# 1099 Osage Exceptional Calculation Narrative

EC  1: EnergyStar Appliances

Process loads in the units were estimated using Energy Star data and specifications from the design team. Energy Star rated refrigerators and dish washers are being installed in the facility.  With a average unit size of 700 SF, the improved appliances reduced the equipment power density from 0.60 W/SF to 0.54 W/SF.

EC  2: Residential Lighting

The lighting is modeled at 1.7 W/SF in both the proposed and baseline models. Credit for efficient lighting is taken as an exceptional calculation.

LEED-NC v2.2 CIR Rulings dated 04/25/07 and 03/23/07 state to use up to 2.0 W/sf for residential units. Table 5-10 of U.S. Lighting Market Characterization: Volume 1: National Lighting Inventory and Energy Consumption Estimate by Navigant Consulting gives the value of 2.0W/SF as a typical installed wattage in multifamily buildings. The Tacoma Public Utilities study, "Residential Lighting: The Data to Date" (1996), found that typical residential lighting power densities are closer to 1.7 W/sf. For this project a conservative value of 1.7 W/SF was used.

Hard-wired lighting fixtures are provided throughout the residential units to provide illuminance for every space in the units.  This was a major cost premium for the developer, and was done as an energy efficiency strategy: to prevent residents from having to bring in their own lamps. The installed lighting power density in the proposed design is 0.33 W/SF.