ECM Implementation and Capital Improvement Plan – Example

The threshold for low-cost energy conservation measures (ECM) has been set at <$ value> for this project. The table below includes all no-cost and low-cost measures identified by the energy audit. Note that all no-cost and low-cost measures were implemented during the LEED-EBOMv4 performance period. All capital measures will be budgeted for and completed within the next five years. The targeted completion timeframe for each capital measure is included in the table below.

| ECM | Description | Type | Implementation Status |
| --- | --- | --- | --- |
| 1 | Implement static pressure reset strategy at floor AHUs | Low-cost | Completed November 15, 2017 |
| 2 | Replace AHU access door gaskets | Low-cost | Completed October 25, 2017 |
| 3 | Install photocell controller for exterior lighting | Low-cost | Completed January 20, 2018 |
| 4 | Upgrade lobby and common areas lighting with LED lamps | Capital | Completion planned for May 2018 |
| 5 | Install carbon monoxide sensors for garage ventilation | Capital | Completion planned for August 2018 |