| Segment ID | Segment status | Source segment | Target segment |
| --- | --- | --- | --- |
| 1a25b1ab6-5fa0-4895-a515-9533b1823771 | Translation Approved (0%) | Updated to reflect the July 1, 2014 LEED v4 Building Operations and Maintenance Addenda | Atualizado para refletir o Adendo de 1º de julho de 2014 do LEED v4 para Operação e Manutenção de Edifícios (LEED v4 Building Operations and Maintenance) |
| 23620f665-2615-4ecf-83cb-83f6589d54f4 | Translation Approved (0%) | <38>LEED </38><39>v</39><40>4</40><41> for</41> | <38>LEED </38><39>v</39><40>4</40><41> para</41> |
| 39d3e7e44-8f65-425c-84f4-cedd01c6a652 | Translation Approved (91%) | BUILDING OPERATIONS AND MAINTENANCE | OPERAÇÕES E MANUTENÇÃO DE EDIFÍCIOS VERDES (LEED v4 for BUILDING OPERATIONS AND MAINTENANCE) |
| 4fc517c35-44d6-430a-807d-5449ce530090 | Translation Approved (0%) | Updated July 1, 2014 | Atualizado em 1º de julho de 2014 |
| 5e2624d43-ed8b-45dd-ae9f-2006b7165ee8 | Translation Approved (0%) | Includes: | Inclui: |
| 62c121430-38a0-4940-a81b-b889de18b4bd | Translation Approved (0%) | LEED O+M: Existing Buildings | LEED O+M: Edifícios Existentes (.Existing Buildings) |
| 7c7ef51ed-4e86-4e64-9e09-696dd4b4a639 | Translation Approved (80%) | LEED O+M: Schools | LEED O+M: Escolas (Schools) |
| 849982708-2c50-41a6-be0e-6e12053ed7f8 | Translation Approved (83%) | LEED O+M: Retail | LEED O+M: Varejo (Retail) |
| 93a72cf47-c348-48b3-9eb9-2128ffc0c609 | Translation Approved (73%) | LEED O+M: Data Centers | LEED O+M: Centros de Dados (Data Centers) |
| 1010d030dc-cfc4-494f-9da9-ce4c075a2a16 | Translation Approved (83%) | LEED O+M: Hospitality | LEED O+M: Hoteleiro (Hospitality) |
| 11f0fd37db-22b2-41a7-a0c0-dab00772b81a | Translation Approved (78%) | LEED O+M: Warehouses and Distribution Centers | LEED O+M: Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) |
| 1228018230-2a14-4dcd-8816-1f3acd27faa7 | Translation Approved (100%) | Location and Transportation (LT) | Local e Transporte (Location and Transportation - LT) |
| 13e3006e41-87bd-47ac-b3da-0ff40873af57 | Translation Approved (0%) | LT Credit: Alternative Transportation | Crédito LT: Transporte Alternativo (LT Credit: Alternative Transportation) |
| 141cd32095-fbbf-483f-9cf6-25623aa5b216 | Translation Approved (0%) | EB:O&M | EB:O&M |
| 1579903eef-0c58-4f44-a731-efbb7103ce03 | Translation Approved (89%) | 1–15 points | 1 a 15 pontos |
| 16e877f481-8289-469f-a16e-c053617092df | Translation Approved (0%) | This credit applies to | Este crédito se aplica a |
| 174e73e5ce-f5ac-4c87-abfd-c96d1519520c | Translation Approved (0%) | Existing Buildings: Operations & Maintenance (1–15 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 a 15 pontos) |
| 18c2c21358-3132-45a5-8af2-ab631cbad17e | Translation Approved (0%) | EB:O&M Schools (1–15 points) | EB:O&M Escolas (Schools) (1 a 15 pontos) |
| 193f2eb1c9-df6b-4546-8e63-5d94b740a4f6 | Translation Approved (89%) | EB:O&M Retail (1–15 points) | EB:O&M Varejo (Retail) (1 a 15 pontos) |
| 2030871bd3-1f85-4526-87c0-85d9ce799ad2 | Translation Approved (0%) | EB:O&M Data Centers (1–15 points) | EB:O&M Centros de Dados (Data Centers) (1 a 15 pontos) |
| 214691562d-2c9b-4c4d-9056-884450871bc8 | Translation Approved (89%) | EB:O&M Hospitality (1–15 points) | EB:O&M Hoteleiro (Hospitality) (1 a 15 pontos) |
| 22b8143449-f615-415d-b2c7-fecb63c28a4e | Translation Approved (81%) | EB:O&M Warehouses & Distribution Centers (1–15 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 a 15 pontos) |
| 23d6696767-533a-4fd5-8ae8-58a08cf27276 | Translation Approved (100%) | Intent | Objetivo |
| 246039e89b-6928-4fdc-96d4-950b19acc740 | Translation Approved (81%) | To reduce pollution and land development effects from automobile use for transportation. | Reduzir os efeitos da poluição e do desenvolvimento de terrenos do uso de automóveis para transporte. |
| 25516d7d87-e2a5-4f0b-b49e-2e517907ff24 | Translation Approved (100%) | Requirements | Requisitos |
| 26977daecc-1e28-48e5-8062-fd698cd555db | Translation Approved (86%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 27e23f1356-8a43-4363-ba7f-a99a28885db8 | Translation Approved (0%) | ESTABLISHMENT | ESTABELECIMENTO |
| 28bd7b55a1-42b5-4d48-9459-2fe0ac743cd7 | Translation Approved (0%) | None. | Nenhum. |
| 291a6915f7-3a44-4fc1-990a-ee1565fd6f78 | Translation Approved (0%) | PERFORMANCE | DESEMPENHO |
| 30b24ac2c7-ca4f-4e8d-bab7-a392ef6549ab | Translation Approved (98%) | Option 1. | Opção 1. |
| 31b24ac2c7-ca4f-4e8d-bab7-a392ef6549ab | Translation Approved (0%) | Transportation Survey (1 point) | Pesquisa sobre transporte (1 ponto) |
| 3217099986-8a0d-437c-9427-12af5b0fec0b | Translation Approved (0%) | Conduct a survey of building occupants on their transportation patterns. | Realize uma pesquisa com os ocupantes do edifício sobre seus padrões de transporte. |
| 3317099986-8a0d-437c-9427-12af5b0fec0b | Translation Approved (0%) | Regular building occupants must be surveyed. | Os ocupantes regulares do edifício devem ser pesquisados. |
| 3417099986-8a0d-437c-9427-12af5b0fec0b | Translation Approved (0%) | Visitors must be surveyed if either the typical peak or daily average is greater than the number of regular building occupants. | Os visitantes podem ser pesquisados se o pico típico ou a média diária for maior do que o número de ocupantes regulares do edifício. |
| 35103bda86-2d45-4feb-9e92-5cd1578e714a | Translation Approved (0%) | Conduct a transportation survey at least once every five years. | Realize uma pesquisa sobre transporte pelo menos uma vez a cada cinco anos. |
| 3617112336-09ef-4f5a-a78c-df008cfcba2c | Translation Approved (100%) | OR | OU |
| 37c8d2939d-a041-4905-9df6-1b076d53b757 | Translation Approved (100%) | Option 2. | Opção 2. |
| 38c8d2939d-a041-4905-9df6-1b076d53b757 | Translation Approved (0%) | Alternative Transportation Rate (3–15 points) | Taxa de transporte alternativo (3 a 15 pontos) |
| 399a150d0c-df79-4d62-93ae-18624fc5fc03 | Translation Approved (100%) | Meet the requirements of Option 1. | Atenda aos requisitos da Opção 1. |
| 40a4cfe083-5681-4995-8b48-c5efe4c2bb07 | Translation Approved (0%) | Demonstrate an alternative transportation rate in accordance with Table 1. | Demonstre uma taxa de transporte alternativo de acordo com a Tabela 1. |
| 41a4cfe083-5681-4995-8b48-c5efe4c2bb07 | Translation Approved (0%) | Alternative transportation strategies that contribute to this reduction include human-powered conveyances (e.g. walking or biking), <62>public transit</62>, telecommuting, rideshare options, compressed workweeks, carpools, and <63>green vehicles</63>. | Estratégias de transporte alternativo que contribuem para essa redução incluem métodos de transporte realizados por seres humanos (como caminhar ou andar de bicicleta), <62>transporte público</62>, teletrabalho, opções de carona solidária, semanas de trabalho comprimidas, caronas solidárias e <63>veículos verdes</63>. |
| 42e2844bb0-8678-479f-8356-ef2dc440191b | Translation Approved (76%) | Calculations are performed relative to a baseline case that assumes all regular occupants commute alone in conventional automobiles. | Os cálculos são realizados tendo em vista um caso de baseline que pressupõe que todos os ocupantes regulares viajam sozinhos ao trabalho em automóveis convencionais. |
| 43e2844bb0-8678-479f-8356-ef2dc440191b | Translation Approved (0%) | The calculations must account for seasonal variations in the use of alternative commuting methods and should indicate the distribution of commuting trips using each type of alternative transportation strategy. | Os cálculos devem levar em conta as variações sazonais no uso de métodos de deslocamento alternativo e devem indicar a distribuição das viagens de deslocamento utilizando cada tipo de estratégia de transporte alternativo. |
| 44b519c50e-7061-4941-8689-076e7f85f427 | Translation Approved (100%) | Table 1. | Tabela 1. |
| 45b519c50e-7061-4941-8689-076e7f85f427 | Translation Approved (0%) | Points for alternative transportation rate | Pontos para taxa de transporte alternativo |
| 46bf503d92-2d2e-4325-a432-17af5dca8810 | Translation Approved (70%) | Alternative Transportation Rate | Taxa de transporte alternativo |
| 4786566bc9-d8e6-4832-913a-80fcd3db9f39 | Translation Approved (100%) | Points | Pontos |
| 484d92e217-9e24-41d8-9d69-6e57d66d15fc | Translation Approved (100%) | 10% | 10% |
| 494618681a-f904-4cfd-959f-6470487a5bd3 | Translation Approved (100%) | 3 | 3 |
| 50a5e12c6e-db2c-4b81-8838-5894d3620314 | Translation Approved (100%) | 15% | 15% |
| 512ad448db-9ca0-4672-a999-1cff099e61d5 | Translation Approved (100%) | 4 | 4 |
| 52361ad913-e94f-4106-827a-a683658c58d4 | Translation Approved (100%) | 20% | 20% |
| 5335bf7c71-1a86-45f5-8c92-e867f54ee4fd | Translation Approved (100%) | 5 | 5 |
| 541b8d2bea-cf97-41b5-bd90-3524b487b1a8 | Translation Approved (100%) | 25% | 25% |
| 55f84810b4-fd0d-4188-8b43-e011060a252a | Translation Approved (100%) | 6 | 6 |
| 565e952380-3820-4049-aca7-e0b71a88fec1 | Translation Approved (100%) | 30% | 30% |
| 5725dc017b-3619-457a-a0c2-e2f502201037 | Translation Approved (100%) | 7 | 7 |
| 588b113e7f-bec1-4a8c-aa5d-9313dfd1039c | Translation Approved (100%) | 35% | 35% |
| 59a634c0c8-c576-4642-a326-e24fc5ad47e0 | Translation Approved (100%) | 8 | 8 |
| 60ae4ad456-6f07-4e44-937c-d6c46fdcbce7 | Translation Approved (100%) | 40% | 40% |
| 61f2c78c4a-5554-49b6-8599-29bcb088bb7b | Translation Approved (100%) | 9 | 9 |
| 6277cdd2a7-87d5-409d-9035-5f08a14bac42 | Translation Approved (100%) | 45% | 45% |
| 63d7e59cb7-942c-4293-a922-105d85122b03 | Translation Approved (100%) | 10 | 10 |
| 64429478b9-6f02-43b7-ae29-dd00a555d36c | Translation Approved (100%) | 50% | 50% |
| 65f61f9dba-ff8a-4c1b-8672-55fecc836c1b | Translation Approved (100%) | 11 | 11 |
| 66a78fcd31-63a0-47b0-bb0b-98b2d4fdb1a2 | Translation Approved (100%) | 55% | 55% |
| 674b0012c3-ab01-4faf-9b77-0569f4eeceb5 | Translation Approved (100%) | 12 | 12 |
| 686729a106-84b0-49e5-a0c5-f3841deca9a6 | Translation Approved (100%) | 60% | 60% |
| 69dd603034-8ada-4743-81ce-f84f9ee75981 | Translation Approved (100%) | 13 | 13 |
| 70e95b5ec0-1066-4f97-ae98-6040df7f8080 | Translation Approved (100%) | 65% | 65% |
| 71b5ba3dcd-2a86-4b11-88dc-a6abf28b7f90 | Translation Approved (100%) | 14 | 14 |
| 7262773404-cbf6-41ae-b004-efd73f0d1029 | Translation Approved (100%) | 70% | 70% |
| 73e2e5314a-b664-431c-a8d0-b8c933804e11 | Translation Approved (100%) | 15 | 15 |
| 74a991233d-8fb3-4c0f-bdc1-7438b1655b20 | Translation Approved (100%) | OR | OU |
| 750a926c42-a69c-4e86-ac0f-d49a20b34d22 | Translation Approved (100%) | Option 3. | Opção 3. |
| 760a926c42-a69c-4e86-ac0f-d49a20b34d22 | Translation Approved (0%) | Comprehensive Alternative Transportation Program (2 points) | Programa abrangente de transporte alternativo (2 pontos) |
| 77a1d9d524-801d-429c-8e0a-8ed42f0a7bb7 | Translation Approved (100%) | Meet the requirements of Option 1. | Atenda aos requisitos da Opção 1. |
| 787acabec1-34ae-4057-af5c-1040cc7e8fec | Translation Approved (0%) | Implement an alternative transportation program to reduce the conventional travel rates of building occupants. | Implemente um programa de transporte alternativo para reduzir as taxas de deslocamento convencional dos ocupantes do edifício. |
| 797acabec1-34ae-4057-af5c-1040cc7e8fec | Translation Approved (0%) | Include at least one element from each of the following three categories: | Inclua no mínimo um elemento de cada uma das três categorias a seguir: |
| 80a49ac2ba-abfd-44ee-8f2e-bb09e0a07758 | Translation Approved (0%) | Education strategies | Estratégias educacionais |
| 81eff9eb8c-a86a-4253-9f6e-d60a8918d4d6 | Translation Approved (0%) | new-hire orientation; | orientação para novas contratações; |
| 822a5867a5-64d7-47cd-b700-70cf622f1d8a | Translation Approved (0%) | employee newsletter, flyer, announcements, memos, letters; | boletim informativo, folheto, anúncios, lembretes, cartas para funcionários; |
| 837b9d8cc0-970b-474a-9886-f78823039c6c | Translation Approved (0%) | carpool matching website; or | site de correspondência de carona solidária; ou |
| 8421207e87-9c28-46bd-bb17-58fbe5dda31f | Translation Approved (0%) | Basic support strategies | Estratégias de suporte básicas |
| 8526ffae4d-cb3c-4b6f-a39d-e8afa9e91cc9 | Translation Approved (0%) | employer carpool events. | eventos de carona solidária do empregador; |
| 863943120b-66c8-4314-966e-ea4cb9cee5fd | Translation Approved (0%) | guaranteed return trip; | viagem de retorno garantida; |
| 8760fc17a7-c8b2-4d2a-b738-c62f1222ab16 | Translation Approved (0%) | preferential parking for rideshare participants; | estacionamento preferencial para participantes de carona solidária; |
| 88d5df31dc-a660-4d37-8ea6-e811d2a5bedf | Translation Approved (0%) | flextime schedule; or | cronograma flexível; ou |
| 89309ba454-c963-480d-9b5d-fb424a2d148b | Translation Approved (0%) | ride-matching service. | serviço de correspondência de caronas. |
| 904708adf9-f38d-4d59-9ec2-d730724a9b4f | Translation Approved (0%) | Direct strategies | Estratégias diretas |
| 912c70afec-bc06-4fe4-9337-72cecb25030f | Translation Approved (0%) | telecommuting; | teletrabalho; |
| 920b1098f9-0939-47f8-adc3-3786c9d74bfa | Translation Approved (0%) | compressed workweek schedule; | cronograma de semana de trabalho comprimida; |
| 930864b10c-af49-4cd1-8cff-0ef2e7f35ed4 | Translation Approved (0%) | transit subsidy; | subsídio para transporte; |
| 94c88989b0-da66-4d89-abeb-ad570480edf8 | Translation Approved (0%) | introduction of a parking fee; | introdução de uma taxa de estacionamento; |
| 95af4fe23d-c6e9-430c-80d5-342c139f7ec9 | Translation Approved (0%) | bicycle program; | programa para bicicletas; |
| 96ae8a8542-7f21-4fcb-ab48-bd2454b80fc4 | Translation Approved (0%) | parking cash-out; | pagamento de estacionamento; |
| 97d3a0b211-5c73-49c0-b09d-413c3cb0ada3 | Translation Approved (0%) | employee clean vehicle purchase program; or | programa de compra de veículo limpo para funcionários; ou |
| 98ff817937-4ff5-4409-af68-3b0d856b9574 | Translation Approved (0%) | carpool program. | programa de carona solidária. |
| 992088a35b-dd1a-45f3-b914-cf9f8e11110e | Translation Approved (100%) | Sustainable Sites (SS) | Terrenos Sustentáveis (Sustainable Sites - SS) |
| 1007b353a57-b1ae-49cb-9a55-0bb5548dab30 | Translation Approved (75%) | SS Prerequisite: Site Management Policy | Pré-requisito SS: Política de Gestão de Terreno (SS Prerequisite: Site Management Policy) |
| 1010bdd2a38-808d-4219-b90a-5d7edb9fbb28 | Translation Approved (100%) | Required | Obrigatório |
| 102cbce1f5d-137b-4c8a-b6e2-3342d79755a9 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 1034b686e5e-fecb-4d1b-8928-fddb8c0b63a9 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 1042267f11d-ff93-4d2d-9603-79a9a4e4b086 | Translation Approved (91%) | Existing Buildings: Operations & Maintenance | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) |
| 105b460d360-9734-4f5b-befb-7231ba124a5f | Translation Approved (72%) | EB:O&M Schools | EB:O&M Escolas (Schools) |
| 10611150897-1193-45d2-9550-8bb097b07cfb | Translation Approved (77%) | EB:O&M Retail | EB:O&M Varejo (Retail) |
| 107560ef612-f54d-4550-a950-39ea392f2c9f | Translation Approved (0%) | EB:O&M Data Centers | EB:O&M Centros de Dados (Data Centers) |
| 108dc0b16c6-2f97-463c-85fd-11750a8f6642 | Translation Approved (77%) | EB:O&M Hospitality | EB:O&M Hoteleiro (Hospitality) |
| 1097c55d741-b3de-49bd-a95b-5b00907e07ff | Translation Approved (72%) | EB:O&M Warehouses & Distribution Centers | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) |
| 110b218a381-984c-4337-b282-a607d5a3691c | Translation Approved (100%) | Intent | Objetivo |
| 1116afc5aa5-20fb-42bc-8599-2698e42ef191 | Translation Approved (0%) | To preserve ecological integrity and encourage environmentally sensitive site management practices that provide a clean, well-maintained, and safe building exterior while supporting high-performance building operations and integration into the surrounding landscape. | Preservar a integridade ecológica e incentivar práticas de gestão do terreno ambientalmente sensíveis que propiciem um exterior de edifício limpo, bem mantido e seguro, mas favorecendo também operações de edifícios de alto desempenho e sua integração à paisagem vizinha. |
| 112442e4af7-2103-46cf-b971-fe2511828885 | Translation Approved (100%) | Requirements | Requisitos |
| 1131ed99ba1-b9be-4d9a-b6ee-36879abb9b8c | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1146ed4eb55-4180-4b92-8cb2-65fb9d45580c | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 115ee3df622-a238-4926-b4e8-8dc428c1f21f | Translation Approved (0%) | Create and implement a site management policy that employs best management practices to reduce harmful chemical use, energy waste, water waste, air pollution, solid waste, and/or chemical runoff for all of the following operational elements on the building and grounds: | Crie e implemente uma política de gestão do terreno que empregue as melhores práticas de gestão para reduzir uso de produtos químicos nocivos, desperdício de energia, desperdício de água, poluição atmosférica, resíduos sólidos e/ou escoamento superficial de produtos químicos para todos os seguintes elementos operacionais no edifício e terreno: |
| 116c151dc25-19e9-4e57-82e4-8f332bed6a72 | Translation Approved (0%) | use of low emissions maintenance equipment; | uso de equipamento de manutenção de baixas emissões; |
| 1176826b7fa-b610-4681-a868-9a0b450d7973 | Translation Approved (0%) | snow and ice removal; | remoção de neve e gelo; |
| 1185c4f62bb-a6da-411d-9687-114c116eb0c9 | Translation Approved (0%) | cleaning of building exterior, pavement, and other impervious surfaces; | limpeza do exterior, da pavimentação e de outras superfícies impermeáveis; |
| 119201b2744-6990-4db9-8611-f28985fbca31 | Translation Approved (0%) | erosion and sedimentation control (for ongoing operations and for construction activity); | controle de erosão e sedimentação (para operações em andamento e para a atividade de construção); |
| 120d2ec0b8a-ca79-4ccc-a906-2fddd01deef8 | Translation Approved (0%) | organic waste management (returned to the site or diverted from landfills); | gestão de resíduos orgânicos (devolvidos ao local ou desviados de aterros sanitários); |
| 12141b01210-56f2-48c8-ab0c-edf4f9d3ab48 | Translation Approved (0%) | invasive and exotic plant species management (through monitoring and eradication); | gestão de espécies de plantas exóticas e invasoras (por meio de monitoramento e erradicação); |
| 122d1468a24-b9b0-4888-8887-8bae4a75e1df | Translation Approved (0%) | fertilizer use (testing soils before using fertilizer to prevent overapplication of nutrients); | uso de fertilizante (testar os solos antes de utilizar um fertilizante para evitar a aplicação excessiva de nutrientes); |
| 123ac6adc61-e0f3-4953-b8d5-e4ca0cd17426 | Translation Approved (0%) | irrigation management (monitor irrigation systems manually or with automated systems at least every two weeks during the operating season for appropriate water usage, system times, leaks, or breaks); and | gestão de irrigação (monitorar sistemas de irrigação manualmente ou utilizando sistemas automáticos pelo menos quinzenalmente durante a época de operação para verificar o uso de água adequado, os tempos do sistema, vazamentos ou rupturas); e |
| 124b538af23-d3d3-4b52-b10a-46e22824f2dc | Translation Approved (0%) | storage of materials and equipment. | armazenamento de materiais e equipamentos. |
| 125333e20f9-30a7-4484-933f-35232171b325 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 126f2c414c3-9744-4b13-9d5a-deba60554b5b | Translation Approved (100%) | None. | Nenhum. |
| 12783bf9437-9ba7-4544-bf5c-25b8040b675f | Translation Approved (100%) | SS Credit: Site Development—Protect or Restore Habitat | Crédito SS: Desenvolvimento do Terreno - Proteção ou Restauração do Habitat (SS Credit: Site Development—Protect or Restore Habitat) |
| 128e445b774-906c-4e53-b31d-314650cb3c58 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 129071fb632-0702-46b3-8595-737674d31da4 | Translation Approved (100%) | 1–2 points | 1 a 2 pontos |
| 130472eaa02-6daa-4305-8e31-b182974312dc | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 1312600e6cf-c075-46f3-b8d5-07d6130f1aa6 | Translation Approved (96%) | Existing Buildings: Operations & Maintenance (1–2 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 a 2 pontos) |
| 132ef1c7ca0-71dc-4280-8029-a2a63461d3b2 | Translation Approved (95%) | EB:O&M Schools (1–2 points) | EB:O&M Escolas (Schools) (1 a 2 pontos) |
| 1337eaaa933-8c48-4e89-9e03-f196bdb063f4 | Translation Approved (95%) | EB:O&M Retail (1–2 points) | EB:O&M Varejo (Retail) (1 a 2 pontos) |
| 134664419fc-14c6-4552-a98e-f7a59d017b99 | Translation Approved (96%) | EB:O&M Data Centers (1–2 points) | EB:O&M Centros de Dados (Data Centers) (1 a 2 pontos) |
| 135c15af1db-c76a-4351-b5d8-cf199a50235f | Translation Approved (95%) | EB:O&M Hospitality (1–2 points) | EB:O&M Hoteleiro (Hospitality) (1 a 2 pontos) |
| 1367db036cd-cc30-4ee1-a8e6-f3bae15bb42c | Translation Approved (97%) | EB:O&M Warehouses & Distribution Centers (1–2 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 a 2 pontos) |
| 13797725cbd-48f7-465d-b4f9-b6097b3e0a75 | Translation Approved (100%) | Intent | Objetivo |
| 138e0eb63e2-3342-4d4e-b890-b795c33de078 | Translation Approved (CM) | To conserve existing natural areas and restore damaged areas to provide habitat and promote biodiversity. | Preservar as áreas naturais existentes e restaurar áreas danificadas para proporcionar habitat e promover a biodiversidade. |
| 139e2366bdd-cc73-4cf5-ab1f-408adfd0e3d1 | Translation Approved (100%) | Requirements | Requisitos |
| 140d5cdc9b2-468e-495a-b4a9-5c7487698cd9 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 14111504a2c-a691-4db6-9bd5-98e6c224e278 | Translation Approved (100%) | Option 1. | Opção 1. |
| 14211504a2c-a691-4db6-9bd5-98e6c224e278 | Translation Approved (0%) | On-Site Restoration (2 points) | Restauração no local (2 pontos) |
| 1431c2f358d-d5de-404a-90b3-65764f569f43 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 144a82248ac-8647-4418-9f7a-72df965d4542 | Translation Approved (0%) | Have in place native or adapted vegetation<176> </176>on 20% of the total site area (including the building footprint), a minimum of 5,000 square feet (465 square meters), to provide habitat and promote biodiversity. | Implante vegetação nativa ou adaptada em 20% do área total do terreno (incluindo a pegada ecológica), mínimo de 5.000 pés quadrados (465 metros quadrados) para fornecer habitat e promover biodiversidade. |
| 14542fa39a0-d2b3-498c-a33d-59ccee1f4a27 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 1467eebd360-e5d2-497c-8dab-0f84f0d4d872 | Translation Approved (100%) | None. | Nenhum. |
| 1475bbee288-1cac-463e-87e1-d555bd9aa979 | Translation Approved (100%) | OR | OU |
| 1489c546af6-d9b4-44b3-93b7-9ca07e2120a9 | Translation Approved (100%) | Option 2. | Opção 2. |
| 1499c546af6-d9b4-44b3-93b7-9ca07e2120a9 | Translation Approved (CM) | Financial Support (1 point) | Apoio financeiro (1 ponto) |
| 1501d04b2ca-9d4f-4f1a-bf54-15c75d13afe3 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 15115ebbbdd-95aa-4655-a109-f11fe1f6d51a | Translation Approved (96%) | Provide financial support equivalent to at least $0.05 per square foot (US$0.50 per square meter) for the total site area (including the building footprint). | Forneça apoio financeiro equivalente a pelo menos US$ 0,05 por pé quadrado (US$ 0,50 por metro quadrado) da área total do terreno (incluindo a pegada ecológica). |
| 152a7da58cd-c996-4441-aead-c63f25368598 | Translation Approved (94%) | Financial support must be provided annually to a nationally or locally recognized land trust or conservation organization within the same EPA Level III ecoregion or the project’s state (or within 100 miles [160 kilometers] for projects outside the U.S.). | O apoio financeiro pode ser oferecido anualmente a um land trust ou uma organização de preservação reconhecida nacional ou localmente na mesma ecorregião Nível III da EPA (EUA) ou no estado do projeto (ou a até 100 milhas [160 km] para projetos fora dos EUA). |
| 153a7da58cd-c996-4441-aead-c63f25368598 | Translation Approved (100%) | For U.S. projects, the land trust must be accredited by the Land Trust Alliance. | Para projetos nos EUA, o land trust deve ser reconhecido pela Land Trust Alliance. |
| 1543a974e21-5004-46a0-8eb2-5453e6f7f24b | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 155adbcf158-65c8-47c6-ad6d-ddd5698940ec | Translation Approved (0%) | Provide the specified financial support annually. | Forneça o apoio financeiro especificado anualmente. |
| 156cdf82ec1-7437-4ffb-8190-7fea0b133d0e | Translation Approved (100%) | SS Credit: Rainwater Management | Crédito SS: Gestão de Águas Pluviais (SS Credit: Rainwater Management) |
| 157502d6d85-a141-46a4-8f6c-4b595d641a8d | Translation Approved (100%) | EB:O&M | EB:O&M |
| 158dfa55878-2ffc-4873-8589-39bc12320456 | Translation Approved (89%) | 2–3 points | 2 a 3 pontos |
| 159cd2ed9ba-81ba-4f07-9950-b891eb19f585 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 16081e529ae-e1ba-412c-a96b-4e339d3b29d9 | Translation Approved (91%) | Existing Buildings: Operations & Maintenance (3 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (3 pontos) |
| 1614c8b5186-606e-49f6-aeae-bf11a7962756 | Translation Approved (89%) | EB:O&M Schools (2 points) | EB:O&M Escolas (Schools) (3 pontos) |
| 1629479625a-9f61-402d-806c-31de25d2e62e | Translation Approved (89%) | EB:O&M Retail (3 points) | EB:O&M Varejo (Retail) (3 pontos) |
| 16353bd33cc-f63a-461c-9faf-df0006b7f796 | Translation Approved (91%) | EB:O&M Data Centers (3 points) | EB:O&M Centros de Dados (Data Centers) (3 pontos) |
| 16467057f74-f9f5-4dbe-99a4-93623d82c40b | Translation Approved (89%) | EB:O&M Hospitality (3 points) | EB:O&M Hoteleiro (Hospitality) (3 pontos) |
| 165c343ee7d-932b-4f77-a46f-c94a6c60fd52 | Translation Approved (92%) | EB:O&M Warehouses & Distribution Centers (3 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (3 pontos) |
| 16672c65ffb-b3ef-4b8c-842b-cc1215baa493 | Translation Approved (100%) | Intent | Objetivo |
| 167a825a865-ab37-461b-bc16-d5a900e6a4ce | Translation Approved (CM) | To reduce runoff volume and improve water quality by replicating the natural hydrology and water balance of the site, based on historical conditions and undeveloped ecosystems in the region. | Reduzir o volume de escoamento superficial e melhorar a qualidade da água replicando a hidrologia natural e o balanço hídrico do terreno, com base em condições históricas e ecossistemas não desenvolvidos na região. |
| 168e7a5f10c-0874-447a-8d29-3d2dfaf95cd6 | Translation Approved (100%) | Requirements | Requisitos |
| 169506199a1-2846-4ba6-912e-00dcbf4eff36 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1708b7dad38-5bb5-4306-9e67-c0c354b7167d | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 1711b9af199-c484-4b4f-8a4e-133756b92a24 | Translation Approved (0%) | Use <186>low-impact development (LID)</186> practices to capture and treat water from 25% of the impervious surfaces for the 95th percentile storm event. | Use práticas de desenvolvimento de baixo impacto (<186>low-impact development</186> - LID) para capturar e tratar a água de 25% das superfícies impermeáveis do evento pluviométrico de percentil 98. |
| 1727f62ed2f-f067-48b5-9c62-db036d434cc9 | Translation Approved (0%) | Establish and implement an annual inspection program of all rainwater management facilities to confirm continued performance. | Estabeleça e implemente um programa de inspeção anual de todas as instalações de gestão de águas pluviais para confirmar o desempenho contínuo. |
| 1730ebe48a6-9b59-4382-be0e-e0f482142388 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 174a6873352-0d4f-4cb9-b0be-3929770f3b6a | Translation Approved (0%) | Document the annual inspections, including identification of areas of erosion, maintenance needs, and repairs. | Documente as inspeções anuais, incluindo identificação das áreas de erosão, as necessidades de manutenção e reparos. |
| 175a6873352-0d4f-4cb9-b0be-3929770f3b6a | Translation Approved (0%) | Perform necessary maintenance, repairs, or stabilization within 60 days of inspection. | Realize a manutenção, os reparos ou a estabilização necessários em até 60 dias após a inspeção. |
| 17625f8bcdc-5eba-4326-bbe1-d40b41b444fc | Translation Approved (100%) | SS Credit: Heat Island Reduction | Crédito SS: Redução de Ilhas de Calor (SS Credit: Heat Island Reduction) |
| 177b7cc456d-6f70-4152-8e3c-61cc6e1f9bd0 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 17891970c80-2b7d-4a6a-9089-6892aa803b6b | Translation Approved (100%) | 2 points | 2 pontos |
| 1795ae7efb4-d95f-412d-aca2-7c7b5f632801 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 180a5f88c74-bd62-415c-b32b-67b9bcfa76d3 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (2 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (2 pontos) |
| 181ccd6f837-5a09-4555-800f-3e87e1769ba5 | Translation Approved (100%) | EB:O&M Schools (2 points) | EB:O&M Escolas (Schools) (3 pontos) |
| 182d697d451-58d3-40ac-8936-84dc1248f5e1 | Translation Approved (100%) | EB:O&M Retail (2 points) | EB:O&M Varejo (Retail) (2 pontos) |
| 183fd27c670-aea7-4eb2-93fd-d70aef338922 | Translation Approved (100%) | EB:O&M Data Centers (2 points) | EB:O&M Centros de Dados (Data Centers) (2 pontos) |
| 184b98f3e23-009c-4e3c-9c54-d7dfa3adf22b | Translation Approved (100%) | EB:O&M Hospitality (2 points) | EB:O&M Hoteleiro (Hospitality) (2 pontos) |
| 1856b722ed3-56f8-4221-9d1f-7d9aef3c3940 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (2 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (2 pontos) |
| 186a44af451-5d7e-4aa5-9672-4abf7bdf085d | Translation Approved (100%) | Intent | Objetivo |
| 1877a98168b-221b-4a2a-85b7-e3dc6730f5ce | Translation Approved (CM) | To minimize effects on microclimates and human and wildlife habitats by reducing heat islands. | Minimizar os efeitos em microclimas e habitats de seres humanos e vida animal reduzindo ilhas de calor. |
| 188a179f2ea-bb39-48f5-a1eb-ac1636e0a555 | Translation Approved (100%) | Requirements | Requisitos |
| 1890dccd922-f0a4-4f2b-bf4f-d0c9ae35878a | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 190adb1c1e1-fc5b-4eda-8b5b-ed50a8b58840 | Translation Approved (99%) | Choose one of the following options. | Escolha uma das opções a seguir. |
| 191eb514b67-cae7-4241-a43e-5925126dd924 | Translation Approved (100%) | Option 1. | Opção 1. |
| 192eb514b67-cae7-4241-a43e-5925126dd924 | Translation Approved (0%) | Nonroof (1 point) | Sem telhado (1 ponto) |
| 193ba4c07c6-7131-44b9-ac20-400844bec4b1 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 1945fc006b7-4f5e-4834-b70b-160f40ad9e05 | Translation Approved (0%) | Use any combination of the following strategies for a minimum of 50% of the site paving. | Use qualquer combinação das estratégias a seguir para um mínimo de 50% da pavimentação do terreno. |
| 1959a5a0527-3942-4142-bc8f-1be33f45e365 | Translation Approved (100%) | Use the existing plant material or install plants that provide shade over paving areas (including playgrounds) on the site within 10 years of planting. | Use material vegetal existente ou instalar plantas que forneçam sombra sobre áreas pavimentadas (incluindo parques infantis) no terreno em até 10 anos após o plantio. |
| 1969a5a0527-3942-4142-bc8f-1be33f45e365 | Translation Approved (0%) | Plants must be in place at the time of certification application. | As plantas devem estar implementadas no momento da inscrição para a certificação. |
| 197fd0975cc-f896-463a-9792-8a68c11362d9 | Translation Approved (100%) | Install vegetated planters. | Instale vasos com vegetação. |
| 198fd0975cc-f896-463a-9792-8a68c11362d9 | Translation Approved (CM) | Plants must be in place at the time of occupancy permit and cannot include artificial turf. | As plantas devem estar no lugar no momento da licença de ocupação e não podem incluir grama artificial. |
| 199a0f3e995-b59f-4659-87de-a2a3d5efaec1 | Translation Approved (CM) | Provide shade with structures covered by energy generation systems, such as solar thermal collectors, photovoltaics, and wind turbines. | Proporcione sombra com estruturas cobertas por sistemas de geração de energia, como coletores térmicos solares, fotovoltaicos e turbinas eólicas. |
| 20022145577-1d43-4fe3-954d-4a5e4482e9ea | Translation Approved (CM) | Provide shade with architectural devices or structures that have a three-year aged <194>solar reflectance (SR)</194> value of at least 0.28. | Proporcione sombra com dispositivos ou estruturas arquitetônicos que tenham um valor de <Italic>refletância solar (solar reflectance - SR)</Italic> após três anos de pelo menos 0,28. |
| 20122145577-1d43-4fe3-954d-4a5e4482e9ea | Translation Approved (100%) | If three-year aged value information is not available, use materials with an initial SR of at least 0.33 at installation. | Se informações de valor após três anos não estiverem disponíveis, use materiais com SR inicial de pelo menos 0,33 na instalação. |
| 202acad9252-5f2f-4ffd-9c7c-2a9e51ef10fc | Translation Approved (CM) | Provide shade with vegetated structures. | Proporcione sombra com estruturas com vegetação. |
| 2035c6d7fd9-df78-4668-82a7-c2f524b3eec4 | Translation Approved (CM) | Use paving materials with a three-year aged <195>solar reflectance (SR)</195> value of at least 0.28. | Use materiais de pavimentação com valor de <Italic>refletância solar (solar reflectance - SR)</Italic> após três anos de pelo menos 0,28. |
| 2045c6d7fd9-df78-4668-82a7-c2f524b3eec4 | Translation Approved (100%) | If three-year aged value information is not available, use materials with an initial SR of at least 0.33 at installation. | Se informações de valor após três anos não estiverem disponíveis, use materiais com SR inicial de pelo menos 0,33 na instalação. |
| 2050dc160ef-6df0-4b1c-967c-677ae8883c03 | Translation Approved (CM) | Use an <196>open-grid pavement system</196> (at least 50% unbound). | Use um <Italic>sistema de pavimentação de blocos vazados</Italic> (pelo menos 50% não consolidado). |
| 2062be03f24-4955-48e5-b52e-8477d094c616 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 207fb4cc93a-0b8f-419e-b0d7-1cacc265ec4d | Translation Approved (0%) | Implement a maintenance program that ensures all high-reflectance paving surfaces are cleaned at least every three years to maintain good reflectance. | Implemente um programa de manutenção que garanta que todas as superfícies de pavimentação de alta refletância sejam limpas a cada três anos, no mínimo, para manter uma boa refletância. |
| 2085390bebb-2877-4079-bf12-47c420549a78 | Translation Approved (100%) | OR | OU |
| 209ee57c89c-f587-4077-9105-7fa48a49b9ac | Translation Approved (100%) | Option 2. | Opção 2. |
| 210ee57c89c-f587-4077-9105-7fa48a49b9ac | Translation Approved (0%) | Roof (1 point) | Com telhado (1 ponto) |
| 2114f8ad2fb-70a8-41e7-8d9e-a6e0dc8698de | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 2121c8d1322-b902-4269-874d-b030e5c7bb30 | Translation Approved (0%) | Use either roofing materials with a SRI equal to or greater than the values in Table 1 for a minimum of 75% of the roof area, or a vegetated roof for a minimum of 50% of the roof area, or both. | Use materiais de telhado com um SRI igual ou superior aos valores na Tabela 1 para um mínimo de 75% da área do telhado ou um telhado com vegetação para um mínimo de 50% da área do telhado, ou ambos. |
| 2131c8d1322-b902-4269-874d-b030e5c7bb30 | Translation Approved (0%) | If using both high-reflectance and vegetated roof surfaces, meet the following criterion: | Se usar superfícies de alta refletância e superfícies de telhado com vegetação, siga o seguinte critério: |
| 214f210b8cb-16ca-4155-ab0c-902e23a1575a | Translation Approved (100%) | Area of High-Reflectance Roof | Área de telhado com alta refletância |
| 21593322379-025d-4ff7-b675-a8c4aa43f62f | Translation Approved (CM) | Area of Vegetated Roof | Área de telhado com cobertura verde |
| 21677a23d70-5351-4fa5-96a5-92c0d112f42f | Translation Approved (100%) | Total Roof Area | Área total do telhado |
| 217371022f0-a03b-406c-a00d-698565c02072 | Translation Approved (0%) | —————— | —————— |
| 218a74957ad-4df6-49cc-8651-c76a6663ce66 | Translation Approved (0%) | + | + |
| 2197a7759e0-a833-4c8f-88f0-5900ea870c74 | Translation Approved (100%) | —————— | —————— |
| 2209318e710-b013-44a2-9058-6a69c3be5ade | Translation Approved (0%) | ≥ | ≥ |
| 22105589b32-497f-4846-b92d-7397fc083646 | Translation Approved (100%) | 0.75 | 0,75 |
| 222066d41a2-edbf-4135-a72f-cf5e2eeb74cb | Translation Approved (100%) | 0.5 | 0,5 |
| 22391a44582-ed88-46b5-b21c-94a0a983a5f6 | Translation Approved (CM) | Alternatively, an SRI and SR weighted average approach may be used to calculate compliance. | Como alternativa, é possível usar uma abordagem de média ponderada SRI e SR para calcular a conformidade. |
| 224fdcc6aae-89f9-4502-b8c1-5bd8d4e62490 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 22580ec8c16-21df-49b0-b311-76ad643932c9 | Translation Approved (0%) | Implement a maintenance program that ensures all high-reflectance roof surfaces are cleaned at least every three years to maintain good reflectance, and all vegetated roofs are maintained for plant health and good structural condition. | Implemente um programa de manutenção que garanta que todas as superfícies do telhado de alta refletância sejam limpas a cada três anos, no mínimo, para manter uma boa refletância e que todos os telhados com vegetação recebam manutenção para manter a saúde das plantas e uma condição estrutural boa. |
| 2267ad1c948-b4f9-48f4-9f3d-0d81dc593a9e | Translation Approved (100%) | OR | OU |
| 227e3ee752e-26c6-420e-9b93-e92c229b4b6b | Translation Approved (100%) | Option 3. | Opção 3. |
| 228e3ee752e-26c6-420e-9b93-e92c229b4b6b | Translation Approved (0%) | Nonroof and Roof (2 points) | Sem telhado e com telhado (2 pontos) |
| 229aaad9423-5df2-4004-a3b0-58180d0e8c9d | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 2307594035f-091a-4235-85da-aae595e450e8 | Translation Approved (100%) | Meet the following criterion: | Satisfaça o critério a seguir: |
| 231fa944c17-8b42-4796-b77b-2b6017fe6010 | Translation Approved (CM) | Area of Nonroof Measures | Medidas da área sem telhado |
| 232b060fdb5-d1bb-494e-b216-e591a2501036 | Translation Approved (100%) | Area of High-Reflectance Roof | Área de telhado com alta refletância |
| 2334aafb74f-1773-4ba5-ab39-0e4c60075b82 | Translation Approved (100%) | Area of Vegetated Roof | Área de telhado com cobertura verde |
| 23409d8e64f-a262-4973-8d48-dcf3df119a90 | Translation Approved (CM) | Total Site Paving Area | Área pavimentada total do terreno |
| 235ab9b233d-b5be-4d6c-b918-06e3bedcf739 | Translation Approved (100%) | Total Roof Area | Área total do telhado |
| 236e1e6b45a-8f54-461b-ad7d-599393c389ae | Translation Approved (100%) | —————— | —————— |
| 2373e52d4d6-cb92-426f-a3f1-298014260771 | Translation Approved (100%) | + | + |
| 238d866e36a-af43-4e46-aa4a-096af7aada67 | Translation Approved (100%) | —————— | —————— |
| 23995c5483c-6385-4fd9-8cae-fdc24b5319dd | Translation Approved (100%) | + | + |
| 240272a7c2a-b193-4add-9cd5-6552e4e2883a | Translation Approved (100%) | —————— | —————— |
| 241dab89b4d-4da1-4097-b2c8-c46bb1b73545 | Translation Approved (100%) | ≥ | ≥ |
| 24241efadae-dd73-44c2-baf3-07712a803b49 | Translation Approved (100%) | + | + |
| 2430fb37342-c957-49ae-86d2-741bc75c74ee | Translation Approved (100%) | 0.5 | 0,5 |
| 2441bd4a5e1-a294-466f-9b6f-5a286496c918 | Translation Approved (100%) | 0.75 | 0,75 |
| 2456a538832-e831-4fd1-a2b4-ea44b31343f9 | Translation Approved (100%) | 0.50 | 0,50 |
| 2460ad64bff-f772-4267-8d79-c64826989f7e | Translation Approved (100%) | Alternatively, an SRI and SR weighted average approach may be used to calculate compliance. | Como alternativa, é possível usar uma abordagem de média ponderada SRI e SR para calcular a conformidade. |
| 2473d4b4b32-e2a4-49ab-b553-96319b306f69 | Translation Approved (CM) | Use any combination of the following strategies. | Use qualquer combinação das estratégias a seguir. |
| 2484bb29a11-43e8-42da-a76b-b9d8a1001069 | Translation Approved (CM) | Nonroof Measures | Medidas sem telhado |
| 249b5dbcbfd-c276-499b-96b8-c11a3bf9f447 | Translation Approved (0%) | Use the measures listed in Option 1. | Use as medidas listadas na Opção 1. |
| 250b5dbcbfd-c276-499b-96b8-c11a3bf9f447 | Translation Approved (84%) | Plant material must be in place at time of certification application. | O material de plantas deve estar implementado no momento da inscrição para a certificação. |
| 2517d591193-69ac-48a9-ab6b-6d1f9eb8124a | Translation Approved (100%) | High-Reflectance Roof | Telhado de alta refletância |
| 252810847a9-378d-4b49-a270-b4997559f9b7 | Translation Approved (CM) | Use roofing materials that have an SRI equal to or greater than the values in Table 1. | Use materiais de telhado com um SRI igual ou superior aos valores da Tabela 1. |
| 253810847a9-378d-4b49-a270-b4997559f9b7 | Translation Approved (CM) | Meet the three-year aged SRI value. | Atenda ao valor de SRI após três anos. |
| 254810847a9-378d-4b49-a270-b4997559f9b7 | Translation Approved (CM) | If three-year aged value information is not available, use materials that meet the initial SRI value. | Se as informações de valor após três anos não estiverem disponíveis, use materiais que atendam ao valor inicial de SRI. |
| 25532784d92-b16c-49bf-8729-10fcfeee1f76 | Translation Approved (100%) | Table 1. | Tabela 1. |
| 25632784d92-b16c-49bf-8729-10fcfeee1f76 | Translation Approved (CM) | Minimum solar reflectance index value, by roof slope | Valor mínimo do índice de refletância solar, por inclinação do telhado |
| 25755796557-418e-4728-88fa-a3bf8e2cc122 | Translation Approved (CM) | Slope | Inclinação |
| 258fa49adbe-9a0c-43fe-ae67-7ccb79492a46 | Translation Approved (CM) | Initial SRI | SRI inicial |
| 2598534e19a-14d9-4803-a448-a3d3c28fc5bc | Translation Approved (CM) | 3-year aged SRI | SRI após 3 anos |
| 26096840349-e50d-4a2f-b1d7-e55c86b22727 | Translation Approved (CM) | Low-sloped roof | Telhado pouco inclinado |
| 261085e7b0c-f433-45cc-84e5-70eb6bc20bb1 | Translation Approved (CM) | ≤ 2:12 | ≤ 2:12 |
| 2627baccef1-d06e-4d67-8773-c0a355657007 | Translation Approved (100%) | 82 | 82 |
| 263d5ffc9bf-c2db-4e71-9ddd-ff3663b88770 | Translation Approved (100%) | 64 | 64 |
| 264f4a43c79-3418-4d2c-bb35-e84672da0276 | Translation Approved (CM) | Steep-sloped roof | Telhado muito inclinado |
| 265aca4bb65-bf80-446d-bf82-38fcf644a421 | Translation Approved (CM) | > 2:12 | > 2:12 |
| 2664322d5b8-2574-4382-bd93-3370cd8c660a | Translation Approved (100%) | 39 | 39 |
| 2673ebe16c3-6b5a-42a6-bd69-dcc1ab1b1c7a | Translation Approved (100%) | 32 | 32 |
| 268a29838b5-e608-4702-9e37-9fc73df46c54 | Translation Approved (CM) | Vegetated Roof | Telhado com cobertura verde |
| 2695eb6c58a-87f1-4676-b1ff-10c8c886268a | Translation Approved (CM) | Install a vegetated roof. | Instale um telhado com cobertura verde. |
| 27077e54912-d1bb-45b4-b537-b1bb1379158f | Translation Approved (0%) | <339>PERFORMANCE </339>Implement a maintenance program that ensures all high-reflectance surfaces are cleaned at least every three years to maintain good reflectance, and all vegetated roofs are maintained for plant health and good structural condition. | <339>DESEMPENHO </339>Implemente um programa de manutenção que garanta que todas as superfícies de alta refletância sejam limpas a cada três anos, no mínimo, para manter uma boa refletância e que todos os telhados com vegetação recebam manutenção para manter a saúde das plantas e uma condição estrutural boa. |
| 271e757c6ef-9d7b-439e-af5b-6a6f3ea26806 | Translation Approved (100%) | OR | OU |
| 272e71ab851-6ef5-4c5d-bf26-0d2c686ca20a | Translation Approved (100%) | Option 4. | Opção 4. |
| 273e71ab851-6ef5-4c5d-bf26-0d2c686ca20a | Translation Approved (CM) | Parking under Cover (1 point) | Estacionamento coberto (1 ponto) |
| 27483f5b85b-3209-4f0e-b487-6d0d4b04c45b | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 2754e6fd69f-4e48-4d57-a966-45a874274f91 | Translation Approved (76%) | Place at least 50% of <342>parking spaces under cover</342>. | Implemente pelo menos 50% das <342>vagas de estacionamento coberto</342>. |
| 2764e6fd69f-4e48-4d57-a966-45a874274f91 | Translation Approved (100%) | Any roof used to shade or cover parking must (1) have a three-year aged SRI of at least 32 (if three-year aged value information is not available, use materials with an initial SRI of at least 39 at installation), (2) be a vegetated roof, or (3) be covered by energy generation systems, such as solar thermal collectors, photovoltaics, and wind turbines. | Qualquer telhado usado para fazer sombra ou cobrir estacionamentos deve (1) ter um SRI após três anos de pelo menos 32 (se as informações de valor após três anos não estiverem disponíveis, use materiais com SRI inicial de pelo menos 39 na instalação), (2) ser um telhado com cobertura verde ou (3) ser coberto por sistemas de geração de energia, como coletores térmicos solares, fotovoltaicos e turbinas eólicas. |
| 277e339636d-f958-4e03-8967-bf710bc27768 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 278c899b811-073e-4b9d-be2e-fa2dbe8bc219 | Translation Approved (96%) | Implement a maintenance program that ensures all SRI surfaces are cleaned at least every three years to maintain good reflectance, and all vegetated roofs are maintained for plant health and good structural condition. | Implemente um programa de manutenção que garanta que todas as superfícies SRI sejam limpas a cada três anos, no mínimo, para manter uma boa refletância e que todos os telhados com vegetação recebam manutenção para manter a saúde das plantas e uma condição estrutural boa. |
| 27988f7f002-5773-4f83-baa5-4a603e840478 | Translation Approved (100%) | SS Credit: Light Pollution Reduction | Crédito SS: Redução da Poluição Luminosa (SS Credit: Light Pollution Reduction) |
| 2809908717f-595f-47b8-8f2d-a88906b9384c | Translation Approved (100%) | EB:O&M | EB:O&M |
| 2817ff2f18c-9a32-4fac-b3d1-750743627ad6 | Translation Approved (100%) | 1 point | 1 ponto |
| 2821027cae8-232b-4395-b8ee-220ce368294b | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 283ef6680cd-11f6-403c-ad18-18073aa8789b | Translation Approved (99%) | Existing Buildings: Operations & Maintenance (1 point) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 ponto) |
| 28488ca365b-50e5-489a-b986-185856ed6c33 | Translation Approved (92%) | EB:O&M Schools (1 point) | EB:O&M Escolas (Schools) (1 ponto) |
| 285cac29524-8fee-4282-9c73-871bfc71163b | Translation Approved (99%) | EB:O&M Retail (1 point) | EB:O&M Varejo (Retail) (1 ponto) |
| 28667a9323b-6d4a-4f18-b890-2e92f1559361 | Translation Approved (99%) | EB:O&M Data Centers (1 point) | EB:O&M Centros de Dados (Data Centers) (1 ponto) |
| 287e16abfcd-0eb5-404d-bb38-3d3967393f0d | Translation Approved (99%) | EB:O&M Hospitality (1 point) | EB:O&M Hoteleiro (Hospitality) (1 ponto) |
| 2884a42e9ba-47a4-462b-9e1f-2df19f6d5747 | Translation Approved (99%) | EB:O&M Warehouses & Distribution Centers (1 point) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 ponto) |
| 289c0e4c851-b4c1-40e5-be89-688f3715b76a | Translation Approved (100%) | Intent | Objetivo |
| 2909af6b99c-5cb6-4aa9-a817-f15b7e044de9 | Translation Approved (CM) | To increase night sky access, improve nighttime visibility, and reduce the consequences of development for wildlife and people. | Aumentar o acesso ao céu noturno, melhorar a visibilidade noturna e reduzir as consequências do empreendimento para a vida animal e as pessoas. |
| 2917aeed3a8-7e14-4504-b77f-1705c235a5b7 | Translation Approved (100%) | Requirements | Requisitos |
| 2929cb43381-8381-413b-a31b-e0896373822a | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 293e9295e5d-1f5a-4b7d-8443-0fec384e5975 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 294777c61ca-dc1c-49e3-927e-c900fc4ae66b | Translation Approved (0%) | Meet the requirements of one of the options below: | Atenda aos requisitos de uma das opções abaixo: |
| 295abddfcee-47d1-4cea-9c89-63b12bd40843 | Translation Approved (100%) | Option 1. | Opção 1. |
| 296abddfcee-47d1-4cea-9c89-63b12bd40843 | Translation Approved (0%) | Fixture Shielding | Proteção de dispositivos |
| 297dc05acb5-ab1c-40de-b680-ea7f8e0d0c5d | Translation Approved (0%) | Shield all exterior fixtures (where the sum of the mean lamp lumens for that fixture exceeds 2,500) such that the installed fixtures do not directly emit any light at a vertical angle more than 90 degrees from straight down. | Proteja todos os dispositivos externos (em que a soma dos lúmens médios da lâmpada do dispositivo exceder 2.500) de forma que os dispositivos instalados não emitam diretamente luz em um ângulo vertical maior que 90 graus direcionado para baixo. |
| 298e9067ba2-cb67-4d34-9e3d-649c9593d5a3 | Translation Approved (100%) | OR | OU |
| 2995ba4d7f3-7166-40c9-8845-7e27ae503e6c | Translation Approved (100%) | Option 2. | Opção 2. |
| 3005ba4d7f3-7166-40c9-8845-7e27ae503e6c | Translation Approved (0%) | Perimeter Measurements | Medições de perímetro |
| 3017bdc11cc-cb7e-44c1-920a-b8fb9e9833a7 | Translation Approved (0%) | Measure the night illumination levels at regularly spaced points on the project boundary, taking the measurements with the building’s exterior and site lights both on and off. | Meça os níveis de iluminação noturna em pontos regularmente espaçados no limite do projeto, fazendo as medições com as luzes do terreno e do exterior do edifício ligadas e desligadas. |
| 3027bdc11cc-cb7e-44c1-920a-b8fb9e9833a7 | Translation Approved (0%) | At least eight measurements are required, at a maximum spacing of 100 feet (30 meters) apart. | São necessárias pelo menos oito medições, com um espaçamento máximo de 100 pés (30 metros) de distância. |
| 3037bdc11cc-cb7e-44c1-920a-b8fb9e9833a7 | Translation Approved (100%) | The illumination level measured with the lights on must not be more than 20% above the level measured with the lights off. | As medições do nível de iluminação com as luzes ligadas não devem estar mais de 20% acima do nível medido com as luzes desligadas. |
| 304efcbb4ef-8de1-44a2-90db-05397cb9687c | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 3052e8b17a5-21ec-49a7-bf3f-b454b7cdc45a | Translation Approved (100%) | None. | Nenhum. |
| 306faff26ab-39e9-4b4f-9124-4043ab618327 | Translation Approved (0%) | SS Credit: Site Management | Crédito SS: Gerenciamento do Terreno (SS Credit: Site Management) |
| 30702d582b4-a7d6-4ae4-81f5-34acae3dd527 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 3082e4fddd3-2e0e-4de5-baf7-3d8fea6c8803 | Translation Approved (100%) | 1 point | 1 ponto |
| 30936fa0bd2-41fd-46cc-bad8-9c31085e45d1 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 310a7961f58-0aa3-4311-9d7e-b29c9aed5954 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1 point) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 ponto) |
| 311796252da-c013-4ff2-b906-f04b598e5bb8 | Translation Approved (100%) | EB:O&M Schools (1 point) | EB:O&M Escolas (Schools) (1 ponto) |
| 3120f5c3bf1-cdf8-4f51-97c1-d60c3fb10998 | Translation Approved (100%) | EB:O&M Retail (1 point) | EB:O&M Varejo (Retail) (1 ponto) |
| 3131140e409-5ce2-400a-8b5f-bda121174278 | Translation Approved (100%) | EB:O&M Data Centers (1 point) | EB:O&M Centros de Dados (Data Centers) (1 ponto) |
| 314534638c0-8242-495a-be56-5beb4fff0521 | Translation Approved (100%) | EB:O&M Hospitality (1 point) | EB:O&M Hoteleiro (Hospitality) (1 ponto) |
| 3150f5ce50c-3ea0-44ce-9cc7-a5d4ba30a5e0 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1 point) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 ponto) |
| 316094a2ffb-8b31-4bd4-a4e2-11e751d07b96 | Translation Approved (100%) | Intent | Objetivo |
| 31736499119-b0e8-4718-b4e9-fe10c51039df | Translation Approved (100%) | To preserve ecological integrity and encourage environmentally sensitive site management practices that provide a clean, well-maintained, and safe building exterior while supporting high-performance building operations and integration into the surrounding landscape. | Preservar a integridade ecológica e incentivar práticas de gestão do terreno ambientalmente sensíveis que propiciem um exterior de edifício limpo, bem mantido e seguro, mas favorecendo também operações de edifícios de alto desempenho e sua integração à paisagem vizinha. |
| 318bb9dd9c8-8620-474e-af57-9c55fe571be8 | Translation Approved (100%) | Requirements | Requisitos |
| 31979df82c2-ab00-4850-bfae-71194a897090 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 3205d9378a9-bc17-4bb2-b2e2-e01e4dc2534c | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 321f71b32a9-73db-4ceb-b6f5-223881b341ea | Translation Approved (100%) | None. | Nenhum. |
| 322773d15fd-9823-4b38-b813-0b72db4e1b62 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 32328932925-4468-4efa-a444-027ee8223548 | Translation Approved (0%) | Demonstrate that the following performance criteria were met: | Demonstre que os seguintes critérios de desempenho foram atendidos: |
| 3244a8729f5-b19d-49ec-a32c-8d54acd3ca40 | Translation Approved (0%) | Use no calcium chloride or sodium chloride deicers, and/or establish reduced treatment areas equal to 50% of applicable paving area. | Não use descongeladores de cloreto de cálcio ou cloreto de sódio e/ou estabeleça áreas de tratamento reduzido igual a 50% da área de pavimentação aplicável. |
| 325651795e7-e735-4dfd-8ce2-f366f15b4bb8 | Translation Approved (0%) | Prevent erosion and sedimentation, and restore any eroded soils. | Evite a erosão e sedimentação e restaure solos erodidos. |
| 326f3651060-47ff-4f56-96a9-5ad3473f0ab1 | Translation Approved (0%) | Prevent air pollution from construction materials and activities. | Evite a poluição atmosférica proveniente de materiais e atividades de construção. |
| 327a66e764c-70e7-4f16-88e0-6e34170849fd | Translation Approved (0%) | Divert from landfills 100% of plant material waste via low-impact means. | Desvie de aterros sanitários 100% dos resíduos de materiais de plantas via meios de baixo impacto. |
| 328b97b4c4c-045d-4139-872d-bf74d019824c | Translation Approved (0%) | Prevent the overapplication of nutrients. | Evite a aplicação excessiva de nutrientes. |
| 329b97b4c4c-045d-4139-872d-bf74d019824c | Translation Approved (0%) | Use no ammonia-based fertilizers, biosolid-based fertilizers (for continuous application), synthetic quick-release fertilizers, or “weed and feed” formulations. | Não use fertilizantes à base de amônia, fertilizantes à base de biossólidos (para aplicação contínua), fertilizantes sintéticos de liberação rápida ou formulações do tipo "weed and feed". |
| 330b97b4c4c-045d-4139-872d-bf74d019824c | Translation Approved (0%) | Blanket applications of herbicides are prohibited; turf weeds may be controlled by spot spraying only. | Aplicações generalizadas de herbicidas são proibidas, ervas daninhas podem ser controladas somente com pulverização direta no local. |
| 331ad5ef59a-dc23-486f-97e3-f3e51c4b7772 | Translation Approved (0%) | Monitor irrigation systems manually or with automated systems at least every two weeks during the operating season and correct any leaks, breaks, inappropriate water usage, or incorrect timing. | Monitore sistemas de irrigação manualmente ou utilizando sistemas automáticos pelo menos quinzenalmente durante a época de operação e corrija vazamentos, rupturas, uso de água inadequado ou momento incorreto. |
| 3327e35e8a0-e0e0-4417-93f4-14932b0c58f6 | Translation Approved (0%) | Store materials and equipment to prevent air and site contamination. | Armazene os materiais e equipamentos para evitar contaminação atmosférica e do terreno. |
| 333be763691-3853-46fd-8d5a-832f72831b9b | Translation Approved (100%) | AND | E |
| 33483cd7564-0fcd-4790-8819-a35ca092652e | Translation Approved (88%) | Meet one of the following options: | Execute uma das opções a seguir: |
| 335586c069f-dd75-4f77-b63a-7115b0a6b4a6 | Translation Approved (100%) | Option 1. | Opção 1. |
| 336586c069f-dd75-4f77-b63a-7115b0a6b4a6 | Translation Approved (0%) | Limited Turf Area | Área limitada de gramado |
| 3378a8c6028-b18b-44b4-89a4-b6a66f12d413 | Translation Approved (0%) | Limit turf to 25% or less of the vegetated area. | Limite o gramado a 25% ou menos da área plantada. |
| 3380d5e711a-9c6a-4d97-ac47-f1b2b14e0201 | Translation Approved (0%) | Playgrounds and athletic fields in schools or parks are excluded from this option. | Parques infantis e quadras esportivas em escolas ou parques são excluídos desta opção. |
| 339f103900c-7265-45f7-a5f3-5eeda1d2a14f | Translation Approved (100%) | OR | OU |
| 34070f883f3-0a18-4731-b098-46aa6bbab652 | Translation Approved (100%) | Option 2. | Opção 2. |
| 34170f883f3-0a18-4731-b098-46aa6bbab652 | Translation Approved (0%) | All Manual or Electric-Powered Equipment | Equipamentos manuais ou elétricos |
| 342b1621a58-a4a5-490e-bb80-7cb26658d07c | Translation Approved (0%) | Use all manual or electric-powered equipment in all site management operations. | Use equipamentos manuais ou elétricos em todas as operações de gestão do terreno. |
| 343ef559055-e6af-438e-932d-dc53cebe027b | Translation Approved (100%) | OR | OU |
| 3445eb1250a-c255-47c3-a7ed-a0f91a043421 | Translation Approved (100%) | Option 3. | Opção 3. |
| 3455eb1250a-c255-47c3-a7ed-a0f91a043421 | Translation Approved (0%) | Reduction in Emissions from Site Management Equipment | Redução nas emissões de equipamentos de gestão do terreno |
| 34616540ced-fbc3-4dff-ac3e-e197470f91b1 | Translation Approved (0%) | Show and maintain a 50% reduction in hydrocarbon (HC) and nitrogen oxide (NOx) emissions, and a 75% reduction in carbon monoxide (CO) emissions from <380>baseline conditions</380>. | Mostre e mantenha uma redução de 50% nas emissões de hidrocarbono (HC) e óxido de nitrogênio (NOx) e uma redução de 75% nas emissões de monóxido de carbono (CO) das <380>condições de baseline</380>. |
| 3477a0e65df-1558-4c9c-b0a2-df2885bbf526 | Translation Approved (72%) | SS Credit: Site Improvement Plan | Crédito SS: Plano de Melhoria do Terreno (SS Credit: Site Improvement Plan) |
| 348645a6cbb-f016-4805-9969-2286cb616a16 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 349e7e58cf9-3252-401a-b0ee-6ddbb1b30d80 | Translation Approved (100%) | 1 point | 1 ponto |
| 35054ab1cb6-1cae-4e1a-925e-d8e0bb3f4ffc | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 35162cc9df7-d55d-464f-8031-aa8bcf55a717 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1 point) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 ponto) |
| 35260c25df9-c412-419f-972a-f420762cce6e | Translation Approved (100%) | EB:O&M Schools (1 point) | EB:O&M Escolas (Schools) (1 ponto) |
| 35344a4e7a6-3cb1-4792-bdb3-52edd23d8468 | Translation Approved (100%) | EB:O&M Retail (1 point) | EB:O&M Varejo (Retail) (1 ponto) |
| 3543fdf2ee0-3563-45c3-8df6-ad290c47db98 | Translation Approved (100%) | EB:O&M Data Centers (1 point) | EB:O&M Centros de Dados (Data Centers) (1 ponto) |
| 355fc72a234-d78f-4f8d-abdc-3ed37af0f9b5 | Translation Approved (100%) | EB:O&M Hospitality (1 point) | EB:O&M Hoteleiro (Hospitality) (1 ponto) |
| 35616796343-0747-4b00-aa04-0bb0f675dfd2 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1 point) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 ponto) |
| 3572e363210-2494-4c1f-892e-c27ee02a4f20 | Translation Approved (100%) | Intent | Objetivo |
| 358c725a71b-5889-4d48-8582-ac3029696385 | Translation Approved (0%) | To preserve and improve ecological integrity while supporting high-performance building operations. | Preservar e melhorar a integridade ecológica, mas favorecendo também operações de edifícios de alto desempenho. |
| 3597e018b7f-d967-4302-b4e6-f31ee66fac3a | Translation Approved (100%) | Requirements | Requisitos |
| 36097561772-f199-4607-8527-24aff3334c6a | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 361cb21f102-45e8-4560-aef7-b65ef1c68f36 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 362bce09fe5-8cc5-47e5-9d98-688ffecdb70f | Translation Approved (0%) | Develop a five-year site improvement plan that includes the following: | Desenvolva um plano de melhoria do terreno de cinco anos que inclua o seguinte: |
| 363a7ba4272-296d-4506-b4a6-ea5392def45e | Translation Approved (0%) | documentation of existing site conditions; | documentação das condições do terreno existente; |
| 364fc3a4894-f320-42d7-9251-5629a731a521 | Translation Approved (70%) | site improvement objectives; | objetivos de melhoria do terreno; |
| 365822c73f2-f367-4576-9b5b-6aa2a9e3e338 | Translation Approved (0%) | performance standards to evaluate ongoing progress; and | normas de desempenho para avaliar o processo contínuo; e |
| 3666492bd56-3967-43c1-b758-bdef6b60ab20 | Translation Approved (0%) | monitoring protocols. | protocolos de monitoramento. |
| 36765bd1a39-3911-4c98-a954-fff4c7e8a6c8 | Translation Approved (0%) | The improvement plan must address the following topics. | O plano de melhoria deve abordar os seguintes tópicos. |
| 3689ab6f6db-f171-485e-8295-2144d05fb751 | Translation Approved (96%) | Hydrology. | Hidrologia. |
| 3699ab6f6db-f171-485e-8295-2144d05fb751 | Translation Approved (0%) | Protection and improvement of water bodies on-site, rainwater management and reuse opportunities, potable water-use reduction. | Proteção e melhoria dos cursos de água no local, oportunidades de reutilização e gestão de águas pluviais, redução do uso de água potável. |
| 370905b0910-5333-4867-9346-8f2cdadccfe0 | Translation Approved (96%) | Vegetation. | Vegetação. |
| 371905b0910-5333-4867-9346-8f2cdadccfe0 | Translation Approved (0%) | Documentation of existing vegetation on-site, turf area reduction, management of native and invasive plants, protection of threatened, endangered or unique species. | Documentação de vegetação existente no local, redução da área de gramado, gestão de plantas nativas e invasoras, proteção de espécies ameaçadas de extinção ou únicas. |
| 372f68ce005-bc39-4bc5-a14a-afd4a0f4eda8 | Translation Approved (0%) | Soils. | Solos. |
| 373f68ce005-bc39-4bc5-a14a-afd4a0f4eda8 | Translation Approved (0%) | Documentation of general soil structure, preservation of healthy soils, remediation of compacted soils, identification of previously disturbed area. | Documentação da estrutura geral do solo, preservação de solos saudáveis, remediação de solos compactados, identificação de áreas previamente impactadas. |
| 37447e04135-165f-4cd7-b0d8-804e5bdcd653 | Translation Approved (0%) | The plan must be developed with professionals trained and experienced in the above disciplines. | O plano deve ser desenvolvido com profissionais treinados e experientes nas disciplinas acima. |
| 375c1172c4d-4490-47b6-85e0-cc01bc2c59f1 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 3764bda5e25-7594-4461-8745-a87a1a5f228a | Translation Approved (0%) | Show that at least 5% of the site is vegetated. | Mostre que pelo menos 5% do terreno possui vegetação. |
| 3774bda5e25-7594-4461-8745-a87a1a5f228a | Translation Approved (0%) | Implement all no-cost and low-cost measures. | Implemente todas as medidas sem custo e de baixo custo. |
| 3784bda5e25-7594-4461-8745-a87a1a5f228a | Translation Approved (0%) | Develop a new improvement plan and implement all new no-cost and low-cost measures every five years. | Desenvolva um novo plano de melhoria e implemente todas as novas medidas sem custo e de baixo custo a cada cinco anos. |
| 37942e7b13a-56a7-461c-96cb-041b3f60366e | Translation Approved (100%) | SS Credit: Joint Use of Facilities | Crédito SS: Uso Conjunto de Instalações (SS Credit: Joint Use of Facilities) |
| 380e9499c23-a52c-4dae-af14-88c299cd23a8 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 381aae40dbb-3006-4736-964d-d3690b057931 | Translation Approved (100%) | 1 point | 1 ponto |
| 38279a4dd3c-6fd8-4598-9012-4b0583fc3097 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 383a91bae91-6a2f-48eb-8e24-875a4e91bce8 | Translation Approved (100%) | EB:O&M Schools | EB:O&M Escolas (Schools) |
| 38429b26cc5-c86b-4a6d-96c7-377d11fd6ec9 | Translation Approved (100%) | Intent | Objetivo |
| 385c3c1384d-daea-4ae7-bf46-c1c26e787c52 | Translation Approved (CM) | To integrate the school with the community by sharing the building and its playing fields for nonschool events and functions. | Integrar a escola à comunidade compartilhando o edifício e seus parques recreativos para eventos e cerimônias não escolares. |
| 3869e7807b0-4053-4792-9773-b8fbf7b7d188 | Translation Approved (100%) | Requirements | Requisitos |
| 387cf2830cd-c2e4-4237-8283-70af3abd6b1d | Translation Approved (0%) | Schools EBOM | EBOM Escolas (Schools) |
| 3889bbd3734-babe-4865-b96e-9bbb5635d6ef | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 38927d78c78-6a15-4063-9195-0c4699b910fb | Translation Approved (100%) | Option 1. | Opção 1. |
| 39027d78c78-6a15-4063-9195-0c4699b910fb | Translation Approved (93%) | Make Building Space Open to General Public (1 point) | Tornar espaço do edifício aberto ao público geral (1 ponto) |
| 3914de83932-4635-4161-9b46-9d076a3d2f06 | Translation Approved (88%) | In collaboration with the school authorities, establish at least three of the following types of spaces as accessible to and available for shared use by the general public: | Em colaboração com as autoridades escolares, estabeleça que pelo menos três dos seguintes tipos de espaços sejam acessíveis e disponibilizados para uso compartilhado pelo público geral: |
| 392f157b41d-af7b-441b-838b-fc5a6dd10c3b | Translation Approved (91%) | auditorium; | auditório; |
| 3931f2a4e86-fc65-48ce-b588-29ccf651f2a5 | Translation Approved (91%) | gymnasium; | ginásio; |
| 39417ba8f5b-7ffd-4ad6-8023-78fc5b7f95ad | Translation Approved (91%) | cafeteria; | lanchonete; |
| 3950698f6f4-b2f3-4776-b72c-6cc09293386c | Translation Approved (87%) | one or more classrooms; | uma ou mais salas de aula; |
| 39671dddfaa-5570-45cf-a338-a633aa661d30 | Translation Approved (0%) | playing fields and stadiums; and | parques recreativos e estádios; e |
| 397450825d3-bdcc-468b-97c3-5eb9ee0f0a52 | Translation Approved (CM) | joint parking. | estacionamento conjunto. |
| 398419a5add-8b9f-4111-8a0a-b806d2826b2f | Translation Approved (CM) | Provide access to toilets in joint-use areas after normal school hours. | Forneça acesso a sanitários em áreas de uso conjunto após o horário escolar normal. |
| 39970f55ead-75b2-49a4-9cec-6e54698f24fb | Translation Approved (100%) | OR | OU |
| 4002d4f7c35-4c44-43c5-956d-0427eb111694 | Translation Approved (100%) | Option 2. | Opção 2. |
| 4012d4f7c35-4c44-43c5-956d-0427eb111694 | Translation Approved (CM) | Contract with Specific Organizations to Share Building Space (1 point) | Contrato com organizações específicas para compartilhar espaços do edifício (1 ponto) |
| 4024aa50ef9-9951-42e3-81d3-62da1edfd47c | Translation Approved (CM) | In collaboration with the school authorities, contract with community or other organizations to provide at least two types of dedicated-use spaces in the building, such as the following: | Em colaboração com as autoridades escolares, firme contratos com a comunidade ou outras organizações para fornecer pelo menos dois tipos de espaços de uso dedicado no edifício, como: |
| 4038855f38a-c2a6-433d-afc3-9d5328685e83 | Translation Approved (CM) | commercial office; | escritório comercial; |
| 40459a20027-379c-4b42-8851-1538ba344f9e | Translation Approved (CM) | health clinic; | clínica de saúde; |
| 40507339109-dbe1-471d-aeb8-9138eb8b3896 | Translation Approved (CM) | community service centers (provided by state or local offices); | centros de serviços comunitários (oferecidos por órgãos estaduais ou municipais); |
| 4065bc85af9-724a-41e8-b782-febda38d5d47 | Translation Approved (96%) | police offices; | postos policiais; |
| 407ebb91a51-9c74-47b5-90cc-8d728c8f31b5 | Translation Approved (CM) | library or media center; | biblioteca ou centro de mídia; |
| 408ae20c0c8-811b-425f-94c2-b656711dc407 | Translation Approved (CM) | parking lot; and | estacionamento; e |
| 40938bb5064-3865-4001-81bc-01a357ee2939 | Translation Approved (CM) | one or more commercial businesses. | uma ou mais empresas comerciais. |
| 4102b3b21c5-6e57-47b7-919f-64c9fb6f1a8b | Translation Approved (100%) | Provide access to toilets in joint-use areas after normal school hours. | Forneça acesso a sanitários em áreas de uso conjunto após o horário escolar normal. |
| 41166b41238-da8c-4822-8494-01625d74c35f | Translation Approved (100%) | OR | OU |
| 41281a7aff3-0260-4abb-ad02-83311aa9acef | Translation Approved (100%) | Option 3. | Opção 3. |
| 41381a7aff3-0260-4abb-ad02-83311aa9acef | Translation Approved (CM) | Use Shared Space Owned by Other Organizations (1 point) | Usar o espaço compartilhado pertencente a outras organizações (1 ponto) |
| 414db6bec33-e82b-413d-bdcc-33ea5543118e | Translation Approved (88%) | In collaboration with the school authorities, establish at least two of the following six types of spaces (owned by other organizations/agencies) are accessible to students: | Em colaboração com as autoridades escolares, estabeleça que pelo menos dois dos seis tipos de espaços a seguir, pertencentes a outras organizações ou agências, sejam acessíveis aos alunos: |
| 415c22aa049-b114-46b7-a7b4-d07ca3aa2a9e | Translation Approved (100%) | auditorium; | auditório; |
| 41694c487c4-26d7-40ce-8040-fc7754959f8e | Translation Approved (100%) | gymnasium; | ginásio; |
| 4173e343aff-2f68-4c2a-8b55-b70efd312d7f | Translation Approved (100%) | cafeteria; | lanchonete; |
| 418b150b555-7982-40e9-89f1-b66f828e6f71 | Translation Approved (100%) | one or more classrooms; | uma ou mais salas de aula; |
| 419b3d26829-221e-4ac1-8dba-9d7725c21362 | Translation Approved (86%) | swimming pool; and | piscina; e |
| 420c40448fe-93fe-4411-b7ea-5dec9af8450e | Translation Approved (CM) | playing fields and stadiums. | parques recreativos e estádios. |
| 421ccac6bfa-a58d-41eb-b93e-40fbbbc698a7 | Translation Approved (CM) | Provide direct pedestrian access to these spaces from the school. | Forneça acesso de pedestre direto da escola a esses espaços. |
| 422ccac6bfa-a58d-41eb-b93e-40fbbbc698a7 | Translation Approved (CM) | In addition, provide signed joint-use agreements with the other organizations or agencies that stipulate how these spaces will be shared. | Além disso, forneça acordos assinados de uso conjunto com as outras organizações ou agências, determinando como os espaços serão compartilhados. |
| 4234639ede4-3714-4d92-865f-55d9a43f0d6d | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 4243b331a79-1de4-4a3f-b39a-01e794728a96 | Translation Approved (100%) | None. | Nenhum. |
| 4251284de1e-58ab-48a8-af9e-4394ec869e07 | Translation Approved (100%) | Water Efficiency (WE) | Eficiência Hídrica (Water Efficiency - WE) |
| 4264582ccd0-0cde-45c5-9679-6ecfa611d035 | Translation Approved (CM) | WE Prerequisite: Indoor Water Use Reduction | Pré-requisito WE: Redução do Uso de Água Interno (WE Prerequisite: Indoor Water Use Reduction) |
| 4271a3970a5-6b90-4bbe-b67f-a531271fd9b5 | Translation Approved (100%) | Required | Obrigatório |
| 428a8ec3ffd-a755-4b8e-8f39-651f2fdb5611 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 4292ea1d171-226a-4d19-999a-ea9c4ef68871 | Translation Approved (100%) | This prerequisite applies to | Este pré-requisito se aplica a: |
| 430a022675e-e129-4765-bddc-fdafbed72382 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) |
| 431cf042ba5-8289-41e4-a7ab-166b9c9ce016 | Translation Approved (100%) | EB:O&M Schools | EB:O&M Escolas (Schools) |
| 4322c172462-8937-4c77-ba8c-26ce142e855d | Translation Approved (100%) | EB:O&M Retail | EB:O&M Varejo (Retail) |
| 4330a47b41f-6dd9-49a3-8c06-9d7c7a600f14 | Translation Approved (100%) | EB:O&M Data Centers | EB:O&M Centros de Dados (Data Centers) |
| 434dfe20b32-d3ef-4c7d-aac4-11a658d68d54 | Translation Approved (100%) | EB:O&M Hospitality | EB:O&M Hoteleiro (Hospitality) |
| 4358bda4069-60fb-408b-91fb-dc5490fade38 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) |
| 4367f45cb4f-21a9-4f9d-b09f-32a05dcde749 | Translation Approved (100%) | Intent | Objetivo |
| 4370868e4c6-909e-47df-b547-01c34fb5c657 | Translation Approved (CM) | To reduce indoor water consumption. | Reduzir o consumo de água interno. |
| 438ee4cbe1b-73b0-4e7f-83f4-5fc951fb8f8b | Translation Approved (100%) | Requirements | Requisitos |
| 439ec3610ac-cf68-4e5b-975f-ab7c157bff2a | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 440c0a98e7e-0815-439f-bef5-e21390e96bb1 | Translation Approved (100%) | Option 1. | Opção 1. |
| 441c0a98e7e-0815-439f-bef5-e21390e96bb1 | Translation Approved (77%) | Calculated Water Use | Uso de água calculado |
| 4423559a51e-6bb6-4be4-b0b3-1236db2d20bd | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 4435b169502-bb46-4c51-b84c-dc205207599b | Translation Approved (0%) | For the indoor plumbing fixtures and fittings listed in Table 1, reduce water consumption to or below the LEED v4 for Existing Buildings: Operations & Maintenance baseline, calculated assuming 100% of the building’s indoor plumbing fixtures and fittings meet the flush and flow rates listed in Table 1. | Para dispositivos e conexões internos de encanamento listados na Tabela 1, reduza o consumo de água para ou abaixo da baseline de LEED v4 para Edifícios Existentes: Operações e Manutenção (LEED v4 for Existing Buildings: Operations & Maintenance), calculado supondo de 100% dos dispositivos e conexões hidráulicos internos do edifício atendem às taxas de fluxo e de descarga listadas na Tabela 1. |
| 444b40e391c-328a-4f80-bbdc-bed41a7a3d7c | Translation Approved (0%) | The LEED v4 for Existing Buildings: Operations & Maintenance water use baseline is set depending on the year of building’s occupancy, as follows: | A baseline de uso de água do LEED v4 para Edifícios Existentes: Operações e Manutenção (LEED v4 for Existing Buildings: Operations & Maintenance) é definida dependendo no ano da ocupação do edifício, da seguinte forma: |
| 445500f511c-e647-4248-85d4-0af46a3167c8 | Translation Approved (0%) | For a building with a certificate of occupancy dated 1995 or later, the baseline is 120% of the water use that would result if all fixtures met the code requirements in Table 1. | Para um edifício com um certificado de ocupação de 1995 ou posterior, a baseline é 120% do uso de água que resultaria se todos os dispositivos atendessem aos requisitos do código na Tabela 1. |
| 446093a5844-6d49-420b-8f4e-8b5b78811dc3 | Translation Approved (94%) | For a building with a certificate of occupancy dated before 1995, the baseline is 150% of the water use that would result if all fixtures met the code requirements in Table 1. | Para um edifício com um certificado de ocupação anterior a 1995, a baseline é 150% do uso de água que resultaria se todos os dispositivos atendessem aos requisitos do código na Tabela 1. |
| 4477598ff39-9c0b-41dd-af3d-1b493eaec80b | Translation Approved (100%) | Table 1. | Tabela 1. |
| 4487598ff39-9c0b-41dd-af3d-1b493eaec80b | Translation Approved (0%) | Fixture and fitting code requirements | Requisitos do código de dispositivos e conexões |
| 44955346d8a-b165-460f-a0f6-a1132149ba59 | Translation Approved (0%) | Fixture or fitting | Dispositivo ou conexão |
| 45020795608-f00c-49ab-a442-bf150fe9b8ec | Translation Approved (CM) | Baseline (IP units) | Baseline (sistema imperial) |
| 451e95d11f6-3954-4649-a8b5-816782f35539 | Translation Approved (CM) | Baseline (SI units) | Baseline (sistema métrico) |
| 452bb22d725-2fa2-4d60-a1fe-d5ab8414018b | Translation Approved (75%) | Toilet (water closet) | Vaso sanitário |
| 4532d2b89bf-edc1-40a5-b6c1-48d6b5896da8 | Translation Approved (0%) | 1.6 gpf | 1,6 gpf |
| 4546228de01-7d37-429a-bbbd-0def3d393539 | Translation Approved (0%) | 6 lpf | 6 lpf |
| 455e26e627f-0991-410e-95ae-796e45941f11 | Translation Approved (100%) | Urinal | Mictório |
| 4569118ea8c-829f-4017-8632-e033f4d4b045 | Translation Approved (100%) | 1.0 gpf | 1,0 gpf |
| 457a69e2462-2707-4dab-9552-c93be25d744b | Translation Approved (100%) | 3.8 lpf | 3,8 lpf |
| 458f10e01c8-594b-493e-bfe2-cc01d1016696 | Translation Approved (CM) | Public lavatory (restroom) faucet | Torneira de pia pública (banheiro) |
| 4593d02daa9-0db2-40b2-aeab-3f4768a8586d | Translation Approved (CM) | 0.5 gpm at 60 psi** all others except private applications | 0,5 gpm a 60 psi**, todas as outras, exceto aplicações privativas |
| 460e06b3f38-cd06-4dc8-86c3-9cb5745a5433 | Translation Approved (CM) | 1.9 lpm at 415 kPa, all others except private applications | 1,9 lpm a 415 kPa, todas as outras, exceto aplicações privativas |
| 461d83ad4f9-1bbb-43be-a386-e08184d1f032 | Translation Approved (97%) | Private lavatory faucet | Torneira de pias privativas |
| 46249ab2ce8-e864-4d1d-b42c-f950dfeb95db | Translation Approved (CM) | 2.2 gpm at 60 psi | 2,2 gpm a 60 psi |
| 463955a79a2-6744-4041-9a27-a94932369532 | Translation Approved (CM) | 8.3 lpm at 415 kPa | 8,3 lpm a 415 kPa |
| 464723d71d9-0cc9-4c25-b23c-05af088d4c9d | Translation Approved (CM) | Kitchen faucet (excluding faucets used exclusively for filling operations) | Torneira de cozinha (exceto torneiras usadas exclusivamente para operações de enchimento) |
| 4656d67dedd-7afb-4015-9fc1-d145e27b7e88 | Translation Approved (100%) | 2.2 gpm at 60 psi | 2,2 gpm a 60 psi |
| 466bf3788e9-c12c-47e3-b4b9-120d72121e38 | Translation Approved (100%) | 8.3 lpm at 415 kPa | 8,3 lpm a 415 kPa |
| 4678764756b-b175-4a2b-93c4-b9daa3e15542 | Translation Approved (96%) | Showerhead | Chuveiro |
| 468086064b8-4e29-4f72-b6a1-f42a3bc06843 | Translation Approved (CM) | 2.5 gpm at 80 psi per shower stall | 2,5 gpm a 80 psi por box de chuveiro |
| 4691dac9159-6116-4094-9155-46b03c9afc09 | Translation Approved (CM) | 9.5 lpm at 550 kPa per shower stall | 9,5 lpm a 550 kPa por box de chuveiro |
| 4706a0c38e0-9f01-4b08-9644-b0aaee37ca9e | Translation Approved (100%) | gpf = gallons per flush | gpf = galões por descarga |
| 47105f4871e-556c-4123-854b-77909464900b | Translation Approved (CM) | gpm = gallons per minute | gpm = galões por minuto |
| 472d1ece158-d9cc-4dec-87dc-c0c5cb676749 | Translation Approved (CM) | psi = pounds per square inch | psi = libras por polegada quadrada |
| 473c96fa8ce-ff03-47ef-b4cc-e9fc2b2249c4 | Translation Approved (CM) | lpf = liters per flush | lpf = litros por descarga |
| 474b514530d-55ee-465f-8752-72602cf3aa63 | Translation Approved (CM) | lpm = liters per minute | lpm = litros por minuto |
| 475893ef151-f4ec-44b1-8cc1-70ec0ed77638 | Translation Approved (CM) | kPa = kilopascals | kPa = quilopascals |
| 476cee207ff-2cdb-451e-9f4a-520438caa7e8 | Translation Approved (0%) | If indoor plumbing systems were renovated after initial occupancy of the building, set a whole-building average baseline by prorating the above limits, based on the proportion of plumbing fixtures installed during the plumbing renovations in each period. | Se os sistemas hidráulicos internos foram reformados após a ocupação inicial do edifício, defina uma baseline média para todo o edifício rateando os limites acima, com base na proporção dos dispositivos hidráulicos instalados durante as reformas hidráulicas em cada período. |
| 477cee207ff-2cdb-451e-9f4a-520438caa7e8 | Translation Approved (0%) | Pre-1995 buildings that have had only minor fixture retrofits (e.g., aerators, showerheads, flushing valves) but no plumbing renovations in or after 1995 may use the 150% baseline for the whole building. | Os edifícios anteriores a 1995 que receberam pequenas modernizações nos dispositivos (por exemplo, arejadores, chuveiros, válvulas de descarga), mas não reformas hidráulicas em ou após 1995 podem usar a baseline de 150% para todo o edifício. |
| 478d1f93a78-ad09-491b-bdce-edd8942f93e9 | Translation Approved (0%) | Calculate fixture and fitting performance to compare the water use of the as-installed fixtures and fittings with the use of Uniform Plumbing Code or International Plumbing Code-compliant (baseline) fixtures and fittings. | Calcule o desempenho dos dispositivos de conexões para comparar o uso de água dos dispositivos e conexões conforme instalados com o uso dos dispositivos e conexões em conformidade com o Código Uniforme de Hidráulica e o Código Internacional de Hidráulica (baseline). |
| 479bf7f5f20-a0f9-429a-a565-6a4fa063b8eb | Translation Approved (0%) | Inspect all existing fittings or fixtures to ensure they are operating properly. | Inspecione todos os dispositivos e conexões existentes para garantir que estão operando de forma correta. |
| 480bf7f5f20-a0f9-429a-a565-6a4fa063b8eb | Translation Approved (0%) | Make any repairs needed to bring all fixtures into good working order or permanently turn off water supply to nonfunctional units. | Faça todos os reparos necessários para deixar todos os dispositivos em bom funcionamento ou desligar permanentemente o fornecimento de água para unidades sem funcionamento. |
| 481e4a4551c-1d6c-4e66-8d30-99a062f38904 | Translation Approved (0%) | Implement a fixture and fitting replacement and retrofit policy specifying that all newly installed toilets, urinals, private lavatory faucets, and showerheads that are eligible for labeling be WaterSense labeled (or a local equivalent for projects outside the U.S.). | Implemente uma política de substituição e modernização de dispositivos e conexões especificando que todos os vaso sanitários, mictórios, torneiras de lavatórios privados e chuveiros recém-instalados são elegíveis para receber o selo WaterSense (ou um equivalente local para projetos fora dos EUA). |
| 4829350035d-1459-4cb3-894f-4565c179fc6a | Translation Approved (0%) | Retail, Hospitality and Schools only | Apenas Varejo (Retail), Hoteleiro (Hospitality) e Escolas (Schools) |
| 483aa26ba70-c4d2-4fb0-9480-114a75940b0e | Translation Approved (0%) | Have in place a process and appliance water equipment purchasing policy for the building and site addressing the products listed in Table 2. | Implemente um processo e política de compra de equipamentos de água e utensílios para o edifício e terreno abordando os produtos listados na Tabela 2. |
| 484aa26ba70-c4d2-4fb0-9480-114a75940b0e | Translation Approved (0%) | The policy must cover at least those products purchased within the building and site management’s control. | A política deve cobrir pelo menos esses produtos adquiridos conforme o controle de gestão do terreno e edifício. |
| 48542a6a334-78ff-4ef1-90e0-bfefe919181e | Translation Approved (100%) | Table 2. | Tabela 2. |
| 48642a6a334-78ff-4ef1-90e0-bfefe919181e | Translation Approved (0%) | Standards for appliances | Normas para utensílios |
| 48732a2639b-73c1-485f-9e99-1de4b1a0216f | Translation Approved (0%) | Appliance | Utensílio |
| 488a133c840-d1ac-4e25-9b70-4decd992b896 | Translation Approved (100%) | Requirement | Requisito |
| 4890c188b38-4251-452b-9fa2-fc71ba9862c1 | Translation Approved (72%) | Residential clothes washer | Máquina de lavar roupa residencial |
| 490cf7d1b1c-4570-4448-b92a-9d61c7ee658b | Translation Approved (100%) | ENERGY STAR or performance equivalent | ENERGY STAR ou de desempenho equivalente |
| 491d0285da4-2e88-43a2-9d02-40ed6862e447 | Translation Approved (90%) | Commercial clothes washer | Máquina de lavar roupa comercial |
| 49235bc4c85-e4a3-4436-baa5-8f6a79d8ae28 | Translation Approved (100%) | CEE Tier 3A | CEE Classe 3A |
| 4936c4a83f6-6460-425a-88a3-323dcf1049c9 | Translation Approved (0%) | Residential dishwasher, standard or compact | Lava-louças residencial, padrão ou compacta |
| 494385410e6-f9a9-4731-a0f1-bd3a1242345c | Translation Approved (100%) | ENERGY STAR or performance equivalent | ENERGY STAR ou de desempenho equivalente |
| 4950c2e67b0-8ac8-411d-942e-e397faa60a7f | Translation Approved (90%) | Prerinse spray valve | Válvula de pulverização pré-lavagem |
| 49645d8d979-e108-49f1-b252-1eb7d64cc25d | Translation Approved (0%) | ≤ 1.3 gpm (4.9 lpm) | ≤ 1,3 gpm (4,9 lpm) |
| 497e702d73b-ab33-42b3-8f57-115364625a5e | Translation Approved (CM) | Ice machine | Máquina de gelo |
| 4985ff8783f-a295-4c0b-8fc2-fe9c5ffb02eb | Translation Approved (99%) | ENERGY STAR or performance equivalent, and use either air-cooled or closed-loop cooling, such as chilled or condenser water system | ENERGY STAR ou de desempenho equivalente e utilizar resfriamento a ar ou de ciclo fechado, como um sistema de água resfriada ou com condensador |
| 4997402bb9d-6b66-4dbc-bf2b-37fdbfd3c850 | Translation Approved (100%) | gpm = gallons per minute | gpm = galões por minuto |
| 500fc6f736a-f983-45a0-b94e-efc6e396dd8b | Translation Approved (100%) | lpm = liters per minute | lpm = litros por minuto |
| 5019bb509d8-df79-44e3-8569-ad283c96b760 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 5021529e9f5-0039-4025-ad09-061d73edb6f0 | Translation Approved (0%) | For building water use, confirm that calculations are up to date. | Para o uso de água do edifício, confirme se os cálculos estão atualizados. |
| 5031529e9f5-0039-4025-ad09-061d73edb6f0 | Translation Approved (0%) | Demonstrate that all applicable purchases made during the performance period meet the requirements of the fixture and fitting replacement and retrofit policy. | Demonstre que todas as compras aplicáveis realizadas durante o período de desempenho atendem aos requisitos a política de substituição e modernização de dispositivos e conexões. |
| 50408b4a502-22c7-4a15-b188-663b822f09ef | Translation Approved (0%) | For appliances, demonstrate that appliances purchased and installed within the building meet the requirements listed in Table 2. | Para utensílios, demonstre que todos os utensílios comprados e instalados no edifício atendem aos requisitos listados na Tabela 2. |
| 50508b4a502-22c7-4a15-b188-663b822f09ef | Translation Approved (0%) | Appliances not listed are not subject to any additional requirements. | Os utensílios não listados não estão sujeitos a requisitos adicionais. |
| 5068cf142ac-a96c-4937-90be-42d56967bef1 | Translation Approved (100%) | Option 2. | Opção 2. |
| 5078cf142ac-a96c-4937-90be-42d56967bef1 | Translation Approved (77%) | Metered Water Use | Uso de água medido |
| 508494fb1ba-41f1-4a01-b695-6e0f2df08640 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 509255e34d1-5ac2-40e6-b203-07b781a01825 | Translation Approved (0%) | Meter fixtures and fittings and record metered data for one year to establish a water-use baseline. | Meça os dispositivos e conexões e registrar os dados medidos por um ano para estabelecer uma baseline de uso da água. |
| 510515d20ed-28b2-4a5e-9066-9000284b7b5c | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 511618118ef-09ee-4853-b96d-ce11f7092baf | Translation Approved (0%) | For projects with at least 80% of fixtures and fittings metered, show that the water-use baseline has been maintained. | Para projetos com no mínimo 80% dos dispositivos e conexões medidos, mostrar que a baseline de uso da água foi mantida. |
| 512747de68e-2c6d-4ac5-9a06-5e6f4bc70473 | Translation Approved (100%) | WE Prerequisite: Building-Level Water Metering | Pré-requisito WE: Medição de Água do Edifício (WE Prerequisite: Building-Level Water Metering) |
| 51391253a97-dbdb-4a0c-8522-8d9f083548b0 | Translation Approved (100%) | Required | Obrigatório |
| 514eb9469c4-a08f-4cc7-bf19-40436c5c446c | Translation Approved (100%) | EB:O&M | EB:O&M |
| 5159b526c73-c6b6-42ea-8397-ed05f521701e | Translation Approved (100%) | This prerequisite applies to | Este pré-requisito se aplica a: |
| 516f745fe18-e4d7-40bd-bb84-6fa02ea4427a | Translation Approved (100%) | Existing Buildings: Operations & Maintenance | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) |
| 5176241de20-deb2-4d11-9864-66aec6e31a1c | Translation Approved (100%) | EB:O&M Schools | EB:O&M Escolas (Schools) |
| 518717cceb6-9b32-41da-81f9-4aaedae4199f | Translation Approved (100%) | EB:O&M Retail | EB:O&M Varejo (Retail) |
| 519163d044a-1642-4bfc-a67b-ebf6ab0ecae9 | Translation Approved (100%) | EB:O&M Data Centers | EB:O&M Centros de Dados (Data Centers) |
| 52051c8cf82-15d3-471f-9154-972c29563fe2 | Translation Approved (100%) | EB:O&M Hospitality | EB:O&M Hoteleiro (Hospitality) |
| 5213a3a0ad3-0495-4b19-9837-180c4033d28f | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) |
| 5228f4960cd-79f5-473d-a11d-00201ab058a5 | Translation Approved (100%) | Intent | Objetivo |
| 523d13c4bc8-f469-4c69-9414-2a10ec9e4a95 | Translation Approved (CM) | To support water management and identify opportunities for additional water savings by tracking water consumption. | Apoiar a gestão da água e identificar oportunidades de economias adicionais de água rastreando o consumo de água. |
| 524b0976e18-c06c-41dc-b565-692cfa05828d | Translation Approved (100%) | Requirements | Requisitos |
| 525156b15d4-15be-461a-8a59-b599603000f1 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 52622f49486-3c9f-4785-811e-33912c143632 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 5272fc23f3b-9a7d-469f-bc0d-e11d711afb91 | Translation Approved (89%) | Have permanently installed water meters that measure the total potable water use for the building and associated grounds. | Instale hidrômetros permanentes que meçam o uso total de água potável para o edifício e os terrenos associados. |
| 5282fc23f3b-9a7d-469f-bc0d-e11d711afb91 | Translation Approved (0%) | Metering of any gray or reclaimed water supplied to the building is encouraged but not required. | A medição de água recuperada ou cinza fornecida ao edifício é incentivada, mas não necessária. |
| 5298f37c1e1-da24-452d-9350-b451f9f03f3f | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 530ba66cae8-88f1-4fc9-a3be-f208cd9afe47 | Translation Approved (0%) | Record meter data on a monthly basis and compile; meter readings can be manual or automated. | Registre os dados do medidor mensalmente e compile; as leituras do medidor podem ser manuais ou automáticas. |
| 5313d0e5b91-51bd-4e54-937a-f8410632b360 | Translation Approved (100%) | Commit to sharing with USGBC the resulting whole-project water usage data for a five-year period beginning on the date the project accepts LEED certification or typical occupancy, whichever comes first. | Comprometa-se em compartilhar com o USGBC os dados de uso de água de todo o projeto por um período de cinco anos, começando na data em que o projeto aceitar a certificação LEED ou a ocupação típica, o que ocorrer primeiro. |
| 532abb2a24b-81d0-4902-afc2-44b5cca289c7 | Translation Approved (CM) | This commitment must carry forward for five years or until the building changes ownership or lessee. | Esse compromisso deve ser mantido por cinco anos ou até que o edifício mude de proprietário ou arrendatário. |
| 5332c270e47-1b42-4ace-b200-e8ff6c44085d | Translation Approved (100%) | WE Credit: Outdoor Water Use Reduction | Crédito WE: Redução do Uso de Água Externo (Outdoor Water Use Reduction) |
| 534d169fe5a-5959-4831-a3c4-a1a7c53ea04c | Translation Approved (100%) | EB:O&M | EB:O&M |
| 5352c95ba8c-479e-4a19-9bb8-bbaf84eae116 | Translation Approved (100%) | 1–2 points | 1 a 2 pontos |
| 536872c1daf-97aa-4734-8e36-8f26d622ed21 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 53709b94915-440f-433f-9f87-6defd6a69dec | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1–2 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 a 2 pontos) |
| 5385d5975bb-02d9-4451-9958-5883a44e2717 | Translation Approved (100%) | EB:O&M Schools (1–2 points) | EB:O&M Escolas (Schools) (1 a 2 pontos) |
| 53929d8b6ca-1b97-4822-b6a1-ec9b391193c7 | Translation Approved (100%) | EB:O&M Retail (1–2 points) | EB:O&M Varejo (Retail) (1 a 2 pontos) |
| 540c7681eba-ea5c-4adc-b668-d6c4c3fb9ef0 | Translation Approved (100%) | EB:O&M Data Centers (1–2 points) | EB:O&M Centros de Dados (Data Centers) (1 a 2 pontos) |
| 541945b15e1-9e17-4bbe-b92c-1d83285ae21e | Translation Approved (100%) | EB:O&M Hospitality (1–2 points) | EB:O&M Hoteleiro (Hospitality) (1 a 2 pontos) |
| 5426fcc487d-3b35-461f-8ecc-d90923ba9efe | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1–2 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 a 2 pontos) |
| 5432ca2252b-f6c7-4d39-8105-ac604448ed01 | Translation Approved (100%) | Intent | Objetivo |
| 544eea174c8-f88a-48b8-a309-f63accf01f2b | Translation Approved (CM) | To reduce outdoor water consumption. | Reduzir o consumo de água externo. |
| 545c8328e84-7e74-4fe4-ae0b-aaa883db6f96 | Translation Approved (100%) | Requirements | Requisitos |
| 5463ef57b5d-dd7c-42c4-a8a1-2e39e62814dd | Translation Approved (89%) | EBOM, Retail, Data Centers, Hospitality, Schools, Warehouses & Distribution Centers | EBOM, Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Escolas (Schools), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 547f3ad2c89-f961-4b7b-b1ab-ae1a321cfd73 | Translation Approved (100%) | Reduce outdoor water use through one of the following options. | Reduza o uso de água externo por meio de uma das seguintes opções. |
| 548f3ad2c89-f961-4b7b-b1ab-ae1a321cfd73 | Translation Approved (CM) | Nonvegetated surfaces, such as permeable or impermeable pavement, should be excluded from landscape area calculations. | Superfícies sem vegetação, como pavimentos permeáveis ou impermeáveis, devem ser excluídas dos cálculos de área de paisagismo. |
| 549f3ad2c89-f961-4b7b-b1ab-ae1a321cfd73 | Translation Approved (100%) | Athletic fields and playgrounds (if vegetated) and food gardens may be included or excluded at the project team’s discretion. | Campos esportivos e parques infantis (se tiverem vegetação) e hortas podem ser incluídos ou excluídos, a critério da equipe do projeto. |
| 55003abce48-1543-41f3-aa58-10498ab55d14 | Translation Approved (0%) | If landscape irrigation is not submetered, use Option 2. | Se a irrigação de paisagismo não for submedida, use a Opção 2. |
| 551d253ae2e-2a87-465b-9ffb-7f1c60c6cf18 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 55257cb0364-74d5-437b-8c27-79657d71ff3b | Translation Approved (100%) | Option 1. | Opção 1. |
| 55357cb0364-74d5-437b-8c27-79657d71ff3b | Translation Approved (0%) | No Irrigation Required (2 points) | Sem irrigação necessária (2 pontos) |
| 554cba869aa-b433-4202-abe0-1e64bda4a32a | Translation Approved (83%) | Show that the landscape does not require irrigation beyond a maximum two-year plant establishment period. | Demonstre que o paisagismo não requer um sistema de irrigação permanente além de um período de estabelecimento máximo de dois anos. |
| 555d8f88386-4b07-4025-b2ed-2c5a31749815 | Translation Approved (100%) | Option 2. | Opção 2. |
| 556d8f88386-4b07-4025-b2ed-2c5a31749815 | Translation Approved (0%) | No Irrigation Meter Installed: Calculated Water Budget (1–2 points) | Sem medidor de irrigação instalado: orçamento de água calculado (1 a 2 pontos) |
| 557e212cbb6-c28a-4e9a-bbce-bb145bb58c9f | Translation Approved (0%) | Use the existing landscape to calculate the landscape water requirement using the EPA WaterSense Water Budget Tool. | Use a paisagem existente para calcular o requisito de água da paisagem usando a Ferramenta de Demanda de Água WaterSense da EPA. |
| 558a2c5488e-c6c4-48c1-98f3-4ab4fa5b69ea | Translation Approved (0%) | Install an irrigation meter. | Instale um medidor de irrigação. |
| 55977232672-5e54-462f-95ae-0f492ae96ac9 | Translation Approved (100%) | Option 3. | Opção 3. |
| 56077232672-5e54-462f-95ae-0f492ae96ac9 | Translation Approved (0%) | Irrigation Meter Installed (1–2 points) | Medidor de irrigação instalado (1 a 2 pontos) |
| 561b9b12982-e580-4eff-b61c-0eb99f894c28 | Translation Approved (0%) | The baseline is established using the annual average of at least 3 years of consecutive data out of the last 5 years. | A baseline é estabelecida usando a média anual de pelo 3 anos de dados consecutivos dos últimos cinco anos. |
| 562ffd883e9-9c4e-4ccb-8c43-07be56d3d719 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 563cd894a01-9214-4e02-8e53-769029f3a8c6 | Translation Approved (100%) | Option 1. | Opção 1. |
| 564cd894a01-9214-4e02-8e53-769029f3a8c6 | Translation Approved (100%) | No Irrigation Required (2 points) | Sem irrigação necessária (2 pontos) |
| 565f8a394f6-257a-452e-8a64-0f3e89c10ee8 | Translation Approved (100%) | None. | Nenhum. |
| 566bd990faf-7b33-4757-8b81-8b109bb7ae49 | Translation Approved (100%) | Option 2. | Opção 2. |
| 567bd990faf-7b33-4757-8b81-8b109bb7ae49 | Translation Approved (93%) | No Irrigation Meter: Calculated Water Budget (1–2 points) | Sem medidor de irrigação: orçamento de água calculado (1 a 2 pontos) |
| 568343fe76e-11e5-4f7c-a0a3-cc09ae79c767 | Translation Approved (0%) | Points are earned according to Table 1. | Os pontos são concedidos de acordo com a Tabela 1. |
| 569718303dc-7f36-47e6-89f5-a5606b9ad2ee | Translation Approved (100%) | Option 3. | Opção 3. |
| 570718303dc-7f36-47e6-89f5-a5606b9ad2ee | Translation Approved (100%) | Irrigation Meter Installed (1–2 points) | Medidor de irrigação instalado (1 a 2 pontos) |
| 5719605f568-3b64-4ab0-8673-83b7f79253cb | Translation Approved (0%) | Demonstrate a reduction in outdoor water use over the most recent 12 months compared with the established baseline. | Demonstre uma redução do uso de água externo durante os 12 meses mais recentes comparado com a baseline estabelecida. |
| 5729605f568-3b64-4ab0-8673-83b7f79253cb | Translation Approved (100%) | Points are earned according to Table 1. | Os pontos são concedidos de acordo com a Tabela 1. |
| 57386fb6010-c8dc-4170-bba5-78a6cf31b526 | Translation Approved (100%) | Table 1. | Tabela 1. |
| 57486fb6010-c8dc-4170-bba5-78a6cf31b526 | Translation Approved (CM) | Points for reducing irrigation water | Pontos por reduzir a água de irrigação |
| 575e2b8e38a-7dd4-4762-a963-0cc5c99deb2b | Translation Approved (100%) | Percentage reduction from baseline | Porcentagem de redução em relação à baseline |
| 576a934596d-ad58-4d7a-8b4a-b467eac476d2 | Translation Approved (100%) | Points | Pontos |
| 577edd056ea-67b5-4a3c-820f-eef549659018 | Translation Approved (100%) | 30% | 30% |
| 5785dcde5df-c35d-4ff9-8f3a-07462de7ff96 | Translation Approved (100%) | 1 | 1 |
| 579a3f5a21f-459c-4136-bd67-94633a86d99a | Translation Approved (100%) | 40% | 40% |
| 58054ec4762-4ef2-401d-8d7c-2e03b4351693 | Translation Approved (100%) | 2 | 2 |
| 58149391007-968e-4a00-9a4d-a0e31140ce0c | Translation Approved (100%) | WE Credit: Indoor Water Use Reduction | Crédito WE: Redução do Uso de Água Interno (WE Credit: Indoor Water Use Reduction) |
| 582dac2a165-c5e8-42e3-9594-c632347b95eb | Translation Approved (100%) | EB:O&M | EB:O&M |
| 583d9155009-6222-493d-961a-7f2749c1dbcc | Translation Approved (100%) | 1–5 points | 1 a 5 pontos |
| 584703e9177-1e74-4b38-b90f-5627ae7bf385 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 5850e8ddfff-1c1e-44f3-aa44-f6fa0750244e | Translation Approved (96%) | Existing Buildings: Operations & Maintenance (1–5 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 a 5 pontos) |
| 586f0ad9c6b-4e0f-449f-b5b5-81ec1d4fa28b | Translation Approved (95%) | EB:O&M Schools (1–5 points) | EB:O&M Escolas (Schools) (1 a 5 pontos) |
| 587173271d6-1f5d-4a7e-8d28-137d05f5c9ac | Translation Approved (95%) | EB:O&M Retail (1–5 points) | EB:O&M Varejo (Retail) (1 a 5 pontos) |
| 588134370dc-c1f9-4416-b0db-59b1e7187cd3 | Translation Approved (96%) | EB:O&M Data Centers (1–4 points) | EB:O&M Centros de Dados (Data Centers) (1 a 4 pontos) |
| 589e618f761-dae5-45de-ae31-80d1e593ec33 | Translation Approved (95%) | EB:O&M Hospitality (1–5 points) | EB:O&M Hoteleiro (Hospitality) (1 a 5 pontos) |
| 59055a39433-dec0-4724-ab33-dbe241027bb4 | Translation Approved (97%) | EB:O&M Warehouses & Distribution Centers (1–5 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 a 5 pontos) |
| 5912468aca2-0183-4ae9-b9cb-6d50cfae3614 | Translation Approved (100%) | Intent | Objetivo |
| 59230fa15a4-d071-4133-9234-c7a317ea26e4 | Translation Approved (100%) | To reduce indoor water consumption. | Reduzir o consumo de água interno. |
| 5932f6e8331-e2bd-46f7-a374-c844e80eed0c | Translation Approved (100%) | Requirements | Requisitos |
| 59479722ad9-18fb-4c47-9261-d0a7facfee41 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 595e64dea32-47b5-4bd1-a1ad-234858acf3b7 | Translation Approved (100%) | Option 1. | Opção 1. |
| 596e64dea32-47b5-4bd1-a1ad-234858acf3b7 | Translation Approved (0%) | Calculated Water Use (1–5 points except Data Centers, 1–4 points Data Centers) | Uso de água calculado (1 a 5 pontos exceto Centros de Dados, 1 a 4 pontos Centros de Dados) |
| 5972f8dfea2-c358-406f-892e-0b6335c22f7d | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 5987dc5e823-5797-4bdb-ab01-c28375cec28c | Translation Approved (100%) | None. | Nenhum. |
| 599b9da6c67-f79d-4435-8153-f1aaadab2f24 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 600b0f1f5ba-961f-4485-a509-626a343471cf | Translation Approved (0%) | Have fixtures that use less water than the baseline calculated in WE Prerequisite Indoor Water-Use Reduction. | Tenha dispositivos que usem menos água que a baseline calculada no Pré-requisito WE: Redução do Uso de Água do Interno (WE Prerequisite: Indoor Water Use Reduction). |
| 601b0f1f5ba-961f-4485-a509-626a343471cf | Translation Approved (91%) | Points are awarded according to Table 1. | Os pontos são concedidos de acordo com a Tabela 1. |
| 6020568f8c3-de6f-4cff-925f-93461caf5ea0 | Translation Approved (100%) | Table 1. | Tabela 1. |
| 6030568f8c3-de6f-4cff-925f-93461caf5ea0 | Translation Approved (0%) | Points for reducing calculated water use beyond the prerequisite level. | Pontos por redução do uso de água calculado além do nível do pré-requisito. |
| 604bc5c97b0-bbe8-410a-b2c9-0b2e1a407772 | Translation Approved (0%) | Additional percentage reduction | Porcentagem de redução adicional |
| 605e66a4f71-3366-44a6-91d5-16b94cdb10b4 | Translation Approved (0%) | Points (except Data Centers) | Pontos (exceto Centros de Dados) |
| 606463f18c0-66d2-4919-86bd-ec150e71fb13 | Translation Approved (80%) | Points (Data Centers) | Pontos (Centros de Dados) |
| 6075227bd43-b764-4543-b5d0-d1b740a8b5aa | Translation Approved (100%) | 10% | 10% |
| 6087a7ca7eb-4581-4514-87f8-a6afe0a65845 | Translation Approved (100%) | 1 | 1 |
| 60907e759b1-9fba-4a1b-9703-4f18c2afeae0 | Translation Approved (100%) | 1 | 1 |
| 610960e1caa-c2bd-4368-bc89-49efd9b631a8 | Translation Approved (100%) | 15% | 15% |
| 6117e08128b-3eae-4089-9f81-9cf2a80878aa | Translation Approved (100%) | 2 | 2 |
| 612e477f931-a66e-4ec5-bbf4-5dda106cb7ae | Translation Approved (100%) | 2 | 2 |
| 6138c1c6cdc-30d8-4b4b-9b19-0ef6f3d08796 | Translation Approved (100%) | 20% | 20% |
| 6145dbb6015-b273-42c9-a509-e31dca51e313 | Translation Approved (100%) | 3 | 3 |
| 615c5e43bbd-6299-4463-80fc-aad685a5f91e | Translation Approved (100%) | 3 | 3 |
| 616d31f8065-764e-45bf-a93f-f219d8068162 | Translation Approved (100%) | 25% | 25% |
| 6172962a1bb-10b3-413f-b3ef-73bbfc9b3a21 | Translation Approved (100%) | 4 | 4 |
| 61881b5e47d-234e-4eca-990b-58db9a6834fb | Translation Approved (100%) | 4 | 4 |
| 619d2e4916c-e64d-44f0-bf2a-9c1809b0da6d | Translation Approved (100%) | 30% | 30% |
| 6203c8549dd-5e72-4b19-a5fc-cc0e41385608 | Translation Approved (100%) | 5 | 5 |
| 62198f07dbd-6360-4a17-b641-7b53da3da372 | Translation Approved (0%) | Confirm that calculations are up to date. | Confirme se os cálculos estão atualizados. |
| 62298f07dbd-6360-4a17-b641-7b53da3da372 | Translation Approved (0%) | Demonstrate that all purchases made since the end of the performance period meet the design performance requirements. | Demonstre que todas as compras realizadas desde o final do período de desempenho atendem aos requisitos de desempenho do projeto. |
| 623742837f7-8f9f-4891-a27f-09439f87972c | Translation Approved (100%) | OR | OU |
| 624fac6cd50-f1be-48f1-ad32-48dc56275e37 | Translation Approved (100%) | Option 2. | Opção 2. |
| 625fac6cd50-f1be-48f1-ad32-48dc56275e37 | Translation Approved (96%) | Metered Water Use (1-5 points except Data Centers, 1-4 points Data Centers) | Uso de água medido (1 a 5 pontos exceto Centros de Dados, 1 a 4 pontos Centros de Dados) |
| 626483e2eb9-2fa6-4199-bb13-488017be4397 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 627b76e47de-598a-47da-81f0-1f67014764a4 | Translation Approved (96%) | Meter fixtures and fittings, and record meter data for one year to establish a water-use baseline. | Meça os dispositivos e conexões e registrar os dados medidos por um ano para estabelecer uma baseline de uso da água. |
| 628d60c0312-2183-49ca-82eb-46a0bfc4c551 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 629b02ee1ba-f8ab-49cd-ae2f-31c25f832a5b | Translation Approved (75%) | For projects with at least 80% of fixtures and fittings metered, show a reduction from the baseline year of meter data. | Para projetos com no mínimo 80% dos dispositivos e conexões medidos, mostre uma redução da baseline dos dados do medidor. |
| 630fad251e6-8721-42bf-8775-12124a41b65d | Translation Approved (100%) | Table 2. | Tabela 2. |
| 631fad251e6-8721-42bf-8775-12124a41b65d | Translation Approved (74%) | Points for reducing metered water use | Pontos por redução do uso de água medido |
| 6321bab5826-f9f6-436e-8e87-cd596c36a4ee | Translation Approved (91%) | percentage reduction | Porcentagem de redução |
| 633a06721df-61dc-4bed-873a-e8c98bc26afd | Translation Approved (100%) | Points (except Data Centers) | Pontos (exceto Centros de Dados) |
| 6340180e825-b9f5-41e0-ad41-31ae352b9a7c | Translation Approved (100%) | Points (Data Centers) | Pontos (Centros de Dados) |
| 6352c3136f1-45b0-48c3-a637-f287ad6d516e | Translation Approved (100%) | <5% | <5% |
| 636bd0e92a1-b48b-472c-b934-a42f279141e0 | Translation Approved (100%) | 1 | 1 |
| 637b05d2f6d-8c9a-4c4b-a870-86614c880157 | Translation Approved (100%) | 1 | 1 |
| 638ad1ecd46-3182-4665-96a3-50ce3b7d048f | Translation Approved (100%) | 5% | 5% |
| 639f605db54-c368-45f0-8b46-2401546b2778 | Translation Approved (100%) | 2 | 2 |
| 6405704f8a7-9f12-433f-b0e4-c36c99788188 | Translation Approved (100%) | 2 | 2 |
| 6417650b396-a603-4e9f-b7b2-f4c295edbd27 | Translation Approved (100%) | 10% | 10% |
| 642160b1e1b-e925-47f6-bc84-fb1781a73b50 | Translation Approved (100%) | 3 | 3 |
| 64375f0248f-3352-42d1-80c1-7f8f4ffa4189 | Translation Approved (100%) | 3 | 3 |
| 644b146d636-6897-46fb-b2f5-14c55ccdaa3c | Translation Approved (100%) | 15% | 15% |
| 6454da9b18a-75ab-4d72-9d10-476a4412f504 | Translation Approved (100%) | 4 | 4 |
| 6469ccaae5f-d118-4ea8-aa9d-751fd61b2d3e | Translation Approved (100%) | 4 | 4 |
| 6470d450bf5-66a9-4b16-a15b-6ada4bd40c69 | Translation Approved (100%) | 20% | 20% |
| 64830140390-b16c-4026-8bc8-46b41181c7d2 | Translation Approved (100%) | 5 | 5 |
| 64991e1d36c-42d1-40a8-99fb-22ec98fa51da | Translation Approved (0%) | -- | -- |
| 6502461df38-3164-439c-a993-d0ba618aa78f | Translation Approved (100%) | WE Credit: Cooling Tower Water Use | Crédito WE: Uso de Água de Torre de Resfriamento (WE Credit: Cooling Tower Water Use) |
| 6518a7f4d55-1adf-4cfa-8679-e982ea3c9453 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 652363182cc-6c64-4be8-a661-3857ccf1f89f | Translation Approved (100%) | 2–4 points | 2 a 4 pontos |
| 6533cb122cf-831a-4246-a550-450abf0529eb | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 654ffd8a221-eadf-47f0-b55a-9068a2d4809f | Translation Approved (92%) | Existing Buildings: Operations & Maintenance (2–3 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (2 a 3 pontos) |
| 65501089541-aa27-416b-b413-31d60c4874e8 | Translation Approved (90%) | EB:O&M Schools (2–3 points) | EB:O&M Escolas (Schools) (2 a 3 pontos) |
| 656a69816ab-2a25-4717-9827-e41564a719da | Translation Approved (90%) | EB:O&M Retail (2–3 points) | EB:O&M Varejo (Retail) (2 a 3 pontos) |
| 657deecb904-d42d-44bb-b9c9-52d312be23a1 | Translation Approved (96%) | EB:O&M Data Centers (2–4 points) | EB:O&M Centros de Dados (Data Centers) (2 a 4 pontos) |
| 658577e5933-6b7d-4792-9b09-4e058a948e1a | Translation Approved (90%) | EB:O&M Hospitality (2–3 points) | EB:O&M Hoteleiro (Hospitality) (2 a 3 pontos) |
| 6598841b912-bfc3-490f-9cbe-9c572a76e3a2 | Translation Approved (93%) | EB:O&M Warehouses & Distribution Centers (2–3 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (2 a 3 pontos) |
| 6604d92f6cb-1f8d-4fe7-a38f-fa732d18078d | Translation Approved (100%) | Intent | Objetivo |
| 661a51dea50-740b-48ff-9dd9-f9bee5dedd00 | Translation Approved (CM) | To conserve water used for cooling tower makeup while controlling microbes, corrosion, and scale in the condenser water system. | Conservar a água usada para reposição da torre de resfriamento enquanto se controla micróbios, corrosão e crostas no sistema de água do condensador. |
| 662189fc2e1-6d1d-4179-a1ad-6dee71e2c471 | Translation Approved (100%) | Requirements | Requisitos |
| 663729d3a98-beef-4188-ad30-08a89ba651c3 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 6649a37719f-252f-4044-a8e7-7e4c818d5cc2 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 665bcc07d51-0412-48cc-9c05-8b567f48d3fb | Translation Approved (0%) | For cooling towers and evaporative condensers, conduct a potable water analysis within five years of submission for certification, measuring at least the five control parameters listed in Table 1. | Para torres de resfriamento e condensadores evaporativos, conduza uma análise de água potável em até cinco anos do envio da certificação, medindo pelo menos os cinco parâmetros de controle listados na Tabela 1. |
| 666876a4cee-d966-46e0-a954-c320bdfe5e28 | Translation Approved (100%) | Table 1. | Tabela 1. |
| 667876a4cee-d966-46e0-a954-c320bdfe5e28 | Translation Approved (CM) | Maximum concentrations for parameters in condenser water | Concentrações máximas para parâmetros na água do condensador |
| 668d7a525c0-000b-4276-a8d3-a7b25027fce5 | Translation Approved (CM) | Parameter | Parâmetro |
| 6696163395a-2c97-4da9-b123-fb7871594227 | Translation Approved (CM) | Maximum level | Nível máximo |
| 670d9ea41fe-6e9f-44d6-aca1-23a9e8d06738 | Translation Approved (CM) | Ca (as CaCO<718>3</718>) | Ca (como CaCO<718>3</718>) |
| 671d1d110cc-90ea-449b-9d80-79f90eebaca2 | Translation Approved (CM) | 1,000 ppm | 1.000 ppm |
| 672f02c15a6-a280-45df-bf73-55654d47eae9 | Translation Approved (CM) | Total alkalinity | Alcalinidade total |
| 673ef62ffe2-6e41-4e57-919f-bf64f52c41fb | Translation Approved (100%) | 1,000 ppm | 1.000 ppm |
| 6744ff5ffff-4b9c-4849-bf9f-38f5731cea16 | Translation Approved (0%) | SiO<731>2</731> | SiO<731>2</731> |
| 675be7d4058-17a1-4a59-b605-d3ac68df3cbd | Translation Approved (100%) | 100 ppm | 100 ppm |
| 6767ff35938-cefb-43dd-9ef5-9fa3c98a0b7b | Translation Approved (0%) | Cl<738>-</738> | Cl<738>-</738> |
| 6778e0de769-e02e-4972-b7e0-b850ee5bed35 | Translation Approved (100%) | 250 ppm | 250 ppm |
| 678f0f31ab8-0ca9-4d5c-9655-0d79f8db12ab | Translation Approved (CM) | Conductivity | Condutividade |
| 67937fbf343-619f-43ee-a544-259eba33abf5 | Translation Approved (0%) | 2000 µS/cm | 2000 µS/cm |
| 680341d8b65-8343-45c1-aa7f-905edeb08199 | Translation Approved (0%) | ppm = parts per million | ppm = partes por milhão |
| 681936ab89a-197f-4091-8220-f66501f722ec | Translation Approved (0%) | µS/cm = micro siemens per centimeter | µS/cm = micro siemens por centímetro |
| 682ade407b6-9f51-4026-a1aa-7b22a850149b | Translation Approved (CM) | Calculate the number of cooling tower cycles by dividing the maximum allowed concentration level of each parameter by the actual concentration level of each parameter found in the potable makeup water. | Calcule o número de ciclos da torre de resfriamento dividindo o nível máximo de concentração permitido de cada parâmetro pelo nível de concentração real de cada parâmetro encontrado na água de reposição potável. |
| 683ade407b6-9f51-4026-a1aa-7b22a850149b | Translation Approved (CM) | Limit cooling tower cycles to avoid exceeding maximum values for any of these parameters. | Limite os ciclos da torre de resfriamento para não exceder os valores máximos em nenhum desses parâmetros. |
| 6849bde2b58-b73b-4863-98c4-4fc94a985bcf | Translation Approved (100%) | Table 2. | Tabela 2. |
| 6859bde2b58-b73b-4863-98c4-4fc94a985bcf | Translation Approved (CM) | Points for cooling tower cycles | Pontos para ciclos da torre de resfriamento |
| 68696f3c02f-201a-4f3a-93d4-a4b553c43156 | Translation Approved (CM) | Cooling tower cycles | Ciclos da torre de resfriamento |
| 687d4c40024-7385-41eb-8175-21d41597290c | Translation Approved (100%) | Points (except Data Centers) | Pontos (exceto Centros de Dados) |
| 68898984c15-6b25-49d0-b56b-5befea8b4cc3 | Translation Approved (100%) | Points (Data Centers) | Pontos (Centros de Dados) |
| 6897acefe15-0d43-44f3-b04c-b7b926a545bc | Translation Approved (100%) | Maximum number of cycles achieved without exceeding any filtration levels or affecting operation of condenser water system (up to maximum of 10 cycles) | Número máximo de ciclos alcançado sem exceder nenhum nível de filtração nem afetar a operação do sistema de água do condensador (até no máximo 10 ciclos) |
| 690a31ea34d-fdda-4b2d-94a5-32331f57d536 | Translation Approved (100%) | 2 | 2 |
| 691773f1355-95b4-4701-bc80-719c7c14cb08 | Translation Approved (100%) | 2 | 2 |
| 692286582f0-c20d-44de-86dd-07786481e6df | Translation Approved (CM) | Achieve a minimum 10 cycles by increasing the level of treatment in condenser or make-up water | Alcançar no mínimo 10 ciclos aumentando o nível de tratamento na água do condensador ou de reposição |
| 6930a87728e-8a73-4391-aa1d-e014198179e5 | Translation Approved (100%) | OR | OU |
| 694796d99d4-e7e5-4c5c-9529-523d0e9f3c68 | Translation Approved (CM) | Meet the minimum number of cycles to earn 1 point and use a minimum 20% recycled nonpotable water | Cumprir com o número mínimo de ciclos para obter 1 ponto e usar no mínimo 20% de água reciclada não potável |
| 695d727fc32-b594-4dab-a2b0-69bd4ecebe3b | Translation Approved (100%) | 3 | 3 |
| 69621c7aa9d-bff4-48cc-94a8-5107e73cb90c | Translation Approved (100%) | 4 | 4 |
| 697731f4507-4ab6-4931-8cd9-9688e1570c04 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 698b5dc0e0b-0297-4a19-9ed4-27fcc4ca87f4 | Translation Approved (100%) | None. | Nenhum. |
| 699435c70ca-0397-4d42-97cd-cbf0249e922c | Translation Approved (100%) | WE Credit: Water Metering | Crédito WE: Medição de Água (WE Credit: Water Metering) |
| 7005e4d5f31-1aac-413f-a9ff-5f8538465303 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 70100ccef81-fe27-4d45-b3a1-c0a7c2bfc4aa | Translation Approved (100%) | 2 points | 2 pontos |
| 70273790cd7-1592-4ae3-b9d4-6068bd38a032 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 703ef37dc91-f1a4-4e3b-9310-bcc5cdaa5724 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1-2 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 a 2 pontos) |
| 7048ef191f4-aa19-4207-8a63-fc8ac8be4714 | Translation Approved (100%) | EB:O&M Schools (1-2 points) | EB:O&M Escolas (Schools) (1 a 2 pontos) |
| 705af633064-d0de-452c-a37b-7198cf3477ed | Translation Approved (100%) | EB:O&M Retail (1-2 points) | EB:O&M Varejo (Retail) (1 a 2 pontos) |
| 70638c36e97-d37f-4d2b-98c1-6ddfb4f54983 | Translation Approved (100%) | EB:O&M Data Centers (1-2 points) | EB:O&M Centros de Dados (Data Centers) (1 a 2 pontos) |
| 70717370f93-ba68-4575-a0b5-9849949cee08 | Translation Approved (100%) | EB:O&M Hospitality (1-2 points) | EB:O&M Hoteleiro (Hospitality) (1 a 2 pontos) |
| 708807cb4c2-8b5b-423b-9b3a-91c6b137cbbf | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1-2 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 a 2 pontos) |
| 709fc67f29d-3d56-494b-8906-4abc5ddb47b1 | Translation Approved (100%) | Intent | Objetivo |
| 7108cb1132c-cfa0-4789-b956-d053275a2bd8 | Translation Approved (100%) | To support water management and identify opportunities for additional water savings by tracking water consumption. | Apoiar a gestão da água e identificar oportunidades de economias adicionais de água rastreando o consumo de água. |
| 711e1de6a8c-517b-4559-9aa6-d389845b2c65 | Translation Approved (100%) | Requirements | Requisitos |
| 712504dbd5c-a2be-47bd-ad88-ecd12e9fe707 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 7139f321e26-fc46-4760-8e34-36351e3e7501 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 714a3dedd3f-3bbc-42ff-b707-d4921263e6b0 | Translation Approved (0%) | Establish permanently installed meters; 1 point for two water subsystems, 2 points for four or more water subsystems: | Estabeleça medidores instalados permanentemente; 1 ponto por dois subsistemas de água, 2 pontos por quatro ou mais subsistemas: |
| 7156666d56b-8893-4cc1-8a7c-352b73654dfd | Translation Approved (96%) | Irrigation. | Irrigação. |
| 7166666d56b-8893-4cc1-8a7c-352b73654dfd | Translation Approved (CM) | Meter water systems serving at least 80% of the irrigated landscaped area. | Meça sistemas de água que atendam a pelo menos 80% da área de paisagismo irrigada. |
| 7176666d56b-8893-4cc1-8a7c-352b73654dfd | Translation Approved (0%) | Calculate the percentage of irrigated landscape area as the total metered irrigated landscape area divided by the total irrigated landscape area. | Calcule a porcentagem da área de paisagem irrigada como a área total de paisagem irrigada medida dividida pela área total de paisagem irrigada. |
| 7186666d56b-8893-4cc1-8a7c-352b73654dfd | Translation Approved (100%) | Landscape areas fully covered with xeriscaping or native vegetation that requires no routine irrigation may be excluded from the calculation. | Áreas de paisagismo totalmente cobertas por xeriscape ou vegetação nativa que não precisam de irrigação rotineira podem ser excluídas do cálculo. |
| 7198785596c-89d1-4f3d-b488-c719ef9a6d1d | Translation Approved (CM) | Indoor plumbing fixtures and fittings. | Dispositivos e conexões hidráulicos internos. |
| 7208785596c-89d1-4f3d-b488-c719ef9a6d1d | Translation Approved (92%) | Meter water systems serving at least 80% of the indoor plumbing fixtures and fittings listed in WE Prerequisite Indoor Water-Use Reduction, either directly or by deducting all other measured water use from the measured total water consumption of the building and grounds. | Meça sistemas de água que atendam a pelo menos 80% dos dispositivos e conexões internos de encanamento descritos no Pré-requisito WE: Redução do Uso de Água Interno (WE Prerequisite Indoor Water Use Reduction), diretamente ou subtraindo todos os outros usos de água medidos do consumo total de água do edifício e do terreno. |
| 721a58a3408-f5d3-434e-81c0-225137a1f6fa | Translation Approved (98%) | Cooling towers. | Torres de resfriamento. |
| 722a58a3408-f5d3-434e-81c0-225137a1f6fa | Translation Approved (100%) | Meter replacement water use of all cooling towers serving the facility. | Meça o uso de água de substituição de todos as torres de resfriamento que servem a instalação. |
| 7238a971ea1-079c-46d0-9154-0aff291c8fff | Translation Approved (99%) | Domestic hot water. | Água quente para uso doméstico. |
| 7248a971ea1-079c-46d0-9154-0aff291c8fff | Translation Approved (CM) | Meter water use of at least 80% of the installed domestic hot water heating capacity (including both tanks and on-demand heaters). | Meça o uso de água de pelo menos 80% da capacidade de aquecimento de água para uso doméstico instalada (incluindo tanques e aquecedores por demanda). |
| 725fa9cab31-86d0-4c13-9796-bccbfdc35424 | Translation Approved (94%) | Reclaimed water. | Água recuperada. |
| 726fa9cab31-86d0-4c13-9796-bccbfdc35424 | Translation Approved (CM) | Meter reclaimed water, regardless of rate. | Meça a água recuperada, independentemente da taxa. |
| 727fa9cab31-86d0-4c13-9796-bccbfdc35424 | Translation Approved (CM) | A reclaimed water system with a makeup water connection must also be metered so that the true reclaimed water component can be determined. | Um sistema de água recuperada com conexão de água de reposição também deve ser medido, para que o verdadeiro componente de água recuperada possa ser determinado. |
| 728397ea8ae-413c-438c-943f-8aef2c2e22fe | Translation Approved (92%) | Other process water. | Água de outros processos. |
| 729397ea8ae-413c-438c-943f-8aef2c2e22fe | Translation Approved (0%) | Meter at least 80% of expected daily water consumption for process end uses, such as humidifiers, dishwashers, clothes washers, and pools. | Meça pelo menos 80% do consumo diário esperado de água para usos finais de processos, como umidificadores, máquinas de lavar louça, máquinas de lavar roupa e piscinas. |
| 730326c5f72-df96-47a9-badb-252a2af689df | Translation Approved (0%) | . | . |
| 73130c14b58-55eb-4b19-8393-cf865d8b89bf | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 73274454a42-c630-4c44-90d5-7619bc35d1b0 | Translation Approved (0%) | All meters, including whole-building meters, must be recorded at least weekly and used in a regular analysis of time trends. | Todos os medidores, incluindo os medidores para todo o edifício, devem ser registrados semanalmente, no mínimo, e usados em uma análise regular das tendências de tempo. |
| 73314014ae1-23bf-4411-9a90-c22be28c2eff | Translation Approved (0%) | Meters must be calibrated within the manufacturer’s recommended interval if the building owner, management organization, or tenant owns the meter. | Os medidores devem ser calibrados de acordo com o intervalo recomentado pelo fabricante se o proprietário, a organização administradora ou o locatário do edifício for dono do medidor. |
| 73414014ae1-23bf-4411-9a90-c22be28c2eff | Translation Approved (100%) | Meters owned by third parties (e.g., utilities or governments) are exempt. | Os medidores de terceiros (por exemplo, concessionárias de serviços públicos ou governos) estão isentos. |
| 73557437217-67eb-416d-9096-a63939add7f3 | Translation Approved (98%) | Commit to sharing with USGBC the resulting water usage data for a five-year period beginning on the date the project accepts LEED certification or typical occupancy, whichever comes first. | Comprometa-se em compartilhar com o USGBC os dados de uso de água resultantes por um período de cinco anos, começando na data em que o projeto aceitar a certificação LEED ou a ocupação típica, o que ocorrer primeiro. |
| 7367d02e5cb-4c3a-4b37-944d-a66b1e3bb8cc | Translation Approved (100%) | Energy and Atmosphere | Energia e Atmosfera (Energy and Atmosphere) |
| 737c99e627a-73c0-493f-b6e2-37f8f6b7dd1f | Translation Approved (79%) | EA Prerequisite: Energy Efficiency Best Management Practices | Pré-requisito EA: Melhores Práticas de Gestão de Eficiência Energética (EA Prerequisite: Energy Efficiency Best Management Practices) |
| 738ff446a47-edb6-43b2-8914-44a2dbef8f0c | Translation Approved (100%) | Required | Obrigatório |
| 739b6e327d8-4af2-40df-a586-fa570ace6c4c | Translation Approved (100%) | EB:O&M | EB:O&M |
| 7404e002627-8b30-435e-a865-13132196e681 | Translation Approved (100%) | This prerequisite applies to | Este pré-requisito se aplica a: |
| 741dd6a03dd-a83a-40bb-87c1-8c06ac30b7ea | Translation Approved (100%) | Existing Buildings: Operations & Maintenance | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) |
| 742d527ba6f-e246-4e23-a8d7-c11ccf80aa40 | Translation Approved (100%) | EB:O&M Schools | EB:O&M Escolas (Schools) |
| 7437b024fdf-75c6-46b1-9922-d5ebce009105 | Translation Approved (100%) | EB:O&M Retail | EB:O&M Varejo (Retail) |
| 7444b33f84c-1b61-4c97-9a7a-c6aa954b05c0 | Translation Approved (100%) | EB:O&M Data Centers | EB:O&M Centros de Dados (Data Centers) |
| 745f402a5ce-787a-461c-b207-668d0f0a2bac | Translation Approved (100%) | EB:O&M Hospitality | EB:O&M Hoteleiro (Hospitality) |
| 7467e144d8d-a00c-4375-b18a-2bc123d78ead | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) |
| 7474054aca5-8db6-4b25-ad5b-5156fcf66981 | Translation Approved (100%) | Intent | Objetivo |
| 7487e640df5-f694-429c-a6a6-ddec10e5c804 | Translation Approved (100%) | To promote continuity of information to ensure that energy-efficient operating strategies are maintained and provide a foundation for training and system analysis. | Promover a continuidade de informações para garantir que estratégias de operação eficazes energeticamente sejam mantidas e sirvam de base para treinamentos e análises de sistemas. |
| 749fbca00a3-8fff-4287-9b71-853e204a1fb7 | Translation Approved (100%) | Requirements | Requisitos |
| 75031129fad-9756-4f4d-a78a-dc83999943f9 | Translation Approved (83%) | EBOM, Schools, Retail, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 751337cfc02-ce34-4f07-a21e-607f9a676a4c | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 7526f5d11c0-e2fe-40ac-a226-7ef3739a4e89 | Translation Approved (0%) | Conduct an energy audit that meets both the requirements of the ASHRAE preliminary energy use analysis and an ASHRAE Level 1 walk-through assessment identified in the ASHRAE Procedures for Commercial Building Energy Audits or equivalent. | Conduza uma auditoria de energia que atenda aos requisitos da análise de uso de energia preliminar ASHRAE e uma avaliação de detecção de Nível 1 ASHRAE identificada em ASHRAE Procedures for Commercial Building Energy Audits ou equivalente. |
| 75391b6af80-2235-4364-b8be-5a13f28cd9cf | Translation Approved (100%) | Prepare and maintain a current facilities requirements and operations and maintenance plan that contains the information necessary to operate the building efficiently. | Prepare e mantenha um plano atualizado de requisitos, operações e manutenção de instalações que contenha as informações necessárias para operar o edifício com eficiência. |
| 75491b6af80-2235-4364-b8be-5a13f28cd9cf | Translation Approved (CM) | The plan must include the following: | O plano deve incluir o seguinte: |
| 755b02497a0-0b1c-42e6-92f3-6d656f3e1c26 | Translation Approved (0%) | a current sequence of operations for the building; | uma sequência atual de operações do edifício; |
| 7567443d094-814e-4882-a66b-842d6b2a0b43 | Translation Approved (100%) | the building occupancy schedule; | a programação de ocupação do edifício; |
| 7571ba08622-a3cc-483f-8846-9593b1509aaf | Translation Approved (CM) | equipment run-time schedules; | programações de tempo de funcionamento de equipamentos; |
| 75837f8db80-0ae0-4a1e-b4c2-fb2ab53b8599 | Translation Approved (CM) | setpoints for all HVAC equipment; | pontos de ajuste de todos os equipamentos de HVAC; |
| 759139242cd-3226-4b2b-b5d1-b5201f7bd856 | Translation Approved (0%) | setpoints for lighting levels throughout the building; | pontos de ajuste para níveis de iluminação em todo o edifício; |
| 760cf036f9c-17a3-41a6-bd6a-7e514c94ac21 | Translation Approved (100%) | minimum outside air requirements; | requisitos mínimos de ar externo; |
| 7618ae765af-8237-4b72-9048-f7b953638ba0 | Translation Approved (CM) | any changes in schedules or setpoints for different seasons, days of the week, and times of day; | quaisquer alterações em programações ou pontos de ajuste para diferentes estações, dias da semana e horas do dia; |
| 7622e300ef5-6ba9-4482-bb7f-a995c0907ea5 | Translation Approved (81%) | a systems narrative describing the mechanical and electrical systems and equipment in the building; and | um relato dos sistemas que descreva os sistemas e equipamentos mecânicos e elétricos no edifício; e |
| 7636e3c0b02-5c6a-41b6-94de-adeec3e5e0e5 | Translation Approved (94%) | a preventive maintenance plan for building equipment described in the systems narrative. | um plano de manutenção preventiva para os equipamentos do edifício descritos no relato dos sistemas. |
| 7643acf8b8f-15b2-4426-9ec0-c6f35004f336 | Translation Approved (100%) | Data Centers | Centros de Dados (Data Centers) |
| 76569e60498-62b6-4857-9d75-dd075a5b6591 | Translation Approved (CM) | Use the U.S. Department of Energy’s DC PRO Profiling Tool to perform a preliminary assessment of energy consumption in data center spaces for critical systems. | Use a ferramenta de perfil DC PRO Profiling Tool do Ministério de Energia dos EUA para realizar uma avaliação preliminar do consumo de energia nos espaços de data center para sistemas críticos. |
| 7663ae72d03-f0c8-42f9-9b8e-afda5fad40be | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 767277bcb58-9eb8-4593-9660-6bb1802510df | Translation Approved (100%) | None. | Nenhum. |
| 7689c1d4bb8-c991-4360-946f-b38cc605202f | Translation Approved (100%) | EA Prerequisite: Minimum Energy Performance | Pré-requisito EA: Desempenho Energético Mínimo (EA Prerequisite: Minimum Energy Performance) |
| 76994c0b7fd-937e-448b-a162-5c816bf260ad | Translation Approved (100%) | Required | Obrigatório |
| 770847ff911-ce7d-49a9-b2bf-a90cc2442d46 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 7719529d028-c1f1-4aa7-b187-1bec99c7ea44 | Translation Approved (100%) | This prerequisite applies to | Este pré-requisito se aplica a: |
| 772c81079ed-8384-4dd9-9ce4-411ca497932e | Translation Approved (100%) | Existing Buildings: Operations & Maintenance | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) |
| 7736a4a908e-5c1a-4ea6-8f10-8ae612578598 | Translation Approved (100%) | EB:O&M Schools | EB:O&M Escolas (Schools) |
| 7743e66253a-a1e6-41da-8ae8-9c3e874adb5c | Translation Approved (100%) | EB:O&M Retail | EB:O&M Varejo (Retail) |
| 775367b54cc-f11a-47b8-8556-64db939bf89d | Translation Approved (100%) | EB:O&M Data Centers | EB:O&M Centros de Dados (Data Centers) |
| 776248d6359-d64b-4ee6-8c4b-224e95993040 | Translation Approved (100%) | EB:O&M Hospitality | EB:O&M Hoteleiro (Hospitality) |
| 7774b1e0434-c835-4c77-a568-52b0459ffcf7 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) |
| 77825b4d92b-eb88-4a02-b962-aff57b6dd4f6 | Translation Approved (100%) | Intent | Objetivo |
| 779fac271d3-2e24-4163-ac27-2fe9cb988651 | Translation Approved (CM) | To reduce the environmental and economic harms associated with excessive energy use by establishing a minimum level of operating energy performance. | Reduzir os danos ambientais e econômicos associados ao uso excessivo de energia estabelecendo um nível mínimo de desempenho de energia em operação. |
| 78070b5387c-1131-4b19-88cd-a32849c89024 | Translation Approved (100%) | Requirements | Requisitos |
| 7815e1cc0e3-4e2c-4757-97f5-a5090429eb69 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 78277981559-fef2-49af-82c6-fcfe3a8bcd26 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 78396eb58b7-6bd2-4476-99b7-bf8dfb968371 | Translation Approved (CM) | Calibrate meters within the manufacturer’s recommended interval if the building owner, management organization, or tenant owns the meter. | Calibre os medidores de acordo com o intervalo recomentado pelo fabricante se o proprietário, a organização administradora ou o locatário do edifício forem donos do medidor. |
| 78496eb58b7-6bd2-4476-99b7-bf8dfb968371 | Translation Approved (100%) | Meters owned by third parties (e.g., utilities or governments) are exempt. | Os medidores de terceiros (por exemplo, concessionárias de serviços públicos ou governos) estão isentos. |
| 78581be7433-ecdf-49ef-a298-58217ee29219 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 7866d41b843-bc13-4abf-9ee0-5c2fccaa2086 | Translation Approved (CM) | Meter the building’s energy use for a full 12 months of continuous operation and achieve the levels of efficiency set forth in the options below. | Meça o uso de energia do edifício por um período de 12 meses de operação contínua e atingir os níveis de eficiência estabelecidos nas opções abaixo. |
| 7876d41b843-bc13-4abf-9ee0-5c2fccaa2086 | Translation Approved (100%) | Each building’s energy performance must be based on actual metered energy consumption for both the LEED project building(s) and all comparable buildings used for the benchmark. | O desempenho energético de cada edifício deve ser baseado no consumo energético medido atual para ambos os edifícios do projeto LEED e todos os edifícios comparáveis usados para referência. |
| 788cc72a5d0-5c9d-4699-953a-85ab59c01ca5 | Translation Approved (100%) | Case 1. | Caso 1. |
| 789cc72a5d0-5c9d-4699-953a-85ab59c01ca5 | Translation Approved (CM) | ENERGY STAR Rating | Classificação ENERGY STAR |
| 790cfebc9b8-689e-4243-8440-0e8dbad45c3e | Translation Approved (78%) | For buildings eligible to receive an energy performance rating using the Environmental Protection Agency (EPA) ENERGY STAR<812>®</812> Portfolio Manager tool, achieve an energy performance rating of at least 75. | Para edifícios qualificados a receber uma avaliação de desempenho energético usando a ferramenta ENERGY STAR Portfolio Manager da EPA (Agência de Proteção Ambiental), é obrigatório alcançar uma avaliação de desempenho energético de pelo menos 75. |
| 791cfebc9b8-689e-4243-8440-0e8dbad45c3e | Translation Approved (100%) | For projects outside the U.S., consult ASHRAE/ASHRAE/IESNA Standard 90.1–2010, Appendixes B and D, to determine the appropriate climate zone. | Para projetos fora dos EUA, consulte a Norma ASHRAE/ASHRAE/IESNA 90.1–2010, Apêndices B e D, para determinar a zona climática adequada. |
| 79248904339-1676-47e1-a5f7-e87d9bea3728 | Translation Approved (100%) | Case 2. | Caso 2. |
| 79348904339-1676-47e1-a5f7-e87d9bea3728 | Translation Approved (0%) | Projects Not Eligible for ENERGY STAR Rating | Projetos não elegíveis para a classificação ENERGY STAR |
| 7949e7070d4-f320-4917-93c0-568d79d5b946 | Translation Approved (0%) | Projects not eligible to use EPA’s rating system may compare their buildings’ energy performance with that of comparable buildings, using national averages or actual buildings, or with the previous performance of the project building. | Os projetos não qualificados para usar o sistema de avaliação da EPA podem comparar o desempenho energético de seus edifícios com os de edifícios comparáveis, usando as médias nacionais ou os edifícios reais, com o desempenho anterior do edifício do projeto. |
| 795ceb145f3-10ad-40d8-bdad-dbbb165e30aa | Translation Approved (100%) | Option 1. | Opção 1. |
| 796ceb145f3-10ad-40d8-bdad-dbbb165e30aa | Translation Approved (0%) | Benchmark against Typical Buildings | Referência em relação a edifícios típicos |
| 7977239f6a6-3a56-43f9-bf09-be58ecad472d | Translation Approved (98%) | Path 1. | Caminho 1. |
| 7987239f6a6-3a56-43f9-bf09-be58ecad472d | Translation Approved (0%) | National Average Data Available | Dados nacionais médios disponíveis |
| 79927a2b8f3-61bb-42b0-a47b-f8a9ae5b25ab | Translation Approved (0%) | Demonstrate energy efficiency performance that is 25% better than the median energy performance of similar buildings by benchmarking against the national source energy data provided in the Portfolio Manager tool. | Demonstre o desempenho de eficiência energética que é 25% melhor que o desempenho energético médio de edifícios semelhantes tendo como referência os dados nacionais de fontes energéticas fornecidos na ferramenta Portfolio Manager. |
| 8001e8e2684-0a42-4d25-85b9-db3383c44e27 | Translation Approved (100%) | Path 2. | Caminho 2. |
| 8011e8e2684-0a42-4d25-85b9-db3383c44e27 | Translation Approved (84%) | National Average Data Not Available | Dados nacionais médios não disponíveis |
| 8027350e8ad-0a79-4d74-9e2c-6f06db570896 | Translation Approved (0%) | If national average source energy data are unavailable for buildings of similar type, benchmark against the building site energy data of at least three similar buildings, normalized for climate, building use, and occupancy. | Se os dados nacionais da média de fontes energéticas estiverem indisponíveis para edifícios de tipo semelhante, tenha como referência os dados da energia do terreno do edifício de pelo menos três edifícios semelhantes, normalizado para clima, uso do edifício e ocupação. |
| 8037350e8ad-0a79-4d74-9e2c-6f06db570896 | Translation Approved (0%) | Demonstrate a 25% improvement. | Demonstre uma melhora de 25%. |
| 80418b06842-bf54-4130-ba09-b085e0dd5f49 | Translation Approved (100%) | OR | OU |
| 8055e842dad-e309-4957-a7dc-de3a27e5a452 | Translation Approved (100%) | Option 2. | Opção 2. |
| 8065e842dad-e309-4957-a7dc-de3a27e5a452 | Translation Approved (0%) | Benchmark against Historical Data | Referência em relação a dados históricos |
| 8078f25b1ad-f3c7-4d27-a714-dcb428ef6ff4 | Translation Approved (0%) | If national average source energy data are unavailable, compare the building’s site energy data for the previous 12 months with the data from three contiguous years of the previous five, normalized for climate, building use, and occupancy. | Se os dados nacionais da média de fontes energéticas estiverem indisponíveis, compare os dados de energia do terreno do edifício pelos 12 meses anteriores com os dados de três anos consecutivos dos cinco anos anteriores, normalizado para clima, uso do edifício e ocupação. |
| 8088f25b1ad-f3c7-4d27-a714-dcb428ef6ff4 | Translation Approved (100%) | Demonstrate a 25% improvement. | Demonstre uma melhora de 25%. |
| 809e2eaf381-e8f6-4123-9c79-cf7ba70889c8 | Translation Approved (100%) | EA Prerequisite: Building-Level Energy Metering | Pré-requisito EA: Medição de Energia do Edifício (EA Prerequisite: Building-Level Energy Metering) |
| 810032737c4-a83e-4c01-a98b-be93897490bb | Translation Approved (100%) | Required | Obrigatório |
| 811abdd2f6a-646e-4cc0-81bd-1136cfe0db24 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 81266a774a4-9e6d-4886-a4b0-da733a94a710 | Translation Approved (100%) | This prerequisite applies to | Este pré-requisito se aplica a: |
| 81306037ac2-816c-4c2f-b23e-2da39a44c3d8 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) |
| 814bc142013-0b54-4f94-ab31-3018397946ce | Translation Approved (100%) | EB:O&M Schools | EB:O&M Escolas (Schools) |
| 815d64eb406-c33a-4480-aa2b-394cb3b013d3 | Translation Approved (100%) | EB:O&M Retail | EB:O&M Varejo (Retail) |
| 81649ef50b3-66ef-4bb8-9683-10e6945b34ac | Translation Approved (100%) | EB:O&M Data Centers | EB:O&M Centros de Dados (Data Centers) |
| 817f30a2fe0-4191-4a26-83d9-31f64d877f85 | Translation Approved (100%) | EB:O&M Hospitality | EB:O&M Hoteleiro (Hospitality) |
| 818321e285e-cafb-417c-8d93-2e982334932b | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) |
| 819fda7cfd6-5818-494d-b962-d529509ac086 | Translation Approved (100%) | Intent | Objetivo |
| 8208bb220e3-af6b-4921-a71d-e7d72031c180 | Translation Approved (CM) | To support energy management and identify opportunities for additional energy savings by tracking building-level energy use. | Apoiar a gestão de energia e identificar oportunidades de economias adicionais de energia rastreando o uso de energia no nível do edifício. |
| 8215e69d006-4a3c-4eb6-bde3-c328738a7d28 | Translation Approved (100%) | Requirements | Requisitos |
| 822b293e1ff-b1cd-4f63-bc4d-d07eca8dece8 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 82394de81af-0829-4cf0-b2ff-7ef64df1d004 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 824c6cfccb4-c0c1-4e7e-84e3-88a7482e5a16 | Translation Approved (98%) | Install new or use existing building-level energy meters or submeters that can be aggregated to provide building-level data representing total building energy consumption (electricity, natural gas, chilled water, steam, fuel oil, propane, etc). | Instale medidores de energia novos ou use os existentes no nível do edifício ou submedidores que possam ser agregados para fornecer dados do nível do edifício que representem o consumo total de energia do edifício (eletricidade, gás natural, água refrigerada, vapor, óleo combustível, propano, etc.). |
| 825c6cfccb4-c0c1-4e7e-84e3-88a7482e5a16 | Translation Approved (100%) | Utility-owned meters capable of aggregating building-level resource use are acceptable. | Medidores de concessionárias de serviços públicos capazes de agregar o uso de recursos no nível do edifício são aceitáveis. |
| 8267043af72-4267-40df-9336-0dba1fdc5b66 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 8272ff61c41-be0c-4b56-a9c9-49bb0772b6ea | Translation Approved (76%) | Compile meter data into monthly and annual summaries; meter readings can be manual or automated. | Compile os dados do medidor em resumos mensais e anuais; as leituras do medidor podem ser manuais ou automáticas. |
| 828209130e0-408a-461c-8b6a-54d928dbf055 | Translation Approved (100%) | Commit to sharing with USGBC the resulting energy consumption data and electrical demand data (if metered) for a five-year period beginning on the date the project accepts LEED certification or typical occupancy, whichever comes first. | Comprometa-se em compartilhar com o USGBC os dados de consumo de energia resultantes e os dados da demanda de energia (se medida) por um período de cinco anos, começando na data em que o projeto aceita a certificação LEED ou ocupação típica, o que ocorrer primeiro. |
| 829209130e0-408a-461c-8b6a-54d928dbf055 | Translation Approved (100%) | At a minimum, energy consumption must be tracked at one-month intervals. | No mínimo, o consumo de energia deve ser rastreado em intervalos de um mês. |
| 8306d9a333c-b3bb-47d0-b9c3-1d1579fca7c1 | Translation Approved (100%) | This commitment must carry forward for five years or until the building changes ownership or lessee. | Esse compromisso deve ser mantido por cinco anos ou até que o edifício mude de proprietário ou arrendatário. |
| 831e6562a35-e691-423c-9716-961e5e0fdba1 | Translation Approved (100%) | EA Prerequisite: Fundamental Refrigerant Management | Pré-requisito EA: Gerenciamento Fundamental de Gases Refrigerantes (EA Prerequisite: Fundamental Refrigerant Management) |
| 8320894f782-fa9a-4d23-93a9-a5292a211989 | Translation Approved (100%) | Required | Obrigatório |
| 83350fe1b81-9e1f-432c-9295-fdec894c8d26 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 834931d0e6a-8354-4ab5-b04c-c927653fe5c9 | Translation Approved (100%) | This prerequisite applies to | Este pré-requisito se aplica a: |
| 8353c52e991-2adb-4d67-85b2-3ab6ca4e0c6e | Translation Approved (100%) | Existing Buildings: Operations & Maintenance | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) |
| 8368ce21ed6-9205-4e7a-978d-552d0b778dac | Translation Approved (100%) | EB:O&M Schools | EB:O&M Escolas (Schools) |
| 837efda33a2-3d94-47c5-bf40-a21c9a32b486 | Translation Approved (100%) | EB:O&M Retail | EB:O&M Varejo (Retail) |
| 8380578a9c7-e3ff-4346-a703-cd2c6e04fed8 | Translation Approved (100%) | EB:O&M Data Centers | EB:O&M Centros de Dados (Data Centers) |
| 8399993756c-0c53-4c1a-a1fb-5fc685fdc9e6 | Translation Approved (100%) | EB:O&M Hospitality | EB:O&M Hoteleiro (Hospitality) |
| 8409ff8beaf-be09-4bc0-a898-f33859a72fce | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) |
| 8410ef66292-95af-4f9b-aff6-589fbcf5038c | Translation Approved (100%) | Intent | Objetivo |
| 842937b2936-d8ab-4fe3-8ae0-ecd0edc48cc7 | Translation Approved (CM) | To reduce stratospheric ozone depletion. | Reduzir o esgotamento do ozônio estratosférico. |
| 843aa7bc41f-639a-48c3-8dcc-dc7688dc56bf | Translation Approved (100%) | Requirements | Requisitos |
| 8448faccdc2-6749-42ee-b87b-597cdbf894c0 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 8453474902a-b2c9-4da6-9505-bd1ae4fe4420 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 8460d01c493-b84a-4ef0-b5dd-9c7dac2dac5f | Translation Approved (88%) | Do not use chlorofluorocarbon (CFC)-based refrigerants in heating, ventilating, air-conditioning, and refrigeration (HVAC&R) systems unless a third-party audit shows that system replacement or conversion is not economically feasible or unless a phase-out plan for CFC-based refrigerants is in place. | Não use refrigerantes à base de clorofluorcarbono (CFC) nos sistemas de aquecimento, ventilação, ar-condicionado e refrigeração (HVAC&R), a menos que uma auditoria realizada por terceiros mostre que a substituição ou conversão desses sistemas não seja economicamente viável ou que há um plano de substituição para refrigerantes à base de CFC em execução. |
| 8470d01c493-b84a-4ef0-b5dd-9c7dac2dac5f | Translation Approved (0%) | Phase-out plans should be scheduled for completion within 10 years. | Os planos de substituição devem ser programados para serem concluídos em 10 anos. |
| 8480d01c493-b84a-4ef0-b5dd-9c7dac2dac5f | Translation Approved (0%) | The replacement or conversion of HVAC&R equipment is considered not economically feasible if the simple payback of the replacement or conversion is greater than 10 years. | A substituição ou conversão dos equipamentos de HVAC&R não são consideradas economicamente viáveis se o retorno do investimento simples da conversão é maior que 10 anos. |
| 8490d01c493-b84a-4ef0-b5dd-9c7dac2dac5f | Translation Approved (0%) | Perform the following economic analysis: | Execute a seguinte análise econômica: |
| 850ae4015cc-6a2d-492d-a402-e48088f8f916 | Translation Approved (96%) | Simple payback = | Retorno do investimento simples = |
| 851664ee019-2f9f-46cd-a80a-d887837cb1ae | Translation Approved (0%) | Cost of replacement or conversion | Custo da substituição ou conversão |
| 85263aec62b-01e7-4ca4-92f9-33bd18538720 | Translation Approved (0%) | > 10 | > 10 |
| 8535bc08c85-f55b-4952-9258-2996c42d2330 | Translation Approved (0%) | Resulting annual energy cost difference | Diferença resultado do custo de energia anual |
| 854e38d6f04-8270-482c-afc2-0bf10325e8da | Translation Approved (100%) | + | + |
| 8557927c781-8eac-497d-afd1-23f2abdb8e6a | Translation Approved (0%) | Resulting annual maintenance and refrigerant cost difference | Diferença resultante de custo de refrigerante e manutenção anual |
| 856a060231e-2e1b-4d83-9792-9f810631ef34 | Translation Approved (0%) | If CFC-based refrigerants are maintained in the building, reduce annual leakage to 5% or less using the procedures in the Clean Air Act, Title VI, Rule 608, governing refrigerant management and reporting (or a local equivalent for projects outside the U.S.), and reduce the total leakage over the remaining life of the unit to less than 30% of its refrigerant charge. | Se refrigerantes à base de CFC forem mantidos no edifício, reduzir o vazamento anual para 5% ou menos usando os procedimento no Clean Air Act da EPA, Título VI, Regra 608, para relatórios e gestão e refrigerantes (ou um equivalente local para projetos fora dos EUA) e reduzir o vazamento total durante a vida restante da unidade para menos de 30% se sua carga de refrigerante. |
| 85767f0c61c-a9de-4302-811d-d07c69dcde4c | Translation Approved (0%) | Small HVAC&R units (defined as containing less than 0.5 pound [225 grams] of refrigerant), standard refrigerators, small water coolers, and any other cooling equipment that contains less than 0.5 pound (225 grams) of refrigerant are exempt. | Unidades pequenas de HVAC&R (definidas como contendo menos de 0,5 libra [225 gramas] de refrigerante), refrigeradores padrão, pequenos refrigeradores de água e qualquer outros equipamento de resfriamento que contenha menos de 0,5 libra (225 gramas) de refrigerante são isentos. |
| 858e67b5300-b19a-49a6-b017-5c4305260755 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 8597c184500-7da6-4add-93d5-3a28e59439ef | Translation Approved (100%) | None. | Nenhum. |
| 860627a0ee7-cbfc-468e-8341-8d3c16c33cd5 | Translation Approved (0%) | EA Credit: Existing Building Commissioning—Analysis | Crédito EA: Comissionamento de Edifício Existente - Análise (EA Credit: Existing Building Commissioning—Analysis) |
| 861d35f8157-60a2-4f29-8d66-d72740b5aaa1 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 862dc5ba13a-145a-445e-89aa-274c38203742 | Translation Approved (100%) | 2 points | 2 pontos |
| 86327848afa-da07-4f6d-8cc4-53195c37aafa | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 86425dfea0e-34e5-426f-9440-1bd838ff0cea | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (2 point) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (2 ponto) |
| 86556e10e7d-90e7-47d4-95e6-53dfae4e43dc | Translation Approved (100%) | EB:O&M Schools (2 point) | EB:O&M Escolas (Schools) (2 ponto) |
| 86635614995-8d4f-4251-be86-c5d20b9e9396 | Translation Approved (100%) | EB:O&M Retail (2 point) | EB:O&M Varejo (Retail) (2 ponto) |
| 867c31a6464-bbc3-405b-a0a7-4793ca8c21d4 | Translation Approved (100%) | EB:O&M Data Centers (2 point) | EB:O&M Centros de Dados (Data Centers) (2 ponto) |
| 8688ba666b2-fe79-46c9-9d74-35e06ac0dc1e | Translation Approved (100%) | EB:O&M Hospitality (2 point) | EB:O&M Hoteleiro (Hospitality) (2 ponto) |
| 8693088d8c1-562d-4bc4-9bd5-1442f1c48a99 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (2 point) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (2 ponto) |
| 8706af945cc-2ff1-40b0-a3f3-96bf92193cf5 | Translation Approved (100%) | Intent | Objetivo |
| 8718bd02986-c356-4bb3-abac-c9c4163bfb09 | Translation Approved (0%) | To use the existing building commissioning process to improve building operations, energy, and resource efficiency. | Usar o processo de comissionamento do edifício existente para melhorar as operações do edifício, a energia e a eficiência de recursos. |
| 872ef8ba3c9-db33-41b3-b201-a3716d40291b | Translation Approved (100%) | Requirements | Requisitos |
| 8730b779621-7ebe-41f2-9a0b-657176b9da4d | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 874e0aefb9a-7db4-4a45-9f11-81a3d090b54e | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 8750430f688-7d16-4d19-b969-2f96bbe2316b | Translation Approved (0%) | Evaluate the current performance of the project building against the performance specifications in the current facilities requirements and the operations and maintenance plan. | Avalie o desempenho atual do edifício do projeto em relação às especificações de desempenho nos requisitos atuais da instalação e no plano de manutenção e operações. |
| 8766fd2ff3c-03ce-40e7-b4bb-198a8d18a4e4 | Translation Approved (0%) | Identify the systems and components in the facility to be investigated and analyzed as part of the existing building commissioning or energy auditing process. | Identifique os sistemas e componentes na instalação a serem investigados e analisados como parte do processo de auditoria de energia ou comissionamento de edifício existente. |
| 8776fd2ff3c-03ce-40e7-b4bb-198a8d18a4e4 | Translation Approved (0%) | Provide a breakdown of estimated resource use for each of these systems. | Forneça uma discriminação do uso estimado de recursos para um desses sistemas. |
| 878fcc055c8-c9fd-4889-a37e-92c90db04452 | Translation Approved (100%) | Option 1. | Opção 1. |
| 879fcc055c8-c9fd-4889-a37e-92c90db04452 | Translation Approved (100%) | Existing Building Commissioning | Comissionamento de edificações existentes (Existing Building Commissioning) |
| 8807e261f5f-7f63-4bdc-88c1-f3bf7634f89e | Translation Approved (0%) | Develop an existing building commissioning plan to effectively inventory and evaluate specific opportunities within the systems being analyzed. | Desenvolva um plano de comissionamento do edifício existente para organizar um inventário e avaliar de forma eficiente as oportunidades específicas dentro dos sistemas sendo analisados. |
| 8817e261f5f-7f63-4bdc-88c1-f3bf7634f89e | Translation Approved (89%) | The commissioning plan must include the following: | O plano de comissionamento deve incluir o seguinte: |
| 88236e1d02e-91ba-46b6-a2f9-b8fb09f8384a | Translation Approved (0%) | updated current facilities requirements; | requisitos atualizados das instalações atuais; |
| 8830ff0b8da-3cc0-431d-a0e3-457c1b845312 | Translation Approved (0%) | the commissioning team members and their roles and responsibilities during the commissioning process; | os membros da equipe de comissionamento e suas funções e responsabilidades durante o processo de comissionamento; |
| 8848672eada-889a-4d56-8ae7-44adbaf7bdab | Translation Approved (0%) | a description of the approach for identifying and analyzing facility improvement opportunities; | uma descrição da abordagem para identificar e analisar as oportunidades de melhoria da instalação; |
| 885c5d51e7c-804d-4a30-87d1-293017b0d87a | Translation Approved (0%) | the process for reviewing and prioritizing identified opportunities with the owner and developing an implementation plan; | o processo para revisar e priorizar as oportunidades identificadas com o proprietário e desenvolver um plano de implementação; |
| 886e143eb23-8249-4bbf-ae2f-f4eacd48774d | Translation Approved (0%) | the format and content of the eventual deliverables from the commissioning process; and | o formato e conteúdo dos eventuais resultados do processo de comissionamento; e |
| 8879e31c5ec-77a7-4aae-b3d0-3e68e55edcd9 | Translation Approved (0%) | the proposed schedule. | a programação proposta. |
| 8886fb3cc4a-4fdf-4207-9724-cf0e057535d2 | Translation Approved (100%) | Option 2. | Opção 2. |
| 8896fb3cc4a-4fdf-4207-9724-cf0e057535d2 | Translation Approved (96%) | Energy Audit | Auditoria de energia |
| 89052ab1ad2-2a0a-4ec4-b4e9-a41b038a7c5a | Translation Approved (0%) | Develop an energy audit plan following the requirements of ASHRAE Level 2, Energy Survey and Analysis, to evaluate efficiency opportunities. | Desenvolva plano de auditoria de energia seguindo os requisitos norma ASHRAE Nível II Energy Survey and Analysis (Pesquisa e Análise de Energia), para avaliar as oportunidades de eficiência. |
| 89152ab1ad2-2a0a-4ec4-b4e9-a41b038a7c5a | Translation Approved (91%) | The audit plan must include the following: | O plano de auditoria deve incluir o seguinte: |
| 89233a8179f-f927-4217-ae64-985d889cd487 | Translation Approved (90%) | the audit team members and their roles and responsibilities during the audit process; | os membros da equipe de auditoria e suas funções e responsabilidades durante o processo de auditoria; |
| 893b061b732-44bb-47f9-99cc-4fd35d13c132 | Translation Approved (100%) | a description of the approach for identifying and analyzing facility improvement opportunities; | uma descrição da abordagem para identificar e analisar as oportunidades de melhoria da instalação; |
| 894cef44834-ada3-4559-805a-608b5bdf43a6 | Translation Approved (100%) | the process for reviewing and prioritizing identified opportunities with the owner and developing an implementation plan; | o processo para revisar e priorizar as oportunidades identificadas com o proprietário e desenvolver um plano de implementação; |
| 895009a8d3e-da06-4e9c-b876-88a78a5ea452 | Translation Approved (95%) | the format and content of the eventual deliverables from the audit process; and | o formato e conteúdo dos eventuais resultados do processo de auditoria; e |
| 896872a94f3-9717-4656-8dd3-25002ad56bb0 | Translation Approved (100%) | the proposed schedule. | a programação proposta. |
| 897fe9e2178-f782-4569-a2ab-cc4e4537ca9d | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 898909e55d2-5f52-4299-829f-876c97ab11ce | Translation Approved (0%) | Apply the requirements below to all direct energy-consuming or energy-producing systems, including lighting, process loads, HVAC&R, domestic water heating, and renewable energy. | Aplique os requisitos abaixo para todos os sistemas consumidores de energia e produtores de energia, incluindo iluminação, cargas de processo, HVAC&R, aquecimento de água doméstica e energia renovável. |
| 899d7300915-4c10-4b57-b511-d298fd495d0c | Translation Approved (79%) | Update the systems and components to be addressed as part of the existing building commissioning or energy auditing process. | Atualize os sistemas e componentes a serem abordados como parte do processo de auditoria de energia ou comissionamento de edifício existente. |
| 900d7300915-4c10-4b57-b511-d298fd495d0c | Translation Approved (100%) | Provide a breakdown of estimated resource use for each of these systems. | Forneça uma discriminação do uso estimado de recursos para um desses sistemas. |
| 90170b74f0f-46f6-4870-834b-3206f9153b7d | Translation Approved (100%) | Option 1. | Opção 1. |
| 90270b74f0f-46f6-4870-834b-3206f9153b7d | Translation Approved (100%) | Existing Building Commissioning | Comissionamento de edificações existentes (Existing Building Commissioning) |
| 90374e993f0-cae1-40c2-8045-ef1c99a8a0ab | Translation Approved (0%) | Update and execute the existing building commissioning plan. | Atualize e execute o plano de comissionamento de edifício existente. |
| 904f4f28b1d-b580-4a3f-8ba9-e546590fe0e9 | Translation Approved (100%) | Option 2. | Opção 2. |
| 905f4f28b1d-b580-4a3f-8ba9-e546590fe0e9 | Translation Approved (100%) | Energy Audit | Auditoria de energia |
| 9065af325bc-0fa3-45b7-8e15-9420085ed8d6 | Translation Approved (0%) | Update and execute the energy audit plan following the requirements of ASHRAE Level 2, Energy Survey and Analysis. | Atualize e execute o plano de auditoria de energia seguindo os requisitos da norma ASHRAE Nível II Energy Survey and Analysis (Pesquisa e Análise de Energia). |
| 907c358c8e2-131d-4b00-a97f-4729b5b0787f | Translation Approved (0%) | For each opportunity, describe the potential improvement, estimated implementation costs, and anticipated savings. | Para cada oportunidade, descreva a melhoria potencial, os custos de implementação estimados e as economias previstas. |
| 9087f674541-d819-420e-9a61-6a78da8251ee | Translation Approved (100%) | Data Centers only | Apenas Centros de Dados (Data Centers) |
| 909d81bb3e2-489f-42d8-b583-4c38595fc839 | Translation Approved (0%) | In addition to the requirements above, data centers must use the Department of Energy’s Save Energy Now Program’s on-line DC Pro Energy Assessment Tools for data center critical systems. | Além dos requisitos acima, os Centros de Dados devem usar as Ferramentas de Avaliação de Energia DC Pro on-line do Programa "Save Energy Now" (Economize energia agora) do Ministério de Energia dos EUA para sistemas críticos de Centros de Dados. |
| 9108941f85d-8a1f-4f64-8934-edd145e72807 | Translation Approved (87%) | EA Credit: Existing Building Commissioning—Implementation | Crédito EA: Comissionamento de Edifício Existente - Implementação (EA Credit: Existing Building Commissioning—Implementation) |
| 91118a213b5-d0c9-41e4-9989-b9bf7fa9527d | Translation Approved (100%) | EB:O&M | EB:O&M |
| 912f0637585-e9e3-4664-b3a7-b1f98df35efa | Translation Approved (100%) | 2 points | 2 pontos |
| 91371263746-56fd-4572-b8b9-340638df6a65 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 91418cd7257-5523-431b-b39f-9267b8d6b0ec | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (2 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (2 pontos) |
| 915274a5266-f2be-41d0-8f5f-b982acf8b853 | Translation Approved (100%) | EB:O&M Schools (2 points) | EB:O&M Escolas (Schools) (3 pontos) |
| 9162e9c944c-47ab-4731-be0b-73d158e46389 | Translation Approved (100%) | EB:O&M Retail (2 points) | EB:O&M Varejo (Retail) (2 pontos) |
| 917262bd464-aa8f-4b20-b5bf-e2968fb6a131 | Translation Approved (100%) | EB:O&M Data Centers (2 points) | EB:O&M Centros de Dados (Data Centers) (2 pontos) |
| 918fecbdee1-e857-44d5-a809-63895140608a | Translation Approved (100%) | EB:O&M Hospitality (2 points) | EB:O&M Hoteleiro (Hospitality) (2 pontos) |
| 9199b80c0a1-cad1-4787-806e-62e2c28209dd | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (2 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (2 pontos) |
| 920b83c2036-e845-45f7-b1c5-7d95a74884cd | Translation Approved (100%) | Intent | Objetivo |
| 92153f1885b-6d49-4473-a56d-532b73cff22b | Translation Approved (99%) | To use the existing building commissioning process to improve building operations, energy and resource efficiency. | Usar o processo de comissionamento do edifício existente para melhorar as operações do edifício, a energia e a eficiência de recursos. |
| 9228a29a092-cbbc-46ff-a44f-3b965798eecf | Translation Approved (100%) | Requirements | Requisitos |
| 9233dc26c58-a0f9-4847-bd8d-1e8b48ac20a4 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 9241baeb4f7-e8de-40c4-98f0-7de991c2595e | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 925b6b5d8c1-2531-4eae-84fe-3a17c40fde5c | Translation Approved (0%) | Meet the requirements of EA Credit Existing Building Commissioning—Analysis. | Cumpra os requisitos do Crédito EA: Comissionamento de Edifício Existente - Análise (EA Credit: Existing Building Commissioning—Analysis). |
| 926e492ecc1-d44f-43f8-a5e1-482536047843 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 92798c0a998-7715-4a42-a81b-461d65c9a9b2 | Translation Approved (100%) | Apply the requirements below to all direct energy-consuming or energy-producing systems, including lighting, process loads, HVAC&R, domestic water heating, and renewable energy. | Aplique os requisitos abaixo para todos os sistemas consumidores de energia e produtores de energia, incluindo iluminação, cargas de processo, HVAC&R, aquecimento de água doméstica e energia renovável. |
| 92866d9003d-e15f-4323-9f0f-d200076562a0 | Translation Approved (0%) | Implement no- or low-cost operational improvements and develop a five-year plan for equipment replacement and major modifications or upgrades based on the analysis phase. | Implemente melhorias sem custo ou de baixo custo e desenvolver um plano de cinco anos para substituição de equipamento e grandes modificações ou atualizações com base na fase de análise. |
| 92945335ff1-c157-457a-91ea-fd5e99fade8b | Translation Approved (0%) | Confirm training of building operations staff so that they can efficiently operate all new or substantially altered building equipment or systems. | Confirme o treinamento dos funcionários de operações do edifícios para que possam operar de forma eficiente todos os equipamentos ou sistemas do edifício novos ou substancialmente alterados. |
| 9303acd95ad-1772-488f-8677-dd1bbbdddac2 | Translation Approved (0%) | Develop a tracking and verification program for all projects implemented as part of the existing building commissioning process. | Desenvolva um programa de monitoramento e verificação para todos os projetos implementados como parte do processo de comissionamento de edifício existente. |
| 9313acd95ad-1772-488f-8677-dd1bbbdddac2 | Translation Approved (0%) | Note factors such as effectiveness, financial costs and benefits, and observed or estimated environmental and human health and comfort benefits. | Observe fatores como eficácia, custos e benefícios financeiros e benefícios ambientais, de conforto e de saúde humana observados e estimados. |
| 932bba3436c-a1b0-4011-a31a-86a7f6438a19 | Translation Approved (0%) | Update the operations and maintenance plan and the current facilities requirements to incorporate the newly implemented improvements. | Atualize o plano de operações e manutenção e os requisitos das instalações atuais para incorporar as melhorias recém-implementadas. |
| 93311f42287-ce50-4f0d-9b56-066d2937c6fb | Translation Approved (90%) | EA Credit: Ongoing Commissioning | Crédito EA: Comissionamento Contínuo (EA Credit: Ongoing Commissioning) |
| 9340df1c0ae-8a2e-4fab-8cff-bcf54464745d | Translation Approved (100%) | EB:O&M | EB:O&M |
| 935395f8b56-653b-4bf5-94b2-2694cdc889fc | Translation Approved (100%) | 3 points | 3 pontos |
| 936f5815e14-9743-424d-8930-f1ff3074fe15 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 937e00f608e-4e22-4f96-ad4b-cb5efc54aa1f | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (3 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (3 pontos) |
| 938d9779a85-bd15-4832-82b8-1683a9e16486 | Translation Approved (99%) | EB:O&M Schools (3 points) | EB:O&M Escolas (Schools) (3 pontos) |
| 9393a7daaa3-7a36-4789-94b7-8f2fed10bd2c | Translation Approved (100%) | EB:O&M Retail (3 points) | EB:O&M Varejo (Retail) (3 pontos) |
| 940bb2b4c75-f109-45b1-ba5a-c8330b36d498 | Translation Approved (100%) | EB:O&M Data Centers (3 points) | EB:O&M Centros de Dados (Data Centers) (3 pontos) |
| 941743b3b95-683f-4448-81bf-8630ce6c2605 | Translation Approved (100%) | EB:O&M Hospitality (3 points) | EB:O&M Hoteleiro (Hospitality) (3 pontos) |
| 9421055ce26-192d-49ce-9793-3310135c823a | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (3 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (3 pontos) |
| 943b715756c-cf4e-415b-8426-c6d7b4694a87 | Translation Approved (100%) | Intent | Objetivo |
| 944411ae2b1-b514-4796-8942-00c203f5a08d | Translation Approved (100%) | To use the existing building commissioning process to improve building operations, energy, and resource efficiency. | Usar o processo de comissionamento do edifício existente para melhorar as operações do edifício, a energia e a eficiência de recursos. |
| 94569ccfecc-4186-4d45-87c0-4bfa1cf87c09 | Translation Approved (100%) | Requirements | Requisitos |
| 94625a91fdd-f950-4ca1-b3de-bdaf9ec712d7 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 94725b670db-fbbf-4d45-9fb6-c668e777a983 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 948a478118b-349e-47fc-a2fa-25d43c179a64 | Translation Approved (0%) | Meet the requirements of EA Credit Existing Building Commissioning–Analysis and EA Credit Existing Building Commissioning–Implementation. | Cumpra os requisitos do Crédito EA: Comissionamento de Edifício Existente - Análise (EA Credit: Existing Building Commissioning—Analysis) e do Crédito EA: Comissionamento de Edifício Existente - Implementação (EA Credit: Existing Building Commissioning—Implementation). |
| 9497c72e81e-74c6-46f8-8239-343154c1c20d | Translation Approved (0%) | Establish an ongoing commissioning process that includes planning, point monitoring, system testing, performance verification, corrective action response, ongoing measurement, and documentation to proactively address operating problems in the systems being commissioned. | Estabeleça um processo de comissionamento contínua que inclua planejamento, monitoramento de ponto, teste de sistema, verificação de desempenho, resposta de ação corretiva, medição contínua e documentação para abordar proativamente os problemas operacionais nos sistemas sendo comissionados. |
| 950a9fde6c2-8670-45f7-8a43-612398379710 | Translation Approved (0%) | Develop an on-going commissioning plan that defines the following: | Desenvolva um plano de comissionamento contínuo que defina o seguinte: |
| 95118b15958-222d-4894-b8b6-13f2697f7e75 | Translation Approved (100%) | roles and responsibilities; | funções e responsabilidades; |
| 952ee9abee9-4932-45bc-b1d8-de45acb5ddba | Translation Approved (CM) | measurement requirements (meters, points, metering systems, data access); | requisitos de medição (medidores, pontos, sistemas de medição, acesso a dados); |
| 9539b9c7de9-2ddb-4789-8744-1e31987bc59e | Translation Approved (CM) | the points to be tracked, with frequency and duration for trend monitoring; | os pontos a serem rastreados, com frequência e duração do monitoramento de tendências; |
| 954d69cfc4c-549e-41ca-9df5-455be105a13a | Translation Approved (0%) | the limits of acceptable values for tracked points and metered values; | os limites dos valores aceitáveis para pontos monitorados e valores medidos; |
| 9558979988c-b5e6-4aca-8b65-7d9e6d1eabdb | Translation Approved (0%) | the review process that will be used to evaluate performance; | o processo de revisão que será usado para avaliar o desempenho; |
| 956d82016b3-c3bb-4c6e-aadb-8e2036e87ded | Translation Approved (0%) | an action plan for identifying and correcting operational errors and deficiencies | um plano de ação para identificar e corrigir erros e deficiências operacionais |
| 957f7d06033-1547-4476-bd91-9e80a944366e | Translation Approved (100%) | planning for repairs needed to maintain performance; | planejamento para reparos necessários a fim de manter o desempenho; |
| 9580da1379e-2521-46ee-a4c8-845adece4ac2 | Translation Approved (0%) | the frequency of analyses in the first year (at least quarterly); and | a frequência de análises no primeiro ano (pelo menos trimestralmente); e |
| 959c2a0a63b-89c0-48bc-b1fa-110c9312f48c | Translation Approved (0%) | the subsequent analysis cycle (at least every 24 months). | o ciclo de análise subsequente (pelo menos a cada 24 meses). |
| 96065750200-7c85-41db-b065-148a7caa56da | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 961733d2c3e-3e17-4859-a963-b9140c5ea1b2 | Translation Approved (100%) | Apply the requirements below to all direct energy-consuming or energy-producing systems, including lighting, process loads, HVAC&R, domestic water heating, and renewable energy. | Aplique os requisitos abaixo para todos os sistemas consumidores de energia e produtores de energia, incluindo iluminação, cargas de processo, HVAC&R, aquecimento de água doméstica e energia renovável. |
| 9628d0bff3b-9e9e-4bf9-be35-fed8153c0363 | Translation Approved (100%) | Update the systems manual with any modifications or new settings, and give the reason for any modifications from the original design. | Atualize o manual dos sistemas com quaisquer modificações ou novos ajustes e forneça o motivo de todas as modificações em relação ao projeto original. |
| 9638d0bff3b-9e9e-4bf9-be35-fed8153c0363 | Translation Approved (0%) | Define methods for improving operations and maintenance. | Defina os métodos para melhorar as operações e a manutenção. |
| 964373473f6-c4d2-4b8e-861b-46567e648fe6 | Translation Approved (0%) | Include quarterly reports during the first year of implementation and annual reports on the performance of building systems. | Inclua relatórios trimestrais durante o primeiro ano da implementação e relatórios anuais do desempenho dos sistemas do edifício. |
| 965eba44fa6-c159-461e-9396-8d488d2b2c3f | Translation Approved (0%) | Continue to update the facility’s operating and maintenance plan and current facilities requirements to reflect actual conditions and issue annual revisions of these documents. | Continue a atualizar o plano de operação e manutenção da instalação e os requisitos das instalações atuais para refletir as condições reais e emita revisões anuais desses documentos. |
| 9665bbc1c4e-95c9-400b-b6d4-609e78d2ea47 | Translation Approved (0%) | Only activities associated with ongoing commissioning completed within two years of the LEED application may be included to show progress. | Somente atividades associadas com o comissionamento contínuo concluído em dois anos da inscrição no LEED podem ser incluídos para mostrar o progresso. |
| 96747c00f3a-c23e-456b-ac69-6350617f8eaf | Translation Approved (100%) | EA Credit: Optimize Energy Performance | Crédito EA: Otimização do Desempenho Energético (EA Credit: Optimize Energy Performance) |
| 968f6f86340-2fb9-4b9c-b383-f34cfefdafe1 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 9696ae30d37-8626-4faa-9068-17daf84f50e9 | Translation Approved (89%) | 1–20 points | 1 a 20 pontos |
| 97010e90c51-e654-4d3e-a746-d74762a8547b | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 9717673167f-e5a7-44c7-bd1d-44a04ca50a6f | Translation Approved (96%) | Existing Buildings: Operations & Maintenance (1–20 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 a 20 pontos) |
| 9728084d2c5-46ca-4c2a-a311-cf00a078e713 | Translation Approved (95%) | EB:O&M Schools (1–20 points) | EB:O&M Escolas (Schools) (1 a 20 pontos) |
| 9731836793a-9e96-49d5-85c1-827c38e2b010 | Translation Approved (95%) | EB:O&M Retail (1–20 points) | EB:O&M Varejo (Retail) (1 a 20 pontos) |
| 9744e190d00-3c84-4d87-b5df-5793ca604e3b | Translation Approved (96%) | EB:O&M Data Centers (1–20 points) | EB:O&M Centros de Dados (Data Centers) (1 a 20 pontos) |
| 975188a0c4b-42c8-41bc-a9d5-32fbf035fbf5 | Translation Approved (95%) | EB:O&M Hospitality (1–20 points) | EB:O&M Hoteleiro (Hospitality) (1 a 20 pontos) |
| 976f4b86d7e-599b-4839-b8ae-3ff1258f9e93 | Translation Approved (97%) | EB:O&M Warehouses & Distribution Centers (1–20 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 a 20 pontos) |
| 977bfdf52ca-e6ce-4e9b-afd4-a3ae341ce3ce | Translation Approved (100%) | Intent | Objetivo |
| 978f9ef3859-a6e2-4cb9-b079-dcc2c3a3094c | Translation Approved (86%) | To reduce environmental and economic harms associated with excessive energy use by achieving higher levels of operating energy performance | Reduzir os danos ambientais e econômicos associados ao uso excessivo de energia alcançando níveis mais altos de desempenho de energia em operação. |
| 9797cb8e41b-2123-42ee-a2b2-b4b9ffe4cdc3 | Translation Approved (100%) | Requirements | Requisitos |
| 980f63f6ed6-b553-492d-9070-9ce137eb563f | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 9817e7e1c3a-e663-4b95-b342-337b54b6412c | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 982e453ee67-e15c-4b9a-9b39-ddaf1715ada5 | Translation Approved (100%) | None. | Nenhum. |
| 98346cc3182-0f2c-4c88-83f9-4a93e0514b06 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 984d629b8bf-c24d-4484-be49-cbcb985ea4ba | Translation Approved (0%) | Demonstrate increased energy efficiency or efficiency improvement beyond EA Prerequisite Minimum Energy Performance as described below. | Demonstre a melhor eficiência energética ou a melhoria de eficiência além do Pré-requisito EA: Desempenho Energético Mínimo (EA Prerequisite: Minimum Energy Performance) como descrito abaixo. |
| 985d629b8bf-c24d-4484-be49-cbcb985ea4ba | Translation Approved (0%) | Each building must provide actual metered energy data. | Cada edifício deve fornecer os dados reais de energia medidos. |
| 986d629b8bf-c24d-4484-be49-cbcb985ea4ba | Translation Approved (0%) | A full 12 months of continuous energy data is required. | São necessários os dados de energia contínua de um total de 12 meses. |
| 9877f57f9fa-b55c-4b22-bae6-f3cfef1f5ff6 | Translation Approved (100%) | . | . |
| 98870fdf225-5cc5-404d-8cb9-7f633389e913 | Translation Approved (0%) | <888>C</888><889>ase</889><890> 1.</890> | <888>C</888><889>aso</889><890> 1.</890> |
| 98970fdf225-5cc5-404d-8cb9-7f633389e913 | Translation Approved (71%) | ENERGY STAR Rating (3–20 points) | Classificação ENERGY STAR (3 a 20 pontos) |
| 9903b29e814-31f6-41b3-95bd-25fe60ecba99 | Translation Approved (72%) | For buildings eligible to receive an energy performance rating using the EPA ENERGY STAR’s Portfolio Manager tool, points are awarded for ENERGY STAR scores above 75, according to Table 1. | Para edifícios qualificados a receber uma avaliação de desempenho energético usando a ferramenta ENERGY STAR Portfolio Manager da EPA ENERGY STAR, os pontos são concedidos para pontuações ENERGY STAR acima de 75 de acordo com a Tabela 1. |
| 9913b29e814-31f6-41b3-95bd-25fe60ecba99 | Translation Approved (100%) | For projects outside the U.S., consult ASHRAE/ASHRAE/IESNA Standard 90.1–2010, Appendixes B and D, to determine the appropriate climate zone. | Para projetos fora dos EUA, consulte a Norma ASHRAE/ASHRAE/IESNA 90.1–2010, Apêndices B e D, para determinar a zona climática adequada. |
| 992247d2c6a-82a7-4df9-b329-d4839e0f859d | Translation Approved (100%) | Table 1. | Tabela 1. |
| 993247d2c6a-82a7-4df9-b329-d4839e0f859d | Translation Approved (0%) | Points for ENERGY STAR performance ratings | Pontos para classificações de desempenho ENERGY STAR |
| 994a9f4fdc1-1c87-40ff-81c5-a06409e5ebea | Translation Approved (100%) | ENERGY STAR rating | Classificação ENERGY STAR |
| 99537d30ada-2049-4b25-9478-96f2b6029686 | Translation Approved (100%) | Points | Pontos |
| 996d6ffa57f-20e7-4c23-b5c2-8e91435cf01f | Translation Approved (100%) | 76 | 76 |
| 9973bd2b165-487c-4dfc-a3c2-d662f6d71fb8 | Translation Approved (100%) | 3 | 3 |
| 9987e5dcd73-3077-41f9-bc5d-6e824137f893 | Translation Approved (100%) | 77 | 77 |
| 9995b177b7b-84f3-4ed7-87a1-49fbce9b1c22 | Translation Approved (100%) | 4 | 4 |
| 1000417be189-f647-46d9-8b84-2df74a76f2dc | Translation Approved (100%) | 78 | 78 |
| 1001d625cbfa-ce3e-4bc3-8cda-bd0b49433848 | Translation Approved (100%) | 5 | 5 |
| 10024bc0fdc9-8828-4544-888c-9d82c7bd3d1b | Translation Approved (100%) | 79 | 79 |
| 10031ac51fb6-55c3-4a70-a8a3-061473c55209 | Translation Approved (100%) | 6 | 6 |
| 10046d455cbe-8838-4d1e-86b8-5305a77de891 | Translation Approved (100%) | 80 | 80 |
| 1005d4711b89-f178-4139-897f-398d1d56365f | Translation Approved (100%) | 7 | 7 |
| 1006d514b18b-fa75-4bd6-b687-31130bcd4663 | Translation Approved (100%) | 81 | 81 |
| 10076a50550d-fc98-451b-b54b-f1d12c800921 | Translation Approved (100%) | 8 | 8 |
| 100825337ac6-c99b-49df-a9f8-b5124479b32b | Translation Approved (100%) | 82 | 82 |
| 10099f6a676e-ea79-47cd-ad80-888c1b86db53 | Translation Approved (100%) | 9 | 9 |
| 1010617f984c-89a5-43b6-a426-bd63ed9da538 | Translation Approved (100%) | 83 | 83 |
| 101133d5d3a0-cf16-4651-9e11-2be1d01254c5 | Translation Approved (100%) | 10 | 10 |
| 1012535ea8c9-9a0c-46c1-bd6f-0e586e344d9e | Translation Approved (100%) | 84 | 84 |
| 101378a1f538-ac49-4c2b-bff1-a3da823bc6f2 | Translation Approved (100%) | 11 | 11 |
| 1014e02a1b8c-8d4a-4354-81d4-08b85a1f67e1 | Translation Approved (100%) | 85 | 85 |
| 10159476336f-f995-4c6a-862a-5a31a8be9854 | Translation Approved (100%) | 12 | 12 |
| 1016a300bf3e-66cc-4ac2-a279-b75c0e34df53 | Translation Approved (100%) | 86 | 86 |
| 10176ab1727a-b81d-4a43-a96c-3cfc2471db35 | Translation Approved (100%) | 13 | 13 |
| 1018655150aa-8a66-4917-81df-9b4c03a5c58d | Translation Approved (100%) | 87 | 87 |
| 10193b8929c5-abb4-4108-9a55-653a753f0d65 | Translation Approved (100%) | 14 | 14 |
| 102076d17918-43a5-482a-b496-977ee0249582 | Translation Approved (100%) | 88 | 88 |
| 1021cb000f7a-5cf2-4cee-b140-25c8fadc8f86 | Translation Approved (100%) | 15 | 15 |
| 1022c0c01ac4-e83b-4264-8e4c-6af66a98cfcf | Translation Approved (100%) | 89 | 89 |
| 1023d0a9af0d-e68e-4e81-9db1-3f44b6c77897 | Translation Approved (100%) | 16 | 16 |
| 1024f7a9bb59-153f-468a-967d-9029001728cc | Translation Approved (100%) | 90 | 90 |
| 10254ec850fa-d425-407b-a838-5bcce6537100 | Translation Approved (100%) | 17 | 17 |
| 1026be902a2b-c48d-4b55-ab8d-8ee2d7f78239 | Translation Approved (100%) | 91 | 91 |
| 1027ea1605dc-179a-414e-92b1-03fb2871e890 | Translation Approved (100%) | 18 | 18 |
| 1028b2ab3a0f-fd80-4ae8-9048-bfdc1eed25fb | Translation Approved (100%) | 93 | 93 |
| 1029753d9509-9716-4e2c-99db-ed4127ef64bc | Translation Approved (100%) | 19 | 19 |
| 1030ac6a48b4-9ba1-4f4b-86da-a532f2f3fa18 | Translation Approved (100%) | 95 | 95 |
| 103122ce4a69-aeba-42f7-8215-79e7066d5a1d | Translation Approved (100%) | 20 | 20 |
| 10327821ec69-4bfd-4cc3-ba0e-c97ce86aa79e | Translation Approved (100%) | Case 2. | Caso 2. |
| 10337821ec69-4bfd-4cc3-ba0e-c97ce86aa79e | Translation Approved (100%) | Projects Not Eligible for ENERGY STAR Rating | Projetos não elegíveis para a classificação ENERGY STAR |
| 10348f94ec16-f937-49c6-8cee-66e02081a808 | Translation Approved (100%) | Projects not eligible to use EPA’s rating system may compare their buildings’ energy performance with that of comparable buildings, using national averages or actual buildings, or with the previous performance of the project building. | Os projetos não qualificados para usar o sistema de avaliação da EPA podem comparar o desempenho energético de seus edifícios com os de edifícios comparáveis, usando as médias nacionais ou os edifícios reais, com o desempenho anterior do edifício do projeto. |
| 103549fa23f2-6fd8-4a12-adeb-7150d12278a3 | Translation Approved (0%) | <1045>O</1045><1046>ption 1.</1046> | <1045>O</1045><1046>pção 1.</1046> |
| 103649fa23f2-6fd8-4a12-adeb-7150d12278a3 | Translation Approved (0%) | Benchmark against Typical Buildings (1–20 points) | Referência em relação a edifícios existentes (1 a 20 pontos) |
| 1037328759ee-56aa-42e8-840a-474065bcc63b | Translation Approved (100%) | Path 1. | Caminho 1. |
| 1038328759ee-56aa-42e8-840a-474065bcc63b | Translation Approved (82%) | National Average Data Available (1–20 points) | Dados nacionais médios disponíveis (1 a 20 pontos) |
| 10393b5ebc2f-2e74-431f-b718-bafbb3e8d324 | Translation Approved (84%) | Demonstrate energy efficiency performance that is at least 26% better than the median energy performance for typical buildings of similar type by benchmarking against national average source energy data provided in the Portfolio Manager tool. | Demonstre o desempenho de eficiência energética que é pelo menos 26% melhor que o desempenho energético médio de edifícios típicos ou de tipo semelhante tendo como referência os dados nacionais da média de fontes energéticas fornecidos na ferramenta Portfolio Manager. |
| 10403b5ebc2f-2e74-431f-b718-bafbb3e8d324 | Translation Approved (100%) | Points are awarded according to Table 2. | Os pontos são concedidos de acordo com a Tabela 2. |
| 10414fd9bbf3-7bbf-42ed-9f77-f8786cfaf98c | Translation Approved (100%) | Table 2. | Tabela 2. |
| 10424fd9bbf3-7bbf-42ed-9f77-f8786cfaf98c | Translation Approved (0%) | Points for percentage improvement over national average (Option 1, Path 1) or comparable buildings and historical data (Option 3) | Pontos para percentual de melhoria em relação à media nacional (Opção 1 Caminho 1) ou edifícios comparáveis e dados históricos (Opção 3). |
| 10436ea6022d-5b40-4d6d-b148-380f664be89b | Translation Approved (0%) | Percentage improvement | Percentual de melhoria |
| 10440fce98b4-1f0f-45f8-9cf3-113df46a44ba | Translation Approved (100%) | Points | Pontos |
| 10454afb1eeb-5e60-4f11-91dc-42efb5720ac5 | Translation Approved (100%) | 26 | 26 |
| 1046ddf8f7e2-207e-45ed-975d-726d273a17bb | Translation Approved (100%) | 1 | 1 |
| 1047aa0ebf65-9e9a-4d0a-b648-6b2a2a2c6f5d | Translation Approved (100%) | 27 | 27 |
| 1048611303d2-e00e-4df6-ba99-4679fb78b1e6 | Translation Approved (100%) | 2 | 2 |
| 104999842fd5-a3d0-4aaf-92af-c48d0c7d6393 | Translation Approved (100%) | 28 | 28 |
| 10507717b93c-a8d2-4b03-abfc-42f6d0955a8c | Translation Approved (100%) | 3 | 3 |
| 10511f4d4fe0-2acd-4b76-be88-a98a1a860548 | Translation Approved (100%) | 29 | 29 |
| 1052547bfe9a-a9c3-4140-b06a-ee2d0e38d359 | Translation Approved (100%) | 4 | 4 |
| 1053df021c13-22d4-48e6-b90f-de1e8bfe65e0 | Translation Approved (100%) | 30 | 30 |
| 105445f0925a-084a-4054-bfe9-3b5992861c12 | Translation Approved (100%) | 5 | 5 |
| 105571c3a251-38da-4e08-b71d-b79eba6f90af | Translation Approved (100%) | 31 | 31 |
| 1056187b50b2-8ba9-48fc-a27d-919eaf60fcbd | Translation Approved (100%) | 6 | 6 |
| 10570323f760-3dfe-41b4-ab23-596142268f5f | Translation Approved (100%) | 32 | 32 |
| 10581b862583-ea1e-43bd-9d52-42c2643b5a03 | Translation Approved (100%) | 7 | 7 |
| 1059a6467420-92a1-4381-812a-8855e4614c2d | Translation Approved (100%) | 33 | 33 |
| 106085ef255a-9e9f-473d-8749-df383955582e | Translation Approved (100%) | 8 | 8 |
| 1061125ac15c-35c9-4a06-b2b5-972d1d4c173c | Translation Approved (100%) | 34 | 34 |
| 106252482d2e-3430-4027-8abb-7c56a967d252 | Translation Approved (100%) | 9 | 9 |
| 10631c1a4f02-d637-4677-a32d-751a69723519 | Translation Approved (100%) | 35 | 35 |
| 10641cd36b98-262c-4100-8d20-d3c24a14045d | Translation Approved (100%) | 10 | 10 |
| 10656ef8d60a-ac86-402b-9ad6-9b77806a67cd | Translation Approved (100%) | 36 | 36 |
| 106654672d7f-f489-4f16-a762-1d45a97b622b | Translation Approved (100%) | 11 | 11 |
| 1067ed6c30d0-9880-469e-abc3-3299eb274603 | Translation Approved (100%) | 37 | 37 |
| 1068a61d30e5-67c0-4d9f-9232-ff454a48a5bd | Translation Approved (100%) | 12 | 12 |
| 1069ef769427-be13-4757-8b38-81f32b558424 | Translation Approved (100%) | 38 | 38 |
| 1070117434ef-3ecc-49f3-9e3d-06f16079cfdb | Translation Approved (100%) | 13 | 13 |
| 107141897dab-5426-4def-b675-35c8a1e8b409 | Translation Approved (100%) | 39 | 39 |
| 1072112e79ac-0a8c-48de-b466-c3b95c11cb0c | Translation Approved (100%) | 14 | 14 |
| 107306690010-5168-4dd5-b773-ef7a21dbc747 | Translation Approved (100%) | 40 | 40 |
| 10748a5b9751-a7f7-44fa-b286-c093d97bd313 | Translation Approved (100%) | 15 | 15 |
| 1075d44a1837-24f0-42cb-a488-ee0341623d99 | Translation Approved (100%) | 41 | 41 |
| 1076032be3c9-b520-4cc1-be09-856141f8f1b1 | Translation Approved (100%) | 16 | 16 |
| 10771eacf192-ac6f-4d84-b1de-f32c92e4e603 | Translation Approved (100%) | 42 | 42 |
| 107802c68b49-acb2-40de-abc4-82d7740312cc | Translation Approved (100%) | 17 | 17 |
| 10793194aa8c-46f2-4ef1-b60d-a324985fd22a | Translation Approved (100%) | 43 | 43 |
| 1080d66a9b6e-ed05-4db7-82e6-f7a053446578 | Translation Approved (100%) | 18 | 18 |
| 1081f0e258d9-b088-48d8-8048-5baf52b55751 | Translation Approved (100%) | 44 | 44 |
| 1082ad2f398d-835a-4eb2-80b9-94ed213525ee | Translation Approved (100%) | 19 | 19 |
| 108397719418-8144-4c95-8950-f9b5080dae13 | Translation Approved (100%) | 45 | 45 |
| 10845c95db2e-bfe9-4df3-bd55-6d39ada8e9ab | Translation Approved (100%) | 20 | 20 |
| 108575da97a6-ed36-427c-b301-1f0d56462314 | Translation Approved (100%) | Path 2. | Caminho 2. |
| 108675da97a6-ed36-427c-b301-1f0d56462314 | Translation Approved (70%) | National Average Data Not Available (2–14 points) | Dados nacionais médios não disponíveis (2 a 14 pontos) |
| 108772f6dd2e-f451-490c-a7e0-fb594623cca9 | Translation Approved (100%) | If national average source energy data are unavailable for buildings of similar type, benchmark against the building site energy data of at least three similar buildings, normalized for climate, building use, and occupancy. | Se os dados nacionais da média de fontes energéticas estiverem indisponíveis para edifícios de tipo semelhante, tenha como referência os dados da energia do terreno do edifício de pelo menos três edifícios semelhantes, normalizado para clima, uso do edifício e ocupação. |
| 108872f6dd2e-f451-490c-a7e0-fb594623cca9 | Translation Approved (100%) | Points are awarded according to Table 3. | Os pontos são concedidos de acordo com a Tabela 3. |
| 1089d1d72353-545f-4cdb-af21-79729d63a340 | Translation Approved (100%) | OR | OU |
| 1090ec7596d8-edba-441d-8df0-446225c1426b | Translation Approved (100%) | Option 2. | Opção 2. |
| 1091ec7596d8-edba-441d-8df0-446225c1426b | Translation Approved (100%) | Benchmark against Historical Data | Referência em relação a dados históricos |
| 109295ff79ae-1278-4877-80db-9b58c7022081 | Translation Approved (100%) | If national average source energy data are unavailable, compare the building’s site energy data for the previous 12 months with the data from three contiguous years of the previous five, normalized for climate, building use, and occupancy. | Se os dados nacionais da média de fontes energéticas estiverem indisponíveis, compare os dados de energia do terreno do edifício pelos 12 meses anteriores com os dados de três anos consecutivos dos cinco anos anteriores, normalizado para clima, uso do edifício e ocupação. |
| 109395ff79ae-1278-4877-80db-9b58c7022081 | Translation Approved (0%) | Use Table 3 to determine points. | Use a Tabela 3 para determinar os pontos. |
| 109480009346-c465-40e4-ab84-8332736a04cc | Translation Approved (100%) | Table 3. | Tabela 3. |
| 109580009346-c465-40e4-ab84-8332736a04cc | Translation Approved (81%) | Points for percentage improvement over comparable buildings (Option 1, Path 2) or historical data (Option 2) | Pontos para percentual de melhoria em relação a edifícios comparáveis (Opção 1, Caminho 2) ou dados históricos (Opção 2). |
| 1096489f82ca-e44d-4314-82ca-f8cb4baa03a4 | Translation Approved (98%) | Percentage improvement | Percentual de melhoria |
| 1097597c1500-6bf8-4e34-91c8-6f6e0bd70093 | Translation Approved (100%) | Points | Pontos |
| 10989a6b398a-a27a-4c36-b6f3-a47645a3c681 | Translation Approved (100%) | 27 | 27 |
| 1099ce165df0-52a9-4f17-9e15-7aead853ef6b | Translation Approved (100%) | 2 | 2 |
| 1100adb2dd31-f6d6-4717-a509-3adf0f6c217a | Translation Approved (100%) | 30 | 30 |
| 11019af598a6-f8b5-4285-91f7-8241371b3b55 | Translation Approved (100%) | 4 | 4 |
| 11025d5a5ea1-9dba-4437-978e-4e32219fa04c | Translation Approved (100%) | 33 | 33 |
| 1103a8ea3c48-9fd6-4f25-a64b-cd536c60e731 | Translation Approved (100%) | 6 | 6 |
| 1104b53c327f-8e88-4034-a9ac-0a8e14753509 | Translation Approved (100%) | 36 | 36 |
| 11057f92e691-5543-46dc-8626-a09163b25d74 | Translation Approved (100%) | 8 | 8 |
| 1106b16d1780-b0ed-41ca-a8d0-c22616d3b8a9 | Translation Approved (100%) | 39 | 39 |
| 11077fdd06ef-cae0-4e83-9815-24c2dc1bc05d | Translation Approved (100%) | 10 | 10 |
| 1108d2e59828-c9ac-4dde-b5b0-125b48598a1f | Translation Approved (100%) | 42 | 42 |
| 110972a9dab4-4881-44c8-8b09-7ed0abe2f0a4 | Translation Approved (100%) | 12 | 12 |
| 1110e17fc7b4-94d1-4141-96fc-c70dddc80e37 | Translation Approved (100%) | 45 | 45 |
| 111172957a65-6cfa-48f0-98b1-0e524954619c | Translation Approved (100%) | 14 | 14 |
| 1112acc9a6dc-eaec-4b0a-ba5c-a51939763fea | Translation Approved (100%) | OR | OU |
| 1113ef1600c6-5564-443a-93cd-20fab285b98b | Translation Approved (100%) | Option 3. | Opção 3. |
| 1114ef1600c6-5564-443a-93cd-20fab285b98b | Translation Approved (0%) | Benchmark against Both Similar Buildings and Historical Data | Referência em relação a edifícios semelhantes e dados históricos |
| 1115ce1c9b11-361a-4ad4-8b95-deaa072d315a | Translation Approved (0%) | Follow the requirements of both Option 1, Path 2, and Option 2 to benchmark against the site energy data for the three similar buildings and the building’s historic data. | Siga os requisitos da Opção 1, Caminho 2 e Opção 2 para ter como referência os dados de energia do terreno dos três edifícios semelhantes e dos dados históricos do edifício. |
| 1116ce1c9b11-361a-4ad4-8b95-deaa072d315a | Translation Approved (100%) | Use Table 2 to determine points. | Use a Tabela 2 para determinar os pontos. |
| 11178ae1fdf5-f7dd-4b77-8475-9748cf842102 | Translation Approved (100%) | EA Credit: Advanced Energy Metering | Crédito EA: Medição de Energia Avançada (EA Credit: Advanced Energy Metering) |
| 11182b885ff7-4ef8-4ff5-a1c1-7c8ce5bbb153 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 111981cad3e9-cdd3-48ce-821f-afe489eaf0a5 | Translation Approved (100%) | 2 points | 2 pontos |
| 1120924adafb-ae92-4ffd-ad3b-cb54ae53c3bd | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 112194ca9ad1-09b3-4ce7-b36e-10e7f1764747 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (2 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (2 pontos) |
| 1122e26f7560-83d1-4779-81e6-bf8d0d26b3a0 | Translation Approved (100%) | EB:O&M Schools (2 points) | EB:O&M Escolas (Schools) (2 pontos) |
| 1123a06744c8-64e1-4ec9-92c9-5ef51e158b0a | Translation Approved (100%) | EB:O&M Retail (2 points) | EB:O&M Varejo (Retail) (2 pontos) |
| 1124310e6ecd-d8cb-4cf8-b40b-34645019346d | Translation Approved (100%) | EB:O&M Data Centers (2 points) | EB:O&M Centros de Dados (Data Centers) (2 pontos) |
| 1125596017e6-1a30-415b-8d6d-03bdfe2209b7 | Translation Approved (100%) | EB:O&M Hospitality (2 points) | EB:O&M Hoteleiro (Hospitality) (2 pontos) |
| 1126fd9fd5d3-05af-4bcd-a1cb-ed51b879e93a | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (2 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (2 pontos) |
| 11279d0e52ca-0c6b-4ec3-83d7-b4d4929b2244 | Translation Approved (100%) | Intent | Objetivo |
| 11287c6d12a7-b029-48e7-97f5-c6d0fbf46ccb | Translation Approved (CM) | To support energy management and identify opportunities for additional energy savings by tracking building-level and system-level energy use. | Apoiar a gestão de energia e identificar oportunidades de economias adicionais de energia rastreando o uso de energia no edifício e nos sistemas. |
| 112910791819-40c3-4921-8108-7e3930909cfd | Translation Approved (100%) | Requirements | Requisitos |
| 1130d03b86d9-4290-4fa1-965b-7b4bf521b4c1 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1131993053b9-5f0e-4ec3-aa0d-4c9ee4260037 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 113221cab752-558b-4b24-a209-9cfc3cdcda86 | Translation Approved (97%) | Install <1294>advanced energy metering </1294>for the following: | Instale <1294>medição avançada de energia</1294> para o seguinte: |
| 11337051c32e-a208-4f4f-9584-429ec7b0d8f2 | Translation Approved (CM) | all whole-building energy sources used by the building; and | todas as fontes de energia do edifício usadas pelo edifício; e |
| 1134fb8b5889-1b1d-47e4-bedd-9b741a53e9df | Translation Approved (89%) | major end uses that represent 20% or more of the total annual consumption of the building minus plug load use. | grandes usos finais que representam 20% ou mais do consumo total anual do edifício menos a carga da tomada. |
| 113581c5516b-3336-4c15-ae37-d3ad91c2ce61 | Translation Approved (100%) | The advanced energy metering must have the following characteristics. | A medição avançada de energia deve ter as seguintes características. |
| 1136617c9bae-9763-4be0-aec4-fee4487289c2 | Translation Approved (CM) | Meters must be permanently installed, record at intervals of one hour or less, and transmit data to a remote location. | Os medidores devem ser instalados de forma permanente, registrar em intervalos de uma hora ou menos e transmitir dados para um local remoto. |
| 1137f737b67a-e4d3-4f8e-859c-3ae13199174f | Translation Approved (CM) | Electricity meters must record both consumption and demand. | Medidores de eletricidade devem registrar o consumo e a demanda. |
| 1138f737b67a-e4d3-4f8e-859c-3ae13199174f | Translation Approved (CM) | Whole-building electricity meters should record the power factor, if appropriate. | Medidores de eletricidade de todo o edifício devem registrar o fator de potência, se adequado. |
| 11398550bbd8-6f37-4c71-ab29-003c4dac8ddb | Translation Approved (CM) | The data collection system must use a local area network, building automation system, wireless network, or comparable communication infrastructure. | O sistema de coleta de dados deve usar uma rede local, sistema de automação predial, rede sem fio ou infraestrutura de comunicação comparável. |
| 1140495152bd-e7bc-40c4-9b3f-e4966645cb5b | Translation Approved (CM) | The system must be capable of storing all meter data for at least 36 months. | O sistema deve ser capaz de armazenar todos os dados de medição por pelo menos 36 meses. |
| 1141975d3194-6831-4506-8795-55f516912679 | Translation Approved (CM) | The data must be remotely accessible. | Deve ser possível acessar os dados remotamente. |
| 1142f6a54653-eb5b-47d0-b687-16652a743ec6 | Translation Approved (CM) | All meters in the system must be capable of reporting hourly, daily, monthly, and annual energy use. | Todos os medidores no sistema devem ser capazes gerar relatórios horários, diários, mensais e anuais do uso de energia. |
| 11439d80bdbe-8440-4f22-ae8c-b0228e328979 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 1144e63558d2-8872-414a-9f0e-13a8bf2861fa | Translation Approved (0%) | Program the facility’s energy management system to set an alarm whenever the energy consumption and peak demand rise above the anticipated amount by more than 5%. | Programe o sistema de gestão de energia da instalação para acionar um alarme sempre que o consumo de energia e a demanda de pico ultrapassar o valor previsto em mais de 5%. |
| 1145e63558d2-8872-414a-9f0e-13a8bf2861fa | Translation Approved (0%) | The anticipated consumption and peak should be determined by analyzing historical facility performance and weather and operating conditions and should be set on at least monthly, preferably daily. | O consumo e pico previstos devem ser determinados analisando o desempenho histórico da instalação e as condições climáticas e operacionais e devem ser definidos no mínimo mensalmente, preferivelmente diariamente. |
| 114698ddd138-8eb1-4e68-bdaa-3029275e86ec | Translation Approved (0%) | Demand measurements must be taken in time increments no longer than the increments used for utility billing or in one-hour increments, whichever is less time. | As medições da demanda devem ser realizadas nos incrementos de tempo sem exceder os incrementos usados para contas de serviços públicos ou nos incrementos de uma hora, o tempo que for menor. |
| 114713c617e7-b8bd-48ac-9aaa-e1654affe82a | Translation Approved (0%) | On at least a monthly basis, report the facility’s utility peak demand and total consumption and compare it with the data for the previous month and the same month from the previous year. | No mínimo mensalmente, registre a demanda de pico de serviços públicos e o consumo total da instalação e compare os registros com os dados do mês anterior e o mesmo mês do ano anterior. |
| 1148b7d7b162-2c18-4a5a-9bb9-77166d71c542 | Translation Approved (100%) | EA Credit: Demand Response | Crédito EA: Resposta à Demanda (EA Credit: Demand Response) |
| 1149e3ad8bb4-3396-4dc7-9661-615f62ec38a8 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 11500d7ac47d-46c0-4900-8a8d-0e6232aa469b | Translation Approved (0%) | 1–3 points | 1 a 3 pontos |
| 1151816a27cf-5481-4d28-8273-64cedde9cb24 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 1152718c9478-069f-4a98-b316-df813a28cb15 | Translation Approved (96%) | Existing Buildings: Operations & Maintenance (1–3 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 a 3 pontos) |
| 1153cf3deff5-1a70-47dd-b586-b0fe1941e54e | Translation Approved (95%) | EB:O&M Schools (1–3 points) | EB:O&M Escolas (Schools) (1 a 3 pontos) |
| 1154fd9f662d-686e-4e4d-b2f9-e6755d59d13d | Translation Approved (95%) | EB:O&M Retail (1–3 points) | EB:O&M Varejo (Retail) (1 a 3 pontos) |
| 115598f6d67d-fbe9-41ea-a951-b55a2ee5ee17 | Translation Approved (96%) | EB:O&M Data Centers (1–3 points) | EB:O&M Centros de Dados (Data Centers) (1 a 3 pontos) |
| 11566919ea35-d254-4129-94f3-b4deb57392ca | Translation Approved (95%) | EB:O&M Hospitality (1–3 points) | EB:O&M Hoteleiro (Hospitality) (1 a 3 pontos) |
| 115737be04fb-5593-4fdd-856b-4c8171f648f7 | Translation Approved (97%) | EB:O&M Warehouses & Distribution Centers (1–3 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 a 3 pontos) |
| 1158f2e84f59-d0cf-4583-9c39-34902805f9d9 | Translation Approved (100%) | Intent | Objetivo |
| 115948c16147-a6ce-4cf1-b557-f7b201488c8c | Translation Approved (CM) | To increase participation in demand response technologies and programs that make energy generation and distribution systems more efficient, increase grid reliability, and reduce greenhouse gas emissions. | Aumentar a participação em tecnologias e programas de resposta à demanda que tornem sistemas de geração e distribuição de energia mais eficientes, aumentar a confiabilidade da rede de energia elétrica e reduzir as emissões de gases do efeito estufa. |
| 1160028d693b-cc8a-416b-85cf-d3896d6bebdb | Translation Approved (100%) | Requirements | Requisitos |
| 1161d5b7818b-360b-4cd4-9af4-ed4e95f03cb7 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1162a0c7c344-be6f-44a3-ae86-8e19fef71acf | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 11632873d3bb-2971-48aa-9262-a86ac6d0fb28 | Translation Approved (0%) | Evaluate building systems and equipment for participation in a demand response program. | Avalie os sistemas e equipamentos do edifício para participação em um programa de resposta à demanda. |
| 11642873d3bb-2971-48aa-9262-a86ac6d0fb28 | Translation Approved (100%) | On-site electricity generation does not meet the intent of this credit. | Geração de eletricidade no local não atende ao objetivo deste crédito. |
| 116595d889a0-a9c5-461b-9244-a4219f8f6c5a | Translation Approved (100%) | Case 1. | Caso 1. |
| 116695d889a0-a9c5-461b-9244-a4219f8f6c5a | Translation Approved (CM) | Demand Response Program Available (3 points) | Programa de resposta à demanda disponível (3 pontos) |
| 1167a553b442-3cba-48d8-816c-0dde9fadd509 | Translation Approved (CM) | Participate in an existing demand response (DR) program and complete the following activities. | Participe de um programa de resposta à demanda (demand response - DR) existente e execute as atividades a seguir. |
| 116807feb513-29df-4981-b408-7248a9be2201 | Translation Approved (0%) | Have in place a system with the capability for real-time, fully automated DR based on external initiation by a DR program provider. | Implemente um sistema com a capacidade de DR totalmente automática e em tempo real com base em uma iniciação externa por um provedor de programa de DR. |
| 116907feb513-29df-4981-b408-7248a9be2201 | Translation Approved (100%) | Semi-automated DR may be utilized in practice. | DR semiautomatizada pode ser utilizada na prática. |
| 11706fa83832-22fc-4df7-b129-63dd02c1d4f4 | Translation Approved (98%) | Enroll in a minimum one-year DR participation amount contractual commitment with a qualified DR program provider, with the intention of multiyear renewal, for at least 10% of the annual peak electricity demand. | Comprometa-se contratualmente com uma participação mínima de DR de um ano com um provedor de programa de DR qualificado, com o objetivo de renovação plurianual, para pelo menos 10% da demanda de pico de eletricidade anual. |
| 11716fa83832-22fc-4df7-b129-63dd02c1d4f4 | Translation Approved (0%) | Peak demand is based on electric utility bills. | A demanda de pico é baseada nas contas de serviços públicos de eletricidade. |
| 11726776e30b-0f79-4e08-81fb-ba4ec52f1e22 | Translation Approved (100%) | Develop a comprehensive plan for meeting the contractual commitment during a Demand Response event. | Desenvolva um plano abrangente para cumprir com o compromisso contratual durante um evento de Resposta à demanda. |
| 117328818ce4-fb07-4053-b3ed-c84108be3db8 | Translation Approved (0%) | Include the DR processes in the current facilities requirements and operations and maintenance plan. | Inclua os processos de DR nos requisitos atuais da instalação e no plano de manutenção e operações. |
| 1174374d60dc-b162-4912-b990-21cb46059705 | Translation Approved (0%) | Initiate at least one full test of the DR plan. | Inicie pelo menos um teste completo do plano de DR. |
| 117554cb1e10-6eb8-4769-804d-2ca901e835c4 | Translation Approved (100%) | Case 2. | Caso 2. |
| 117654cb1e10-6eb8-4769-804d-2ca901e835c4 | Translation Approved (CM) | Demand Response Program Not Available (1 point) | Programa de resposta à demanda não disponível (1 ponto) |
| 11774aa3bc27-af31-4550-a4ee-e641c48f5621 | Translation Approved (0%) | Have infrastructure in place to take advantage of future demand response programs or dynamic, real-time pricing programs and complete the following activities.Develop a comprehensive plan for shed at least 10% of the annual peak electricity demand. | Implemente a infraestrutura para tirar vantagem dos programas futuros de resposta à demanda ou dos programas de preço dinâmicos e em tempo real e concluir as atividades a seguir. Desenvolva um plano abrangente para reduzir pelo menos 10% da demanda de eletricidade do pico anual. |
| 11784aa3bc27-af31-4550-a4ee-e641c48f5621 | Translation Approved (100%) | Peak demand is based on electric utility bills. | A demanda de pico é baseada nas contas de serviços públicos de eletricidade. |
| 1179ffd7d304-1e4e-456f-867d-1107c38f899b | Translation Approved (100%) | Include the DR processes in the current facilities requirements and operations and maintenance plan. | Inclua os processos de DR nos requisitos atuais da instalação e no plano de manutenção e operações. |
| 1180ca32957b-fe12-4847-8a9d-977a2164078d | Translation Approved (100%) | Initiate at least one full test of the DR plan. | Inicie pelo menos um teste completo do plano de DR. |
| 1181e1405000-b43d-4c55-b189-aed342bf9af8 | Translation Approved (100%) | Contact local utility representatives to discuss participation in future DR programs. | Entre em contato com representantes de serviços públicos locais para discutir a participação em futuros programas de DR. |
| 11822e9ecd82-1c96-46a1-9db1-30a730238e5e | Translation Approved (0%) | Case 3: Permanent Load Shifting (2 points) | Caso 3: Mudança de carga permanente (2 pontos) |
| 1183fc32dec3-4fe3-485a-b2bc-1d9835b520c7 | Translation Approved (0%) | Implement electrical load shifting measures with the following requirements: | Implemente medidas de mudança de carga elétrica com os seguintes requisitos: |
| 1184d020e079-2cd2-45c8-a347-da11caf7b174 | Translation Approved (0%) | Have in place during the performance period a system which permanently transfers electricity demand from peak hours to off-peak hours as defined by the local utility provider. | Durante o período de desempenho, implemente um sistema que transfira permanentemente a demanda de eletricidade do horário de pico para o horário fora de pico como definido pelo provedor de serviços públicos local. |
| 1185365366a1-6ab4-4364-94ba-fcca10aa0164 | Translation Approved (0%) | Demonstrate that the facility is successfully reducing peak demand by 10% during the performance period as compared to peak electrical demand by: | Demonstre que a instalação está reduzindo de forma bem-sucedida a demanda de pico em 10% durante o período de desempenho em comparação com a demanda elétrica do pico ao: |
| 1186829923d6-a286-43f8-8059-8a81d83b889c | Translation Approved (0%) | Identifying all load shifting measures and their intended peak electrical load shift | Identifique todas as medidas de mudança de carga e sua mudança de carga elétrica do pico pretendida |
| 11879514ca8e-4c33-4c58-b22c-231444a7734b | Translation Approved (0%) | Verifying a corresponding peak electrical load reduction for each measure | Verifique uma redução da carga elétrica do pico correspondente para cada medida |
| 11888faa9f4d-6801-4bcf-be5e-3e78a0cee339 | Translation Approved (87%) | Verifying a corresponding off-peak electrical load increase for each measure | Verifique um aumento da carga elétrica nos horários fora de pico correspondente para cada medida |
| 1189fb7cd3cd-a96a-4396-85c3-1ccc989c8dc4 | Translation Approved (75%) | Include the load shifting measures in the Current Facilities Requirements and Operations and Maintenance Plan. | Inclua as medidas de mudança de carga nos Requisitos atuais da instalação e no Plano de manutenção e operações. |
| 1190cc60138f-e315-4719-8e0c-eb9b8c341660 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 11911b3b63a4-b29c-4119-8fe8-dd11dfc1c3bf | Translation Approved (100%) | None. | Nenhum. |
| 1192f69c86b6-dd56-422d-9b31-195c71a6c299 | Translation Approved (79%) | EA Credit: Renewable Energy and Carbon Offsets | Crédito EA: Energia Renovável e Compensações de Carbono (EA Credit: Renewable Energy and Carbon Offsets) |
| 1193cc6baef9-2bff-40a6-8cb1-150b1cda02fc | Translation Approved (100%) | EB:O&M | EB:O&M |
| 11943d4acb7e-d879-4b15-9eb3-297971c8cadd | Translation Approved (100%) | 1–5 points | 1 a 5 pontos |
| 11959d1050a2-2942-4594-abd2-88d6bced6384 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 11967fc0d320-6ca4-41a2-a7b5-17a3e81293b1 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1–5 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 a 5 pontos) |
| 1197b07c93b1-7ab3-48a0-a7f6-761db91e7015 | Translation Approved (100%) | EB:O&M Schools (1–5 points) | EB:O&M Escolas (Schools) (1 a 5 pontos) |
| 11980f40f3ce-ccf7-4a00-a1d2-1f23916b5a55 | Translation Approved (100%) | EB:O&M Retail (1–5 points) | EB:O&M Varejo (Retail) (1 a 5 pontos) |
| 1199abe67d58-b70e-4134-95cd-4bf911c3522c | Translation Approved (96%) | EB:O&M Data Centers (1–5 points) | EB:O&M Centros de Dados (Data Centers) (1 a 5 pontos) |
| 120019150e4a-1259-4694-8647-324cbad052ea | Translation Approved (100%) | EB:O&M Hospitality (1–5 points) | EB:O&M Hoteleiro (Hospitality) (1 a 5 pontos) |
| 1201700bb7b2-620f-4be0-85cc-48d3aaac76d0 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1–5 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 a 5 pontos) |
| 1202a31241af-183c-4bd4-b62f-1d5ff90487ae | Translation Approved (100%) | Intent | Objetivo |
| 1203334698db-2eca-4ff7-aee9-4808c2cf44f2 | Translation Approved (93%) | To encourage the reduction of greenhouse gas emissions through the use of local and grid-source renewable energy technologies and carbon mitigation projects. | Incentivar a redução de emissões de gases do efeito estufa com o uso de fontes locais e provenientes da rede de energia, tecnologias de energia renovável e projetos de mitigação de carbono. |
| 120427d00bfd-504f-458d-9e81-5aa390abf818 | Translation Approved (100%) | Requirements | Requisitos |
| 12055aaa81db-289f-4968-bf50-3dbf35dcce28 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1206ed9b0d06-96c4-4efd-a8b6-4710b6c42160 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 1207829b8a20-a8a2-4ba9-bf7d-03cdf582449a | Translation Approved (0%) | Demonstrate one or both of the following for at least a portion of the building’s total energy use. | Demonstre uma ou ambas as opções a seguir por pelo menos uma porção do uso total de energia do edifício. |
| 12081c01510b-9292-4c05-b295-b3ed6f8e05ac | Translation Approved (0%) | Total energy use is met directly with renewable energy systems. | O uso total de energia é atendido diretamente com os sistemas de energia renovável. |
| 1209fa31aeb7-e003-42b1-b5fe-648e81c678db | Translation Approved (0%) | A minimum two-year contract is in place, with the commitment to renew on an ongoing basis, to purchase qualified resources that will be delivered at least annually. | Um contrato de no mínimo dois anos é necessário, com o compromisso de renovar continuamente, para comprar recursos qualificados que serão entregues anualmente, no mínimo. |
| 1210fa31aeb7-e003-42b1-b5fe-648e81c678db | Translation Approved (0%) | Resources must have come online after January 1, 2005. | Os recursos devem ter aparecido on-line após 1 de janeiro de 2005. |
| 121101c208fc-ba90-42de-af79-6cde003262dd | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 1212c5fb49e8-1242-4854-b0b0-ca76f1e337ca | Translation Approved (0%) | Meet at least some of the building’s total energy use directly with renewable energy systems, or engage in a contract to purchase green power, carbon offsets, or Renewable Energy Certificates (RECs). | Atenda ao menos parte do uso total de energia do edifício diretamente com os sistemas de energia renovável, ou participar de um contrato para adquirir energia verde, compensações de carbono ou certificados de energia renovável (Renewable Energy Certificates - RECs). |
| 12133a4a1d53-2487-4f8e-82c0-2ff8e98b14b9 | Translation Approved (0%) | Green power and RECs must be Green-e Energy Certified or the equivalent. | A energia verde e os RECs devem ter certificação Green-e ou equivalente. |
| 12143a4a1d53-2487-4f8e-82c0-2ff8e98b14b9 | Translation Approved (0%) | RECs can be used only to mitigate the effects of Scope 2, electricity use. | Os RECs podem ser usados apenas para mitigar os efeitos do Escopo 2, uso de eletricidade. |
| 1215fbd64c7e-a31f-4d80-9a7b-5a4c183511bd | Translation Approved (100%) | Carbon offsets may be used to mitigate Scope 1 or Scope 2 emissions on a metric ton of carbon dioxide–equivalent basis and must be Green-e Climate certified, or the equivalent. | Compensações de carbono podem ser usadas para atenuar emissões do Escopo 1 ou do Escopo 2 em uma tonelada métrica de dióxido de carbono ou base equivalente e devem ter a certificação Green-e Climate ou equivalente. |
| 12169f71b4b0-5c9a-47cc-9975-2f1fe2e86264 | Translation Approved (96%) | For U.S. projects, the offsets must come from greenhouse gas emissions reduction projects within the U.S. | Para projetos nos EUA, as compensações devem ser de projetos de redução de emissões de gases do efeito estufa dentro dos EUA. |
| 1217e25c8675-f15a-494c-a95c-fbf95b18c4b7 | Translation Approved (0%) | Use the following equation to calculate credit, up to the 5-point limit: | Use a seguinte equação para calcular o crédito, até o limite de 5 pontos: |
| 1218586d1b69-7d48-473f-9b5a-56030fc5e005 | Translation Approved (91%) | Points = | Pontos = |
| 1219a2c41432-7800-4033-b767-941daafe4d2c | Translation Approved (74%) | Renewable energy generated % | % de energia renovável gerada |
| 1220d62c72a4-bb2d-49f0-a29d-18661d602796 | Translation Approved (100%) | + | + |
| 12212fd58317-0d14-40cb-bc51-0a489866a2d5 | Translation Approved (0%) | Energy purchased/offset % (Not to exceed 100%) | % de energia adquirida/compensação (Não deve exceder 100%) |
| 122282a6fad9-8136-4e10-9139-e6676ae720e9 | Translation Approved (100%) | 1.5% | 1,5% |
| 122337f0865f-5c30-4b5f-97f3-933f14007b1e | Translation Approved (100%) | 25% | 25% |
| 12240a844161-756c-4482-ada4-3bb4cdae8c25 | Translation Approved (100%) | EA Credit: Enhanced Refrigerant Management | Crédito EA: Gerenciamento Aprimorado de Gases Refrigerantes (EA Credit: Enhanced Refrigerant Management) |
| 1225449e2e47-9b5d-47a2-b489-51a699cc7c21 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 1226153fdb82-c869-4244-80ec-d0c114f30d8e | Translation Approved (100%) | 1 point | 1 ponto |
| 1227ed49ecee-0c47-4917-ae6b-277c0216072c | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 1228a58aea84-41d4-40f6-afab-1a1e291bbce4 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1 point) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 ponto) |
| 122965665488-d45b-4a6c-b87c-685fe910efb6 | Translation Approved (100%) | EB:O&M Schools (1 point) | EB:O&M Escolas (Schools) (1 ponto) |
| 12304ff7a14f-89b2-4f74-be53-6013c6e6fa9e | Translation Approved (100%) | EB:O&M Retail (1 point) | EB:O&M Varejo (Retail) (1 ponto) |
| 1231e12e792d-797c-4b32-ab9c-10a4642fce1f | Translation Approved (100%) | EB:O&M Data Centers (1 point) | EB:O&M Centros de Dados (Data Centers) (1 ponto) |
| 123281797247-bb60-401e-a34f-f35aeef74812 | Translation Approved (100%) | EB:O&M Hospitality (1 point) | EB:O&M Hoteleiro (Hospitality) (1 ponto) |
| 12337f2e5843-8195-4fee-9ad6-90ab628dc893 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1 point) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 ponto) |
| 123470b77033-c93c-4973-974e-2a209bfbdcf8 | Translation Approved (100%) | Intent | Objetivo |
| 1235b84f6d3d-baa1-46bb-9c49-dfca0e7850bf | Translation Approved (CM) | To reduce ozone depletion and support early compliance with the Montreal Protocol while minimizing direct contributions to climate change. | Reduzir a destruição da camada de ozônio e promover conformidade antecipada com o Protocolo de Montreal minimizando ao mesmo tempo as contribuições diretas para a mudança climática. |
| 1236042b6cb1-cd6d-43c6-b7a7-d59961c26915 | Translation Approved (100%) | Requirements | Requisitos |
| 1237d8380917-cd3b-42bb-b8e0-f6bf6cc46ae9 | Translation Approved (92%) | EBOM, Schools, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 12387f42aa42-307c-4950-b0cb-37ca2fcd43e5 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 12390e5ebe1d-0877-4a0c-8a5a-9a26b7366863 | Translation Approved (100%) | Option 1. | Opção 1. |
| 12400e5ebe1d-0877-4a0c-8a5a-9a26b7366863 | Translation Approved (CM) | No Refrigerants or Low-Impact Refrigerants (1 point) | Sem refrigerantes ou com refrigerantes de baixo impacto (1 ponto) |
| 124105ec08a0-036a-41f9-b837-8cbabaa57906 | Translation Approved (CM) | Do not use refrigerants, or use only refrigerants (naturally occurring or synthetic) that have an ozone depletion potential (ODP) of zero and a global warming potential (GWP) of less than 50. | Não use refrigerantes ou use apenas refrigerantes (naturais ou sintéticos) que tenham um potencial de destruição da camada de ozônio (ozone depletion potential - ODP) de zero e um potencial de aquecimento global (global warming potential - GWP) inferior a 50. |
| 12426f209717-ad46-4232-92d9-873edb6c6655 | Translation Approved (100%) | OR | OU |
| 1243b74a3a93-2ae0-43f2-85d1-043eee8b3b89 | Translation Approved (100%) | Option 2. | Opção 2. |
| 1244b74a3a93-2ae0-43f2-85d1-043eee8b3b89 | Translation Approved (0%) | Calculation of Refrigerant Impact (1 point) | Cálculo do impacto do refrigerante (1 ponto) |
| 12452676e5e0-cd22-4415-a237-67771c846285 | Translation Approved (CM) | Select refrigerants that are used in heating, ventilating, air-conditioning, and refrigeration (HVAC&R) equipment to minimize or eliminate the emission of compounds that contribute to ozone depletion and climate change. | Selecione refrigerantes usados em equipamentos de aquecimento, ventilação, ar-condicionado e refrigeração (HVAC&R) para minimizar ou eliminar a emissão de compostos que contribuem para a destruição da camada de ozônio e a mudança climática. |
| 12462676e5e0-cd22-4415-a237-67771c846285 | Translation Approved (CM) | The combination of all new and existing base building and tenant HVAC&R equipment that serve the project must comply with the following formula: | A combinação de equipamentos de HVAC&R novos e existentes do edifício base e de locatários que atendam ao projeto deve seguir a seguinte fórmula: |
| 12470b004333-2ad8-4740-b11c-af5ba4f228c8 | Translation Approved (CM) | IP units | Sistema imperial |
| 124837c91924-afaa-4e41-8f60-af973ca6f290 | Translation Approved (100%) | LCGWP | LCGWP |
| 1249392c558f-a89e-405c-803b-6338264bc5db | Translation Approved (100%) | + | + |
| 12504844f209-235d-4d68-81fa-9ac81c90f653 | Translation Approved (100%) | LCODP | LCODP |
| 12512bf472a2-7edb-418a-ba96-616843b8684f | Translation Approved (100%) | x | x |
| 1252c853c030-70dd-4036-a176-55a9ad04b963 | Translation Approved (100%) | 10<1361>5</1361> | 10<1361>5</1361> |
| 1253f4b421e7-ce1c-45b3-8cdc-0fb34890ba5c | Translation Approved (0%) | ≤ | ≤ |
| 125417336ec4-8975-44af-892c-6584e5ad1d17 | Translation Approved (100%) | 100 | 100 |
| 12551d7f22c0-3cf8-435f-9ee7-35d073eb5721 | Translation Approved (CM) | SI units | Sistema métrico |
| 12561e8dd262-247c-4886-86d5-4f3d6c21758c | Translation Approved (100%) | LCGWP | LCGWP |
| 1257e1ed84c6-553f-4663-a9c6-b2e8a8db746b | Translation Approved (100%) | + | + |
| 12587aecfdcd-d539-4b76-9e6b-e05711f32855 | Translation Approved (100%) | LCODP | LCODP |
| 12596b6f5709-f962-4898-b033-f881c2880f73 | Translation Approved (100%) | x | x |
| 12607313494b-fb2d-4db6-a0dd-da206b1ec012 | Translation Approved (100%) | 10<1387>5</1387> | 10<1387>5</1387> |
| 12618dbf5102-f252-4b17-a46e-78add6c1cce4 | Translation Approved (100%) | ≤ | ≤ |
| 12628fbdade7-4285-4e35-b4e4-1de951717b26 | Translation Approved (100%) | 13 | 13 |
| 1263b60a1d93-600d-4cee-8740-59a3597f2091 | Translation Approved (CM) | Calculation definitions for LCGWP + LCODP x 10<1398>5</1398> ≤ 100 | Definições de cálculo de LCGWP + LCODP x 10<1398>5</1398> ≤ 100 |
| 12641b322551-b89f-4e0c-91b7-e8114618ea47 | Translation Approved (CM) | (IP units) | (Sistema imperial) |
| 126523510c4f-634e-4852-b37d-5da5d2858ae8 | Translation Approved (100%) | Calculation definitions for LCGWP + LCODP x 10<1407>5</1407> ≤ 13 | Definições de cálculo de LCGWP + LCODP x 10<1407>5</1407> ≤ 13 |
| 12665a4d8d63-5610-4a2a-b2f8-f74bb22ef359 | Translation Approved (CM) | (SI units) | (Sistema métrico) |
| 12670c5c7dc7-8d33-46d0-957a-4fcff2296f11 | Translation Approved (CM) | LCODP = [ODPr x (Lr x Life +Mr) x Rc]/Life | LCODP = [ODPr x (Lr x Vida útil +Mr) x Rc]/Vida útil |
| 126860d72bb3-4a8f-46dd-be6e-9e401738ee1e | Translation Approved (100%) | LCODP = [ODPr x (Lr x Life +Mr) x Rc]/Life | LCODP = [ODPr x (Lr x Vida útil +Mr) x Rc]/Vida útil |
| 12698b3c83c9-7f3d-4429-8c14-1f1292468ccf | Translation Approved (CM) | LCGWP = [GWPr x (Lr x Life +Mr) x Rc]/Life | LCGWP = [GWPr x (Lr x Vida útil +Mr) x Rc]/Vida útil |
| 12700a106193-6284-48f0-b37f-53098d8281b5 | Translation Approved (100%) | LCGWP = [GWPr x (Lr x Life +Mr) x Rc]/Life | LCGWP = [GWPr x (Lr x Vida útil +Mr) x Rc]/Vida útil |
| 12719bd07cac-3d52-4a6a-bc90-3c94ce5c3dc2 | Translation Approved (CM) | LCODP: Lifecycle Ozone Depletion Potential (lb CFC 11/Ton-Year) | LCODP: Potencial de destruição da camada de ozônio do ciclo de vida (lb CFC 11/Ton-Ano) |
| 12727040f73c-9a8a-478c-94de-cceaf1e56844 | Translation Approved (CM) | LCODP: Lifecycle Ozone Depletion Potential (kg CFC 11/(kW/year)) | LCODP: Potencial de destruição da camada de ozônio do ciclo de vida (kg CFC 11/(kW/ano)) |
| 127392e97f0d-22f1-4d46-a7d1-0ef5c17b5502 | Translation Approved (CM) | LCGWP: Lifecycle Direct Global Warming Potential (lb CO<1433>2</1433>/Ton-Year) | LCGWP: Potencial de aquecimento global direto do ciclo de vida (lb CO<1433>2</1433>/Ton-Ano) |
| 1274eee47d0a-64c5-4ac8-a5ff-ee435d553161 | Translation Approved (CM) | LCGWP: Lifecycle Direct Global Warming Potential (kg CO<1437>2</1437>/kW-year) | LCGWP: Potencial de aquecimento global direto do ciclo de vida (kg CO<1437>2</1437>/kW-ano) |
| 1275ae334aec-c570-4e0f-baba-4942143512af | Translation Approved (CM) | GWPr: Global Warming Potential of Refrigerant (0 to 12,000 lb CO<1441>2</1441>/lbr) | GWPr: Potencial de aquecimento global do refrigerante (0 a 12.000 lb CO<1441>2</1441>/lbr) |
| 127620e5b820-a47a-4800-8d48-1cc22e84f638 | Translation Approved (CM) | GWPr: Global Warming Potential of Refrigerant (0 to 12,000 kg CO<1445>2</1445>/kg r) | GWPr: Potencial de aquecimento global do refrigerante (0 a 12.000 kg CO<1445>2</1445>/kg r) |
| 1277860f28b8-5368-4d0a-8bf5-42573b75c1d5 | Translation Approved (CM) | ODPr: Ozone Depletion Potential of Refrigerant (0 to 0.2 lb CFC 11/lbr) | ODPr: Potencial de destruição da camada de ozônio do refrigerante (0 a 0,2 lb CFC 11/lbr) |
| 1278caf0f550-6799-4cb7-abdd-45c36774bb4e | Translation Approved (CM) | ODPr: Ozone Depletion Potential of Refrigerant (0 to 0.2 kg CFC 11/kg r) | ODPr: Potencial de destruição da camada de ozônio do refrigerante (0 a 0,2 kg CFC 11/kg r) |
| 1279162748a3-c2d0-4453-8e59-48c76848efa0 | Translation Approved (CM) | Lr: Refrigerant Leakage Rate (2.0%) | Lr: Taxa de vazamento do refrigerante (2,0%) |
| 1280655e0299-da5e-4671-9cb4-967563f949d9 | Translation Approved (100%) | Lr: Refrigerant Leakage Rate (2.0%) | Lr: Taxa de vazamento do refrigerante (2,0%) |
| 1281f986e793-48d6-4412-bdc6-01c5ed1a972f | Translation Approved (CM) | Mr: End-of-life Refrigerant Loss (10%) | Mr: Perda de refrigerante no final da vida útil (10%) |
| 1282a6ec3cdd-ad5e-4aab-8774-44a61e8f1168 | Translation Approved (100%) | Mr: End-of-life Refrigerant Loss (10%) | Mr: Perda de refrigerante no final da vida útil (10%) |
| 128328d32667-b0be-4188-b84e-bf2b55e68176 | Translation Approved (CM) | Rc: Refrigerant Charge (0.5 to 5.0 lbs of refrigerant per ton of gross AHRI rated cooling capacity) | Rc: Carga de refrigerante (0,5 a 5,0 lb de refrigerante por tonelada de capacidade nominal bruta de resfriamento AHRI) |
| 1284314e080c-ded5-41d3-8813-3f57d76a3057 | Translation Approved (CM) | Rc: Refrigerant Charge (0.065 to 0.65 kg of refrigerant per kW of AHRI rated or Eurovent Certified cooling capacity) | Rc: Carga de refrigerante (0,065 a 0,65 kg de refrigerante por kW da capacidade nominal de resfriamento AHRI ou com certificação Eurovent) |
| 1285b5649f8b-3a2f-4e0e-a6ad-ee71588a9762 | Translation Approved (CM) | Life: Equipment Life (10 years; default based on equipment type, unless otherwise demonstrated) | Vida útil: Vida útil dos equipamentos (10 anos; padrão com base no tipo de equipamento, salvo de demonstrado de outra forma) |
| 1286953827d6-3d5a-441c-9bd8-8b64ae5b0db0 | Translation Approved (100%) | Life: Equipment Life (10 years; default based on equipment type, unless otherwise demonstrated) | Vida útil: Vida útil dos equipamentos (10 anos; padrão com base no tipo de equipamento, salvo de demonstrado de outra forma) |
| 1287e29ed1d2-0d47-4349-9255-8fbf30c6249b | Translation Approved (CM) | For multiple types of equipment, calculate a weighted average of all base building HVAC&R equipment, using the following formula: | Para vários tipos de equipamentos, calcule uma média ponderada de todos os equipamentos de HVAC&R do edifício base utilizando a seguinte fórmula: |
| 12887f1d6ce1-7cf8-4760-a27c-89d1f7c487fe | Translation Approved (100%) | IP units | Sistema imperial |
| 128921071112-bdbc-41ec-b347-d1d866dcf097 | Translation Approved (100%) | SI units | Sistema métrico |
| 1290d88f231a-5202-4228-8499-a9ac424c6fbc | Translation Approved (CM) | ∑ ( LCGWP + LCODP x 10<1487>5</1487> ) x Qunit | ∑ ( LCGWP + LCODP x 10<1487>5</1487> ) x Qunit |
| 1291543062d8-248d-4341-8edd-d00431d228ca | Translation Approved (0%) | ≤ 100 | ≤ 100 |
| 1292c166bb13-b507-4148-b842-0215ad2a7e01 | Translation Approved (100%) | ∑ ( LCGWP + LCODP x 10<1494>5</1494> ) x Qunit | ∑ ( LCGWP + LCODP x 10<1494>5</1494> ) x Qunit |
| 129329d4ca07-4c83-484d-871f-eed5b9743f76 | Translation Approved (100%) | ≤ 13 | ≤ 13 |
| 1294ba901afc-907c-44e6-8f22-ddd7ee26a4ce | Translation Approved (0%) | ——————————————————————————— | ——————————————————————————— |
| 1295fb2870c0-422a-4586-b7cf-4a55bb4d0a7d | Translation Approved (100%) | Qtotal | Qtotal |
| 1296e1e2ce92-58fb-4f7d-a098-3f981518f126 | Translation Approved (0%) | ——————————————————————— | ——————————————————————— |
| 129799416843-d929-4290-9717-1ec2681c613c | Translation Approved (100%) | Qtotal | Qtotal |
| 12988194d893-8035-40a5-b71f-67336ed56aef | Translation Approved (CM) | Calculation definitions for [ ∑ (LCGWP + LCODP x 10<1514>5</1514>) x Qunit ] / Qtotal ≤ 100 | Definições de cálculo para [ ∑ (LCGWP + LCODP x 10<1514>5</1514>) x Qunid ] / Qtotal ≤ 100 |
| 12992a00b82e-4858-4641-9b7f-8749acd3d127 | Translation Approved (100%) | (IP units) | (Sistema imperial) |
| 13008411aa40-ae51-4351-8530-6dfd76e7ea9f | Translation Approved (100%) | Calculation definitions for [ ∑ (LCGWP + LCODP x 10<1523>5</1523>) x Qunit ] / Qtotal ≤ 13 | Definições de cálculo para [ ∑ (LCGWP + LCODP x 10<1523>5</1523>) x Qunid ] / Qtotal ≤ 13 |
| 1301401dbeac-1b45-4b1f-aa4a-516096556244 | Translation Approved (100%) | (SI units) | (Sistema métrico) |
| 1302b0520f4e-c234-41ae-bea3-b46313662c66 | Translation Approved (98%) | Qunit = Gross ARI rated cooling capacity of an individual HVAC or refrigeration unit (Tons) | Qunid = capacidade nominal bruta de resfriamento ARI de uma unidade de HVAC ou refrigeração individual (toneladas) |
| 13032353b3ed-2cd4-461d-ba2a-1916413d3d9d | Translation Approved (CM) | Qunit = Eurovent Certified cooling capacity of an individual HVAC or refrigeration unit (kW) | Qunid = capacidade de resfriamento com Certificação Eurovent de uma unidade de HVAC ou refrigeração individual (kW) |
| 1304e6d48e07-91ba-4c27-8354-be616d40f042 | Translation Approved (98%) | Qtotal = Total gross ARI rated cooling capacity of all HVAC or refrigeration | Qtotal = capacidade nominal bruta total de resfriamento ARI de todas as unidades de HVAC ou refrigeração |
| 1305325aa781-0494-4ab5-b1ba-552a279409d2 | Translation Approved (CM) | Qtotal = Total Eurovent Certified cooling capacity of all HVAC or refrigeration (kW) | Qtotal = capacidade de resfriamento total com Certificação Eurovent de todas as unidades de HVAC ou refrigeração (kW) |
| 13064acf1679-28f5-4e49-9318-6685ff6dbcf6 | Translation Approved (0%) | Retail EBOM | EBOM Varejo (Retail) |
| 13077426e94b-8ab6-4339-b166-c9355cfdb99a | Translation Approved (100%) | Meet Option 1 or 2 for all HVAC systems. | Atenda à Opção 1 ou 2 para todos os sistemas de HVAC. |
| 1308c8cba394-7ecd-446a-91c1-432bc9613a68 | Translation Approved (CM) | Stores with commercial refrigeration systems must comply with the following. | Lojas com sistemas de refrigeração comercial devem cumprir o seguinte. |
| 13092c2526f3-8a54-416f-9aae-5447e7d8a816 | Translation Approved (CM) | Use only non-ozone-depleting refrigerants. | Use apenas refrigerantes que não destruam a camada de ozônio. |
| 1310ee1ca7cf-b3e3-43dc-875d-3626bc9bdccf | Translation Approved (87%) | Achieve an average HFC refrigerant charge of no more than 1.75 pounds of refrigerant per 1,000 Btu/h (2.72 grams of refrigerant per kW) total evaporator cooling load. | Alcance uma carga de refrigerante HFC média de no máximo 1,75 libra de refrigerante por 1.000 Btu/h (2,72 gramas de refrigerante por kW) de carga total de resfriamento do evaporador. |
| 1311da1a6810-6e8b-4e46-ac28-13445b4ae143 | Translation Approved (0%) | Achieve a store-wide annual refrigerant emissions rate of no more than 15%. | Alcance uma taxa de emissões de refrigerante anual para toda a loja de não mais de 15%. |
| 131207e88096-1a91-497e-a941-429fe0e0426b | Translation Approved (0%) | Alternatively, stores with commercial refrigeration systems may provide proof of attainment of EPA GreenChill’s silver-level store certification for fully operational food retail stores. | Ou, lojas com sistemas de refrigeração comercial podem fornecer prova de obtenção da certificação de loja de nível prata da GreenChill da EPA para lojas de varejo de alimentos em pleno funcionamento. |
| 13131f9694b9-ebea-4801-b40b-13e83f547022 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 13143605dcec-4b9b-407d-a928-fde73987ed2c | Translation Approved (100%) | None. | Nenhum. |
| 1315ac74774f-8c94-416c-8585-4a687a5f9847 | Translation Approved (100%) | Materials and Resources (MR) | Materiais e Recursos (Materials and Resources - MR) |
| 1316b05f8e07-9a99-4726-a891-d6c12ab77881 | Translation Approved (79%) | MR Prerequisite: Ongoing Purchasing and Waste Policy | Pré-requisito MR: Política de Compras Contínuas e Resíduos (MR Prerequisite: Ongoing Purchasing and Waste Policy) |
| 1317e819770e-121f-4686-bdbe-e2e5f95bb44f | Translation Approved (100%) | Required | Obrigatório |
| 13188f1f90d1-a478-4d37-b388-8c59dc6f3934 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 1319862877f8-0452-4340-a686-c97d1317aa5d | Translation Approved (100%) | This prerequisite applies to | Este pré-requisito se aplica a: |
| 1320b4ff1c63-7293-4fea-a8a1-998688987740 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) |
| 1321bd5bbe23-13e9-4352-bb7a-ce5a02a0653c | Translation Approved (100%) | EB:O&M Schools | EB:O&M Escolas (Schools) |
| 13225df8ce21-268f-4932-990f-79d29571334a | Translation Approved (100%) | EB:O&M Retail | EB:O&M Varejo (Retail) |
| 1323c64eaa96-d0a4-4001-9654-29f075abc074 | Translation Approved (100%) | EB:O&M Data Centers | EB:O&M Centros de Dados (Data Centers) |
| 132437d37f06-e86b-40f2-b7a5-9d46df831fa7 | Translation Approved (100%) | EB:O&M Hospitality | EB:O&M Hoteleiro (Hospitality) |
| 1325d9971a17-5373-4c94-9734-7e0e27974d95 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) |
| 13268efb6727-2ead-4dc9-a460-d17d50316032 | Translation Approved (100%) | Intent | Objetivo |
| 13271a36d972-3882-4869-9661-2a1053ac026d | Translation Approved (0%) | To reduce the environmental harm from materials purchased, used, and disposed of in the operations within buildings. | Reduzir o dano ambiental de materiais comprados, usados e descartados nas operações dentro dos edifícios. |
| 1328aa6a60d5-23d4-4c7b-a507-11d5230921f8 | Translation Approved (100%) | Requirements | Requisitos |
| 13295ab9979c-1f70-40ca-ab56-bac2590c0d48 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 13306a994c7c-1f68-448b-8a4e-3977f257ce24 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 13312aabefc3-b633-4092-8b11-bbb6560a8587 | Translation Approved (77%) | Environmentally Preferable Purchasing | Aquisição ecologicamente responsável |
| 133206de4323-e9d3-4c80-9dea-a42008bb2bce | Translation Approved (0%) | Have in place an environmentally preferable purchasing (EPP) policy for products purchased during regular operations of the building. | Implemente uma polícia de aquisição ecologicamente responsável (EPP) para produtos comprados durante as operações regulares do edifício. |
| 133306de4323-e9d3-4c80-9dea-a42008bb2bce | Translation Approved (100%) | Include at a minimum: | Inclua no mínimo: |
| 1334357b361b-da88-4b6d-86a7-2c2b76a20c11 | Translation Approved (0%) | Ongoing Purchases | Compras contínuas |
| 1335a6a9922a-4594-4bdf-96b1-29f5673feef4 | Translation Approved (0%) | The five most purchased product categories based on total annual purchases. | As cinco categorias de produtos mais comprados com base nas compras totais anuais. |
| 1336fc063226-046a-43fc-8885-ab291780fcde | Translation Approved (81%) | Paper, toner cartridges, binders, batteries, and desk accessories. | Papel, cartuchos de toner, pastas, pilhas, baterias e acessórios de mesa. |
| 133758711cfe-fd66-44b5-9592-c7db90549321 | Translation Approved (0%) | Lamps (indoor and outdoor, hard-wired and portable fixtures) | Lâmpadas (dispositivos cabeados e portáteis internos ou externos) |
| 1338f4ceb01c-8e3f-41f7-81f0-0325b787fd2e | Translation Approved (0%) | Food (required for Schools and Hospitality only) | Alimentos (necessário somente para Escolas e Hoteleiro) |
| 1339cf3f5232-4d30-4076-8080-d404fe4dde12 | Translation Approved (70%) | Durable Goods Purchases | Compras de bens duráveis |
| 1340c62facbf-2489-4b86-b503-e56eb2b70750 | Translation Approved (0%) | Office equipment, appliances, and audiovisual equipment | Equipamentos para escritório, utensílios e equipamentos audiovisuais |
| 13414073f8c7-6931-4e81-baf9-4e4966754c34 | Translation Approved (0%) | Electric powered equipment | Equipamentos elétricos |
| 13427e410362-bdf5-4d4b-b268-756a6c63b6d7 | Translation Approved (0%) | The policy should address the criteria in the following credits: | A política deve abordar os critérios nos seguintes créditos: |
| 1343786ec606-912c-487c-9f9d-aba354f633ef | Translation Approved (0%) | Materials and Resources Credit: Purchasing—Ongoing | Crédito de Materiais de Recursos: Compras - Contínuas (Materials and Resources Credit: Purchasing—Ongoing) |
| 1344f4d0ac5a-e9c8-47d8-ac95-0f9453510664 | Translation Approved (84%) | Materials and Resources Credit: Purchasing—Lamps | Crédito de Materiais de Recursos: Compras - Lâmpadas (Materials and Resources Credit: Purchasing—Lamps) |
| 1345a7cf6591-9846-4c45-957a-dd3bf035a795 | Translation Approved (99%) | The policy must cover at least those product purchases within the building and site management’s control. | A política deve cobrir pelo menos essas compras de produto conforme o controle de gestão do terreno e edifício. |
| 1346f6422d50-47d2-49c5-bbdf-e6be8a50b691 | Translation Approved (100%) | Solid Waste Management | Gestão de resíduos sólidos |
| 1347bcf97047-09a3-4260-942d-af46d0995c8b | Translation Approved (0%) | Establish storage locations for recyclable materials, including mixed paper, corrugated cardboard, glass, plastics, and metals. | Estabeleça locais de armazenamento para materiais reciclados incluindo tipos variados de papel, papelão ondulado, vidro, plástico e metal. |
| 1348bcf97047-09a3-4260-942d-af46d0995c8b | Translation Approved (0%) | Establish safe storage areas for batteries and mercury-containing lamps. | Estabeleça áreas de armazenamento seguro para baterias e lâmpadas que contêm mercúrio. |
| 13491d0b2ddd-720a-43fa-ad94-1ffc49d4bd43 | Translation Approved (0%) | Have in place an environmentally preferable solid waste management policy that addresses reuse, recycling, or composting of products purchased during regular operations of the building. | Implemente uma política de gestão de resíduos sólidos ecologicamente responsável que aborda o reuso, a reciclagem, ou a compostagem de produtos comprados durante as operações regulares do edifício. |
| 13501d0b2ddd-720a-43fa-ad94-1ffc49d4bd43 | Translation Approved (100%) | Include at a minimum: | Inclua no mínimo: |
| 1351c3fef006-2ca1-45ff-83c0-2f94773fa98b | Translation Approved (0%) | Ongoing waste | Resíduos contínuos |
| 13523f97bec6-3087-4f87-9220-dd9a39130857 | Translation Approved (100%) | The five most purchased product categories based on total annual purchases. | As cinco categorias de produtos mais comprados com base nas compras totais anuais. |
| 1353f844d3cf-07e6-455b-b52a-055cb907c173 | Translation Approved (91%) | Food (required for EBOM Schools and Hospitality only) | Alimentos (necessário somente para EBOM Escolas e Hoteleiro) |
| 135476cd4b1d-d88f-4070-828d-66ea67673490 | Translation Approved (76%) | Durable goods waste | Resíduos de bens duráveis |
| 135582a481f0-2f24-4be8-92c7-3ea8cd6f93a1 | Translation Approved (100%) | Office equipment, appliances, and audiovisual equipment | Equipamentos para escritório, utensílios e equipamentos audiovisuais |
| 1356ba8cef6c-e969-4d23-84d2-d5a193044ae5 | Translation Approved (100%) | Electric powered equipment | Equipamentos elétricos |
| 1357d87d02dd-4006-423d-bd29-84ebfc9205cd | Translation Approved (0%) | Hazardous waste | Resíduos nocivos |
| 13582a146e97-95aa-417d-82c0-1bf894c0b9dc | Translation Approved (0%) | Safe disposal of batteries and lamps (indoor and outdoor, hard-wired and portable fixtures) | Descarte seguro de baterias e lâmpadas (dispositivos cabeados e portáteis internos ou externos) |
| 1359525679ce-57ce-4971-82a8-e92ed5dfaa3b | Translation Approved (100%) | The policy must cover at least those product purchases within the building and site management’s control. | A política deve cobrir pelo menos essas compras de produto conforme o controle de gestão do terreno e edifício. |
| 13604b90efc7-767e-4f54-8778-5c6667190a88 | Translation Approved (100%) | Retail | Varejo (Retail) |
| 1361dde0da31-03b1-424a-92d1-1a8bc57d20dd | Translation Approved (0%) | In addition to the requirements above, retail projects should promote environmentally responsible sourcing of retail merchandise through one of the four following options. | Além dos requisitos acima, os projetos de varejo devem promover o fornecimento ecologicamente responsável de mercadorias de varejo por meio de uma das quatro opções a seguir. |
| 1362d934f0e7-66a3-4e6f-a9ac-298ecdaf1fc1 | Translation Approved (0%) | <1550>O</1550><1551>ption<1552> 1.</1552></1551> | <1550>O</1550><1551>pção<1552> 1.</1552></1551> |
| 1363d934f0e7-66a3-4e6f-a9ac-298ecdaf1fc1 | Translation Approved (0%) | Supply Chain Survey | Pesquisa de cadeia de fornecimento |
| 136423b6c3cb-3486-470f-98c4-22d38ab99354 | Translation Approved (0%) | Establish a supply chain survey. | Estabeleça uma pesquisa de cadeia de fornecimento. |
| 1365909d97bc-6a01-412b-91a1-4e563fdd9cd7 | Translation Approved (0%) | The survey should collect information from each supplier regarding the following: | A pesquisa deve coletar informações de cada fornecedor em relação ao seguinte: |
| 1366f8f7ffdf-9273-4301-a82b-932351d21f67 | Translation Approved (0%) | social equity practices; | práticas de igualdade social; |
| 1367b460f1ca-0e17-4d2c-b1d5-5b3dbbe349ca | Translation Approved (0%) | energy and carbon reduction measures; | medidas de redução de energia e carbono; |
| 1368bc600ded-f3ad-4e6b-a506-63115569b5f3 | Translation Approved (0%) | material selection practices for products, packaging, and distribution; | praticas de seleção de material para produtos, embalagens e distribuição; |
| 13699139929c-d64d-4cf6-9053-96a92e520922 | Translation Approved (0%) | waste reduction and waste management measures; and | medidas de redução de resíduos e gestão de resíduos; e |
| 137081282d65-4330-4be3-b68b-9b9c7e4ed2dc | Translation Approved (0%) | human health protection measures. | medidas de proteção da saúde humana. |
| 1371b957d1d4-e63c-43ca-9610-26128784be49 | Translation Approved (100%) | OR | OU |
| 1372f0c185f3-04ea-4270-be19-e965084592e4 | Translation Approved (100%) | <1558>O</1558><1559>ption<1560> 2.</1560></1559> | <1558>O</1558><1559>pção<1560> 2.</1560></1559> |
| 1373f0c185f3-04ea-4270-be19-e965084592e4 | Translation Approved (0%) | Supply Chain Education Program for Retail Employees and/or Retail Tenant Representatives | Programa educacional da cadeia de fornecimento para funcionários de varejo e/ou representantes de locatários de varejo |
| 1374e8bdc04d-ed5f-4d5f-a753-1951772d4955 | Translation Approved (0%) | Establish a program to inform employees and tenants engaged in merchandise purchasing, packaging, and distribution about environmentally preferable supply chain strategies. | Estabeleça um programa para informar ao funcionários e locatários envolvidos na compra, embalagem e distribuição de mercadorias sobre as estratégias de cadeia de fornecimento ecologicamente responsáveis. |
| 137577289c23-56fd-4029-a43f-3e880237db99 | Translation Approved (0%) | Include the following in the education program: | Inclua o seguinte no programa educacional: |
| 1376c9852f03-3f3d-44e0-9c1b-2b5ca3e8f14a | Translation Approved (0%) | environmental best practices for supply chain decisions; | melhores práticas ambientais para decisões de cadeia de fornecimento; |
| 1377cd59d326-dd07-4ec4-bb48-6deb3a52b6e9 | Translation Approved (0%) | resources for additional information; and | recursos para informações adicionais; e |
| 1378843a8742-2813-4431-a6a0-f40d6749d43c | Translation Approved (0%) | internal contacts for more information. | contatos internos para mais informações. |
| 13792ab880d4-f16b-4a12-93aa-e3552abd76af | Translation Approved (100%) | OR | OU |
| 13804f7c638e-15f2-442f-a708-0760b1297ade | Translation Approved (100%) | <1564>O</1564><1565>ption<1566> 3.</1566></1565> | <1564>O</1564><1565>pção<1566> 3.</1566></1565> |
| 13814f7c638e-15f2-442f-a708-0760b1297ade | Translation Approved (0%) | Supply Chain Environmental Criteria List | Lista de critérios ambientais para cadeias de fornecimento |
| 13822333af62-c630-4920-8dbc-a78dcca96f27 | Translation Approved (0%) | Establish criteria for retail products encouraging an environmentally preferable supply chain strategy addressing the following areas: | Estabeleça critérios para produtos de varejo incentivando uma estratégia de cadeia de fornecimento ecologicamente responsável abordando as seguintes áreas: |
| 1383e4f750ea-1564-4db8-83ea-8ec2da74d63e | Translation Approved (0%) | purchasing; | compras; |
| 1384df24d3c5-ec95-41e5-8a25-29fd7f3522fa | Translation Approved (0%) | materials handling and packaging; | manuseio e embalagem de materiais; |
| 1385d8296d9d-06a1-4f94-9518-2580165666bf | Translation Approved (0%) | inventory; | inventário; |
| 13863f2bdeea-0245-4fb9-ba0e-f8f272c21dec | Translation Approved (0%) | materials recovery during manufacturing; | recuperação de materiais durante a fabricação; |
| 13877a60004d-3a48-47ca-b4cd-a8b95ca2099a | Translation Approved (0%) | waste disposal; and | descarte de resíduos; e |
| 1388a2fcedde-ac24-4447-a17b-37f419ef1e53 | Translation Approved (0%) | product take-back. | coleta de produto. |
| 13897d943a5a-b290-4321-ba49-ee2e71dc75fe | Translation Approved (100%) | OR | OU |
| 1390bf65a932-48f9-4e03-811a-026ab5a5edb1 | Translation Approved (100%) | Option 4. | Opção 4. |
| 1391bf65a932-48f9-4e03-811a-026ab5a5edb1 | Translation Approved (0%) | Sustainable Purchasing Education for Customers | Educação sobre compras sustentáveis para clientes |
| 139254928f75-54c7-4e33-8b58-5ba72952230f | Translation Approved (0%) | Install an educational program display for customers displaying environmental initiatives that the store has implemented. | Instale uma exibição de programa educacional para clientes mostrando as iniciativas ambientais que a loja implementou. |
| 139354928f75-54c7-4e33-8b58-5ba72952230f | Translation Approved (0%) | The educational display should incorporate information that includes the supply chain environmental criteria listed in Option 3. | A exibição educacional deve incorporar as informações que incluem os critérios ambientais para cadeias de fornecimento listados na Opção 3. |
| 1394de6f26df-87f6-450c-b1d0-6d91657e26a5 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 139529d616d2-660b-4b0a-8c5f-48b05c596577 | Translation Approved (0%) | Maintain a high-performing solid waste management program by conducting a waste stream audit of ongoing consumables at least once every five years or by diverting 75% of ongoing waste and achieving Materials and Resources Credit Solid Waste Management—Ongoing. | Mantenha um programa de gestão de resíduos sólidos de alto desempenho conduzindo uma audição de fluxo de resíduos de materiais de consumo contínuo pelo menos uma vez a cada cinco anos ou desviando 75% dos resíduos contínuos e alcançando o Crédito Materiais e Recursos, Gestão de Resíduos Sólidos - Contínua (Materials and Resources, Solid Waste Management—Ongoing) |
| 139616186f10-9ffc-42d7-a2e0-3dae5b5053af | Translation Approved (78%) | MR Prerequisite: Facility Maintenance and Renovation Policy | Pré-requisito MR: Política de Manutenção e Reforma de Instalações (MR Prerequisite: Facility Maintenance and Renovation Policy) |
| 1397885cd88b-4349-4b27-a5e5-10ba6ba8489e | Translation Approved (100%) | Required | Obrigatório |
| 1398e4fbe173-bc9a-4709-bf88-90a311bb8ddc | Translation Approved (100%) | EB:O&M | EB:O&M |
| 1399b0476610-6f9a-44d5-94ec-58405859ecd8 | Translation Approved (100%) | This prerequisite applies to | Este pré-requisito se aplica a: |
| 1400ed07c132-b9b6-4fa0-b6f5-62bcde1259e5 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) |
| 14017c39db10-44d8-4f31-9fb8-40acdd2d9436 | Translation Approved (100%) | EB:O&M Schools | EB:O&M Escolas (Schools) |
| 14020372178e-fbc4-49bb-89d1-3745a40e4922 | Translation Approved (100%) | EB:O&M Retail | EB:O&M Varejo (Retail) |
| 140351f0c3c1-ecb1-4ed2-89e4-b9cc3dd90bb1 | Translation Approved (100%) | EB:O&M Data Centers | EB:O&M Centros de Dados (Data Centers) |
| 1404337fcf1c-3ba5-4fc5-86ca-c156c6673eb1 | Translation Approved (100%) | EB:O&M Hospitality | EB:O&M Hoteleiro (Hospitality) |
| 140506b674be-0978-4df7-9d18-647892334612 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) |
| 1406efc46a7e-1c25-4290-a0ed-f060b86a7ef2 | Translation Approved (100%) | Intent | Objetivo |
| 14078e0b284f-f881-4abd-a849-58e3d6712be4 | Translation Approved (0%) | To reduce the environmental harms associated with the materials purchased, installed, and disposed of during maintenance and renovation of buildings. | Reduzir os danos ambientais associados com os materiais comprados, instalados e descartados durante a manutenção e reforma de edifícios. |
| 1408690936f5-0460-4142-98bb-047d7ff99219 | Translation Approved (100%) | Requirements | Requisitos |
| 140987d3aa46-a472-4136-99d4-f453d6ee8d61 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1410fa834df5-5022-4b45-8e4b-e60a6a70db28 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 1411f3bce514-adfc-4363-8903-f9591cfc3933 | Translation Approved (0%) | Have in place a facility maintenance and renovation policy that includes guidelines for renovation and maintenance activities, using LEED rating system strategies, to be implemented at the discretion of building owners, operators, or tenants. | Implemente uma política de manutenção reforma da instalação que inclua as diretrizes para as atividades de reforma e manutenção, usando estratégias do sistema de classificação LEED, a serem implementadas a critério dos proprietários, operadores ou locatários do edifício. |
| 1412f3bce514-adfc-4363-8903-f9591cfc3933 | Translation Approved (0%) | Renovation activities include building improvements and tenant fit-outs. | As atividades de reforma incluem melhorias no edifícios e modernizações realizadas pelos locatários. |
| 1413f3bce514-adfc-4363-8903-f9591cfc3933 | Translation Approved (0%) | Maintenance activities include general repair and replacement. | As atividades de manutenção incluem reparos e substituições em geral. |
| 1414c02bebff-64be-4766-98c4-11ff48d6d82c | Translation Approved (100%) | The policy must cover at least those product purchases within the building and site management’s control. | A política deve cobrir pelo menos essas compras de produto conforme o controle de gestão do terreno e edifício. |
| 1415c02bebff-64be-4766-98c4-11ff48d6d82c | Translation Approved (0%) | The policy must address purchasing, waste management and indoor air quality. | A política deve abordar a compra, a gestão de resíduos e a qualidade do ar interno. |
| 1416d77dcb8f-924d-4259-b412-ef2cf1f17d45 | Translation Approved (0%) | Purchasing Policy for Maintenance and Renovations | Política de compra para manutenção e reformas |
| 1417f5fb1f75-b483-4fc6-b428-d07499ad6f57 | Translation Approved (0%) | Have in place a purchasing policy for product and materials purchased for facility maintenance and renovation activities. | Implemente uma política de compra para produtos e materiais comprados para atividades de manutenção e reforma da instalação. |
| 1418f5fb1f75-b483-4fc6-b428-d07499ad6f57 | Translation Approved (100%) | Include at a minimum: | Inclua no mínimo: |
| 14192f324ddc-d171-40a0-81e5-30ef5614039a | Translation Approved (0%) | Base building elements permanently or semi- permanently attached to the building (mechanical, electrical and plumbing components and specialty items such as elevators are excluded). | Elementos do edifício base fixados de forma permanente ou semipermanente ao edifício (componentes mecânicos, elétricos e hidráulicos e itens especiais, como elevadores, estão excluídos). |
| 14202f324ddc-d171-40a0-81e5-30ef5614039a | Translation Approved (83%) | Exclude fixtures, and equipment, which are not considered base building elements; | Exclua dispositivos e equipamentos que não são considerados elementos do edifício base; |
| 1421f43861bb-32b6-4102-9048-8ec92290a241 | Translation Approved (0%) | Furniture and furnishings as well as components and parts needed to maintain them | Móveis e acessórios, bem como componentes e peças necessários para mantê-los |
| 1422b944c382-48d5-425f-af34-5a314b15d30a | Translation Approved (100%) | The policy should address the criteria in the following credits: | A política deve abordar os critérios nos seguintes créditos: |
| 14237db46fa1-e7e9-4473-b101-c2c0870d2b4b | Translation Approved (79%) | MR Credit: Purchasing—Facility Maintenance and Renovation | Crédito MR: Compras - Manutenção e Reforma de Instalações (MR Credit: Purchasing—Facility Maintenance and Renovation) |
| 1424c6034ced-2bcd-4adc-a8c1-e1ceae5a1c00 | Translation Approved (76%) | Waste Management Policy for Maintenance and Renovations | Política de gestão de resíduos para manutenção e reformas |
| 1425176b4e4c-72f6-4594-a930-1bc47466838a | Translation Approved (0%) | Have in place a waste management policy addressing the following: | Implemente uma política de gestão de resíduos abordando o seguinte: |
| 14262442582e-233d-472b-9575-910cc8673611 | Translation Approved (0%) | <1584>Facility maintenance waste</1584>. | <1584>Resíduos de manutenção da instalação</1584>. |
| 14272442582e-233d-472b-9575-910cc8673611 | Translation Approved (0%) | The policy should address safe storage and recycling and diversion of waste associated with maintenance activities. | A política deve abordar o armazenamento seguro e a reciclagem e desvio de resíduos associados às atividades de manutenção. |
| 14285f64a2fd-6a63-4cc2-b7cc-97f30de53757 | Translation Approved (0%) | <1585>Renovation waste</1585>. | <1585>Resíduos de reforma</1585>. |
| 14295f64a2fd-6a63-4cc2-b7cc-97f30de53757 | Translation Approved (0%) | The policy should describe the procedure for creating an individual plan for each renovation project. | A política deve descrever o procedimento para criar um plano individual para cada projeto de reforma. |
| 14305f64a2fd-6a63-4cc2-b7cc-97f30de53757 | Translation Approved (0%) | Each renovation project should establish waste diversion goals, target five materials for diversion, approximate the volume of waste anticipated, and identify waste diversion strategies to be used. | Cada projeto de reforma deve estabelecer metas de desvio de resíduos, definir cinco materiais para o desvio, aproximar o volume de resíduo previsto e identificar as estratégias de desvio de resíduos a serem usadas. |
| 14315e1aa4c6-4e7b-4472-9f8a-2041e04d5319 | Translation Approved (0%) | Indoor Air Quality Policy for Maintenance and Renovations | Política de qualidade do ar interno para manutenção e reformas |
| 1432cb11efeb-2e5b-455d-b8a3-2232baae7b33 | Translation Approved (0%) | Have in place an indoor air quality policy for facility maintenance and renovation activities addressing the criteria below. | Implemente uma política de qualidade do ar interno para as atividades de manutenção e reforma da instalação abordando os critérios abaixo. |
| 1433cb11efeb-2e5b-455d-b8a3-2232baae7b33 | Translation Approved (0%) | For maintenance activities implement the policy as applicable. | Para atividades de manutenção, implemente a política conforme aplicável. |
| 1434cb11efeb-2e5b-455d-b8a3-2232baae7b33 | Translation Approved (0%) | For renovation activities create an individual plan for each renovation project as outlined in the policy. | Para as atividades de reforma, crie um plano individual para cada projeto de reforma conforme descrito na política. |
| 1435aecdd231-64d1-48c7-872a-a1fbc9fd19ca | Translation Approved (87%) | Follow the recommended control measures of the Sheet Metal and Air Conditioning National Contractors Association (SMACNA) IAQ Guidelines for Occupied Buildings under Construction, 2nd edition (2007), ANSI/SMACNA 008–2008, Chapter 3 | Siga as medidas de controle recomendadas da Associação Nacional de Empreiteiros de Ar-condicionado e Serralheria (Sheet Metal and Air Conditioning Contractors National Association - SMACNA), Diretrizes de QAI para Edifícios Ocupados (IAQ Guidelines for Occupied Buildings) em Construction (Construção), 2ª edição, 2007, ANSI/SMACNA 008–2008, Capítulo 3. |
| 1436724befbd-c293-402d-9e06-433c49daf247 | Translation Approved (94%) | Protect stored on-site and installed absorptive materials from moisture damage. | Proteja os materiais de absorção armazenados no local (in loco) e instalados contra danos provocados pela umidade. |
| 1437e4c8ebc9-aa1a-46f5-b6b5-66a4bcfbdde0 | Translation Approved (98%) | Do not operate permanently-installed air handling equipment during construction unless filtration media with a minimum efficiency reporting value (MERV) of 8, as determined by ASHRAE 52.2–2007, with errata (or equivalent filtration media class of F5 or higher, as defined by CEN Standard EN 779–2002, Particulate Air Filters for General Ventilation, Determination of the Filtration Performance), are installed at each return air grille and return or transfer duct inlet opening such that there is no bypass around the filtration media. | Não opere equipamentos de manipulação de ar instalados permanentemente durante a construção, a não ser que um meio filtrante com valor mínimo de eficiência relatada (minimum efficiency reporting value – MERV) de 8, conforme determinado pela Norma ASHRAE 52.2–2007, com errata (ou meio filtrante equivalente da classe F5 ou superior, conforme definido pela Norma EN 779–2002 da CEN, Filtros de particulado no ar para ventilação geral, Determinação do desempenho de filtração (Particulate Air Filters for General Ventilation, Determination of the Filtration Performance)), seja instalado em cada grelha de retorno de ar ou abertura de entrada de duto de transferência de forma que não haja como desviar o meio filtrante. |
| 1438f62c0b34-e499-45c7-a6e0-7a7f7d8ee1e1 | Translation Approved (0%) | Develop a procedure to, before occupancy, replace all filtration media with the final design filtration media. | Desenvolva um procedimento para, após a ocupação, substituir todos os meios filtrantes com meio filtrante do projeto final. |
| 14395a45b591-5de1-4bbf-99f4-66d9961706c2 | Translation Approved (0%) | Develop a plan to determine whether a flush-out or air quality testing is needed after construction ends and all interior finishes are installed but before occupancy. | Desenvolva um plano para determinar se um teste de lavagem ou qualidade do ar é necessário após o término da construção e todos os acabamentos interiores sejam instalados antes da ocupação. |
| 144032bc921e-0cfa-4da4-b0d1-a06add4ba903 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 144134d9c108-c1c9-486e-a8bb-f83a8b6aa4b3 | Translation Approved (100%) | None. | Nenhum. |
| 14421d861679-1366-4dad-8faf-00fcea82c28c | Translation Approved (76%) | MR Credit: Purchasing—Ongoing | Crédito MR: Compras - Contínuas (MR Credit: Purchasing—Ongoing) |
| 1443e3c62266-adf7-491d-b6ec-e5c0f2905b3e | Translation Approved (100%) | EB:O&M | EB:O&M |
| 144446f3a291-f670-4272-b5c2-1532366fc196 | Translation Approved (100%) | 1 point | 1 ponto |
| 1445ea07b784-7ca4-42c4-8aa9-199941705add | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 1446f3c05798-69fe-46e0-b19f-bb25dc2b00aa | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1 point) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 ponto) |
| 1447f513d665-9a4d-44f4-b882-3a18d1d6ed53 | Translation Approved (100%) | EB:O&M Schools (1 point) | EB:O&M Escolas (Schools) (1 ponto) |
| 14489cb27df6-77ef-4d0f-957a-38d842903c90 | Translation Approved (100%) | EB:O&M Retail (1 point) | EB:O&M Varejo (Retail) (1 ponto) |
| 14493ce4bb73-a3e6-4cba-98b6-1bf29b1a8da0 | Translation Approved (100%) | EB:O&M Data Centers (1 point) | EB:O&M Centros de Dados (Data Centers) (1 ponto) |
| 1450dce99e98-ebdd-4965-ad3b-e8f7b76f1ba1 | Translation Approved (100%) | EB:O&M Hospitality (1 point) | EB:O&M Hoteleiro (Hospitality) (1 ponto) |
| 1451f69617f9-496b-442e-9c51-517d0c1e9906 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1 point) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 ponto) |
| 1452820a37f8-4055-4243-9479-2dfd5752d183 | Translation Approved (100%) | Intent | Objetivo |
| 145324e03b56-514f-450f-b884-ab340f901fcc | Translation Approved (0%) | To reduce environmental harm from materials used in the operations and maintenance of buildings. | Reduzir o dano ambiental de materiais usados nas operações e manutenção de edifícios. |
| 1454dcd1fc22-ce72-46a1-965c-93727054481d | Translation Approved (100%) | Requirements | Requisitos |
| 14553b1099ea-a6b1-44a4-a4e0-edb7e3c30def | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1456de7e5aa0-80c8-4c73-8f70-2706e13e9714 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 14574d1a4b6a-c77d-45d2-8f76-3be7c214053a | Translation Approved (100%) | None. | Nenhum. |
| 1458b1590146-68aa-4824-ad00-191dde8ab5c8 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 145915591638-059a-4dd5-aca8-e4472b0ab463 | Translation Approved (100%) | Ongoing Consumables | Materiais de consumo contínuo (Solid Waste Management—Ongoing Consumables) |
| 1460eddd2085-4310-4a81-846e-404b02b431e9 | Translation Approved (0%) | Purchase at least 60%, by cost, of total ongoing consumables that meet at least one of the following criteria. | Compre no mínimo 60%, por custo, do total de materiais de consumo continuo que atendam pelo menos um dos critérios a seguir. |
| 1461eddd2085-4310-4a81-846e-404b02b431e9 | Translation Approved (0%) | Include the product categories identified in the Materials and Resources prerequisite: Ongoing Purchasing and Waste Policy. | Inclua as categorias de produto identificadas no Pré-requisito MR: Política de Compras Contínuas e Resíduos (MR Prerequisite: Ongoing Purchasing and Waste Policy) |
| 1462eddd2085-4310-4a81-846e-404b02b431e9 | Translation Approved (72%) | Lamps are excluded from the calculation. | As lâmpadas devem ser excluídas do cálculo. |
| 1463eddd2085-4310-4a81-846e-404b02b431e9 | Translation Approved (0%) | Each purchase can receive credit for each criterion met. | Cada compra pode receber crédito para cada critério atendido. |
| 1464f2e84661-3e34-4497-9f51-fd7ac885796d | Translation Approved (99%) | Postconsumer recycled content. | Conteúdo reciclado pós-consumo. |
| 1465f2e84661-3e34-4497-9f51-fd7ac885796d | Translation Approved (0%) | The content of purchases must meet or exceed the levels listed in the U.S. Environmental Protection Agency Comprehensive Procurement Guidelines. | O conteúdo de comprar deve atender ou exceder os níveis listados nas Diretrizes abrangentes de aquisição da Agência de Proteção Ambiental dos EUA. |
| 1466f2e84661-3e34-4497-9f51-fd7ac885796d | Translation Approved (0%) | Products not covered by the Guidelines can get credit for their recycled content with no minimum. | Os produtos não cobertos pelas Diretrizes podem receber crédito pelo seu conteúdo reciclado sem nenhum mínimo. |
| 146732a3f33f-e32d-43bc-be88-74322d00dd61 | Translation Approved (0%) | Extended use. | Uso estendido. |
| 146832a3f33f-e32d-43bc-be88-74322d00dd61 | Translation Approved (0%) | Batteries must be rechargeable. | As baterias devem ser recarregáveis. |
| 146932a3f33f-e32d-43bc-be88-74322d00dd61 | Translation Approved (0%) | Toner cartridges for laser printers must be remanufactured. | Os cartuchos de toner para impressoras a laser devem ser remanufaturados. |
| 1470fb02a15e-6b2e-4aa1-8bb4-dcb455380f17 | Translation Approved (0%) | Sustainable agriculture. | Agricultura sustentável. |
| 1471fb02a15e-6b2e-4aa1-8bb4-dcb455380f17 | Translation Approved (0%) | Food and beverages must be labeled USDA Organic, Food Alliance Certified, Rainforest Alliance Certified, Protected Harvest Certified, Fair Trade, or Marine Stewardship Council’s Blue Eco-Label, Canada Organic logo in accordance with Organic Products Regulations (SOR/2009-176) or labeled with the European Community Organic Production logo in accordance with Regulations (EC) No. 834/2007 and (EC) No. 889/2008. | Alimentos e bebidas devem receber o selo orgânico do USDA (USDA Organic), a certificação Food Alliance, a certificação Rainforest Alliance, a certificação Protected Harvest, Fair Trade ou o Selo ecológico azul do Marine Stewardship Council, o logotipo orgânico Canada Organic de acordo com as Normas de produtos orgânicos (Organic Products Regulations - SOR/2009-176) ou receber o selo do logo de Produção orgânica da comunidade europeia (European Community Organic Production) de acordo com as Normas (UE) Nº 834/2007 e (UE) Nº 889/2008. |
| 147235f1c0c1-bdca-4933-93a1-1ab1d497b7ab | Translation Approved (0%) | Local sourcing of food and beverages. | Fornecimento local de alimentos e bebidas. |
| 147335f1c0c1-bdca-4933-93a1-1ab1d497b7ab | Translation Approved (0%) | The food or beverage must contain raw materials harvested and produced within 100 miles (160 kilometers) of the site. | Os alimentos e bebidas devem conter matérias-primas colhidas e produzidas em uma distância de até 100 milhas (160 quilômetros) do terreno. |
| 147470e6e195-6635-4919-a155-db07a6c965fe | Translation Approved (89%) | Bio-based materials. | Materiais biodegradáveis. |
| 147570e6e195-6635-4919-a155-db07a6c965fe | Translation Approved (CM) | Bio-based products must meet the Sustainable Agriculture Network’s Sustainable Agriculture Standard. | Produtos biodegradáveis devem atender à Norma para Agricultura Sustentável da Rede de Agricultura Sustentável (Sustainable Agriculture Network’s Sustainable Agriculture Standard). |
| 147670e6e195-6635-4919-a155-db07a6c965fe | Translation Approved (CM) | Bio-based raw materials must be tested using ASTM Test Method D6866 and be legally harvested, as defined by the exporting and receiving country. | Matérias-primas biodegradáveis devem ser testadas utilizando o Método de Teste ASTM D6866 e serem coletadas de acordo com as leis dos países de exportação e destino. |
| 147770e6e195-6635-4919-a155-db07a6c965fe | Translation Approved (CM) | Exclude hide products, such as leather and other animal skin material. | Exclua produtos de couro cru, como couro e outras peles de animais. |
| 147801ce1fc5-9e1e-4af1-a62d-26e7038bca1c | Translation Approved (0%) | Paper and wood products. | Papel e produtos de madeira. |
| 147901ce1fc5-9e1e-4af1-a62d-26e7038bca1c | Translation Approved (89%) | Paper and wood products must be certified by the Forest Stewardship Council or USGBC-approved equivalent. | Produtos de papel e madeira devem ser certificados pelo Conselho de Manejo Florestal (Forest Stewardship Council - FSC) ou equivalente aprovado pelo USGBC. |
| 14802294bf6f-3211-4523-a92f-e32b89cbeca9 | Translation Approved (100%) | Electric-Powered Equipment | Equipamentos elétricos |
| 1481f12cf5d1-5024-4d90-acdd-80ac73bceb65 | Translation Approved (87%) | Purchase at least 40%, by cost, electric-powered equipment that meets at least one of the following criteria. | Compre no mínimo 40%, por custo, equipamentos elétricos que atendem pelo menos um dos critérios a seguir. |
| 1482f12cf5d1-5024-4d90-acdd-80ac73bceb65 | Translation Approved (86%) | Include product categories specified in Materials and Resources prerequisite: Ongoing Purchasing and Waste Policy. | Inclua as categorias de produto especificadas no Pré-requisito MR: Política de Compras Contínuas e Resíduos (MR Prerequisite: Ongoing Purchasing and Waste Policy) |
| 1483f12cf5d1-5024-4d90-acdd-80ac73bceb65 | Translation Approved (0%) | In addition, create a phase-out plan to replace remaining products with compliant equipment at the end of their useful life. | Além disso, criar um plano de substituição para substituir os produtos restantes com equipamentos em conformidade no final de sua vida útil. |
| 14849d33c242-444f-4846-b2e0-11ad6c328a0f | Translation Approved (82%) | EPEAT rating. | Classificação EPEAT. |
| 14859d33c242-444f-4846-b2e0-11ad6c328a0f | Translation Approved (0%) | The equipment must have a silver Electronic Product Environmental Assessment Tool (EPEAT) rating or better. | Os equipamentos devem ter uma classificação prata ou superior da Electronic Product Environmental Assessment Tool (EPEAT). |
| 14865c3991ca-5995-4cf8-9760-48f2c8401b96 | Translation Approved (99%) | ENERGY STAR rating. | Classificação ENERGY STAR. |
| 14875c3991ca-5995-4cf8-9760-48f2c8401b96 | Translation Approved (0%) | If the equipment does not yet fall under the EPEAT rating systems, it must be ENERGY STAR® qualified or performance equivalent for projects outside the U.S. | Se o equipamento não cair nos sistemas de classificação EPEAT, deve ter a qualificação ENERGY STAR® ou desempenho equivalente para projetos fora dos EUA. |
| 1488a1ceaa0b-ea1c-4b4c-b33b-f2cd847ad502 | Translation Approved (0%) | Schools and Hospitality only | Apenas Escolas (Schools) e Hoteleiro (Hospitality) |
| 1489a9499ec9-d63a-4252-a912-afb613e82837 | Translation Approved (80%) | Purchase at least 25%, by cost, of total combined food and beverage purchases must meet at least one of the following criteria. | Compre no mínimo 25%, por custo, do total combinado de compras de alimentos e bebidas que devem atender a pelo menos um dos critérios a seguir. |
| 1490a9499ec9-d63a-4252-a912-afb613e82837 | Translation Approved (0%) | Exclude wine, beer, and liquor purchases from the credit calculations. | Exclua compras de vinho, cerveja e destilados dos cálculos do crédito. |
| 1491a5cc4710-903e-4dfd-8c3e-12a746f7a30d | Translation Approved (100%) | Sustainable agriculture. | Agricultura sustentável. |
| 1492a5cc4710-903e-4dfd-8c3e-12a746f7a30d | Translation Approved (82%) | The food or beverage must be labeled USDA Organic, Food Alliance Certified, Rainforest Alliance Certified, Protected Harvest Certified, Fair Trade, or Marine Stewardship Council’s Blue Eco-Label, or labeled with the European Community Organic Production logo in accordance with Regulations (EC) No. 834/2007 and (EC) No. 889/2008. | Alimentos e bebidas devem receber o selo orgânico do USDA (USDA Organic), a certificação Food Alliance, a certificação Rainforest Alliance, a certificação Protected Harvest, Fair Trade ou o Selo ecológico azul do Marine Stewardship Council, ou receber o selo do logo de Produção orgânica da comunidade europeia (European Community Organic Production) de acordo com as Normas (UE) Nº 834/2007 e (UE) Nº 889/2008. |
| 1493a3dfae55-4e32-4176-9946-c3b305b7c013 | Translation Approved (0%) | Local sourcing. | Fornecimento local. |
| 1494a3dfae55-4e32-4176-9946-c3b305b7c013 | Translation Approved (100%) | The food or beverage must contain raw materials harvested and produced within 100 miles (160 kilometers) of the site. | Os alimentos e bebidas devem conter matérias-primas colhidas e produzidas em uma distância de até 100 milhas (160 quilômetros) do terreno. |
| 1495269ad12e-d085-419c-8f91-5491418a73d0 | Translation Approved (78%) | MR Credit: Purchasing—Lamps | Crédito MR: Compras - Lâmpadas (MR Credit: Purchasing—Lamps) |
| 1496851fe29c-e353-43c9-a05f-fd83a584fdc0 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 1497be8da816-1596-4e25-9b4a-ed95210a6b26 | Translation Approved (100%) | 1 point | 1 ponto |
| 14983da04c53-083d-4c11-944d-a0e43389fb60 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 14991c733cd6-de4a-4419-bddf-074db83b0102 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 pontos) |
| 1500f38125df-949a-4cbb-a287-6a1f512da858 | Translation Approved (100%) | EB:O&M Schools (1 points) | EB:O&M Escolas (Schools) (1 pontos) |
| 1501cc882a68-632c-43b9-8e0e-b0d5146b4d8a | Translation Approved (100%) | EB:O&M Retail (1 points) | EB:O&M Varejo (Retail) (1 pontos) |
| 15026fc767ec-3350-4a3c-bb82-504580b3e3eb | Translation Approved (100%) | EB:O&M Data Centers (1 points) | EB:O&M Centros de Dados (Data Centers) (1 pontos) |
| 1503a125cad2-565f-4a39-bcc2-b56d8275bbe8 | Translation Approved (100%) | EB:O&M Hospitality (1 points) | EB:O&M Hoteleiro (Hospitality) (1 pontos) |
| 150443d43195-de1d-47d3-bfb0-c91b7b309c9b | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 pontos) |
| 150517909fe8-d915-4983-8b48-cf461ae49f11 | Translation Approved (100%) | Intent | Objetivo |
| 15062f399ecf-3d26-43cd-9b48-3f0b9db496e7 | Translation Approved (100%) | To establish and maintain a toxic material source reduction program to reduce the amount of mercury brought onto the building site through purchases of lamps. | Estabelecer e manter um programa de redução de fontes de materiais tóxicos para reduzir a quantidade de mercúrio levada para o local do edifício por meio de compras de lâmpadas. |
| 1507c8ec8e5d-7dc9-4be2-9eca-be88c60f3b10 | Translation Approved (100%) | Requirements | Requisitos |
| 15082dc60f2f-9499-40e1-92de-baab8dbd20d3 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 15096e82b82b-13d3-4189-9443-c634ce1d7226 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 15107f506373-fc1f-4f20-9051-4c3b51a79bd4 | Translation Approved (100%) | None. | Nenhum. |
| 1511642fcd71-048e-44b1-847a-5cfc66105d47 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 1512f7641632-edff-4194-9f7a-0d14605c9bb4 | Translation Approved (0%) | Implement the lighting purchasing plan that specifies an overall building average of 70 picograms of mercury per lumen-hour or less for all mercury-containing lamps purchased for the building and associated grounds within the project boundary. | Implemente o plano de compra de produtos de iluminação que especifica uma média geral do edifício de 10 picogramas de mercúrio por lúmen-hora ou menos para todas as lâmpadas que contêm mercúrio compradas para o edifício e terrenos associados dentro do limite do projeto. |
| 1513f7641632-edff-4194-9f7a-0d14605c9bb4 | Translation Approved (88%) | Include lamps for both indoor and outdoor fixtures, as well as both hard-wired and portable fixtures. | Inclua lâmpadas para dispositivos internos e externos, bem como para dispositivos cabeados e portáteis. |
| 1514f7641632-edff-4194-9f7a-0d14605c9bb4 | Translation Approved (0%) | Lamps containing no mercury may be counted only if their energy efficiency at least equals that of their mercury-containing counterparts. | As lâmpadas que não contêm mercúrio podem ser contadas somente se apresentarem eficiência energética igual a suas correspondentes que contêm mercúrio. |
| 1515aea6dbe8-b62f-4eef-825e-14fea6dbb965 | Translation Approved (100%) | MR Credit: Purchasing—Facility Maintenance and Renovation | Crédito MR: Compras - Manutenção e Reforma de Instalações (MR Credit: Purchasing—Facility Maintenance and Renovation) |
| 15168c41f8ac-29da-4423-b6ec-df519e64e77e | Translation Approved (100%) | EB:O&M | EB:O&M |
| 151755027a9c-29e1-4f7a-9410-16cfbe8fd905 | Translation Approved (100%) | 1–2 points | 1 a 2 pontos |
| 1518e1575400-e02f-4197-909a-a7dbc2d1bbaf | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 15190d872e0e-f316-4fe2-bb2c-459a061e702f | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1–2 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 a 2 pontos) |
| 1520302fe1a5-bf93-4df0-ad2b-961234fdabbe | Translation Approved (100%) | EB:O&M Schools (1–2 points) | EB:O&M Escolas (Schools) (1 a 2 pontos) |
| 15217ffdfc89-28e6-4250-9e22-a84faba674d9 | Translation Approved (100%) | EB:O&M Retail (1–2 points) | EB:O&M Varejo (Retail) (1 a 2 pontos) |
| 1522da9f9333-3dcc-4f4e-99e6-38af7bac7d96 | Translation Approved (100%) | EB:O&M Data Centers (1–2 points) | EB:O&M Centros de Dados (Data Centers) (1 a 2 pontos) |
| 15237f9eecd3-31ec-4a18-95ff-bb76495dabcc | Translation Approved (100%) | EB:O&M Hospitality (1–2 points) | EB:O&M Hoteleiro (Hospitality) (1 a 2 pontos) |
| 1524dfc9b47e-672d-4a60-a3a0-7d18b1eac278 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1–2 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 a 2 pontos) |
| 1525f06493cf-59e3-49f7-8138-404acd040778 | Translation Approved (100%) | Intent | Objetivo |
| 15264803e267-b2e5-4a32-83b5-6b56da5b3b0c | Translation Approved (0%) | To reduce the environmental harm from materials used in building renovations. | Reduzir os prejuízos ambientais de materiais usados em reformas de edifícios. |
| 1527eb303a48-b6b0-4c59-9698-9b2d274c701e | Translation Approved (100%) | Requirements | Requisitos |
| 15280bfa4dcb-b5db-47c7-871a-35ffed1b3381 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1529879fc5f9-9e91-4908-81bc-1e96799b864c | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 15300e7b9d63-617e-4391-aff5-c7e1acdc8c7f | Translation Approved (100%) | None. | Nenhum. |
| 15313f067319-68e2-45b1-a906-aa72dd4f29e7 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 1532ef35d753-0462-4efc-8f0c-00bcbb9d5cd1 | Translation Approved (100%) | Option 1. | Opção 1. |
| 1533ef35d753-0462-4efc-8f0c-00bcbb9d5cd1 | Translation Approved (0%) | Products and Materials (1 point) | Produtos e materiais (1 ponto) |
| 153445c1ca45-bf24-40bd-8aea-4745ce41843c | Translation Approved (83%) | Purchase at least 50%, by cost, of the total maintenance and renovation materials that meet at least one of the following criteria. | Compre no mínimo 50%, por custo, do total de materiais de manutenção e reforma que atendam pelo menos um dos critérios a seguir. |
| 153545c1ca45-bf24-40bd-8aea-4745ce41843c | Translation Approved (76%) | Include products specified in Materials and Resources prerequisite: Facility Maintenance and Renovation Policy. | Inclua os produtos especificados no Pré-requisito MR: Política de Manutenção e Reforma de Instalações (MR Prerequisite: Facility Maintenance and Renovation Policy) |
| 153645c1ca45-bf24-40bd-8aea-4745ce41843c | Translation Approved (0%) | There is no minimum scope of renovation or new construction work required for eligibility of this credit. | Não é necessário um escopo mínimo de reforma ou trabalho novo de construção para qualificação para esse crédito. |
| 153745c1ca45-bf24-40bd-8aea-4745ce41843c | Translation Approved (100%) | Each purchase can receive credit for each criterion met. | Cada compra pode receber crédito para cada critério atendido. |
| 15384ea1b12b-ab56-4d23-860e-08d8f5a2174d | Translation Approved (98%) | Recycled content. | Conteúdo reciclado. |
| 15394ea1b12b-ab56-4d23-860e-08d8f5a2174d | Translation Approved (0%) | Recycled content is the sum of postconsumer recycled content plus one-half the preconsumer recycled content. | O conteúdo reciclado é a soma do conteúdo reciclado pós-consumo mais metade do conteúdo reciclado pré-consumo. |
| 154083bd8367-1d43-4c24-9e10-fc2daad747a8 | Translation Approved (0%) | Wood products. | Produtos de madeira. |
| 154183bd8367-1d43-4c24-9e10-fc2daad747a8 | Translation Approved (CM) | Wood products must be certified by the Forest Stewardship Council or USGBC-approved equivalent. | Produtos de madeira devem ser certificados pelo Conselho de Manejo Florestal (Forest Stewardship Council - FSC) ou equivalente aprovado pelo USGBC. |
| 15424fb5ccc1-c9a0-4a9e-bbbc-4c3acce3ff91 | Translation Approved (100%) | Bio-based materials. | Materiais biodegradáveis. |
| 15434fb5ccc1-c9a0-4a9e-bbbc-4c3acce3ff91 | Translation Approved (100%) | Bio-based products must meet the Sustainable Agriculture Network’s Sustainable Agriculture Standard. | Produtos biodegradáveis devem atender à Norma para Agricultura Sustentável da Rede de Agricultura Sustentável (Sustainable Agriculture Network’s Sustainable Agriculture Standard). |
| 15444fb5ccc1-c9a0-4a9e-bbbc-4c3acce3ff91 | Translation Approved (100%) | Bio-based raw materials must be tested using ASTM Test Method D6866 and be legally harvested, as defined by the exporting and receiving country. | Matérias-primas biodegradáveis devem ser testadas utilizando o Método de Teste ASTM D6866 e serem coletadas de acordo com as leis dos países de exportação e destino. |
| 15454fb5ccc1-c9a0-4a9e-bbbc-4c3acce3ff91 | Translation Approved (100%) | Exclude hide products, such as leather and other animal skin material. | Exclua produtos de couro cru, como couro e outras peles de animais. |
| 15463fffb4d1-3f89-4bf5-829c-2f2fa77a897e | Translation Approved (97%) | Materials reuse<1636>.</1636> | Reuso de materiais. |
| 15473fffb4d1-3f89-4bf5-829c-2f2fa77a897e | Translation Approved (CM) | Reuse includes<1637> </1637>salvaged, refurbished, or reused products. | Reuso inclui produtos recuperados, recondicionados ou reutilizados. |
| 1548675a3a12-aadc-47df-be2a-c25d6f7dc112 | Translation Approved (100%) | Extended producer responsibility. | Responsabilidade estendida do produtor. |
| 1549675a3a12-aadc-47df-be2a-c25d6f7dc112 | Translation Approved (CM) | Products purchased from a manufacturer (producer) that participates in an extended producer responsibility program or is directly responsible for extended producer responsibility. | Produtos adquiridos de um fabricante (produtor) que participe de um programa de responsabilidade estendida do produtor ou seja diretamente responsável pela responsabilidade estendida do produtor. |
| 1550675a3a12-aadc-47df-be2a-c25d6f7dc112 | Translation Approved (0%) | Products valued at 50% of their cost. | Produtos com valor de 50% do seu custo. |
| 15515be66d8c-4483-40ca-9a94-8ef410e2b951 | Translation Approved (100%) | <1640>GreenScreen v1.2 Benchmark</1640><1641>.</1641> | <1640>Referência GreenScreen v1.2</1640><1641>.</1641> |
| 15525be66d8c-4483-40ca-9a94-8ef410e2b951 | Translation Approved (99%) | Products that have fully inventoried chemical ingredients to 100 ppm that have no Benchmark 1 hazards. | Produtos com inventários completos de ingredientes químicos a 100 ppm que não têm perigos da Referência 1. |
| 155349f0c8b9-c4be-4d55-914d-d626cf44099e | Translation Approved (CM) | If any ingredients are assessed with the GreenScreen List Translator, value these products at 100% of cost. | Se algum ingrediente for avaliado com o GreenScreen List Translator, avalie esses produtos a 100% do custo. |
| 1554e3d0012e-2661-44ba-a6fc-756e04285fc3 | Translation Approved (CM) | If all ingredients are have undergone a full GreenScreen Assessment, value these products at 150% of cost. | Se todos os ingredientes passaram por uma Avaliação GreenScreen completa, avalie os produtos a 150% do custo. |
| 15550ed2c742-084c-4cfd-9fa5-33976d5d1784 | Translation Approved (CM) | <1642>Cradle to Cradle Certified</1642><1643>.</1643> | <1642>Certificação Cradle to Cradle (berço a berço)</1642><1643>.</1643> |
| 15560ed2c742-084c-4cfd-9fa5-33976d5d1784 | Translation Approved (CM) | End use products are certified Cradle to Cradle. | Produtos de uso final têm a certificação Cradle to Cradle. |
| 15570ed2c742-084c-4cfd-9fa5-33976d5d1784 | Translation Approved (CM) | Products will be valued as follows: | Os produtos serão avaliados da seguinte forma: |
| 155813e8efc3-4223-4bc6-a8c1-a97905c774ff | Translation Approved (CM) | Cradle to Cradle v2 Gold: 100% of cost | Cradle to Cradle v2 Gold: 100% do custo |
| 15592572f1d1-4658-404b-8714-d35b090615f3 | Translation Approved (CM) | Cradle to Cradle v2 Platinum: 150% of cost | Cradle to Cradle v2 Platinum: 150% do custo |
| 15608406444b-6c4e-4400-8690-45ab091bf004 | Translation Approved (CM) | Cradle to Cradle v3 Silver: 100% of cost | Cradle to Cradle v3 Silver: 100% do custo |
| 15614f22155b-237b-44ae-b3de-fbe54e256a2b | Translation Approved (CM) | Cradle to Cradle v3 Gold or Platinum: 150% of cost | Cradle to Cradle v3 Gold ou Platinum: 150% do custo |
| 1562079103bf-3e0e-41a8-b65c-fba1b2670598 | Translation Approved (CM) | <1644>International Alternative Compliance Path – REACH Optimization</1644>. | <1644>Caminho internacional alternativo para conformidade – Otimização REACH</1644>. |
| 1563079103bf-3e0e-41a8-b65c-fba1b2670598 | Translation Approved (CM) | End use products and materials that do not contain substances that meet REACH criteria for substances of very high concern. | Produtos e materiais de uso final que não contenham substâncias que atendam aos critérios REACH para substâncias de preocupação muito alta. |
| 1564079103bf-3e0e-41a8-b65c-fba1b2670598 | Translation Approved (CM) | If the product contains no ingredients listed on the REACH Authorization or Candidate list, value at 100% of cost. | Se o produto não tiver ingredientes listados na lista de Autorização ou Candidatos REACH, avalie a 100% do custo. |
| 15655738db0b-ed6b-4349-99ab-dcbed925521b | Translation Approved (0%) | <1645>Product Manufacturer Supply Chain Optimization</1645>. | Otimização da cadeia de fornecimento do fabricante do produto. |
| 15665738db0b-ed6b-4349-99ab-dcbed925521b | Translation Approved (0%) | Use building products that: | Use produtos de construção que: |
| 1567728437a6-705c-4cbf-9fe7-fdd7665f4d1d | Translation Approved (100%) | Are sourced from product manufacturers who engage in validated and robust safety, health, hazard, and risk programs which at a minimum document at least 99% (by weight) of the ingredients used to make the building product or building material, and | Sejam adquiridos de fabricantes envolvidos em programas rígidos e comprovados para segurança, saúde, perigo e riscos, que documentem no mínimo 90% (por peso) dos ingredientes usados para fazer o produto de construção ou material de construção; e |
| 1568e8b45a52-108b-4eec-9296-5f2a55e48e17 | Translation Approved (CM) | Are sourced from product manufacturers with independent third party verification of their supply chain that at a minimum verifies: | Sejam adquiridos de fabricantes que apliquem uma verificação independente de sua cadeia de fornecimento, feita por terceiros e que verifique, no mínimo, que haja: |
| 1569576dd072-2d3d-474e-8781-064166041026 | Translation Approved (CM) | Processes are in place to communicate and transparently prioritize chemical ingredients along the supply chain according to available hazard, exposure and use information to identify those that require more detailed evaluation | Processos em vigor para comunicar e priorizar de maneira transparente ingredientes químicos em toda a cadeia de fornecimento de acordo com as informações disponíveis de risco, exposição e uso, para identificar os que precisam de avaliações mais detalhadas |
| 1570798379aa-ce19-4362-80c6-92ce422e062d | Translation Approved (CM) | Processes are in place to identify, document, and communicate information on health, safety and environmental characteristics of chemical ingredients | Processos em vigor para identificar, documentar e comunicar informações sobre saúde, segurança e as características ambientais dos ingredientes químicos |
| 157158c463cf-32e8-407c-83ad-f8be41779a2f | Translation Approved (CM) | Processes are in place to implement measures to manage the health, safety and environmental hazard and risk of chemical ingredients | Processos em vigor para implementar medidas para gerenciar perigos e riscos de saúde, segurança e ambientais de ingredientes químicos |
| 1572d4f743ab-b64f-4608-ac02-35ac840bc8d1 | Translation Approved (CM) | Processes are in place to optimize health, safety and environmental impacts when designing and improving chemical ingredients | Processos em vigor para otimizar os impactos ambientais, de saúde e segurança ao projetar e aprimorar ingredientes químicos |
| 15734feef96c-67ad-4a49-b3ee-4dc7155a8f89 | Translation Approved (CM) | Processes are in place to communicate, receive and evaluate chemical ingredient safety and stewardship information along the supply chain | Processos em vigor para comunicar, receber e avaliar informações de administração e segurança de ingredientes químicos ao longo da cadeia de fornecimento |
| 15745b8757a2-ccf1-4455-9407-bb7a0ca0a92f | Translation Approved (CM) | Safety and stewardship information about the chemical ingredients is publicly available from all points along the supply chain | Informações de administração e segurança dos ingredientes químicos publicamente disponíveis em todos os pontos da cadeia de fornecimento |
| 1575bd1ffcb1-e97b-434d-98f2-eccae47febeb | Translation Approved (0%) | Low emissions of volatile organic compounds. | Baixas emissões de compostos orgânicos voláteis. |
| 1576bd1ffcb1-e97b-434d-98f2-eccae47febeb | Translation Approved (0%) | The following products must either be inherently nonemitting or be<1648> </1648>tested and determined compliant in accordance with California Department of Public Health Standard Method V1.1–2010, using the applicable exposure scenario. | Os produtos a seguir devem ser inerentemente não emissores ou serem testados e determinados em conformidade de acordo com o Método padrão V1.1–2010 do Departamento de Saúde Pública da Califórnia (California Department of Public Health Standard Method V1.1–2010), usando o cenário de exposição aplicável. |
| 1577bd1ffcb1-e97b-434d-98f2-eccae47febeb | Translation Approved (0%) | The default scenario is the private office scenario; classroom furniture may use the school classroom scenario. | O cenário padrão é o cenário do escritório privado; o mobiliário escolar pode usar o cenário de sala de aula escolar. |
| 1578bd1ffcb1-e97b-434d-98f2-eccae47febeb | Translation Approved (100%) | Both first-party and third-party statements of product compliance must follow the guidelines in CDPH SM V1.1–2010, Section 8. | Declarações de conformidade de produto, de primeira fonte e de terceiros, devem seguir as diretrizes em CDPH SM V1.1–2010, Seção 8. |
| 1579bd1ffcb1-e97b-434d-98f2-eccae47febeb | Translation Approved (CM) | Organizations that certify manufacturers’ claims must be accredited under ISO Guide 65. | Organizações que certificam alegações de fabricantes devem ser certificadas pelo Guia ISO 65. |
| 1580bd1ffcb1-e97b-434d-98f2-eccae47febeb | Translation Approved (0%) | Laboratories that conduct the tests must be accredited under ISO/IEC 17025 for the test methods they use. | Os laboratórios que conduzirem os testes devem ser acreditados pela ISO/IEC 17025 para os métodos de teste que utilizarem. |
| 1581bd1ffcb1-e97b-434d-98f2-eccae47febeb | Translation Approved (0%) | Projects outside the U.S. may use (1) the CDPH standard method or (2) the German AgBB Testing and Evaluation Scheme (2010). | Os projetos fora dos EUA podem usar (1) o método padrão do CDPH ou (2) o Esquema de avaliação e teste AgBB alemão (German AgBB Testing and Evaluation Scheme) (2010). |
| 1582bd1ffcb1-e97b-434d-98f2-eccae47febeb | Translation Approved (0%) | Test products either with (1) ISO 16000-3: 2010, ISO 16000-6: 2011, ISO 16000-9: 2006, ISO 16000-11:2006, or (2) the DIBt testing method (2010). | Teste os produtos com as normas (1) ISO 16000-3: 2010, ISO 16000-6: 2011, ISO 16000-9: 2006, ISO 16000-11:2006 ou com (2) o método de teste DIBt (2010). |
| 1583bd1ffcb1-e97b-434d-98f2-eccae47febeb | Translation Approved (100%) | U.S. projects must follow the CDPH standard method. | Projetos nos EUA devem seguir o método da norma do CDPH. |
| 1584f994f613-574a-473c-ab44-333420af0474 | Translation Approved (0%) | thermal and acoustic insulation | isolamento térmico e acústico |
| 15857719479f-b246-4301-8170-15eafcb69733 | Translation Approved (0%) | flooring materials and finishes | materiais e acabamentos do piso |
| 15866db86c72-ef8c-4255-a84b-538dfc7b4039 | Translation Approved (78%) | ceiling materials and finishes | materiais e acabamentos do telhado |
| 1587edd2870e-3876-4b48-b5b5-0e41ca887024 | Translation Approved (78%) | wall materials and finishes | materiais e acabamentos da parede |
| 1588614152a3-ab62-4676-a2ab-af8b55a436bd | Translation Approved (0%) | VOC content requirements for wet-applied products. | Requisitos de concentração de COV para produtos de aplicação úmida. |
| 1589614152a3-ab62-4676-a2ab-af8b55a436bd | Translation Approved (100%) | In addition to meeting the general requirements for VOC emissions (above), on-site wet-applied products must not contain excessive levels of VOCs, for the health of the installers and other tradesworkers who are exposed to these products. | Além de atender aos requisitos gerais para emissões de COV (acima), produtos com aplicação úmida no local não devem conter níveis excessivos de COVs, para a saúde dos aplicadores e de outros trabalhadores que são expostos aos produtos. |
| 1590614152a3-ab62-4676-a2ab-af8b55a436bd | Translation Approved (CM) | To demonstrate compliance, a product or layer must meet the following requirements, as applicable. | Para demonstrar conformidade, um produto ou camada deve atender aos requisitos a seguir, conforme for aplicável. |
| 1591614152a3-ab62-4676-a2ab-af8b55a436bd | Translation Approved (CM) | Disclosure of VOC content must be made by the manufacturer. | A divulgação da concentração de COV deve ser feita pelo fabricante. |
| 1592614152a3-ab62-4676-a2ab-af8b55a436bd | Translation Approved (CM) | Any testing must follow the test method specified in the applicable regulation. | Todos os testes devem seguir o método de teste especificado no regulamento aplicável. |
| 159307080df2-dde1-4df4-8266-d90754b46571 | Translation Approved (CM) | All paints and coatings wet-applied on site must meet the applicable VOC limits of the California Air Resources Board (CARB) 2007, Suggested Control Measure (SCM) for Architectural Coatings, or the South Coast Air Quality Management District (SCAQMD) Rule 1113, effective June 3, 2011. | Todas as tintas e revestimentos com aplicação úmida no local devem atender aos limites de COV aplicáveis do Conselho de Recursos Atmosféricos da Califórnia (California Air Resources Board – CARB) de 2007, Medida de controle sugerida (SCM) para revestimentos arquitetônicos (Suggested Control Measure (SCM) for Architectural Coatings), ou do Distrito de Gestão da Qualidade do Ar da Costa Sul (South Coast Air Quality Management District – SCAQMD), Regra 1113, de 3 de junho de 2011. |
| 15940d53418c-950b-4deb-84f4-c3c7eba1e9bf | Translation Approved (CM) | All adhesives and sealants wet-applied on site must meet the applicable chemical content requirements of SCAQMD Rule 1168, July 1, 2005, Adhesive and Sealant Applications, as analyzed by the methods specified in Rule 1168. | Todos os adesivos e selantes com aplicação úmida no local devem atender aos requisitos aplicáveis para concentração química da Regra 1168 da SCAQMD, de 1 de julho de 2005, Aplicações de adesivo e selante (Adhesive and Sealant Applications), conforme analisada pelos métodos especificados na Regra 1168. |
| 15950d53418c-950b-4deb-84f4-c3c7eba1e9bf | Translation Approved (100%) | The provisions of SCAQMD Rule 1168 do not apply to adhesives and sealants subject to state or federal consumer product VOC regulations. | As disposições da Regra 1168 da SCAQMD não se aplicam a adesivos e selantes sujeitos a regulamentos estaduais ou federais para produtos com COV. |
| 1596787fe94d-8513-46dc-9b65-707794e28584 | Translation Approved (98%) | For projects outside North America, all paints, coatings, adhesives, and sealants wet-applied on site must either meet the technical requirements of the above regulations, or comply with applicable national VOC control regulations, such as the European Decopaint Directive (2004/42/EC), the Canadian VOC Concentration Limits for Architectural Coatings, or the Hong Kong Air Pollution Control (VOC) Regulation. | Para projetos fora da América no Norte, todas as tintas, revestimentos, adesivos e selantes com aplicação úmida no local devem atender aos requisitos técnicos dos regulamentos acima ou cumprir regulamentos nacionais de controle de COV, como a Diretriz Europeia Decopaint (2004/42/EC), os Limites de concentração de COV para revestimentos arquitetônicos do Canadá (Canadian VOC Concentration Limits for Architectural Coatings) ou o Regulamento de controle de poluição atmosférica (COV) de Hong Kong (Hong Kong Air Pollution Control (VOC) Regulation). |
| 1597276f2df0-c29d-424e-81aa-ccf8fe0acf17 | Translation Approved (100%) | If the applicable regulation requires subtraction of exempt compounds, any content of intentionally added exempt compounds larger than 1% weight by mass (total exempt compounds) must be disclosed. | Se o regulamento aplicável exigir a subtração de compostos isentos, qualquer concentração intencionalmente adicionada de compostos isentos superior a 1% do peso por massa (total de compostos isentos) deve ser divulgada. |
| 1598db7e1466-a1d6-4fcc-9c84-97293e092163 | Translation Approved (CM) | If a product cannot reasonably be tested as specified above, testing of VOC content must comply with ASTM D2369-10; ISO 11890, part 1; ASTM D6886-03; or ISO 11890-2. | Se um produto não puder ser testado de maneira razoável conforme especificado acima, o teste da concentração de COV deve ser feito em conformidade com ASTM D2369-10; ISO 11890, parte 1; ASTM D6886-03; ou ISO 11890-2. |
| 1599274e5623-aef5-43e2-993c-b21d3f6fd934 | Translation Approved (CM) | For projects in North America, methylene chloride and perchloroethylene may not be intentionally added in paints, coatings, adhesives, or sealants. | Para projetos na América do Norte, cloreto de metileno e percloroetileno não podem ser intencionalmente adicionados a tintas, revestimentos, adesivos ou selantes. |
| 16002d70d732-e616-4632-b6b3-c92cbd39908b | Translation Approved (0%) | Low emissions of formaldehyde. | Baixas emissões de formaldeídos. |
| 16012d70d732-e616-4632-b6b3-c92cbd39908b | Translation Approved (0%) | Built-in cabinetry and architectural millwork containing composite woods must be constructed from materials documented to have low formaldehyde emissions that meet the California Air Resources Board requirements for ultra-low-emitting formaldehyde (ULEF) resins or no-added formaldehyde based resins. | Armários embutidos e madeiramento arquitetônico que contêm madeira composta devem ser construídos de materiais documentados com baixas emissões de formaldeídos que atendem aos requisitos do Conselho de Recursos Atmosféricos da Califórnia (California Air Resources Board) para resinas de formaldeídos de emissão ultrabaixa (ULEF) ou resinas sem adição de formaldeídos. |
| 16022d70d732-e616-4632-b6b3-c92cbd39908b | Translation Approved (100%) | Salvaged and reused architectural millwork more than one year old at the time of occupancy is considered compliant, provided it meets the requirements for any site-applied paints, coatings, adhesives, and sealants. | Trabalhos arquitetônicos em madeira recuperados e reutilizados com mais de um ano no momento da ocupação são considerados em conformidade se atenderem aos requisitos para qualquer tinta, revestimento, adesivo e selante aplicados no local. |
| 16035f8b8129-53e7-4541-a2f3-8069933edf78 | Translation Approved (100%) | USGBC approved program<1653>.</1653> | Programa aprovado pelo USGBC<1653>.</1653> |
| 16045f8b8129-53e7-4541-a2f3-8069933edf78 | Translation Approved (CM) | Other USGBC approved programs meeting leadership extraction criteria. | Outros programas aprovados pelo USGBC que atendem aos principais critérios. |
| 160549ec8744-7435-408e-a1a3-765f9fc43515 | Translation Approved (99%) | For credit achievement calculation, products sourced (extracted, manufactured, and purchased) within 100 miles (160 km) of the project site are valued at 200% of their base contributing cost. | Para o cálculo de obtenção do crédito, os produtos originados (extraídos, fabricados e adquiridos) a até 100 milhas (160 km) do terreno do projeto são avaliados a 200% de seu custo base de contribuição. |
| 1606a0812328-670a-4cd7-8877-991f7ba43798 | Translation Approved (100%) | AND/OR | E/OU |
| 1607844bb66d-dc14-483d-909f-a1a763df3b12 | Translation Approved (100%) | Option 2. | Opção 2. |
| 1608844bb66d-dc14-483d-909f-a1a763df3b12 | Translation Approved (0%) | Furniture (1 point) | Móveis (1 ponto) |
| 16099889fcc4-3d9f-4c9b-9b37-001f64909a58 | Translation Approved (79%) | Purchase at least 75%, by cost, of total furniture and furnishings that meet one or more of the following criteria. | Compre no mínimo 75%, por custo, do total de móveis e acessórios que atendam pelo menos um ou mais critérios a seguir. |
| 16109889fcc4-3d9f-4c9b-9b37-001f64909a58 | Translation Approved (100%) | Each purchase can receive credit for each criterion met. | Cada compra pode receber crédito para cada critério atendido. |
| 1611c33bcb02-61ef-4559-8340-d3db72b30bf7 | Translation Approved (100%) | Recycled content. | Conteúdo reciclado. |
| 1612c33bcb02-61ef-4559-8340-d3db72b30bf7 | Translation Approved (CM) | Recycled content is the sum of postconsumer recycled content plus one-half the preconsumer recycled content, based on cost. | O conteúdo reciclado é a soma do conteúdo reciclado pós-consumo mais metade do conteúdo reciclado pré-consumo, com base no custo. |
| 16130bddaf0c-029c-4f08-a3a2-1b3aa5965fa0 | Translation Approved (100%) | Wood products. | Produtos de madeira. |
| 16140bddaf0c-029c-4f08-a3a2-1b3aa5965fa0 | Translation Approved (100%) | Wood products must be certified by the Forest Stewardship Council or USGBC-approved equivalent. | Produtos de madeira devem ser certificados pelo Conselho de Manejo Florestal (Forest Stewardship Council - FSC) ou equivalente aprovado pelo USGBC. |
| 16151b89239e-55ab-4c58-b2b3-70f74e6df75a | Translation Approved (100%) | Bio-based materials. | Materiais biodegradáveis. |
| 16161b89239e-55ab-4c58-b2b3-70f74e6df75a | Translation Approved (100%) | Bio-based products must meet the Sustainable Agriculture Network’s Sustainable Agriculture Standard. | Produtos biodegradáveis devem atender à Norma para Agricultura Sustentável da Rede de Agricultura Sustentável (Sustainable Agriculture Network’s Sustainable Agriculture Standard). |
| 16171b89239e-55ab-4c58-b2b3-70f74e6df75a | Translation Approved (100%) | Bio-based raw materials must be tested using ASTM Test Method D6866 and be legally harvested, as defined by the exporting and receiving country. | Matérias-primas biodegradáveis devem ser testadas utilizando o Método de Teste ASTM D6866 e serem coletadas de acordo com as leis dos países de exportação e destino. |
| 16181b89239e-55ab-4c58-b2b3-70f74e6df75a | Translation Approved (100%) | Exclude hide products, such as leather and other animal skin material. | Exclua produtos de couro cru, como couro e outras peles de animais. |
| 161945daf996-e6bf-4e1a-93cd-43098ad543f1 | Translation Approved (100%) | Materials reuse<1663>.</1663> | Reuso de materiais. |
| 162045daf996-e6bf-4e1a-93cd-43098ad543f1 | Translation Approved (100%) | Reuse includes<1664> </1664>salvaged, refurbished, or reused products. | Reuso inclui produtos recuperados, recondicionados ou reutilizados. |
| 16215c1a72d2-addb-4d91-a42f-d6a41f02b777 | Translation Approved (100%) | Extended producer responsibility. | Responsabilidade estendida do produtor. |
| 16225c1a72d2-addb-4d91-a42f-d6a41f02b777 | Translation Approved (100%) | Products purchased from a manufacturer (producer) that participates in an extended producer responsibility program or is directly responsible for extended producer responsibility. | Produtos adquiridos de um fabricante (produtor) que participe de um programa de responsabilidade estendida do produtor ou seja diretamente responsável pela responsabilidade estendida do produtor. |
| 16235c1a72d2-addb-4d91-a42f-d6a41f02b777 | Translation Approved (100%) | Products valued at 50% of their cost. | Produtos com valor de 50% do seu custo. |
| 1624e7ab88c4-5406-450d-a305-3eced1b3895a | Translation Approved (100%) | <1667>GreenScreen v1.2 Benchmark</1667><1668>.</1668> | <1667>Referência GreenScreen v1.2</1667><1668>.</1668> |
| 1625e7ab88c4-5406-450d-a305-3eced1b3895a | Translation Approved (100%) | Products that have fully inventoried chemical ingredients to 100 ppm that have no Benchmark 1 hazards. | Produtos com inventários completos de ingredientes químicos a 100 ppm que não têm perigos da Referência 1. |
| 1626e3b7a0a3-073d-4ef8-8676-eb1f41130326 | Translation Approved (100%) | If any ingredients are assessed with the GreenScreen List Translator, value these products at 100% of cost. | Se algum ingrediente for avaliado com o GreenScreen List Translator, avalie esses produtos a 100% do custo. |
| 162745fe4db8-26b9-431f-8fc3-418f0b5a4a78 | Translation Approved (100%) | If all ingredients are have undergone a full GreenScreen Assessment, value these products at 150% of cost. | Se todos os ingredientes passaram por uma Avaliação GreenScreen completa, avalie os produtos a 150% do custo. |
| 162897d559d1-f286-4d45-95f6-42ecdb55bacf | Translation Approved (100%) | <1669>Cradle to Cradle Certified</1669><1670>.</1670> | <1669>Certificação Cradle to Cradle (berço a berço)</1669><1670>.</1670> |
| 162997d559d1-f286-4d45-95f6-42ecdb55bacf | Translation Approved (100%) | End use products are certified Cradle to Cradle. | Produtos de uso final têm a certificação Cradle to Cradle. |
| 163097d559d1-f286-4d45-95f6-42ecdb55bacf | Translation Approved (100%) | Products will be valued as follows: | Os produtos serão avaliados da seguinte forma: |
| 163185d4d934-9505-486b-8b72-252b6a442626 | Translation Approved (100%) | Cradle to Cradle v2 Gold: 100% of cost | Cradle to Cradle v2 Gold: 100% do custo |
| 1632b07f1654-425f-4823-b0cc-ae5b9ddb605a | Translation Approved (100%) | Cradle to Cradle v2 Platinum: 150% of cost | Cradle to Cradle v2 Platinum: 150% do custo |
| 1633a9eb7ddf-8658-4ea8-9d70-9c1bf0f7d714 | Translation Approved (100%) | Cradle to Cradle v3 Silver: 100% of cost | Cradle to Cradle v3 Silver: 100% do custo |
| 163483b7155e-b715-4100-895a-f7735eec258b | Translation Approved (100%) | Cradle to Cradle v3 Gold or Platinum: 150% of cost | Cradle to Cradle v3 Gold ou Platinum: 150% do custo |
| 1635cb67b0ee-1b07-4257-9277-938461cb7add | Translation Approved (100%) | <1671>International Alternative Compliance Path – REACH Optimization</1671>. | <1671>Caminho internacional alternativo para conformidade – Otimização REACH</1671>. |
| 1636cb67b0ee-1b07-4257-9277-938461cb7add | Translation Approved (100%) | End use products and materials that do not contain substances that meet REACH criteria for substances of very high concern. | Produtos e materiais de uso final que não contenham substâncias que atendam aos critérios REACH para substâncias de preocupação muito alta. |
| 1637cb67b0ee-1b07-4257-9277-938461cb7add | Translation Approved (100%) | If the product contains no ingredients listed on the REACH Authorization or Candidate list, value at 100% of cost. | Se o produto não tiver ingredientes listados na lista de Autorização ou Candidatos REACH, avalie a 100% do custo. |
| 1638b736b388-c9c7-40c0-bb93-dc2285ff440b | Translation Approved (100%) | <1672>Product Manufacturer Supply Chain Optimization</1672>. | Otimização da cadeia de fornecimento do fabricante do produto. |
| 1639b736b388-c9c7-40c0-bb93-dc2285ff440b | Translation Approved (100%) | Use building products that: | Use produtos de construção que: |
| 1640758ef6b6-a9a3-4f1f-9290-328195ec73ad | Translation Approved (100%) | Are sourced from product manufacturers who engage in validated and robust safety, health, hazard, and risk programs which at a minimum document at least 99% (by weight) of the ingredients used to make the building product or building material, and | Sejam adquiridos de fabricantes envolvidos em programas rígidos e comprovados para segurança, saúde, perigo e riscos, que documentem no mínimo 90% (por peso) dos ingredientes usados para fazer o produto de construção ou material de construção; e |
| 1641e08b0007-ad86-4bc3-9301-6d9bb8df4e66 | Translation Approved (100%) | Are sourced from product manufacturers with independent third party verification of their supply chain that at a minimum verifies: | Sejam adquiridos de fabricantes que apliquem uma verificação independente de sua cadeia de fornecimento, feita por terceiros e que verifique, no mínimo, que haja: |
| 164204470681-abd0-415a-865b-cbef9e37e16f | Translation Approved (100%) | Processes are in place to communicate and transparently prioritize chemical ingredients along the supply chain according to available hazard, exposure and use information to identify those that require more detailed evaluation | Processos em vigor para comunicar e priorizar de maneira transparente ingredientes químicos em toda a cadeia de fornecimento de acordo com as informações disponíveis de risco, exposição e uso, para identificar os que precisam de avaliações mais detalhadas |
| 164393e149d2-a3d1-40ad-adfb-e0c519e22bb0 | Translation Approved (100%) | Processes are in place to identify, document, and communicate information on health, safety and environmental characteristics of chemical ingredients | Processos em vigor para identificar, documentar e comunicar informações sobre saúde, segurança e as características ambientais dos ingredientes químicos |
| 164494fa8585-3376-4f9b-bfd6-4d940c66725e | Translation Approved (100%) | Processes are in place to implement measures to manage the health, safety and environmental hazard and risk of chemical ingredients | Processos em vigor para implementar medidas para gerenciar perigos e riscos de saúde, segurança e ambientais de ingredientes químicos |
| 16456a17a9f7-76c5-41b6-86c7-e5cf3d5bdc5a | Translation Approved (100%) | Processes are in place to optimize health, safety and environmental impacts when designing and improving chemical ingredients | Processos em vigor para otimizar os impactos ambientais, de saúde e segurança ao projetar e aprimorar ingredientes químicos |
| 1646f3796223-6559-46b8-8cf6-91c2af2d4f12 | Translation Approved (100%) | Processes are in place to communicate, receive and evaluate chemical ingredient safety and stewardship information along the supply chain | Processos em vigor para comunicar, receber e avaliar informações de administração e segurança de ingredientes químicos ao longo da cadeia de fornecimento |
| 1647016c1345-d9c1-4d57-8f00-bbf15b8677d0 | Translation Approved (100%) | Safety and stewardship information about the chemical ingredients is publicly available from all points along the supply chain | Informações de administração e segurança dos ingredientes químicos publicamente disponíveis em todos os pontos da cadeia de fornecimento |
| 1648a1c958ff-3a45-4dfd-9842-86cbad2f1e9d | Translation Approved (100%) | Low emissions of volatile organic compounds. | Baixas emissões de compostos orgânicos voláteis. |
| 1649a1c958ff-3a45-4dfd-9842-86cbad2f1e9d | Translation Approved (0%) | Products must have been tested, following ANSI/BIFMA Standard Method M7.1–2011, and must comply with ANSI/BIFMA e3-2011 Furniture Sustainability Standard, Sections 7.6.1 (valued at 50% cost) or 7.6.2 (valued at 100% cost), using either the concentration modeling approach or the emissions factor approach. | Os produtos devem ter sido testados, seguindo o Método padrão ANSI/BIFMA M7.1–2011 (ANSI/BIFMA Standard Method M7.1–2011) e devem estar em conformidade com a Norma de sustentabilidade de móveis ANSI/BIFMA e3-2011 (ANSI/BIFMA e3-2011 Furniture Sustainability Standard), seções 7.6.1 (avaliado a 50% do custo) ou 7.6.2 (avaliado a 100% do custo), usando abordagem de modelo da concentração ou a abordagem de fatores de emissão. |
| 1650a1c958ff-3a45-4dfd-9842-86cbad2f1e9d | Translation Approved (100%) | For classroom furniture, use the standard school classroom model in CDPH Standard Method v1.1. | Para mobiliário escolar, use o modelo de sala de aula padrão do Método da Norma v.1.1 do CDPH. |
| 1651a1c958ff-3a45-4dfd-9842-86cbad2f1e9d | Translation Approved (100%) | Salvaged and reused furniture more than one year old at the time of use is considered compliant, provided it meets the requirements for any site-applied paints, coatings, adhesives, and sealants. | Móveis recuperados e reutilizados com mais de um ano no momento do uso são considerados em conformidade se atenderem aos requisitos para qualquer tinta, revestimento, adesivo e selante aplicados no local. |
| 16522d14b842-e49f-487c-8ad7-882f12db8955 | Translation Approved (100%) | USGBC approved program<1675>.</1675> | Programa aprovado pelo USGBC<1675>.</1675> |
| 16532d14b842-e49f-487c-8ad7-882f12db8955 | Translation Approved (100%) | Other USGBC approved programs meeting leadership extraction criteria. | Outros programas aprovados pelo USGBC que atendem aos principais critérios. |
| 1654c73ee1e0-c8ed-473c-948b-4102309c998d | Translation Approved (100%) | For credit achievement calculation, products sourced (extracted, manufactured, purchased) within 100 miles (160 km) of the project site are valued at 200% of their base contributing cost. | Para o cálculo de obtenção do crédito, os produtos originados (extraídos, fabricados, adquiridos) a até 100 milhas (160 km) do terreno do projeto são avaliados em 200% de seu custo base de contribuição. |
| 16555c2f8152-5629-44ab-93e6-8c063e891e1f | Translation Approved (100%) | OR | OU |
| 16564009961f-c9bb-4c9f-b253-233d08fbf5fe | Translation Approved (100%) | Option 3. | Opção 3. |
| 16574009961f-c9bb-4c9f-b253-233d08fbf5fe | Translation Approved (0%) | No Alterations or Furniture Purchasing (1 point) | Sem alterações ou compra de móveis (1 ponto) |
| 16584064276b-88cc-4aa3-8c5f-b0827e8710b3 | Translation Approved (0%) | Make no alterations to the project space and do not purchase any furniture. | Não faça alterações no espaço do projeto e não compre móveis. |
| 165944e37c35-e840-4573-ae7f-4e3223cd61df | Translation Approved (86%) | MR Credit: Solid Waste Management—Ongoing | Crédito MR: Gestão de Resíduos Sólidos - Contínua (MR Credit: Solid Waste Management—Ongoing) |
| 1660eea81134-d13a-4808-92d5-63754a35afb5 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 1661c33fd365-86b3-4dec-b90e-0e2aac130514 | Translation Approved (100%) | 2 points | 2 pontos |
| 1662cef247f6-9728-403c-8c31-daa3bb942454 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 166329a7c7fd-e100-469e-b0cd-1f26f71d1f47 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (2 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (2 pontos) |
| 16644562a461-46d2-4c20-8e97-efc9b116e843 | Translation Approved (100%) | EB:O&M Schools (2 points) | EB:O&M Escolas (Schools) (2 pontos) |
| 16653395c920-9262-49d3-8aff-b2c6c4aea5d0 | Translation Approved (100%) | EB:O&M Retail (2 points) | EB:O&M Varejo (Retail) (2 pontos) |
| 1666651041b6-a405-4fba-93c3-5c9ed729906a | Translation Approved (100%) | EB:O&M Data Centers (2 points) | EB:O&M Centros de Dados (Data Centers) (2 pontos) |
| 1667fc4ddba7-82fa-481e-aec4-f446d35a40d2 | Translation Approved (100%) | EB:O&M Hospitality (2 points) | EB:O&M Hoteleiro (Hospitality) (2 pontos) |
| 1668a92096a2-c5e2-4ec1-a2d8-98c438aba45c | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (2 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (2 pontos) |
| 166958040408-f8c4-43d0-b282-4b2abbbfa344 | Translation Approved (100%) | Intent | Objetivo |
| 1670307bdaa0-e16c-48fd-b5be-390f3a9db2c3 | Translation Approved (73%) | To reduce the waste that is generated by building occupants and hauled to and disposed of in landfills and incinerators. | Reduzir os resíduos gerados por ocupantes de edifícios e transportados e descartados em aterros sanitários ou incineradores. |
| 167146a1feb7-6040-4499-a822-21bc45d816f6 | Translation Approved (100%) | Requirements | Requisitos |
| 1672cfd08fc3-eaab-46a2-98b8-298162bd0176 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 16736d19baaa-9239-4353-aece-d455262967ad | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 16745f9baec0-d9a5-47e7-90f1-de1b2a889e9f | Translation Approved (100%) | None. | Nenhum. |
| 16753c68de6d-10d9-4431-9c2d-2aa85d8b947e | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 1676449be14c-795f-471b-831d-e09e79619d40 | Translation Approved (0%) | Maintain a waste reduction and recycling program that reuses, recycles, or composts the following: | Mantenha um programa de redução e reciclagem de resíduos que reutilize, recicle ou faça a compostagem do seguinte: |
| 167736a16960-5bfa-4177-9a07-5c764aca94d7 | Translation Approved (0%) | at least 50% of the ongoing waste as specified in Materials and Resources Prerequisite: Ongoing Purchasing and Waste Policy (by weight or volume); and | pelo menos 50% dos resíduos contínuos conforme especificado no Pré-requisito MR: Política de Compras Contínuas e Resíduos (MR Prerequisite: Ongoing Purchasing and Waste Policy) (por peso ou volume); e |
| 167873c18d99-4a37-45da-a858-3b9f4cdfbf8b | Translation Approved (82%) | at least 75% of the durable goods waste as specified in Materials and Resources Prerequisite: Ongoing Purchasing and Waste Policy (by weight, volume or replacement value). | pelo menos 75% dos resíduos de bens duráveis conforme especificado no Pré-requisito MR: Política de Compras Contínuas e Resíduos (MR Prerequisite: Ongoing Purchasing and Waste Policy) (por peso, volume e valor de substituição). |
| 167912944253-cdbd-4ea9-b663-208628ca51fa | Translation Approved (0%) | In addition, safely dispose of the following: | Além disso, fazer o descarte de forma segura o seguinte: |
| 1680d8f1a6f1-5517-46a0-9a77-642a74932942 | Translation Approved (0%) | all discarded batteries; and | todas as baterias descartadas; e |
| 1681bbb3be0b-1c8b-4a91-9bd0-5d8b9f9a3c38 | Translation Approved (0%) | all mercury-containing lamps. | todas as lâmpadas de contêm mercúrio. |
| 1682f725db93-4c2d-42fc-a7d9-1b030b5f3682 | Translation Approved (100%) | Schools only | Apenas Escolas (Schools) |
| 16839c14f74b-9433-45e0-ac18-c66f682fc10d | Translation Approved (0%) | K–12 schools may exclude food waste from the final performance calculations of the total building waste stream by meeting both of the following requirements. | Escolas de educação infantil e fundamental podem excluir os restos de alimentos dos cálculos de desempenho final do fluxo total de resíduos do edifício atendendo ambos os requisitos a seguir. |
| 1684d151aee2-f21e-425e-be3a-0e5b555ec51f | Translation Approved (0%) | Provide documentation that food waste composting services are not available in the region or are not economically feasible, based on the school or district’s operational budget for solid waste management. | Forneça documentação de que os serviços de compostagem dos restos de alimentos não estão disponíveis na região e não são viáveis economicamente, com base no orçamento operacional da escola ou distrito para gestão de resíduos sólidos. |
| 16859d47db30-8510-483f-9489-f2be1d1dfdaf | Translation Approved (0%) | During the performance period, implement an awareness program that encourages occupants to reduce food waste. | Durante o período de desempenho, implementar um programa de conscientização que incentiva os ocupantes a reduzir o desperdício de alimentos. |
| 16869d47db30-8510-483f-9489-f2be1d1dfdaf | Translation Approved (0%) | Compliant programs should include at least two of the following: | Os programas de conformidade devem incluir pelo menos dois do seguinte: |
| 16877a1c21f5-1038-4ebf-909a-d5233efa3671 | Translation Approved (0%) | signage in food service and cafeteria areas; | sinalização em áreas de serviços de alimentação e lanchonetes; |
| 168875039afb-a696-4a23-9dd1-43813158fc92 | Translation Approved (0%) | food service employee training on reducing waste in food preparation and selecting menu options to reduce the potential for food waste; and | treinamento para funcionários do serviço de alimentação sobre a redução de resíduos na preparação de alimentos e a seleção de opções de menu para reduzir os potenciais restos de alimentos; e |
| 168999b76e54-f4b5-4893-af8b-422c1d2c3b55 | Translation Approved (0%) | extracurricular activities or student organizations that promote awareness of the environmental benefits associated with composting food waste. | atividades extracurriculares ou organizações estudantis que promovam a conscientização dos benefícios ambientais associados à compostagem de restos de alimentos. |
| 1690d28dadd4-8b49-44a3-96f1-b47ec8e8fa0a | Translation Approved (73%) | MR Credit: Solid Waste Management—Facility Maintenance and Renovation | Crédito MR: Gestão de Resíduos Sólidos - Manutenção e Reforma de Instalações (MR Credit: Solid Waste Management—Facility Maintenance and Renovation) |
| 169119f13f75-8478-4623-82eb-5df7040bae9b | Translation Approved (100%) | EB:O&M | EB:O&M |
| 169227df6b35-898e-410e-8598-05f121a4df89 | Translation Approved (100%) | 2 points | 2 pontos |
| 1693fc3633d8-6cb0-4de1-b4b5-5c138cfffe87 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 169438d7a4c6-850c-4023-a7b0-5d378f05edb7 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (2 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (2 pontos) |
| 169516e08eb6-666d-4eee-bd69-46d604abb59c | Translation Approved (100%) | EB:O&M Schools (2 points) | EB:O&M Escolas (Schools) (2 pontos) |
| 1696ef9d8533-df03-48ea-9a0f-3285bff89c6e | Translation Approved (100%) | EB:O&M Retail (2 points) | EB:O&M Varejo (Retail) (2 pontos) |
| 1697bacd515b-89eb-45c2-8aba-61f55d780ab2 | Translation Approved (100%) | EB:O&M Data Centers (2 points) | EB:O&M Centros de Dados (Data Centers) (2 pontos) |
| 16986abe3ed0-9660-4f89-a4c2-b4f5ff0342e0 | Translation Approved (100%) | EB:O&M Hospitality (2 points) | EB:O&M Hoteleiro (Hospitality) (2 pontos) |
| 1699fe57d42c-bc28-4209-a7f7-913d057cdc84 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (2 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (2 pontos) |
| 17003036ba33-6a36-4877-ab58-b392dbbefa8c | Translation Approved (100%) | Intent | Objetivo |
| 1701ea1ac71c-8d7c-4fa2-9067-666832c7183b | Translation Approved (0%) | To divert construction, renovation, and demolition debris from disposal in landfills and incinerators and recover and recycle reusable materials. | Desviar os entulhos de construção, reformas e demolição do descarte em aterros sanitários e incineradores e recuperar e reciclar os materiais reutilizáveis. |
| 1702457fd3b7-559b-478e-9e40-b433e491d364 | Translation Approved (100%) | Requirements | Requisitos |
| 170338a9ad90-a5df-43f3-887f-d9edb8c22189 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1704bb47a974-7331-40ab-b585-4011a5b6c91f | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 1705c8956f04-873c-48bf-bcbb-e102c47b20a9 | Translation Approved (100%) | None. | Nenhum. |
| 17063e6904bb-dbd1-4714-ba1a-1a55f7363914 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 1707cb1e42d5-5e89-4b80-977c-3c30cbeb0ef5 | Translation Approved (0%) | Divert at least 70% of the waste (by weight or volume) generated by facility maintenance and renovation activities from disposal in landfills and incinerators. | Desviar pelo menos 70% do resíduo (por peso ou volume) gerado por atividades de manutenção e reforma da instalação do descarte em aterros sanitários e incineradores. |
| 1708cb1e42d5-5e89-4b80-977c-3c30cbeb0ef5 | Translation Approved (77%) | Include base building elements as specified in the Materials and Resources prerequisite: Facility Maintenance and Renovation Policy. | Inclua os elementos do edifício base conforme especificado no Pré-requisito MR: Política de Manutenção e Reforma de Instalações (MR Prerequisite: Facility Maintenance and Renovation Policy) |
| 1709483f2277-1fdc-4b24-a8c7-ac6a6e30efed | Translation Approved (0%) | Exclude furniture and furnishings that pose human health concerns (e.g., mold) as well as components not considered base building elements; mechanical, electrical, and plumbing components; and specialty items, such as elevators. | Exclua os móveis e acessórios que apresentam preocupações relacionadas à saúde humana (por exemplo, mofo), bem como componentes não considerados elementos do edifício base; componentes mecânicos, elétricos e hidráulicos e itens especiais, como elevadores. |
| 1710d180cade-e78b-4cee-9ceb-890c3a6e3456 | Translation Approved (100%) | Indoor Environmental Quality (EQ) | Qualidade do ambiente interno (Indoor Environmental Quality – EQ) |
| 17118f0d9efd-1b3f-4c3f-b6bd-88f638797bdb | Translation Approved (CM) | EQ Prerequisite: Minimum Indoor Air Quality Performance | Pré-requisito EQ: Desempenho Mínimo da Qualidade do Ar Interno (EQ Prerequisite: Minimum Indoor Air Quality Performance) |
| 1712d517ddc7-80e6-4311-ac8f-25d7b0a2e0fe | Translation Approved (100%) | Required | Obrigatório |
| 1713506173d6-2942-4f09-9f08-ec87b99a1292 | Translation Approved (98%) | EB: O&M | EB: O&M |
| 171436973f7e-0ac5-4d0e-97bf-c5550b587a41 | Translation Approved (100%) | This prerequisite applies to | Este pré-requisito se aplica a: |
| 1715d95fb403-ea78-478a-add3-d637deb32689 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) |
| 1716685594df-a0f2-4ca6-bc05-8ac4e19f2e58 | Translation Approved (100%) | EB:O&M Schools | EB:O&M Escolas (Schools) |
| 1717d9f331ee-982f-41f0-a291-60e4196c77b7 | Translation Approved (100%) | EB:O&M Retail | EB:O&M Varejo (Retail) |
| 17185fc3d045-54b5-4e5b-8fff-76c0b04ddd08 | Translation Approved (100%) | EB:O&M Data Centers | EB:O&M Centros de Dados (Data Centers) |
| 1719b57ee557-5427-4003-8abc-7c6e037f65bc | Translation Approved (100%) | EB:O&M Hospitality | EB:O&M Hoteleiro (Hospitality) |
| 172045447447-e8e0-4c91-b7c9-3f9a247bd87e | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) |
| 17217e68ba36-b72a-47a6-87e5-0e0fecf4379f | Translation Approved (100%) | Intent | Objetivo |
| 17228289a2ee-f9f6-4145-a3a5-a6f1b53bdce3 | Translation Approved (CM) | To contribute to the comfort and well-being of building occupants by establishing minimum standards for indoor air quality (IAQ). | Contribuir para o conforto e bem-estar dos ocupantes do edifício estabelecendo padrões mínimos para a qualidade do ar interno (QAI). |
| 172310ea4d39-7f2e-4ddf-a9b9-a956177d3543 | Translation Approved (100%) | Requirements | Requisitos |
| 17249a7993d1-198f-4be5-8060-6a76e72f51db | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1725a1d290f4-2b2d-4874-9892-cef5d26c3cac | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 1726122e2bd4-ff3c-4c92-9481-fa95c77fc4b8 | Translation Approved (100%) | Each air-handling unit in the building must comply with either Case 1 or Case 2. | Todas as unidades de tratamento do ar devem estar em conformidade com o Caso 1 ou o Caso 2. |
| 1727122e2bd4-ff3c-4c92-9481-fa95c77fc4b8 | Translation Approved (0%) | If some air-handling units can provide the outdoor airflow required by Case 1 and others cannot, those that can must do so. | Se algumas unidades de tratamento de ar forem capazes de propiciar a vazão de ar externo exigido pelo Caso 1 e outras não forem capazes, as que forem capazes devem ser responsáveis por fornecer tal fluxo. |
| 1728b15c31f8-b4d8-43ae-bd84-a65660e3d138 | Translation Approved (100%) | Mechanically Ventilated Spaces | Espaços ventilados mecanicamente |
| 172949c28ee5-6d62-4d29-a996-4c1cc0a3d60b | Translation Approved (97%) | For mechanically ventilated spaces (and for mixed-mode systems when the mechanical ventilation is activated), chose one of the following: | Para espaços ventilados mecanicamente (e para sistemas de modo misto quando a ventilação mecânica está ativada), escolher um dos casos a seguir. |
| 17306fe56abc-b5bc-4c45-97df-ad88c76d2479 | Translation Approved (100%) | Case 1. | Caso 1. |
| 17316fe56abc-b5bc-4c45-97df-ad88c76d2479 | Translation Approved (CM) | Systems Able to Meet Required Outdoor Airflow Rates | Sistemas capazes de alcançar as vazões de ar externo |
| 17324ee4b2db-98f3-4340-9074-0417d4519173 | Translation Approved (100%) | Option 1. | Opção 1. |
| 17334ee4b2db-98f3-4340-9074-0417d4519173 | Translation Approved (CM) | ASHRAE Standard 62.1-2010 | Norma ASHRAE 62.1-2010 |
| 1734f640c9fc-52fa-4ba1-8de2-8a2028e12663 | Translation Approved (0%) | Modify or maintain each outdoor air intake, supply air fan, and ventilation distribution system to meet the outdoor air intake flow rates, using the ASHRAE ventilation rate procedure or a local equivalent, whichever is more stringent and meet the minimum requirements of ASHRAE Standard 62.1–2010, Sections 4–7, Ventilation for Acceptable Indoor Air Quality (with errata), or a local equivalent, whichever is more stringent. | Modifique ou mantenha cada admissão de ar externo, fornecer ventilador e sistema de distribuição de ventilação para atingir as taxas de admissão de ar externo, usando o procedimento de taxa de ventilação ASHRAE ou equivalente local, o que for mais rigoroso e atender os requisitos mínimos da Norma ASHRAE 62.1–2010, Seções 4 a 7, Ventilação para qualidade do ar interno aceitável (Ventilation for Acceptable Indoor Air Quality) (com errata), ou um equivalente local, o que for mais rigoroso. |
| 173592105b5e-ca1e-496c-b3dc-9d7ff2c1d77d | Translation Approved (100%) | Option 2. | Opção 2. |
| 173692105b5e-ca1e-496c-b3dc-9d7ff2c1d77d | Translation Approved (0%) | CEN Standards EN 15251–2007 and EN 13779–2007 | Normas CEN EN 15251–2007 e EN 13779–2007 |
| 17370a3cb584-304d-4d06-9e4d-1b155e6cc4f4 | Translation Approved (CM) | Projects outside the U.S. may instead meet the minimum outdoor air requirements of Annex B of Comité Européen de Normalisation (CEN) Standard EN 15251–2007, Indoor environmental input parameters for design and assessment of energy performance of buildings addressing indoor air quality, thermal environment, lighting and acoustics; and meet the requirements of CEN Standard EN 13779–2007, Ventilation for nonresidential buildings, Performance requirements for ventilation and room conditioning systems, excluding Section 7.3, Thermal environment; 7.6, Acoustic environment; A.16; and A.17. | Projetos fora dos EUA podem atender aos requisitos mínimos de ar externo do Anexo B do Comitê Europeu de Normalização (CEN), Norma EN 15251–2007, Parâmetros de entrada do ambiente interno, para projeto e avaliação do desempenho energético de edifícios que tratam da qualidade do ar interno, ambiente térmico, iluminação e acústica; e atender aos requisitos da Norma CEN EN 13779–2007, Ventilação para edifícios não residenciais, Requisitos de desempenho para sistemas de ventilação e condicionamento de ambientes, exceto a Seção 7.3, Ambiente térmico; 7.6, Ambiente acústico; A.16 e A.17. |
| 1738ea8d819a-e3c6-479d-ba70-04628a6cc961 | Translation Approved (100%) | Case 2. | Caso 2. |
| 1739ea8d819a-e3c6-479d-ba70-04628a6cc961 | Translation Approved (CM) | Systems Unable to Meet Required Outdoor Airflow Rates | Sistemas incapazes de alcançar as vazões de ar externo |
| 1740594d4b16-3447-4393-9177-129bd16bc530 | Translation Approved (CM) | If meeting the outdoor airflow rates in Case 1 is not feasible because of the physical constraints of the existing ventilation system, complete an engineering assessment of the system’s maximum outdoor air delivery rate. | Se alcançar as vazões de ar externo no Caso 1 não for viável devido a restrições físicas do sistema de ventilação existente, realize uma avaliação de engenharia da taxa máxima de distribuição de ar externo do sistema. |
| 1741594d4b16-3447-4393-9177-129bd16bc530 | Translation Approved (CM) | Supply the maximum possible to reach the minimum setpoint in Case 1 and not less than 10 cubic feet per minute (5 liters per second) of outdoor air per person. | Forneça o máximo possível para alcançar o ponto de ajuste mínimo no Caso 1 e não menos do que 10 pés cúbicos por minuto (5 litros por segundo) de ar externo por pessoa. |
| 17421bf243a3-d931-4d87-96bd-41c14ccebce8 | Translation Approved (100%) | Naturally Ventilated Spaces | Espaços ventilados naturalmente |
| 17438e2222cc-3626-49a2-809d-bc013a8e9ba8 | Translation Approved (CM) | For naturally ventilated spaces (and for mixed-mode systems when the mechanical ventilation is inactivated), determine the minimum outdoor air opening and space configuration requirements using the natural ventilation procedure from ASHRAE Standard 62.1–2010 or a local equivalent, whichever is more stringent. | Para espaços ventilados naturalmente (e para sistemas de modo misto quando a ventilação mecânica está desativada), determine os requisitos mínimos de configuração de abertura de ar externo e espaço, utilizando o procedimento de ventilação natural da Norma ASHRAE 62.1–2010 ou norma local equivalente, a que for mais rigorosa. |
| 17448e2222cc-3626-49a2-809d-bc013a8e9ba8 | Translation Approved (CM) | Confirm that natural ventilation is an effective strategy for the project by following the flow diagram in the Chartered Institution of Building Services Engineers (CIBSE) Applications Manual AM10, March 2005, Natural Ventilation in Nondomestic Buildings, Figure 2.8 and meet the requirements of ASHRAE Standard 62.1–2010, Section 4, or a local equivalent, whichever is more stringent. | Confirme se a ventilação natural é uma estratégia eficaz para o projeto, seguindo o diagrama de fluxo do Manual de Aplicações AM10 do CIBSE (Chartered Institution of Building Services Engineers), de março de 2005, Ventilação natural em edifícios não residenciais, Figura 2.8, e atenda aos requisitos da Norma ASHRAE 62.1–2010, Seção 4, ou norma local equivalente, a que for mais rigorosa. |
| 17459bdc3a8d-2016-4c61-b354-9d82fc5ce049 | Translation Approved (CM) | All Spaces | Todos os espaços |
| 1746e73dc45f-e618-44e8-8d27-c5a05275b953 | Translation Approved (CM) | The indoor air quality procedure defined in ASHRAE Standard 62.1–2010 may not be used to comply with this prerequisite. | O procedimento de qualidade do ar interno definido na Norma ASHRAE 62.1–2010 não pode ser usado para cumprir este pré-requisito. |
| 17474735f31f-955a-4588-8c06-4caf53c5dd2c | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 1748fd86e1e1-ebe2-4f97-bee4-8f0a8715f977 | Translation Approved (0%) | Show compliance through measurements taken at the system level within five years of the end of the performance period. | Mostre conformidade por meio de medidas realizadas no nível do sistema em até cinco anos após o final do período de desempenho. |
| 1749f756fa1d-3e9a-4751-b5da-6e1ed9dcf632 | Translation Approved (0%) | Implement and maintain an HVAC system maintenance program, based on ASHRAE 62.1–2010, Section 8, or a local equivalent, whichever is more stringent, to ensure the proper operations and maintenance of HVAC components as they relate to outdoor air introduction and exhaust. | Implemente e manter um programa de manutenção do sistema HVAC, com base na ASHRAE 62.1–2010, Seção 8 ou um equivalente local, o que for mais rígido, para garantir as operações e manutenção corretas dos componentes HVAC com relação à entrada e ao escape de ar externo. |
| 17505878a7f4-2b11-48e8-886d-91ea0b58a8f8 | Translation Approved (100%) | EQ Prerequisite: Environmental Tobacco Smoke Control | Pré-requisito EQ: Controle de Fumaça Ambiental do Tabaco (EQ Prerequisite: Environmental Tobacco Smoke Control) |
| 17519587a9bb-b6b4-4438-8da7-52ee2eb57bd1 | Translation Approved (100%) | Required | Obrigatório |
| 1752c27a0b01-892a-4df0-bda2-92e554f936f8 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 1753805166e7-354b-4c19-b774-300371346162 | Translation Approved (100%) | This prerequisite applies to | Este pré-requisito se aplica a: |
| 17544174d0eb-45f0-4a2f-af4a-0d769bf930b2 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) |
| 1755c11c3e12-e2c9-4142-87da-c5f0cf0acc03 | Translation Approved (100%) | EB:O&M Schools | EB:O&M Escolas (Schools) |
| 1756f2628b09-fda3-4d4c-a3e0-8cca8c5ad371 | Translation Approved (100%) | EB:O&M Retail | EB:O&M Varejo (Retail) |
| 1757d0a63cce-d1af-411a-bc9b-fa3e8142868e | Translation Approved (100%) | EB:O&M Data Centers | EB:O&M Centros de Dados (Data Centers) |
| 175869894c5d-2932-4494-8b0b-19875f24e293 | Translation Approved (100%) | EB:O&M Hospitality | EB:O&M Hoteleiro (Hospitality) |
| 17590feef08d-5ce9-4210-aed6-d8ad51622c5f | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) |
| 1760f7c4c448-ce7d-4f43-abcd-378466dc36be | Translation Approved (100%) | Intent | Objetivo |
| 1761536842ea-58a8-4926-8f36-4ed634f52a90 | Translation Approved (CM) | To prevent or minimize exposure of building occupants, indoor surfaces, and ventilation air distribution systems to environmental tobacco smoke. | Evitar ou minimizar a exposição de ocupantes do edifício, superfícies internas e sistemas de distribuição do ar de ventilação à fumaça ambiental do tabaco. |
| 17621e977e53-d8f1-4593-8a30-974e6b5afe09 | Translation Approved (100%) | Requirements | Requisitos |
| 1763bd57520a-649b-4239-8e2f-047fb6a16273 | Translation Approved (92%) | EBOM, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1764e8686a47-f2c5-477b-b35c-c688e890d202 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 1765f49738b0-9997-4514-99d0-218e74172258 | Translation Approved (84%) | Prohibit smoking in the building. | Proíba o fumo no edifício. |
| 1766488704b7-d9fa-4e23-a78c-d1e2a4f0e151 | Translation Approved (100%) | Prohibit smoking outside the building except in designated smoking areas located at least 25 feet (7.5 meters) from all entries, outdoor air intakes, and operable windows. | Proíba o fumo fora do edifício, exceto em áreas designadas localizadas a pelo menos 25 pés (7,5 metros) de todas as entradas, entradas de ar externo e janelas operáveis. |
| 1767488704b7-d9fa-4e23-a78c-d1e2a4f0e151 | Translation Approved (CM) | Also prohibit smoking outside the property line in spaces used for business purposes. | Também proíba o fumo fora da linha de propriedade em espaços usados para fins comerciais. |
| 17682e0cec04-9ab2-460a-821a-24849a7943ec | Translation Approved (CM) | If the requirement to prohibit smoking within 25 feet (7.5 meters) cannot be implemented because of code, provide documentation of these regulations. | Se o requisito de proibição de fumo em até 25 pés (7,5 metros) não puder ser implementado devido a códigos, forneça documentação desses regulamentos. |
| 1769854d2c28-c310-4b3e-a454-f2492225ab37 | Translation Approved (CM) | Signage must be posted within 10 feet (3 meters) of all building entrances indicating the no-smoking policy. | Deve haver sinalização a 10 pés (3 metros) de todas as entradas do edifício indicando a política de proibição de fumo. |
| 17704e992991-df48-47ea-9c33-8dbd7e105f72 | Translation Approved (100%) | Residential only | Apenas residencial |
| 1771313bec61-743b-4f6c-af5c-9a3915a581c4 | Translation Approved (100%) | Option 1. | Opção 1. |
| 1772313bec61-743b-4f6c-af5c-9a3915a581c4 | Translation Approved (CM) | No Smoking | Proibido fumar |
| 1773e196620c-8910-4a03-a9ab-23cce55aecf2 | Translation Approved (CM) | Meet the requirements above. | Atenda aos requisitos acima. |
| 17740ab8af33-7316-49dc-9791-14710586139a | Translation Approved (100%) | OR | OU |
| 17756a9d0d72-7a2b-4a9d-a021-1d3b537c7fa4 | Translation Approved (100%) | Option 2. | Opção 2. |
| 17766a9d0d72-7a2b-4a9d-a021-1d3b537c7fa4 | Translation Approved (CM) | Compartmentalization of Smoking Areas | Compartimentalização de áreas para fumantes |
| 177762092f29-0fa6-41c4-9870-0c3d1447bbf5 | Translation Approved (93%) | Prohibit smoking in all common areas of the building. | Proíba fumar em todas as áreas comuns do edifício. |
| 177862092f29-0fa6-41c4-9870-0c3d1447bbf5 | Translation Approved (100%) | The prohibition must be communicated in building rental or lease agreements or condo or coop association covenants and restrictions. | A proibição deve ser comunicada em contratos de locação do edifício ou em acordos e restrições da associação do condomínio ou cooperativa. |
| 177962092f29-0fa6-41c4-9870-0c3d1447bbf5 | Translation Approved (CM) | Make provisions for enforcement. | Tome medidas para impor a política. |
| 178048869c81-d769-46c4-b2df-604bbccfbf9f | Translation Approved (99%) | Prohibit smoking outside the building except in designated smoking areas located at least 25 feet (7.5 meters) from all entries, outdoor air intakes, and operable windows. | Proíba o fumo fora do edifício, exceto em áreas designadas localizadas a pelo menos 25 pés (7,5 metros) de todas as entradas, entradas de ar externo e janelas operáveis. |
| 178148869c81-d769-46c4-b2df-604bbccfbf9f | Translation Approved (CM) | The no-smoking policy also applies to spaces outside the property line used for business purposes. | A política de proibição do fumo também se aplica a espaços fora da linha da propriedade usados para fins comerciais. |
| 178262311e08-7007-4f40-a6a1-605a8efb6830 | Translation Approved (100%) | If the requirement to prohibit smoking within 25 feet (7.5 meters) cannot be implemented because of code, provide documentation of these regulations. | Se o requisito de proibição de fumo em até 25 pés (7,5 metros) não puder ser implementado devido a códigos, forneça documentação desses regulamentos. |
| 17839fd01d28-0a52-4d5a-a1a2-d94ef3b95f12 | Translation Approved (100%) | Signage must be posted within 10 feet (3 meters) of all building entrances indicating the no-smoking policy. | Deve haver sinalização a 10 pés (3 metros) de todas as entradas do edifício indicando a política de proibição de fumo. |
| 17841e7689f1-63e7-4df7-9853-5717bb20f8a8 | Translation Approved (CM) | Each unit must be compartmentalized to prevent excessive leakage between units: | Cada unidade deve ser compartimentalizada para evitar vazamento excessivo entre unidades: |
| 1785d9a9e360-4f5f-47d6-95d2-b133e488655e | Translation Approved (CM) | Weather-strip all exterior doors and operable windows in the residential units to minimize leakage from outdoors. | Vede com fita adesiva todas as portas externas e janelas operáveis nas unidades residenciais para minimizar vazamentos vindos de áreas externas. |
| 17868a69e7df-4d47-4ec3-8df4-5634257fc26f | Translation Approved (CM) | Weather-strip all doors leading from residential units into common hallways. | Vede com fita adesiva todas as portas que levam de unidades residenciais para pontos de circulação comuns. |
| 1787f5158c01-f57d-4eb5-a225-d27ccfbdac78 | Translation Approved (CM) | Minimize uncontrolled pathways for the transfer of smoke and other indoor air pollutants between residential units by sealing penetrations in the walls, ceilings, and floors and by sealing vertical chases (including utility chases, garbage chutes, mail drops, and elevator shafts) adjacent to the units. | Minimize caminhos não controlados para a transferência de fumaça e outros poluentes do ar interno entre unidades residenciais vedando penetrações em paredes, tetos e pisos e vedando espaços verticais em paredes (incluindo espaços para serviços públicos, calhas para lixo, caixas de correio e poços de elevadores) adjacentes às unidades. |
| 1788d216d15b-afb7-4c12-bbf0-3e28b786f529 | Translation Approved (88%) | Demonstrate a maximum leakage of 0.50 cubic feet per minute per square foot (2.54 liters per second per square meter) at 50 Pa of enclosure (i.e., all surfaces enclosing the apartment, including exterior and party walls, floors, and ceilings) or establish a baseline for a future 30% improvement. | Demonstre um vazamento máximo de 0,50 pé cúbicos por minuto por pé quadrado (2,54 litro por segundo por metro quadrado) a 50 Pa de invólucro (isto é, superfícies que envolvem o apartamento, incluindo paredes, pisos e tetos externos e divisórias) ou estabeleça uma baseline para uma melhoria futura de 30%. |
| 17890c5c72bc-a251-43d2-8d94-faa3aff2912b | Translation Approved (100%) | Schools | Escolas (Schools) |
| 179055817e01-cc65-4d92-9803-28a320f37514 | Translation Approved (CM) | Prohibit smoking on site. | Proíba fumar no terreno. |
| 1791cca90067-29d8-4a04-9a82-22668be8aed9 | Translation Approved (CM) | Signage must be posted at the property line indicating the no-smoking policy. | Deve haver sinalização na linha da propriedade indicando a política de proibição do fumo. |
| 1792a4969936-72c6-41fb-86d4-9ea518a92c22 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 17931988e5ab-e2a6-4d66-84de-063e4ee66e79 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1794dae1a2ea-af92-4da4-a9f8-bcde8cedfdca | Translation Approved (100%) | None. | Nenhum. |
| 17956aecedc1-81e2-4850-b15b-b1133111119a | Translation Approved (100%) | Residential only | Apenas residencial |
| 17961ad48bd6-1438-45b5-b8a3-830f7be24f0f | Translation Approved (87%) | Demonstrate on a regular basis (at least once every five years) a maximum leakage of 0.50 cubic feet per minute per square foot (2.54 liters per second per square meter) at 50 Pa of enclosure (i.e., all surfaces enclosing the apartment, including exterior and party walls, floors, and ceilings). | Demonstre regulamente (pelo menos uma vez a cada cinco anos) um vazamento máximo de 0,50 pé cúbicos por minuto por pé quadrado (2,54 litro por segundo por metro quadrado) a 50 Pa de invólucro (isto é, superfícies que envolvem o apartamento, incluindo paredes, pisos e tetos externos e divisórias). |
| 17975cc886ed-be0c-4a9b-b593-d5a983996251 | Translation Approved (0%) | Projects that do not meet the leakage requirement may demonstrate a 30% improvement over the most recent baseline. | Projetos que não atenderem ao requisito de vazamento podem demonstrar uma melhoria de 30% em relação à baseline mais recente. |
| 17985cc886ed-be0c-4a9b-b593-d5a983996251 | Translation Approved (0%) | The current measurement establishes the new baseline. | A medida atual estabelece a nova baseline. |
| 179998b77e72-4d05-4766-b7a2-06b6f86b535f | Translation Approved (92%) | EQ Prerequisite: Green Cleaning Policy | Pré-requisito EQ: Política de Limpeza Verde (EQ Prerequisite: Green Cleaning Policy). |
| 180012d4d53f-d5e6-418f-9a6f-7e3ab5421f79 | Translation Approved (100%) | Required | Obrigatório |
| 180111bf6c77-8916-49f4-ba4e-dd87294ec7e5 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 180270aae780-e7bb-4ef5-9251-a7cfaf96a33c | Translation Approved (100%) | This prerequisite applies to | Este pré-requisito se aplica a: |
| 1803a419cf86-6e2c-4ffb-8daf-2954db7767f1 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) |
| 180442a2f1f5-d98f-4829-9209-d141e8efcad8 | Translation Approved (100%) | EB:O&M Schools | EB:O&M Escolas (Schools) |
| 18052c21e577-b1f2-4e9d-9d72-1054c0fb2cb3 | Translation Approved (100%) | EB:O&M Retail | EB:O&M Varejo (Retail) |
| 1806c0dba957-c781-4ba5-9572-52738b5d9e69 | Translation Approved (100%) | EB:O&M Data Centers | EB:O&M Centros de Dados (Data Centers) |
| 1807e9d29a1f-2ce5-4549-ae91-6687dfad5b25 | Translation Approved (100%) | EB:O&M Hospitality | EB:O&M Hoteleiro (Hospitality) |
| 18086ae4a152-83c3-4575-9a99-5ed2f4976afb | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) |
| 1809ed73bfd8-e76d-4f5d-a139-cbcdc97b933e | Translation Approved (100%) | Intent | Objetivo |
| 181055627e6c-2c89-4e54-97c2-f5412b622d7d | Translation Approved (71%) | To reduce levels of chemical, biological, and particulate contaminants that can compromise air quality, human health, building finishes, building systems, and the environment. | Reduzir os níveis de contaminantes químicos, biológicos e particulados que podem comprometer a qualidade do ar, a saúde humana, acabamentos e sistemas de edifícios e o meio ambiente. |
| 1811504e32f1-be89-4097-8c67-e7293d41cb6a | Translation Approved (100%) | Requirements | Requisitos |
| 181220ebf1dd-1f97-4d82-99f7-61864a3b5126 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1813cdf61124-6636-4f56-8a83-6b8e41290ae5 | Translation Approved (100%) | Option 1. | Opção 1. |
| 1814cdf61124-6636-4f56-8a83-6b8e41290ae5 | Translation Approved (79%) | In-House Green Cleaning Policy | Política de limpeza verde interna |
| 1815b7ca58cf-fa4e-4b7e-8225-4814e73ce09f | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 1816699557d4-1864-4028-b495-7a3e42709cba | Translation Approved (0%) | Have in place a green cleaning policy for the building and site addressing the green cleaning credits, goals and strategies, and personnel listed below. | Implemente uma política de limpeza verde para o edifício e o terreno abordando os créditos de limpeza verde, as metas e estratégias e os pessoal listados abaixo. |
| 1817699557d4-1864-4028-b495-7a3e42709cba | Translation Approved (0%) | At a minimum, the policy must cover green cleaning procedures, materials, and services that are within the building and site management’s control, and include the organization responsible for cleaning the building and building site. | No mínimo, a política deve cobrir os procedimentos, materiais e serviços de limpeza verde que fazem parte do controle da gestão do edifício e do terreno e incluir a organização responsável pela limpeza do edifício e do terreno do edifício. |
| 18187731dd5f-a294-4766-b879-df47fdbd5b2f | Translation Approved (0%) | Address the requirements of the following credits: | Aborde os requisitos dos seguintes créditos: |
| 1819d6ba9c4f-dece-4a9b-a282-b7005f11c145 | Translation Approved (79%) | EQ Credit: Green Cleaning—Purchase of Cleaning Products and Materials | Crédito EQ: Limpeza Verde – Compra de Produtos e Materiais de Limpeza (EQ Credit: Green Cleaning—Purchase of Cleaning Products and Materials) |
| 1820d24d4ebb-3ff4-438b-844f-70773a67df76 | Translation Approved (0%) | EQ Credit: Green Cleaning—Cleaning Equipment | Crédito EQ: Limpeza Verde – Equipamentos de Limpeza (EQ Credit: Green Cleaning—Cleaning Equipment) |
| 182143627e1d-5e7d-4b51-ae95-4c1d3accc985 | Translation Approved (0%) | Goals and Strategies | Metas e estratégias |
| 1822d107fc0c-4545-48b6-a35a-42c9c4bd0372 | Translation Approved (82%) | Establish standard operating procedures addressing how an effective cleaning and hard floor and carpet maintenance system will be consistently used, managed, and audited. | Estabeleça procedimentos padrão de operação para definindo como a limpeza eficaz, pisos duros e o sistema de manutenção de carpetes serão utilizados, controlados e auditados de forma consistente. |
| 1823bd2f40f9-31d9-4148-aa9a-6370fb62659e | Translation Approved (0%) | Address protection of vulnerable building occupants during cleaning. | Defina a proteção de ocupantes vulneráveis do edifício durante a limpeza. |
| 18244ff5bed3-74f0-469e-849a-17e3beb3d42e | Translation Approved (0%) | Address selection and appropriate use of disinfectants and sanitizers. | Defina a seleção e o uso correto de desinfetantes e higienizadores. |
| 1825d43ba884-3246-4905-bf37-aa4420ff8905 | Translation Approved (89%) | Develop guidelines addressing the safe handling and storage of cleaning chemicals used in the building, including a plan for managing hazardous spills and mishandling incidents. | Desenvolva diretrizes que abordam o manuseio e o armazenamento seguros dos produtos químicos de limpeza usados no edifício, incluindo um plano para a administração de vazamentos perigosos ou tratamento incorreto de incidentes. |
| 18263dc838d7-df58-4418-a184-7d9569511772 | Translation Approved (0%) | Develop goals and strategies for reducing the toxicity of the chemicals used for laundry, ware washing, and other cleaning activities. | Desenvolva metas e estratégias para redução da toxicidade dos produtos químicos usados na lavagem de roupa, lavagem de louça e outras atividades de limpeza. |
| 182778071d5a-a768-42a0-80a3-2b7aaeefca79 | Translation Approved (0%) | Develop goals and strategies for promoting the conservation of energy, water, and chemicals used for cleaning. | Desenvolva metas e estratégias para promover a conservação de energia, água e produtos químicos usados para limpeza. |
| 1828e177217f-40a7-431c-b1be-0c0871f8dd89 | Translation Approved (80%) | Develop strategies for promoting and improving hand hygiene. | Desenvolva estratégias para promover e melhorar a higiene das mãos |
| 18297e96e73f-f3c2-4df6-be70-03d259f45116 | Translation Approved (0%) | Personnel | Pessoal |
| 18301ad217f4-e258-491b-a6c4-25dcbb4c3f80 | Translation Approved (0%) | Develop requirements for maintenance personnel. | Desenvolva requisitos para o pessoal de manutenção. |
| 18311ad217f4-e258-491b-a6c4-25dcbb4c3f80 | Translation Approved (0%) | Specifically address contingency planning to manage staffing shortages under a variety of conditions to ensure that basic cleaning services are met and critical cleaning needs are addressed. | Defina especificamente um plano de contingência para gerenciar a escassez de funcionários devido a diversas condições para garantir que os serviços básicos de limpeza sejam realizados e as necessidades críticas de limpeza sejam atendidas. |
| 18321ad217f4-e258-491b-a6c4-25dcbb4c3f80 | Translation Approved (0%) | Include a process to obtain occupant and custodial staff input and feedback after contingency plans are implemented. | Inclua um processo para obter informações e feedback de funcionários de vigilância e limpeza e de ocupantes após os planos de contingência terem sido implementados. |
| 183306c513ae-21f4-445b-9f8c-75b60ea90954 | Translation Approved (82%) | Determine the timing and frequency of training for maintenance personnel in the hazards of use, disposal, and recycling of cleaning chemicals, dispensing equipment, and packaging. | Determine o calendário e a frequência do treinamento de funcionários de manutenção quanto aos perigos do uso, descarte e na reciclagem de produtos químicos de limpeza, equipamentos de distribuição e embalagens. |
| 183465241091-4bff-4fbb-8d2d-ddf29281e5fd | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 1835edc4bf8a-00f5-4ef9-9873-8ae0dd0045a0 | Translation Approved (0%) | Implement a high-performance cleaning program based on the above policy and track performance goals associated with this policy. | Implemente um programa de limpeza de alto desempenho com base na política acima e monitorar as metas de desempenho associadas a essa política. |
| 1836e7896ab1-0011-45fe-a724-da3f71895717 | Translation Approved (100%) | Option 2. | Opção 2. |
| 1837e7896ab1-0011-45fe-a724-da3f71895717 | Translation Approved (0%) | Certified Cleaning Service | Serviço de limpeza certificado |
| 18381bb1b1eb-36f8-4f3b-a359-7f8bc18ddbee | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 18392a2f52af-1e54-419c-9a6a-e9bb54e085f9 | Translation Approved (100%) | None. | Nenhum. |
| 18409e7a4fd9-745d-4560-9a28-fb7be8fc64a4 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 18414e892542-bdbd-481e-b961-3457ab71f96e | Translation Approved (0%) | Clean the building with a cleaning service provider, either in-house custodial staff or a contracted service contractor, certified under one of the following: | Limpe o edifício com um provedor de serviços de limpeza, seja os funcionários de vigilância e limpeza internos ou uma empresa de serviços contratada, certificada com um dos seguinte: |
| 184226cb2f79-41af-4936-9bd8-fffe3017231a | Translation Approved (0%) | Green Seal’s Environmental Standard for Commercial Cleaning Services (GS-42); or | Norma ambiental para serviços comerciais de limpeza do Green Seal (Green Seal’s Environmental Standard for Commercial Cleaning Services) (GS-42); ou |
| 1843ab6a36d9-1271-4749-9ab6-5dd351463eab | Translation Approved (0%) | International Sanitary Supply Association (ISSA) Cleaning Industry Management Standard for Green Buildings (CIMS-GB); or | Norma de gestão do setor de limpeza para edifícios verdes (Cleaning Industry Management Standard for Green Buildings) (CIMS-GB) da ISSA (International Sanitary Supply Association); ou |
| 18446affb2fb-24c1-4b4d-b9ab-f62b5f728127 | Translation Approved (0%) | Local equivalent for projects outside the U.S. | Equivalente local para projetos fora dos EUA. |
| 184581d401cb-8e97-49b6-82ef-e46f693ea2c0 | Translation Approved (0%) | Confirm that the building or contractor was audited by the third party within 12 months of the end of the performance period. | Confirme se o edifício ou empresa passou por auditoria por terceiros em até 12 meses após o período de desempenho. |
| 1846ec2caa5d-7fdf-40a8-bc00-6e247dc66334 | Translation Approved (73%) | In addition, the cleaning contractor must develop goals and strategies for promoting the conservation of energy, water, and chemicals used for cleaning the building. | Além disso, a empresa de limpeza deve desenvolver metas e estratégias para promover a conservação de energia, água e produtos químicos usados para limpeza do edifício. |
| 18470f20b2f1-757a-4e92-8150-53a468cf1e38 | Translation Approved (94%) | EQ Credit: Indoor Air Quality Management Program | Crédito EQ: Programa de Gerenciamento da Qualidade do Ar Interno (EQ Credit: Indoor Air Quality Management Program) |
| 18480070c8df-c33b-4a45-b5e9-bb34cdf7ae1f | Translation Approved (100%) | EB:O&M | EB:O&M |
| 1849c4e4ac0f-e85f-4130-b9f8-6556222e0e44 | Translation Approved (100%) | 2 points | 2 pontos |
| 1850e9cdfefd-75c1-45bd-a566-23ba716b431b | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 18512a2d4c2e-1597-42d6-9448-6e4e5eb21ede | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (2 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (2 pontos) |
| 1852aabbc99e-38df-4bb6-b81c-b7f208db531e | Translation Approved (100%) | EB:O&M Schools (2 points) | EB:O&M Escolas (Schools) (2 pontos) |
| 185318247e24-fa08-4fbc-96b1-92a993df53ec | Translation Approved (100%) | EB:O&M Retail (2 points) | EB:O&M Varejo (Retail) (2 pontos) |
| 1854738d6ba9-5dad-42d3-99b2-53aee38f8239 | Translation Approved (100%) | EB:O&M Data Centers (2 points) | EB:O&M Centros de Dados (Data Centers) (2 pontos) |
| 1855c3c29c4b-63e3-43fb-82b9-1b37e0f20fa7 | Translation Approved (100%) | EB:O&M Hospitality (2 points) | EB:O&M Hoteleiro (Hospitality) (2 pontos) |
| 1856ba20bdb1-bd9d-462a-b419-37f4252da6e3 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (2 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (2 pontos) |
| 185703c30559-88b4-4218-a9e8-96ce7cdb7dc9 | Translation Approved (100%) | Intent | Objetivo |
| 1858837270ef-08ec-4fbc-b792-92a3749e8969 | Translation Approved (0%) | To maintain the well-being of occupants by preventing and correcting indoor air quality problems. | Manter o bem-estar dos ocupantes evitando e corrigindo os problemas de qualidade do ar interno. |
| 18594daed189-3b6e-432a-a0f7-7959b0768fc9 | Translation Approved (100%) | Requirements | Requisitos |
| 18603262074a-6ce1-4401-9bb1-750e94877444 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 18616938872a-9852-4369-a975-e1ee0081d4bb | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 186219170a29-1d3b-4ce1-a11d-0008a30b1fa8 | Translation Approved (88%) | Develop and implement an indoor air quality (IAQ) management program based on the EPA Indoor Air Quality Building Education and Assessment Model (I-BEAM). | Desenvolva e implemente um programa de gestão de qualidade do ar interno (QAI) com base no Modelo de educação e avaliação da qualidade do ar interno de edifícios (Indoor Air Quality Building Education and Assessment Model - I-BEAM) da EPA . |
| 186319170a29-1d3b-4ce1-a11d-0008a30b1fa8 | Translation Approved (79%) | Include the IAQ management program in the project’s current facilities requirements and operations and maintenance Plan. | Inclua o programa de gestão de IAQ nos requisitos atuais da instalação e no plano de manutenção e operações. |
| 1864c2a2b0ae-dfb9-461c-8cbc-c9d3678e72f8 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 18653c8b74c8-b53e-40c5-8269-3834fb6c7838 | Translation Approved (0%) | Conduct an I-BEAM audit on a regular basis (at least once every five years) and revise the IAQ management program as appropriate. | Conduza uma auditoria I-BEAM regularmente (pelo menos uma vez a cada cinco anos) e revisar o programa de gestão de QAI conforme adequado. |
| 1866055e1af5-e33d-4c35-a808-c48931e3ef2a | Translation Approved (100%) | EQ Credit: Enhanced Indoor Air Quality Strategies | Crédito EQ: Estratégias Aprimoradas de Qualidade do Ar Interno (EQ Credit: Enhanced Indoor Air Quality Strategies) |
| 1867b28a63ce-4198-4fee-a942-b52b345436fa | Translation Approved (100%) | EB:O&M | EB:O&M |
| 1868631b9cd0-cd52-450d-a287-78034efd4efe | Translation Approved (100%) | 1–2 points | 1 a 2 pontos |
| 18698a33d841-6325-4873-a660-5f226f648c1b | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 1870f41607cf-88f3-4142-8e81-14e32d1d6cca | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1–2 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 a 2 pontos) |
| 1871a13a73bd-4594-4a02-ab00-ff3ba212ac53 | Translation Approved (100%) | EB:O&M Schools (1–2 points) | EB:O&M Escolas (Schools) (1 a 2 pontos) |
| 187283f44fa8-2f89-4d35-add2-7185afcf45dd | Translation Approved (100%) | EB:O&M Retail (1–2 points) | EB:O&M Varejo (Retail) (1 a 2 pontos) |
| 18732d40884c-b292-4ff1-a61c-9b0e7134a92c | Translation Approved (100%) | EB:O&M Data Centers (1–2 points) | EB:O&M Centros de Dados (Data Centers) (1 a 2 pontos) |
| 18741af8e809-8463-4905-8aac-8de839bbd2d5 | Translation Approved (100%) | EB:O&M Hospitality (1–2 points) | EB:O&M Hoteleiro (Hospitality) (1 a 2 pontos) |
| 1875c0631b19-76a5-46f8-8bea-801da54228e2 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1–2 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 a 2 pontos) |
| 187670d83bfa-8070-40cb-b052-2752d2d516b6 | Translation Approved (100%) | Intent | Objetivo |
| 18779b59bae1-bfb7-4d8d-8cc4-c849ce759be1 | Translation Approved (CM) | To promote occupants’ comfort, well-being, and productivity by improving indoor air quality. | Promover o conforto, bem-estar e produtividade dos ocupantes melhorando a qualidade do ar interno. |
| 1878c5af2da4-d7b3-4adc-a8f2-d354150b5153 | Translation Approved (100%) | Requirements | Requisitos |
| 1879a6954504-52c3-41c0-91f3-40da0ad8bcc4 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 188072f702cf-5223-4654-95c3-deabbf46350e | Translation Approved (100%) | Option 1. | Opção 1. |
| 188172f702cf-5223-4654-95c3-deabbf46350e | Translation Approved (0%) | Entryway Systems (1 point) | Sistemas de entrada (1 ponto) |
| 188216a5ed23-0c76-4cdd-b1b1-caad08de9409 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 1883cbaac613-df26-4942-a2b3-d02ff38921fd | Translation Approved (95%) | Have in place permanent entryway systems at least 10 feet (3 meters) long in the primary direction of travel to capture dirt and particulates entering the building at regularly used exterior entrances. | Implemente sistemas de entrada permanentes com pelo menos 10 pés (3 metros) de comprimento na direção de deslocamento principal para capturar sujeira e partículas que entram no edifício por entradas externas usadas regularmente. |
| 1884cbaac613-df26-4942-a2b3-d02ff38921fd | Translation Approved (96%) | Acceptable entryway systems include permanently installed grates, grilles, slotted systems that allow for cleaning underneath, rollout mats, and any other materials manufactured as entryway systems with equal to or better performance. | Os sistemas de entrada aceitáveis incluem grelhas instaladas permanentemente, sistemas ranhurados que permitem a limpeza por baixo, capachos e qualquer outro material fabricado como sistema de entrada com desempenho igual ou superior. |
| 1885cbaac613-df26-4942-a2b3-d02ff38921fd | Translation Approved (100%) | Maintain all on a weekly basis. | Faça a manutenção de todos os sistemas semanalmente. |
| 18867e15943c-4860-469f-93ea-6aa478aa130d | Translation Approved (CM) | Warehouses & Distribution Centers only | Apenas Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1887ec133212-925b-4d0a-888f-82b0f1e131a4 | Translation Approved (0%) | Buildings are not required to provide entryway systems at doors leading from the exterior to the loading dock/garage, but must provide them between these spaces and adjacent office areas. | Não é necessário que os edifícios forneçam sistemas de entrada em portas que conduzem do exterior à plataforma de carga/garagem, mas devem fornecê-los entre esses espaços e áreas de escritórios adjacentes. |
| 1888eba6a530-8302-45d3-9c51-198aad51d0b3 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 1889d71582a1-2b41-4104-990a-96c465ba86cd | Translation Approved (0%) | Confirm that entryway systems have been maintained on a weekly basis. | Confirme se os sistemas de entrada receberam manutenção semanalmente. |
| 18905d637fdd-bd60-4d0a-9718-9c182fdd589a | Translation Approved (100%) | Option 2. | Opção 2. |
| 18915d637fdd-bd60-4d0a-9718-9c182fdd589a | Translation Approved (CM) | Additional Enhanced IAQ Strategies (1 point) | Estratégias de QAI aprimoradas adicionais (1 ponto) |
| 1892a7a94a2f-3fc8-4d48-90fb-f7be69060b0a | Translation Approved (0%) | Comply with the requirements of at least one of the following. | Cumpra os requisitos de pelo menos uma das opções a seguir. |
| 189327bd42b4-ea8b-4e22-8560-4a706e1491dd | Translation Approved (75%) | Filtration for Mechanically Ventilated Spaces | Filtração para espaços mecanicamente ventilados |
| 1894f2617512-42c0-41ab-8444-c0e2e695608a | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 1895152d536c-c630-411b-b4db-334ac1b16367 | Translation Approved (0%) | Each ventilation system that supplies outdoor air to occupied spaces must have particle filters or air cleaning devices. | Cada sistema de ventilação que fornece ar exterior a espaços ocupados devem ter filtros de partícula ou dispositivos de limpeza de ar. |
| 1896152d536c-c630-411b-b4db-334ac1b16367 | Translation Approved (0%) | These filters or devices must meet one of the following filtration media requirements: | Esses filtros ou dispositivos devem atender a um dos seguintes requisitos de meios filtrantes: |
| 189719e19496-421d-423a-9790-2798982c9982 | Translation Approved (0%) | minimum efficiency reporting value (MERV) of 13 or higher, in accordance with ASHRAE Standard 52.2–2007; | valor relatado de eficiência mínima (minimum efficiency reporting value - MERV) de 13, ou superior, conforme determinado pela Norma ASHRAE 52.2–2007; |
| 1898eb61c549-9e1c-4ac7-84b3-ffd8d7609485 | Translation Approved (100%) | Class F7 or higher as defined by CEN Standard EN 779–2002, Particulate Air Filters for General Ventilation, Determination of the Filtration Performance. | Classe F7 ou superior, conforme definido pela Norma CEN EN 779–2002, Filtros de particulado no ar para ventilação geral, Determinação do desempenho de filtração (Particulate Air Filters for General Ventilation, Determination of the Filtration Performance). |
| 18994465653a-734c-4a65-aac3-50f135b0434b | Translation Approved (0%) | Establish a regular schedule for maintenance and replacement of filtration media according to the manufacturer’s recommended interval. | Estabeleça um cronograma regular para manutenção e substituição de meios filtrantes de acordo com o intervalo recomendado pelo fabricante. |
| 19003e2242a3-c3e5-4f83-92c9-eb696f69c063 | Translation Approved (100%) | Data Centers only | Apenas Centros de Dados (Data Centers) |
| 1901f60db07d-2c31-4823-8fa6-b881e408c76a | Translation Approved (CM) | The above filtration media requirements are required only for ventilation systems serving regularly occupied spaces. | Os requisitos de meios filtrantes acima são necessários apenas para sistemas de ventilação que servem espaços ocupados regularmente. |
| 1902163fcd68-a6b7-4eed-af88-39e75357a980 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 1903fa86771e-2b57-4a1e-8be8-2802616dd185 | Translation Approved (0%) | Follow the schedule for maintenance and replacement of filtration media. | Siga a programação de manutenção e substituição de meios filtrantes. |
| 19045a7afecb-a5e2-4c53-b5b7-2e870c0abc45 | Translation Approved (77%) | Carbon Dioxide Monitors | Monitoramentos de dióxido de carbono |
| 19054de0d7f3-cfbc-4b39-ab75-7e67e2ec1fa6 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 19069a2fbe2c-3973-4ba9-bd7f-0858841e8980 | Translation Approved (0%) | Have in place CO<1758>2</1758> monitors in all densely occupied spaces. | Implemente monitoramentos de CO<1758>2</1758> em todos os espaços densamente ocupados. |
| 19079a2fbe2c-3973-4ba9-bd7f-0858841e8980 | Translation Approved (0%) | Rooms smaller than 150 square feet (14 square meters) are exempt. | Salas menores que 150 pés quadrados (14 metros quadrados) são isentas. |
| 19089a2fbe2c-3973-4ba9-bd7f-0858841e8980 | Translation Approved (100%) | CO<1759>2</1759> monitors must be between 3 and 6 feet (900 and 1 800 millimeters) above the floor. | Os monitores de CO<1759>2</1759> devem ficar entre 3 e 6 pés (900 e 1.800 milímetros) acima do piso. |
| 1909ab381f9c-9f09-4231-86c0-59b6e8df7a7a | Translation Approved (0%) | Configure the system to generate a visual alarm to the system operator if the differential CO<1760>2</1760> concentration in any zone rises more than 15% above that corresponding to the minimum outdoor air rate required in the ventilation section of EQ Prerequisite Minimum Indoor Air Quality Performance. | Configure o sistema para gerar um alarme visual ao operador do sistema se a concentração de CO<1758>2</1758> diferencial qualquer área aumentar mais de 15% acima da que corresponde à taxa mínima de ar exterior necessária na seção de ventilação do Pré-requisito EQ: Desempenho Mínimo da Qualidade do Ar Interno (EQ Prerequisite: Minimum Indoor Air Quality Performance). |
| 1910649d2594-c65f-4305-99a1-9c53928ec18a | Translation Approved (0%) | Test and calibrate CO<1761>2</1761> sensors to have an accuracy of no less than 75 parts per million or 5% of the reading, whichever is greater. | Teste e calibre os sensores de CO<1761>2</1761> para não ter uma precisão abaixo de 75 partes por milhão ou 5% da leitura, o que for maior. |
| 1911a2bab57a-70b8-42b6-8111-5ce82117a1f1 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 1912bfe2f7f1-4870-4825-9ec6-d166e8c59aa6 | Translation Approved (0%) | Sensors must be tested and calibrated at least once every five years or per the manufacturer’s recommendation, whichever is shorter. | Os sensores devem ser testados e calibrados pelo menos uma vez a cada cinco anos ou conforme recomendação do fabricante, qual tempo for mais curto. |
| 19132a104de0-4042-42b4-9c8c-9aeee7037716 | Translation Approved (0%) | Monitor CO<1763>2</1763> sensors with a system configured to trend CO<1764>2</1764> concentrations in intervals no greater than 30 minutes. | Monitore os sensores de CO<1761>2</1761> com um sistema configurado para mostrar as tendências de concentrações de CO<1761>2</1761> em intervalos de até 30 minutos. |
| 191425bf620f-4dc4-4e96-a453-9e6ab03d3cfe | Translation Approved (0%) | Outdoor Air Monitoring for Mechanically Ventilated Spaces | Monitoramento de ar externo para espaços mecanicamente ventilados |
| 1915ab1320b0-69b0-40bf-8992-ebd92f90a825 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 1916bb706f7d-8010-4188-bc2b-ce6999de5e5a | Translation Approved (0%) | For variable air volume systems, provide a direct outdoor airflow measurement device capable of measuring the minimum outdoor air intake flow for at least 80% of the outdoor air flow. | Para sistemas de volume de ar variável, fornecer um dispositivo de medição de vazão de ar exterior direto capaz de medir a vazão mínima de admissão de ar exterior por melo menos 80% da vazão de ar exterior. |
| 1917bb706f7d-8010-4188-bc2b-ce6999de5e5a | Translation Approved (0%) | This device must measure the minimum outdoor air intake flow with an accuracy of +/–10% of the design minimum outdoor airflow rate required in the ventilation section of EQ Prerequisite Minimum Indoor Air Quality Performance. | Esse dispositivo deve medir a vazão mínima de admissão de ar exterior com uma precisão de +/–10% da taxa mínima de vazão de ar exterior necessária na seção de ventilação do Pré-requisito EQ: Desempenho Mínimo da Qualidade do Ar Interno (EQ Prerequisite: Minimum Indoor Air Quality Performance). |
| 1918bb706f7d-8010-4188-bc2b-ce6999de5e5a | Translation Approved (0%) | An alarm must indicate when the outdoor airflow value varies by 15% or more from the outdoor airflow setpoint. | Um alarme deve indicar quando o valor de vazão de ar exterior variar em 15% ou mais do ponto de ajuste da vazão de ar exterior. |
| 1919410de500-98f4-4226-8bbb-3d36f9642e5b | Translation Approved (0%) | For constant-volume systems, balance outdoor airflow to the design minimum outdoor airflow rate required in the ventilation section of EQ Prerequisite Minimum Indoor Air Quality Performance, or higher. | Para sistemas de volume constante, equilibre a vazão de ar exterior à taxa mínima de vazão de ar exterior do projeto na seção de ventilação do Pré-requisito EQ: Desempenho Mínimo da Qualidade do Ar Interno (EQ Prerequisite: Minimum Indoor Air Quality Performance)., ou superior. |
| 1920410de500-98f4-4226-8bbb-3d36f9642e5b | Translation Approved (0%) | Install a current transducer on the supply fan, an airflow switch, or similar monitoring device. | Instalar um transdutor atual no ventilador de alimentação, um interruptor de vazão de ar ou um dispositivo de monitoramento similar. |
| 192197c1712f-b648-43c3-8f3d-ed21fccc343e | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 1922455cbfc8-624e-4364-9afd-88c50f992c92 | Translation Approved (0%) | Calibrate all measurement devices within the manufacturer’s recommended interval. | Calibrar todos os dispositivos de medição de acordo com o intervalo recomendados pelo fabricante. |
| 1923d3f58244-b5ea-4b9c-be1a-c68e04c10b4f | Translation Approved (91%) | Outdoor Air Monitoring for Naturally Ventilated Spaces | Monitoramento de ar exterior para espaços naturalmente ventilados |
| 1924e99c0a82-e75f-45aa-a24c-7d54b1bc03a1 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 19256f6c1e1e-c291-4d2f-8f03-b375c5e35b38 | Translation Approved (0%) | Provide a direct exhaust airflow measurement device capable of measuring the exhaust airflow. | Forneça um dispositivo de medição de vazão de ar de escape direto capaz de medir a vazão de ar de escape. |
| 19266f6c1e1e-c291-4d2f-8f03-b375c5e35b38 | Translation Approved (0%) | This device must measure the exhaust airflow with an accuracy of +/–10% of the design minimum exhaust airflow rate. | Esse dispositivo deve medir a vazão de ar de escape com uma precisão de +/–10% da taxa mínima de vazão de ar de escape do projeto. |
| 19276f6c1e1e-c291-4d2f-8f03-b375c5e35b38 | Translation Approved (88%) | An alarm must indicate when airflow values vary by 15% or more from the exhaust airflow setpoint. | Um alarme deve indicar quando o valor de vazão de ar variar em 15% ou mais do ponto de ajuste da vazão de ar de escape. |
| 19284e213053-b5b9-4472-baa2-faf63024eb30 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 1929eacb312c-b5a0-4bd6-afbf-1b56f8cca506 | Translation Approved (CM) | Calibrate all measurement devices within the manufacturer’s recommended interval. | Calibrar todos os dispositivos de medição de acordo com o intervalo recomendados pelo fabricante. |
| 1930475f52ab-d46a-46ac-a849-9864c05da0ae | Translation Approved (0%) | Alarmed Openings for Naturally Ventilated Spaces | Aberturas com alarme para espaços naturalmente ventilados |
| 1931e1c677ee-a425-4dad-9ab7-6a3d4523491b | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 1932fc063657-ac64-4742-83fd-f88c1c6dffdf | Translation Approved (0%) | Provide automatic indication devices on all openings intended to meet the minimum opening requirements. | Forneça dispositivos automáticos de indicação em todas as aberturas com o intuito de atender aos requisitos mínimos de abertura. |
| 1933fc063657-ac64-4742-83fd-f88c1c6dffdf | Translation Approved (0%) | An alarm must indicate when any one of the openings is closed during occupied hours. | Um alarme deve indicar quando qualquer uma das aberturas está fechada durante as horas ocupadas. |
| 1934eaaad80f-e7cb-4965-98c6-11184961b05d | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 19355cfd7043-e649-485c-b817-b1fd659c36cd | Translation Approved (100%) | None. | Nenhum. |
| 19362d8999d4-f4d6-46af-ae2a-999eb2f6c07e | Translation Approved (99%) | EQ Credit: Thermal Comfort | Crédito EQ: Conforto Térmico (EQ Credit: Thermal Comfort) |
| 1937db4328b1-5961-4bce-9a98-fbe79acdd84b | Translation Approved (100%) | EB:O&M | EB:O&M |
| 19383ebc3e46-e59e-4e42-96e4-baf72a2033b7 | Translation Approved (100%) | 1 point | 1 ponto |
| 193923b93eb2-e3b5-404c-b4d5-a96902e8f3cf | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 19402a1ed156-1ef9-4dc9-b0d1-46d05b3533d7 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1 point) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 ponto) |
| 1941b4a7aa39-73c0-4cd5-976d-da65260fd84c | Translation Approved (100%) | EB:O&M Schools (1 point) | EB:O&M Escolas (Schools) (1 ponto) |
| 1942d56fa2bb-4674-47d5-a007-da030baef0dc | Translation Approved (100%) | EB:O&M Retail (1 point) | EB:O&M Varejo (Retail) (1 ponto) |
| 1943a24e1f19-d997-46d0-bf9b-9bb26ddc1ae2 | Translation Approved (100%) | EB:O&M Data Centers (1 point) | EB:O&M Centros de Dados (Data Centers) (1 ponto) |
| 1944bf835594-e8cd-40a4-a94d-bbf46ab24533 | Translation Approved (100%) | EB:O&M Hospitality (1 point) | EB:O&M Hoteleiro (Hospitality) (1 ponto) |
| 194555c3d1e6-0ec8-4af8-a567-707a328ce389 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1 point) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 ponto) |
| 1946c21f15c8-f5b7-49c3-b883-1e41ca67ab26 | Translation Approved (100%) | Intent | Objetivo |
| 19478a5f1691-03d6-4841-b605-b760f7625e79 | Translation Approved (0%) | To promote occupants’ productivity, comfort, and well-being by providing quality thermal comfort. | Promover a produtividade, o conforto e o bem-estar dos ocupantes por meio do fornecimento de conforto térmico de qualidade. |
| 1948281081b8-5e1f-48e1-9b38-b8b7348e3e8c | Translation Approved (100%) | Requirements | Requisitos |
| 1949f554c611-a2ae-497e-987c-dfb8f04d983c | Translation Approved (0%) | Meet the requirements for both thermal comfort design and thermal comfort control. | Atender aos requisitos de projeto de conforto técnico e do controle de conforto térmico. |
| 19501930298d-f9b4-43eb-ad34-1dd873c0c8ce | Translation Approved (90%) | Thermal Comfort Design | Projeto de conforto térmico |
| 195125308536-ac02-4b00-8d73-7df8a36b2c34 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 195285a4ad19-dd72-48df-91cb-9a835a00cac0 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 1953f2bc1de3-d7c1-4085-92ad-388a4f9b460c | Translation Approved (0%) | Have in place a system for continuous tracking and optimization of systems that regulate indoor comfort and conditions (air temperature, radiant temperature, humidity, and air speed) in occupied spaces. | Implemente um sistema para monitorar e otimizar continuamente os sistemas que regulam o conforto e as condições internas (temperatura do ar, temperatura radiante, umidade e velocidade do ar) em espaços ocupados. |
| 1954116961fa-58c3-4dfa-b2d3-4ae82af57b03 | Translation Approved (100%) | Option 1. | Opção 1. |
| 1955116961fa-58c3-4dfa-b2d3-4ae82af57b03 | Translation Approved (0%) | ASHRAE 55–2010 | ASHRAE 55–2010 |
| 1956729401e1-acca-457a-a628-ce4caf5e33bc | Translation Approved (0%) | Have a permanent monitoring system to ensure ongoing building performance to the desired comfort criteria, as specified by ASHRAE Standard 55–2010, Thermal Comfort Conditions for Human Occupancy, Section 5.2 or 5.3, with errata, or a local equivalent. | Implemente um sistema de monitoração permanente para garantir o desempenho contínuo do edifício de acordo com os critérios de conforto desejados, conforme especificado pela Norma ASHRAE 55–2010, Condições de conforto térmico para ocupação humana (Thermal Comfort Conditions for Human Occupancy), Seção 5.2 ou 5.3 com errata, ou equivalente local. |
| 1957b33dfff6-d5e5-45bd-887b-2b140c8b07b4 | Translation Approved (100%) | OR | OU |
| 19588eef7f63-d27c-48b4-895d-dd02e7bf9bc2 | Translation Approved (100%) | Option 2. | Opção 2. |
| 19598eef7f63-d27c-48b4-895d-dd02e7bf9bc2 | Translation Approved (76%) | ISO and CEN Standards | Normas ISO E CEN |
| 19607c046525-e51a-46fc-9ffd-c1b3aa1fd8e8 | Translation Approved (0%) | Have a permanent monitoring system to ensure ongoing building performance of the desired comfort criteria, as specified by the applicable standard: | Implemente um sistema de monitoração permanente para garantir o desempenho contínuo do edifício de acordo com os critérios de conforto desejados, conforme especificado pela norma aplicável: |
| 1961217103f2-7a07-4845-8c49-45bf7e7a59b4 | Translation Approved (0%) | ISO 7730:2005 Ergonomics of the Thermal Environment, analytical determination and interpretation of thermal comfort using calculation of the PMV and PPD indices and local thermal comfort criteria; and | ISO 7730:2005 Ergonomia do ambiente térmico (Ergonomics of the Thermal Environment), determinação e interpretação analítica do conforto térmico usando o cálculo dos índices de PMV e PPD, bem como os critérios de conforto térmico local; e |
| 1962bc0586a1-ca0d-4220-b0d0-14a3a750a716 | Translation Approved (0%) | CEN Standard EN 15251:2007, Indoor Environmental Input Parameters for Design and Assessment of Energy Performance of Buildings, addressing indoor air quality, thermal environment, lighting, and acoustics, Section A2. | Norma CEN EN 15251:2007, Parâmetros de entrada do ambiente interno para projeto e avaliação de desempenho energético de edifícios, abordando qualidade do ar interno, ambiente térmico, iluminação e acústica, Seção A2. |
| 1963fcf8c374-f95d-4428-b8bb-54efe633bac7 | Translation Approved (100%) | Data Centers only | Apenas Centros de Dados (Data Centers) |
| 196460f380a6-57fa-403f-a84c-3c6693e9cc05 | Translation Approved (0%) | Meet the above requirements for occupants in regularly occupied spaces. | Atender aos requisitos acima para ocupantes em espaços regularmente ocupados. |
| 1965039c4199-9fca-4108-8dc8-a6aea60e402a | Translation Approved (0%) | Hospitality only | Somente Hoteleiro |
| 1966f8475f25-6b53-4b2e-884c-e1472a6bd1d9 | Translation Approved (0%) | Guest rooms are assumed to provide adequate thermal comfort and are therefore not included in the credit calculations. | Os quartos de hóspede devem fornecer conforto térmico adequado e, portanto, não são incluídos nos cálculos do crédito. |
| 19677669aa19-9489-4a13-a75b-6b0d16e1fc6d | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 19682e760ccb-8880-4a40-88ab-df49b5aadf3c | Translation Approved (0%) | The monitoring system must meet the following requirements. | O sistema de monitoramento deve atender aos seguintes requisitos. |
| 1969103f6c0d-0429-469c-bf81-f65224b48bd4 | Translation Approved (0%) | Continuous monitoring. | Monitoramento contínuo. |
| 1970103f6c0d-0429-469c-bf81-f65224b48bd4 | Translation Approved (0%) | Monitor at least air temperature and humidity in occupied spaces, at sampling intervals of 15 minutes or less. | Monitorar pelo menos a temperatura do ar e a umidade em espaços ocupados, em intervalos de coleta de amostra de 15 minutos ou menos. |
| 1971fd78fd58-a1a4-43c0-9fd8-4d8fd430a66e | Translation Approved (98%) | Periodic testing. | Testes periódicos. |
| 1972fd78fd58-a1a4-43c0-9fd8-4d8fd430a66e | Translation Approved (0%) | Monitor air speed and radiant temperature in occupied spaces. | Monitorar a velocidade do ar e a temperatura radiante em espaços ocupados. |
| 1973fd78fd58-a1a4-43c0-9fd8-4d8fd430a66e | Translation Approved (100%) | Using handheld meters is permitted. | É permitido usar medidores manuais. |
| 1974e5e97303-9776-4011-ae85-31ae67e08e59 | Translation Approved (0%) | Alarms. | Alarmes. |
| 1975e5e97303-9776-4011-ae85-31ae67e08e59 | Translation Approved (79%) | An alarm must indicate conditions that require system adjustment or repair. | Um alarme deve indicar as condições que exigem ajuste ou reparo do sistema |
| 19767121d7df-4041-4d82-93ef-ab7cc83c9d7a | Translation Approved (0%) | Prompt repair. | Reparo imediato. |
| 19777121d7df-4041-4d82-93ef-ab7cc83c9d7a | Translation Approved (0%) | Specify procedures for adjustments or repairs to be made in response to problems identified. | Especificar procedimentos para que ajustes ou reparos sejam feitos em resposta aos problemas identificados. |
| 1978c27d8ff8-177b-471b-90e4-671fb0b2473d | Translation Approved (0%) | Calibration. | Calibração. |
| 1979c27d8ff8-177b-471b-90e4-671fb0b2473d | Translation Approved (100%) | All monitoring devices must be calibrated within the manufacturer’s recommended interval. | Todos os dispositivos de monitoramento devem ser calibrados de acordo com o intervalo recomendado pelo fabricante. |
| 198055d65452-8e47-41b8-b1af-63bfcce01129 | Translation Approved (0%) | EQ Credit: Interior Lighting | Crédito EQ: Iluminação Interior (EQ Credit: Interior Lighting) |
| 1981bc744bd0-b98d-4e66-a75b-f0b1cc1e976b | Translation Approved (100%) | EB:O&M | EB:O&M |
| 19828d984053-4af8-47f8-bf0a-4d1ac9ecdf3a | Translation Approved (100%) | 1–2 points | 1 a 2 pontos |
| 1983e0836a51-347d-45f6-9567-4aa009a5b6e8 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 19849087abe6-5452-41c8-bcc7-ecd4a82d6711 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1–2 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 a 2 pontos) |
| 1985bdf5ae0b-759c-4197-91e5-7b075957e7c0 | Translation Approved (100%) | EB:O&M Schools (1–2 points) | EB:O&M Escolas (Schools) (1 a 2 pontos) |
| 19867ae91bf0-dfea-4b91-9876-8b0b4593d335 | Translation Approved (100%) | EB:O&M Retail (1–2 points) | EB:O&M Varejo (Retail) (1 a 2 pontos) |
| 1987708cfca3-959e-4842-8da4-1e7768c1fe71 | Translation Approved (100%) | EB:O&M Data Centers (1–2 points) | EB:O&M Centros de Dados (Data Centers) (1 a 2 pontos) |
| 1988f60cb39e-f9e8-4357-8e33-8b3f9baf022c | Translation Approved (100%) | EB:O&M Hospitality (1–2 points) | EB:O&M Hoteleiro (Hospitality) (1 a 2 pontos) |
| 1989751e5716-be07-431d-b108-f993115b634f | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1–2 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 a 2 pontos) |
| 199001189fad-0d8c-4514-b144-63020350384b | Translation Approved (100%) | Intent | Objetivo |
| 1991c0626b44-fe64-408d-bd2d-758d33f6d637 | Translation Approved (80%) | To promote occupants’ productivity, comfort, and well-being by providing high-quality lighting. | Promover a produtividade, o conforto e o bem-estar dos ocupantes por meio do fornecimento de iluminação de qualidade. |
| 199258f4e8f3-6d84-4834-a5ba-4fa0e40a60af | Translation Approved (100%) | Requirements | Requisitos |
| 1993a597adc4-fd10-470d-988a-152f0b0dafcd | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 1994f5d1dbef-b5bf-4129-ac88-5f361d4692c9 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 19957b447d0f-748f-43e7-8bc3-8e026af865f6 | Translation Approved (100%) | Option 1. | Opção 1. |
| 19967b447d0f-748f-43e7-8bc3-8e026af865f6 | Translation Approved (0%) | Lighting Control (1 point) | Controle de iluminação (1 ponto) |
| 1997422ad351-54cc-443e-92a8-5cd3688876fe | Translation Approved (0%) | For at least 50% of individual occupant spaces, have in place individual lighting controls that enable occupants to adjust the lighting to suit their individual tasks and preferences, with at least three lighting levels or scenes (on, off, midlevel). | Para pelo menos 50% dos espaços individuais de ocupantes, implementar controles individuais de iluminação que permitem aos ocupantes ajustarem a iluminação para se adaptar às suas tarefas e preferências individuais, com pelo menos três níveis ou cenas de iluminação (ligado, desligado, intermediário). |
| 1998422ad351-54cc-443e-92a8-5cd3688876fe | Translation Approved (0%) | Midlevel is 30% to 70% of the maximum illumination level (not including daylight contributions). | O nível intermediário é 30% a 70% do nível máximo de iluminação (não incluindo as contribuições de luz natural). |
| 19991bf56f2c-c8cd-46a3-b130-b2884c437113 | Translation Approved (0%) | For all shared multi-occupant spaces, meet all of the following requirements. | Para todos os espaços multiocupante compartilhados, atender a todos os seguintes requisitos. |
| 20009d9b63ec-4943-4ec6-81c1-ac72861b7f57 | Translation Approved (72%) | Have in place multizone control systems that enable occupants to adjust the lighting to meet group needs and preferences, with at least three lighting levels or scenes (on, off, midlevel). | Implemente sistemas de controle individuais em diversas zonas que permitem aos ocupantes ajustarem a iluminação para se adaptar às necessidades e preferências em grupo, com pelo menos três níveis ou cenas de iluminação (ligado, desligado, intermediário). |
| 20011fd114d2-1620-445b-818a-69e55a7257cc | Translation Approved (0%) | Lighting for any presentation or projection wall must be separately controlled. | A iluminação de qualquer parede de apresentação ou projeção deve ser controlada separadamente. |
| 20021441e284-d28b-40df-be92-0ad55655f63c | Translation Approved (0%) | Switches or manual controls must be located in the same space as the controlled luminaires. | Os interruptores ou controles manuais devem ser localizados no mesmo espaço das luminárias controladas. |
| 20031441e284-d28b-40df-be92-0ad55655f63c | Translation Approved (0%) | A person operating the controls must have a direct line of sight to the controlled luminaires. | A pessoa que opera os controles deve ter uma linha de visão direta das luminárias controladas. |
| 20044d7b2450-9542-430e-8a52-920fcc587361 | Translation Approved (100%) | Hospitality only | Somente Hoteleiro |
| 2005432e4da7-eb22-4010-b662-d26aaefafc0b | Translation Approved (89%) | Guest rooms are assumed to have adequate lighting controls and are therefore not included in the credit calculations. | Os quartos de hóspede devem fornecer controles de iluminação adequado e, portanto, não são incluídos nos cálculos do crédito. |
| 200685756ac4-d245-490a-b2b6-3d9b68053478 | Translation Approved (100%) | AND/OR | E/OU |
| 2007014036fc-daba-43bc-8a6d-ebaf41d527d1 | Translation Approved (100%) | Option 2. | Opção 2. |
| 2008014036fc-daba-43bc-8a6d-ebaf41d527d1 | Translation Approved (84%) | Lighting Quality (1 point) | Qualidade da iluminação (1 ponto) |
| 2009cd69d2ab-d2f3-49c5-a83f-913539175c5f | Translation Approved (78%) | Choose four of the following strategies. | Escolher quatro das seguintes estratégias. |
| 20100823c952-6208-4b05-8a08-767a7d49fb27 | Translation Approved (0%) | For all regularly occupied spaces, have in place light fixtures with a luminance of less than 2,500 cd/m<1805>2</1805> between 45 and 90 degrees from nadir. | Para todos os espaços regularmente ocupados, implementar dispositivos de iluminação com uma luminância menor que 2.500 cd/m<1805>2</1805> entre 45 e 90 graus de nadir. |
| 2011d187bebc-32d2-40b6-8776-8cd9bb5ca1d2 | Translation Approved (0%) | Exceptions include wallwash fixtures properly aimed at walls, as specified by manufacturer’s data, indirect uplighting fixtures, provided there is no view down into these uplights from a regularly occupied space above, and any other specific applications (i.e. adjustable fixtures). | Exceções incluem dispositivos de banho de luz (wall wash) propriamente apontados para paredes, conforme especificado pelos dados do fabricante, dispositivos de iluminação direcionada para cima indireta, desde que não haja uma vista para baixo dessa iluminação direcionada para cima a partir de um espaço regularmente ocupado acima e qualquer outra aplicação especifica (como dispositivos ajustáveis). |
| 2012589a25dd-38bd-4e5e-bbe4-7856addf7633 | Translation Approved (0%) | For the entire project, have in place light sources with a CRI of 80 or higher. | Para todo o projeto, implementar fontes de luz com um CRI de 80 ou superior. |
| 2013589a25dd-38bd-4e5e-bbe4-7856addf7633 | Translation Approved (0%) | Exceptions include lamps or fixtures specifically designed to provide colored lighting for effect, site lighting, or other special use. | Exceções incluem lâmpadas ou dispositivos projetados especificamente para fornecer iluminação colorida para uma iluminação do terreno de efeito ou outro uso especial. |
| 20143d31e43c-5875-4546-b8e4-3204c60b0dd8 | Translation Approved (0%) | For at least 75% of the total connected lighting load, have in place light sources that have a rated life (or L70 for LED sources) of at least 24,000 hours (at 3-hour per start, if applicable). | Para pelo menos 75% da carga total de iluminação conectada, implementar fontes de luz que possuem um vida nominal (ou L70 para fontes LED) de pelo menos 24.000 horas (a 3 horas por início, se aplicável). |
| 2015121eca36-2de8-4d23-9d7e-1aaec1223e2a | Translation Approved (0%) | Have in place direct-only overhead lighting for 25% or less of the total connected lighting load for all regularly occupied spaces. | Implemente iluminação suspensa exclusivamente direta para 25% ou menos da carga total de iluminação conectada para todos os espaços regularmente ocupados. |
| 201683a3d2b2-1748-4069-a657-ea6cf0e56200 | Translation Approved (0%) | For at least 90% of the regularly occupied floor area, meet or exceed the following thresholds for area-weighted average surface reflectance: 85% for ceilings, 60% for walls, and 25% for floors. | Para pelo menos 90% da área de piso regularmente ocupada, atender ou exceder os seguintes limiares para a refletância média ponderada por área da superfície: 85% para telhados, 60% para paredes e 35% para pisos. |
| 2017cc5ccc9b-1fc2-4010-a256-915d317ba848 | Translation Approved (0%) | Meet or exceed the following thresholds for area-weighted average surface reflectance: 45% for work surfaces and 50% for movable partitions. | Atender ou exceder os seguintes limiares para a refletância média ponderada por área da superfície: 45% para superfícies de trabalho e 50% para partições móveis. |
| 20189c47cedc-e2be-4dc8-94c7-c0fa0e7b681c | Translation Approved (0%) | For at least 75% of the regularly occupied floor area, meet a ratio of average wall surface illuminance (excluding fenestration) to average work surface illuminance that does not exceed 1:10. | Para pelo menos 75% da área de piso regularmente ocupada, atender a uma razão entre a iluminação média da superfície da parede (excluindo fenestração) e a luminância média da superfície de trabalho que não exceda 1:10. |
| 20199c47cedc-e2be-4dc8-94c7-c0fa0e7b681c | Translation Approved (0%) | Must also meet strategy E, strategy F, or demonstrate area-weighted surface reflectance of at least 60% for walls. | Também deve atender à estratégia E, estratégia F ou demonstrar uma refletância ponderada por área da superfície de pelo menos 60% para paredes. |
| 2020646b22cb-b7da-453a-87d4-60ea3438d507 | Translation Approved (93%) | For at least 75% of the regularly occupied floor area, meet a ratio of average ceiling illuminance (excluding fenestration) to work surface illuminance that does not exceed 1:10. | Para pelo menos 75% da área de piso regularmente ocupada, atender a uma razão entre a iluminação média do telhado (excluindo fenestração) e a luminância da superfície de trabalho que não exceda 1:10. |
| 2021646b22cb-b7da-453a-87d4-60ea3438d507 | Translation Approved (97%) | Must also meet strategy E, strategy F, or demonstrate area-weighted surface reflectance of at least 85% for ceilings. | Também deve atender à estratégia E, estratégia F ou demonstrar uma refletância ponderada por área da superfície de pelo menos 85% para telhados. |
| 20221c5f2ff2-74e7-4f08-82fd-cba4805bc0ab | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 202377e01e51-386a-4669-a40e-011d2161a635 | Translation Approved (100%) | None. | Nenhum. |
| 2024bb2996b2-e6d9-4dae-a797-c684d2cfe593 | Translation Approved (75%) | EQ Credit: Daylight and Quality Views | Crédito EQ: Luz Natural e Vistas de Qualidade (EQ Credit: Daylight and Quality Views) |
| 2025a9beb117-f16a-4a25-8025-5f0b89d182a4 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 20267ea09e7b-8a09-4d74-abd3-8dbbb7db3245 | Translation Approved (100%) | 2–4 points | 2 a 4 pontos |
| 20279d2fbfbd-25de-40fd-8a55-86a135f424f8 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 20286b5e261e-8f4c-4229-8af6-a093ab5f4fa9 | Translation Approved (96%) | Existing Buildings: Operations & Maintenance (2–4 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (2 a 4 pontos) |
| 2029809f1379-764d-45c7-acf0-f04846877a48 | Translation Approved (95%) | EB:O&M Schools (2–4 points) | EB:O&M Escolas (Schools) (2 a 4 pontos) |
| 2030e573597b-db6c-4352-8b1b-8a386008bc9c | Translation Approved (95%) | EB:O&M Retail (2–4 points) | EB:O&M Varejo (Retail) (2 a 4 pontos) |
| 203112ece76d-929f-4624-8b18-597f34bc4f2e | Translation Approved (100%) | EB:O&M Data Centers (2–4 points) | EB:O&M Centros de Dados (Data Centers) (2 a 4 pontos) |
| 203276f6c7c4-5935-4daa-bb8e-7c6333809c2b | Translation Approved (95%) | EB:O&M Hospitality (2–4 points) | EB:O&M Hoteleiro (Hospitality) (2 a 4 pontos) |
| 20336cffd3e5-fce9-4c40-b3fa-a00956c98225 | Translation Approved (97%) | EB:O&M Warehouses & Distribution Centers (2–4 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (2 a 4 pontos) |
| 2034a14f7912-6a26-45af-b1d3-72a26d024545 | Translation Approved (100%) | Intent | Objetivo |
| 2035612b6f30-b6f1-425d-944b-ce5ec88d3c93 | Translation Approved (0%) | To connect building occupants with the outdoors, reinforce circadian rhythms, and reduce the use of electrical lighting by introducing daylight and views into the space. | Conectar os ocupantes do edifício com o ambiente ao ar livre, reforçar os ritmos circadianos e reduzir o uso de iluminação elétrica por meio da introdução de luz natural e vistas ao espaço. |
| 20368daada8f-6112-4cb2-9a42-5bc9f20b7776 | Translation Approved (100%) | Requirements | Requisitos |
| 2037083abe64-05cb-4d33-babe-3fdfc375191a | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 20389e82f2b5-ea4d-47b8-9930-28e38b2a0ad9 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 20393ad2db8b-e777-4782-905d-dd02d5449d16 | Translation Approved (100%) | Option 1. | Opção 1. |
| 20403ad2db8b-e777-4782-905d-dd02d5449d16 | Translation Approved (0%) | Daylight Measurement (2 points) | Medição de luz natural (2 pontos) |
| 20411308f2a1-0e91-4bce-bedb-33abcb8c04e6 | Translation Approved (0%) | Achieve illuminance levels between 300 lux and 3,000 lux for at least 50% of the regularly occupied floor area. | Atingir níveis de iluminação entre 300 lux e 3.000 lux para pelo menos 50% da área de piso regularmente ocupada. |
| 2042551312c9-5d52-4262-900c-cf45bdafd40d | Translation Approved (0%) | With furniture, fixtures, and equipment in place, measure illuminance levels as follows: | Com os móveis, dispositivos e equipamentos instalados, medir os níveis de iluminação da seguinte forma: |
| 20439ee28d76-0504-433c-8189-48f707943d03 | Translation Approved (0%) | Measure at appropriate work plane height during any hour between 9 a.m. and 3 p.m. | Meça a altura adequada do plano de trabalho durante qualquer horário entre 9h e 15h. |
| 204451c98f0c-3e0b-4e3a-9ee4-9d23502a9124 | Translation Approved (0%) | Take one measurement in any regularly occupied month, and take a second as indicated in Table 1. | Fazer uma medição em qualquer mês de ocupação regular e fazer uma segunda medição conforme indicado na Tabela 1. |
| 2045152dafda-23e0-4d2a-b3f2-0183554e20d8 | Translation Approved (0%) | For spaces larger than 150 square feet (14 square meters), take measurements on a maximum 10 foot (3 meter) square grid. | Para espaços maiores que 150 pés quadrados (14 metros quadrados), fazer medições em uma rede de no máximo 10 pés (3 metros) quadrados. |
| 204698485ffd-298e-476f-bbaf-db1b68037bbd | Translation Approved (81%) | For spaces 150 square feet (14 square meters) or smaller, take measurements on a maximum 3 foot (900 millimeter) square grid. | Para espaços de 150 pés quadrados (14 metros quadrados) ou menores, fazer medições em uma rede de no máximo 3 pés (900 milímetros) quadrados. |
| 204736d48b4c-3b38-4a83-a4df-bfc7129f0365 | Translation Approved (100%) | Table 1. | Tabela 1. |
| 204836d48b4c-3b38-4a83-a4df-bfc7129f0365 | Translation Approved (0%) | Timing of measurements for illuminance | Calendário de medições para iluminação |
| 2049453d3433-1b46-4580-92e0-799a4b025c5e | Translation Approved (0%) | If first measurement is taken in … | Se a primeira medição foi realizada em... |
| 2050271ebc92-b14d-467c-8768-b85700054a3b | Translation Approved (0%) | take second measurement in … | fazer a segunda medição em... |
| 2051be5dd442-ff04-4f26-9771-ce488065b3db | Translation Approved (0%) | January | Janeiro |
| 2052c2b69057-0ccf-4964-ba75-dfa546373fab | Translation Approved (0%) | May-September | Maio-setembro |
| 2053287d33ba-1882-4eb3-81a5-5602eaa81bcf | Translation Approved (0%) | February | Fevereiro |
| 2054c431713e-c037-465f-9977-438dc3717cfc | Translation Approved (0%) | June-October | Junho-outubro |
| 205561acbcdf-52df-4ae2-aa53-ec106e07c81f | Translation Approved (0%) | March | Março |
| 20568f443dc3-2666-43ca-8048-cdb3b992fcb7 | Translation Approved (0%) | June-July, November-December | Junho/julho, novembro/dezembro |
| 20571aa0bd67-23e4-4710-976f-7b3cfb7e6da8 | Translation Approved (75%) | April | Abril |
| 2058a9217bed-19da-4f1e-a0ff-8974384beaa7 | Translation Approved (0%) | August-December | Agosto-dezembro |
| 205920e20281-2489-4b38-855c-c1fe8553c25d | Translation Approved (0%) | May | Maio |
| 2060b93c6913-dac4-4257-8ba3-62debb703339 | Translation Approved (0%) | September-January | Setembro-janeiro |
| 206163325b41-c65f-483c-922a-ddeb7c350e16 | Translation Approved (75%) | June | Junho |
| 20624d8d273a-24f7-4c46-a73c-0500be22bdda | Translation Approved (0%) | October-February | Outubro-fevereiro |
| 20637795009d-4162-49a4-8f8d-39573b970e79 | Translation Approved (75%) | July | Julho |
| 20646a436ae5-4af8-44fe-a206-77ba7a15ebf7 | Translation Approved (0%) | November-March | Novembro-março |
| 2065ffa788a5-2a7a-414d-aef8-c36fa0d7604f | Translation Approved (0%) | August | Agosto |
| 2066d3cf6a3f-5e7e-44da-81c3-735106f7f6d8 | Translation Approved (0%) | December-April | Dezembro-abril |
| 2067d7286638-3516-47ed-9a27-1ecb024f0fc1 | Translation Approved (0%) | September | Setembro |
| 20683beb2064-2b95-4358-8ee9-a77e669f46a0 | Translation Approved (0%) | December-January, May-June | Dezembro/janeiro, maio/junho |
| 2069e97292d1-f6ad-4438-84e9-163afeb9d2aa | Translation Approved (0%) | October | Outubro |
| 2070f4dc7c5a-e57f-40c4-a0f7-e525dcd538f6 | Translation Approved (0%) | February-June | Fevereiro-junho |
| 20716f2c5408-26c1-40e0-82b8-feeb32191cb4 | Translation Approved (0%) | November | Novembro |
| 2072f1311339-7a7a-4c15-a878-052300044afd | Translation Approved (0%) | March-July | Março-julho |
| 2073218af4b4-c467-4d32-a1db-dd42a7f27bbe | Translation Approved (0%) | December | Dezembro |
| 2074306cafdb-8764-4f4b-8edf-5a6e33d0ca74 | Translation Approved (0%) | April-August | Abril-agosto |
| 20750e81e443-967a-43b7-99dc-1a08c160a63b | Translation Approved (100%) | AND/OR | E/OU |
| 2076007d4e44-fd6e-4591-915e-9feb19bfa8c0 | Translation Approved (100%) | Option 2. | Opção 2. |
| 2077007d4e44-fd6e-4591-915e-9feb19bfa8c0 | Translation Approved (0%) | Quality Views (2 points) | Vistas de qualidade (2 pontos) |
| 20787bf72376-b658-4996-8292-362cbed8aab5 | Translation Approved (0%) | Achieve a direct line of sight to the outdoors via vision glazing for 50% of all regularly occupied floor area. | Atingir uma linha direta de visão para o ambiente ao ar livre por meio de vidraça para 50% de toda a área de piso regularmente ocupada. |
| 20797bf72376-b658-4996-8292-362cbed8aab5 | Translation Approved (0%) | View glazing in the contributing area must provide a clear image of the exterior, not obstructed by frits, fibers, patterned glazing, or added tints that distort color balance. | A visão de vidraça na área de contribuição deve possibilitar uma imagem clara da área externa, sem a obstrução de fritas, fibras, vidraças estampadas ou tintas que distorçam o equilíbrio de cores. |
| 2080bbccab8c-141d-4a4b-9070-79112d6d96fc | Translation Approved (0%) | Additionally, 50% of all regularly occupied floor area must have at least two of the following four kinds of views: | Além disso, 50% de toda a área de piso regularmente ocupada deve ter pelo menos dois dos seguintes quatro tipos de vistas: |
| 2081dd5cce76-51bc-4856-80b1-b39dfea1272b | Translation Approved (0%) | multiple lines of sight to vision glazing in different directions at least 90 degrees apart; | múltiplas linhas de visão para a vidraça em direções diferentes afastadas pelo menos 90 graus; |
| 2082b0b1a978-1fed-4da2-a190-c41c11e2ae22 | Translation Approved (0%) | views that include at least two of the following: (1) flora, fauna, or sky; (2) movement; and (3) objects at least 25 feet (7.5 meters) from the exterior of the glazing; | vistas que incluem pelo menos dois dos seguintes: (1) flora, fauna ou céu; (2) movimento; e (3) objetos a pelo menos 25 pés (7,5 metros) do exterior da vidraça; |
| 2083167c4560-d66a-4e50-a20e-40ed22678ba4 | Translation Approved (0%) | unobstructed views located within the distance of three times the head height of the vision glazing; and | vistas sem obstrução localizadas dentro da distância de três vezes a altura da vidraça; e |
| 20844f82429e-225b-4402-a67b-5b4c55ff16f9 | Translation Approved (0%) | views with a view factor of 3 or greater, as defined in “Windows and Offices; A Study of Office Worker Performance and the Indoor Environment.” | vistas com um fator de visão de 3 ou superior, conforme definido em “Windows and Offices; A Study of Office Worker Performance and the Indoor Environment.” |
| 2085ed547573-f5b7-42f7-b5e6-15d5cd02a29f | Translation Approved (0%) | Include in the calculations any permanent interior obstructions . | Inclua qualquer obstrução interior permanente nos cálculos. |
| 2086ed547573-f5b7-42f7-b5e6-15d5cd02a29f | Translation Approved (0%) | Movable furniture and partitions may be excluded. | Móveis e partições móveis podem ser excluídos. |
| 208709b1d43b-1da3-4cfd-9672-562e80bfaaa0 | Translation Approved (0%) | Views into interior atria may be used to meet up to 30% of the required area. | Vistas do átrio interior podem ser usadas para atender até 30% da área necessária. |
| 2088b159bf83-e220-47d1-908a-961355e920b8 | Translation Approved (100%) | Warehouses & Distribution Centers only | Apenas Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 20899c13cbc1-78dc-495c-9d26-2df118574630 | Translation Approved (0%) | For the office portion of the building, meet the requirements above. | Para a porção de escritórios do edifício, atender aos requisitos acima. |
| 20903b8b298b-56e8-47ab-9a9b-be08ef3c8e7d | Translation Approved (0%) | For the bulk storage, sorting, and distribution portions of the building, meet the requirements above for 25% of the regularly occupied floor area. | Para as porções de distribuição, separação, armazenamento em bloco do edifício, atender aos requisitos acima para 25% da área de piso regularmente ocupada. |
| 209120462a62-9948-491b-9a83-8e83ed06e1a5 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 20928040d2c6-3d98-4c69-aa23-8838d4315c44 | Translation Approved (100%) | None. | Nenhum. |
| 20931948ee90-c530-4ea2-8e05-d6d6544c9045 | Translation Approved (78%) | EQ Credit: Green Cleaning—Custodial Effectiveness Assessment | Crédito EQ: Limpeza Verde - Avaliação da Eficiência de Limpeza (EQ Credit: Green Cleaning—Custodial Effectiveness Assessment) |
| 2094d6fd2475-8a54-4fee-94b3-5212f9b3d75d | Translation Approved (100%) | EB:O&M | EB:O&M |
| 209502b3e071-c25f-4737-a641-7f5ca6f337dd | Translation Approved (100%) | 1 point | 1 ponto |
| 20966c203bcb-b0e0-4e74-b898-da8c889954d0 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 2097403b0c1e-0f31-4ee3-94f9-f9e8d7425886 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1 point) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 ponto) |
| 2098eeed7615-2725-47de-961b-3332bedd7d55 | Translation Approved (100%) | EB:O&M Schools (1 point) | EB:O&M Escolas (Schools) (1 ponto) |
| 20998eb9bbfd-5946-4796-a1a5-18814fb99708 | Translation Approved (100%) | EB:O&M Retail (1 point) | EB:O&M Varejo (Retail) (1 ponto) |
| 21004268d399-7f73-4d5d-baaf-872882af48cc | Translation Approved (100%) | EB:O&M Data Centers (1 point) | EB:O&M Centros de Dados (Data Centers) (1 ponto) |
| 2101e48b89cd-3591-4510-804d-2ab11a4b8d34 | Translation Approved (100%) | EB:O&M Hospitality (1 point) | EB:O&M Hoteleiro (Hospitality) (1 ponto) |
| 21027f4bf9a5-8250-43ed-b9b6-d409ee6715fa | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1 point) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 ponto) |
| 2103ac00cfcf-492c-4e28-bbb1-cd976f2d4e15 | Translation Approved (100%) | Intent | Objetivo |
| 2104ed2ec401-59e5-40c3-98bf-d7a3fa75897f | Translation Approved (0%) | To reduce levels of chemical, biological, and particulate contaminants, which can compromise human health, building finishes and systems, and the environment, by implementing effective cleaning procedures. | Reduzir os níveis de produtos químicos, agentes biológicos e contaminantes particulados que podem comprometer a saúde humana, acabamentos e sistemas de edifícios e o meio ambiente por meio da implementação de procedimentos de limpeza eficazes. |
| 21052c86db0e-98ec-4747-b165-af0fde6fdece | Translation Approved (100%) | Requirements | Requisitos |
| 21063a07b97f-d50a-4205-a1a0-efc5a3174b0e | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 2107dbc9b3da-b238-4dc2-b755-fec9cb10614c | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 2108d7b51f46-1e8e-4ec6-aea5-eb208424ed1a | Translation Approved (100%) | None. | Nenhum. |
| 2109f23143ca-a1ed-48ab-be2e-fef367406478 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 21103bbe83fa-1382-4e01-a292-086b7f1a1518 | Translation Approved (0%) | Implement the strategies set forth in the facility’s green cleaning policy and perform routine inspection and monitoring. | Implemente as estratégias estabelecidas na política de limpeza verde da instalação e realizar a inspeção e o monitoramento de rotina. |
| 21113bbe83fa-1382-4e01-a292-086b7f1a1518 | Translation Approved (0%) | This inspection must verify that the specified strategies have been implemented and must identify areas in need of improvement. | Essa inspeção deve verificar que as estratégias especificadas foram implementadas e deve identificar as áreas que precisam de melhoria. |
| 2112735b36ef-2ab9-4f0b-abb5-d8b421f484bd | Translation Approved (0%) | Additionally, conduct an annual audit in accordance with APPA Leadership in Educational Facilities’ Custodial Staffing Guidelines, or a local equivalent, whichever is more stringent, to determine the appearance level of the facility. | Além disso, conduzir uma auditoria anual de acordo com as Diretrizes de recursos humanos para pessoal de vigilância e limpeza da Liderança da Associação de Administradores de Planta Física (Association of Physical Plant Administrators – APPA), ou um equivalente local, o que for mais rígido, para determinar o nível de aparência da instalação. |
| 2113735b36ef-2ab9-4f0b-abb5-d8b421f484bd | Translation Approved (0%) | The facility must score 2.5 or better. | A instalação deve obter pontuação de 2,5 ou superior. |
| 2114bcd5ec3a-9bf8-475f-82eb-dabacae7e1b8 | Translation Approved (0%) | EQ Credit: Green Cleaning—Products and Materials | Crédito EQ: Limpeza Verde - Produtos e Materiais (EQ Credit: Green Cleaning—Products and Materials) |
| 2115c15cf0bb-c972-4fbd-99d1-969b287a8f70 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 2116ee4877ad-5829-420c-9550-e3e582eb8021 | Translation Approved (100%) | 1 point | 1 ponto |
| 2117c5ffdba0-eb10-4f4a-a098-939299ef0aea | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 21180bb7422e-9734-4214-b19d-54851da82a05 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1 point) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 ponto) |
| 2119708a376f-27be-43c1-b9ab-a691cd4cf91d | Translation Approved (100%) | EB:O&M Schools (1 point) | EB:O&M Escolas (Schools) (1 ponto) |
| 21208d096d33-7d28-402e-aba9-7b2c0144f43f | Translation Approved (100%) | EB:O&M Retail (1 point) | EB:O&M Varejo (Retail) (1 ponto) |
| 21212c24ccd0-9632-4127-9326-2ac530c21ba4 | Translation Approved (100%) | EB:O&M Data Centers (1 point) | EB:O&M Centros de Dados (Data Centers) (1 ponto) |
| 2122c38d2066-b840-44cd-bfe8-367e0f695be0 | Translation Approved (100%) | EB:O&M Hospitality (1 point) | EB:O&M Hoteleiro (Hospitality) (1 ponto) |
| 21237f2cd3ca-b322-4fbf-b97e-af0ef07ad24b | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1 point) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 ponto) |
| 2124e75fc4e0-abf7-4b5c-b8b3-cee7965e5998 | Translation Approved (100%) | Intent | Objetivo |
| 2125c01b30a2-2ab6-47e9-9b32-4c912dd826ce | Translation Approved (96%) | To reduce the environmental effects of cleaning products, disposable janitorial paper products, and trash bags. | Reduzir os efeitos ambientais de produtos de limpeza, produtos descartáveis de papel para limpeza e sacos de lixo. |
| 2126fe8e5c67-219f-48fe-a5fd-72a26ea72fb5 | Translation Approved (100%) | Requirements | Requisitos |
| 2127e2910f3c-b4d5-455c-8fd3-990afea02a04 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 21287c168b10-325c-419d-b90e-a6cc4cb76f3b | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 212988fbdf0c-24b7-45ce-8ac0-20734ab57116 | Translation Approved (100%) | None. | Nenhum. |
| 213074acad89-1a9f-4631-aa57-7d7d95591013 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 213166b8fc9b-3c46-4562-8ed6-218ae5e9588e | Translation Approved (0%) | Purchase green cleaning materials and products such as floor finishes and strippers, disposable janitorial paper products, and trash bags. | Compre materiais e produtos de limpeza verde como revestimentos de piso e removedores, produtos descartáveis de papel para limpeza e sacos de lixo. |
| 213266b8fc9b-3c46-4562-8ed6-218ae5e9588e | Translation Approved (71%) | Include items used by in-house staff or outsourced service providers. | Inclua itens usados por funcionários internos ou por prestadores de serviço terceirizados. |
| 213319a5b330-9deb-4ca0-b2d6-337d40116377 | Translation Approved (75%) | At least 75%, by cost, of the total annual purchases of these products must meet at least one of the following standards. | Pelo menos 75%, por custo, do total de compras anuais desses produtos deve atender a pelo menos uma das normas a seguir. |
| 21346ecf0823-eab0-456f-aad5-d2a7b814f9a8 | Translation Approved (0%) | Cleaning products must meet one or more of the following standards, or a local equivalent for projects outside the U.S.: | Os produtos de limpeza devem atender a uma ou mais normas a seguir, ou um equivalente local para projetos fora dos EUA. |
| 2135e469b57b-dfe6-4eb2-9de6-4b849eec59e3 | Translation Approved (99%) | Green Seal GS-37, for general-purpose, bathroom, glass and carpet cleaners used for industrial and institutional purposes; | Green Seal GS-37, para produtos de limpeza de uso geral, para banheiros, vidros e carpetes usados para fins industriais e institucionais; |
| 213609a6c1b4-f46d-48ef-a4de-2a6e3599928a | Translation Approved (0%) | <1919>,</1919> <1920>UL EcoLogo 2792 for cleaning and degreasing compounds;</1920> | UL EcoLogo 2792, para compostos de limpeza e desengordurantes; |
| 213793f53c54-833e-4e86-880c-d4f76a9ce271 | Translation Approved (0%) | UL EcoLogo 2759 for hard-surface cleaners; | UL EcoLogo 2759, para produtos de limpeza de superfícies duras; |
| 21387d726051-bace-45d2-85bf-5b3f7086b55a | Translation Approved (72%) | UL EcoLogo 2795, for carpet and upholstery care; | UL EcoLogo 2795, para produtos de conservação de carpetes e estofamentos; |
| 2139409dbfc7-d51b-4bae-b420-b54912539c92 | Translation Approved (99%) | Green Seal GS-40, for industrial and institutional floor care products; | Green Seal GS-40, para produtos de conservação de piso industriais e institucionais; |
| 2140f6c5cf5f-eca8-45d0-bfa0-1d77f6c39c92 | Translation Approved (74%) | UL EcoLogo 2777, for hard-floor care; | UL EcoLogo 2777 para conservação de pisos duros; |
| 2141221ba00e-bde3-4acb-a711-e3975779a52f | Translation Approved (0%) | EPA Design for the Environment Program’s Standard for Safer Cleaning Products; and/or | Norma para produtos de limpeza mais seguros do projeto do programa ambiental da EPA; e/ou |
| 2142c89a980c-fc60-4f1a-9783-7a46bd1d7390 | Translation Approved (0%) | Cleaning devices that use only ionized water or electrolyzed water and have third-party-verified performance data equivalent to the other standards mentioned above (if the device is marketed for antimicrobial cleaning, performance data must demonstrate antimicrobial performance comparable to EPA Office of Pollution Prevention and Toxics and Design for the Environment requirements, as appropriate for use patterns and marketing claims). | Dispositivos de limpeza que usam água ionizada ou água eletrolisada e possuem dados de desempenho verificados por terceiros equivalentes às outras normas mencionadas acima (se o dispositivo é comercializado para limpeza antimicrobiana, os dados de desempenho devem demonstrar o desempenho antimicrobiano comparável aos requisitos do Escritório de prevenção de poluição e tóxicos e projeto do programa ambiental da EPA, conforme adequado para padrões de uso e declarações de marketing). |
| 2143606e07ea-4473-482e-9a3a-2ab6f749d29f | Translation Approved (70%) | Disinfectants, metal polish, or other products not addressed by the above standards must meet one or more of the following standards (or a local equivalent for projects outside the U.S.): | Desinfetantes, polidores de metais ou outros produtos não abordados pelas normas acima devem atender a uma ou mais normas a seguir (ou um equivalente local para projetos fora dos EUA). |
| 21448f94938e-167c-40e4-aac0-13199ee8afaa | Translation Approved (78%) | UL EcoLogo 2798 for digestion additives for cleaning and odor control; | UL EcoLogo 2798, para aditivos de absorção para limpeza e controle de odores; |
| 2145b810b31a-95c2-4b82-b580-2d46e60644f4 | Translation Approved (0%) | UL EcoLogo 2791, for drain or grease trap additives; | UL EcoLogo 2791, para aditivos de drenagem ou filtros de gordura; |
| 2146153555b7-fdfe-4f75-a416-300b0f41e50e | Translation Approved (0%) | UL EcoLogo 2796 for odor control additives; | UL EcoLogo 2796, para aditivos de controle de odores; |
| 2147c9dc8fb7-74f1-4af6-8c70-2440fa33435f | Translation Approved (0%) | Green Seal GS-52/53, for specialty cleaning products; | Green Seal GS-52/53, para produtos de limpeza especiais; |
| 2148a49deec3-da8d-41a5-af5e-59aca6b17d52 | Translation Approved (99%) | California Code of Regulations maximum allowable VOC levels for the specific product category; | Níveis máximos de COVs permitidos do Código de Regulamentações da Califórnia para a categoria de produto específica; |
| 214996cf7a58-74d6-43bf-bab0-e37a17be0945 | Translation Approved (95%) | EPA Design for the Environment Program’s standard for safer cleaning products; and/or | Norma para produtos de limpeza mais seguros do projeto do programa ambiental da EPA; e/ou |
| 2150ac99b75b-3a7c-402d-898c-8da62f002498 | Translation Approved (99%) | Cleaning devices that use only ionized water or electrolyzed water and have third-party-verified performance data equivalent to the other standards mentioned above (if the device is marketed for antimicrobial cleaning, performance data must demonstrate antimicrobial performance comparable to EPA Office of Pollution Prevention and Toxics and Design for the Environment requirements, as appropriate for use patterns and marketing claims). | Dispositivos de limpeza que usam água ionizada ou água eletrolisada e possuem dados de desempenho verificados por terceiros equivalentes às outras normas mencionadas acima (se o dispositivo é comercializado para limpeza antimicrobiana, os dados de desempenho devem demonstrar o desempenho antimicrobiano comparável aos requisitos do Escritório de prevenção de poluição e tóxicos e projeto do programa ambiental da EPA, conforme adequado para padrões de uso e declarações de marketing). |
| 2151c011a6f7-d233-44fe-9906-d5a7c634bc20 | Translation Approved (70%) | Disposable janitorial paper products and trash bags must meet the minimum requirements of one or more of the following programs, or a local equivalent for projects outside the U.S.: | Os produtos descartáveis de papel para limpeza e sacos de lixo devem atender a uma ou mais normas a seguir, ou um equivalente local para projetos fora dos EUA.: |
| 2152f39fb317-b829-423a-9a5e-58f7f009ba57 | Translation Approved (0%) | EPA comprehensive procurement guidelines, for janitorial paper; | Diretrizes abrangentes de aquisição da EPA, para papel para limpeza; |
| 2153dfdee096-fe2f-4766-b061-60c087091ea5 | Translation Approved (77%) | Green Seal GS-01, for tissue paper, paper towels and napkins; | Green Seal GS-01, para papel toalha, toalhas de papel e guardanapos; |
| 2154854f9e07-a602-4579-8df9-70329f367f61 | Translation Approved (75%) | UL EcoLogo 175, for toilet tissue; | UL EcoLogo 175, para papel higiênico; |
| 21559de02c3a-c810-4274-a10b-6edf1549c988 | Translation Approved (75%) | UL EcoLogo 175, for hand towels; | UL EcoLogo 175, para toalha de mão; |
| 21569340d8ae-25a6-41a6-9763-78a817285660 | Translation Approved (99%) | Janitorial paper products derived from rapidly renewable resources or made from tree-free fibers; | Produtos de papel para limpeza fabricados com recursos rapidamente ou de fibras recicladas; |
| 2157aceeb6e7-a3e4-454b-85ca-12fc1368e380 | Translation Approved (0%) | FSC certification, for fiber procurement; | Certificação FSC, para aquisição de fibras; |
| 2158a2c630cb-c1db-4f5a-8bc5-e99bebf96081 | Translation Approved (0%) | EPA comprehensive procurement guidelines, for plastic trash can liners; and/or | Diretrizes abrangentes de aquisição da EPA, para sacos de lixo; e/ou |
| 2159772bb1ad-62de-4c8c-a1ac-6e128bcb0cfd | Translation Approved (0%) | California integrated waste management requirements, for plastic trash can liners (California Code of Regulations Title 14, Chapter 4, Article 5, or SABRC 42290-42297 Recycled Content Plastic Trash Bag Program). | Os requisitos de gestão de resíduos integrados da Califórnia para sacos de lixo (Código de Regulamentações da Califórnia Título 14, Capítulo 4, Artigo 5 ou SABRC 42290-42297 Programa de saco de lixo reciclado). |
| 216007e42344-a398-4590-a144-e62f8ba40bc1 | Translation Approved (84%) | Hand soaps and hand sanitizers must meet one or more of the following standards, or a local equivalent for projects outside the U.S.: | Sabonetes para mãos e higienizadores de mão devem atender a uma ou mais normas a seguir, ou um equivalente local para projetos fora dos EUA.: |
| 21619bb6fc35-06b8-443a-b1bc-ede8e06dd8f1 | Translation Approved (94%) | no antimicrobial agents (other than as a preservative) except where required by health codes and other regulations (e.g., food service and health care requirements); | nenhum agente antimicrobiano (exceto preservativos), salvo se exigido por códigos de saúde e outros regulamentos (por exemplo, requisitos do serviço de alimentação e do setor de saúde); |
| 21625438c74f-eb76-4985-8e4b-35f466dc36ac | Translation Approved (99%) | Green Seal GS-41, for industrial and institutional hand cleaners; | Green Seal GS-41, para produtos de limpeza para as mãos industriais e institucionais; |
| 216362a9d2dd-ed98-4ee0-ba59-e4a03dd2e88e | Translation Approved (74%) | UL EcoLogo 2784 for hand cleaners and hand soaps; | UL EcoLogo 2784, para produtos de limpeza para as mãos e sabonetes para as mãos; |
| 216424fea67e-0989-4d25-af9a-55bebdc861af | Translation Approved (86%) | UL EcoLogo 2783 for hand sanitizers; | UL EcoLogo 2783, para higienizadores de mão; |
| 2165cf720916-3e90-4e85-b928-07352c95c14f | Translation Approved (92%) | EPA Design for the Environment Program’s standard for safer cleaning products. | Norma para produtos de limpeza mais seguros do projeto do programa ambiental da EPA. |
| 2166f633a1f4-be5d-4861-bac4-b6acdeb5344e | Translation Approved (0%) | For projects outside the U.S., any Type 1 eco-labeling program as defined by ISO 14024: 1999 developed by a member of the Global Ecolabelling Network may be used in lieu of Green Seal or Environmental Choice standards. | Para projetos fora dos EUA, qualquer programa de selo ecológico Tipo 1 conforme definido pela ISO 14024: 1999 desenvolvido por um membro da Global Ecolabelling Network pode ser usado no lugar nas normas Green Seal ou Environmental Choice. |
| 21676dcfa587-e099-469d-b431-422f48f4ff3c | Translation Approved (0%) | EQ Credit: Green Cleaning—Equipment | Crédito EQ: Limpeza Verde - Equipamentos (EQ Credit: Green Cleaning—Equipment) |
| 216864820a17-7153-4e86-969f-f4f5532578c9 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 2169aeda0113-bff9-498e-8591-d61dee44ea88 | Translation Approved (100%) | 1 point | 1 ponto |
| 217079d02faa-e4d4-4fd2-8a32-08974be3ed92 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 2171a5073800-e3d8-4669-a385-4a165bb9707d | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1 point) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 ponto) |
| 21720b699eaa-d8b4-44db-8a84-098e31f327d0 | Translation Approved (100%) | EB:O&M Schools (1 point) | EB:O&M Escolas (Schools) (1 ponto) |
| 21731858b2b0-3568-401f-990b-e21ecbb64071 | Translation Approved (100%) | EB:O&M Retail (1 point) | EB:O&M Varejo (Retail) (1 ponto) |
| 2174a07f79fa-1133-4d18-b132-3ec5903104ce | Translation Approved (100%) | EB:O&M Data Centers (1 point) | EB:O&M Centros de Dados (Data Centers) (1 ponto) |
| 2175c2a288d2-6292-40fa-80ca-fea1425d1bf5 | Translation Approved (100%) | EB:O&M Hospitality (1 point) | EB:O&M Hoteleiro (Hospitality) (1 ponto) |
| 21765639b386-896c-4fc1-9f7d-5d026cdd9826 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1 point) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 ponto) |
| 2177b50683a0-2b26-42ee-97d6-7512f81d621d | Translation Approved (100%) | Intent | Objetivo |
| 2178a888314b-e4bf-4aa4-b99e-dcde43009493 | Translation Approved (0%) | To reduce chemical, biological, and particulate contaminants from powered cleaning equipment. | Reduzir os níveis de produtos químicos, agentes biológicos e contaminantes particulados de equipamentos de limpeza elétricos. |
| 21796d8061ec-20b3-46bc-b8ce-ea4cc298f6c6 | Translation Approved (100%) | Requirement | Requisito |
| 21807b8a1562-3b63-4bdc-8311-4837b9fb4a72 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 2181886a2ec9-291a-48cf-9591-51ea80e79155 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 2182ea19792c-956f-425e-9f81-6e4fb5397951 | Translation Approved (0%) | Create an inventory of existing interior and exterior equipment, including what is brought on site by vendors. | Criar um inventário dos equipamentos internos e externos existentes, incluindo o que é trazido no terreno por vendedores. |
| 2183ea19792c-956f-425e-9f81-6e4fb5397951 | Translation Approved (0%) | At least 40% of all powered janitorial equipment (purchased, leased, or used by contractors) must meet the following criteria. | Pelo menos 40% de todos os equipamentos se limpeza elétricos (comprados, alugados ou usados por empreiteiro) devem seguir os critérios a seguir. |
| 2184ea19792c-956f-425e-9f81-6e4fb5397951 | Translation Approved (0%) | For existing equipment that does not meet the criteria, develop a phase-out plan for its replacement with environmentally preferable products at the end of its useful life. | Para os equipamentos existentes que não atendem aos critérios, desenvolver um plano de substituição para serem substituídos por produtos ecologicamente responsáveis no final de sua vida útil. |
| 2185c455c1b5-6a71-43c7-a585-a502e0845c5f | Translation Approved (0%) | All powered equipment must have the following features: | Todos os equipamentos elétricos devem atender aos seguintes recursos: |
| 218676027dbb-d58b-49ff-bdbf-cd5e471ec664 | Translation Approved (75%) | safeguards, such as rollers or rubber bumpers, to avoid damage to building surfaces; | protetores, como rolos ou amortecedores de borracha, para evitar danos às superfícies de construção; |
| 2187ca15f27d-1c8b-4941-8805-a89c1949f2e3 | Translation Approved (0%) | ergonomic design to minimize vibration, noise, and user fatigue, as reported in the user manual in accordance with ISO 5349-1 for arm vibrations, ISO 2631–1 for vibration to the whole body, and ISO 11201 for sound pressure at operator’s ear; and | projeto ergonômico para minimizar a vibração, o ruído e a fatiga do usuário, conforme registrado no manual do usuário de acordo com a ISO 5349-1 para vibrações do braço, a ISO 2631-1 para vibração de todo o corpo e a ISO 11201 para pressão acústica no ouvido do operador; e |
| 2188d2a21147-1387-41a7-9fd8-59cecaa114f5 | Translation Approved (0%) | as applicable, environmentally preferable batteries (e.g., gel, absorbent glass mat, lithium-ion) except in applications requiring deep discharge and heavy loads where performance or battery life is reduced by the use of sealed batteries. | conforme aplicável, baterias ecologicamente responsáveis (por exemplo, gel, tapete absorvente de vidro, íon de lítio) exceto em aplicações que exigem descarga profunda e cargas pesadas em que o desempenho ou a vida útil da bateria são reduzidos pelo uso de baterias seladas. |
| 21896ae5bbc0-ea2e-4c73-831d-c1b539accb63 | Translation Approved (0%) | Vacuum cleaners must be certified by the Carpet and Rug Institute Seal of Approval/Green Label Vacuum Program and operate with a maximum sound level of 70 dBA or less in accordance with ISO 11201. | Aspiradores de pó devem ser certificados pelo Programa para aspiradores de pó Green Label/Selo de aprovação do Instituto do Carpete e Tapete (Carpet and Rug Institute - CRI) e operar com nível sonoro máximo de 70 dBA, ou inferior, de acordo com a ISO 11201. |
| 21903dc027fc-7966-46d4-98e0-b8e18b7c7c8c | Translation Approved (0%) | Carpet extraction equipment, for restorative deep cleaning, must be certified by the Carpet and Rug Institute's Seal of Approval Deep Cleaning Extractors and Seal of Approval Deep Cleaning Systems program. | Os equipamentos de extração de carpete, para limpeza profunda restauradora, devem ser certificados pelo programa de Selo de aprovação para extratores de limpeza profunda e Selo de aprovação para sistemas de limpeza profunda do Instituto do Carpete e Tapete (Carpet and Rug Institute - CRI). |
| 21910654467d-fc20-4b45-9d08-942dce2f653b | Translation Approved (0%) | Powered floor maintenance equipment must have vacuums, guards, or other devices for capturing fine particles, and must operate with a maximum level of 70 dBA, in accordance with ISO 11201. | Os equipamentos elétricos de manutenção de piso devem possuir dispositivos de sistema a vácuo, protetores ou outros dispositivos para capturar particulados finos e devem operar com nível sonoro máximo de 70 dBA, de acordo com a ISO 11201. |
| 2192986b0fda-82c8-469b-b3e3-6d01d4d4a2b2 | Translation Approved (0%) | Propane-powered floor equipment must have high-efficiency, low-emissions engines with catalytic converters and mufflers that meet the California Air Resources Board or EPA standards for the specific engine size and operate with a sound level of 90 dBA or less, in accordance with ISO 11201. | Os equipamentos de piso movidos a propano devem possuir motores de alta eficiência e baixa emissão, com conversores catalíticos e silenciadores que atendem às normas do CARB ou da EPA para o tamanho de motor específico e operar com um nível sonoro máximo de 90 dBA, ou inferior, de acordo com a ISO 11201. |
| 21933e6a806f-8fe1-4d3d-96e4-16b3b8fe706f | Translation Approved (0%) | Automated scrubbing machines must be equipped with variable-speed feed pumps and either (1) on-board chemical metering to optimize the use of cleaning fluids or (2) dilution control systems for chemical refilling. | Lavadoras de piso automáticas devem ser equipadas com bombas de alimentação com velocidades variáveis e (1) sistema de medição de produtos químicos incorporado para otimizar o uso de líquidos de limpeza ou (2) sistemas de controle de diluição para reabastecimento de produtos químicos. |
| 21943e6a806f-8fe1-4d3d-96e4-16b3b8fe706f | Translation Approved (0%) | Alternatively, scrubbing machines may use tap water only, with no added cleaning products. | Ou, as lavadoras de piso podem usar somente água de torneira com nenhum produto de limpeza adicionado. |
| 21950c6e65e3-a1b6-4021-ad7f-c12500b0eefe | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 219665d6a9f6-6ce2-42bd-9fea-698cbb3fc17c | Translation Approved (97%) | At least 40% of all powered janitorial equipment (purchased, leased, or used by contractors) must meet the above criteria. | Pelo menos 40% de todos os equipamentos se limpeza elétricos (comprados, alugados ou usados por empreiteiro) devem seguir os critérios acima. |
| 219765d6a9f6-6ce2-42bd-9fea-698cbb3fc17c | Translation Approved (100%) | For existing equipment that does not meet the criteria, develop a phase-out plan for its replacement with environmentally preferable products at the end of its useful life. | Para os equipamentos existentes que não atendem aos critérios, desenvolver um plano de substituição para serem substituídos por produtos ecologicamente responsáveis no final de sua vida útil. |
| 219825e14223-b0de-4c78-b6df-3227c4a358c5 | Translation Approved (76%) | EQ Credit: Integrated Pest Management | Crédito EQ: Gerenciamento Integrado de Pragas (EQ Credit: Integrated Pest Management) |
| 2199347de6a9-e9ee-41b2-beee-6250483a87a4 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 2200e2a69326-6551-41f5-b60a-6684572009d9 | Translation Approved (100%) | 2 points | 2 pontos |
| 22018efe6558-4c36-400c-ad53-e5960a824cd1 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 22023ec53b40-2d91-4f25-968f-92b873323626 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (2 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (2 pontos) |
| 22039a0d0ac3-598e-4c72-b34e-785d2d42aa26 | Translation Approved (100%) | EB:O&M Schools (2 points) | EB:O&M Escolas (Schools) (2 pontos) |
| 2204ea072bbd-8b69-4827-a2db-ebd3f210886f | Translation Approved (100%) | EB:O&M Retail (2 points) | EB:O&M Varejo (Retail) (2 pontos) |
| 2205cde1eec3-59b2-412c-a45d-9fe5697d3145 | Translation Approved (100%) | EB:O&M Data Centers (2 points) | EB:O&M Centros de Dados (Data Centers) (2 pontos) |
| 2206e949defe-525d-41b5-a2fe-af4b4aabaa7e | Translation Approved (100%) | EB:O&M Hospitality (2 points) | EB:O&M Hoteleiro (Hospitality) (2 pontos) |
| 22075cdedac4-eb1c-4de5-96a9-6d087f4ac54f | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (2 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (2 pontos) |
| 2208a680c30c-e5a9-4e4b-8e85-644849cae498 | Translation Approved (100%) | Intent | Objetivo |
| 2209a7bc50d5-26a9-4863-8d8b-50fe2a158f4a | Translation Approved (0%) | To minimize pest problems and exposure to pesticides. | Minimizar os problemas de pragas e a exposição a pesticidas. |
| 2210e09255c6-dc00-4d93-991e-1ed526e37606 | Translation Approved (100%) | Requirement | Requisito |
| 221136926f37-7792-4d5b-9cb6-788190dced50 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 2212486fbff8-1baa-4448-8890-bbfc2ea5042e | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 2213b3d0c8ab-0d6d-45e8-95c7-460428e3fcda | Translation Approved (0%) | Have in place an integrated pest management (IPM) plan for the building and grounds within the project boundary. | Implemente um plano de controle integrado de pragas (IPM) para o edifício e o terreno dentro das limitações do projeto. |
| 2214b3d0c8ab-0d6d-45e8-95c7-460428e3fcda | Translation Approved (90%) | The IPM plan must include the following elements. | O plano de IPM deve incluir os seguintes elementos. |
| 2215fa87da80-63a3-4e13-86c1-16dc57718307 | Translation Approved (0%) | Identification of an IPM team. | Identificação de uma equipe de IPM. |
| 2216fa87da80-63a3-4e13-86c1-16dc57718307 | Translation Approved (0%) | Identify roles for building management, pest management contractors, maintenance staff, and liaisons with building occupants. | Identificar as funções da gestão do edifício, dos empreiteiros de controle de pragas, dos funcionários de manutenção e conexões com os ocupantes do edifício. |
| 221748c5e971-5b75-4e89-9400-207fec706c35 | Translation Approved (0%) | Provisions for identifying and monitoring pests. | As providências para identificar e monitorar pragas. |
| 221848c5e971-5b75-4e89-9400-207fec706c35 | Translation Approved (0%) | Specify inspections, pest population monitoring, and a reporting system that allows occupants, maintenance staff, and others to report evidence of pest infestations. | Especificar as inspeções, o monitoramento da população de pragas e um sistema de registro que permite que ocupantes, funcionários de manutenção e outros relatem a evidência de infestações de pragas. |
| 221901df66f9-d7e9-4d59-a795-935667674afd | Translation Approved (0%) | Action thresholds for all pests likely encountered in the building. | Os limites de ação para todas as pestes provavelmente encontradas no edifício. |
| 222001df66f9-d7e9-4d59-a795-935667674afd | Translation Approved (0%) | Also describe a process for modifying action thresholds, if necessary, through active communication between occupants and the IPM team. | Também descrever um processo para modificar os limites de ação, se necessária, por meio da comunicação ativa entre os ocupantes e a equipe de IPM. |
| 2221cfe94207-210f-4035-9b34-087695602e21 | Translation Approved (0%) | Nonchemical pest preventive measures, either designed into the structure or implemented as part of pest management activities. | Medidas preventivas de pragas sem uso de produtos químicos, designadas na estrutura ou implementadas como parte as atividades de controle de pragas. |
| 22222fd0bbc5-ad53-4745-b237-5d5e240fc7ea | Translation Approved (0%) | Pest control methods to be used when action thresholds are exceeded. | Métodos de controle de pragas a serem usados quando o limites de ação forem excedidos. |
| 22232fd0bbc5-ad53-4745-b237-5d5e240fc7ea | Translation Approved (0%) | For each pest, list all potential control methods considered and adopt the lowest-risk options, considering the risks to the applicator, building occupants, and the environment. | Para cada praga, listar todos os métodos de controle potenciais considerados e adotar as opções de risco mais baixo, considerando os riscos ao aplicador, aos ocupantes do edifício e ao meio ambiente. |
| 22242fd0bbc5-ad53-4745-b237-5d5e240fc7ea | Translation Approved (0%) | The plan must preferentially require nonchemical approaches, with pesticides registered for the site applied only if those approaches fail. | O plano deve preferencialmente exigir abordagem sem uso de produtos químicos, com pesticidas registrados para o terreno aplicado somente se essas abordagens falharem. |
| 22252fd0bbc5-ad53-4745-b237-5d5e240fc7ea | Translation Approved (0%) | Give preference to the use of least-risk pesticides based on inherent toxicity and exposure potential. | Dar preferência ao uso de pesticidas de menor risco com base na toxicidade inerente e no potencial de exposição. |
| 22262fd0bbc5-ad53-4745-b237-5d5e240fc7ea | Translation Approved (0%) | If a pesticide that is not in the least-risk category is selected, document the reason. | Se um pesticida que não é da categoria de menor risco for selecionado, documentar o motivo. |
| 2227365e7060-b235-455c-9fa2-a1d8e9ef33ad | Translation Approved (0%) | A mechanism for documentation of inspection, monitoring, prevention, and control methods and for evaluation of the effectiveness of the IPM plan. | Um mecanismo para documentação dos métodos de inspeção, monitoramento, prevenção e controle e para avaliação da eficácia do plano de IPM. |
| 2228365e7060-b235-455c-9fa2-a1d8e9ef33ad | Translation Approved (0%) | Specify the metrics by which performance will be measured, and describe the quality assurance process to evaluate and verify successful implementation of the plan. | Especificar as métricas pelas quais o desempenho será medido e descrever o processo de garantia de qualidade para avaliar e verificar a implementação bem-sucedida do plano. |
| 2229790ab3a4-950c-4005-8f3c-5bdaff8e79f4 | Translation Approved (0%) | A strategy for communications between the IPM team and the building occupants (for schools, faculty and staff). | Um estratégia de comunicação entre a equipe de IPM e os ocupantes do edifício (para escolas, docentes e funcionário) |
| 2230790ab3a4-950c-4005-8f3c-5bdaff8e79f4 | Translation Approved (0%) | This strategy should include education about the IPM plan, participation in problem solving, feedback mechanisms (e.g., a system for recording pest complaints), and provision for notification of pesticide applications. | Essa estratégia deve incluir instrução sobre o plano de IPM, participação na resolução de problemas, mecanismos de feedback (como um sistema para registrar as reclamações quanto a pragas) e providência para a notificação de aplicações de pesticidas. |
| 2231790ab3a4-950c-4005-8f3c-5bdaff8e79f4 | Translation Approved (0%) | At a minimum, the facility manager must notify any building occupant or employee who requests it and post a sign at the application site, which must remain in place for 24 hours. | No mínimo, o gerente da instalação deve notificar os ocupantes e funcionários do edifício que solicitarem as informações e colocar placas de aviso no local de aplicação, que devem ser mantidas por 24 horas. |
| 2232790ab3a4-950c-4005-8f3c-5bdaff8e79f4 | Translation Approved (0%) | Notifications must include the pesticide name, EPA registration number, treatment location, and date of application. | As notificações devem incluir o nome do pesticida, o número de registro EPA, o local de tratamento e a data da aplicação. |
| 2233790ab3a4-950c-4005-8f3c-5bdaff8e79f4 | Translation Approved (0%) | Applications of least-risk pesticides do not require notification. | As aplicações de pesticidas de menor risco não precisam de notificação. |
| 2234790ab3a4-950c-4005-8f3c-5bdaff8e79f4 | Translation Approved (0%) | For an emergency application of a pesticide, anyone who requested notice must be notified within 24 hours of the application and given an explanation of the emergency. | Para uma aplicação de emergência de um pesticida, qualquer pessoa que solicitar um aviso deve ser notificada em até 24 horas da aplicação e receber uma explicação da emergência. |
| 2235076d64e9-1834-4d81-8565-56ec82313703 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 2236d8c6f125-9210-4d61-b23f-6787c38c7f66 | Translation Approved (0%) | Implement the strategies set forth in the IPM plan and evaluate the plan annually. | Implemente as estratégias definidas no plano de IPM e avaliar o plano anualmente. |
| 2237d8c6f125-9210-4d61-b23f-6787c38c7f66 | Translation Approved (0%) | This evaluation must verify that the strategies specified in the IPM plan have been implemented and identify any chemical applications that did not comply with the plan. | Essa avaliação deve verificar se as estratégias especificadas no plano de IPM foram implementadas e identificar qualquer aplicação de produtos químicos que não estiveram em conformidade com o plano. |
| 22388388a144-9260-48ff-b4d4-f6dd5d9800bd | Translation Approved (0%) | Perform recordkeeping and documentation required under the IPM plan. | Realizar a manutenção de registros e a documentação necessária do plano de IPM. |
| 22398388a144-9260-48ff-b4d4-f6dd5d9800bd | Translation Approved (0%) | Maintain records of IPM team participation and decisions, as well as pesticide applications. | Mantenha registros da participação e das decisões da equipe de IPM, bem como das aplicações de pesticidas. |
| 2240ebe2051a-e2fb-4895-80d7-92c996887f2e | Translation Approved (0%) | A project meets the requirements if the IPM service is provided by a certified member in good standing of GreenPro, EcoWise, or GreenShield, or a program with equivalent IPM standards, who complies with the program’s standards. | Um projeto atende aos requisitos se o serviço de IPM for fornecido por membro certificado com boa reputação GreenPro, EcoWise ou GreenShield ou em um programa com normas de IPM equivalente, que está em conformidade com as normas do programa. |
| 22413b2eff80-3491-433d-9625-d418d6fcb588 | Translation Approved (75%) | EQ Credit: Occupant Comfort Survey | Crédito EQ: Pesquisa de Conforto do Ocupante (EQ Credit: Occupant Comfort Survey) |
| 22425b3b0b7f-24a1-4ce9-8f96-0540fe5c9d41 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 224385f5c998-3a66-4d7c-ac65-8990801f752f | Translation Approved (100%) | 1 point | 1 ponto |
| 2244f5de2f11-8887-4f56-a5e1-409b01e1d384 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 22458c468875-76d4-4b26-8bf6-f19faba42350 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1 point) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 ponto) |
| 2246b61a7047-ec98-4393-88d7-8dc2201b4130 | Translation Approved (100%) | EB:O&M Schools (1 point) | EB:O&M Escolas (Schools) (1 ponto) |
| 22473b25db78-06e7-4057-9592-06e2aeda9990 | Translation Approved (100%) | EB:O&M Retail (1 point) | EB:O&M Varejo (Retail) (1 ponto) |
| 224851e7d46a-b6e7-4560-b8cc-10289bcba6f2 | Translation Approved (100%) | EB:O&M Data Centers (1 point) | EB:O&M Centros de Dados (Data Centers) (1 ponto) |
| 224943b90887-bc9a-4084-b7da-63593769c61e | Translation Approved (100%) | EB:O&M Hospitality (1 point) | EB:O&M Hoteleiro (Hospitality) (1 ponto) |
| 225033a5697e-e0a4-48fa-997f-74a00b1d8e37 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1 point) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 ponto) |
| 2251206a55dc-3da2-479d-a5f5-67542fa96375 | Translation Approved (100%) | Intent | Objetivo |
| 22523d38c68e-6398-417c-92b7-62e6a601c665 | Translation Approved (0%) | To assess building occupants’ comfort. | Avaliar o conforto dos ocupantes do edifício. |
| 22536373c18a-8841-4592-973a-b67ad08ac006 | Translation Approved (100%) | Requirements | Requisitos |
| 225428d04757-0244-4efd-948a-218fa3ecd8ca | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 2255e5fa18cb-5488-4c5a-ac74-3fcefbdbdfb6 | Translation Approved (100%) | ESTABLISHMENT | ESTABELECIMENTO |
| 2256d7d82efd-8ba9-4662-9819-673f7b858979 | Translation Approved (100%) | None. | Nenhum. |
| 225793c139d6-92da-46dc-89af-73506e0aca21 | Translation Approved (100%) | PERFORMANCE | DESEMPENHO |
| 2258c749481a-67da-4037-b519-47d20da4ca0f | Translation Approved (0%) | Administer at least one occupant comfort survey to collect anonymous responses regarding at least the following: | Administrar pelo menos uma pesquisa de conforto do ocupante para coletar respostas anônimas com relação ao seguinte: |
| 2259e8493e5f-28eb-4c9c-9c4a-69c055f41f10 | Translation Approved (91%) | acoustics; | acústica, |
| 22605ae08b6a-d1e7-41aa-afa5-fee51992d73c | Translation Approved (94%) | building cleanliness; | limpeza do edifício; |
| 22619043ced2-7aa2-49b7-8767-5f9c5901b035 | Translation Approved (99%) | indoor air quality; | qualidade do ar interno; |
| 2262b16f1df4-3343-476e-808a-56d1d5d0f09e | Translation Approved (0%) | lighting; and | iluminação; e |
| 2263763e5bcc-5b27-45a4-9787-7d6deaa8bacb | Translation Approved (94%) | thermal comfort. | conforto térmico; |
| 22642fb8afa3-c46b-4f38-aeab-0fb99d53a57f | Translation Approved (0%) | The responses must be collected from a representative sample of building occupants making up at least 30% of the total occupants. | As respostas devem ser coletadas de uma amostra representativa de ocupantes do edifício que representam pelo menos 30% do total de ocupantes. |
| 22653d3c207e-2d63-41d6-8adc-e4734432c0af | Translation Approved (0%) | Document survey results. | Documentar os resultados da pesquisa. |
| 22663d3c207e-2d63-41d6-8adc-e4734432c0af | Translation Approved (0%) | Develop and implement a corrective action plan to address comfort issues if the results indicate that more than 20% of occupants are dissatisfied. | Desenvolva e implementar um plano de ação corretiva para abordar os problemas de conforto se os resultados indicarem que mais de 20% dos ocupantes estão insatisfeitos. |
| 2267ee8c9bbe-4624-4dee-9df9-d14a645f3006 | Translation Approved (0%) | Perform at least one survey and implement corrective actions. | Realizar pelo menos uma entrevista e implementar as ações corretivas. |
| 2268ee8c9bbe-4624-4dee-9df9-d14a645f3006 | Translation Approved (0%) | At a minimum, perform one new survey at least once every 2 years. | Realizar uma nova pesquisa pelo menos uma vez a cada 2 anos. |
| 22698c694820-331b-47c0-99a8-0bb91ecfeadd | Translation Approved (71%) | Innovation (IN) | Inovação (IN) (Innovation (IN)) |
| 2270d26f8685-fa70-4dce-9e1f-35aa7ced7679 | Translation Approved (0%) | IN Credit: Innovation | Crédito IN: Inovação (IN Credit: Innovation) |
| 2271deabade5-4aa2-4e8a-b511-d455f82823a7 | Translation Approved (100%) | EB:O&M | EB:O&M |
| 22728a7b11c9-6dfd-4273-bfd7-b6209537d229 | Translation Approved (100%) | 1–5 points | 1 a 5 pontos |
| 2273c7814228-4c9d-4220-8e37-4f8f6dc7e1ce | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 2274fcf4434a-561b-489e-bfc9-eeff949f8488 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1–5 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 a 5 pontos) |
| 22758b44667a-6bce-4b56-919d-d4c52e5380d2 | Translation Approved (100%) | EB:O&M Schools (1–5 points) | EB:O&M Escolas (Schools) (1 a 5 pontos) |
| 227645cb084d-9f09-4ab7-9016-17810ed67b98 | Translation Approved (100%) | EB:O&M Retail (1–5 points) | EB:O&M Varejo (Retail) (1 a 5 pontos) |
| 227774549e01-0fad-4314-a1fa-423731b456f7 | Translation Approved (100%) | EB:O&M Data Centers (1–5 points) | EB:O&M Centros de Dados (Data Centers) (1 a 5 pontos) |
| 2278f6457857-4977-48e7-8ee6-35cc44900a47 | Translation Approved (100%) | EB:O&M Hospitality (1–5 points) | EB:O&M Hoteleiro (Hospitality) (1 a 5 pontos) |
| 2279bb780ca1-82fa-49f3-a912-f9bd4fbd0867 | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) |
| 2280ea7dc273-c337-4508-b6fb-eb2e8c27b1a6 | Translation Approved (100%) | Intent | Objetivo |
| 22812fc9a3df-0594-44f3-b54f-ac30a9956985 | Translation Approved (0%) | To encourage projects to achieve exceptional or innovative performance | Incentivar projetos a atingirem desempenho excepcional ou inovador |
| 2282c7261359-a701-4d38-86b5-dbf40493be6b | Translation Approved (100%) | Requirements | Requisitos |
| 22833a4d9b8f-26bf-4274-837f-6fffb0b9d013 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 228471499558-c26a-4242-9445-561bf920ca53 | Translation Approved (0%) | Project teams can use any combination of innovation, pilot, and exemplary performance strategies. | As equipes do projeto podem usar qualquer combinação de estratégias de desempenho exemplar, piloto e de inovação. |
| 22856341192a-9c6b-4d5d-ad18-61b2c670564b | Translation Approved (100%) | Option 1. | Opção 1. |
| 22866341192a-9c6b-4d5d-ad18-61b2c670564b | Translation Approved (89%) | <1978>Innovation</1978> <1979>(1 point)</1979> | <1978>Inovação (1 ponto)</1978> |
| 22874e703fd2-f82a-43c9-ba7c-f8b4c49aa487 | Translation Approved (0%) | Achieve significant, measurable environmental performance using a strategy not addressed in the LEED green building rating system. | Obter desempenho ambiental mensurável e significativo usando uma estratégia não abordada no sistema de classificação de edifício verde do LEED. |
| 228898255db1-3c40-45ac-8da3-ea2cd1fd45c6 | Translation Approved (0%) | Identify the following: | Identificar o seguinte: |
| 228902f273d8-097d-4d93-b19f-7fc26eeda03e | Translation Approved (99%) | the intent of the proposed innovation credit; | o objetivo do crédito de inovação proposto; |
| 2290d972a37d-f7b7-49e9-ade2-d236e5146453 | Translation Approved (88%) | proposed requirements for compliance; | os requisitos propostos para conformidade; |
| 229130b4c870-2734-4bb8-bb09-1fee61ea478e | Translation Approved (89%) | proposed submittals to demonstrate compliance; and | as submissões propostas para demonstrar a conformidade; e |
| 2292f58a2586-6a4e-4564-bfe6-7927182db5b5 | Translation Approved (0%) | the design approach or strategies used to meet the requirements. | a abordagem ou estratégias do projeto usadas atender aos requisitos. |
| 2293b93766d0-9722-42ef-aec0-2c9e684d1c34 | Translation Approved (100%) | AND/OR | E/OU |
| 22941efdda2c-549e-4691-bfa3-b6f64f904764 | Translation Approved (100%) | Option 2. | Opção 2. |
| 22951efdda2c-549e-4691-bfa3-b6f64f904764 | Translation Approved (88%) | Pilot (1 point) | Piloto (1 ponto) |
| 229650d67e62-fed2-496b-9437-7c51d739ff51 | Translation Approved (0%) | Achieve one pilot credit from USGBC’s LEED Pilot Credit Library | Obter um crédito piloto Biblioteca de crédito piloto LEED do USGBC. |
| 229712ba5d27-779d-4ea2-adc5-fce255e5bd6b | Translation Approved (100%) | AND/OR | E/OU |
| 229893b15241-5870-44e1-b201-feab987c2c93 | Translation Approved (100%) | Option 3. | Opção 3. |
| 229993b15241-5870-44e1-b201-feab987c2c93 | Translation Approved (98%) | Additional Strategies | Estratégias Adicionais |
| 230014a2a755-f0ee-4d2c-ae48-9b8d53a02531 | Translation Approved (74%) | <1982>Innovation</1982> <1983>(1-3 points)</1983> | <1982>Inovação (1 a 3 pontos)</1982> |
| 2301a0c9ef8c-510c-451d-a8af-1c5056585e51 | Translation Approved (0%) | Defined in Option 1 above. | Definido na Opção 1 acima. |
| 2302c4f96cda-4861-4843-820e-961128c101d3 | Translation Approved (74%) | <1984>Pilot</1984> <1985>(1-3 points)</1985> | <1984>Piloto (1 a 3 pontos)</1984> |
| 23039995f184-148d-47d5-a557-f2b4ec51136b | Translation Approved (100%) | Meet the requirements of Option 2. | Atenda aos requisitos da Opção 2. |
| 2304ef319879-059f-46ca-8f12-108ca30592e1 | Translation Approved (89%) | <1986>Exemplary Performance</1986> <1987>(1–2 points)</1987> | <1986>Desempenho exemplar (1 a 2 pontos)</1986> |
| 23050ce9c2d0-a081-4037-ae19-111647816ee8 | Translation Approved (0%) | Achieve exemplary performance in an existing LEED v4 prerequisite or credit that allows exemplary performance, as specified in the LEED Reference Guide, v4 edition. | Obter desempenho exemplar em um pré-requisito ou crédito existente LEED v4 que permite o desempenho exemplar, conforme especificado no Guia de referência LEED, edição v4. |
| 23060ce9c2d0-a081-4037-ae19-111647816ee8 | Translation Approved (74%) | An exemplary performance point is typically earned for achieving double the credit requirements or the next incremental percentage threshold. | Um ponto do desempenho exemplar é normalmente obtido cumprindo em dobro os requisitos do crédito ou atingindo o limiar de porcentagem incremental seguinte. |
| 23071ab2f6be-3a7e-4b68-a0a1-9afd4327b250 | Translation Approved (73%) | IN Credit: LEED Accredited Professional | Crédito IN: Profissional Acreditado LEED (IN Credit: LEED Accredited Professional) |
| 230897d124bb-f2f9-44ce-9b3f-c0987d39ea0d | Translation Approved (100%) | EB:O&M | EB:O&M |
| 2309828daa0c-44b3-4ee6-8d80-225f41c0912e | Translation Approved (100%) | 1 point | 1 ponto |
| 2310d6dad7b4-ea7d-4819-9dfa-ec6aa8513e73 | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 231144adf925-4e00-419d-9028-349fbb02b528 | Translation Approved (100%) | Existing Buildings: Operations & Maintenance (1 point) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 ponto) |
| 2312da18648f-7e7d-4351-9b73-250309ec3013 | Translation Approved (100%) | EB:O&M Schools (1 point) | EB:O&M Escolas (Schools) (1 ponto) |
| 2313504c2f9f-5d17-4eda-b90a-29d13a72c929 | Translation Approved (100%) | EB:O&M Retail (1 point) | EB:O&M Varejo (Retail) (1 ponto) |
| 2314fdf247a5-7ee1-4b26-9d34-5fc66780d05a | Translation Approved (100%) | EB:O&M Data Centers (1 point) | EB:O&M Centros de Dados (Data Centers) (1 ponto) |
| 231578017d8e-ffc9-4eec-b40f-876c6d9599d9 | Translation Approved (100%) | EB:O&M Hospitality (1 point) | EB:O&M Hoteleiro (Hospitality) (1 ponto) |
| 2316139f7cb6-42d7-465f-91ac-035724b0170c | Translation Approved (100%) | EB:O&M Warehouses & Distribution Centers (1 point) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 ponto) |
| 2317f25b3506-9d35-42c6-8e70-d7e1aeb6e61e | Translation Approved (100%) | Intent | Objetivo |
| 231866a2f62c-b896-426d-bc8d-39bf356685bc | Translation Approved (0%) | To encourage the team integration required by a LEED project and to streamline the application and certification process. | Incentivar a integração da equipe necessária por um projeto LEED e simplificar o processo de certificação e inscrição. |
| 2319c4a9eb8b-f273-4300-b846-9f9f32733f22 | Translation Approved (100%) | Requirements | Requisitos |
| 2320d6313c83-42fd-48dd-863e-2ee86dc9a1cd | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 2321364b59d6-d7b1-4d8b-9654-04fd963fda6d | Translation Approved (0%) | At least one principal participant of the project team must be a LEED Accredited Professional (AP) with a specialty appropriate for the project. | Pelo menos um participante principal da equipe do projeto deve ser um Profissional acreditado LEED (LEED Accredited Professional - AP) com uma especialidade adequada para o projeto. |
| 23221a2e2f59-1209-4fd9-8bd2-471579411b2d | Translation Approved (100%) | Regional Priority (RP) | Prioridade Regional (RP) |
| 2323a80e2de3-79ba-4007-b749-2ac796a0ef44 | Translation Approved (0%) | RP Credit: Regional Priority | Crédito RP: Prioridade Regional (RP Credit: Regional Priority) |
| 232476828726-7e64-4e4b-82b9-3c8a7ca24b3c | Translation Approved (100%) | EB:O&M | EB:O&M |
| 2325a25bdf1d-f435-454f-a41f-8e57dc994ec5 | Translation Approved (100%) | 4 points | 4 pontos |
| 2326c0c87814-c695-46df-940f-c24efcbbec5c | Translation Approved (100%) | This credit applies to | Este crédito se aplica a |
| 232737c6f25d-b5f5-43a8-831b-7bf8d043c15a | Translation Approved (96%) | Existing Buildings: Operations & Maintenance (1-4 points) | Edifícios Existentes: Operações e Manutenção (Existing Buildings: Operations and Maintenance) (1 a 4 pontos) |
| 2328e8eca76f-eba8-4cf6-adb9-4ac2bbe92c4a | Translation Approved (95%) | EB:O&M Schools (1-4 points) | EB:O&M Escolas (Schools) (1 a 4 pontos) |
| 2329a87abe1c-3bd4-4342-b92a-ec2ce56b0fcc | Translation Approved (95%) | EB:O&M Retail (1-4 points) | EB:O&M Varejo (Retail) (1 a 4 pontos) |
| 23305f4719e3-101e-447c-b9e7-0af3c62d22c5 | Translation Approved (100%) | EB:O&M Data Centers (1-4 points) | EB:O&M Centros de Dados (Data Centers) (1 a 4 pontos) |
| 233113276d7e-4688-418b-960e-fbff453e7a1e | Translation Approved (95%) | EB:O&M Hospitality (1-4 points) | EB:O&M Hoteleiro (Hospitality) (1 a 4 pontos) |
| 2332227a3944-a1d1-411b-8b77-d8928095098a | Translation Approved (97%) | EB:O&M Warehouses & Distribution Centers (1-4 points) | EB:O&M Armazéns e Centros de Distribuição (Warehouses and Distribution Centers) (1 a 4 pontos) |
| 233350e4f736-1d12-4ec1-ad71-8ab8b137231c | Translation Approved (100%) | Intent | Objetivo |
| 233482c06cb0-21fe-4e21-a59d-d2fcf6baedde | Translation Approved (79%) | To provide an incentive for the achievement of credits that address geographically specific environmental, social equity, and public health priorities. | Oferecer um incentivo para a obtenção de créditos que abordam prioridades ambientais geograficamente específicas, prioridades de igualdade social e prioridades de saúde pública. |
| 2335b00cc46f-a516-489a-b47e-aa3e5e402e4d | Translation Approved (100%) | Requirements | Requisitos |
| 23368b4b140b-2043-45d2-988f-6d3aa1415eb4 | Translation Approved (100%) | EBOM, Schools, Retail, Data Centers, Hospitality, Warehouses & Distribution Centers | EBOM, Escolas (Schools), Varejo (Retail), Centros de Dados (Data Centers), Hoteleiro (Hospitality), Armazéns e Centros de Distribuição (Warehouses & Distribution Centers) |
| 2337f59ccd69-4509-40b5-bc98-4c7c657fdf8e | Translation Approved (0%) | Earn up to four of the six Regional Priority credits. | Obter até quatro dos seis créditos de Prioridade Regional. |
| 2338f59ccd69-4509-40b5-bc98-4c7c657fdf8e | Translation Approved (0%) | These credits have been identified by the USGBC regional councils and chapters as having additional regional importance for the project’s region. | Esses créditos devem identificados pelos conselhos regionais e por capítulos do USGBC como tendo importância ambiental adicional para a região de um projeto. |
| 2339f59ccd69-4509-40b5-bc98-4c7c657fdf8e | Translation Approved (96%) | A database of Regional Priority credits and their geographic applicability is available on the USGBC website, <1997>http://www.usgbc.org</1997>. | Um banco de dados de créditos de Prioridade Regional e sua aplicabilidade geográfica está disponível no site do USGBC, <1997>http://www.usgbc.org</1997>. |
| 2340e440c1a1-0366-4567-b438-821eaee8ef0a | Translation Approved (100%) | http://www.usgbc.org | http://www.usgbc.org |
| 2341ed108ab4-27a1-476c-9619-17265b6cb9bd | Translation Approved (0%) | One point is awarded for each Regional Priority credit achieved, up to a maximum of four. | Um ponto é concedido para cada crédito de Prioridade Regional obtido, até um máximo de quatro. |
| 2342f70df8ec-1a6b-47e0-8b35-f9926198ebeb | Translation Approved (100%) | Appendices | Apêndices |
| 23433dc98812-74c3-47a1-a0a1-6bc2e8f8c1f5 | Translation Approved (72%) | Appendix 1. | Apêndice 1. |
| 23443dc98812-74c3-47a1-a0a1-6bc2e8f8c1f5 | Translation Approved (0%) | Use Types and Categories | Tipos de uso e categorias |
| 23459a28a219-b14b-4c32-9dbf-b2f13cd1e3cb | Translation Approved (100%) | Table 1. | Tabela 1. |
| 23469a28a219-b14b-4c32-9dbf-b2f13cd1e3cb | Translation Approved (100%) | Use Types and Categories | Tipos de uso e categorias |
| 23473829c49e-6df7-4029-afd2-ba7b2656e0ff | Translation Approved (86%) | Category | Categoria |
| 234852a4a31d-e5fd-46b7-8819-372b90df8122 | Translation Approved (0%) | Use type | Tipos de uso |
| 2349e703eb0c-6320-4922-810a-6af1aaa5a2a2 | Translation Approved (100%) | Food retail | Varejo de alimentos |
| 2350aa5fc188-4edb-4211-95c6-5d460db77d7a | Translation Approved (100%) | Supermarket | Supermercado |
| 23511ffd18fe-80d6-4c1d-b932-da875ace1935 | Translation Approved (0%) | Grocery with produce section | Mercado com sessão de hortifrúti |
| 23522a281367-389b-4127-b764-5239c3cbeb38 | Translation Approved (100%) | Community-serving retail | Varejo que atende à comunidade |
| 2353605dae73-aa93-4563-a877-4ed016678a3d | Translation Approved (0%) | Convenience store | Loja de conveniência |
| 2354df2ac7a6-99f1-4a6e-b3a6-e3cb8a303ec6 | Translation Approved (0%) | Farmers market | Mercado de agricultores |
| 2355fb12d28a-b472-4a17-86cd-d066f7d0c258 | Translation Approved (0%) | Hardware store | Loja de ferragens |
| 2356ab1b0310-cfb5-47ea-8de1-1269799834eb | Translation Approved (100%) | Pharmacy | Farmácia |
| 23572a6c7ea9-ae92-4ed0-981a-c53a19aafd34 | Translation Approved (CM) | Other retail | Outro varejo |
| 235878d84489-d9ab-4b8f-af5f-9881309dd867 | Translation Approved (0%) | Services | Serviços |
| 2359b29b1e33-dd2d-4dc3-9b62-9ffa3a4ea3e8 | Translation Approved (100%) | Bank | Banco |
| 2360e106e5d4-749f-4566-84b0-e8492e93b31a | Translation Approved (0%) | Family entertainment venue (e.g., theater, sports) | Espaço de entretenimento familiar (teatro, esportes) |
| 2361584ef71d-c4b1-4748-b55c-b78fe3bcacd5 | Translation Approved (0%) | Gym, health club, exercise studio | Academia, clube de saúde, estúdio de exercício |
| 23627fdfcb64-dd25-4c4f-9f20-43004b2eadcd | Translation Approved (0%) | Hair care | Cabeleireiro |
| 2363f8103b88-1e36-42df-9d09-d3ed3b42572f | Translation Approved (0%) | Laundry, dry cleaner | Lavanderia, empresa de limpeza a seco |
| 2364e679b4a9-2a71-45ee-b1e4-b1fad93d5578 | Translation Approved (0%) | Restaurant, café, diner (excluding those with only drive-thru service) | Restaurante, cafeteria, lanchonete (excluindo aquelas com apenas serviço drive-thru) |
| 23657f050cea-3099-4fb4-a595-879a4f3e5cde | Translation Approved (0%) | Civic and community facilities | Instalações cívicas e comunitárias |
| 23660c36249f-e4f4-413c-94e1-d5c7c2b38522 | Translation Approved (0%) | Adult or senior care (licensed) | Asilos e casas de repouso (licenciados) |
| 2367ef2d14ac-91fd-4986-875a-2cd856c38dce | Translation Approved (0%) | Child care (licensed) | Creches (licenciadas) |
| 2368c83b2b90-5bb7-43ed-81a5-3deb39336dc9 | Translation Approved (0%) | Community or recreation center | Centro comunitário ou recreativo |
| 23697a8db179-0167-454d-b14c-cba44ef082df | Translation Approved (0%) | Cultural arts facility (museum, performing arts) | Instalação de arte cultural (museu, artes cênicas) |
| 23704fdf7608-42c0-4210-9ae5-290af1a136ff | Translation Approved (0%) | Education facility (e.g., K—12 school, university, adult education center, vocational school, community college) | Instituição educacional (escolas infantis, universidade, centro de educação para adultos, escola vocacional, centro técnico) |
| 2371c569db3e-2952-4da2-8bfe-6588f3151e77 | Translation Approved (0%) | Government office that serves public on-site | Órgão público que serve ao público no local |
| 237210917963-8cd2-470c-ac02-bcf1d8d40a41 | Translation Approved (0%) | Medical clinic or office that treats patients | Clínica média ou escritório que trata pacientes |
| 23731b471522-8ec3-4833-8576-0715fdf46feb | Translation Approved (0%) | Place of worship | Local de adoração |
| 23749b92cc48-297b-475b-b90d-e0ab8eb8ef47 | Translation Approved (0%) | Police or fire station | Polícia ou corpo de bombeiros |
| 23755c15f163-3eb4-4f1f-bbf5-d0a2d70eb701 | Translation Approved (0%) | Post office | Correio |
| 2376282c8dfd-0ebd-4f17-8b09-316bf885afa6 | Translation Approved (0%) | Public library | Biblioteca pública |
| 2377efdaef68-384b-449d-acc9-276a0e634007 | Translation Approved (0%) | Public park | Parque público |
| 2378ba1b2b35-7762-4a46-af22-0475fda90f07 | Translation Approved (0%) | Social services center | Centro de serviço social |
| 2379f4faca5a-2136-41a2-9060-7a76f6a84f3d | Translation Approved (0%) | Community anchor uses (BD&C and ID&C only) | Usos de suporte comunitário (somente BD&C e ID&C) |
| 23807bdebf8c-e68e-4269-9cf7-58f7ea031f80 | Translation Approved (0%) | Commercial office (100 or more full-time equivalent jobs) | Escritório comercial (100 ou mais empregos de tempo integral equivalentes) |
| 23813d3978b0-4b29-4323-bf65-bee6e61852e1 | Translation Approved (0%) | Housing (100 or more dwelling units) | Habitação (100 ou mais unidades residenciais) |
| 2382ed875db3-36eb-4645-a661-9ad6a9f78976 | Translation Approved (0%) | Adapted from Criterion Planners, INDEX neighborhood completeness indicator, 2005. | Adaptado do indicador de integridade de bairro INDEX da Criterion Planners, 2005. |
| 2383f38f2dc9-485c-4932-8cd7-3743ec98d511 | Translation Approved (100%) | Appendix 2. | Apêndice 2. |
| 2384f38f2dc9-485c-4932-8cd7-3743ec98d511 | Translation Approved (99%) | Default Occupancy Counts | Contagens de ocupação padrão |
| 2385f97e8e21-b7bb-4337-ab87-f3625c118fc1 | Translation Approved (0%) | Use Table 1 to calculate default occupancy counts. | Usar a Tabela 1 para calcular as contagens de ocupação padrão. |
| 2386f97e8e21-b7bb-4337-ab87-f3625c118fc1 | Translation Approved (0%) | Only use the occupancy estimates if occupancy is unknown. | Somente usar as estimativas de ocupação se a ocupação for desconhecida. |
| 2387318b4778-a52b-449a-9eb4-a3aaab7dda0c | Translation Approved (0%) | For the calculation, use gross floor area, not net or leasable floor area. | Para o cálculo, use a área de piso bruta, não a área de piso líquida ou locável. |
| 2388318b4778-a52b-449a-9eb4-a3aaab7dda0c | Translation Approved (0%) | Gross floor area is defined as the sum of all areas on all floors of a building included within the outside faces of the exterior wall, including common areas, mechanical spaces, circulation areas, and all floor penetrations that connect one floor to another. | A área de piso bruto é definida como a soma de todas as áreas em todos os andares de um edifício incluídas nas fazes exteriores da parede exterior, incluindo áreas de uso comum, espaços mecânicos, áreas de circulação e todas as penetrações de piso que conectam um andar ao outro. |
| 2389318b4778-a52b-449a-9eb4-a3aaab7dda0c | Translation Approved (0%) | To determine gross floor area, multiply the building footprint (in square feet or square meters) by the number of floors in the building. | Para determinar a área de piso brita, é necessário multiplicar a pegada ecológica (em pés quadrados ou metros quadrados) pelo número de andares do edifício. |
| 2390318b4778-a52b-449a-9eb4-a3aaab7dda0c | Translation Approved (0%) | Exclude underground or structured parking from the calculation. | Exclua estacionamentos subterrâneos ou estruturados do cálculo. |
| 23912373d572-d5a9-45e0-89f7-46b2e098272a | Translation Approved (100%) | Table 1. | Tabela 1. |
| 23922373d572-d5a9-45e0-89f7-46b2e098272a | Translation Approved (77%) | Default Occupancy Numbers | Números de ocupação padrão |
| 23934dfff027-d022-4de4-9b05-a83cb954f3ff | Translation Approved (0%) | Gross square feet per occupant | Pés quadrados brutos por ocupante |
| 239460fc7c1c-b0e5-43de-bb43-713460120cba | Translation Approved (84%) | Gross square meters per occupant | Metros quadrados brutos por ocupante |
| 23952010037b-ccf3-450d-88bf-6d28ceb4c03a | Translation Approved (0%) | Employees | Funcionários |
| 23966567e9ac-fa41-4e03-88a4-923400875b01 | Translation Approved (0%) | Transients | Ocupantes esporádicos |
| 2397e1c68467-d784-4e98-b557-6a7ace632cd3 | Translation Approved (100%) | Employees | Funcionários |
| 2398176d8534-3e0a-40d0-8cad-191b750bfe10 | Translation Approved (CM) | Transients | Ocupantes esporádicos |
| 2399335b82db-9a25-4118-9d75-ad016a8ddadb | Translation Approved (100%) | General office | Escritório geral |
| 240003b41ccd-8b01-48f3-a328-85737ea3ee9a | Translation Approved (100%) | 250 | 250 |
| 240177c2a63d-3bb7-4702-9114-1516d19182e1 | Translation Approved (100%) | 0 | 0 |
| 2402775d8ff0-480a-4fa1-86a4-7ad41e598e34 | Translation Approved (100%) | 23 | 23 |
| 240343b9abe7-d2f5-4213-b8af-d316403c7bb4 | Translation Approved (100%) | 0 | 0 |
| 240442e4788b-8db6-4f75-87a0-7d23e888c9f8 | Translation Approved (CM) | Retail, general | Varejo, geral |
| 2405b94da0b6-2772-49ac-9b5d-4934ad46f982 | Translation Approved (100%) | 550 | 550 |
| 24060898c756-2ae0-4c5c-a8cb-12537673c4f3 | Translation Approved (100%) | 130 | 130 |
| 2407ea5ba990-2464-444f-b89b-d1a02b653a23 | Translation Approved (100%) | 51 | 51 |
| 2408ced71bd9-3e06-4b94-8063-d4fa24aed84b | Translation Approved (100%) | 12 | 12 |
| 2409b35263a0-5fe2-44a5-b06b-f857a6571e4d | Translation Approved (CM) | Retail or service (e.g., financial, auto) | Varejo ou serviço (ex.: financeiro, automotivo) |
| 2410fb6a8fa0-3563-4648-9b76-b747dc15a711 | Translation Approved (100%) | 600 | 600 |
| 24110a4ab2ec-24c4-4382-9615-7406654dcf0b | Translation Approved (100%) | 130 | 130 |
| 2412e91a0f44-187a-4429-b9d6-e7ebaf17f1e9 | Translation Approved (100%) | 56 | 56 |
| 241304b067d2-c075-4948-bf61-a3b1f41089a8 | Translation Approved (100%) | 12 | 12 |
| 24143fd26177-e7f3-4fe9-971b-f9d8f70c5533 | Translation Approved (100%) | Restaurant | Restaurante |
| 24158703771a-710b-4acc-b40f-02522ae2d784 | Translation Approved (100%) | 435 | 435 |
| 24168c2959de-8e68-4093-be29-5965e885d55d | Translation Approved (100%) | 95 | 95 |
| 2417cd42dc8f-071b-4bcd-822e-ed788e99b7b8 | Translation Approved (100%) | 40 | 40 |
| 2418724c8dbb-b710-465e-8de7-011aa78afb05 | Translation Approved (100%) | 9 | 9 |
| 2419e1afff5f-02db-4669-9e98-64d83d7ef8fc | Translation Approved (0%) | Grocery store | Mercado |
| 242029d20604-1e5f-49ba-b558-0ce01be044cc | Translation Approved (100%) | 550 | 550 |
| 24215cb90b0f-f14d-4059-943e-2d74767c087a | Translation Approved (100%) | 115 | 115 |
| 2422fdbcd569-efb2-4331-a494-eef10fbe59d4 | Translation Approved (100%) | 51 | 51 |
| 242339609739-dd7e-4784-880b-12931a21a8bf | Translation Approved (100%) | 11 | 11 |
| 24240f1600e0-f5ff-4dd7-b990-86e6a702a82e | Translation Approved (0%) | Medical office | Clínica médica |
| 2425a325e987-8eb6-413b-a4c3-e74c3c17bb45 | Translation Approved (100%) | 225 | 225 |
| 24268ce73a9c-c95b-4ee8-81af-b467c86839e9 | Translation Approved (100%) | 330 | 330 |
| 2427af0b345c-2396-4694-944b-51ee2b7c3d87 | Translation Approved (100%) | 21 | 21 |
| 242823b50407-3c6e-4f2c-a074-2abb9c5ca989 | Translation Approved (100%) | 31 | 31 |
| 24299af1ad9b-981a-4d7e-92f9-ca1725ad4a6b | Translation Approved (0%) | R&D or laboratory | P&D ou laboratório |
| 24305f98bf69-e4c6-4e1f-a563-46d4966d8684 | Translation Approved (100%) | 400 | 400 |
| 243135976477-cdde-43c8-a8b9-9db093328e92 | Translation Approved (100%) | 0 | 0 |
| 24320d197f23-c5f0-4b14-a34f-9ad2fbe3fde7 | Translation Approved (100%) | 37 | 37 |
| 24331063d4fd-d2f6-4870-a9d5-33b4959c9abd | Translation Approved (100%) | 0 | 0 |
| 24342c426e43-6323-412c-bafd-60faab861da4 | Translation Approved (0%) | Warehouse, distribution | Galpão, distribuição |
| 243508708490-6f24-49e6-ba77-a4213b4a69c5 | Translation Approved (100%) | 2,500 | 2.500 |
| 2436cd0f3148-3c2f-424d-ade2-3acd7c9e5c76 | Translation Approved (100%) | 0 | 0 |
| 24377d3b659f-88b7-4e12-8a78-2dbd0b4c1fa0 | Translation Approved (100%) | 232 | 232 |
| 2438d7333df8-3e11-4a63-ba56-af4adf404205 | Translation Approved (100%) | 0 | 0 |
| 2439f912fa07-85a5-41b3-921c-4f4c4840e720 | Translation Approved (0%) | Warehouse, storage | Galpão, armazenamento |
| 24405b3d325a-cc86-4ad9-9dfd-ad87a651e2dc | Translation Approved (100%) | 20,000 | 20.000 |
| 2441d10be3da-c4f1-4299-b579-0deb9b003914 | Translation Approved (100%) | 0 | 0 |
| 24427f11633d-92da-420b-b960-3634626491ad | Translation Approved (100%) | 1860 | 1860 |
| 2443e3e486dd-ba3b-4260-874b-5e9fa63278de | Translation Approved (100%) | 0 | 0 |
| 244489a4f032-3a44-478d-b480-629f3b90464c | Translation Approved (100%) | Hotel | Hotel |
| 24450878aa8a-1b39-421e-bf6f-caac178eca6e | Translation Approved (100%) | 1,500 | 1.500 |
| 2446ea572da2-5f53-4a57-b61e-e97f4073d907 | Translation Approved (100%) | 700 | 700 |
| 2447104e6f41-c5b5-4393-999f-e9bacac300d8 | Translation Approved (100%) | 139 | 139 |
| 24489ac8df69-e648-42ce-a68c-c5367441d986 | Translation Approved (100%) | 65 | 65 |
| 2449ce15787c-ec6c-42a8-b64e-e8ba97689139 | Translation Approved (0%) | Educational, daycare | Educacional, creche |
| 2450f8b45309-94df-4f81-b6b8-753f6ab26036 | Translation Approved (100%) | 630 | 630 |
| 245101889aae-d3e5-488e-9288-b483fe9ebabd | Translation Approved (100%) | 105 | 105 |
| 245224694d60-66b3-4ebd-a57b-a753382d7ddb | Translation Approved (100%) | 59 | 59 |
| 245307757bdb-c5de-40b4-93e3-65fd70e97205 | Translation Approved (100%) | 10 | 10 |
| 2454fa632973-870c-438a-bbb1-8e592423f175 | Translation Approved (0%) | Educational, K–12 | Educacional, ensino infantil |
| 24556b78fc2d-fdd1-4f06-821f-b1c183a80c57 | Translation Approved (100%) | 1,300 | 1.300 |
| 2456a3408e17-984a-4e76-b119-34830012d772 | Translation Approved (100%) | 140 | 140 |
| 245795bed86d-08e6-45fa-b5cb-41e5a6180c68 | Translation Approved (100%) | 121 | 121 |
| 245846911246-fcb6-414b-9127-c8b0207f22f4 | Translation Approved (100%) | 13 | 13 |
| 24595d0ae851-28b1-4b5b-8058-518ca63fc898 | Translation Approved (0%) | Educational, postsecondary | Educacional, ensino superior |
| 2460943992cf-bd79-45d7-9bf6-0b49e9950d3b | Translation Approved (100%) | 2,100 | 2.100 |
| 2461bab9f2e3-1950-49f1-a97a-f47531cf16cb | Translation Approved (100%) | 150 | 150 |
| 2462a32fec75-ee2e-4471-9817-18ee4449ca33 | Translation Approved (100%) | 195 | 195 |
| 2463f679b8e9-7733-4c40-9716-4c9de402bf9c | Translation Approved (100%) | 14 | 14 |
| 24642f00949e-2531-45c0-ae3f-c400c198daa5 | Translation Approved (91%) | Sources: | Fontes: |
| 246581d7fffe-cbda-4715-9bac-3685ee77d735 | Translation Approved (0%) | ANSI/ASHRAE/IESNA Standard 90.1–2004 (Atlanta, GA, 2004). | ANSI/ASHRAE/IESNA Standard 90.1–2004 (Atlanta, GA, 2004). |
| 2466b09eb1fc-23ec-4f58-95e0-751c1f6cd320 | Translation Approved (0%) | 2001 Uniform Plumbing Code (Los Angeles, CA) | 2001 Uniform Plumbing Code (Los Angeles, CA) |
| 246775f83e3c-39d8-4a46-beab-9d32f75a853c | Translation Approved (0%) | California Public Utilities Commission, 2004–2005 Database for Energy Efficiency Resources (DEER) Update Study (2008). | California Public Utilities Commission, 2004–2005 Database for Energy Efficiency Resources (DEER) Update Study (2008). |
| 2468c1259f58-cb5c-40da-9b58-9fbc59840c0f | Translation Approved (CM) | California State University, Capital Planning, Design and Construction Section VI, Standards for Campus Development Programs ( Long Beach, CA, 2002). | California State University, Capital Planning, Design and Construction Section VI, Standards for Campus Development Programs (Long Beach, CA, 2002). |
| 2469f97c1218-dd35-4f44-b452-0765ad026c33 | Translation Approved (CM) | City of Boulder Planning Department, Projecting Future Employment—How Much Space per Person (Boulder, 2002). | City of Boulder Planning Department, Projecting Future Employment—How Much Space per Person (Boulder, 2002). |
| 24709022c5b9-a2f7-414c-a78c-42021079215e | Translation Approved (CM) | Metro, 1999 Employment Density Study (Portland, OR 1999). | Metro, 1999 Employment Density Study (Portland, OR 1999). |
| 2471f24e2808-9c0e-4114-9894-697ea73e4cab | Translation Approved (0%) | American Hotel and Lodging Association, Lodging Industry Profile Washington, DC, 2008. | American Hotel and Lodging Association, Lodging Industry Profile Washington, DC, 2008. |
| 2472d7f7f26e-243e-455c-9a28-a9a6119ba5b1 | Translation Approved (0%) | LEED for Core & Shell Core Committee, personal communication (2003 - 2006). | LEED for Core & Shell Core Committee, personal communication (2003 - 2006). |
| 24730a6f150e-169b-414f-ae2f-f6f7c04e51e4 | Translation Approved (CM) | LEED for Retail Core Committee, personal communication (2007) | LEED for Retail Core Committee, personal communication (2007) |
| 2474b882e5e8-e123-4ee1-8c2f-e0c5541a6d00 | Translation Approved (CM) | OWP/P, Medical Office Building Project Averages (Chicago, 2008). | OWP/P, Medical Office Building Project Averages (Chicago, 2008). |
| 2475922dc48b-fceb-4537-850e-dd24543137af | Translation Approved (CM) | OWP/P, University Master Plan Projects (Chicago, 2008). | OWP/P, University Master Plan Projects (Chicago, 2008). |
| 24764d3b11d8-310b-4efe-a186-2bed4ea9ca1d | Translation Approved (0%) | U.S. General Services Administration, Childcare Center Design Guide (Washington, DC,2003). | U.S. General Services Administration, Childcare Center Design Guide (Washington, DC,2003). |
| 24771ddc8c0a-764d-4d5c-9814-adf34783330d | Translation Approved (100%) | Appendix 3. | Apêndice 3. |
| 24781ddc8c0a-764d-4d5c-9814-adf34783330d | Translation Approved (CM) | Retail Process Load Baselines | Baselines de cargas de processos de varejo |
| 247999a384c0-b92f-4bbc-9c6d-984966b9422c | Translation Approved (0%) | Table 1a. | Tabela 1a. |
| 248099a384c0-b92f-4bbc-9c6d-984966b9422c | Translation Approved (CM) | Commercial kitchen appliance prescriptive measures and baseline for energy cost budget (IP units) | Baseline e medidas prescritivas para utensílios para cozinhas comerciais para orçamento de custos de energia (sistema imperial) |
| 2481e5d3da25-e202-41e0-8063-222d8e48438c | Translation Approved (100%) | Baseline energy usage for energy modeling path | Baseline do uso de energia do caminho de modelagem de energia |
| 2482a9bcb8d3-4f84-43e6-a5b2-6a089084233c | Translation Approved (CM) | Levels for prescriptive path | Níveis do caminho prescritivo |
| 2483186cd7e1-0b1a-40d5-848c-1ba4f64107e5 | Translation Approved (100%) | Appliance type | Tipo de utensílio |
| 2484b4719d0a-68af-4f39-9f3e-688837246fc0 | Translation Approved (100%) | Fuel | Combustível |
| 24852bdcbdd4-a0ed-47c6-b0e2-48af5dcd3e4a | Translation Approved (100%) | Function | Função |
| 248609970e09-1e90-43e7-8986-1df25aca0740 | Translation Approved (100%) | Baseline efficiency | Eficiência da baseline |
| 2487873dde56-9e52-43c6-ac39-e246be0b2e3c | Translation Approved (100%) | Baseline idle rate | Taxa de inatividade da baseline |
| 2488429107f6-127a-420a-9df9-b6ea8e26520b | Translation Approved (100%) | Prescriptive efficiency | Eficiência prescritiva |
| 248921a508c3-b7d2-4205-a156-b6d7ff25f400 | Translation Approved (100%) | Prescriptive idle rate | Taxa de inatividade prescritiva |
| 2490cb7b42dd-9173-4065-a93f-6529eb90929c | Translation Approved (100%) | Broiler, underfired | Grelha, aquecida por baixo |
| 2491323f58e7-704e-4492-85a2-3fdc078e0d13 | Translation Approved (100%) | Gas | Gás |
| 249264ced36a-5b9d-4b09-9992-5464b9368722 | Translation Approved (100%) | Cooking | Culinária |
| 24930d107bf4-5e39-4e5e-a7a7-00c445f1fd33 | Translation Approved (100%) | 30% | 30% |
| 2494e8aae8d2-45ba-4f17-9cb8-d237ecd2e5fe | Translation Approved (CM) | 16,000 Btu/h/ft<2459>2</2459> peak input | Pico de entrada 16.000 Btu/h/ft<2459>2</2459> |
| 2495a5f42511-b021-46f4-a325-44bdc374615f | Translation Approved (100%) | 35% | 35% |
| 2496c76df4a6-0f88-4030-a309-ba8f0a1e2403 | Translation Approved (100%) | 12,000 Btu/h/ft<2466>2</2466> peak input | Pico de entrada 12.000 Btu/h/ft<2466>2</2466> |
| 249793177f79-d073-4ddc-aaa9-4345dbfd5ead | Translation Approved (CM) | Combination ovens, steam mode (P = pan capacity) | Fornos de combinação, modo vapor (P = capacidade da panela) |
| 2498b847812f-b1e6-496c-b835-435df1d108d4 | Translation Approved (0%) | Elec | Elét. |
| 24992cb1393e-e353-4403-8684-08d26eafb374 | Translation Approved (100%) | Cooking | Culinária |
| 250009a2b0f1-1030-4a80-8d86-b24fd153e9d1 | Translation Approved (CM) | 40% steam mode | 40% modo vapor |
| 2501b3ce7804-20a1-4144-bfa9-84f8272fdec8 | Translation Approved (CM) | 0.37P+4.5 kW | 0,37P + 4,5 kW |
| 2502fb981e35-cf98-4dc6-b73d-fd4acff8be4d | Translation Approved (100%) | 50% steam mode | 50% modo vapor |
| 25035207af79-89cf-4dcd-a1f1-ef60e39cb512 | Translation Approved (0%) | 0.133P+0.6400 kW | 0,133P+0,6400 kW |
| 25044283c57d-30f7-40e1-b26b-ebe6738e73a6 | Translation Approved (0%) | Combination ovens, steam mode | Fornos combinados, modo de vapor |
| 250566dd8bb6-0b26-42f7-a1a3-f7395a63cfbe | Translation Approved (100%) | Gas | Gás |
| 250607885a18-eb20-4bce-91c6-2c76327c32ea | Translation Approved (100%) | Cooking | Culinária |
| 250705ffe660-323f-43e7-ab2c-237116ec248e | Translation Approved (100%) | 20% steam mode | 20% modo vapor |
| 2508f73ba473-5440-4fc7-b233-84aa80a41529 | Translation Approved (0%) | 1,210P+35,810 Btu/h | 1.210P+35.810 Btu/h |
| 2509c8b82785-d634-4c6f-9122-304a0ba08438 | Translation Approved (100%) | 38% steam mode | 38% modo vapor |
| 25100dbec5f7-90c2-4ae2-b643-999487c4207a | Translation Approved (0%) | 200P+6,511 Btu/h | 200P+6.511 Btu/h |
| 25110c800426-64cd-4c63-9609-ca26034c9eaa | Translation Approved (83%) | Combination ovens, convection mode | Fornos combinados, modo de convecção |
| 2512ef18a7f5-ce26-41ac-ba85-e37c4807b02a | Translation Approved (100%) | Elec | Elét. |
| 2513e0fc4fa5-8060-4107-abf2-903875cd0cd4 | Translation Approved (100%) | Cooking | Culinária |
| 251410faa169-db6e-4a77-8b17-10960fe3eaee | Translation Approved (77%) | 65% convection mode | 65% em modo de convecção |
| 251558513f53-b1f6-4349-8299-3be6d9598371 | Translation Approved (0%) | 0.1P+1.5 kW | 0,1P+1,5 kW |
| 2516c123cb00-ecb3-4f35-96cf-107125037e29 | Translation Approved (100%) | 70% convection mode | 70% em modo de convecção |
| 251705017786-3440-4003-b02e-933624d0d83a | Translation Approved (0%) | 0.080P+0.4989 kW | 0,080P+0,4989 kW |
| 25181cd11aed-f3cf-4e8a-bc2d-ecfeb9f0c69c | Translation Approved (100%) | Combination ovens, convection mode | Fornos combinados, modo de convecção |
| 2519eee9cc19-762b-4fcc-bb77-cd7b75d3f19c | Translation Approved (100%) | Gas | Gás |
| 2520917e012e-0474-4a5b-9699-5229433a16bf | Translation Approved (100%) | Cooking | Culinária |
| 2521dcb960fa-9b99-4c62-aec5-bb49368813dc | Translation Approved (CM) | 35% convection mode | 35% em modo de convecção |
| 252261605c40-1b48-4a8e-b6e2-9ec933e4827f | Translation Approved (0%) | 322P+13,563 Btu/h | 322P+13.563 Btu/h |
| 2523f210009e-8ac2-466c-872c-f270b67e1576 | Translation Approved (100%) | 44% convection mode | 44% em modo de convecção |
| 25244d6e5a12-6eed-4708-b95a-5a7882966f3c | Translation Approved (0%) | 150P+5,425 Btu/h | 150P+5.425 Btu/h |
| 25255149924f-bce7-477d-a2bb-618932c6aa9f | Translation Approved (0%) | Convection oven, full-size | Forno de convecção, tamanho grande |
| 252648659c10-245a-4d8a-a474-bb10f8c929af | Translation Approved (100%) | Elec | Elét. |
| 252751822e99-542a-4d3c-9563-198d82d499f3 | Translation Approved (100%) | Cooking | Culinária |
| 2528ecf2dfeb-9616-4d31-9bd5-bc7a91896a96 | Translation Approved (100%) | 65% | 65% |
| 252917989847-84cb-4c95-94fc-1aba62923e59 | Translation Approved (95%) | 2.0 kW | 2,0 kW |
| 2530e02b79ff-bb5d-47c5-a054-9a5c542af301 | Translation Approved (100%) | 71% | 71% |
| 2531f10e882b-c114-4e9e-90b6-494bc38c54c4 | Translation Approved (100%) | 1.6 kW | 1,6 kW |
| 2532e83101a7-42b3-465f-9884-da6712792ec2 | Translation Approved (100%) | Convection oven, full-size | Forno de convecção, tamanho grande |
| 253352261b8b-0c6b-4c99-867c-c90bd6a141d6 | Translation Approved (100%) | Gas | Gás |
| 25344bdc3f74-d74c-43c7-bb4a-cd08be0c1068 | Translation Approved (100%) | Cooking | Culinária |
| 2535b979d35e-7d71-4fb7-bdd8-41f0e5fb4d3a | Translation Approved (100%) | 30% | 30% |
| 25362280f61c-2ad5-4341-a8b0-499c9b0c1698 | Translation Approved (100%) | 18,000 Btu/h | 18.000 Btu/h |
| 25373def9980-14a3-4214-918c-6edc221d38e4 | Translation Approved (100%) | 46% | 46% |
| 253819ec865f-1a47-4190-a614-6c80f4d8e43b | Translation Approved (CM) | 12,000 Btu/h | 12.000 Btu/h |
| 25392d41023a-b830-465e-a7a5-5bdd2c5aa9d0 | Translation Approved (78%) | Convection oven, half-size | Forno de convecção, tamanho médio |
| 2540f7207e03-b1d9-4044-938d-de6aa1f5b4d3 | Translation Approved (100%) | Elec | Elét. |
| 25416885dce0-5485-4195-b12c-3e32460d4ce4 | Translation Approved (100%) | Cooking | Culinária |
| 2542bcf094c2-b8f1-4aae-9fb9-e88424f79aa2 | Translation Approved (100%) | 65% | 65% |
| 2543eb2fff38-5c4a-4324-8708-9d16136fc10d | Translation Approved (CM) | 1.5 kW | 1,5 kW |
| 2544cd66d58a-0313-4e5b-be54-39db7f96183d | Translation Approved (100%) | 71% | 71% |
| 2545e5d5662f-556a-4f1e-bc60-bffb92d620e1 | Translation Approved (CM) | 1.0 kW | 1,0 kW |
| 2546205ed788-000c-4b06-87f0-f0831dfb3e5c | Translation Approved (0%) | Conveyor oven, > 25-inch belt | Forno de esteira, correia > 25 pol. |
| 25471dee178f-ffbd-46dd-bce4-8a02a428e4e1 | Translation Approved (100%) | Gas | Gás |
| 2548cf65a84b-1b16-41ae-9a04-d1871e58c26f | Translation Approved (100%) | Cooking | Culinária |
| 2549c634a56f-3f93-43a2-bf4b-cf639cfe0306 | Translation Approved (100%) | 20% | 20% |
| 2550b90f36e0-80cf-40a8-9057-2e4b5b17a938 | Translation Approved (80%) | 70,000 Btu/h | 70.000 Btu/h |
| 255138631ca6-5dae-4aed-9e8e-08a7be12c2e6 | Translation Approved (100%) | 42% | 42% |
| 2552eb7528ed-1318-4a3f-a369-7649e79c2b3a | Translation Approved (CM) | 57,000 Btu/h | 57.000 Btu/h |
| 2553d27b0c0e-1d2c-4098-bd88-9610ffb594e4 | Translation Approved (99%) | Conveyor oven, ≤ 25-inch belt | Forno de esteira, correia ≤ 25 pol. |
| 255483cd59f4-247b-4a69-be86-c475de7837cc | Translation Approved (100%) | Gas | Gás |
| 25555e3aca27-3ac3-4dc8-9db1-820981680b99 | Translation Approved (100%) | Cooking | Culinária |
| 2556c64a2d5a-42bd-4b51-ae52-603c8bef52f8 | Translation Approved (100%) | 20% | 20% |
| 255702630a4c-ef02-4ac8-8e23-3754f9f0f735 | Translation Approved (CM) | 45,000 Btu/h | 45.000 Btu/h |
| 2558949462a2-72aa-4d50-ae50-b1715d96fbbe | Translation Approved (100%) | 42% | 42% |
| 25595a5ee742-eaa6-4b1d-a158-c9c8382e2ae7 | Translation Approved (CM) | 29,000 Btu/h | 29.000 Btu/h |
| 2560b760d6b3-a682-4223-bf3f-91fa61ab0792 | Translation Approved (0%) | Fryer | Fritadeira |
| 256149805fb7-718f-4d3a-a353-6065750ac619 | Translation Approved (100%) | Elec | Elét. |
| 25627f9db915-abe5-4c4b-af29-549246fc4002 | Translation Approved (100%) | Cooking | Culinária |
| 25635347f5fe-69b6-4151-bbee-0b2b14308dd1 | Translation Approved (100%) | 75% | 75% |
| 256419f7f533-0dde-42eb-a58e-11a8a807274f | Translation Approved (CM) | 1.05 kW | 1,05 kW |
| 25656df41cf9-9d62-484e-9c6a-3fd6214e09eb | Translation Approved (100%) | 80% | 80% |
| 2566ef4dd288-23bf-4f3a-9eaf-8a6555415ea5 | Translation Approved (CM) | 1.0 kW | 1,0 kW |
| 2567c38cb771-068a-433f-9fea-f07fb92fd0a3 | Translation Approved (100%) | Fryer | Fritadeira |
| 2568721944ae-bb4b-4cbd-aa61-e331715d4c2b | Translation Approved (100%) | Gas | Gás |
| 25698a669aa5-e010-4852-8d46-3b128a0bebd4 | Translation Approved (100%) | Cooking | Culinária |
| 2570ed8e2ca9-6a75-4744-bbd3-cca08ae46afa | Translation Approved (100%) | 35% | 35% |
| 2571a2994f52-3b29-4426-8f8a-2eecbc9f8105 | Translation Approved (CM) | 14,000 Btu/h | 14.000 Btu/h |
| 25723342394d-de21-405e-9b07-7d761a1c8be7 | Translation Approved (100%) | 50% | 50% |
| 25738e67e9e6-2d00-4ef6-a484-f62aca64345e | Translation Approved (CM) | 9,000 Btu/h | 9.000 Btu/h |
| 2574fbb29929-f22b-49ea-8416-cc4f7811f9cb | Translation Approved (CM) | Griddle (based on 3 ft model) | Frigideira (com base no modelo de 3 pés) |
| 2575fae71042-e83b-4826-a379-3ba59556decb | Translation Approved (100%) | Elec | Elét. |
| 2576b90ac3a0-4124-4e35-b2f5-dfa19dd7e360 | Translation Approved (100%) | Cooking | Culinária |
| 2577ee9b1b5a-bd61-4ab4-ac88-247e03518313 | Translation Approved (100%) | 60% | 60% |
| 2578116a0c98-93a6-46ae-871d-c51972341b01 | Translation Approved (0%) | 400 W/ft<2713>2</2713> | 400 W/ft<2713>2</2713> |
| 2579ab94e805-d2ed-4a4d-80bb-a1268a87900d | Translation Approved (100%) | 70% | 70% |
| 258020fe0a1c-a10d-4c51-92b6-4af11050bf9e | Translation Approved (100%) | 320 W/ft<2720>2</2720> | 320 W/ft<2720>2</2720> |
| 2581a488e7f4-27ee-44ab-a0b9-f9b0772421f9 | Translation Approved (100%) | Griddle (based on 3 ft model) | Frigideira (com base no modelo de 3 pés) |
| 258277138ac8-087c-46a9-b4b5-6eaccfdf368c | Translation Approved (100%) | Gas | Gás |
| 258355767992-e052-4321-a72b-bfec6a2806ce | Translation Approved (100%) | Cooking | Culinária |
| 2584ec709f72-97ab-4ba5-9721-083538181e52 | Translation Approved (100%) | 30% | 30% |
| 25853e739ce0-4181-4db5-8239-c5ce22c51d3e | Translation Approved (0%) | 3,500 Btu/h/ft<2736>2</2736> | 3.500 Btu/h/ft<2736>2</2736> |
| 258656a72c4f-974d-467f-9cb8-480f1a02482e | Translation Approved (100%) | 38% | 38% |
| 25873cb02011-9f63-49cc-b774-bcb6b4e323f6 | Translation Approved (CM) | 2,650 Btu/h/ft<2743>2</2743> | 2.650 Btu/h/ft<2743>2</2743> |
| 25884049f6de-ec7e-416b-b532-df631fc49b7f | Translation Approved (CM) | Hot food holding cabinets (excluding drawer warmers and heated display), 0 < V < 13 ft³ (V = volume) | Armários de armazenamento de alimentos quentes (exceto aquecedores de gavetas e prateleiras aquecidas), 0 < V < 13 ft³ (V = volume) |
| 25893e3c0e76-85d6-40ba-be7a-5234c4d7edb5 | Translation Approved (100%) | Elec | Elét. |
| 259093082e17-e395-4f32-8733-1a21d584474d | Translation Approved (100%) | Cooking | Culinária |
| 2591c9503e0d-922b-4b19-b208-d54759bf33cb | Translation Approved (CM) | na | n/d |
| 25926a8591c9-fe04-4f29-a326-92f09d42307f | Translation Approved (100%) | 40 W/ft<2759>3</2759> | 40 W/ft<2759>3</2759> |
| 2593142fdc57-fff3-4eab-965e-10704efc07a1 | Translation Approved (100%) | na | n/d |
| 2594cd82396f-3f6d-44dc-89e3-df02b46fb62b | Translation Approved (CM) | 21.5V Watts | 21,5 V Watts |
| 2595e0cb8e58-e1b8-4ddd-9409-68d437385205 | Translation Approved (CM) | Hot food holding cabinets (excluding drawer warmers and heated display), 13 ≤ V < 28 ft³ | Armários de armazenamento de alimentos quentes (exceto aquecedores de gavetas e prateleiras aquecidas), 13 ≤ V < 28 ft³ |
| 2596778552a2-78cb-4b6e-9751-b7b36370d3fd | Translation Approved (100%) | Elec | Elét. |
| 2597f9c81abf-7682-4e41-8de7-37aa03d3891e | Translation Approved (100%) | Cooking | Culinária |
| 25982ea4f3dd-2006-4dd6-a914-3e33e474b620 | Translation Approved (100%) | na | n/d |
| 25997fb0a04b-09c3-4b06-836b-56d3785b9c46 | Translation Approved (CM) | 40 W/ft<2781>3</2781> | 40 W/ft<2781>3</2781> |
| 2600ceb0a091-dbc9-42c4-b77d-1e01ea5162cb | Translation Approved (100%) | na | n/d |
| 260130b39a8f-6544-47a4-9a27-0857c701ba31 | Translation Approved (0%) | 2.0V + 254 Watts | 2,0 V + 254 Watts |
| 26026e2dab63-14fa-4eea-bacc-c9cd6bff750d | Translation Approved (CM) | Hot food holding cabinets (excluding drawer warmers and heated display), 28 ft³ ≤ V | Armários de armazenamento de alimentos quentes (exceto aquecedores de gavetas e prateleiras aquecidas), 28 ft³ ≤ V |
| 26035b74cd4f-7992-4280-88ea-0969c7a5ed33 | Translation Approved (100%) | Elec | Elét. |
| 2604de6ec3f9-52fd-4eca-9a37-b197c7306c75 | Translation Approved (100%) | Cooking | Culinária |
| 2605109450cf-591f-4cfd-b17e-ed444f6f66ae | Translation Approved (100%) | na | n/d |
| 2606b80d47a1-af64-4658-a850-2b22d4532306 | Translation Approved (CM) | 40 W/ft<2803>3</2803> | 40 W/ft<2803>3</2803> |
| 260768b27ffe-2150-430d-b34b-ab7f80a4a4eb | Translation Approved (100%) | na | n/d |
| 26085aa5a03c-a422-45a5-9e1e-86d0fbff3e81 | Translation Approved (CM) | 3.8V + 203.5 Watts | 3,8V + 203,5 Watts |
| 2609c848b6b8-e5d3-4688-814b-0e706d065856 | Translation Approved (CM) | Large vat fryer | Fritadeira de tanque grande |
| 2610c527e020-18b8-4136-b71e-f810c86894dd | Translation Approved (100%) | Elec | Elét. |
| 2611fba15af2-6a28-4e1a-af15-b7d9fd1587df | Translation Approved (100%) | Cooking | Culinária |
| 2612f958b8c7-b3a0-4154-8891-3adc2eb5df9d | Translation Approved (100%) | 75% | 75% |
| 26132db25152-de59-41e5-87e2-fe7eb06156e0 | Translation Approved (CM) | 1.35 kW | 1,35 kW |
| 26149af96fd5-828f-43c0-bb36-35dbd8fcd44c | Translation Approved (100%) | 80% | 80% |
| 261503bbc99e-ae26-4169-88f8-fadc3e223615 | Translation Approved (CM) | 1.1 kW | 1,1 kW |
| 2616457f4859-9ad4-4abe-9b7c-38e0a259501e | Translation Approved (100%) | Large vat fryer | Fritadeira de tanque grande |
| 2617e1af9a7d-9448-40ae-bb1d-6f5f01f36b12 | Translation Approved (100%) | Gas | Gás |
| 2618cc6e51e4-d24b-44ce-bf0b-48975f9892cd | Translation Approved (100%) | Cooking | Culinária |
| 26190ab0a2f0-701c-4ae2-8896-e227768ba984 | Translation Approved (100%) | 35% | 35% |
| 26208d9dab88-bf33-446f-a86a-4657595957b8 | Translation Approved (CM) | 20,000 Btu/h | 20.000 Btu/h |
| 2621f372e9ee-4fb7-4f37-9652-245a9b90c916 | Translation Approved (100%) | 50% | 50% |
| 2622b14e8bb0-8c81-47b3-90ad-250c0ad45219 | Translation Approved (CM) | 12,000 Btu/h | 12.000 Btu/h |
| 2623019d7390-16bb-4cb9-93eb-bf9ba22d4de4 | Translation Approved (100%) | Rack oven, double | Forno rotativo, duplo |
| 2624ab74fa04-bbe8-4c61-b133-d40f6fadac47 | Translation Approved (100%) | Gas | Gás |
| 26259142d2ee-bf26-44ea-9172-e97ae5b931ef | Translation Approved (100%) | Cooking | Culinária |
| 262641afbde2-6ab4-4a71-850f-ea1ea0e22045 | Translation Approved (100%) | 30% | 30% |
| 262766be937b-b45d-443d-93ee-e66bd1cd3acb | Translation Approved (CM) | 65,000 Btu/h | 65.000 Btu/h |
| 26281c44f18a-0f3a-47f9-a63c-f4dc78a01c94 | Translation Approved (100%) | 50% | 50% |
| 2629a4fca2f7-df6a-4d79-b3fe-efaad567dea4 | Translation Approved (CM) | 35,000 Btu/h | 35.000 Btu/h |
| 2630bc5d910d-28fd-4c9e-ac88-d3d8f98bcbdd | Translation Approved (100%) | Rack oven, single | Forno rotativo, simples |
| 263132ec701c-345b-4893-9600-851e30515897 | Translation Approved (100%) | Gas | Gás |
| 2632600c7d0a-2b3d-4dec-a9f9-7a325f549318 | Translation Approved (100%) | Cooking | Culinária |
| 26332eff4b2a-4c59-47c0-8165-642b0448ee21 | Translation Approved (100%) | 30% | 30% |
| 2634830aa1bb-7149-4d9e-8ba3-17734f6afa6e | Translation Approved (CM) | 43,000 Btu/h | 43.000 Btu/h |
| 2635be8d7542-5414-4d92-9d59-854c7c4f57b6 | Translation Approved (100%) | 50% | 50% |
| 263653d26e65-e17a-426c-a12d-c92777f28745 | Translation Approved (CM) | 29,000 Btu/h | 29.000 Btu/h |
| 263772d2c55b-460b-459a-872d-e6a641d052cb | Translation Approved (CM) | Range | Fogão |
| 263801f107b2-1e9a-43f5-99ef-4a88e414785d | Translation Approved (100%) | Elec | Elét. |
| 263943f57c94-c39e-47f8-a37f-3ac7e9f56bc4 | Translation Approved (100%) | Cooking | Culinária |
| 2640cfa73b06-3ad5-4e03-8e99-bac451fd02ea | Translation Approved (100%) | 70% | 70% |
| 26415ef362a8-1591-49b1-9e21-01e4329713f6 | Translation Approved (100%) | 80% | 80% |
| 2642e6911ed4-19a4-4b9f-93c2-87771db063f0 | Translation Approved (100%) | Range | Fogão |
| 26433af9dd92-09f0-49f6-8292-e2e7dae6a1de | Translation Approved (100%) | Gas | Gás |
| 264449a22cbb-ebf0-42eb-adbc-e71b53455f6c | Translation Approved (100%) | Cooking | Culinária |
| 26450c2710c4-0520-4ebe-8ff8-2de519feefff | Translation Approved (100%) | 35% | 35% |
| 2646056df6d3-9729-4dba-8d76-6ee58fb843e9 | Translation Approved (100%) | na | n/d |
| 26478c746367-688c-4710-8bc8-eb2595418938 | Translation Approved (0%) | 40% and no standing pilots | 40% e sem chama piloto |
| 2648f4069ec6-14bf-4354-b2bf-5bb2ea07d185 | Translation Approved (100%) | na | n/d |
| 2649fe71116a-646f-45f9-b58c-07df4280893c | Translation Approved (CM) | Steam cooker, batch cooking | Panela a vapor, cozinhar em lote |
| 26508a8e57a0-6b53-4c8d-8666-80cc22dd307d | Translation Approved (100%) | Elec | Elét. |
| 2651f6a83e05-44bb-4778-a546-ee9bca2ba118 | Translation Approved (100%) | Cooking | Culinária |
| 2652bb7e817d-f4f8-4e92-9d7e-85d3282a4127 | Translation Approved (100%) | 26% | 26% |
| 2653d0cd42b6-35d2-473e-b457-ef90a3a0db96 | Translation Approved (0%) | 200 W/pan | 200 W/panela |
| 2654f1decd87-befc-49d1-ab84-1ab6bbff09f1 | Translation Approved (100%) | 50% | 50% |
| 26557b6fb39c-0182-40a1-8cff-17346563c154 | Translation Approved (CM) | 135 W/pan | 135 W/panela |
| 26568b7f5627-ab1e-4542-9102-204090d6c1eb | Translation Approved (100%) | Steam cooker, batch cooking | Panela a vapor, cozinhar em lote |
| 265732ad2ace-cbe3-4e0d-81fb-40344b8710e4 | Translation Approved (100%) | Gas | Gás |
| 265874c74fb9-810f-46b0-b3cb-43c410d49bfb | Translation Approved (100%) | Cooking | Culinária |
| 2659cfe9c27c-fb6d-42fc-8cf4-2ef7df150a52 | Translation Approved (100%) | 15% | 15% |
| 2660eb028b87-732d-4f21-9fb3-84b9ca1e42ec | Translation Approved (CM) | 2,500 Btu/h/pan | 2.500 Btu/h/panela |
| 26614c00ac33-485f-475d-b828-6f2df0cde2e8 | Translation Approved (100%) | 38% | 38% |
| 2662335ec863-f3af-45f1-b609-e1f593e8da71 | Translation Approved (100%) | 2,100 Btu/h/pan | 2.100 Btu/h/panela |
| 266395ed719f-4fda-49aa-85e9-a4dc9567ee6f | Translation Approved (CM) | Steam cooker, high production or cook to order | Panela a vapor, alta produção ou "cook to order" |
| 2664faa84d55-da26-4183-8e4c-c4b992939f74 | Translation Approved (100%) | Elec | Elét. |
| 2665d7219604-cc9e-41bc-bd13-dd4d45dbaad2 | Translation Approved (100%) | Cooking | Culinária |
| 2666586d9484-57ec-44ad-b73a-8cd0dea9dcce | Translation Approved (100%) | 26% | 26% |
| 266736b2a238-a36b-4d88-b5c2-2b789cc3fba5 | Translation Approved (CM) | 330 W/pan | 330 W/panela |
| 2668b7725eb1-d986-48e2-aeee-70fb2dbbf872 | Translation Approved (100%) | 50% | 50% |
| 26690cd43eeb-ec00-4639-94f1-9a0f0321abd8 | Translation Approved (CM) | 275 W/pan | 275 W/panela |
| 26707b0c8a3e-940f-4ed8-ad51-a1987bbde974 | Translation Approved (100%) | Steam cooker, high production or cook to order | Panela a vapor, alta produção ou "cook to order" |
| 26717b7ec9bc-bb67-4d52-8e61-ad5a2bfd20f7 | Translation Approved (100%) | Gas | Gás |
| 267213f6af9f-6eac-4cb5-963f-64d9712cf931 | Translation Approved (100%) | Cooking | Culinária |
| 26732e68f6c5-e28a-4d04-9f58-4db9234ae17d | Translation Approved (100%) | 15% | 15% |
| 2674cf871f06-b1d6-4c65-b927-eca6c0f2edbe | Translation Approved (100%) | 5,000 Btu/h/pan | 5.000 Btu/h/panela |
| 2675316277ab-8d45-41d2-919f-def8b09912a1 | Translation Approved (100%) | 38% | 38% |
| 2676ff626557-c07a-451e-8b21-0fabdc9d50a1 | Translation Approved (100%) | 4,300 Btu/h/pan | 4.300 Btu/h/panela |
| 2677e0c72ab0-f173-4ef8-a03f-59b5496f7c7f | Translation Approved (CM) | Toaster | Torradeira |
| 26783c445759-6795-4fc1-9889-dbf1dcd6aee2 | Translation Approved (100%) | Elec | Elét. |
| 267930ca0731-0673-4543-9c0c-ee95453aeda8 | Translation Approved (100%) | Cooking | Culinária |
| 2680c220a181-cfd5-4e51-a3fc-3c1bbb7b9873 | Translation Approved (0%) | — | — |
| 268113ca2ea8-0591-49cc-bb02-de8be6c0bd79 | Translation Approved (CM) | 1.8 kW average operating energy rate | 1,8 kW de taxa de energia média de operação |
| 268253d4c78a-75aa-48f2-88d9-502cbaf1e498 | Translation Approved (100%) | na | n/d |
| 26835209b188-cea8-495e-b62c-95ccd74fa89a | Translation Approved (100%) | 1.2 kW average operating energy rate | 1,2 kW de taxa de energia média de operação |
| 26842c7b1bb4-e455-4a20-bbd7-b62d7abd0df8 | Translation Approved (CM) | Ice machine, IMH (ice-making head, H = ice harvest), H <3044>></3044> 450 lb/day | Máquina de gelo, IMH (cabeçote de produção de gelo, H = coleta de gelo), H <3044>></3044> 450 lb/dia |
| 268565fdb6c0-7912-439d-b291-fa40d24cd5e1 | Translation Approved (100%) | Elec | Elét. |
| 2686d140f3d6-b116-4c55-bf22-2bf144a1c48a | Translation Approved (0%) | Ice | Gelo |
| 26875eda7d77-ed39-4d36-9b2c-c93dc168c1ef | Translation Approved (0%) | 6.89 - 0.0011H kWh/100 lb ice | 6,89 - 0,0011H kWh/100 lb de gelo |
| 26884b41e77d-a09d-43d1-832c-69bb45ef14c2 | Translation Approved (100%) | na | n/d |
| 26899117c466-dd65-43e7-9ad6-e35477e68f10 | Translation Approved (0%) | 37.72*H<3060>-0.298 </3060>kWh/100 lb ice | 37,72*H<3060>-0,298 </3060>kWh/100 lb de gelo |
| 269089e37960-80f3-44f0-a285-50d4908c9ffe | Translation Approved (100%) | na | n/d |
| 26919b025e16-0536-4793-8f29-c8407101241c | Translation Approved (CM) | Ice machine, IMH (ice-making head), H <3067>< </3067>450 lb/day | Máquina de gelo, IMH (cabeçote de produção de gelo), H <3067>< </3067>450 lb/dia |
| 269280c628b9-7963-43e1-8851-e9dab27a6aff | Translation Approved (100%) | Elec | Elét. |
| 269366e7bc15-e5e4-4c58-a85e-de5a4b87b7e6 | Translation Approved (100%) | Ice | Gelo |
| 26949f1d1790-e821-4ef4-be2b-f5f66062e31e | Translation Approved (0%) | 10.26 – 0.0086H kWh/100 lb ice | 10,26 – 0,0086H kWh/100 lb de gelo |
| 269567809550-d145-4ba5-9061-645ad1251bb8 | Translation Approved (100%) | na | n/d |
| 26967ad1e194-323a-416e-ab31-4d13849225ea | Translation Approved (CM) | 37.72*H<3083>-0.298 </3083>kWh/100 lb ice | 37,72*H<3083>-0,298 </3083>kWh/100 lb de gelo |
| 2697ae353a27-4560-4207-9843-417d941b6bca | Translation Approved (100%) | na | n/d |
| 2698192b90e7-6921-4c6d-8948-0554ee4498bd | Translation Approved (CM) | Ice machine, RCU (remote condensing unit, w/o remote compressor, H < 1,000 lb/day | Máquina de gelo, RCU (unidade de condensação remota, sem compressor remoto), H < 1.000 lb/dia |
| 26998024ec58-afc3-4666-9ea7-8d7e6d8cf5a0 | Translation Approved (100%) | Elec | Elét. |
| 270028a4098a-63a8-40fb-8f08-408fd0fc352c | Translation Approved (CM) | Ice | Gelo |
| 2701bc19b68b-4aa0-46b3-bfff-c502cad64c29 | Translation Approved (0%) | 8.85 - 0.0038H kWh/100lb ice | 8,85 - 0,0038H kWh/100lb de gelo |
| 27022dc517bb-bcdc-4087-840a-b26ff84a0ccd | Translation Approved (100%) | na | n/d |
| 2703178bef62-2d55-4edc-b5d4-e43fe3626198 | Translation Approved (0%) | 22.95*H<3105>-0.258 </3105>+ 1.00 kWh/100 lb ice | 22,95*H<3105>-0,258 </3105>+ 1,00 kWh/100 lb de gelo |
| 270462e7c3e3-d0de-4e39-be5f-8b000e8c5231 | Translation Approved (100%) | na | n/d |
| 2705d420e36a-9182-4530-8c84-e48aa08e0ac7 | Translation Approved (CM) | Ice machine, RCU (remote condensing unit), 1600 > H <3112>></3112> 1000 lb/day | Máquina de gelo, RCU (unidade de condensação remota), 1600 > H <3112>></3112> 1.000 lb/dia |
| 2706e0147095-d3f6-48d6-9701-286e69964a31 | Translation Approved (100%) | Elec | Elét. |
| 270750776f7b-ffa9-4ced-a98e-bc1b458cf397 | Translation Approved (CM) | ice | gelo |
| 270801df707f-a88f-40dd-8db5-76035ac9399f | Translation Approved (0%) | 5.10 kWh/100 lb ice | 5,10 kWh/100 lb de gelo |
| 2709b82eff47-d067-4843-9a7f-225d7d651a12 | Translation Approved (100%) | na | n/d |
| 27105fe42ecb-585e-44c9-957c-cd954f13188d | Translation Approved (0%) | 22.95*H<3128>-0.258</3128> + 1.00 kWh/100 lb ice | 22,95*H<3128>-0,258</3128> + 1,00 kWh/100 lb de gelo |
| 27114e52c313-30e0-46f0-9fa1-247ca42d5610 | Translation Approved (100%) | na | n/d |
| 2712cd12ad32-0112-45c7-914c-9d985626caf6 | Translation Approved (CM) | Ice machine, RCU (remote condensing unit), H ≥ 1600 lb/day | Máquina de gelo, RCU (unidade de condensação remota), H ≥ 1600 lb/dia |
| 27133be7c5ba-9f20-484c-87d2-67624050646b | Translation Approved (100%) | Elec | Elét. |
| 2714facb4be1-62a1-414d-9d72-b0172cc99298 | Translation Approved (CM) | Ice | Gelo |
| 2715b7d3e417-6fe9-49dd-b2fb-12df97008a01 | Translation Approved (CM) | 5.10 kWh/100lb ice | 5,10 kWh/100lb de gelo |
| 2716ead69a86-72a0-45ae-bf6d-5d5541d598dc | Translation Approved (100%) | na | n/d |
| 271714837b84-e680-460e-b8ad-5531ab96cc19 | Translation Approved (0%) | -0.00011*H + 4.60 kWh/100 lb ice | -0,00011*H + 4,60 kWh/100 lb de gelo |
| 27182b20335e-1126-4c2c-9cfa-4ada10cb05c1 | Translation Approved (100%) | na | n/d |
| 2719d4b88db5-d992-4a12-935b-51e8f72db55a | Translation Approved (CM) | Ice machine, SCU (self-contained unit), H < 175 lb/day | Máquina de gelo, SCU (unidade autônoma), H < 175 lb/dia |
| 27202a0ccc94-23c8-4276-80d4-9cdedaf1d7f5 | Translation Approved (100%) | Elec | Elét. |
| 272187dd7fc7-ef39-4e0e-9d23-c96c2596e7fa | Translation Approved (CM) | Ice | Gelo |
| 27227e779b1a-7080-44ce-857f-b890b9a3b717 | Translation Approved (0%) | 18.0 - 0.0469H kWh/100lb ice | 18,0 - 0,0469H kWh/100lb de gelo |
| 27236c84b329-5fca-4dd0-962a-5d99cf3d5468 | Translation Approved (100%) | na | n/d |
| 2724e6cc9174-ac4f-4960-8a5c-645068403597 | Translation Approved (CM) | 48.66*H<3171>-0.326</3171> + 0.08 kWh/100 lb ice | 48,66*H<3171>-0,326</3171> + 0,08 kWh/100 lb de gelo |
| 272531d1ba0e-7ad0-412a-b434-4a2ab301c155 | Translation Approved (100%) | na | n/d |
| 27268f586f14-0177-4d90-9c8e-09f0582f8cf0 | Translation Approved (CM) | Ice machine self-contained unit, H <3178>></3178> 175 lb/day | Máquina de gelo, unidade autônoma, H <3178>></3178> 175 lb/dia |
| 2727715698de-9068-4310-951e-8b37140d3a91 | Translation Approved (100%) | Elec | Elét. |
| 2728aef0fe11-44f5-4be5-8811-d6e63968be1b | Translation Approved (CM) | Ice | Gelo |
| 27295de1f54a-e4d7-40f5-ad34-c3971c2d9119 | Translation Approved (CM) | 9.80 kWh/100 lb ice | 9,80 kWh/100 lb de gelo |
| 27305745c49b-69ef-4b08-ab2f-a79ae116e37f | Translation Approved (100%) | na | n/d |
| 27319d7c59d2-18c8-4b5d-9fd3-4fbd9d5afbde | Translation Approved (CM) | 48.66*H<3194>-0.326</3194> + 0.08 kWh/100 lb ice | 48,66*H<3194>-0,326</3194> + 0,08 kWh/100 lb de gelo |
| 2732eea94a40-5db0-47bf-b82d-58b8e0977176 | Translation Approved (100%) | na | n/d |
| 2733720bde73-a791-4c57-bff4-a997b41bf5f3 | Translation Approved (CM) | Ice machine, water-cooled ice-making head, H <3201>></3201> 1436 lb/day (must be on chilled loop) | Máquina de gelo, cabeçote de produção de gelo resfriado à água, H <3201>></3201> 1436 lb/dia (deve estar em ciclo resfriado) |
| 27342087d8fa-43d9-41e8-8deb-441d43bc39bb | Translation Approved (100%) | Elec | Elét. |
| 27350627f877-19ec-411e-9482-d9760f103692 | Translation Approved (CM) | Ice | Gelo |
| 2736a338bc66-770a-4224-8b84-58cd729b0e24 | Translation Approved (CM) | 4.0 kWh/100 lb ice | 4,0 kWh/100 lb de gelo |
| 273731784878-0d60-4053-9647-a16b731e9a66 | Translation Approved (100%) | na | n/d |
| 27381d564f49-70ff-43da-b372-051096633648 | Translation Approved (100%) | 3.68 kWh/100 lb ice | 3,68 kWh/100 lb de gelo |
| 273917497d0f-0121-45ac-b03b-db081fe414d2 | Translation Approved (100%) | na | n/d |
| 27408645891c-1c09-44c5-8315-ce6de0864702 | Translation Approved (CM) | Ice machine, water-cooled ice-making head, 500 lb/day < H < 1436 (must be on chilled loop) | Máquina de gelo, cabeçote de produção de gelo resfriado à água, 500 lb/dia < H < 1436 (deve estar em ciclo resfriado) |
| 2741c428282a-3b92-4b27-b63e-7fee3f4963a1 | Translation Approved (100%) | Elec | Elét. |
| 27427c18af85-9fa1-42c9-9b62-b485b268e78a | Translation Approved (CM) | Ice | Gelo |
| 274357e195de-15a1-41a1-9d77-fc9f9d8d7138 | Translation Approved (0%) | 5.58 – 0.0011H kWh/100 lb ice | 5,58 – 0,0011H kWh/100 lb de gelo |
| 274438598405-3e15-42cf-88e2-8321362d664b | Translation Approved (100%) | na | n/d |
| 2745c670aaeb-a31a-459e-8cf7-656725bcbb84 | Translation Approved (CM) | 5.13 - 0.001H kWh/100 lb ice | 5,13 – 0,001H kWh/100 lb de gelo |
| 2746edf9fd81-9a11-42ed-96d2-30f9b2721baf | Translation Approved (100%) | na | n/d |
| 274731523340-e510-443a-8717-2b9f88a63752 | Translation Approved (CM) | Ice machine, water-cooled ice-making head, H < 500 lb/day (must be on chilled loop) | Máquina de gelo, cabeçote de produção de gelo resfriado à água, H < 500 lb/dia (deve estar em ciclo resfriado) |
| 274818e048f7-6c0f-4379-b60d-c1cc687fdadd | Translation Approved (100%) | Elec | Elét. |
| 2749f9014fdd-70d5-4580-bbfc-f67cb77f5ae2 | Translation Approved (CM) | Ice | Gelo |
| 27501f4abb4e-19a4-4869-9660-df5574a482c5 | Translation Approved (0%) | 7.80 – 0.0055H kWh/100 lb ice | 7,80 – 0,0055H kWh/100 lb de gelo |
| 27516e72e98c-c2cc-42d0-8fee-c40d7c7b0ab3 | Translation Approved (100%) | na | n/d |
| 2752de06d0b5-807d-4d96-b874-a9d9c3863d9e | Translation Approved (0%) | 7.02 - 0.0049H kWh/100 lb ice | 7,02 - 0,0049H kWh/100 lb de gelo |
| 275344db3729-7cb7-4d06-909c-dc809ef2922b | Translation Approved (100%) | na | n/d |
| 2754a4397cf5-015b-45bf-b882-08f0d6fb0548 | Translation Approved (CM) | Ice machine, water-cooled once-through (open loop) | Máquina de gelo, resfriamento à água em uma passagem (ciclo aberto) |
| 27556afc39d1-7b02-43a8-af95-d38d8e7f3587 | Translation Approved (100%) | Elec | Elét. |
| 27568d2dfcc7-aa10-46d1-b269-60b6efd478f0 | Translation Approved (CM) | Ice | Gelo |
| 2757bfb5fb5a-0fd9-4806-9783-ba7f5010b4cd | Translation Approved (CM) | Banned | Proibido |
| 2758d3154305-c7a2-4f06-8795-4115bd59ea83 | Translation Approved (100%) | Banned | Proibido |
| 2759dc86af88-5d50-4eaf-9359-48454a9ff626 | Translation Approved (100%) | Banned | Proibido |
| 2760dd4fe146-3828-4e5d-92df-080e31f61513 | Translation Approved (100%) | Banned | Proibido |
| 2761287d977c-f2fd-45d2-9c89-b6054093103e | Translation Approved (CM) | Ice machine, water-cooled SCU (self-contained unit), H < 200 lb/day (must be on chilled loop) | Máquina de gelo, SCU resfriada à água (unidade autônoma), H < 200 lb/dia (deve estar em ciclo resfriado) |
| 2762164aab9c-cb40-4835-941b-a5bdf5a91776 | Translation Approved (100%) | Elec | Elét. |
| 276381c15954-3ad0-4f82-8c1e-7bf5fbba7da9 | Translation Approved (CM) | Ice | Gelo |
| 27645448899d-b42d-4d41-b8db-b6d8d75f698f | Translation Approved (0%) | 11.4 – 0.0190H kWh/100 lb ice | 11,4 – 0,0190H kWh/100 lb de gelo |
| 276585e37fe4-b50f-4be6-932c-c8ad4d309a71 | Translation Approved (100%) | na | n/d |
| 2766c0e98b45-3ab8-40a0-a871-f23a7c234760 | Translation Approved (0%) | 10.6 - 0.177H kWh/100 lb ice | 10,6 - 0,177H kWh/100 lb de gelo |
| 27679bd71f16-d8d5-43bd-925c-9c705c14865c | Translation Approved (100%) | na | n/d |
| 2768e68760a4-cb56-44a6-ae48-b2ed07722fea | Translation Approved (CM) | Ice machine, water-cooled self-contained unit, H <3307>></3307> 200 lb/day (must be on chilled loop) | Máquina de gelo, unidade autônoma resfriada à água, H <3307>></3307> 200 lb/dia (deve estar em ciclo resfriado) |
| 2769874e1f73-a1ec-4b5c-86fe-580ac88eec35 | Translation Approved (100%) | Elec | Elét. |
| 27701643397d-7150-48dc-8cb3-27b67a42739c | Translation Approved (CM) | Ice | Gelo |
| 2771900a6d88-a96d-44e2-b290-058e63c5c256 | Translation Approved (CM) | 7.6 kWh/100 lb ice | 7,6 kWh/100 lb de gelo |
| 2772d198b936-92ec-4a0a-b87a-6027ca4d04bd | Translation Approved (100%) | na | n/d |
| 27739e6d3671-44c8-46f9-911f-721b42978c63 | Translation Approved (CM) | 7.07 kWh/100 lb ice | 7,07 kWh/100 lb de gelo |
| 2774684e1f0a-2f98-4052-bc89-1be7e5575195 | Translation Approved (100%) | na | n/d |
| 27758d8664bb-ec4e-46a3-9ea0-329c528d4d71 | Translation Approved (100%) | Chest freezer, solid or glass door | Freezer horizontal, porta sólida ou de vidro |
| 27763267c200-8688-4d45-a36d-d94839b5ba4a | Translation Approved (100%) | Elec | Elét. |
| 277793a7a1cd-c11f-40be-bed8-fb36f414d444 | Translation Approved (0%) | Refrig | Refrig. |
| 27787dff24aa-a056-4d68-8d1f-00a32fe1bfb2 | Translation Approved (0%) | 0.45V + 0.943 kWh/day | 0,45 V + 0,943 kWh/dia |
| 2779a0c926b4-afb1-48d7-8341-427fd9bc86fa | Translation Approved (100%) | na | n/d |
| 27804d19c3d1-8e59-4e27-89c8-96b52d642c25 | Translation Approved (0%) | ≤ 0.270V + 0.130 kWh/day | ≤ 0,270 V + 0,130 kWh/dia |
| 2781ab7d3e38-0c4c-464c-a550-95adfb0d0b19 | Translation Approved (100%) | na | n/d |
| 2782ddbb0f40-bfda-4348-83b5-ff224c6cba26 | Translation Approved (100%) | Chest refrigerator, solid or glass door | Refrigerador horizontal, porta sólida ou de vidro |
| 278330e767da-b435-415d-b3e7-c27b9db19f65 | Translation Approved (100%) | Elec | Elét. |
| 27841d817948-99ef-4edf-8d3f-9018c097e77c | Translation Approved (CM) | Refrig | Refrig. |
| 278544a547c6-83af-48d3-b253-139961c7b0c5 | Translation Approved (CM) | 0.1V + 2.04 kWh/day | 0,1 V + 2,04 kWh/dia |
| 278646b14180-f628-4301-ba12-d2b7b272dab5 | Translation Approved (100%) | na | n/d |
| 2787d082100f-6040-4845-af16-b8b3d49d2a27 | Translation Approved (CM) | ≤ 0.125V + 0.475 kWh/day | ≤ 0,125 V + 0,475 kWh/dia |
| 2788912c4d99-9221-4ed2-b9e3-72c55f24e228 | Translation Approved (100%) | na | n/d |
| 278981876191-444b-4582-ba3c-2a69df9e17bd | Translation Approved (100%) | Glass-door reach-in freezer, 0 < V < 15 ft³ | Freezer vertical com porta de vidro, 0 < V < 15 ft³ |
| 2790452c6da9-4145-4501-abb2-248eb0b171b2 | Translation Approved (100%) | Elec | Elét. |
| 27910b37a5dc-2d98-4961-a642-64ba9160570b | Translation Approved (CM) | Refrig | Refrig. |
| 2792d56716f9-ae3a-4bdb-9e56-7cd62a6ef114 | Translation Approved (CM) | 0.75V + 4.10 kWh/day | 0,75 V + 4,10 kWh/dia |
| 2793a0d08c29-4513-40ec-b0f6-eb90310d7c77 | Translation Approved (100%) | na | n/d |
| 2794df764ebf-cc96-4dea-9170-a368e3c11796 | Translation Approved (CM) | ≤ 0.607V + 0.893 kWh/day | ≤ 0,607 V + 0,893 kWh/dia |
| 2795982f0c1d-34d7-418b-804f-1cb16c1c8ea1 | Translation Approved (100%) | na | n/d |
| 279672df8cb2-8938-4fd2-9520-9ee76f572533 | Translation Approved (100%) | Glass-door reach-in freezer, 15 ≤ V < 30 ft³ | Freezer vertical com porta de vidro, 15 ≤ V < 30 ft³ |
| 2797e8c1ff8c-83fb-40d7-9f63-d0a9d9791c2e | Translation Approved (CM) | Elec | Elét. |
| 279899d317b3-2fdc-4821-a34f-2b758c3cc456 | Translation Approved (CM) | Refrig | Refrig. |
| 2799cca2d30b-a090-44e0-bab4-7091899bd042 | Translation Approved (100%) | .75V + 4.10 kWh/day | 0,75V + 4,10 kWh/dia |
| 2800b1d767d8-66b8-4e99-9331-16bb2379fa2e | Translation Approved (100%) | na | n/d |
| 2801f8b00312-e830-4cc3-af75-561971b4760f | Translation Approved (0%) | ≤ 0.733V – 1.00 kWh/day | ≤ 0,733 V – 1,00 kWh/dia |
| 2802ae32f5b2-c2d2-4c93-9642-7ad6a4b266c9 | Translation Approved (100%) | na | n/d |
| 28033273b4a7-125d-48f2-904a-78ae1b4a1290 | Translation Approved (100%) | Glass-door reach-in freezer, 30 ≤ V < 50 ft³ | Freezer vertical com porta de vidro, 30 ≤ V < 50 ft³ |
| 28047892896a-83f2-41aa-a9cc-811ad2a2d483 | Translation Approved (CM) | Elec | Elét. |
| 2805aaa230c1-cca3-4481-a91a-c7284bb37d2c | Translation Approved (CM) | Refrig | Refrig. |
| 28062c4aad39-bb96-466c-856b-6780218ad3cd | Translation Approved (100%) | .75V + 4.10 kWh/day | 0,75V + 4,10 kWh/dia |
| 2807356c86c4-8fec-4af7-a35a-5c2602e822a8 | Translation Approved (100%) | na | n/d |
| 2808d4e63df7-16bd-4cff-8cf0-da7991f0a07a | Translation Approved (CM) | ≤ 0.250V + 13.50 kWh/day | ≤ 0,250 V + 13,50 kWh/dia |
| 28091875225d-8b3f-48b6-afee-ed6df4cd9a96 | Translation Approved (100%) | na | n/d |
| 281076ef93fd-4bf5-4a86-bb4c-a6f37056485c | Translation Approved (100%) | Glass-door reach-in freezer, 50 ≤ V ft³ | Freezer vertical com porta de vidro, 50 ≤ V ft³ |
| 2811f068e15b-7d4c-4881-b2c5-9aadc5fa3f3e | Translation Approved (100%) | Elec | Elét. |
| 2812910bdacf-5f22-423e-9c87-bb7407f58ceb | Translation Approved (CM) | Refrig | Refrig. |
| 2813b5b11382-9df2-4161-a16f-bacb67e31ab2 | Translation Approved (CM) | 0.75V + 4.10 kWh/day | 0,75V + 4,10 kWh/dia |
| 2814fc10220c-6970-469e-95d8-ab686ecf5b5d | Translation Approved (100%) | na | n/d |
| 28157fe13bde-c9e0-4d39-a294-7fbc60fd1d12 | Translation Approved (CM) | ≤ 0.450V + 3.50 kWh/day | ≤ 0,450 V + 3,50 kWh/dia |
| 2816248d3037-a24a-4719-bcc6-9d5858ceaf70 | Translation Approved (100%) | na | n/d |
| 281739120ba2-bcf3-4932-bfea-8a771dfe552c | Translation Approved (100%) | Glass-door reach-in refrigerator, 0 < V < 15 ft³ | Refrigerador vertical com porta de vidro, 0 < V < 15 ft³ |
| 28183b8d2a95-6a5f-4b21-bac1-6d119c6150c4 | Translation Approved (100%) | Elec | Elét. |
| 2819866f7c29-c9ac-4c89-9291-3b1ed101173c | Translation Approved (CM) | Refrig | Refrig. |
| 2820e6c88174-ffb5-4a10-b537-c9ac418485d0 | Translation Approved (CM) | 0.12V + 3.34 kWh/day | 0,12 V + 3,34 kWh/dia |
| 2821573e43ac-7a0b-4629-90ae-be65380d781d | Translation Approved (100%) | na | n/d |
| 28227ac18540-3d86-4ecf-897e-ec0aaba6220c | Translation Approved (CM) | ≤ 0.118V + 1.382 kWh/day | ≤ 0,118 V + 1,382 kWh/dia |
| 2823161f3c42-d8fb-42d3-ba39-21663b6f7fe4 | Translation Approved (100%) | na | n/d |
| 282475d1b827-980b-4ddb-b932-82ca7f5280cf | Translation Approved (100%) | Glass-door reach-in refrigerator, 15 ≤ V < 30 ft³ | Refrigerador vertical com porta de vidro, 15 ≤ V < 30 ft³ |
| 282599c18f1a-8a77-4bd5-b800-24d2ab672393 | Translation Approved (CM) | Elec | Elét. |
| 28260c88d3ee-033b-4b0d-ac12-312395bcc883 | Translation Approved (CM) | Refrig | Refrig. |
| 28276d3ea87f-ad57-4448-95c1-d006bd19ebc2 | Translation Approved (CM) | 0.12V + 3.34 kWh/day | 0,12 V + 3,34 kWh/dia |
| 282808f3d4bc-0e1d-414b-8930-8758ba68f4c3 | Translation Approved (100%) | na | n/d |
| 2829496def71-8695-4a18-a1b9-e4d8f4fddb18 | Translation Approved (CM) | ≤ 0.140V + 1.050 kWh/day | ≤ 0,140 V + 1,050 kWh/dia |
| 28301d7f539f-fef5-4913-ac9b-b1ae9b761678 | Translation Approved (100%) | na | n/d |
| 2831a3de9088-90c7-45e2-be8c-fef532ae25b0 | Translation Approved (100%) | Glass-door reach-in refrigerator, 30 ≤ V < 50 ft³ | Refrigerador vertical com porta de vidro, 30 ≤ V < 50 ft³ |
| 28327183cf5d-eeac-4664-a6a2-8008dff37513 | Translation Approved (CM) | Elec | Elét. |
| 2833c62962e5-a40c-42d8-8cb7-fc029c3666b2 | Translation Approved (CM) | Refrig | Refrig. |
| 2834cea5d06b-7b3f-402d-993b-19762cff5919 | Translation Approved (CM) | 0.12V + 3.34 kWh/day | 0,12V + 3,34 kWh/dia |
| 2835b22fbc74-cdd9-40cf-9fc2-1cf47a6a38a8 | Translation Approved (100%) | na | n/d |
| 2836734960bb-62ad-46ec-ac9c-3f70923743aa | Translation Approved (CM) | ≤ 0.088V + 2.625 kWh/day | ≤ 0,088V + 2,625 kWh/dia |
| 2837da0ef230-ca10-4b80-b280-796cf725596d | Translation Approved (100%) | na | n/d |
| 2838f72eaebb-21e1-43c6-bbfc-73a2ae5d3e51 | Translation Approved (100%) | Glass-door reach-in refrigerator, 50 ≤ V ft³ | Refrigerador vertical com porta de vidro, 50 ≤ V ft³ |
| 2839ae79e8a5-a392-4d9d-9adc-c16f025b1741 | Translation Approved (100%) | Elec | Elét. |
| 28405a90a566-3e71-40cb-ab07-8a270c41f7db | Translation Approved (CM) | Refrig | Refrig. |
| 2841140a6f4b-070f-4239-ad39-29737766bc3b | Translation Approved (CM) | 0.12V + 3.34 kWh/day | 0,12 V + 3,34 kWh/dia |
| 284295557b8b-6111-4438-be13-2f87dfee1562 | Translation Approved (100%) | na | n/d |
| 2843d1b97024-4497-4595-865d-866453c5a9b1 | Translation Approved (CM) | ≤ 0.110V + 1.500 kWh/day | ≤ 0,110 V + 1,500 kWh/dia |
| 28443f43ca87-e988-4f74-9f11-a6903a0a2626 | Translation Approved (100%) | na | n/d |
| 284544f3cd2b-a70f-4a61-94c5-5595d7ae7cfa | Translation Approved (100%) | Solid-door reach-in freezer, 0 < V < 15 ft³ | Freezer vertical com porta sólida, 0 < V < 15 ft³ |
| 2846dcfdc6d8-3233-4790-a6c9-73ceda890ec6 | Translation Approved (100%) | Elec | Elét. |
| 28473e699a05-0629-4822-ac92-3b575496f6bf | Translation Approved (CM) | Refrig | Refrig. |
| 28482c98ee67-4657-4cb1-9707-64d8e338c60a | Translation Approved (CM) | 0.4V + 1.38 kWh/day | 0,4 V + 1,38 kWh/dia |
| 28492efd1a82-9216-450b-bfe2-c73ac4ab8a71 | Translation Approved (100%) | na | n/d |
| 2850c2029448-98cf-4118-a240-99d6b402f959 | Translation Approved (CM) | ≤ 0.250V + 1.25 kWh/day | ≤ 0,250 V + 1,25 kWh/dia |
| 2851988b1a40-d749-4af5-a2a8-dd6feacef027 | Translation Approved (100%) | na | n/d |
| 28528ce7be9c-bf36-4f95-8173-86496f1d3f63 | Translation Approved (100%) | Solid-door reach-in freezer, 15 ≤ V < 30 ft³ | Freezer vertical com porta sólida, 15 ≤ V < 30 ft³ |
| 285306278ffd-e874-4dca-8927-c1289c17f780 | Translation Approved (CM) | Elec | Elét. |
| 28544ace5cfc-8962-4337-a5e0-5890a627c6af | Translation Approved (CM) | Refrig | Refrig. |
| 2855403b6e71-6369-4ed1-9435-96cde037c128 | Translation Approved (CM) | 0.4V + 1.38 kWh/day | 0,4 V + 1,38 kWh/dia |
| 2856eb5e9079-cf17-4b9b-9dcf-1cc546f4ca8b | Translation Approved (100%) | na | n/d |
| 2857a903d689-fad5-45d9-97a9-ac0ec08defc5 | Translation Approved (CM) | ≤ 0.400V – 1.000 kWh/day | ≤ 0,400 V – 1,000 kWh/dia |
| 285833b56e19-0c65-48fa-ba68-c4ea2d16fa3b | Translation Approved (100%) | na | n/d |
| 28590c8a266a-52eb-46e6-9715-87f96e179c4f | Translation Approved (100%) | Solid-door reach-in freezer, 30 ≤ V < 50 ft³ | Freezer vertical com porta sólida, 30 ≤ V < 50 ft³ |
| 2860a8aad12d-42b8-44de-aaca-469b38844c31 | Translation Approved (CM) | Elec | Elét. |
| 2861a7579b5a-7963-44eb-b769-034f9121b8fb | Translation Approved (CM) | Refrig | Refrig. |
| 2862ac1d481b-74cc-4ef2-9bef-0eb72cb8b4fd | Translation Approved (CM) | 0.4V + 1.38 kWh/day | 0,4 V + 1,38 kWh/dia |
| 2863bf56adfc-74a9-4e1a-974b-b00502ef8ee4 | Translation Approved (100%) | na | n/d |
| 28646f3e0921-e4ad-4017-95ff-f00e3bee2739 | Translation Approved (CM) | ≤ 0.163V + 6.125 kWh/day | ≤ 0,163 V + 6,125 kWh/dia |
| 286587fff26f-2f8a-4de0-8dba-7a2bd43a2177 | Translation Approved (100%) | na | n/d |
| 28667ad3fff2-4bf7-4c00-90a6-209004de8b95 | Translation Approved (100%) | Solid-door reach-in freezer, 50 ≤ V ft³ | Freezer vertical com porta sólida, 50 ≤ V ft³ |
| 286788d9dca7-3f45-4694-98fb-2b698aea7a69 | Translation Approved (100%) | Elec | Elét. |
| 28689d1a8c5a-14fa-4ccf-94d2-c6d4d56ba875 | Translation Approved (CM) | Refrig | Refrig. |
| 28697aac0fc5-1106-4a4e-a92f-4d18782bf712 | Translation Approved (CM) | 0.4V + 1.38 kWh/day | 0,4 V + 1,38 kWh/dia |
| 2870aef510e0-3f5f-461a-831d-c004846d190f | Translation Approved (100%) | na | n/d |
| 2871ef2e45d1-9f45-45f9-8abd-334101c28380 | Translation Approved (CM) | ≤ 0.158V + 6.333 kWh/day | ≤ 0,158 V + 6,333 kWh/dia |
| 2872946cfdc0-bc99-4603-84f6-c72f43a1b59e | Translation Approved (100%) | na | n/d |
| 287306ab69e1-ff88-4060-94b1-6392b6b04381 | Translation Approved (100%) | Solid-door reach-in refrigerator, 0 < V < 15 ft³ | Refrigerador vertical com porta sólida, 0 < V < 15 ft³ |
| 2874a16ac8db-3b27-4939-a859-6e7ee7ed1f10 | Translation Approved (100%) | Elec | Elét. |
| 2875898fec98-9b3b-47d4-8518-3d4e65ec3bcb | Translation Approved (CM) | Refrig | Refrig. |
| 2876df734f18-dcf9-4ac2-8140-4c5b571f8051 | Translation Approved (CM) | 0.1V + 2.04 kWh/day | 0,1 V + 2,04 kWh/dia |
| 2877d0643aec-db9f-49ec-bab6-95527ec4ad42 | Translation Approved (100%) | na | n/d |
| 28786340befc-a38a-4cf6-afdb-7ad71e68d3a7 | Translation Approved (CM) | ≤ 0.089V + 1.411 kWh/day | ≤ 0,089 V + 1,411 kWh/dia |
| 28792323f9d4-6056-4f7c-a5be-5c848e1374de | Translation Approved (100%) | na | n/d |
| 28800fe87da1-5148-478e-b950-4a30011be7ab | Translation Approved (100%) | Solid-door reach-in refrigerator, 15 ≤ V < 30 ft³ | Refrigerador vertical com porta sólida, 15 ≤ V < 30 ft³ |
| 2881df58fece-3237-4b7e-ac2c-adc97cd999ba | Translation Approved (CM) | Elec | Elét. |
| 2882a950e043-264f-4cc2-bca6-219a716f4f95 | Translation Approved (CM) | Refrig | Refrig. |
| 2883e7f1c478-a4f8-4e53-8966-61a00cf7da4d | Translation Approved (CM) | 0.1V + 2.04 kWh/day | 0,1 V + 2,04 kWh/dia |
| 288419c606ea-3460-4ff3-bf1d-452b102db677 | Translation Approved (100%) | na | n/d |
| 28850b17bfee-58aa-46af-82ab-590a5bc96aeb | Translation Approved (CM) | ≤ 0.037V + 2.200 kWh/day | ≤ 0,037 V + 2,200 kWh/dia |
| 2886319e142d-795c-43c2-a58f-0a55574a472b | Translation Approved (100%) | na | n/d |
| 2887a276aa38-2ea1-471c-81a9-135a80ed7b8f | Translation Approved (100%) | Solid-door reach-in refrigerator, 30 ≤ V < 50 ft³ | Refrigerador vertical com porta sólida, 30 ≤ V < 50 ft³ |
| 2888f824fee3-97dd-407d-b3b4-fccc327f355d | Translation Approved (CM) | Elec | Elét. |
| 2889f2ff2656-cd14-4318-bbba-0fed0dc17269 | Translation Approved (CM) | Refrig | Refrig. |
| 2890685c8410-db02-4d33-8dc5-04d17b108864 | Translation Approved (CM) | 0.1V + 2.04 kWh/day | 0,1 V + 2,04 kWh/dia |
| 28917e965d60-9f54-4eb3-9359-1c83afade0b4 | Translation Approved (100%) | na | n/d |
| 289229bc601d-6485-4d04-9a12-8951a9bd5853 | Translation Approved (CM) | ≤ 0.056V + 1.635 kWh/day | ≤ 0,056 V + 1,635 kWh/dia |
| 289343ca85f5-001c-4ed9-b540-5a44cc270a31 | Translation Approved (100%) | na | n/d |
| 2894deea5575-09bd-4f87-9ece-b6b7ea07e1e8 | Translation Approved (100%) | Solid-door reach-in refrigerator, 50 ≤ V ft³ | Refrigerador vertical com porta sólida, 50 ≤ V ft³ |
| 28956db72193-b1c2-40ba-890d-ea775dc72e1e | Translation Approved (100%) | Elec | Elét. |
| 289622cff7c8-3db1-4cf8-a8c5-dda593a31052 | Translation Approved (CM) | Refrig | Refrig. |
| 289776285188-e0c6-42c8-a713-2b646a584b6c | Translation Approved (CM) | 0.1V + 2.04 kWh/day | 0,1 V + 2,04 kWh/dia |
| 2898ce92fe7b-5ff6-49c4-a2c5-0eda129c7151 | Translation Approved (100%) | na | n/d |
| 2899daa1eb69-d291-48bf-ab5f-67f2bf7245f7 | Translation Approved (CM) | ≤ 0.060V + 1.416 kWh/day | ≤ 0,060 V + 1,416 kWh/dia |
| 2900ba045b0b-cc84-4002-968f-cce82f6191be | Translation Approved (100%) | na | n/d |
| 2901b36ed66b-89ef-4daf-8bce-9e0d4f05cf86 | Translation Approved (100%) | Clothes washer | Máquina de lavar roupa |
| 2902d8a1c7c8-7941-47fa-a633-db14be10cb79 | Translation Approved (100%) | Gas | Gás |
| 29032a2993b5-843d-4b21-ba3f-b14423cd3d57 | Translation Approved (100%) | Sanitation | Saneamento |
| 290405562b05-7a2b-47d3-aeb5-776b03caeb30 | Translation Approved (100%) | 1.72 MEF | 1,72 MEF |
| 29056b119d24-55c4-4220-816b-5c8120b3c155 | Translation Approved (100%) | na | n/d |
| 2906d0c29d04-f1ff-42a9-a300-fe416a5f67e1 | Translation Approved (100%) | 2.00 MEF | 2,00 MEF |
| 290722e90d8e-3831-4105-aa2b-655aa9029355 | Translation Approved (100%) | na | n/d |
| 290892f94c50-f10a-4ce3-a540-dc5317d06e66 | Translation Approved (100%) | Door-type dish machine, high temp | Lava-louças tipo porta, alta temp. |
| 290944bbdd3c-e080-4955-aa13-d5fd45899cf9 | Translation Approved (100%) | Elec | Elét. |
| 2910cbb0f53d-2b83-48e9-8af8-0b13d446a05b | Translation Approved (100%) | Sanitation | Saneamento |
| 29119ddec370-4079-44dd-a90d-e9ec96a76233 | Translation Approved (100%) | na | n/d |
| 2912e5ff9c12-f4b2-4600-8c1d-84e01fbaa278 | Translation Approved (100%) | 1.0 kW | 1,0 kW |
| 291317dd0ccf-94aa-4ec8-8274-d2ab8da8855d | Translation Approved (100%) | na | n/d |
| 29149136f949-7451-4fd2-88db-209b57b872d6 | Translation Approved (CM) | 0.70 kW | 0,70 kW |
| 29155f9cc93b-3a83-4c98-8ea4-2680580a795c | Translation Approved (100%) | Door-type dish machine, low temp | Lava-louças tipo porta, baixa temp. |
| 29169f8ae7c3-1a87-4c21-9322-7f9714981d01 | Translation Approved (100%) | Elec | Elét. |
| 2917a06a5f0e-8aa6-4323-a8f2-f723066969d2 | Translation Approved (100%) | Sanitation | Saneamento |
| 29180ddaef3d-34f7-4636-8999-fef5b32396f1 | Translation Approved (100%) | na | n/d |
| 2919fa207a80-efb5-47eb-ae14-ac0b78c509de | Translation Approved (CM) | 0.6 kW | 0,6 kW |
| 29203f0e7a10-4678-4ec9-b669-335da22d469b | Translation Approved (100%) | na | n/d |
| 2921f18b2d72-c518-4aca-8ec7-9f45ed6b1365 | Translation Approved (CM) | 0.6 kW | 0,6 kW |
| 292236d70353-d3ef-4600-b6e3-1f6098e4482d | Translation Approved (100%) | Multitank rack conveyor dish machine, high temp | Lava-louças industrial multitanque com transportador, alta temp. |
| 29232c459393-d051-4da1-a13c-be38e93dcbba | Translation Approved (100%) | Elec | Elét. |
| 29241bf519c5-df52-4aa8-ada4-d50335dc7768 | Translation Approved (100%) | Sanitation | Saneamento |
| 2925cda26d1d-e8a6-4310-8ce1-ee3ab3b13741 | Translation Approved (100%) | na | n/d |
| 29267b8f4505-9f0c-4e0c-ac53-c3d043b25feb | Translation Approved (CM) | 2.6 kW | 2,6 kW |
| 292708910669-94d5-427c-b389-c86d3760f9fb | Translation Approved (100%) | na | n/d |
| 2928aa65b6dc-a07a-45a4-8ce8-a0bc3d180c52 | Translation Approved (CM) | 2.25 kW | 2,25 kW |
| 2929a29973e7-1a9d-4b42-b919-2d7fdde2a5e1 | Translation Approved (100%) | Multitank rack conveyor dish machine, low temp | Lava-louças industrial multitanque com transportador, baixa temp. |
| 2930323193a2-4d93-4b73-87ad-311cbaf74c7d | Translation Approved (100%) | Elec | Elét. |
| 2931fb52914a-f8b1-437a-bc65-931906b80b86 | Translation Approved (100%) | Sanitation | Saneamento |
| 29328f1f095b-5500-4229-ac99-f23f4f6a8c8b | Translation Approved (100%) | na | n/d |
| 29331472568b-fe57-43ae-b144-0ae23796b03a | Translation Approved (CM) | 2.0 kW | 2,0 kW |
| 293415d6898b-f036-4cc6-979e-ad030f576fad | Translation Approved (100%) | na | n/d |
| 293571680237-3011-4ed1-a2d1-fba373c70b5f | Translation Approved (CM) | 2.0 kW | 2,0 kW |
| 2936088066be-434a-4081-a846-53fc9b5e0fff | Translation Approved (100%) | Single-tank rack conveyor dish machine, high temp | Lava-louças industrial tanque único com transportador, alta temp. |
| 2937b7902272-154b-436b-8707-fecc38380f67 | Translation Approved (100%) | Elec | Elét. |
| 2938e35a9473-6eb0-4fbb-903e-e5c285d507e1 | Translation Approved (100%) | Sanitation | Saneamento |
| 29394ea0154a-3f0c-44b0-bdcd-0383e236ed1f | Translation Approved (100%) | na | n/d |
| 2940f9a31db1-64bf-47e5-912b-46fa4749cf97 | Translation Approved (CM) | 2.0 kW | 2,0 kW |
| 2941b0b1323a-e0dc-4244-aca1-0638185ce025 | Translation Approved (100%) | na | n/d |
| 29428edaf26d-bc80-4aa0-96b5-060b6b697d1d | Translation Approved (CM) | 1.5 kW | 1,5 kW |
| 294364de6157-d4c6-4deb-bbd6-f508db3517d0 | Translation Approved (100%) | Single-tank rack conveyor dish machine, low temp | Lava-louças industrial tanque único com transportador, baixa temp. |
| 2944306847c7-27f3-4d6b-95dc-81c87fac18c9 | Translation Approved (100%) | Elec | Elét. |
| 29453cfc31e1-11b3-4ee8-81d3-26c68dd6b795 | Translation Approved (100%) | Sanitation | Saneamento |
| 29462ad2ddd3-0e15-49e5-8783-c3857e19bbef | Translation Approved (100%) | na | n/d |
| 294771144c1f-ff8f-4525-bbf4-7bcdbec273e1 | Translation Approved (CM) | 1.6 kW | 1,6 kW |
| 29489402ba13-388c-4533-b2ab-107858a7725f | Translation Approved (100%) | na | n/d |
| 29498ea46bac-c12d-4e7f-ab34-6283d601888b | Translation Approved (CM) | 1.5 kW | 1,5 kW |
| 2950f5d9ab7a-cf11-4173-a002-10fa6dea80cd | Translation Approved (100%) | Undercounter dish machine, high temp | Lava-louças sob bancada, alta temp. |
| 295152154848-9112-486e-940c-bd8fffd05e68 | Translation Approved (100%) | Elec | Elét. |
| 2952849cd4d4-39bb-4a55-b847-28d1a4300f3a | Translation Approved (100%) | Sanitation | Saneamento |
| 2953bcf5bcc5-082e-4576-9f6b-3f075e7b631b | Translation Approved (100%) | na | n/d |
| 295400b0961b-b117-4e27-b5ab-448b14455264 | Translation Approved (CM) | 0.9 kW | 0,9 kW |
| 2955a8df9f68-f236-4fd2-8691-19ed659894e1 | Translation Approved (100%) | na | n/d |
| 2956116d3aed-4ab4-453c-aae5-8cfdf413b665 | Translation Approved (CM) | 0.5 kW | 0,5 kW |
| 29578cf72803-09d3-4e8a-a28b-f2e57de2d1f9 | Translation Approved (100%) | Undercounter dish machine, low temp | Lava-louças sob bancada, baixa temp. |
| 2958720a0f66-ec31-426a-9adb-20173b915ad7 | Translation Approved (100%) | Elec | Elét. |
| 29592f931ee7-0329-4588-b0a2-fb4577a58754 | Translation Approved (100%) | Sanitation | Saneamento |
| 29604949ba55-677c-4156-b9e7-2f2b7128e22c | Translation Approved (100%) | na | n/d |
| 296161f8b4f1-73d0-404c-97ff-d5142e1bb0c8 | Translation Approved (CM) | 0.5 kW | 0,5 kW |
| 29625bae2a30-dfca-45bb-b986-35115c6dccbc | Translation Approved (100%) | na | n/d |
| 2963f4911d7a-8726-464a-9588-69180b6ed6fa | Translation Approved (CM) | 0.5 kW | 0,5 kW |
| 2964c68f9b8b-aa52-44c8-9266-bf31dced0bdd | Translation Approved (100%) | The energy efficiency, idle energy rates, and water use requirements, where applicable, are based on the following test methods: | Os requisitos de eficiência energética, taxas de energia em inatividade e uso de água, quando aplicáveis, se baseiam nos seguintes métodos de teste: |
| 296557e0e4b6-a52e-4d2e-b47c-12fadcd5433f | Translation Approved (100%) | ASTM F1275 Standard Test Method for Performance of Griddles | ASTM F1275 – Método de teste padrão de desempenho de frigideiras (Standard Test Method for Performance of Griddles) |
| 2966810fe0cb-4a73-4334-940f-8de34722118b | Translation Approved (100%) | ASTM F1361 Standard Test Method for Performance of Open Deep Fat Fryers | ASTM F1361 – Método de teste padrão de desempenho de fritadeiras de imersão (Standard Test Method for Performance of Open Deep Fat Fryers) |
| 296778258384-bb70-46ca-8f07-1944740b7554 | Translation Approved (100%) | ASTM F1484 Standard Test Methods for Performance of Steam Cookers | ASTM F1484 – Métodos de teste padrão de desempenho de panelas a vapor (Standard Test Methods for Performance of Steam Cookers) |
| 2968f5353519-e26b-4a23-81cc-ee9dbd3da581 | Translation Approved (100%) | ASTM F1496 Standard Test Method for Performance of Convection Ovens | ASTM F1496 – Método de teste padrão de desempenho de fornos de convecção (Standard Test Method for Performance of Convection Ovens) |
| 29693b2c37ce-a244-418c-8b45-4d1cf4f00ee0 | Translation Approved (100%) | ASTM F1521 Standard Test Methods for Performance of Range Tops | ASTM F1521 – Métodos de teste padrão de desempenho de "cooktops" (Standard Test Methods for Performance of Range Tops) |
| 29703d968e33-06eb-4af3-bdc2-8854e38dee0c | Translation Approved (100%) | ASTM F1605 Standard Test Method for Performance of Double-Sided Griddles | ASTM F1605 – Método de teste padrão de desempenho de frigideiras de dois lados (Standard Test Method for Performance of Double-Sided Griddles) |
| 297117ccc23f-7b2f-4cba-b077-86de949561eb | Translation Approved (100%) | ASTM F1639 Standard Test Method for Performance of Combination Ovens | ASTM F1639 – Método de teste padrão de desempenho de fornos de combinação (Standard Test Method for Performance of Combination Ovens) |
| 2972454d088a-5bd8-4643-873e-c2b382269cdb | Translation Approved (100%) | ASTM F1695 Standard Test Method for Performance of Underfired Broilers | ASTM F1695 – Método de teste padrão de desempenho de grelhas aquecidas por baixo (Standard Test Method for Performance of Underfired Broilers) |
| 2973a0012a78-612b-4199-8a7a-edaecddc389a | Translation Approved (100%) | ASTM F1696 Standard Test Method for Energy Performance of Single-Rack Hot Water Sanitizing, ASTM Door-Type Commercial Dishwashing Machines | ASTM F1696 – Método de teste padrão de desempenho energético de lava-louças comerciais tipo porta ASTM, limpeza com água quente e prateleira única (Standard Test Method for Energy Performance of Single-Rack Hot Water Sanitizing, ASTM Door-Type Commercial Dishwashing Machines) |
| 2974b143f722-b6c6-4058-b558-223bcc3ef228 | Translation Approved (100%) | ASTM F1704 Standard Test Method for Capture and Containment Performance of Commercial Kitchen Exhaust Ventilation Systems | ASTM F1704 – Método de teste padrão de desempenho de captura e contenção de sistemas de ventilação de exaustão de cozinhas (Standard Test Method for Capture and Containment Performance of Commercial Kitchen Exhaust Ventilation Systems) |
| 29754ca1129b-fda7-4cdc-ac24-be44694bbbd9 | Translation Approved (100%) | ASTM F1817 Standard Test Method for Performance of Conveyor Ovens | ASTM F1817 – Método de teste padrão de desempenho de fornos com transportador (Standard Test Method for Performance of Conveyor Ovens) |
| 29764d5089aa-ff0a-469e-bace-965af5d4d12c | Translation Approved (100%) | ASTM F1920 Standard Test Method for Energy Performance of Rack Conveyor, Hot Water Sanitizing, Commercial Dishwashing Machines | ASTM F1920 – Método de teste padrão de desempenho energético de lava-louças comerciais, limpeza com água quente, com transportador e prateleiras (Standard Test Method for Energy Performance of Rack Conveyor, Hot Water Sanitizing, Commercial Dishwashing Machines) |
| 2977353f4a25-12eb-4241-833e-f85deb167b6e | Translation Approved (100%) | ASTM F2093 Standard Test Method for Performance of Rack Ovens | ASTM F2093 – Método de teste padrão para o desempenho de fornos rotativos (Standard Test Method for Performance of Rack Ovens) |
| 29783710fdb0-61a5-4db6-b41a-82d43f4d53aa | Translation Approved (100%) | ASTM F2140 Standard Test Method for Performance of Hot Food Holding Cabinets | ASTM F2140 – Método de teste padrão de desempenho de armários de armazenamento de alimentos quentes (Standard Test Method for Performance of Hot Food Holding Cabinets) |
| 297931856b93-59ea-4454-b5f4-b382ac98756e | Translation Approved (100%) | ASTM F2144 Standard Test Method for Performance of Large Open Vat Fryers | ASTM F2144 – Método de teste padrão de desempenho de fritadeiras grandes de tanque aberto (Standard Test Method for Performance of Large Open Vat Fryers) |
| 29800289ae89-92ae-48a2-b51a-f25c27dbd428 | Translation Approved (100%) | ASTM F2324 Standard Test Method for Prerinse Spray Valves | ASTM F2324 – Método de teste padrão de válvulas de pulverização pré-lavagem (Standard Test Method for Prerinse Spray Valves) |
| 29812945098a-05f5-42e0-9924-650955eb69a2 | Translation Approved (100%) | ASTM F2380 Standard Test Method for Performance of Conveyor Toasters | ASTM F2380 – Método de teste padrão de desempenho de torradeiras com transportador (Standard Test Method for Performance of Conveyor Toasters) |
| 2982ca4a4a02-41fe-4eb8-9065-b15c7ef8f5f6 | Translation Approved (100%) | ARI 810-2007: Performance Rating of Automatic Commercial Ice Makers | ARI 810-2007: Avaliação de desempenho de máquinas de gelo comerciais automáticas (Performance Rating of Automatic Commercial Ice Makers) |
| 2983f373e09f-6922-4a0c-9f01-a15c9cf59903 | Translation Approved (100%) | ANSI/ASHRAE Standard 72–2005: Method of Testing Commercial Refrigerators and Freezers with temperature setpoints at 38°F for medium-temp refrigerators, 0°F for low-temp freezers, and -15°F for ice cream freezers | Norma ANSI/ASHRAE 72–2005: Método de teste de refrigeradores e freezers comerciais com pontos de ajuste de temperatura a 38°F (3°C) para refrigeradores de temperatura média, 0°F (-18°C) para freezers de baixa temperatura e -15°F (-26°C) para freezers para sorvete (Method of Testing Commercial Refrigerators and Freezers with temperature setpoints at 38°F for medium-temp refrigerators, 0°F for low-temp freezers, and -15°F for ice cream freezers). |
| 29848ee12a29-dc6c-459c-be84-4ab33dfc1c30 | Translation Approved (100%) | Table 1b. | Tabela 1b. |
| 29858ee12a29-dc6c-459c-be84-4ab33dfc1c30 | Translation Approved (100%) | Commercial Kitchen Appliance Prescriptive Measures and Baseline for Energy Cost Budget (SI units) | Baseline e medidas prescritivas para utensílios de cozinhas comerciais para orçamento de custos de energia (sistema métrico) |
| 29864c841d9f-2b27-4df2-92db-60e9fa12b3b7 | Translation Approved (100%) | Baseline energy usage for energy modeling path | Baseline do uso de energia do caminho de modelagem de energia |
| 298721c552e6-3d58-42cb-9aad-8c203b62b3a8 | Translation Approved (100%) | Levels for prescriptive path | Níveis do caminho prescritivo |
| 29884a91bf82-b4bf-43b0-8308-b6ad8e299eec | Translation Approved (100%) | Appliance type | Tipo de utensílio |
| 2989cc166746-14af-41a4-b94b-55ad78c36c52 | Translation Approved (100%) | Fuel | Combustível |
| 2990dfd5670c-2e65-4585-8fb9-300af56dbc83 | Translation Approved (100%) | Function | Função |
| 2991f104c436-fa88-4d28-9d84-a1f76d944bd1 | Translation Approved (100%) | Baseline efficiency | Eficiência da baseline |
| 2992a163f7b7-df30-4e51-a248-82a7cb431363 | Translation Approved (100%) | Baseline idle rate | Taxa de inatividade da baseline |
| 299355d4be9b-7740-4c5a-ac7e-944013230ce8 | Translation Approved (100%) | Prescriptive efficiency | Eficiência prescritiva |
| 2994c650bd83-d6c7-4d8a-8a4a-0df79fea2c1a | Translation Approved (100%) | Prescriptive idle rate | Taxa de inatividade prescritiva |
| 299519fdc5b1-b6b6-4cbc-a470-163efe3f4528 | Translation Approved (100%) | Broiler, underfired | Grelha, aquecida por baixo |
| 299653c43e72-ee60-4934-ba24-7ee33c1c0d1b | Translation Approved (100%) | Gas | Gás |
| 29970df27aaa-4fbc-4ab6-9418-1ca249805663 | Translation Approved (100%) | Cooking | Culinária |
| 29989f724c1e-88a2-48f1-a826-9817f78de1da | Translation Approved (100%) | 30% | 30% |
| 2999bb396bd4-c59e-4cb9-879b-493729044a81 | Translation Approved (100%) | 50.5 kW/m<4001>2</4001> | 50,5 kW/m<4001>2</4001> |
| 30007e273440-614e-4331-8c7b-715d32e3892a | Translation Approved (100%) | 35% | 35% |
| 3001a8139c8a-cdc8-4586-841a-61395f16c772 | Translation Approved (100%) | 37.9 kW/m<4008>2</4008> | 37,9 kW/m<4008>2</4008> |
| 30027da063e6-8e8a-4c95-8883-870685edb72c | Translation Approved (100%) | Combination oven, steam mode (P = pan capacity) | Forno de combinação, modo vapor (P = capacidade da panela) |
| 300352b8994a-84e5-4a70-8a38-789cfe6f47c1 | Translation Approved (100%) | Elec | Elét. |
| 30042c4bc92b-46d7-441f-ac40-4020faf7bf32 | Translation Approved (100%) | Cooking | Culinária |
| 300545aaff3f-360f-4f7f-b5dc-e763764d10ee | Translation Approved (100%) | 40% steam mode | 40% modo vapor |
| 300617505f58-74c9-4893-8ea1-8593b8c91ffb | Translation Approved (100%) | 0.37P+4.5 kW | 0,37P + 4,5 kW |
| 3007ddc92529-7905-48fe-bd1a-0556d8b6a3f3 | Translation Approved (100%) | 50% steam mode | 50% modo vapor |
| 30085bce6c97-6f7c-40a5-88c9-0f6cdf1bc393 | Translation Approved (CM) | 0.133P+0.6400 kW | 0,133P+0,6400 kW |
| 300952765d9e-e515-4272-9081-07c93484cb93 | Translation Approved (100%) | Combination oven, steam mode | Forno de combinação, modo vapor |
| 3010ca897965-2d29-4118-a573-c247d2176ce4 | Translation Approved (100%) | Gas | Gás |
| 3011d48471c5-68bf-4b56-9d4c-4d4fb22fa62a | Translation Approved (100%) | Cooking | Culinária |
| 30123a5b4dad-2987-4d11-b8c5-fe1375830d7b | Translation Approved (100%) | 20% steam mode | 20% modo vapor |
| 30138cd970eb-121b-442b-bdd5-4de8d8d644c9 | Translation Approved (100%) | (1 210P+ | (1.210P + |
| 3014d83f32ef-4d19-4001-9992-3ce97fa48567 | Translation Approved (100%) | 35 810)/3 412 kW | 35.810) / 3.412 kW |
| 3015af99159f-ebcf-4db4-88fc-f41199b1d987 | Translation Approved (100%) | 38% steam mode | 38% modo vapor |
| 30168c02dda5-0a38-4b78-a63e-d2b4b97c07d8 | Translation Approved (100%) | (200P+6 511)/ | (200P + 6.511)/ |
| 301795c11d6a-cda7-4e50-bb22-025790f7911f | Translation Approved (100%) | 3 412 kW | 3.412 kW |
| 30182925a19b-2976-4510-828a-f0e73d79d36f | Translation Approved (100%) | Combination oven, convection mode | Forno de combinação, modo de convecção |
| 30190ebaea5b-7a6f-4d34-81c4-2e578e3857a6 | Translation Approved (CM) | Elec | Elét. |
| 3020378e276d-4c61-4a1a-896f-9dd1727b2882 | Translation Approved (100%) | Cooking | Culinária |
| 3021474e6d2a-3e57-4eaf-98c0-42093ccfc80a | Translation Approved (CM) | 65% convection mode | 65% em modo de convecção |
| 3022ce178b62-fb92-4ab3-9d67-52b5c6a4975c | Translation Approved (100%) | 0.1P+1.5 kW | 0,1P+1,5 kW |
| 302328d610da-6394-43a0-8e4b-a2d44d483759 | Translation Approved (CM) | 70% convection mode | 70% em modo de convecção |
| 3024f34b6541-617a-4d58-8a11-4d28357b216c | Translation Approved (100%) | 0.080P+0.4989 kW | 0,080P+0,4989 kW |
| 3025ff930353-9f02-4f0b-92d2-ad5e2da2a636 | Translation Approved (100%) | Combination oven, convection mode | Forno de combinação, modo de convecção |
| 3026b1f2379b-a1b7-4c3f-bcfd-4e4d267ac6be | Translation Approved (100%) | Gas | Gás |
| 3027c8e94802-4efe-4729-911f-68b4286856b9 | Translation Approved (100%) | Cooking | Culinária |
| 3028ed9e5b48-d701-4d64-a02e-955a2caf166b | Translation Approved (CM) | 35% convection mode | 35% em modo de convecção |
| 3029279137c2-39be-4cd0-a68d-8cf0bd01f02c | Translation Approved (100%) | (322P+ | (322P + |
| 3030d5861ec0-87f7-46c3-86f0-dc61ea93f5ad | Translation Approved (100%) | 13 563)/ | 13.563)/ |
| 3031b46c5cc2-41d9-45f9-8aaa-be7ee95a5f96 | Translation Approved (100%) | 3412 kW | 3.412 kW |
| 3032967b11f4-fabc-404d-a17a-08d9ebcd4628 | Translation Approved (100%) | 44% convection mode | 44% em modo de convecção |
| 30332b0ce756-c823-4f37-8e66-9e2d419a7434 | Translation Approved (100%) | (150P+5 425)/ | (150P + 5.425)/ |
| 30340bfab7be-5318-46da-a527-9303e4914047 | Translation Approved (100%) | 3412 kW | 3.412 kW |
| 303543067e95-aaca-42f2-879c-57e7a6e6a984 | Translation Approved (CM) | Convection oven, full-size | Forno de convecção, tamanho grande |
| 30360845e63c-02b6-4265-bd1f-6fd86539d04b | Translation Approved (CM) | Elec | Elét. |
| 30374712298c-7e26-4c0f-ad73-6bb2d4c395fc | Translation Approved (100%) | Cooking | Culinária |
| 303828e077c2-5e65-4ca8-8e79-609868b66e3f | Translation Approved (100%) | 65% | 65% |
| 303916f76bf9-f570-46fd-8af9-1d6966eaecd8 | Translation Approved (CM) | 2.0 kW | 2,0 kW |
| 3040fee94627-bddd-4de6-ad73-2662b3969d77 | Translation Approved (100%) | 71% | 71% |
| 3041d1069ac1-9d52-4782-800f-40f53d8e13b2 | Translation Approved (CM) | 1.6 kW | 1,6 kW |
| 304251d39815-4216-4475-a5e9-205db0c0a8b4 | Translation Approved (CM) | Convection oven, full-size | Forno de convecção, tamanho grande |
| 3043e1fbbaeb-b2c7-42ac-be17-3e85ab7eb7f8 | Translation Approved (100%) | Gas | Gás |
| 3044297222a2-2671-4116-bb1f-c9abf7d813cc | Translation Approved (100%) | Cooking | Culinária |
| 3045ea4388c6-fa65-4b55-bfe4-769c05ada711 | Translation Approved (100%) | 30% | 30% |
| 304625d55e6e-40ef-42bf-b362-b525ef8dac1c | Translation Approved (CM) | 5.3 kW | 5,3 kW |
| 30471e156f55-9742-4b3b-a3ba-8f1260691f8d | Translation Approved (100%) | 46% | 46% |
| 3048686a1d21-5497-4d0c-ac9b-d66a81d04944 | Translation Approved (CM) | 3.5 kW | 3,5 kW |
| 30496a9a4560-2d2e-4ca9-b784-b2bf35ce09af | Translation Approved (100%) | Convection oven, half-size | Forno de convecção, tamanho médio |
| 30503eca9d0a-e666-4709-a8eb-625d15608d58 | Translation Approved (CM) | Elec | Elét. |
| 3051c2188449-ea92-44e0-b11e-bc7898357447 | Translation Approved (100%) | Cooking | Culinária |
| 3052db0b518d-8300-471c-95a9-731a1ebc4769 | Translation Approved (100%) | 65% | 65% |
| 30535c9e5078-68fb-41aa-82d1-afd835d2e69d | Translation Approved (CM) | 1.5 kW | 1,5 kW |
| 30545588399f-354f-413c-84ce-51a308f9710f | Translation Approved (100%) | 71% | 71% |
| 3055f5014b0c-0b69-45c4-9d86-9ad54fd58b8d | Translation Approved (CM) | 1.0 kW | 1,0 kW |
| 305688cedd57-4763-48a6-954e-16684fd3940c | Translation Approved (100%) | Conveyor oven, > 63.5 cm belt | Forno com transportador, correia > 63,5 cm |
| 3057487c82c6-d99b-4529-96c5-eb349403e0bf | Translation Approved (100%) | Gas | Gás |
| 30580105e2fd-5754-4f90-8a57-cf5b7ee2e4a7 | Translation Approved (100%) | Cooking | Culinária |
| 3059cbe050d3-d8cf-4d7c-9c2c-8f6be8e06b9a | Translation Approved (100%) | 20% | 20% |
| 3060e568c1e4-c2aa-4aec-bdee-cefa38d89866 | Translation Approved (CM) | 20.5 kW | 20,5 kW |
| 306196718206-f626-47fb-93a7-2f8f148d1288 | Translation Approved (100%) | 42% | 42% |
| 30625a677950-1d4a-432b-8205-d547f71a553e | Translation Approved (CM) | 16.7 kW | 16,7 kW |
| 3063017aa47b-30fe-4a52-b5f4-1974b4a3cbb3 | Translation Approved (100%) | Conveyor oven, < 63.5 cm belt | Forno com transportador, correia < 63,5 cm |
| 3064f1e6eefb-273b-4d8a-8911-2ab68d7a5c03 | Translation Approved (100%) | Gas | Gás |
| 3065206e2629-e283-4a04-920b-c3be5712243c | Translation Approved (100%) | Cooking | Culinária |
| 306676913755-02a7-4526-842e-f4a66487c43b | Translation Approved (100%) | 20% | 20% |
| 30678f164d5f-d93f-4476-a62a-f23efe2f8c04 | Translation Approved (CM) | 13.2 kW | 13,2 kW |
| 30685b2ed548-1ec8-4ebf-9a82-be46656c01be | Translation Approved (100%) | 42% | 42% |
| 30696079ddec-0953-47fe-a55c-d335778b2912 | Translation Approved (CM) | 8.5 kW | 8,5 kW |
| 30703d7dbd3a-86b3-48b5-9a0d-dfa905cbd18f | Translation Approved (CM) | Fryer | Fritadeira |
| 3071abff8888-f7c2-4309-b624-5a1437af819a | Translation Approved (CM) | Elec | Elét. |
| 30726121120c-2d69-4d89-8729-ba22f0dc8e03 | Translation Approved (100%) | Cooking | Culinária |
| 3073ecfe005d-3c13-456c-a93c-56f87e084d0f | Translation Approved (100%) | 75% | 75% |
| 30745c1b431f-6862-4d09-a636-601fd7dc78e2 | Translation Approved (CM) | 1,05 kW | 1,05 kW |
| 30752cd3d744-0d8d-42d8-9d69-1cf52442f3c5 | Translation Approved (100%) | 80% | 80% |
| 3076bbd57234-e0ca-46fa-b313-4608121e4e5b | Translation Approved (CM) | 1.0 kW | 1,0 kW |
| 3077eeff2167-8747-4d2b-bb1f-5de59ae913ec | Translation Approved (CM) | Fryer | Fritadeira |
| 30788cc18948-f143-425e-839f-d6b53dc7e956 | Translation Approved (100%) | Gas | Gás |
| 3079b0d6f7c5-f038-4154-bc0d-66ab6274e6c3 | Translation Approved (100%) | Cooking | Culinária |
| 30801eef1fb6-a307-4f09-9b82-58e25c5165fd | Translation Approved (100%) | 35% | 35% |
| 30818a65e616-8103-4d08-a9c8-83eaf1d976ab | Translation Approved (CM) | 4.1 kW | 4,1 kW |
| 30826a0d5cda-d543-44da-8d43-a067feb1a5fe | Translation Approved (100%) | 50% | 50% |
| 30836273f426-e7f4-4ee3-986a-6c5a212bd6ef | Translation Approved (CM) | 2.64 kW | 2,64 kW |
| 3084cec82b0b-0b4c-4578-ba31-a23fc77793fc | Translation Approved (100%) | Griddle (based on 90-cm model) | Frigideira (com base no modelo de 90 cm) |
| 308585b87a93-55d2-4b9b-baed-58e028ae13b5 | Translation Approved (100%) | Elec | Elét. |
| 3086390adc20-79dd-4118-8802-d52e9e316725 | Translation Approved (100%) | Cooking | Culinária |
| 3087ed809b54-6aa7-4e5b-8ec6-e50b2000de9e | Translation Approved (100%) | 60% | 60% |
| 3088e964a029-be11-48a6-952d-a96462004807 | Translation Approved (100%) | 4.3 kW/m<4270>2</4270> | 4,3 kW/m<4270>2</4270> |
| 308961de84ec-1fdb-4543-9e3e-1b5ed8a84263 | Translation Approved (100%) | 70% | 70% |
| 309033434898-8c79-43c7-9c41-ce9e90eebad6 | Translation Approved (0%) | 3 .45 kW/m<4277>2</4277> | 3,45 kW/m<4277>2</4277> |
| 3091af6d1411-2e34-4cdd-a4cc-83a3bf304731 | Translation Approved (100%) | Griddle (based on 90-cm model) | Frigideira (com base no modelo de 90 cm) |
| 3092f6a21ad4-05ca-4c09-8284-0ecdce69ba15 | Translation Approved (100%) | Gas | Gás |
| 3093e6302d64-bb2c-42fd-8932-cac2e0376408 | Translation Approved (100%) | Cooking | Culinária |
| 3094996add1a-867d-432b-9fc3-67ab82d6f563 | Translation Approved (100%) | 30% | 30% |
| 3095e71e93da-81db-4048-9951-95e8ba3eb204 | Translation Approved (100%) | 11 kW/m<4293>2</4293> | 11 kW/m<4293>2</4293> |
| 3096c81d64fe-8e08-4eda-99dc-8a91ad3fd266 | Translation Approved (100%) | 33% | 33% |
| 3097927ea54c-28dd-4e35-81c8-a9c73f9f380a | Translation Approved (100%) | 8.35 kW/m<4300>2</4300> | 8,35 kW/m<4300>2</4300> |
| 309835727aaa-5e80-4aa3-837f-b09b756423bd | Translation Approved (CM) | Hot food holding cabinets (excluding drawer warmers and heated display) 0 < V < 0.368 m<4304>3</4304> (V = volume) | Armários de armazenamento de alimentos quentes (exceto aquecedores de gavetas e prateleiras aquecidas), 0 < V < 0,368 m<4304>3</4304> (V = volume) |
| 309911386ff7-224f-4f7d-9417-2b04279e18e7 | Translation Approved (100%) | Elec | Elét. |
| 3100da10269f-8f97-46cd-9db8-6cf4cff41bd6 | Translation Approved (100%) | Cooking | Culinária |
| 3101cf200056-11bd-43c5-b83c-d39bf34a4233 | Translation Approved (100%) | na | n/d |
| 31025115bd6b-fb2d-4335-8ec9-4c63553ac748 | Translation Approved (100%) | 1.4 kW/m<4317>3</4317> | 1,4 kW/m<4317>3</4317> |
| 3103077f410d-2307-410e-8ab0-f015ad10da64 | Translation Approved (100%) | na | n/d |
| 310441bdaaa6-2b23-4701-b37e-4d5b9667cedb | Translation Approved (0%) | (21.5*V)/0.0283 kW/m<4324>3</4324> | (21,5*V)/0,0283 kW/m<4324>3</4324> |
| 31059c422004-6ecb-4342-9bdc-ae8e2c745770 | Translation Approved (CM) | Hot food holding cabinets (excluding drawer warmers and heated display), 0.368 ≤ V < 0.793 m<4328>3</4328> | Armários de armazenamento de alimentos quentes (exceto aquecedores de gavetas e prateleiras aquecidas), 0,368 ≤ V < 0,793 m<4328>3</4328> |
| 31068ed8c310-6b74-4fa1-9e9c-d9602567c158 | Translation Approved (100%) | Elec | Elét. |
| 3107ecef2630-8e51-4bd3-aed9-db1263fdd2f5 | Translation Approved (100%) | Cooking | Culinária |
| 3108a2c0eec1-ff4b-4af1-83b4-305c483b4d67 | Translation Approved (100%) | na | n/d |
| 310980961ff7-0bf8-48ee-9847-351603b6fb2b | Translation Approved (100%) | 1.4 kW/m<4341>3</4341> | 1,4 kW/m<4341>3</4341> |
| 311014892e80-bbaf-4709-9131-92be9a7e1e65 | Translation Approved (100%) | na | n/d |
| 31115d762ce8-0514-4203-bcd7-e7c1f4693aa4 | Translation Approved (0%) | (2.0*V + 254)/0.0283 kW/m<4348>3</4348> | (2,0*V + 254)/0,0283 kW/m<4348>3</4348> |
| 31121f80c09f-1380-4da4-b2b6-596bada10e94 | Translation Approved (CM) | Hot food holding cabinets (excluding drawer warmers and heated display), 0.793 m<4352>3</4352> ≤ V | Armários para armazenamento de alimentos quentes (exceto aquecedores de gavetas e prateleiras aquecidas), 0,793 m<4352>3</4352> ≤ V |
| 3113cdc54db7-08a1-4aae-be6c-f48b9213a848 | Translation Approved (100%) | Elec | Elét. |
| 311476cd1247-c94b-4555-b37f-016f7268b4f7 | Translation Approved (100%) | Cooking | Culinária |
| 3115bf87b9b1-1777-4b1d-9bbf-afbd5b83d80e | Translation Approved (100%) | na | n/d |
| 3116c8c56603-4776-4f95-82e6-b02b67b47695 | Translation Approved (100%) | 1.4 kW/m<4365>3</4365> | 1,4 kW/m<4365>3</4365> |
| 311776c1fe7d-0f52-4fe6-a7cc-fd7fb3ab62f1 | Translation Approved (100%) | na | n/d |
| 31182f2debff-132b-4c2e-9f18-37151ac1a772 | Translation Approved (CM) | (3.8*V + 203.5)/0.0283 kW/m<4372>3</4372> | (3,8*V + 203,5)/0,0283 kW/m<4372>3</4372> |
| 3119867d1987-5234-407d-9599-7fad4338af5d | Translation Approved (100%) | Large vat fryer | Fritadeira de tanque grande |
| 3120b9abad27-ef66-4d59-b875-7b46957b2deb | Translation Approved (CM) | Elec | Elét. |
| 3121d5d15020-61d1-4d20-bf8a-3b2324a6220b | Translation Approved (100%) | Cooking | Culinária |
| 3122a4c050db-9e29-4800-89fb-453cc7448eb5 | Translation Approved (100%) | 75% | 75% |
| 312325950a79-11c9-4c55-a0c9-581e95b26faa | Translation Approved (CM) | 1.35 kW | 1,35 kW |
| 3124ed91c66e-a412-472c-ac99-a95f9c9a3e8a | Translation Approved (100%) | 80% | 80% |
| 312503406fe3-e7b5-432f-be8c-a30c94f79ed5 | Translation Approved (CM) | 1.1 kW | 1,1 kW |
| 3126ecd75e93-75bf-4355-a191-44e5d6e803b3 | Translation Approved (100%) | Large vat fryer | Fritadeira de tanque grande |
| 31278706e6a1-8572-4f3f-8233-ba201298615a | Translation Approved (100%) | Gas | Gás |
| 3128a651422c-220e-47c4-a0ca-18e6f5f070cb | Translation Approved (100%) | Cooking | Culinária |
| 3129260dfe8d-d840-400d-bf18-a21f178ffa17 | Translation Approved (100%) | 35% | 35% |
| 31300227a6a4-db16-469b-819c-2446705d9cb4 | Translation Approved (CM) | 5.86 kW | 5,86 kW |
| 313148ff10f6-ef6d-48a0-9810-fc06941db9de | Translation Approved (100%) | 50% | 50% |
| 31322d9e8d5b-dee3-4ab1-8d62-a7add62fb0f8 | Translation Approved (CM) | 3.5 kW | 3,5 kW |
| 313300a4e3c7-f08e-4bcd-8744-06e3a434d542 | Translation Approved (100%) | Rack oven, double | Forno rotativo, duplo |
| 3134dc7e262d-d774-45bf-8391-fd708ccde40d | Translation Approved (100%) | Gas | Gás |
| 3135a8e395ca-6b39-4a4d-953f-3f58aebd3f94 | Translation Approved (100%) | Cooking | Culinária |
| 3136d8980f2b-5a3e-4054-90a4-9489a229fc1f | Translation Approved (100%) | 30% | 30% |
| 31372279d462-ec4a-4dd9-b23f-cec2b1076ad1 | Translation Approved (CM) | 19 kW | 19 kW |
| 3138a353778f-ad23-4030-ba62-c0207ec7e75e | Translation Approved (100%) | 50% | 50% |
| 313906aa33c4-fe76-4e4b-8a6b-cf19277361b6 | Translation Approved (CM) | 10.25 kW | 10,25 kW |
| 3140fb1cc789-8cf3-47a6-9816-f3ce3b5d6477 | Translation Approved (100%) | Rack oven, single | Forno rotativo, simples |
| 3141a556cb92-94cf-4dcb-af86-6b341066c764 | Translation Approved (100%) | Gas | Gás |
| 314277c783a0-fc57-4de1-8a32-15743dc52cb4 | Translation Approved (100%) | Cooking | Culinária |
| 31433c5c01d5-b24d-4dec-ab82-7920a762ad74 | Translation Approved (100%) | 30% | 30% |
| 3144d0aba432-89bd-43fd-9a14-53d26b03a6c8 | Translation Approved (CM) | 12.6 kW | 12,6 kW |
| 31458fdf2916-dcfe-43de-aaa3-78cea961ec39 | Translation Approved (100%) | 50% | 50% |
| 3146af14b6f1-7bca-44ad-9af1-5be24027a54b | Translation Approved (CM) | 8.5 kW | 8,5 kW |
| 3147bd057243-0cab-4b36-afbf-aeb9c10cdf2f | Translation Approved (100%) | Range | Fogão |
| 314829465fab-e1cd-4b13-a800-da1b853a7d21 | Translation Approved (CM) | Elec | Elét. |
| 3149f205cb24-0396-47a0-8ae4-8e2fad3b60d9 | Translation Approved (100%) | Cooking | Culinária |
| 31508d2db3cc-a748-4bf7-b13b-a70379ee9398 | Translation Approved (100%) | 70% | 70% |
| 31519a13d98b-dd99-4cdb-b2a9-7bbbc35491e0 | Translation Approved (100%) | na | n/d |
| 3152e4f8f7f5-9b81-44a8-8ac3-a1634ed6a05b | Translation Approved (100%) | 80% | 80% |
| 3153adf4d3ce-5156-4c4a-a77f-707db2a843e6 | Translation Approved (100%) | na | n/d |
| 31548a3328b8-c0c1-4e28-960f-4526e19abb99 | Translation Approved (100%) | Range | Fogão |
| 315512cada52-b718-4c1f-8da7-28d22d45ca1b | Translation Approved (100%) | Gas | Gás |
| 315670c06991-1df2-4dc3-ad7a-d56b63e980e5 | Translation Approved (100%) | Cooking | Culinária |
| 3157d048d5a8-8141-45d9-8a5c-95b02fd7ff51 | Translation Approved (100%) | 35% | 35% |
| 3158a4b0f078-ddc6-4c26-a5e6-07c1a3f02ad5 | Translation Approved (100%) | na | n/d |
| 3159e06ce8b0-b36a-4a9c-a460-1f9cd1e81905 | Translation Approved (CM) | 40% and no standing pilots | 40% e sem chama piloto |
| 31602cde3184-4958-4148-a7ac-3f295a12b4fc | Translation Approved (100%) | na | n/d |
| 316127c2b790-99e7-4e37-bfe4-89bc74e74528 | Translation Approved (100%) | Steam cooker, batch cooking | Panela a vapor, cozinhar em lote |
| 3162d09f85e3-a5e0-46b7-a841-bed724916f77 | Translation Approved (100%) | Elec | Elét. |
| 31635ea0ced5-2eca-495c-b5b0-464c85763acf | Translation Approved (100%) | Cooking | Culinária |
| 31644abf581c-b433-4cb1-9b33-6561685cc58f | Translation Approved (100%) | 26% | 26% |
| 31655e139933-6b9f-4068-adb4-5016ed0622ae | Translation Approved (CM) | 200 W/pan | 200 W/panela |
| 31665242d995-d335-4708-9ada-c7c3947abe1c | Translation Approved (100%) | 50% | 50% |
| 316719d99c83-c176-4576-a18e-e43919add7fe | Translation Approved (CM) | 135 W/pan | 135 W/panela |
| 31681b406e0d-8bbc-4d2c-ab05-587a8d99c082 | Translation Approved (100%) | Steam cooker, batch cooking | Panela a vapor, cozinhar em lote |
| 31693f0fbf71-ddb6-416f-a48c-a7e3f745bf00 | Translation Approved (100%) | Gas | Gás |
| 3170ebd51f26-f75c-4f2a-9ded-0a869cee1c10 | Translation Approved (100%) | Cooking | Culinária |
| 3171c5dfc645-91cc-458a-b253-898952d3ee8a | Translation Approved (100%) | 15% | 15% |
| 3172707e0a1e-3d82-44aa-845f-5040e898aee9 | Translation Approved (CM) | 733 W/pan | 733 W/panela |
| 31732a4d1d56-733c-4c22-b309-f24270817897 | Translation Approved (100%) | 38% | 38% |
| 3174f63fc568-37c9-42f7-9a0d-c1b6c4318458 | Translation Approved (CM) | 615 W/pan | 615 W/panela |
| 3175660475f1-b094-4922-903f-28cce44013c9 | Translation Approved (100%) | Steam cooker, high production or cook to order | Panela a vapor, alta produção ou "cook to order" |
| 317672070c14-c86c-4484-bb20-9ce7ff5b5bd0 | Translation Approved (CM) | Elec | Elét. |
| 31776991303b-2f91-49ab-b73a-b616212a9b8d | Translation Approved (100%) | Cooking | Culinária |
| 31785cb5778f-860b-46de-91e6-cf5f9c8f4050 | Translation Approved (100%) | 26% | 26% |
| 31793cd5d2c4-295f-4f1f-801a-1608db5438c6 | Translation Approved (CM) | 330 W/pan | 330 W/panela |
| 3180926a07a0-8bc6-4766-b2d4-902f8d7ee6dc | Translation Approved (100%) | 50% | 50% |
| 31813bd22963-7015-4ed4-89cf-094fbceb52bb | Translation Approved (CM) | 275 W/pan | 275 W/panela |
| 3182e2f4ab1b-f823-49e3-9560-08f13f6d3e25 | Translation Approved (100%) | Steam cooker, high production or cook to order | Panela a vapor, alta produção ou "cook to order" |
| 31836d767ded-2d12-49bd-97c7-59e5fdab00c9 | Translation Approved (100%) | Gas | Gás |
| 3184f7bb5532-38a2-4821-ac26-ad3f22c7d72a | Translation Approved (100%) | Cooking | Culinária |
| 318574509c59-bc5f-4772-abf8-12f79cfc8f68 | Translation Approved (100%) | 15% | 15% |
| 3186caf92485-31a2-4781-a18b-a0009852c1a8 | Translation Approved (CM) | 1.47 kW/pan | 1,47 kW/panela |
| 31870b4fd9ff-2ccc-4e1f-af8e-80a28b6ee778 | Translation Approved (100%) | 38% | 38% |
| 318879d2c467-9af0-4860-85bd-f8ba86a84cd0 | Translation Approved (CM) | 1.26 kW/pan | 1,26 kW/panela |
| 318902f026a1-d701-4a47-ba4a-3bac4b44ff1b | Translation Approved (100%) | Toaster | Torradeira |
| 31909cb4566c-b009-4f8d-9af8-0443d8e92eb0 | Translation Approved (CM) | Elec | Elét. |
| 319160594de6-7c3c-4f96-9826-217a752a7229 | Translation Approved (100%) | Cooking | Culinária |
| 3192a52acadc-4109-4adb-aa7a-d7e619307735 | Translation Approved (100%) | na | n/d |
| 3193258355a8-a265-42ff-93b5-1bcb91cbd403 | Translation Approved (100%) | 1.8 kW average operating energy rate | 1,8 kW de taxa de energia média de operação |
| 31940a0fd333-7a33-45dc-8fa2-93100b0409ff | Translation Approved (100%) | na | n/d |
| 319512f0eddf-1264-4e0e-833a-65ed6ac43473 | Translation Approved (100%) | 1.2 kW average operating energy rate | 1,2 kW de taxa de energia média de operação |
| 319643ec2187-7a9c-4e1c-a2f1-948f2f602ae9 | Translation Approved (CM) | Ice machine, IMH (ice making head, H = ice harvest) H <4610>≥</4610> 204 kg/day | Máquina de gelo, IMH (cabeçote de produção de gelo, H = coleta de gelo), H <4610>≥</4610> 204 kg/dia |
| 319724c8c23e-506b-486f-9f8f-402419d6fda1 | Translation Approved (100%) | Elec | Elét. |
| 319896963e0b-4ff7-475c-ba56-230bddfff349 | Translation Approved (CM) | Ice | Gelo |
| 319904bd4985-1040-490e-90ae-c183de8a1222 | Translation Approved (0%) | 0.0015 - 5.3464E<4620>-07</4620> kWh/kg ice | 0,0015 - 5,3464E<4620>-07</4620> kWh/kg de gelo |
| 32006e5d8faf-4c36-4f27-ae35-6ca9b78914b9 | Translation Approved (100%) | na | n/d |
| 3201dbd68130-d532-4774-92b0-e217ff6b302c | Translation Approved (0%) | ≤ 13.52*H<4627>-0.298</4627> kWh/100 kg ice | ≤ 13,52*H<4627>-0,298</4627> kWh/100 kg de gelo |
| 3202ec827acf-53ea-41d8-b90d-43a250b89f2e | Translation Approved (100%) | na | n/d |
| 32034d0ce5d5-c4f1-4a47-a064-da4034196736 | Translation Approved (CM) | Ice machine, IMH (ice making head), H < 204 kg/day | Máquina de gelo, IMH (cabeçote de produção de gelo), H < 204 kg/dia |
| 320413015ca4-2a13-40ae-ad21-d37bf10283f7 | Translation Approved (100%) | Elec | Elét. |
| 32055aef3a57-4965-47e1-9edd-702b0483a6c0 | Translation Approved (CM) | Ice | Gelo |
| 320622e4d2a9-5252-4bec-86ff-721b335dea5e | Translation Approved (0%) | 0.2262 - 4.18E<4643>-04</4643> kWh/kg ice | 0,2262 - 4,18E<4643>-04</4643> kWh/kg de gelo |
| 320746123a9b-c832-4149-ac30-1d5ec81a3218 | Translation Approved (100%) | na | n/d |
| 320840ef2792-59d0-4a4c-9d8e-7d57b2f2b423 | Translation Approved (CM) | ≤ 13.52*H<4650>-0.298</4650> kWh/100 kg ice | ≤ 13,52*H<4650>-0,298</4650> kWh/100 kg de gelo |
| 3209bafd1839-4f42-46f8-8df8-189e4c6bcf2a | Translation Approved (100%) | na | n/d |
| 321019125e26-7e67-411b-af39-b1342e09477b | Translation Approved (CM) | Ice machine, RCU <4657> </4657>(remote condensing unit, w/o remote compressor) H < 454 kg/day | Máquina de gelo, RCU (unidade de condensação remota, sem compressor remoto), H < 454 kg/dia |
| 3211397f84ed-2721-4a86-b16c-30651d9ac6eb | Translation Approved (CM) | Elec | Elét. |
| 3212f4bcd34e-59a4-40fb-9536-088e941ab4eb | Translation Approved (CM) | Ice | Gelo |
| 3213d827091f-6d92-411a-8124-7e6857921293 | Translation Approved (0%) | 0.1951 - 1.85E<4667>-04</4667> kWh/kg ice | 0,1951 - 1,85E<4667>-04</4667> kWh/kg de gelo |
| 321448217c2c-0ad0-4d4d-bb4d-9cbc13aef48d | Translation Approved (100%) | na | n/d |
| 3215ff99b634-7b6b-48b2-9c6f-e329f7cc4820 | Translation Approved (0%) | ≤ 111.5835H<4674>-0.258</4674>) + 2.205 kWh/100 kg ice | ≤ 111,5835H<4674>-0,258</4674>) + 2,205 kWh/100 kg de gelo |
| 3216c59518a8-423a-42a6-b01a-16fc13588174 | Translation Approved (100%) | na | n/d |
| 321715a2275d-b26d-4761-b4ef-63e8532bf540 | Translation Approved (CM) | Ice machine, RCU (remote condensing unit) 726 > H <4681>≥</4681> 454 kg/day | Máquina de gelo, RCU (unidade de condensação remota) 726 > H <4681>≥</4681> 454 kg/dia |
| 3218663ba0c3-6b68-4d61-ac54-daca8cf6193a | Translation Approved (CM) | Elec | Elét. |
| 32199b1ca2a2-445a-4ce7-a739-ad9b4926476b | Translation Approved (CM) | Ice | Gelo |
| 32200f4549cb-d004-4ab4-b47f-77c259a80b89 | Translation Approved (0%) | 0.1124 kWh/kg ice | 0,1124 kWh/kg de gelo |
| 3221391e02a4-a7f2-4566-8817-b3486ca3ee92 | Translation Approved (100%) | na | n/d |
| 3222dd444272-1afe-4c90-bd14-1f80d108fb0d | Translation Approved (CM) | ≤ 111.5835H<4697>-0.258</4697>) + 2.205 kWh/100 kg ice | ≤ 111,5835H<4697>-0,258</4697>) + 2,205 kWh/100 kg de gelo |
| 32234b29d41d-15b0-4de6-a7fb-66b6b3f5e9b6 | Translation Approved (100%) | na | n/d |
| 32240240a1e4-2fed-4faf-8c8f-349861bc764d | Translation Approved (CM) | Ice machine, RCU (remote condensing unit), H <4704>></4704> 726kg/day | Máquina de gelo, RCU (unidade de condensação remota), H <4704>></4704> 726 kg/dia |
| 322540133ead-9c88-40a8-b04f-5923e47ef3f9 | Translation Approved (CM) | Elec | Elét. |
| 3226bc0ac3c2-abf3-4aff-92ad-68dbc3142b03 | Translation Approved (CM) | Ice | Gelo |
| 3227dbbd28e7-0b12-4920-ab01-ce87aaf76507 | Translation Approved (CM) | 0.1124 kWh/kg ice | 0,1124 kWh/kg de gelo |
| 3228b83bcc9a-6d51-4c01-ab90-f107ff830bd5 | Translation Approved (100%) | na | n/d |
| 322946ab2af9-33f6-45ef-9ef2-95fd9eebeafc | Translation Approved (0%) | ≤ -0.00024H + 4.60 kWh/100 kg ice | ≤ -0,00024H + 4,60 kWh/100 kg de gelo |
| 32309686b904-ec9b-44ba-bf10-9e1d3156e311 | Translation Approved (100%) | na | n/d |
| 3231fc6bfd6a-482d-4c78-9006-9b31a0b8fa06 | Translation Approved (CM) | Ice machine, SCU (self contained unit), H < 79 kg/day | Máquina de gelo, SCU (unidade autônoma), H < 79 kg/dia |
| 32327a8bc47d-4a17-46ec-b4dd-cfaa6dcc1620 | Translation Approved (100%) | Elec | Elét. |
| 3233570c54e5-1c04-4c43-a7fe-09a8497ae12b | Translation Approved (CM) | Ice | Gelo |
| 32348e6cc6d5-7ed6-43f0-a343-6d6b4391a25f | Translation Approved (0%) | 0.3968 - 2.28E<4735>-03</4735> kWh/kg ice | 0,3968 - 2,28E<4735>-03</4735> kWh/kg de gelo |
| 3235acd6f32c-2822-4ccc-b84b-bfe5fe5fdd0a | Translation Approved (100%) | na | n/d |
| 3236a7f6d2d0-43b5-4f8d-a1c5-0932004e7dcf | Translation Approved (0%) | 236.59H<4742>-0.326 </4742>+0.176 kWh/100 kg ice | 236,59H<4742>-0.326 </4742>+0,176 kWh/100 kg de gelo |
| 3237e9044fdb-7af9-413a-b548-cb3d0059390a | Translation Approved (100%) | na | n/d |
| 323895d64473-1385-4a0e-ae21-6efc2e7898b6 | Translation Approved (CM) | Ice machine, SCU (self contained unit), H <4749>≥</4749> 79 kg/day | Máquina de gelo, SCU (unidade autônoma), H <4749>≥</4749> 79 kg/dia |
| 32396621d807-ec10-4a90-a7cd-9a156469cad4 | Translation Approved (CM) | Elec | Elét. |
| 3240e2694991-7053-4daa-a29e-f145e48c82c3 | Translation Approved (CM) | Ice | Gelo |
| 3241b1df6d9a-f666-4e65-9dc7-0165db22c78e | Translation Approved (100%) | 0.2161 kWh/kg ice | 0,2161 kWh/kg de gelo |
| 3242eb22247b-1383-4740-ad34-74d5d1c71e79 | Translation Approved (100%) | na | n/d |
| 3243b1d4081a-0a52-4240-aa97-08b4bdbf7ff9 | Translation Approved (CM) | 236.59H<4765>-0.326 </4765>+0.176 kWh/100 kg ice | 236,59H<4765>-0.326 </4765>+0,176 kWh/100 kg de gelo |
| 3244b849fc36-aff3-49ce-af65-0a5db4da7241 | Translation Approved (100%) | na | n/d |
| 32454cf50ccf-15aa-4823-8ece-04396b5aef1c | Translation Approved (CM) | Ice machine, water-cooled ice-making head, H <4772>≥</4772> 651 kg/day(must be on a chilled loop) | Máquina de gelo, cabeçote de produção de gelo resfriado à água, H <4772>≥</4772> 651 kg/dia (deve estar em ciclo resfriado) |
| 32463c6b6191-9183-4c2c-8abf-f2264ea02da8 | Translation Approved (100%) | Elec | Elét. |
| 324793177bab-17ae-44fa-a469-dc197457bdc5 | Translation Approved (CM) | Ice | Gelo |
| 3248a586fdda-084e-4c3e-8075-359215f8ef95 | Translation Approved (100%) | 0.0882 kWh/kg ice | 0,0882 kWh/kg de gelo |
| 32491a1483d5-0917-4c24-b628-73ce848d76e6 | Translation Approved (100%) | na | n/d |
| 3250cef18eea-c1b4-4d42-8ef8-592a1d468e77 | Translation Approved (0%) | ≤ 8.11 kWh/100 kg ice | ≤ 8,11 kWh/100 kg de gelo |
| 325181799b8c-7329-4c06-a03b-e78afc7807f2 | Translation Approved (100%) | na | n/d |
| 32526a1435a0-2aab-47c5-8a22-1594462b6c39 | Translation Approved (CM) | Ice machine, water-cooled ice-making head, 227 <4794>≤ </4794>H < 651 kg/day (must be on a chilled loop) | Máquina de gelo, cabeçote de produção de gelo resfriado à água, 227 <4794>≤ </4794>H < 651 kg/dia (deve estar em ciclo resfriado) |
| 325354f1c25c-2d17-41d9-bfd7-f2847e2a4fa2 | Translation Approved (100%) | Elec | Elét. |
| 3254715e8ce7-c8a1-4c87-98e6-3d451000abc4 | Translation Approved (CM) | Ice | Gelo |
| 3255649f4d9f-858e-4c1e-97f2-ee1235a92cbd | Translation Approved (96%) | 0.1230 - 5.35E<4804>-05</4804> kWh/kg ice | 0,1230 - 5,35E<4804>-05</4804> kWh/kg de gelo |
| 325687e64063-3a20-430c-a9e9-eddee00f1c00 | Translation Approved (100%) | na | n/d |
| 3257bcfc0bd7-3716-4166-b415-84d874b4fb3c | Translation Approved (0%) | ≤ 11.31 - 0.065H kWh/100 kg ice | ≤ 11,31 - 0,065H kWh/100 kg de gelo |
| 32587145624c-4d94-47cc-89af-20e78657b54f | Translation Approved (100%) | na | n/d |
| 32593802b953-eced-406b-a509-e2fad9b36960 | Translation Approved (CM) | Ice machine, water-cooled ice-making head, H < 227 kg/day( must be on a chilled loop) | Máquina de gelo, cabeçote de produção de gelo resfriado à água, H < 227 kg/dia (deve estar em ciclo resfriado) |
| 3260e6fe231c-9052-4d54-b3b5-c344e4ebcc1b | Translation Approved (CM) | Elec | Elét. |
| 3261252aa6fa-f6ef-40e3-a137-4d05fb16a924 | Translation Approved (CM) | Ice | Gelo |
| 32621173bdc8-6c2a-4c47-b0db-60c174944ff1 | Translation Approved (92%) | 0.1720 - 2.67E<4826>-04</4826> kWh/kg ice | 0,1720 - 2,67E<4826>-04</4826> kWh/kg de gelo |
| 3263d9bf0cf1-39b2-4b09-ac48-d25f6c257bac | Translation Approved (100%) | na | n/d |
| 3264c1c0b02c-1ea9-4e3f-9e68-2d53ab4bb16a | Translation Approved (0%) | ≤ 15.48 - 0.0238H kWh/100 kg ice | ≤ 15,48 - 0,0238H kWh/100 kg de gelo |
| 326563a3d020-2a6a-4bc1-86e6-ca9d556ee99b | Translation Approved (100%) | na | n/d |
| 3266d46f3169-9815-473b-8a7c-b5740a036073 | Translation Approved (100%) | Ice machine, water-cooled once-through (open loop) | Máquina de gelo, resfriamento à água em uma passagem (ciclo aberto) |
| 3267c0e926e4-6ec1-4b48-b726-4727c28f70da | Translation Approved (CM) | Elec | Elét. |
| 3268dbbda1c8-9c72-44fc-8cc9-c4b25a130d8b | Translation Approved (CM) | Ice | Gelo |
| 326934a538ce-e308-4d05-9e1f-7a67578ee6b6 | Translation Approved (100%) | Banned | Proibido |
| 32706041e4c5-3abe-46f0-be4f-a8e28de2fd68 | Translation Approved (100%) | Banned | Proibido |
| 327167c01b28-88df-456b-98e4-c0e5305fee48 | Translation Approved (100%) | Banned | Proibido |
| 32725655d09e-70ed-4a26-bfbd-43f2c5203700 | Translation Approved (100%) | Banned | Proibido |
| 3273b52f4fab-ace3-41d6-9248-132f3fc31575 | Translation Approved (CM) | Ice machine, water cooled SCU (self-contained unit) H < 91 kg/day (must be on a chilled loop) | Máquina de gelo, SCU resfriada à água (unidade autônoma), H < 91 kg/dia (deve estar em ciclo resfriado) |
| 327439d7effc-e214-452a-b118-bf1b7b8a74ac | Translation Approved (100%) | Elec | Elét. |
| 327523bbf9f7-6b1b-43c7-a740-518f6c4d2471 | Translation Approved (CM) | Ice | Gelo |
| 3276195b6a4b-fb55-4da0-9125-a5e3b9ec48cc | Translation Approved (96%) | 0.2513 - 9.23E<4869>-04</4869> kWh/kg ice | 0,2513 - 9,23E<4869>-04</4869> kWh/kg de gelo |
| 32770fb79e5b-b49e-456d-9108-8ea40d58ebc8 | Translation Approved (100%) | na | n/d |
| 32784e5bbfcb-5082-4b59-9b0a-6b9797c0aabb | Translation Approved (0%) | ≤ 23.37-0.086H kWh/100 kg ice | ≤ 23,37-0,086H kWh/100 kg de gelo |
| 327905b1bd78-59b8-464b-93e8-c39cd3ba1295 | Translation Approved (100%) | na | n/d |
| 32802a4143e1-7cc6-4f3f-ac9e-cc3000ddf84e | Translation Approved (CM) | Ice machine, water cooled SCU (self-contained unit) H <4882>></4882> 91 kg/day (must be on a chilled loop) | Máquina de gelo, SCU resfriada à água (unidade autônoma), H <4882>></4882> 91 kg/dia (deve estar em ciclo resfriado) |
| 3281289fe7ec-a5dd-4fd6-b74e-42fe21ce786d | Translation Approved (CM) | Elec | Elét. |
| 3282b88b4b3e-04dd-4832-8502-7417e6e756bf | Translation Approved (CM) | Ice | Gelo |
| 3283d4c84dd6-cca1-4c1b-b5b7-75c2cf04736d | Translation Approved (100%) | 0.1676 kWh/kg ice | 0,1676 kWh/kg de gelo |
| 3284414a213b-d5bc-4ff4-bf6e-b3aa7cc9eb28 | Translation Approved (100%) | na | n/d |
| 328526fbe30b-8c12-4065-9749-1f25b52d68a3 | Translation Approved (CM) | 15.57 kWh/100 kg ice | 15,57 kWh/100 kg de gelo |
| 328664b3e3b2-b769-478d-a44a-158e73c5b243 | Translation Approved (100%) | na | n/d |
| 328706299e13-21e7-4ac4-9b90-1bacd31ed9e2 | Translation Approved (100%) | Chest freezer, solid or glass door | Freezer horizontal, porta sólida ou de vidro |
| 328894547e26-4a7c-4078-8668-798573674b97 | Translation Approved (CM) | Elec | Elét. |
| 32891d3aaca8-d4f2-4ff1-b5e6-b6e503f52ac6 | Translation Approved (CM) | Refrig | Refrig. |
| 3290a3d57f1c-bb77-4951-b0b5-58de71f447f8 | Translation Approved (CM) | 15.90V + 0.943 kWh/day | 15,90 V + 0,943 kWh/dia |
| 3291ecbdc7e1-3d55-4957-b13f-f28959c7602b | Translation Approved (100%) | na | n/d |
| 3292999b9a76-35f8-4381-a554-da6c8bdefc21 | Translation Approved (100%) | 9.541V + 0.130 kWh/day | 9,541 V + 0,130 kWh/dia |
| 32937ee76ffd-4652-4e3b-9b6d-04d6a124f23b | Translation Approved (100%) | na | n/d |
| 3294995134a9-54d3-4de5-a2e8-eceb2c2c0f21 | Translation Approved (100%) | Chest refrigerator, solid or glass door | Refrigerador horizontal, porta sólida ou de vidro |
| 3295bba58597-ce9f-41c3-941b-58f3f302878f | Translation Approved (CM) | Elec | Elét. |
| 32969d38ef62-fb6c-4b50-8303-aa55d7610a39 | Translation Approved (CM) | Refrig | Refrig. |
| 3297fa47eacb-6b8e-4fcc-b58c-0805166493b5 | Translation Approved (CM) | 3.53V + 2.04 kWh/day | 3,53 V + 2,04 kWh/dia |
| 329854500db5-fc76-4687-aa87-b72f030e53ba | Translation Approved (100%) | na | n/d |
| 3299e74c3ad3-20e1-4693-baa4-8bf7561e73e2 | Translation Approved (CM) | ≤ 4.417 V + 0.475 kWh/day | ≤ 4,417 V + 0,475 kWh/dia |
| 3300f2cac7b8-9143-41f7-b03f-76fa13e109e2 | Translation Approved (100%) | na | n/d |
| 33016d05cad6-5071-4229-a0f8-46ba0f189996 | Translation Approved (80%) | Glass-door reach-in freezer, 0 < V < 0.42 m<4946>3</4946> | Freezer vertical com porta de vidro, 0 < V < 0,42 m<4946>3</4946> |
| 330242ee044c-595a-4c3e-a158-ecc4c80a496a | Translation Approved (100%) | Elec | Elét. |
| 33032af78794-703b-4f71-b126-ee208f3346b1 | Translation Approved (CM) | Refrig | Refrig. |
| 3304aeac0f2a-3188-4ff8-a2e8-118afe5f55c2 | Translation Approved (CM) | 26.50V + 4.1 kWh/day | 26,50 V + 4,1 kWh/dia |
| 3305697e7f41-3186-4dd5-9812-252626b3c3cd | Translation Approved (100%) | na | n/d |
| 3306482a1bec-256a-446c-b1e6-4374c6bb660c | Translation Approved (CM) | ≤ 21.449V + 0.893 kWh/day | ≤ 21,449 V + 0,893 kWh/dia |
| 3307ea8bc972-b9e9-4e0a-a85a-cdbb3b56a3ad | Translation Approved (100%) | na | n/d |
| 3308371cac91-605c-4c7f-ae54-d7d92d0f7e75 | Translation Approved (99%) | Glass-door reach-in freezer, 0.42 ≤ V < 0.85 m<4968>3</4968> | Freezer vertical com porta de vidro, 0,42 ≤ V < 0,85 m<4968>3</4968> |
| 33098cec1e22-05d8-427d-8e71-56c472a9eee5 | Translation Approved (CM) | Elec | Elét. |
| 33104b81764c-344c-45f1-9bd9-097cad8f3217 | Translation Approved (CM) | Refrig | Refrig. |
| 3311da15f79e-348a-48f0-8e39-22bc61a31479 | Translation Approved (CM) | 26.50V + 4.1 kWh/day | 26,50 V + 4,1 kWh/dia |
| 331205f9070d-5fef-466d-8a25-a51abc256eda | Translation Approved (100%) | na | n/d |
| 331360d82258-5238-4682-aba6-c9b0386076f3 | Translation Approved (CM) | ≤ 25.901V – 1.00 kWh/day | ≤ 25,901 V – 1,00 kWh/dia |
| 3314cebae804-c766-4a2d-8f32-43a99f5ff251 | Translation Approved (100%) | na | n/d |
| 3315255c47be-4ce6-4d15-a534-d081ca3aba1c | Translation Approved (CM) | Glass-door reach-in freezer, 0.85 ≤ V < 1.42 m<4990>3</4990> | Freezer vertical com porta de vidro, 0,85 ≤ V < 1,42 m<4990>3</4990> |
| 331692892063-3692-47f2-8a7a-bdf50a387e1d | Translation Approved (CM) | Elec | Elét. |
| 3317c755136d-c98e-460f-af8b-741010ccc73c | Translation Approved (CM) | Refrig | Refrig. |
| 3318581a37af-2d79-46a9-a1c5-23da15eb2078 | Translation Approved (CM) | 26.50V + 4.1 kWh/day | 26,50V + 4,1 kWh/dia |
| 3319d653e026-1504-4948-afeb-fb710a4a372d | Translation Approved (100%) | na | n/d |
| 332098b417b1-9844-4f95-9f2b-1031575f9499 | Translation Approved (CM) | ≤ 8.834V + 13.50 kWh/day | ≤ 8,834 V + 13,50 kWh/dia |
| 332134f5ec10-f9ed-415b-b5fb-69c1abd2ce23 | Translation Approved (100%) | na | n/d |
| 33225fe17d2d-a3d7-47f0-b014-18d5924b469a | Translation Approved (90%) | Glass-door reach-in freezer, 1.42 ≤ V m<5012>3</5012> | Freezer vertical com porta de vidro, 1,42 ≤ V m<5012>3</5012> |
| 3323398132cc-cbdc-4b8b-bfd8-b2cf3fee9ef6 | Translation Approved (100%) | Elec | Elét. |
| 33242a142d6f-dc5f-47be-b4e6-f3252731cbb3 | Translation Approved (CM) | Refrig | Refrig. |
| 33257fea9e89-184a-4874-b15d-72a4fe0f34b2 | Translation Approved (CM) | 26.50V + 4.1 kWh/day | 26,50 V + 4,1 kWh/dia |
| 3326d9e40ed0-6d83-4c0a-8ca2-e2632f11e7ce | Translation Approved (100%) | na | n/d |
| 33270b03e5b6-9c95-4f3b-abae-9bedd19ca6d8 | Translation Approved (CM) | ≤ 15.90V + 3.50 kWh/day | ≤ 15,90 V + 3,50 kWh/dia |
| 3328a6c0a358-823c-4859-ba77-8a9f40fd1c50 | Translation Approved (100%) | na | n/d |
| 33298445167d-d5e7-4d69-aa0d-1b45837c45a6 | Translation Approved (91%) | Glass-door reach-in refrigerator, 0 < V < 0.42m<5034>3</5034> | Refrigerador vertical com porta de vidro, 0 < V < 0,42m<5034>3</5034> |
| 3330114dab83-1095-460c-ac82-cd33e2d9b4a0 | Translation Approved (100%) | Elec | Elét. |
| 3331df25c7ed-436e-4eb1-b51c-81bbce185ed2 | Translation Approved (CM) | Refrig | Refrig. |
| 3332d9af10d5-af77-4d19-b6f6-49572260e800 | Translation Approved (CM) | 4.24V + 3.34 kWh/day | 4,24 V + 3,34 kWh/dia |
| 333398767adf-8c42-466d-8146-314ee29f3c21 | Translation Approved (100%) | na | n/d |
| 3334c08e9340-0bde-40e2-a644-94f15fe4f3e0 | Translation Approved (CM) | ≤ 4.169V + 1.382 kWh/day | ≤ 4,169 V + 1,382 kWh/dia |
| 33350309cee1-8312-4db8-9934-0e05d6b81aec | Translation Approved (100%) | na | n/d |
| 3336bcf28320-c8c5-4eb5-8a35-db02d3c6b1b1 | Translation Approved (99%) | Glass-door reach-in refrigerator, 0.42 ≤ V < 0.85 m<5056>3</5056> | Refrigerador vertical com porta de vidro, 0,42 ≤ V < 0,85 m<5056>3</5056> |
| 333745be6b91-567c-43ab-887d-f7d1db17dbb4 | Translation Approved (CM) | Elec | Elét. |
| 33380dcb5581-2cfe-465f-a052-123bd96aff53 | Translation Approved (CM) | Refrig | Refrig. |
| 333968c48744-54fe-41d6-9790-655193e10260 | Translation Approved (CM) | 4.24V + 3.34 kWh/day | 4,24 V + 3,34 kWh/dia |
| 3340758b8cb7-114c-44cf-b084-a86fc54fefab | Translation Approved (100%) | na | n/d |
| 334173f06326-ec94-4818-9852-d60fef2a1b94 | Translation Approved (CM) | ≤ 4.947V + 1.050 kWh/day | ≤ 4,947 V + 1,050 kWh/dia |
| 33421067215b-b10f-4c5a-8b0c-f102d64cd63d | Translation Approved (100%) | na | n/d |
| 3343d392a00b-866e-403f-b48e-708d47938ff5 | Translation Approved (CM) | Glass-door reach-in refrigerator, 0.85 ≤ V < 1.42 m<5078>3</5078> | Refrigerador vertical com porta de vidro, 0,85 ≤ V < 1,42 m<5078>3</5078> |
| 3344c9183a75-44bf-4d11-b460-56eecb4b6bd9 | Translation Approved (CM) | Elec | Elét. |
| 3345278f2e09-93c1-44e4-a09d-a9ca39b092c0 | Translation Approved (CM) | Refrig | Refrig. |
| 3346af03eee0-862a-4472-a565-f06368353b84 | Translation Approved (CM) | 4.24V + 3.34 kWh/day | 4,24 V + 3,34 kWh/dia |
| 33479be94f18-cda4-4ef3-b26f-195b16b16d46 | Translation Approved (100%) | na | n/d |
| 3348a1147a28-5f9c-4116-8d30-7003e4e79c5c | Translation Approved (CM) | ≤ 3.109V + 2.625 kWh/day | ≤ 3,109 V + 2,625 kWh/dia |
| 334935dbe6c9-516d-453f-b3e4-7b94cec2ec09 | Translation Approved (100%) | na | n/d |
| 3350a3ee3346-e6a9-4240-976d-09824acdec72 | Translation Approved (91%) | Glass-door reach-in refrigerator, 1.42 ≤ V m<5100>3</5100> | Refrigerador vertical com porta de vidro, 1,42 ≤ V m<5100>3</5100> |
| 33513ac0c4e3-3b3d-4ec6-ae2e-edb29995cb7f | Translation Approved (100%) | Elec | Elét. |
| 3352d7dd300d-56be-4e3d-9a77-9d372754261e | Translation Approved (CM) | Refrig | Refrig. |
| 335385d74b7c-208a-4717-ba8d-1794062eea02 | Translation Approved (CM) | 4.24V + 3.34 kWh/day | 4,24 V + 3,34 kWh/dia |
| 3354d1f7cccc-dafd-404a-9770-cadbaa75fbf5 | Translation Approved (100%) | na | n/d |
| 335543bca9e7-482c-41ef-956b-e714959a3a1e | Translation Approved (CM) | ≤ 3.887V + 1.500 kWh/day | ≤ 3,887 V + 1,500 kWh/dia |
| 3356c8e4f0d3-5763-4138-92c0-2d54f04de1f0 | Translation Approved (100%) | na | n/d |
| 335750314642-0e78-4717-aade-ae9b259b6487 | Translation Approved (91%) | Solid-door reach-in freezer, 0 < V < 0.42 m<5122>3</5122> | Freezer vertical com porta sólida, 0 < V < 0,42 m<5122>3</5122> |
| 3358c04123d6-a6cb-4d39-ab1f-fc67a31c6316 | Translation Approved (100%) | Elec | Elét. |
| 3359271ad6d6-7bd0-4d71-a8d9-6ddbc0d9bccc | Translation Approved (CM) | Refrig | Refrig. |
| 3360abec9082-f7f2-4d0d-a7a5-f6241d25849c | Translation Approved (CM) | 14.13V + 1.38 kWh/day | 14,13 V + 1,38 kWh/dia |
| 3361186d303b-f413-4191-80d1-f1826e9594e7 | Translation Approved (100%) | na | n/d |
| 336237bbb6f6-bf29-4320-8461-4b230b97c573 | Translation Approved (CM) | ≤ 8.834V + 1.25 kWh/day | ≤ 8,834 V + 1,25 kWh/dia |
| 336355fe45be-3526-4fec-bd55-bae5ce16dbe8 | Translation Approved (100%) | na | n/d |
| 336432cf6620-b72a-40b4-a668-dc126b3b0bfb | Translation Approved (99%) | Solid-door reach-in freezer, 0.42 ≤ V < 0.85 m<5144>3</5144> | Freezer vertical com porta sólida, 0,42 ≤ V < 0,85 m<5144>3</5144> |
| 3365b510256c-a35c-41c1-8f1e-027b620a82f4 | Translation Approved (CM) | Elec | Elét. |
| 3366bc845b4f-a2f6-4449-b313-585554f1c4b9 | Translation Approved (CM) | Refrig | Refrig. |
| 3367979b7fae-19eb-455d-a347-472f18e0b787 | Translation Approved (CM) | 14.13V + 1.38 kWh/day | 14,13 V + 1,38 kWh/dia |
| 336810654efb-6f47-48bf-872b-9409387fc002 | Translation Approved (100%) | na | n/d |
| 3369234bcfc5-bb73-4bd4-b2b3-9a9d26a0920d | Translation Approved (CM) | ≤ 4.819V – 1.000 kWh/day | ≤ 4,819 V – 1,000 kWh/dia |
| 3370760f1c90-452e-4044-b5a9-949b425d418e | Translation Approved (100%) | na | n/d |
| 33711f0f401d-ec13-42c0-a237-d4e6a9b1d5e3 | Translation Approved (CM) | Solid-door reach-in freezer, 0.85 ≤ V < 1.42 m<5166>3</5166> | Freezer vertical com porta sólida, 0,85 ≤ V < 1,42 m<5166>3</5166> |
| 337281b4a881-b09b-480f-828e-5e3543b00fde | Translation Approved (CM) | Elec | Elét. |
| 3373eea723ee-e98e-49fe-b0a3-515644c951a9 | Translation Approved (CM) | Refrig | Refrig. |
| 3374283f7f0b-1457-4565-b774-a45fc6ef5ea0 | Translation Approved (CM) | 14.13V + 1.38 kWh/day | 14,13 V + 1,38 kWh/dia |
| 3375451fde6f-fd67-436e-bb07-642db8d8bca8 | Translation Approved (100%) | na | n/d |
| 3376ef7dd8a8-0429-433d-9771-82f0107eed8d | Translation Approved (CM) | ≤ 5.760V + 6.125 kWh/day | ≤ 5,760 V + 6,125 kWh/dia |
| 3377c6e10438-ddd9-4005-9205-a94004375ee3 | Translation Approved (100%) | na | n/d |
| 33787e840626-c51c-4395-8863-b5e9c57b6af3 | Translation Approved (91%) | Solid-door reach-in freezer, 1.42 ≤ V m<5188>3</5188> | Freezer vertical com porta sólida, 1,42 ≤ V m<5188>3</5188> |
| 3379f2bd197a-b75a-4dd0-aed9-f8fd3690ea7c | Translation Approved (100%) | Elec | Elét. |
| 3380b89edb56-fef3-47ab-b92b-4b555a510b78 | Translation Approved (CM) | Refrig | Refrig. |
| 3381f6bb9559-06ec-4b5b-924d-54921058af19 | Translation Approved (CM) | 14.13V + 1.38 kWh/day | 14,13 V + 1,38 kWh/dia |
| 3382422a9d78-5669-4bcd-a5f7-44ed37696d80 | Translation Approved (100%) | na | n/d |
| 3383654c5462-a9bc-47a3-b3f7-c89b2a60fc01 | Translation Approved (CM) | ≤ 5.583V + 6.333 kWh/day | ≤ 5,583 V + 6,333 kWh/dia |
| 3384288f34ce-513c-467a-8f2d-c7d38d601be7 | Translation Approved (100%) | na | n/d |
| 3385bcca53ec-2b6c-4398-ba04-99dd1b0fbfcd | Translation Approved (91%) | Solid-door reach-in refrigerator, 0 < V < 0.42m<5210>3</5210> | Refrigerador vertical com porta sólida, 0 < V < 0,42m<5210>3</5210> |
| 3386972d417f-ba08-4c41-a3c3-94d70bdec026 | Translation Approved (100%) | Elec | Elét. |
| 3387bc143a39-34b3-4af6-99db-cffef9f4be46 | Translation Approved (CM) | Refrig | Refrig. |
| 3388e3f7f76e-eee6-4409-ba9b-c5d36e87d311 | Translation Approved (CM) | 3.53V + 2.04 kWh/day | 3,53 V + 2,04 kWh/dia |
| 3389b50cb2b1-5fd6-4073-81d8-b442b3916f8a | Translation Approved (100%) | na | n/d |
| 33901bc81696-ac36-481d-a021-67656ec8caed | Translation Approved (CM) | ≤ 3.145V + 1.411 kWh/day | ≤ 3,145 V + 1,411 kWh/dia |
| 3391793bf708-a18b-4e66-879b-71fe7454232d | Translation Approved (100%) | na | n/d |
| 3392e014666f-77e9-4d6b-825d-667157b8049f | Translation Approved (99%) | Solid-door reach-in refrigerator, 0.42 ≤ V < 0.85 m<5232>3</5232> | Refrigerador vertical com porta sólida, 0,42 ≤ V < 0,85 m<5232>3</5232> |
| 339340819be4-645a-4e44-a7d3-5810eaae8c6f | Translation Approved (100%) | Elec | Elét. |
| 33947f6db115-e546-486c-9126-163399e2b7c6 | Translation Approved (CM) | Refrig | Refrig. |
| 3395fae21b37-97a9-4900-b551-22a2fce21d53 | Translation Approved (CM) | 3.53V + 2.04 kWh/day | 3,53 V + 2,04 kWh/dia |
| 33963c542781-eb30-411c-bc8a-fb40b3814521 | Translation Approved (100%) | na | n/d |
| 33974f339768-7e17-40e7-be35-043c28dadd3f | Translation Approved (CM) | ≤ 1.307V + 2.200 kWh/day | ≤ 1,307 V + 2,200 kWh/dia |
| 3398ecd862d9-db22-4c5a-934e-541acb1ea16e | Translation Approved (100%) | na | n/d |
| 339938bfeaf3-e895-4c8b-bbf2-de5dd7c4544c | Translation Approved (CM) | Solid-door reach-in refrigerator, 0.85 ≤ V < 1.42 m<5254>3</5254> | Refrigerador vertical com porta sólida, 0,85 ≤ V < 1,42 m<5254>3</5254> |
| 3400d7f2c254-fe35-499b-b6c9-514de1e7791f | Translation Approved (CM) | Elec | Elét. |
| 3401fe4e21bd-74f4-4840-8efb-817d73dbd72e | Translation Approved (CM) | Refrig | Refrig. |
| 34021b4bde4a-2804-42c1-b61e-670489adae86 | Translation Approved (CM) | 3.53V + 2.04 kWh/day | 3,53 V + 2,04 kWh/dia |
| 3403e3aba154-71b9-4b59-ae8d-1e7fba454787 | Translation Approved (100%) | na | n/d |
| 3404d75bb224-ab56-4b86-a01c-a912dd66cbb4 | Translation Approved (CM) | ≤ 1.979V + 1.635 kWh/day | ≤ 1,979 V + 1,635 kWh/dia |
| 3405640c9b9e-cd73-4f4c-bfe5-6e46f7a0dd9d | Translation Approved (100%) | na | n/d |
| 34064adf88bb-e821-4db5-a8a0-cab850332f78 | Translation Approved (91%) | Solid-door reach-in refrigerator, 1.42 ≤ V m<5276>3</5276> | Refrigerador vertical com porta sólida, 1,42 ≤ V m<5276>3</5276> |
| 34073a9d91c9-fcc7-4bf2-bdc6-4b957148bbfe | Translation Approved (100%) | Elec | Elét. |
| 34081b79b583-5d23-4fc3-8e24-99362e01a405 | Translation Approved (CM) | Refrig | Refrig. |
| 34092c0afd9c-5e13-40b6-a8e0-c4660987c915 | Translation Approved (100%) | 3.53V + 2.04 kWh/day | 3,53 V + 2,04 kWh/dia |
| 341016901646-45ef-4e04-a849-2e789a458298 | Translation Approved (100%) | na | n/d |
| 3411ace7e86f-26dd-4146-9f4a-57fc8daa7edb | Translation Approved (CM) | ≤ 2.120V + 1.416 kWh/day | ≤ 2,120 V + 1,416 kWh/dia |
| 341299960192-dc21-4f80-86a7-ef32846a5f79 | Translation Approved (100%) | na | n/d |
| 341344ec735f-d547-41ee-8f2a-7b2f92628c74 | Translation Approved (100%) | Clothes washer | Máquina de lavar roupa |
| 3414d6032d25-da95-4189-93fb-d6b2c5b20b6f | Translation Approved (100%) | Gas | Gás |
| 3415d039bfec-c414-4c6b-a795-c42b1ddabc0a | Translation Approved (100%) | Sanitation | Saneamento |
| 3416aa90a33a-e8ae-47b6-960b-f7f0eac073a6 | Translation Approved (100%) | 1.72 MEF | 1,72 MEF |
| 3417616197e5-3686-4d61-9533-1aeddaf44bdb | Translation Approved (100%) | 2.00 MEF | 2,00 MEF |
| 3418c1c5335d-54c0-4e19-8df0-7e268d86d400 | Translation Approved (100%) | Door-type dish machine, high temp | Lava-louças tipo porta, alta temp. |
| 341975081d46-084a-42b2-b520-b9df10433745 | Translation Approved (100%) | Elec | Elét. |
| 34205ce7959b-8f1f-42f7-8d78-bf971dae5e79 | Translation Approved (100%) | Sanitation | Saneamento |
| 3421e0337756-e3d1-40c8-b88f-1e85c35f1897 | Translation Approved (100%) | na | n/d |
| 34227261f63e-5aae-492b-a961-c9fb29fa90b3 | Translation Approved (CM) | 1.0 kW | 1,0 kW |
| 34231f58a45b-aafb-4dfa-a1e5-162a37f50a42 | Translation Approved (100%) | na | n/d |
| 3424462f534c-61b4-4ff5-9f96-063cd99feed7 | Translation Approved (CM) | 0.70 kW | 0,70 kW |
| 34255034feec-0b03-406f-a938-69b2b6490c07 | Translation Approved (100%) | Door-type dish machine, low temp | Lava-louças tipo porta, baixa temp. |
| 34267ce8fbb8-7681-47ab-8528-e5faa2f904e3 | Translation Approved (100%) | Elec | Elét. |
| 34273b599aa8-632c-4e1c-9e0b-fdb6abda61a2 | Translation Approved (100%) | Sanitation | Saneamento |
| 3428e8c67f16-2225-42a3-bc0b-dc85a8ca8f16 | Translation Approved (100%) | na | n/d |
| 34296eabb249-729b-41e8-9ef5-318e950edf83 | Translation Approved (CM) | 0.6 kW | 0,6 kW |
| 34309acc99aa-5046-4f40-b378-eb2d1f270934 | Translation Approved (100%) | na | n/d |
| 3431c640cfc5-f918-4f08-8945-647d2deeccdc | Translation Approved (CM) | 0.6 kW | 0,6 kW |
| 34324323175b-641d-42df-a9e3-165b84137e25 | Translation Approved (100%) | Multitank rack conveyor dish machine, high temp | Lava-louças industrial multitanque com transportador, alta temp. |
| 3433026ba30c-3cc6-4428-9937-d2bb452a71f8 | Translation Approved (CM) | Elec | Elét. |
| 3434e4c44888-7423-4ee1-ae04-b3e86e741b87 | Translation Approved (100%) | Sanitation | Saneamento |
| 3435a2d2cbc8-1000-4f50-99d4-df0d740d7b8c | Translation Approved (100%) | na | n/d |
| 3436ce3e0fac-a998-4694-a792-8d72d38ae7f5 | Translation Approved (CM) | 2.6 kW | 2,6 kW |
| 34372de7dab5-5cac-48e5-acbb-fdf9895240ea | Translation Approved (100%) | na | n/d |
| 34383bfa1f85-fb20-4331-928b-6f857100c998 | Translation Approved (CM) | 2.25 kW | 2,25 kW |
| 3439ba7bcafd-092c-42f5-814b-5d2aed8e1522 | Translation Approved (100%) | Multitank rack conveyor dish machine, low temp | Lava-louças industrial multitanque com transportador, baixa temp. |
| 34407134b097-d1ee-4145-adbf-f9ac3cf1eb7f | Translation Approved (CM) | Elec | Elét. |
| 3441d6885a1f-79e9-47e3-9b94-07ad9805cd10 | Translation Approved (100%) | Sanitation | Saneamento |
| 344278e17423-9dc7-4197-9864-f1de21edcf21 | Translation Approved (100%) | na | n/d |
| 3443039f9209-9103-4711-a713-f5d05bbf14c8 | Translation Approved (CM) | 2.0 kW | 2,0 kW |
| 344446a24805-4a49-486b-a46e-530c3cc775ce | Translation Approved (100%) | na | n/d |
| 34459ba3bce0-e588-4ddb-8a54-2686a0db7a93 | Translation Approved (CM) | 2.0 kW | 2,0 kW |
| 34469917c2f5-eb2a-492e-9571-ee2f67dfb51c | Translation Approved (100%) | Single-tank rack conveyor dish machine, high temp | Lava-louças industrial tanque único com transportador, alta temp. |
| 34476cd22353-54e7-4f62-aec2-b5f76208ea20 | Translation Approved (CM) | Elec | Elét. |
| 344894e8bcbc-963e-40c2-a00f-c3c6f0be577f | Translation Approved (100%) | Sanitation | Saneamento |
| 344905b25a9c-6b95-4dfe-8227-0dece5ab35d8 | Translation Approved (100%) | na | n/d |
| 34507d3e02c7-572d-4e71-abdc-602c07630563 | Translation Approved (CM) | 2.0 kW | 2,0 kW |
| 3451e0c985c9-e3e2-453c-bc8d-cb988a8889b1 | Translation Approved (100%) | na | n/d |
| 3452ef24150f-3a7f-45e3-8192-1c1fa5f0fec3 | Translation Approved (CM) | 1.5 kW | 1,5 kW |
| 345355e0a4da-b43e-41a6-9204-de0e0d784a9f | Translation Approved (100%) | Single-tank rack conveyor dish machine, low temp | Lava-louças industrial tanque único com transportador, baixa temp. |
| 34543c02463c-efdd-4f24-9676-c13984853715 | Translation Approved (CM) | Elec | Elét. |
| 34556df9b467-2301-4ed4-8d91-ad22c2c518af | Translation Approved (100%) | Sanitation | Saneamento |
| 34568ad8b65a-483d-492c-b7a9-d81f0153f0a6 | Translation Approved (100%) | na | n/d |
| 3457811f7a01-71a0-4850-8e5a-9d53f0a45946 | Translation Approved (CM) | 1.6 kW | 1,6 kW |
| 345888b3e125-9d17-4e7b-8417-06bc3c511728 | Translation Approved (100%) | na | n/d |
| 34598ec89398-788b-4b67-94d1-64bfc04dd07a | Translation Approved (CM) | 1.5 kW | 1,5 kW |
| 3460d51555df-60de-4ccb-8fe6-eb3ddac361bd | Translation Approved (100%) | Undercounter dish machine, high temp | Lava-louças sob bancada, alta temp. |
| 3461a30e302e-1c23-4dba-8e74-e9e06b21caa0 | Translation Approved (CM) | Elec | Elét. |
| 3462a534ce9e-2e29-4ee9-b561-6a02a6188bf8 | Translation Approved (100%) | Sanitation | Saneamento |
| 34637882c0f2-3aca-4de2-9790-93db128f22f4 | Translation Approved (100%) | na | n/d |
| 346421c56240-ac4e-4137-85ea-119d1203cfac | Translation Approved (CM) | 0.9 kW | 0,9 kW |
| 34659d42d99b-6ef2-4e5d-9cc8-c665200c197d | Translation Approved (100%) | na | n/d |
| 3466d08cccde-2dd6-41a4-b264-ff305cf7e07c | Translation Approved (CM) | 0.5 kW | 0,5 kW |
| 346744d943ef-4f4c-4f89-bd75-176bf5cd670b | Translation Approved (100%) | Undercounter dish machine, low temp | Lava-louças sob bancada, baixa temp. |
| 3468faa94dc6-4cf8-4085-a054-5918574ac2a9 | Translation Approved (CM) | Elec | Elét. |
| 3469888ecf41-db23-4b6d-a995-b2e51a47f3ea | Translation Approved (100%) | Sanitation | Saneamento |
| 34707268e9c9-28e6-48a1-ac41-7ed30a3548ff | Translation Approved (100%) | na | n/d |
| 3471b914af7b-c438-403f-9183-b8947224f049 | Translation Approved (CM) | 0.5 kW | 0,5 kW |
| 3472288e9bfc-58e9-4709-9fea-568cb61c7e42 | Translation Approved (100%) | na | n/d |
| 34737f5e2fb8-3a0c-47da-b300-28a04a2e72c8 | Translation Approved (CM) | 0.5 kW | 0,5 kW |
| 3474c6d6c090-9704-49f1-9ce5-950c306088ae | Translation Approved (100%) | The energy efficiency, idle energy rates, and water use requirements, where applicable, are based on the following test methods: | Os requisitos de eficiência energética, taxas de energia em inatividade e uso de água, quando aplicáveis, se baseiam nos seguintes métodos de teste: |
| 347535b5b2f4-5773-4509-9d78-2533905e776f | Translation Approved (100%) | ASTM F1275 Standard Test Method for Performance of Griddles | ASTM F1275 – Método de teste padrão de desempenho de frigideiras (Standard Test Method for Performance of Griddles) |
| 3476ff6f3fa7-c06a-47b1-b700-4949aa1d6c69 | Translation Approved (100%) | ASTM F1361 Standard Test Method for Performance of Open Deep Fat Fryers | ASTM F1361 – Método de teste padrão de desempenho de fritadeiras de imersão (Standard Test Method for Performance of Open Deep Fat Fryers) |
| 3477bed91591-364f-4ce4-bc9c-03c4d96e1cc6 | Translation Approved (100%) | ASTM F1484 Standard Test Methods for Performance of Steam Cookers | ASTM F1484 – Métodos de teste padrão de desempenho de panelas a vapor (Standard Test Methods for Performance of Steam Cookers) |
| 34783723c9ec-2742-423b-a2c4-4e4dc28941f0 | Translation Approved (100%) | ASTM F1496 Standard Test Method for Performance of Convection Ovens | ASTM F1496 – Método de teste padrão de desempenho de fornos de convecção (Standard Test Method for Performance of Convection Ovens) |
| 3479b4d5acc0-1cbe-40bd-b3e2-0f05734112a9 | Translation Approved (100%) | ASTM F1521 Standard Test Methods for Performance of Range Tops | ASTM F1521 – Métodos de teste padrão de desempenho de "cooktops" (Standard Test Methods for Performance of Range Tops) |
| 34802ca4e3d1-4aae-4b16-b647-9eb22629e816 | Translation Approved (100%) | ASTM F1605 Standard Test Method for Performance of Double-Sided Griddles | ASTM F1605 – Método de teste padrão de desempenho de frigideiras de dois lados (Standard Test Method for Performance of Double-Sided Griddles) |
| 3481d4aa4e40-8360-4f62-a20d-6c50119b7b47 | Translation Approved (100%) | ASTM F1639 Standard Test Method for Performance of Combination Ovens | ASTM F1639 – Método de teste padrão de desempenho de fornos de combinação (Standard Test Method for Performance of Combination Ovens) |
| 348288fbe478-8f4d-4f41-92a7-b5f6a2dfaa74 | Translation Approved (100%) | ASTM F1695 Standard Test Method for Performance of Underfired Broilers | ASTM F1695 – Método de teste padrão de desempenho de grelhas aquecidas por baixo (Standard Test Method for Performance of Underfired Broilers) |
| 3483cf5fd030-6044-4731-a04a-916f28403b4d | Translation Approved (100%) | ASTM F1696 Standard Test Method for Energy Performance of Single-Rack Hot Water Sanitizing, ASTM Door-Type Commercial Dishwashing Machines | ASTM F1696 – Método de teste padrão de desempenho energético de lava-louças comerciais tipo porta ASTM, limpeza com água quente e prateleira única (Standard Test Method for Energy Performance of Single-Rack Hot Water Sanitizing, ASTM Door-Type Commercial Dishwashing Machines) |
| 348403cb11b7-7059-4fc2-a5bf-bb21024a31f4 | Translation Approved (100%) | ASTM F1704 Standard Test Method for Capture and Containment Performance of Commercial Kitchen Exhaust Ventilation Systems | ASTM F1704 – Método de teste padrão de desempenho de captura e contenção de sistemas de ventilação de exaustão de cozinhas (Standard Test Method for Capture and Containment Performance of Commercial Kitchen Exhaust Ventilation Systems) |
| 348545c97595-e2a0-4a0a-b736-4fe18d33859d | Translation Approved (100%) | ASTM F1817 Standard Test Method for Performance of Conveyor Ovens | ASTM F1817 – Método de teste padrão de desempenho de fornos com transportador (Standard Test Method for Performance of Conveyor Ovens) |
| 3486aee1b8ea-87c2-474e-81b0-cba562ca6c3f | Translation Approved (100%) | ASTM F1920 Standard Test Method for Energy Performance of Rack Conveyor, Hot Water Sanitizing, Commercial Dishwashing Machines | ASTM F1920 – Método de teste padrão de desempenho energético de lava-louças comerciais, limpeza com água quente, com transportador e prateleiras (Standard Test Method for Energy Performance of Rack Conveyor, Hot Water Sanitizing, Commercial Dishwashing Machines) |
| 348700b676dd-feb9-470e-bc9e-9c6fa1cb6802 | Translation Approved (100%) | ASTM F2093 Standard Test Method for Performance of Rack Ovens | ASTM F2093 – Método de teste padrão para o desempenho de fornos rotativos (Standard Test Method for Performance of Rack Ovens) |
| 34888e7e218b-041f-4a71-a5b6-1182f48ced1c | Translation Approved (100%) | ASTM F2140 Standard Test Method for Performance of Hot Food Holding Cabinets | ASTM F2140 – Método de teste padrão de desempenho de armários de armazenamento de alimentos quentes (Standard Test Method for Performance of Hot Food Holding Cabinets) |
| 3489cc202f07-e952-4bb0-863b-59a81b75317b | Translation Approved (100%) | ASTM F2144 Standard Test Method for Performance of Large Open Vat Fryers | ASTM F2144 – Método de teste padrão de desempenho de fritadeiras grandes de tanque aberto (Standard Test Method for Performance of Large Open Vat Fryers) |
| 34907a60a693-0ef4-4626-bc7f-f1d57f4b7f6d | Translation Approved (100%) | ASTM F2324 Standard Test Method for Prerinse Spray Valves | ASTM F2324 – Método de teste padrão de válvulas de pulverização pré-lavagem (Standard Test Method for Prerinse Spray Valves) |
| 34919c4a4c0d-5811-49ac-a4ff-5c3298984af9 | Translation Approved (100%) | ASTM F2380 Standard Test Method for Performance of Conveyor Toasters | ASTM F2380 – Método de teste padrão de desempenho de torradeiras com transportador (Standard Test Method for Performance of Conveyor Toasters) |
| 3492bc72afcb-113f-4b9c-9234-a025d07dbbf7 | Translation Approved (100%) | ARI 810-2007: Performance Rating of Automatic Commercial Ice Makers | ARI 810-2007: Avaliação de desempenho de máquinas de gelo comerciais automáticas (Performance Rating of Automatic Commercial Ice Makers) |
| 34939018a642-949b-4d29-adcd-fb93fa85711c | Translation Approved (100%) | ANSI/ASHRAE Standard 72–2005: Method of Testing Commercial Refrigerators and Freezers with temperature setpoints at 38°F (3°C) for mediumtemp refrigerators, -18°C for low-temp freezers, and -26°C for ice cream freezers. | Norma ANSI/ASHRAE 72–2005: Método de teste de refrigeradores e freezers comerciais com pontos de ajuste de temperatura a 38°F (3°C) para refrigeradores de temperatura média, -18°C (-18°C) para freezers de baixa temperatura e -26°C (-26°C) para freezers para sorvete (Method of Testing Commercial Refrigerators and Freezers with temperature setpoints at 38°F for medium-temp refrigerators, 0°F for low-temp freezers, and -15°F for ice cream freezers). |
| 3494d6ba3289-53fb-489e-ac88-2f72ff050a95 | Translation Approved (100%) | Table 2. | Tabela 2. |
| 3495d6ba3289-53fb-489e-ac88-2f72ff050a95 | Translation Approved (CM) | Supermarket refrigeration prescriptive measures and baseline for energy cost budget | Medidas prescritivas e baseline de refrigeração de supermercados para orçamento do custo de energia |
| 34962fcbeac1-7fd3-45ba-92ab-eaeec993eef2 | Translation Approved (100%) | Item | Item |
| 34976d090dc6-2891-4113-bf6a-61185418a4ec | Translation Approved (0%) | Attribute | Atributo |
| 34981f15d30b-63fc-48f3-a103-9698ad4c9f8f | Translation Approved (0%) | Prescriptive measure | Medida prescritiva |
| 3499b2414478-70a1-4a8f-9505-4e5441280cfd | Translation Approved (CM) | Baseline for energy modeling path | Baseline do caminho de modelagem de energia |
| 35009b0e2026-f685-4b1a-b205-1f87b431fa25 | Translation Approved (0%) | Commercial Refrigerator and Freezers | Refrigeradores e freezers comerciais |
| 350105aa0c17-a948-4ac0-8a24-359c681de7c5 | Translation Approved (77%) | Energy Use Limits | Limites de uso de energia |
| 350201629d51-7255-4874-a1f1-e4a01e17f8ff | Translation Approved (0%) | ASHRAE 90.1-2010 Addendum g. Table 6.8.1L | ASHRAE 90.1-2010 Adendo g. Tabela 6.8.1L |
| 3503fd40e2d1-d2cb-4bd1-80f0-bac816a18364 | Translation Approved (100%) | ASHRAE 90.1-2010 Addendum g. Table 6.8.1L | ASHRAE 90.1-2010 Adendo g. Tabela 6.8.1L |
| 350464df9779-a124-4e2e-a83c-31b9d5627d9c | Translation Approved (73%) | Commercial Refrigeration Equipment | Equipamentos de refrigeração comercial |
| 3505c92dc523-4d16-4205-8c58-1c628fd5c78f | Translation Approved (100%) | Energy Use Limits | Limites de uso de energia |
| 35068b016ad3-f1e0-4285-9764-9b6d07b2f7b7 | Translation Approved (96%) | ASHRAE 90.1-2010 Addendum g. Table 6.8.1M | ASHRAE 90.1-2010 Adendo g. Tabela 6.8.1M |
| 35078c4fe1a6-1f05-4e52-9da9-625b8dc7c8d1 | Translation Approved (100%) | ASHRAE 90.1-2010 Addendum g. Table 6.8.1M | ASHRAE 90.1-2010 Adendo g. Tabela 6.8.1M |
| 3508201b8b82-d202-4dbc-a9e3-768a96065c33 | Translation Approved (100%) | Table 3. | Tabela 3. |
| 3509201b8b82-d202-4dbc-a9e3-768a96065c33 | Translation Approved (100%) | Walk-in coolers and freezers prescriptive measures and baseline for energy cost budget | Medidas prescritivas e baseline de refrigeradores e freezers tipo câmara fria ("walk-in") para orçamento do custo de energia |
| 351081670d19-5d5b-4b55-a5ff-845b4f79f110 | Translation Approved (100%) | Item | Item |
| 3511a9e8524d-c43b-40a6-8344-5d68cb6f596b | Translation Approved (CM) | Attribute | Atributo |
| 3512c2fd4d25-3b34-478c-a470-c65e7e70c49b | Translation Approved (CM) | Prescriptive measure | Medida prescritiva |
| 351340966fdc-248f-4f13-ae5e-c122205514bf | Translation Approved (100%) | Baseline for energy modeling path | Baseline do caminho de modelagem de energia |
| 351480e12275-33ec-4519-b523-e302e94fefdf | Translation Approved (0%) | Envelope | Envelope |
| 3515178ca5a3-5d66-4b09-9761-511ecfa606b5 | Translation Approved (CM) | Freezer insulation | Isolamento do freezer |
| 3516d4832e0a-1785-48ba-a902-aa8ab6ddfb80 | Translation Approved (0%) | R-46 | R-46 |
| 35175399a595-8d53-406f-8cd2-49ca7b4fbb02 | Translation Approved (0%) | R-36 | R-36 |
| 35188487d8f7-4a06-45c4-a17a-c6387fe2d93a | Translation Approved (CM) | Cooler insulation | Isolamento do refrigerador |
| 351920598573-3f2a-442f-98dc-44597e5b3a02 | Translation Approved (100%) | R-36 | R-36 |
| 3520702b2752-0673-4551-bdad-1f32238204e2 | Translation Approved (0%) | R-20 | R-20 |
| 3521778de496-bbc6-4fe5-a1e9-e41714195678 | Translation Approved (0%) | Automatic closer doors | Portas de fechamento automático |
| 3522340d2d6b-1d3b-411c-8e71-27e7ca5d7b8f | Translation Approved (100%) | Yes | Sim |
| 3523dce35f8e-0e67-46f6-a00e-2cae50244334 | Translation Approved (100%) | No | Não |
| 35248e07651e-45f2-4ba0-95cc-d0dd298f9bd9 | Translation Approved (CM) | High-efficiency low- or no-heat reach-in doors | Portas verticais de alta eficiência, com baixo ou sem aquecimento |
| 352589ac9ca1-3370-4fb2-871c-77dc246f5626 | Translation Approved (CM) | 40W/ft (130W/m) of door frame (low temperature), 17W/ft (55W/m) of door frame (medium temperature) | 40 W/ft (130 W/m) da estrutura da porta (baixa temperatura), 17 W/ft (55 W/m) da estrutura da porta (temperatura média) |
| 35265056203e-b418-4b4d-a1c9-d41c41667252 | Translation Approved (100%) | 40W/ft (130W/m) of door frame (low temperature), 17W/ft (55W/m) of door frame (medium temperature) | 40 W/ft (130 W/m) da estrutura da porta (baixa temperatura), 17 W/ft (55 W/m) da estrutura da porta (temperatura média) |
| 35272c9de1a3-ccfb-4b81-9f7f-6a463f6f0417 | Translation Approved (0%) | Evaporator | Evaporador |
| 35282604451d-be58-4825-b748-238eea9eb65f | Translation Approved (CM) | Evaporator fan motor and control | Motor e controle do ventilador do evaporador |
| 3529cebca8a8-37eb-421b-9858-690f26ded78a | Translation Approved (CM) | Shaded pole and split phase motors prohibited; use PSC or EMC motors | Motores de polo sombreados ou fase dividida são proibidos, use motores PSC ou EMC |
| 3530cc84f86a-c027-4201-9d27-532f16f5d6cd | Translation Approved (0%) | Constant-speed fan | Ventilador de velocidade constante |
| 3531ba7d305c-d5f6-444f-b80f-8ab84a8234dc | Translation Approved (CM) | Hot gas defrost | Descongelamento com gás quente |
| 3532ad43966d-d774-4bfe-8c1e-277b141005cf | Translation Approved (CM) | No electric defrosting. | Sem descongelamento elétrico |
| 3533005b838d-9d45-4ce3-b7b4-e8495dfaa7f2 | Translation Approved (CM) | Electric defrosting | Com descongelamento elétrico |
| 35344b3347b5-140a-4810-9a56-e8c34c92722c | Translation Approved (0%) | Condenser | Condensador |
| 353526be0fb6-45bf-46ec-bd72-31c80e417b0d | Translation Approved (CM) | Air-cooled condenser fan motor and control | Motor e controle do ventilador do condensador resfriado a ar |
| 3536bd60961d-099d-4c27-beef-f6dc51dd7b18 | Translation Approved (CM) | Shaded pole and split phase motors prohibited; use PSC or EMC motors; add condenser fan controllers | Motores de polo sombreados ou fase dividida são proibidos, use motores PSC ou EMC; adicione controladores do ventilador do condensador |
| 35374c4d7daa-70d3-4b93-a948-2959781c2110 | Translation Approved (CM) | Cycling one-speed fan | Ventilador de ciclo de uma velocidade |
| 35388351d731-c268-4c7b-80e1-0634c36dfbf2 | Translation Approved (CM) | Air Cooled condenser design approach | Abordagem de projeto de condensador resfriado a ar |
| 3539363b423d-8547-4951-a0be-6a381c85abdf | Translation Approved (CM) | Floating head pressure controls or ambient subcooling | Controles de pressão de cabeçote flutuante ou sub-resfriamento do ambiente |
| 35401bc2e302-5db1-4cc7-ac5a-a5f1dc6d209e | Translation Approved (0%) | 10°F (-12°C) to 15°F | 10°F (-12°C) a 15°F |
| 3541f4172aed-c97e-4d5e-b8c5-65aae905ca9a | Translation Approved (CM) | (-9°C) dependent on suction temperature | (-9 °C) dependendo da temperatura de sucção |
| 35422069fdbc-9451-4449-afcd-76faa1231b9e | Translation Approved (100%) | Lighting | Iluminação |
| 35438bf9aaad-e22b-476e-a8a6-dee855afb8f5 | Translation Approved (CM) | Lighting power density (W/sq.ft.) | Densidade da potência de iluminação (W/pés quad.) |
| 354467d3f068-b68b-4dd7-a430-640fed9e68b3 | Translation Approved (0%) | 0.6 W/sq.ft. | 0,6 W/pés quad. |
| 3545d0a0eeea-a327-4eca-9265-5a25ac9cf6c6 | Translation Approved (0%) | (6.5 W/sq. meter) | (6,5 W/metros quad.) |
| 35461dde4600-e861-4f29-8afb-9dded9209619 | Translation Approved (100%) | 0.6 W/sq.ft. | 0,6 W/pés quad. |
| 3547ee6557ff-8f5c-4011-a7e6-468c4c710274 | Translation Approved (100%) | (6.5 W/sq. meter) | (6,5 W/metros quad.) |
| 3548f94b524d-4ad9-4dcb-a519-f6a72da6a08b | Translation Approved (100%) | Commercial Refrigerator and Freezers | Refrigeradores e freezers comerciais |
| 35494b66cd17-81d4-4753-84e2-1a95f63ce9d4 | Translation Approved (100%) | Energy Use Limits | Limites de uso de energia |
| 3550c6743464-023d-40a7-b50f-b62c9ce291dc | Translation Approved (100%) | N/A | N/A |
| 355178701dbe-dcd4-4221-9ee4-79fdc19c1bc8 | Translation Approved (CM) | Use an Exceptional Calculation Method if attempting to take savings | Use um Método de Cálculo Excepcional ao tentar obter economias |
| 35526b4db54d-5e52-43eb-bac9-7dd489f89aa8 | Translation Approved (100%) | Commercial Refrigerator and Freezers | Refrigeradores e freezers comerciais |
| 3553ac441e2b-a5ff-461c-9074-82a71635bfdb | Translation Approved (100%) | Energy Use Limits | Limites de uso de energia |
| 35544c48bd75-c01c-4765-b8bc-c8ce51cf5606 | Translation Approved (100%) | N/A | N/A |
| 35553430e177-69ca-4ed6-84a8-48327d04af80 | Translation Approved (100%) | Use an Exceptional Calculation Method if attempting to take savings | Use um Método de Cálculo Excepcional ao tentar obter economias |
| 3556941b7207-faa1-485c-b5fc-650104c6f95a | Translation Approved (100%) | Table 4. | Tabela 4. |
| 3557941b7207-faa1-485c-b5fc-650104c6f95a | Translation Approved (CM) | Commercial kitchen ventilation prescriptive measures and baseline for energy cost budget | Medidas prescritivas e baseline de ventilação comercial de cozinhas para orçamento de custo de energia |
| 35589a178d9d-ee25-45b4-85fc-8d0cbc673f04 | Translation Approved (100%) | Strategies | Estratégias |
| 3559313b99ea-1287-4559-892b-a30744434221 | Translation Approved (CM) | Prescriptive measure | Medida prescritiva |
| 3560adc6b4a3-a376-4ae0-b8bc-494980e1fc72 | Translation Approved (0%) | Baseline | Baseline |
| 3561b8462a3b-d092-4466-9e8e-ab28e0c873cd | Translation Approved (CM) | Kitchen hood control | Controle de coifa de cozinha |
| 3562acc80161-ac46-4c3b-9748-f2575945f855 | Translation Approved (CM) | ASHRAE 90.1-2010 Section 6.5.7.1, except that Section 6.5.7.1.3 and Section 6.5.7.1.4 shall apply if the total kitchen exhaust airflow rate exceeds 2,000 cfm (960 L/s) (as opposed to 5,000 cfm (2,400 L/s) noted in the ASHRAE 90.1-2010 requirements) | ASHRAE 90.1-2010 Seção 6.5.7.1, exceto que Seção 6.5.7.1.3 e Seção 6.5.7.1.4 devem se aplicar se a vazão de ar total da exaustão da cozinha exceder os 2.000 cfm (960 l/s) (contra 5.000 cfm (2.400 l/s) indicados nos requisitos na Norma ASHRAE 90.1-2010) |
| 356386e5a3b0-246f-4242-a15d-fbf483574ab7 | Translation Approved (100%) | ASHRAE 90.1-2010 Section 6.5.7.1 and Section G3.1.1 Exception (d) where applicable | ASHRAE 90.1-2010 Seção 6.5.7.1 e Seção G3.1.1 Exceção (d) quando aplicável |