| Segment ID | Segment status | Source segment | Target segment |
| --- | --- | --- | --- |
| 1e89f9a90-d0bd-431e-bcd4-db8295de06bc | Translated (0%) | Updated to reflect the July 1, 2014 LEED v4 Interior Design and Construction Addenda | 经过更新以反映 2014 年 7 月 1 日的 LEED v4 室内设计与施工附录 |
| 20c02298c-5095-474a-8a9c-dc02b3a7fc79 | Translated (0%) | <24>LEED </24><25>v<26>4</26> for</25> | <24>LEED </24><25>v<26>4</26> </25> |
| 367d81c0c-c5b1-4ac7-b1ec-a9d1c7878056 | Translated (70%) | INTERIOR DESIGN AND CONSTRUCTION | 室内设计与施工 |
| 4d255af64-fb79-4d27-80d7-c33b5fe72b74 | Translated (0%) | Updated July 1, 2014 | 更新于 2014 年 7 月 1 日 |
| 5106999fc-3912-4634-a8b9-e71e38969eb1 | Translated (0%) | Includes: | 包括： |
| 6660bb064-7ec8-40dd-baff-801cc3f357d1 | Translated (70%) | LEED ID+C: Commercial Interiors | LEED ID+C: 商业室内(LEED ID+C: Commercial Interiors) |
| 74703fffa-6bda-444a-a8dd-8f7913aca600 | Translated (80%) | LEED ID+C: Retail | LEED ID+C: 零售(LEED ID+C: Retail) |
| 804c2c6c7-2756-48d4-a29b-f06dd9046fb6 | Translated (80%) | LEED ID+C: Hospitality | LEED ID+C: 宾馆接待(LEED ID+C: Hospitality) |
| 92515d98f-73dc-4e59-a7b3-2bcad95210ed | Translated (0%) | Credit: Integrative Process | 得分点：整合过程 (Integrative Process) |
| 10063ba532-e66c-42df-a3b9-802f4c545521 | Translated (0%) | ID&C | ID&C |
| 114f4c87f6-d2c4-42b5-891c-3b33fb6f5a58 | Translated (0%) | 2 points | 2 分 |
| 128b277f8c-2bc9-4c51-8583-5b3966cd7912 | Translated (0%) | This credit applies to | 该得分点适用于 |
| 137c92d1fa-6a8c-4768-8975-1653dc9d9fde | Translated (100%) | Commercial Interiors | 商业室内 |
| 145eba9b05-5c4b-428d-b3cb-6f052066d7e2 | Translated (100%) | Retail | 零售 |
| 15eb6df80b-c3df-41ec-bced-ae6f67c99c8e | Translated (100%) | Hospitality | 宾馆接待 |
| 1633af3c31-412f-42b0-8bc2-f6f0ee070d7f | Translated (100%) | Intent | 目的 |
| 17d3cb33bb-2ba3-4264-b0de-dd8bc88c05c5 | Translated (0%) | To support high-performance, cost-effective project outcomes through an early analysis of the interrelationships among systems. | 通过对系统间的相互关系进行早期分析以支持实现高性能、高经济效益的项目成果。 |
| 18a0d37ce2-183f-480b-99e3-846bb52f24a6 | Translated (100%) | Requirements | 要求 |
| 19eb648c5e-4884-4be3-ad5b-dbefc7e6f51e | Translated (0%) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 207bc11a81-ad95-4b4c-a937-eed8d161344b | Translated (0%) | Site Selection and Energy-Related Systems (1 point) | 选址与能源相关系统（1 分） |
| 21af5bd039-3dde-460a-9acd-f6a3bb2655e0 | Translated (0%) | Starting in predesign and continuing throughout the design phases, identify and use opportunities to achieve synergies across disciplines and building systems. | 从设计预案直到设计阶段，识别和利用在不同学科和建筑系统之间实现协同效应的机会。 |
| 22af5bd039-3dde-460a-9acd-f6a3bb2655e0 | Translated (0%) | Use the analyses described below to inform the owner’s project requirements (OPR), basis of design (BOD), design documents, and construction documents. | 使用以下分析来为业主项目要求 (OPR)、设计基础要求 (BOD)、设计文件和施工文件提供信息。 |
| 23af5bd039-3dde-460a-9acd-f6a3bb2655e0 | Translated (71%) | Conduct analyses in site selection and energy-related systems (1 point). | 在选址与能源相关系统中进行分析（1 分）。 |
| 247725bdd0-4057-4d04-8608-8ece8fb282d2 | Translated (98%) | Site Selection: | 选址： |
| 2557121c76-4a3c-42f4-b7d9-85894baca9fb | Translated (0%) | <41>Discovery</41>: Before site selection, analyze project goals to identify and select the building site that will provide the most opportunities and fewest barriers for the tenant improvement project. | <Bold>探索</Bold>：在选址之前，分析项目目标以确定和选择能够为租户装修项目提供最多机会和最少障碍的建筑场址。 |
| 2657121c76-4a3c-42f4-b7d9-85894baca9fb | Translated (0%) | Assess at least two potential locations or base building options, taking into consideration at least the following: | 评估至少 2 个潜在的地点或基本建筑方案，至少考虑以下方面： |
| 27e2f5d1a3-85cf-4d28-92f1-93426297a3e2 | Translated (0%) | Building site attributes. | 建筑场址属性。 |
| 28e2f5d1a3-85cf-4d28-92f1-93426297a3e2 | Translated (0%) | Assess the base building’s location and site design characteristics. | 评估基本建筑的地点和场址设计特点。 |
| 2956979cc4-de28-46d2-a8d1-120795913414 | Translated (96%) | Transportation. | 交通。 |
| 3056979cc4-de28-46d2-a8d1-120795913414 | Translated (0%) | Assess the tenant occupants’ transportation needs for commuting to and from the site, including convenient access to alternative transportation that meets occupants’ needs. | 评估租户驻户对往返场址的通勤需求，包括能否方便地使用符合驻户需求的替代交通方式。 |
| 312c7149b0-40fa-4aff-b206-61035e5c296d | Translated (0%) | Building features. | 建筑特色。 |
| 322c7149b0-40fa-4aff-b206-61035e5c296d | Translated (0%) | Assess the base building’s envelope, mechanical and electrical systems that will affect tenant space (e.g., controls, HVAC, plumbing fixtures, renewable energy supply), adaptability to future needs, and resilience in the event of disaster or infrastructure failure. | 评估基本建筑的外围护结构、影响承租空间的机械和电气系统（如控制装置、暖通空调、管道装置、可再生能源供应）、对将来需求的适应性，以及在出现灾害或基础设施故障时的恢复性。 |
| 3345560085-5fe1-4b0c-82a5-7dd44847de98 | Translated (0%) | Occupants’ well-being. | 驻户的健康。 |
| 3445560085-5fe1-4b0c-82a5-7dd44847de98 | Translated (0%) | Assess the base building’s ability to provide daylight and views, indoor air quality, and other indoor environmental quality characteristics. | 评估基本建筑提供自然采光和视野、室内空气质量和其他室内环境质量特性的能力。 |
| 3542741500-6c24-4505-98fa-4e773c8b9d6e | Translated (0%) | <48>Implementation: </48>Document how the above analysis informed selection of a building site for the project’s tenant improvement and informed the OPR and BOD and site selection for the interior design project, including the following, as applicable: | <Bold>实施</Bold>：记录上述分析如何告知项目租户装修的建筑选址，以及 OPR、BOD 和室内设计项目的选址，包括以下内容（如适用）： |
| 36172cb78a-e9fd-4a7e-b1f2-9b26da38a605 | Translated (0%) | suitability of the base building for meeting project goals relative to the building’s site attributes; | 基本建筑在满足与建筑场址属性相关的项目目标方面的适应性； |
| 376c69e2e1-fb16-4ff2-a871-5e194e1b06c4 | Translated (0%) | suitability of the base building site location for meeting daily occupants’ commuting needs; | 基本建筑场址地点在满足日常驻户通勤需求方面的适应性； |
| 3812a1b818-30b4-4ba0-b820-d23f46af3f83 | Translated (0%) | suitability of the base building mechanical and electrical systems for meeting project goals; | 基本建筑机械和电气系统在满足项目目标方面的适应性； |
| 394ab8a268-05a7-48e7-9fe9-96f4cbc088ef | Translated (0%) | capability of the tenant space for meeting the project goals related to indoor environmental quality and occupant well-being; and | 承租空间满足与室内环境质量和驻户健康相关的项目目标方面的能力；以及 |
| 40327970b9-de01-4f65-95ef-8442514d9c87 | Translated (0%) | other systems. | 其他系统。 |
| 41a560f775-1257-45ba-843f-d52fbf8048fc | Translated (0%) | Commit to the establishment and use of ongoing feedback mechanisms that provide information about tenant space performance and occupants’ satisfaction. | 致力于建立和使用提供承租空间性能和驻户满意度相关信息的持续反馈机制。 |
| 42a560f775-1257-45ba-843f-d52fbf8048fc | Translated (0%) | Provide documentation of methods planned to gather feedback on occupants’ satisfaction. | 提供计划收集驻户满意度反馈的方法的文件。 |
| 4336403d05-fdc9-44f7-bc16-f90ef32cb50d | Translated (0%) | Energy-Related Systems | 能源相关系统 |
| 447bdb12e5-4263-4978-af95-34cfe063be82 | Translated (0%) | <51>Discovery:</51> Perform a preliminary energy analysis before the completion of schematic design that explores how to reduce energy loads for the interior design project and accomplish related sustainability goals by questioning default assumptions and testing options. | <Bold>探索</Bold>：在设计企划完成之前执行初步能源分析，以调查如何减少室内设计项目的能源负载，并通过质疑默认假定和测试方案来完成相关可持续性目标。 |
| 457bdb12e5-4263-4978-af95-34cfe063be82 | Translated (0%) | Assess at least two potential options associated with each of the following in terms of project and human performance: | 评估至少 2 个在项目和人的表现方面与以下每一项相关的潜在方案： |
| 4655130d3b-5aa1-44a4-a8ef-01f58b2c8f72 | Translated (0%) | Basic envelope attributes. | 基本外围护结构属性。 |
| 4755130d3b-5aa1-44a4-a8ef-01f58b2c8f72 | Translated (0%) | Insulation values, window-to-wall ratios, glazing characteristics, shading, window operability. | 保温值、窗地面积比、玻璃特性、遮阳、窗户可操作性。 |
| 48daf9fa30-9b5b-4f39-92ff-f4b846bd2f77 | Translated (0%) | Programmatic and operational parameters. | 计划和运营参数。 |
| 49daf9fa30-9b5b-4f39-92ff-f4b846bd2f77 | Translated (0%) | Multifunctioning spaces, operating schedules, space allotment per person, teleworking, reducing building area, ongoing operations and maintenance issues. | 多功能空间、运营安排、每个人的空间分配、远程办公、减少建筑面积、持续运营和维护问题。 |
| 50fa15520b-ee1a-477d-adf0-a9b9b18beefa | Translated (70%) | Lighting levels. | 照明程度。 |
| 51fa15520b-ee1a-477d-adf0-a9b9b18beefa | Translated (0%) | Interior surface reflectance values and lighting levels in occupied spaces. | 使用空间的内部表面反射率和照明程度。 |
| 529afa4b14-7a21-4566-96c3-b86b46691c1a | Translated (73%) | Thermal comfort ranges. | 热舒适范围。 |
| 539afa4b14-7a21-4566-96c3-b86b46691c1a | Translated (0%) | Assess thermal comfort range options. | 评估热舒适范围方案。 |
| 548ed01fea-133d-4322-8a6c-5da99387062f | Translated (0%) | Plug and process load needs. | 插座和工艺负载需求。 |
| 558ed01fea-133d-4322-8a6c-5da99387062f | Translated (0%) | Reducing plug and process loads through programmatic solutions such as equipment and purchasing policies or layout options. | 通过有计划的解决方案，如设备和采购政策或者布局方案来减少插座和工艺负载。 |
| 56fefc3c41-71f8-443e-850b-b1b76b27faf7 | Translated (0%) | <60>Implementation</60>: Document how the above analysis informed interior design decisions in the project’s OPR and BOD and the interior design of the project, including the following, as applicable: | <Bold>实施</Bold>：记录上述分析如何为项目 OPR 、BOD 的室内设计决策以及项目室内设计提供参考信息，包括以下项（如适用）： |
| 57a51a74d3-fee6-4d1b-bfee-30aa91a10ac6 | Translated (0%) | building envelope and façade conditions; | 建筑外围护结构和立面状况。 |
| 58abf5a45a-0bd9-4335-928b-536d4c886245 | Translated (0%) | elimination and/or significant downsizing of building systems (e.g., HVAC, lighting, controls, exterior materials, interior finishes, functional program elements); | 消除和/或大幅减少建筑系统（如，暖通空调、照明、控制、外部材料、室内面层、功能计划元素）； |
| 59a8fec8e6-b84e-46ab-a542-20863a088835 | Translated (0%) | methods planned to gather feedback on energy performance and occupants’ satisfaction during operations; and | 计划收集有关运营期间能源性能和驻户满意度的反馈的方法；以及 |
| 60d89d8120-5f41-4a58-91b8-bb2cb12fc1e3 | Translated (100%) | other systems. | 其他系统。 |
| 6195c42b14-3aad-48cf-b73b-c1aca4a775d3 | Translated (0%) | Project teams may also choose Option 1 for an additional point. | 项目团队还可以选择选项 1 来获得额外的分数。 |
| 620442349b-1fe5-4a0d-ac59-8d30d91150b6 | Translated (98%) | Option 1. | 选项 1. |
| 630442349b-1fe5-4a0d-ac59-8d30d91150b6 | Translated (0%) | Water-Related Systems (1 point) | 水相关系统（1 分） |
| 649dc9d448-4953-430f-8bc0-e1ee8595843e | Translated (0%) | <62>Discovery</62>: Perform a preliminary water budget analysis before the completion of schematic design that explores how to reduce potable water loads and accomplish related sustainability goals. | <Bold>探索</Bold>：在完成调查如何减少饮用水负载和完成相关可持续性目标的设计企划之前，执行初步的用水预算分析。 |
| 659dc9d448-4953-430f-8bc0-e1ee8595843e | Translated (0%) | Assess and estimate the project’s potential nonpotable water supply sources and water demand volumes, including the following: | 评估项目潜在的非自来水供应源以及水需求量，包括： |
| 66b4e1769a-e5f1-4413-a781-d3739956f184 | Translated (0%) | Fixture and fitting water demand. | 器具和配件用水需求。 |
| 67b4e1769a-e5f1-4413-a781-d3739956f184 | Translated (0%) | Assess flow and flush fixture demand volumes, calculated in accordance with WE Prerequisite Indoor Water Use Reduction. | 评估流动和冲洗器具需求量，根据 WE 先决条件: 室内用水减量 (Indoor Water Use Reduction) 计算。 |
| 680eb48c79-0747-420b-a913-6fcce1a39698 | Translated (0%) | Process water demand. | 工艺用水需求。 |
| 690eb48c79-0747-420b-a913-6fcce1a39698 | Translated (0%) | Assess kitchen, laundry, cooling tower, and other equipment demand volumes, as applicable. | 评估厨房、洗衣房、冷却塔和其他设备的需求量（如适用）。 |
| 70cb90f437-ff2f-4e7e-9a78-3804e1028cab | Translated (0%) | Supply sources. | 供应源。 |
| 71cb90f437-ff2f-4e7e-9a78-3804e1028cab | Translated (0%) | Assess all potential nonpotable water supply source volumes, such as on-site rainwater and graywater, municipally supplied nonpotable water, and HVAC equipment condensate. | 评估所有潜在非自来水供应量，如场址内雨水和灰水、市政供应的非自来水，以及暖通空调设备冷凝水。 |
| 72182d0ec5-7564-401f-8c91-1afb00050f15 | Translated (0%) | <69>Implementation</69>: Document how the above analysis informed interior design decisions in the OPR and BOD. | <Bold>实施</Bold>：记录上述分析如何为 OPR 和 BOD 中的室内设计决策提供信息。 |
| 73182d0ec5-7564-401f-8c91-1afb00050f15 | Translated (0%) | Demonstrate how at least one on-site nonpotable water supply source was used to reduce the burden on municipal supply and/or wastewater treatment systems by contributing to the water demand components listed above. | 说明至少一个场址内非自来水供应源是如何通过满足上述用水需求因素来减少市政供水和/或废水处理系统的负担。 |
| 74182d0ec5-7564-401f-8c91-1afb00050f15 | Translated (0%) | Demonstrate how the analysis informed the interior design and systems affected by the project, as applicable, for the following: | 说明分析是如何为受到项目影响的以下室内设计和系统提供参考信息的（如适用）： |
| 75f2c31e0c-839f-4582-954e-b4c5ab2def79 | Translated (0%) | plumbing systems; | 管道系统； |
| 7618de95ef-9fbb-46da-868c-a728521ff92f | Translated (0%) | sewage conveyance and/or on-site treatment systems; | 污水输送和/或场址内处理系统； |
| 77a88f2cb7-bc25-4ca9-9f2e-83324d8a4543 | Translated (73%) | process water systems; | 工艺用水系统； |
| 785541c019-f89f-4d92-82c0-032105095925 | Translated (77%) | methods planned to gather feedback on the performance and efficiency of water-related systems during operations; and | 计划收集有关运营期间性能和水相关系统效率的反馈的方法；以及 |
| 797b32b3c3-83c4-41f2-be2e-881b39d8f1b4 | Translated (100%) | other systems. | 其他系统。 |
| 804130e51a-80fd-4962-b780-3a30e6d47f44 | Translated (77%) | Location and Transportation (LT) | 选址与交通 (Location and Transportation (LT)) |
| 814aac2947-4f7d-41e2-a909-8d2aeb83888c | Translated (75%) | LT Credit: LEED for Neighborhood Development Location | LT 得分点: LEED 社区开发选址( LEED for Neighborhood Development Location) |
| 82fa0cae5b-709a-4b91-bc6a-30c032dce039 | Translated (100%) | ID&C | ID&C |
| 8309630cc8-34e3-405f-a8eb-b7bc0844e9cf | Translated (0%) | 8-18 points | 8-18 分 |
| 8473818e58-e70a-4494-83f8-632c3dbfabe8 | Translated (100%) | This credit applies to | 该得分点适用于 |
| 8570b032d0-1c0f-4caa-8a41-7d2e70b3702b | Translated (0%) | Commercial Interiors (8-18 points) | 商业室内（8-18 分） |
| 86a459b728-5998-4f86-a250-834d11bae2d2 | Translated (80%) | Hospitality (8-18 points) | 宾馆接待（8-18 分） |
| 87a2ff558b-40c1-47d4-8887-0f557abdc1f2 | Translated (80%) | Retail (8-18 points) | 零售（8-18 分） |
| 88d698b95b-c9f8-4aa3-a3e1-c767d87925e8 | Translated (100%) | Intent | 目的 |
| 8996a6b54c-7c51-4fa0-a3c7-9adeac0aa5db | Translated (0%) | To avoid development on inappropriate sites. | 避免在不合适的场址上开发。 |
| 9096a6b54c-7c51-4fa0-a3c7-9adeac0aa5db | Translated (0%) | To reduce vehicle distance traveled. | 减少车辆行驶的距离。 |
| 9196a6b54c-7c51-4fa0-a3c7-9adeac0aa5db | Translated (0%) | To enhance livability and improve human health by encouraging daily physical activity. | 鼓励日常体育锻炼，提高宜居性，改善人类健康。 |
| 9295049826-1882-4257-a1ec-0b8ebae8cf74 | Translated (100%) | Requirements | 要求 |
| 938e10a14b-c7c4-42b2-9efb-f9faa8a27181 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 948db3db86-1b4d-4eb8-9aa7-798e49770850 | Translated (0%) | Locate the project within the boundary of a development certified under LEED for Neighborhood Development (Stage 2 or Stage 3 under the Pilot or 2009 rating systems, Certified Plan or Certified Project under the LEED v4 rating system). | 将项目选址在经过 LEED 社区开发 (LEED for Neighborhood Development) 认证的开发边界内（试行或 2009 评估体系中为阶段 2 或 3，LEED v4 评估体系中为经过认证的计划或经过认证的项目）。 |
| 95388e0b7d-7c22-4ecc-863b-2206c9c2dc63 | Translated (0%) | Projects attempting this credit are not eligible to earn points under other Location and Transportation credits. | 尝试获得该得分点的项目无法在其他选址与交通 (Location and Transportation) 得分点中获得分数。 |
| 96eaea7cd5-c2e3-45d6-aa74-bbdc8a3ff8b9 | Translated (0%) | Table 1. | 表 1. |
| 97eaea7cd5-c2e3-45d6-aa74-bbdc8a3ff8b9 | Translated (0%) | Points for LEED ND location. | LEED ND 地点的分数。 |
| 987838972f-1674-4b88-aa75-84bd2ccd3d2a | Translated (94%) | Certification level | 认证等级 |
| 993a5b1f22-7cc2-4a0a-91f0-ed21deb01008 | Translated (100%) | Points | 分数 |
| 1004a47df37-c23f-4786-9b7a-7662f2241006 | Translated (100%) | Certified | 认证级 |
| 101dd61ad57-e7ed-4d25-a66e-6272df29525e | Translated (0%) | 8 | 8 |
| 10216133a40-1f2b-4c30-a007-c8c472ec6ccc | Translated (100%) | Silver | 银级 |
| 103638c5455-e448-4860-8cc0-f701f48c359b | Translated (100%) | 10 | 10 |
| 104ce0d9963-31d2-4aa7-98bb-54958b6c2148 | Translated (100%) | Gold | 金级 |
| 105ee8649c4-e025-4445-b67e-f3ac63198b5e | Translated (100%) | 12 | 12 |
| 106830125b8-8dea-4833-ba97-1f3084552f7a | Translated (100%) | Platinum | 铂金级 |
| 1078c796141-1cdd-4cd3-8bde-bdf3b8720815 | Translated (100%) | 18 | 18 |
| 108b33f495f-b99b-4b8e-b2de-048d7dfc7c7e | Translated (75%) | LT Credit: Surrounding Density and Diverse Uses | LT得分点: 周边密度和多样化土地使用 (LT Credit:Surrounding Density and Diverse Uses) |
| 1093ae090f1-5277-4636-a916-2c34c5a5d7a0 | Translated (100%) | ID&C | ID&C |
| 11018681625-396d-4399-a54d-73e91e66b736 | Translated (CM) | 1–8 points | 1-8 分 |
| 11125629131-8bdd-4d00-ae54-5e529424ae00 | Translated (CM) | This credit applies to | 该得分点适用于 |
| 11236030e7c-066f-46aa-8a53-b01a9c548801 | Translated (CM) | Commercial Interiors (1–8 points) | 商业室内（1-8 分） |
| 113996f07ae-74dd-45c8-b1df-d724d07afd4b | Translated (100%) | Retail (1–8 points) | 零售（1-8 分） |
| 1144c5360a0-7bee-44e4-8b8a-a3679a217cc3 | Translated (100%) | Hospitality (1–8 points) | 宾馆接待（1-8 分） |
| 1153168c673-a42b-4620-9dec-0b2758609177 | Translated (100%) | Intent | 目的 |
| 116fd105ee4-82bc-400d-9ec5-c1d3f5858cf9 | Translated (0%) | To conserve land and protect farmland and wildlife habitat by encouraging development in areas with existing infrastructure. | 鼓励在已有基础设施的区域进行开发，以节约土地，保护农场和野生动物栖息地。 |
| 117fd105ee4-82bc-400d-9ec5-c1d3f5858cf9 | Translated (0%) | To promote walkability, and transportation efficiency and reduce vehicle distance traveled. | 提高可步行性和交通效率，减少车辆行驶的距离、 |
| 118fd105ee4-82bc-400d-9ec5-c1d3f5858cf9 | Translated (73%) | To improve public health by encouraging daily physical activity. | 鼓励日常体育锻炼，改善公众健康。 |
| 1198ca376e1-1430-4821-95cf-7f2c0286c7ff | Translated (100%) | Requirements | 要求 |
| 120042bbe9a-988c-4d47-a14e-76abe34e5a3a | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 1211c976997-6f86-4427-b26a-32978dd724d6 | Translated (100%) | Option 1. | 选项 1. |
| 1221c976997-6f86-4427-b26a-32978dd724d6 | Translated (0%) | Surrounding Density (3–6 points) | 周边密度（3-6 分） |
| 1235827f4bc-9d1a-43b2-8fed-8f7abd310f96 | Translated (0%) | Locate on a site whose surrounding existing density within a ¼-mile (400-meter) radius of the project boundary meets the values in Table 1. | 选择项目边界 ¼ 英里（400 米）半径范围内的周边既有密度符合表 1 值的场址。 |
| 1245827f4bc-9d1a-43b2-8fed-8f7abd310f96 | Translated (0%) | Use either the “separate residential and nonresidential densities” or the “combined density” values. | 使用“独立住宅和非住宅密度”或“组合密度”值。 |
| 125f5226f09-4f4c-495f-99bb-390b67df1304 | Translated (0%) | Table 1a. | 表 1a. |
| 126f5226f09-4f4c-495f-99bb-390b67df1304 | Translated (0%) | Points for average density within 1/4 mile of project (IP units) | 项目 1/4 英里范围内平均密度的分数（IP 单位） |
| 12769ab93f4-cb69-4172-b92f-800f700b5697 | Translated (0%) | Combined density | 组合密度 |
| 1284986385d-64dc-431c-a619-1c94fe2c59a6 | Translated (0%) | Separate residential and nonresidential densities | 独立住宅和非住宅密度 |
| 129927cbab2-8294-4664-8c39-380e48442d67 | Translated (100%) | Points | 分数 |
| 130a6dea238-3fae-4c4d-8bdc-51429a4240bc | Translated (0%) | Square feet per acre of buildable land | 每英亩可建造土地的平方英尺数 |
| 131336a4019-b193-4d20-9688-9c998022fb6e | Translated (0%) | Residential density (DU/acre) | 住宅密度（DU/英亩） |
| 132873d1689-77ea-4359-836e-ac65bc35944f | Translated (0%) | Nonresidential density (FAR) | 非住宅密度 (FAR) |
| 1338278ffc3-bb44-46a8-90d1-be2b924c014f | Translated (100%) | 22,000 | 22,000 |
| 1349027cc9f-6607-48d0-86cd-49a6ca678257 | Translated (100%) | 7 | 7 |
| 1357206cdfa-6e48-4ca5-adda-4be7d2501e33 | Translated (CM) | 0.5 | 0.5 |
| 136a139ab08-74d7-4711-bf66-7287dde0f8ca | Translated (CM) | 3 | 3 |
| 137a1290c29-4032-444f-a8dd-fe49b9a1c96c | Translated (CM) | 35,000 | 35,000 |
| 13807538687-61b4-4f26-97e7-866c509f8fad | Translated (CM) | 12 | 12 |
| 139ce21f3ef-6e0c-4a88-9477-b7fae41329f4 | Translated (CM) | 0.8 | 0.8 |
| 140f21a7bdd-0ea2-4cf7-85e9-ed6a2cd7aea6 | Translated (CM) | 6 | 6 |
| 141866a3bb9-e7b5-4715-8da7-81cb20f80b3b | Translated (0%) | Table 1b. | 表 1b. |
| 142866a3bb9-e7b5-4715-8da7-81cb20f80b3b | Translated (87%) | Points for average density within 400 meters of project (SI units) | 项目 400 米范围内平均密度的分数（SI 单位） |
| 14355f5afe1-af7e-4b48-936d-5aadbe7d67c9 | Translated (100%) | Combined density | 组合密度 |
| 14460bfeb56-2299-4b44-bfba-80f2a81df991 | Translated (CM) | Separate residential and nonresidential densities | 独立住宅和非住宅密度 |
| 145d73eb6bc-abb5-4ddf-a54d-ac01ff86d9d9 | Translated (100%) | Points | 分数 |
| 146315f091d-569f-42ad-b1a1-ba428361a67f | Translated (0%) | Square meters per hectare of buildable land | 每公顷可建造土地的平方米数 |
| 1470e35c630-09d9-4cd3-9cb3-7a3af5b5a1ec | Translated (84%) | Residential density (DU/hectare) | 住宅密度（DU/公顷） |
| 148bf0a69fb-eadc-4b65-a1e1-a84cb4661804 | Translated (100%) | Nonresidential density (FAR) | 非住宅密度 (FAR) |
| 1490dc89cc9-5925-41cc-986d-299ed43926bd | Translated (100%) | 5 050 | 5 050 |
| 150891cb6ef-2444-4510-9201-cca4745bfe03 | Translated (100%) | 17.5 | 17.5 |
| 1518733b641-2aad-4f11-b754-852087e581d9 | Translated (CM) | 0.5 | 0.5 |
| 152a844a860-1615-4684-b78c-534b476f76d0 | Translated (CM) | 3 | 3 |
| 15349e5dd7f-2ca6-47fe-9aeb-6f5ecdb0c4da | Translated (0%) | 8 035 | 8 035 |
| 15472626d18-60c9-4f87-b5e4-333d42dc69bf | Translated (CM) | 30 | 30 |
| 15598d920ed-2950-4698-a754-9244808a9c84 | Translated (CM) | 0.8 | 0.8 |
| 15680890090-bbca-4ce0-90d3-fea0a061b3c4 | Translated (CM) | 6 | 6 |
| 157acf3b49a-96fb-4d8d-90b9-3c9bdc90818f | Translated (0%) | DU = dwelling unit; FAR = floor-area ratio. | DU = 居住单元；FAR = 容积率。 |
| 15867d40d59-d188-4840-b5fe-ddde27f69c11 | Translated (100%) | AND/OR | 和/或 |
| 1598a2e881f-a169-4546-8760-d5dc4ce39d8a | Translated (100%) | Option 2. | 选项 2. |
| 1608a2e881f-a169-4546-8760-d5dc4ce39d8a | Translated (0%) | Diverse Uses (1–2 points) | 多样化使用（1-2 分） |
| 161d65d0805-fb1a-46e8-be73-0291e6001f16 | Translated (0%) | Construct or renovate a building or a space within a building such that the building’s main entrance is within a ½-mile (800-meter) walking distance of the main entrance of four to seven (1 point) or eight or more (2 points) existing and publicly available diverse uses (listed in Appendix 1). | 建造或翻新建筑或建筑中的空间，使建筑的主入口到 4 - 7 个（1 分）或 8 个及更多（2 分）为公众提供的既有多样化用途（在附录 1 中列出）主入口的步行距离为 ½ 英里（800 米）之内。 |
| 162aabfcfd6-d50c-4321-a38d-ca63f66dd1aa | Translated (0%) | The following restrictions apply. | 以下限制适用。 |
| 1639809c4dd-d685-4aa8-b9a4-0f30b4268129 | Translated (0%) | A use counts as only one type (e.g., a retail store may be counted only once even if it sells products in several categories). | 一种用途只能作为一种类型计入（如，即使零售店出售多种产品，它也只能计入一次）。 |
| 164f27f9ca4-8a42-485b-bb10-e2d04553b5f1 | Translated (0%) | No more than two uses in each use type may be counted (e.g. if five restaurants are within walking distance, only two may be counted). | 每种类型的用途最多计入两次（如，如果在步行距离内有 5 家餐馆，只能计入 2 家）。 |
| 165c58ac726-fc1d-41ac-ba0b-153f5df93847 | Translated (0%) | The counted uses must represent at least three of the five categories, exclusive of the building’s primary use. | 计入的用途至少必须能代表 5 个类别中的 3 个（建筑的主要用途除外）。 |
| 16699fd0137-ceb7-4df9-b3c6-374f0741e083 | Translated (87%) | LT Credit: Access to Quality Transit | LT 得分点: 优良公共交通可达 (Access to Quality Transit) |
| 167e6022320-944b-4735-ab2e-d6cd68311b40 | Translated (100%) | ID&C | ID&C |
| 168fbb322a7-a63a-4b1c-8d52-826295729ff3 | Translated (100%) | 1–7 points | 1-7 分 |
| 169ca936a28-e8be-4ed1-85fc-828eee1536cb | Translated (100%) | This credit applies to | 该得分点适用于 |
| 170fd715874-1688-49ad-8073-1911ed33aa2b | Translated (100%) | Commercial Interiors (1–7 points) | 商业室内（1-7 分） |
| 171c69ac3ea-2fb5-4f95-8f66-167aa60dfd7d | Translated (CM) | Hospitality (1–7 points) | 宾馆接待（1-7 分） |
| 1727e68b262-1c54-4712-a2fe-836b426c0bf9 | Translated (CM) | Retail (1–7 points) | 零售（1-7 分） |
| 1733fba2ce6-2f22-4c1f-8135-c86c5405ccec | Translated (100%) | Intent | 目的 |
| 17405e5b32d-7fa5-45c1-9b08-eeffea03b495 | Translated (0%) | To encourage development in locations shown to have multimodal transportation choices or otherwise reduced motor vehicle use, thereby reducing greenhouse gas emissions, air pollution, and other environmental and public health harms associated with motor vehicle use<224>.</224> | 鼓励在拥有多种运输模式选择或可降低汽车使用率的场址开发项目，从而减少温室气体排放、空气污染以及其他与汽车使用相关的环境和公众健康危害。 |
| 1756a88d126-8cbd-42a3-918e-f4286a06fb42 | Translated (100%) | Requirements | 要求 |
| 176693d81c3-fc8f-4317-b2f1-19d2e311a1bb | Translated (0%) | CI, Hospitality, Retail-CI | 商业室内，宾馆接待，零售商业室内 |
| 1774a79cbee-d32c-42a8-8f84-efe853be9da5 | Translated (0%) | Locate any <226>functional entry</226> of the project within a ¼-mile (400-meter) <227>walking distance</227> of existing or planned bus, <228>streetcar</228>, or <229>rideshare stops,</229> or within a ½-mile (800-meter) walking distance of existing or planned <230>bus rapid transit</230> stops, light<231> </231>or heavy rail stations, commuter rail stations, or commuter ferry terminals. | 使项目的任意功能性入口位于距既有或规划的公交车、有轨电车或共乘站 ¼ 英里（400 米）的步行距离之内，或者是既有或规划的公车快速站台、轻轨站或普通有轨列车站台、公交通勤铁路车站或公交通勤渡轮码头 ½ 英里（800 米）的步行距离之内。 |
| 1784a79cbee-d32c-42a8-8f84-efe853be9da5 | Translated (0%) | The transit service at those stops and stations in aggregate must meet the minimums listed in Tables 1 and 2. | 这些车站的交通服务总的来说必须满足表 1 和 2 列出的最低要求。 |
| 1794a79cbee-d32c-42a8-8f84-efe853be9da5 | Translated (0%) | Planned stops and stations may count if they are sited, funded, and under construction by the date of the certificate of occupancy and are complete within 24 months of that date. | 如果规划的车站在驻户证书发放之日已选址、拨付资金，同时正在建设，并且将在从该日起的 24 个月内完成，则可计入在内。 |
| 1802096f865-f090-4fda-a041-3bc6d7c7fedf | Translated (0%) | Both weekday and weekend trip minimums must be met. | 必须满足工作日和周末班次的最低要求。 |
| 181599da94f-40b3-4df9-988e-dd41324423c6 | Translated (0%) | Qualifying transit routes must have paired route service (service in opposite directions). | 合格的交通线路必须拥有双向路线服务（相反方向的服务）。 |
| 1828abbb7a4-e466-44b3-bb16-a20efae209b8 | Translated (0%) | For each qualifying transit route, only trips in one direction are counted towards the threshold. | 对于每条合格的交通路线，只计入一个方向的班次。 |
| 1837a277766-6869-48c1-89c7-00488b6cb18b | Translated (0%) | If a qualifying transit route has multiple stops within the required walking distance, only trips from one stop are counted towards the threshold. | 如果合格的交通路线在要求的步行距离内有多个站台，那么只计入来自一个站台的班次。 |
| 184561589b7-67b7-4133-baaf-f789e2bb633a | Translated (100%) | Table 1. | 表 1. |
| 185561589b7-67b7-4133-baaf-f789e2bb633a | Translated (0%) | Minimum daily transit service for projects with multiple transit types (bus, <233>streetcar</233>, rail, or ferry) | 具有多种交通类型（公交、有轨电车、铁路或渡轮）的项目的日常交通服务最低要求 |
| 1862d38c3ef-19dc-4f80-b547-bba4c0e0f4a3 | Translated (0%) | Weekday trips | 工作日班次 |
| 1875f416bda-8dcb-4f09-ab28-ef6476b00b86 | Translated (0%) | Weekend trips | 周末班次 |
| 188c8e7e7a8-b932-4465-8531-3ce21e7e3c48 | Translated (100%) | Points | 分数 |
| 189ac690012-515f-4dc4-b160-addcf9c7c4d6 | Translated (100%) | 72 | 72 |
| 19073753ae4-31d7-4ca0-80ff-3eeea461a051 | Translated (CM) | 40 | 40 |
| 1916262dca7-548d-4554-bbc3-472d168d413b | Translated (CM) | 2 | 2 |
| 192ebff5b20-c9fe-4377-ac37-695ce0dd2e75 | Translated (CM) | 144 | 144 |
| 193661893d1-ea45-4473-bede-23ffb00e5bec | Translated (CM) | 108 | 108 |
| 194fa0fdec5-ce83-4621-9a68-c521f98c59bb | Translated (CM) | 5 | 5 |
| 1950a77a7d3-3476-4738-9081-46496e0e4144 | Translated (CM) | 360 | 360 |
| 19637f38785-6f7d-4426-b5f1-5418d728cdd0 | Translated (CM) | 216 | 216 |
| 197923f80fc-bd31-4123-a068-1b31b7781639 | Translated (CM) | 7 | 7 |
| 19832687650-f18b-406f-a97f-0b6aa259f391 | Translated (100%) | Table 2. | 表 2. |
| 19932687650-f18b-406f-a97f-0b6aa259f391 | Translated (0%) | Minimum daily transit service for projects with commuter rail or ferry service only | 仅具有通勤火车或渡轮的项目的日常交通服务最低要求 |
| 2001eb01803-2e40-466a-b522-1246a429b007 | Translated (100%) | Weekday trips | 工作日班次 |
| 2012a857c66-170b-4b76-8751-e32ae9160a33 | Translated (CM) | Weekend trips | 周末班次 |
| 2021723aba6-48ec-4fad-ba60-9de6fd92a3de | Translated (100%) | Points | 分数 |
| 2038cd7dfdc-26c1-4d81-94de-d482636fdf00 | Translated (100%) | 24 | 24 |
| 204c37e9f15-4836-4082-a3da-68b02be62e02 | Translated (CM) | 6 | 6 |
| 205bca5bf1d-55d0-45af-a689-d6030602d3ea | Translated (CM) | 1 | 1 |
| 2069a3b1008-f691-4b16-b6c4-49daaaf58fa9 | Translated (100%) | 40 | 40 |
| 207caacc7f3-d035-4777-911a-c9eeea7320f2 | Translated (100%) | 8 | 8 |
| 208918dfca5-47b1-4cfd-8632-7c82e4224546 | Translated (100%) | 2 | 2 |
| 209205cffd9-eb50-427b-aa04-9d9492f04453 | Translated (100%) | 60 | 60 |
| 2105a175268-6726-4bc7-bdd8-1a21f40cc98b | Translated (100%) | 12 | 12 |
| 2113b58367a-9b31-445e-a39e-16c0e7a111f3 | Translated (100%) | 3 | 3 |
| 2120228a6e0-1270-42de-b64b-a64ffeb653b2 | Translated (0%) | Projects served by two or more transit routes such that no one route provides more than 60% of the documented levels may earn one additional point, up to the maximum number of points. | 在由两条或更多交通路线提供交通服务的项目中，如果没有任何一条路线提供的班次超过总水平的 60%，则可以额外获得一分，最多可得到最高分数。 |
| 213915d796e-b3c6-4dc4-90c3-d332aa1b0700 | Translated (0%) | If <307>existing</307> transit service is temporarily rerouted outside the required distances for less than two years, the project may meet the requirements, provided the local transit agency has committed to restoring the routes with service at or above the prior level. | 如果既有的交通运输服务临时更改路线，使其在少于两年的时间里超出要求的距离，该项目也可满足得分要求，前提条件是当地的交通运输部门承诺恢复路线并提供与之前的级别相当或更高的服务。 |
| 2141760d686-0fa0-47aa-9ea4-1ffbd67782e5 | Translated (0%) | All Options | 所有选项 |
| 2151f31fbf7-ff88-49a0-a149-36ce70e44e88 | Translated (0%) | For all options, provide dedicated walking or bicycling lanes toward the transit lines. | 对于所有选项，提供专门的步行或骑车道路通往交通线路。 |
| 2161f31fbf7-ff88-49a0-a149-36ce70e44e88 | Translated (0%) | The lanes must extend from the school building to at least the end of the school property and may not have any barriers (e.g., fences). | 这些道路至少必须从学校建筑延伸至学校物业的尽头，并且不得有任何障碍物（如栅栏）。 |
| 2171f31fbf7-ff88-49a0-a149-36ce70e44e88 | Translated (0%) | School grounds may be enclosed with fences during class hours for security purposes, provided the fences are open before and after class hours for traveling students, faculty, and staff. | 学校场地在上课期间必须用栅栏围住，以确保安全，但这些栅栏在课外时间必须打开，让学生和教职员工通过。 |
| 2184c6ab557-5433-4683-873d-a2bbff686399 | Translated (0%) | LT Credit: Bicycle Facilities | LT 得分点: 自行车设施 (Bicycle Facilities) |
| 2192b995467-91ca-4274-9167-f703be81b3fa | Translated (100%) | ID&C | ID&C |
| 220295c9643-93de-45e6-a859-3098c4ff375e | Translated (100%) | 1 point | 1 分 |
| 221b9e6b06f-5696-45e8-9ca6-61b11c54bd88 | Translated (100%) | This credit applies to | 该得分点适用于 |
| 2224dcc9d64-02b2-4b9b-a873-2bb8277d9013 | Translated (78%) | Commercial Interiors (1 point) | 商业室内（1 分） |
| 2232d40e505-0fc4-4f85-bd47-bfd7f70fe3ab | Translated (72%) | Hospitality (1 point) | 宾馆接待（1 分） |
| 22409b42e71-1750-49a1-accb-de5552d71233 | Translated (72%) | Retail (1 point) | 零售（1 分） |
| 225db70d2d6-3d52-42a0-b075-94e346381259 | Translated (100%) | Intent | 目的 |
| 2269b004e59-5845-44e4-9f4d-10e653dc15a1 | Translated (94%) | To promote bicycling and transportation efficiency and reduce vehicle distance traveled. | 提高骑车和交通效率，减少车辆行驶的距离。 |
| 2279b004e59-5845-44e4-9f4d-10e653dc15a1 | Translated (79%) | To improve public health by encouraging utilitarian and recreational physical activity. | 鼓励实用和休闲体育锻炼，改善公众健康状况。 |
| 2285c02b14b-2512-46b2-ab90-350b4b5b5feb | Translated (100%) | Requirements | 要求 |
| 2294348ba7c-865a-459f-94eb-1402ee9bdda5 | Translated (70%) | CI, Hospitality | 商业室内，宾馆接待 |
| 230dd454e24-1310-4f53-b270-6b85dc195f21 | Translated (0%) | Bicycle Network | 自行车道网络 |
| 2310143f4bd-6a54-4577-aee5-32531f853980 | Translated (0%) | Locate the space in a building such that a <314>functional entry</314> and/or the bicycle storage is within a 200-yard (180-meter) <315>walking distance</315> or <316>bicycling distance</316> of a <317>bicycle network</317> that connects to at least one of the following: | 在建筑内部找到一处空间，使功能性入口和/或自行车存车处距至少连通以下其中一项的自行车道网络 200 码（180 米）步行距离或自行车骑行距离之内： |
| 23285a02142-0e11-4a93-90f8-d9286ff38f31 | Translated (0%) | at least diverse 10 uses (see Appendix 1); or | 至少 10 种不同用途（参见附录 1）；或 |
| 2330e1d9dd1-5a64-4f6d-bd57-c311475e5a1e | Translated (0%) | a<318> bus rapid transit</318> stop, light or heavy rail station, commuter rail station, or ferry terminal. | 公车快速/专用通道站、轻轨站或火车站、通勤火车站或渡轮码头。 |
| 234af0bb16a-cea0-4af5-a9bb-41fd6306c190 | Translated (0%) | All destinations must be within a 3-mile (4800-meter) bicycling distance of the project boundary. | 所有目的地都必须在距项目边界 3 英里（4800 米）自行车骑行距离之内。 |
| 235231e5c8a-623a-4e89-ba46-5955ffdfe4d6 | Translated (0%) | Planned bicycle trails or lanes may be counted if they are fully funded by the date of the certificate of occupancy and are scheduled for completion within one year of that date. | 如果规划的自行车道在获得驻户证书之日起得到全部建设资金，并且计划在从这一天起的 1 年之内完成，则可以计入。 |
| 2364d7cb3b4-a69b-4358-b751-d4df7b9f0577 | Translated (87%) | Bicycle Storage and Shower Rooms | 自行车存车和淋浴房 |
| 23726b4dc6f-87bc-4fcd-bf92-8f1b5dd6d294 | Translated (0%) | Provide <320>short-term bicycle storage</320> for at least 2.5% or more of all peak visitors, but no fewer than two storage spaces per project. | 至少为高峰时期所有访客中的 2.5% 或更多的人提供短期自行车存放，且每个项目不得少于 2 个存放处。 |
| 238b2111533-0d4b-41fb-aeb7-d08d85688389 | Translated (0%) | Provide <321>long-term bicycle storage</321> for at least 5% of regular building occupants but no fewer than 2 spaces per project in addition to the short-term bicycle spaces. | 至少为常规建筑驻户中的 5% 提供长期自行车存放，除了短期自行车存放处之外，每个项目不得少于 2 个长期存放处。 |
| 239d7134afe-8e3b-402f-bc05-5846fd2cf965 | Translated (0%) | Provide at least one on-site shower with changing facility for the first 100 regular building occupants and one additional shower for every 150 regular building occupants thereafter. | 在场址内为前 100 位常规建筑驻户至少提供一个带更衣室的淋浴房，并且要为之后的每 150 位常规建筑驻户提供一个额外的淋浴房。 |
| 240f5ee1da8-9696-483c-8748-21cffe785be0 | Translated (0%) | <322>Short-term bicycle storage</322> must be within 100 feet (30 meters) walking distance of any main entrance. | 短期自行车存放处必须位于距任何主入口 100 英尺（30 米）的步行距离内。 |
| 241f5ee1da8-9696-483c-8748-21cffe785be0 | Translated (86%) | <323>Long-term bicycle storage</323> must be within 100 feet (30 meters) walking distance of any <324>functional entry</324>. | 长期自行车存放处必须位于距任何功能性入口 100 英尺（30 米）的步行距离内。 |
| 24251a2c27f-07e1-4491-b971-73d9bc51860e | Translated (0%) | Bicycle storage capacity may not be double-counted: storage that is fully allocated to the occupants of nonproject facilities cannot also serve project occupants. | 自行车存车容量不得重复计入：完全分配给非项目设施驻户的存放处无法同时为项目驻户提供服务。 |
| 243eb3e3a11-3657-4751-8199-6db733c05c7b | Translated (0%) | Retail-CI | 零售商业室内 |
| 2449bfdbdba-aea7-4e7c-8987-1135b441b7e7 | Translated (100%) | Bicycle Network | 自行车道网络 |
| 245bcc93ae7-b084-4a9c-9cae-86d6cd89d502 | Translated (87%) | Design or locate the <326>project</326> such that a <327>functional entry</327> and/or bicycle storage is within a 200-yard (180-meter) <328>walking distance</328> or <329>bicycling distance</329> of a <330>bicycle network</330> that connects to at least one of the following: | 设计或确定项目位置时，使功能性入口和/或自行车存车处距至少连通以下其中一项的自行车道网络 200 码（180 米）步行距离或自行车骑行距离之内： |
| 24647a5e5a6-67a6-44a3-ac0e-d5319d7b6528 | Translated (86%) | at least 10 diverse uses (see Appendix 1); or | 至少 10 种多样化用途（参见附录 1）；或 |
| 247bb146204-f8d3-409a-8a46-891752df0617 | Translated (99%) | a bus rapid transit stop, light or heavy rail station, commuter rail station, or ferry terminal. | 公车快速/专用通道站、轻轨站或火车站、通勤火车站或渡轮码头。 |
| 248832d9ece-4f36-4317-8a36-5d9257d5813e | Translated (CM) | All destinations must be within a 3-mile (4800-meter) bicycling distance of the project boundary. | 所有目的地都必须在距项目边界 3 英里（4800 米）自行车骑行距离之内。 |
| 24917726d9b-1b38-42d0-9786-20d4a91d2797 | Translated (CM) | Planned bicycle trails or lanes may be counted if they are fully funded by the date of the certificate of occupancy and are scheduled for completion within one year of that date. | 如果规划的自行车道在获得驻户证书之日起得到全部建设资金，并且计划在从这一天起的 1 年之内完成，则可以计入。 |
| 2507ebb94ee-ffdf-4d86-8d75-5702e481a59c | Translated (CM) | Bicycle Storage and Shower Rooms | 自行车存车和淋浴房 |
| 251a35e7106-79e8-4699-a442-4fbe55252312 | Translated (0%) | Provide two <332>short-term bicycle storage</332> spaces for every 5,000 square feet (465 square meters), but no fewer than two storage spaces per tenant space. | 每 5000 平方英尺（465 平方米）提供 2 个短期自行车存放空间，但每个承租空间不少于两个存放空间。 |
| 2526ecc7cf6-4f4c-4478-8721-6238e4da1791 | Translated (90%) | Provide <333>long-term bicycle storage</333> for at least 5% of regular building occupants, but no fewer than two storage spaces per building in addition to the short-term bicycle storage spaces. | 至少为常规建筑驻户中的 5% 提供长期自行车存放，除了短期自行车存放空间之外，每座建筑不得少于 2 个长期存放空间。 |
| 253333ca3df-9928-4d0e-af5c-ed74af55151c | Translated (100%) | <334>Short-term bicycle storage</334> must be within 100 feet (30 meters) walking distance of any main entrance. | 短期自行车存放处必须位于距任何主入口 100 英尺（30 米）的步行距离内。 |
| 254333ca3df-9928-4d0e-af5c-ed74af55151c | Translated (CM) | <335>Long-term bicycle storage</335> must be within 100 feet (30 meters) walking distance of any <336>functional entry</336>. | 长期自行车存放处必须位于距任何功能性入口 100 英尺（30 米）的步行距离内。 |
| 255504acec7-680e-4b03-847c-813b5d162ffd | Translated (CM) | Bicycle storage capacity may not be double-counted: storage that is fully allocated to the occupants of nonproject facilities cannot also serve project occupants. | 自行车存车容量不得重复计入：完全分配给非项目设施驻户的存放处无法同时为项目驻户提供服务。 |
| 256c6541d5b-5e21-42b8-9dc0-71ea49a165c5 | Translated (0%) | Provide a bicycle maintenance program for employees or bicycle route assistance for employees and customers. | 为员工提供自行车维护计划，或者为员工和客户提供自行车道辅助设施。 |
| 257c6541d5b-5e21-42b8-9dc0-71ea49a165c5 | Translated (0%) | Route assistance must be provided in a manner easily accessible to both employees and customers. | 提供的车道辅助设施必须便于员工和客户使用。 |
| 2585e0cc11a-0960-4cc5-b0b1-b3451557910b | Translated (0%) | For projects that are part of a master plan development only: If bicycle storage has been provided by the development in which the project is located, determine the number of spaces that may be attributed to the project by dividing the floor area of the retail project by the total floor area of the development (buildings only) and multiplying the percentage result by the total number of spaces. | 对于仅为总体开发区一部分的项目来说：如果项目所在的开发区提供了自行车存车处，则可以通过将零售项目的建筑面积除以开发区的总建筑面积（仅建筑）并将百分比结果乘以空间总数，来确定属于该项目的空间数量。 |
| 2595e0cc11a-0960-4cc5-b0b1-b3451557910b | Translated (0%) | If this number does not meet the credit requirement, the project must provide additional bicycle storage. | 如果这一数量不符合得分点要求，那么项目必须提供额外的自行车存车处。 |
| 260d71f69d2-5f9e-40ea-be9b-7bef66204ba3 | Translated (86%) | LT Credit: Reduced Parking Footprint | LT 得分点: 停车面积减量 (Reduced Parking Footprint) |
| 261dc0cd2f2-dd30-42bf-8d4c-ac7deeb65a6a | Translated (100%) | ID&C | ID&C |
| 26215afd98a-0db9-41cd-9960-23c5ed98fa90 | Translated (100%) | 1–2 points | 1-2 分 |
| 263a4c88653-7a90-4e0b-8298-259954aac2c1 | Translated (100%) | This credit applies to | 该得分点适用于 |
| 2648162475a-26b7-4333-987a-394bf3c717f3 | Translated (100%) | Commercial Interiors (1–2 points) | 商业室内（1-2 分） |
| 26511bb05f6-1cea-4327-bb99-ead87f6c367c | Translated (100%) | Hospitality (1–2 points) | 宾馆接待（1-2 分） |
| 266d6d85427-ff64-4687-b3dc-e1972fe28da3 | Translated (100%) | Retail (1–2 points) | 零售（1-2 分） |
| 267240538aa-fe4a-4edb-a9ad-81269cbbd82a | Translated (100%) | Intent | 目的 |
| 26870696896-8200-40fe-932c-d94e03276bc2 | Translated (0%) | To minimize the environmental harms associated with parking facilities, including automobile dependence, land consumption, and rainwater runoff. | 尽量减少与停车设施相关的环境危害，包括对汽车的依赖、土地占用和雨水径流。 |
| 26959fd20e8-d9a7-43b6-91a9-2fe5cc9173dc | Translated (100%) | Requirements | 要求 |
| 270b0e793e2-b75f-4117-84d8-3c005521a9c9 | Translated (98%) | CI, Hospitality. | 商业室内，宾馆接待。 |
| 271b0e793e2-b75f-4117-84d8-3c005521a9c9 | Translated (100%) | Retail | 零售 |
| 2724ae0ccf8-f7c4-4d57-9ee3-2888de255be7 | Translated (0%) | Do not exceed the minimum local code requirements for parking capacity. | 不得超过本地法规的最低停车容量要求。 |
| 2732e0da0ca-e3e3-4365-ad8f-a64975914e50 | Translated (0%) | Provide parking capacity that is a percentage reduction below the base ratios recommended by the Parking Consultants Council, as shown in the Institute of Transportation Engineers’ Transportation Planning Handbook, 3rd edition, Tables 18-2 through 18-4. | 提供停车容量低于停车咨询委员会 (Parking Consultants Council) 建议的基本比率的百分比，如交通工程师协会 (Institute of Transportation Engineers)《交通规划手册》(Transportation Planning Handbook) 第三版表格 18-2 到 18-4 所示。 |
| 274dba125ba-ada8-42e6-8e6d-1b70ee074fea | Translated (0%) | Case 1. | 情况 1. |
| 275dba125ba-ada8-42e6-8e6d-1b70ee074fea | Translated (0%) | Baseline Location | 基线地点 |
| 276e82ea236-0922-43e7-ae74-63efe32f6f5f | Translated (0%) | Projects that have not earned points under LT Credit Surrounding Density and Diverse Uses or LT Credit Access to Quality Transit must achieve a 20% (1 point) or 40% (2 points) reduction from the base ratios. | 没有在 LT 得分点：周边密度和多样化土地使用 (Surrounding Density and Diverse Uses) 或 LT 得分点：优良公共交通可达 (Access to Quality Transit) 上获得分数的项目必须比基本比率低 20%（1 分）或 40%（2 分）。 |
| 277c1410d23-fa5f-407f-bfea-fc6e6ff493d1 | Translated (100%) | Case 2. | 情况 2. |
| 278c1410d23-fa5f-407f-bfea-fc6e6ff493d1 | Translated (0%) | Dense and/or Transit-Served Location | 密度和/或交通服务地点 |
| 2790e6d03d2-b2be-4a27-baaf-d7a862fe6071 | Translated (88%) | Projects earning 1 or more points under either <345>LT Credit Surrounding Density and Diverse Uses or LT Credit Access to Quality Transit</345> much achieve a 40% (1 point) or 60% (2 points) reduction from the base ratios. | 在<345> LT 得分点：周边密度和多样化土地使用 (Surrounding Density and Diverse Uses) 或 LT 得分点：优良公共交通可达 (Access to Quality Transit) </345>上获得 1 分或更多分的项目必须比基本比率低 40%（1 分）或 60%（2 分）。 |
| 280eb357191-4bfd-4fde-ba5f-22c9388f0b23 | Translated (77%) | For All Projects | 对于所有项目 |
| 28121f3153b-24f9-45f7-9a11-6314c1391bdd | Translated (0%) | The credit calculations must include all existing and new off-street parking spaces that are leased or owned by the project, including parking that is outside the project boundary but is used by the project. | 得分点计算必须包括项目租赁或拥有的所有既有的和新的路外停车空间，包括位于项目边界以外但被项目使用的停车空间。 |
| 28221f3153b-24f9-45f7-9a11-6314c1391bdd | Translated (0%) | On-street parking in public rights-of-way is excluded from these calculations. | 这些计算中不包括公共道路用地中的路旁停车空间。 |
| 2835736e6ee-14d8-4796-9b00-8062e636e981 | Translated (0%) | For projects that use pooled parking, calculate compliance using the project’s share of the pooled parking. | 对于使用拼车停车场的项目，使用项目的拼车停车份额来计算是否符合要求。 |
| 284e2466876-bd42-40c4-9300-e747eb12d58d | Translated (0%) | Provide preferred parking for carpools for 5% of the total parking spaces after reductions are made from the base ratios. | 在从基本比率中减去相应百分比之后，为总停车空间 5% 的共乘提供优先车位。 |
| 285e2466876-bd42-40c4-9300-e747eb12d58d | Translated (0%) | Preferred parking is not required if no off-street parking is provided. | 如果未提供路外停车位，则不需要提供优先车位。 |
| 286674be0cc-8d02-4abd-8176-cd32edc61ca9 | Translated (0%) | Mixed-use projects should determine the percentage reduction by first aggregating the parking amount of each use (as specified by the base ratios) and then determining the percentage reduction from the aggregated parking amount. | 混合用途项目应确定减少的百分比，具体方法为首先汇总每种用途的停车数量（由基本比率确定），然后确定从汇总的停车数量中减去的百分比。 |
| 2875020e89e-d495-4e19-811a-1079c3a7ca72 | Translated (0%) | Do not count parking spaces for fleet and inventory vehicles unless these vehicles are regularly used by employees for commuting as well as business purposes. | 不要计入车队和库存车辆的停车空间，除非员工会定期使用这些车辆进行通勤以及商业用途。 |
| 288d9a5b6a6-6df8-4b91-8824-740dbc200995 | Translated (100%) | Sustainable Sites (SS) | 可持续场址 (SS) |
| 2899f74f69b-b1f9-42da-aa42-ab0aea578e12 | Translated (100%) | Water Efficiency (WE) | 用水效率 (WE) |
| 290fa513dfe-42cc-460a-987b-93b280df0865 | Translated (77%) | WE Prerequisite: Indoor Water Use Reduction | WE 先决条件: 室内用水减量 (Indoor Water Use Reduction) |
| 291fbbbf445-8b65-40a0-8c5a-f054f33c89b0 | Translated (100%) | Required | 必要项 |
| 292d0d1d5dc-7e2c-44b0-bf7c-1a6e00b53f74 | Translated (100%) | ID&C | ID&C |
| 293162139fe-7a17-4b50-b4d3-13f985dda2c7 | Translated (83%) | This prerequisite applies to | 该先决条件适用于 |
| 2944baf547b-d3e6-4f91-833b-4582894bc081 | Translated (100%) | Commercial Interiors | 商业室内 |
| 295c620fd36-b7d5-456a-b132-51e882f997ec | Translated (100%) | Retail | 零售 |
| 296d148f8f2-4cfb-4766-89bd-ad07e8d10b92 | Translated (100%) | Hospitality | 宾馆接待 |
| 297b7ed1f33-ae17-4757-8c18-32a41d3b8b8f | Translated (100%) | Intent | 目的 |
| 2988c74e8b1-151e-4129-9583-ba8b6f9339d0 | Translated (0%) | To reduce indoor water consumption. | 减少室内用水量。 |
| 29940f12ef0-39ab-4525-8fe1-6b82e302f56f | Translated (100%) | Requirements | 要求 |
| 300155abd5a-a9b0-4aaf-aa9e-5075f05043bd | Translated (0%) | CI, CI-Retail, CI-Hospitality | 商业室内，商业室内零售，商业室内宾馆接待 |
| 30102afa615-dbcd-4e37-a4aa-208f75516f94 | Translated (77%) | Building Water Use | 建筑用水 |
| 302b9100760-a2e0-4c32-855b-4a262dcaebfe | Translated (0%) | For the fixtures and fittings listed in Table 1, as applicable to the project scope, reduce aggregate water consumption by 20% from the baseline. | 对于表 1 中列出的器具和配件（适用于项目范围），减少 20% 的基线总用水量。 |
| 303b9100760-a2e0-4c32-855b-4a262dcaebfe | Translated (0%) | Base calculations on the volumes and flow rates shown in Table 1. | 基本的用量和流量计算列在表 1 中。 |
| 304fdd7c85a-039d-423b-bb7f-857c920d0798 | Translated (0%) | All newly installed toilets, urinals, private lavatory faucets, and showerheads that are eligible for labeling must be WaterSense labeled (or a local equivalent for projects outside the U.S.). | 所有新安装的符合贴标要求的卫生间、小便器、自用厕所水龙头和淋浴喷头都必须贴有 WaterSense 标签（美国以外国家/地区的项目为本地对应的标签）。 |
| 305d985e92e-0a65-4b33-a53b-29f2cc02d685 | Translated (90%) | Projects where fixtures or fixture fittings are not within the tenant spaces are exempt from this prerequisite. | 器具和器具配件不在承租空间的项目不必满足该先决条件。 |
| 306b9aee480-0026-4cd7-b4cc-a20d2b602820 | Translated (100%) | Table 1. | 表 1. |
| 307b9aee480-0026-4cd7-b4cc-a20d2b602820 | Translated (0%) | Baseline water consumption of fixtures and fittings | 器具和配件的用水量基线 |
| 3080ac6b58e-d801-444a-985d-98fec7846982 | Translated (0%) | Fixture or fitting | 器具或配件 |
| 3092a5b933b-81cf-4832-9782-265521285eb9 | Translated (0%) | Baseline (IP units) | 基线（IP 单位） |
| 3102c8d94f2-2ba3-431e-ab8e-018bca2f587b | Translated (78%) | Baseline (SI units) | 基线（SI 单位） |
| 3113898895f-44b8-433d-8fca-7b449b6db97e | Translated (74%) | Toilet (water closet)* | 卫生间（盥洗室）* |
| 31259a466a2-1fb6-49d1-a2c3-4ead15210a3c | Translated (0%) | 1.6 gpf | 1.6 gpf |
| 313a33d97a4-8f3b-4fa6-b70e-c63e22fd4776 | Translated (0%) | 6 lpf | 6 lpf |
| 314db6f7dc5-dcbb-4597-a1c8-3ad38713e826 | Translated (96%) | Urinal* | 小便器* |
| 315f922041f-c76e-4d5f-9395-8ac426b7eee2 | Translated (100%) | 1.0 gpf | 1.0 gpf |
| 3164968c949-a906-4d6a-b34d-2c4eb3b7d405 | Translated (CM) | 3.8 lpf | 3.8 lpf |
| 317ceb035bf-b2f1-41c4-b574-eeddd9040446 | Translated (76%) | Public lavatory (restroom) faucet | 公用面盆（盥洗室）水龙头 |
| 318d6bad3fc-3ff3-4ce7-ac4f-8ff84e107576 | Translated (99%) | 0.5 gpm at 60 psi** all others except private applications | 60 psi** 下 0.5 gpm，私人应用以外的其他所有应用 |
| 3190c231b9d-94e8-4528-9c45-aadf242daa09 | Translated (86%) | 1.9 lpm at 415 kPa, all others except private applications | 415 kPa 下 1.9 lpm，私人应用以外的其他所有应用 |
| 320064ccecb-c599-42f8-884a-54e848fa235c | Translated (0%) | Private lavatory faucets | 自用厕所水龙头 |
| 321ef778b9a-f90b-421f-9ec2-93c1d651b85a | Translated (99%) | 2.2 gpm at 60 psi | 60 psi 下 2.2 gpm |
| 322537e201a-1b1a-4815-8572-18fb125d8ee2 | Translated (73%) | 8.3 lpm at 415 kPa | 415 kPa 下 8.3 lpm |
| 3239f2002e4-64c2-423f-9e2f-56c441df2ad1 | Translated (0%) | Kitchen faucet (excluding faucets used exclusively for filling operations) | 厨房水龙头（专用于灌装操作的水龙头除外） |
| 324f42d8af6-5af5-4bda-9856-3b8ffa50793a | Translated (100%) | 2.2 gpm at 60 psi | 60 psi 下 2.2 gpm |
| 3257769d7c3-0f77-4e37-bf2c-e5babdcd4ac9 | Translated (CM) | 8.3 lpm at 415 kPa | 415 kPa 下 8.3 lpm |
| 326e87b9039-ae4d-4690-abbf-3e679a50168f | Translated (0%) | Showerhead* | 淋浴喷头* |
| 3271a6ea1a3-ab59-442b-b63d-276d52b93c41 | Translated (98%) | 2.5 gpm at 80 psi per shower stall | 每个淋浴间 80 psi 下 2.5 gpm |
| 3282045dca5-bbfd-405f-9c59-2193eed5849b | Translated (83%) | 9.5 lpm at 550 kPa per shower stall | 每个淋浴间 550 kPa 下 9.5 lpm |
| 329be888eca-afdb-4e8f-ac46-9be303cae080 | Translated (0%) | * WaterSense label available for this product type | * 该产品类型可使用 WaterSense 标签 |
| 330a6de2049-77be-47fc-8853-29e3f9212599 | Translated (77%) | gpf = gallons per flush | gpf = 每次冲水加仑数 |
| 331b0cc5d20-1683-41c6-87a3-bb693ccd9e6c | Translated (81%) | gpm = gallons per minute | gpm = 每分钟加仑数 |
| 332861f62f0-9be5-4701-954b-5977234e61ee | Translated (0%) | psi = pounds per square inch | psi = 每平方英寸磅数 |
| 333133812b5-f0ca-49c6-aa8b-c1455a658bd1 | Translated (0%) | lpf = liters per flush | lpf = 每次冲水升数 |
| 3348f2cd811-898d-4e77-ad65-30ee312cf778 | Translated (0%) | lpm = liters per minute | lpm = 每分钟升数 |
| 33574f21642-a056-4a50-9caf-a37de06d904b | Translated (0%) | kPa = kilopascals | kPa = 千帕 |
| 336e1ddb1ae-16be-498d-a628-f72f8b934136 | Translated (0%) | Appliance and Process Water Use | 电器和工艺用水量 |
| 3373bf534f8-4477-4738-a7f3-fdf00a3db402 | Translated (0%) | Install appliances, equipment, and processes within the project scope that meet the requirements listed in the tables below. | 在项目内安装符合下表所列要求的电器、设备和工艺。 |
| 338a46d2a9b-23ce-4197-ab08-942bb9973a7f | Translated (100%) | Table 2. | 表 2. |
| 339a46d2a9b-23ce-4197-ab08-942bb9973a7f | Translated (0%) | Standards for appliances | 电器标准 |
| 340fdebf20c-e144-477d-92be-acbc9ac49f6c | Translated (0%) | Appliance | 电器 |
| 341d426a22b-e78f-4cd2-8959-b1cfaaa99670 | Translated (100%) | Requirement | 要求 |
| 342412e0d64-3ecd-47bb-8b44-4946e8084732 | Translated (94%) | Residential clothes washers | 住宅洗衣机 |
| 343277aae2d-263a-455e-b02b-6c9035c64bc1 | Translated (0%) | ENERGY STAR or performance equivalent | “能源之星”（ENERGY STAR） 或同等性能 |
| 344669aa059-165e-4f53-bf5a-0ea84d97e26c | Translated (77%) | Commercial clothes washers | 商用洗衣机 |
| 3458c0be1e2-237b-4db4-b13d-949fea434456 | Translated (75%) | CEE Tier 3A | CEE 3A 级 |
| 34651797e96-2942-438e-8615-605b6f1e57c6 | Translated (0%) | Residential dishwashers (standard and compact) | 住宅洗碗机（标准和紧凑型） |
| 347b5bb56cc-214b-470b-92b9-5cfc2804a2f9 | Translated (100%) | ENERGY STAR or performance equivalent | “能源之星”（ENERGY STAR） 或同等性能 |
| 34804850f42-3c68-4237-a8da-1e94cf42d388 | Translated (100%) | Prerinse spray valves | 预清洗喷雾阀 |
| 349125b941f-9b77-42bd-93fd-556da92d42d8 | Translated (0%) | ≤ 1.3 gpm (4.9 lpm) | ≤ 1.3 gpm (4.9 lpm) |
| 35040f435e6-bf16-4160-93f4-85bbc2b875d9 | Translated (0%) | Ice machine | 制冰机 |
| 35182352897-8c9a-4b4e-a236-03412bbafc40 | Translated (0%) | ENERGY STAR or performance equivalent and use either air-cooled or closed-loop cooling, such as chilled or condenser water system | “能源之星”（ENERGY STAR）或同等性能，并使用气冷或闭环冷却，如冷冻或冷凝水系统 |
| 3522c46c1da-66a1-4458-b5d5-c5d34faa4c4f | Translated (100%) | gpm = gallons per minute | gpm = 每分钟加仑数 |
| 35370861828-0e25-4497-95d9-830277b07bc4 | Translated (100%) | lpm = liters per minute | lpm = 每分钟升数 |
| 354e81e6bb8-f0a0-4be3-bd58-5a62d8df666f | Translated (100%) | Table 3. | 表 3. |
| 355e81e6bb8-f0a0-4be3-bd58-5a62d8df666f | Translated (77%) | Standards for processes | 工艺标准 |
| 3565aff2607-e390-4474-8fe2-013f1bc92a1c | Translated (0%) | Process | 工艺 |
| 35718ae2850-0ed4-46d4-9d36-ecb14f285446 | Translated (100%) | Requirement | 要求 |
| 3581c50281b-4272-4a84-ba9c-26290316cbdc | Translated (0%) | Heat rejection and cooling | 排热和冷却 |
| 35938c03dd4-569f-4bec-b620-48ddb7e0a462 | Translated (0%) | No once-through cooling with potable water for any equipment or appliances that reject heat | 对于任何排热设备或电器，不得使用直流饮用水冷却 |
| 3604ac31379-aad7-446c-8b3d-f1face3bc8a1 | Translated (0%) | Cooling towers and evaporative condensers | 冷却塔和蒸发式冷凝器 |
| 3616ccb9841-b1ef-43d0-aa40-da8979d68e3a | Translated (0%) | Equip with | 配备有 |
| 362de38786c-aa20-44eb-878e-7fecd9ee71c2 | Translated (0%) | makeup water meters | 补充水表 |
| 36333ab5191-c987-45dc-8ec7-2e6c14d956db | Translated (0%) | conductivity controllers and overflow alarms | 电导率控制器和溢流警报 |
| 364f5c23037-3ad3-4a40-8796-ab9462169f79 | Translated (0%) | efficient drift eliminators that reduce drift to maximum of 0.002% of recirculated water volume for counterflow towers and 0.005% of recirculated water flow for cross-flow towers | 高效除水器，在逆流式冷却塔中最多可以将飘滴减少为再循环水量的 0.002%，在横流塔中最多可以将飘滴减少为再循环水流的 0.005% |
| 365292e65a5-ca1f-45a4-83da-0d9e70b7956b | Translated (0%) | Retail and Hospitality Only | 仅限零售和宾馆接待 |
| 366d0bcab05-2391-43e0-b33b-bdffb064fda2 | Translated (0%) | In addition, water-consuming appliances, equipment, and processes must meet the requirements listed in Tables 4 and 5. | 此外，用水电器、设备和工艺必须满足表 4 和 5 中所列要求。 |
| 3677b4353b4-1bf6-4943-826b-7f5c2b712d5b | Translated (100%) | Table 4. | 表 4. |
| 3687b4353b4-1bf6-4943-826b-7f5c2b712d5b | Translated (100%) | Standards for appliances | 电器标准 |
| 36950740401-e970-4ac8-94dd-d1941fb5250b | Translated (0%) | Kitchen equipment | 厨房设备 |
| 370f571bc92-a92d-4991-be41-db22366f90bf | Translated (78%) | Requirement (IP units) | 要求（IP 单位） |
| 371dd4361eb-83d6-418a-bfce-e6f914d778f3 | Translated (78%) | Requirement (SI units) | 要求（SI 单位） |
| 3722c5c9918-f441-4aad-a651-08b72a034cbf | Translated (0%) | Dishwasher | 洗碗机 |
| 37340f90607-ba5a-4461-a9cc-b628b6f79534 | Translated (0%) | Undercounter | 台下式 |
| 374cc33d4c1-0bc0-4e47-9e0a-c1da459f4eb4 | Translated (0%) | ≤ 1.6 gal/rack | ≤ 1.6 加仑/架 |
| 37573c13997-c40e-4328-94e7-9878513dd8be | Translated (0%) | ≤ 6.0 liters/rack | ≤ 6.0 升/架 |
| 376cf2829d7-72dd-4b2a-8e1f-a04e580943be | Translated (0%) | Stationary, single tank, door | 固定，单缸，门式 |
| 377f74f0a26-424f-4917-91ec-4c642931edf8 | Translated (0%) | ≤ 1.4 gal/rack | ≤ 1.4 加仑/架 |
| 3783bfd8cc4-f266-4012-867e-e25c361a0cac | Translated (CM) | ≤ 5.3 liters/rack | ≤ 5.3 升/架 |
| 379851debf7-bb10-42e9-903a-f4e7c7fce8e8 | Translated (0%) | Single tank, conveyor | 单缸，传送式 |
| 3807c528190-a1a3-4d63-b632-347650379217 | Translated (85%) | ≤ 1.0 gal/rack | ≤ 1.0 加仑/架 |
| 38118b19714-e45b-4edb-93b2-418121b1d8ed | Translated (CM) | ≤ 3.8 liters/rack | ≤ 3.8 升/架 |
| 3829cb6b01d-c1eb-4741-b442-a4eac08c33a2 | Translated (78%) | Multiple tank, conveyor | 多缸，传送式 |
| 3838d09b31e-935c-4f6c-937a-151760fbefb3 | Translated (85%) | ≤ 0.9 gal/rack | ≤ 0.9 加仑/架 |
| 384b99ec631-d5de-409f-be31-eabfc44d681a | Translated (CM) | ≤ 3.4 liters/rack | ≤ 3.4 升/架 |
| 385e3b7fc7b-3829-4ee9-a9b7-fb6c6839d9c5 | Translated (0%) | Flight machine | 履带式 |
| 3867815d08c-102b-438e-9f2c-7f6cdebd5667 | Translated (0%) | ≤ 180 gal/hour | ≤ 180 加仑/小时 |
| 3874b9f1dee-8489-4238-b27a-d36ff5f94cad | Translated (78%) | ≤ 680 liters/hour | ≤ 680 升/小时 |
| 388ddcc62a8-8a6d-4daa-ab11-de13fcc3f969 | Translated (84%) | Food steamer | 食物蒸笼 |
| 389dd795d64-581f-4916-896f-6e2c1921504f | Translated (0%) | Batch | 批量 |
| 390f1a2fad4-66aa-46f0-be18-d2f7ba08ab61 | Translated (0%) | ≤ 6 gal/hour/pan | ≤ 6 加仑/小时/盘 |
| 391fa8d7cd4-75bb-4bd3-9c13-9802035a2467 | Translated (80%) | ≤ 23 liters/hour/pan | ≤ 23 升/小时/盘 |
| 39232efa679-661b-4c19-b755-3da22284e314 | Translated (0%) | Cook-to-order | 按订单烹饪 |
| 393b3ea2db9-1ba8-4460-8d07-861047c1664f | Translated (90%) | ≤ 10 gal/hour/pan | ≤ 10 加仑/小时/盘 |
| 394334c731c-3931-4ec9-acad-6f34f3731ea0 | Translated (CM) | ≤ 38 liters/hour/pan | ≤ 38 升/小时/盘 |
| 395555711ec-326f-4b19-a3ca-a4214f6c34ac | Translated (0%) | Combination oven, | 组合烤箱， |
| 39666067776-4e52-4921-b1a6-821fd145b7e3 | Translated (0%) | Countertop or stand | 台式或直立式 |
| 397dc6780c7-947c-49c3-a65a-a3bbeb71df1c | Translated (90%) | ≤ 3.5 gal/hour/pan | ≤ 3.5 加仑/小时/盘 |
| 3983c5595be-8a8f-4e44-be81-ad7c2166d70c | Translated (CM) | ≤ 13 liters/hour/pan | ≤ 13 升/小时/盘 |
| 39941df2eff-6ce8-4ae7-9345-9e7605414a7a | Translated (0%) | Roll-in | 旋转式 |
| 4007b8621b2-d7d2-4aeb-a25d-128a877c9483 | Translated (100%) | ≤ 3.5 gal/hour/pan | ≤ 3.5 加仑/小时/盘 |
| 401cfd4de39-0aee-474a-9d34-4c4de48488fb | Translated (CM) | ≤ 13 liters/hour/pan | ≤ 13 升/小时/盘 |
| 40214e52acb-3740-495e-a423-b6c589b50d72 | Translated (100%) | Table 5. | 表 5. |
| 40314e52acb-3740-495e-a423-b6c589b50d72 | Translated (0%) | Process requirements | 工艺要求 |
| 404fd6408f3-9f3e-4a1c-afdb-209f4c3f5736 | Translated (0%) | Discharge water temperature tempering | 排水温度回火 |
| 405790f6226-38e8-4f03-a18e-6ce941ac3add | Translated (0%) | Where local requirements limit discharge temperature of fluids into drainage system, use tempering device that runs water only when equipment discharges hot water | 如果本地要求限制了排放到排水系统中的液体温度，则使用仅在设备排放热水时出水的回火装置 |
| 406eac851bd-bf35-4e37-a14a-0c5ff6caab29 | Translated (100%) | OR | 或 |
| 4076a93e06e-e01e-43f0-bb42-621e5d9edb8a | Translated (0%) | Provide thermal recovery heat exchanger that cools drained discharge water below code-required maximum discharge temperatures while simultaneously preheating inlet makeup water | 提供热回收换热器，将排放出来的水冷却到法规要求的最高排放温度以下，同时预热入口补充水 |
| 40858fd3652-4d6d-4b12-a0f0-fdc5731430be | Translated (100%) | OR | 或 |
| 40993a23a87-dcc2-4594-b5e7-9e9fe53663c1 | Translated (0%) | If fluid is steam condensate, return it to boiler | 如果液体是蒸汽冷凝水，则使其回到锅炉 |
| 4100938d7ab-732e-4f1c-ad98-2dadfd81517f | Translated (0%) | Venturi-type flow-through vacuum generators or aspirators | 文氏管型流过式真空发生器或抽吸器 |
| 4119f5fb888-1c69-4227-9528-e863f9b5eac2 | Translated (0%) | Use no device that generates vacuum by means of water flow through device into drain | 不使用通过让水经过设备流到排水管中来生成真空的设备 |
| 4121fa1b632-aa49-43a1-b693-31707b3acc04 | Translated (89%) | WE Credit: Indoor Water Use Reduction | WE 得分点：室内用水减量 (Indoor Water Use Reduction) |
| 4139380f077-8653-479e-b259-4baeb064db1d | Translated (100%) | ID&C | ID&C |
| 41485d42b87-f276-4c8f-8a0c-616049f076b2 | Translated (100%) | 2–12 points | 2-12 分 |
| 4151e831e2c-5324-4c02-9b25-78af83d38060 | Translated (100%) | This credit applies to | 该得分点适用于 |
| 416998fecdc-0775-4d95-b55a-dc566cfa9ba7 | Translated (100%) | Commercial Interiors (2–12 points) | 商业室内（2-12 分） |
| 417b91d90b3-c168-49a0-8afe-2b143de37070 | Translated (100%) | Retail (2–12 points) | 零售（2-12 分） |
| 418afd6cafa-75d6-494c-8527-5ccf1ce41d82 | Translated (100%) | Hospitality (2–12 points) | 宾馆接待（2-12 分） |
| 419e6f72c6b-0c62-4723-8308-e31ba29f970e | Translated (100%) | Intent | 目的 |
| 4209bfff356-7c4d-45e0-87c1-3600a06df436 | Translated (100%) | To reduce indoor water consumption. | 减少室内用水量。 |
| 4211786d4fd-e343-4814-88aa-4e31a10f7d1e | Translated (100%) | Requirements | 要求 |
| 4224f8a9ce1-5afb-40bc-aeaa-49a17f1e192b | Translated (100%) | CI, CI-Retail, CI-Hospitality | 商业室内，商业室内零售，商业室内宾馆接待 |
| 4236c37398d-b911-4301-83e9-9801204362ee | Translated (0%) | Further reduce fixture and fitting water use from the calculated baseline in WE Prerequisite Indoor Water Use Reduction. | 以 WE 先决条件：室内用水减量 (Indoor Water Use Reduction) 中计算的基线为基础，进一步减少器具和配件用水量。 |
| 4246c37398d-b911-4301-83e9-9801204362ee | Translated (0%) | Additional potable water savings can be earned above the prerequisite level using alternative water sources. | 还可以使用替代性水资源来节省比先决条件水平要求的量更多的饮用水。 |
| 4256c37398d-b911-4301-83e9-9801204362ee | Translated (0%) | Include fixtures and fittings necessary to meet the needs of the occupants. | 包括满足驻户需求所需的器具和配件。 |
| 4266c37398d-b911-4301-83e9-9801204362ee | Translated (0%) | Some of these fittings and fixtures may be outside the tenant space. | 其中某些配件和器具可能在承租空间以外。 |
| 4276c37398d-b911-4301-83e9-9801204362ee | Translated (0%) | Points are awarded according to Table 1. | 根据表 1 获得分数。 |
| 4288302ae96-5aba-4302-b3fd-0ce7bef5c07a | Translated (100%) | Table 1. | 表 1. |
| 4298302ae96-5aba-4302-b3fd-0ce7bef5c07a | Translated (74%) | Points for reducing water use | 减少用水的分数 |
| 430c5870823-60d2-4937-863e-4da2250953dc | Translated (100%) | Percentage | 减少 |
| 431f1b26971-76f1-48fd-823a-e2a178addede | Translated (0%) | reduction | 百分比 |
| 4329667bed5-9fea-47ce-b443-e5aa9de4cb75 | Translated (0%) | Points (Commercial Interiors) | 分数（商业室内） |
| 433944d3d70-db6f-4b3e-bc49-8281e37ed392 | Translated (100%) | Points | 分数 |
| 434f7920d26-4559-4e5f-80eb-2738e919263c | Translated (91%) | (Retail) | （零售） |
| 435074d08d0-d90e-42c4-8128-3bd839fab6ee | Translated (100%) | Points | 分数 |
| 4360c172e2a-632e-42e9-86c4-87467ca0cea5 | Translated (91%) | (Hospitality) | （宾馆接待） |
| 437cdce1648-0edc-422c-979c-1ba95e099398 | Translated (100%) | 25% | 25% |
| 438d1e72073-e1f3-4415-b395-6fd8a05e8ab4 | Translated (100%) | 2 | 2 |
| 439bd92404c-586c-4327-8b3f-fd822876cc37 | Translated (100%) | 2 | 2 |
| 44042aa424b-9f79-4b03-a3fb-25f3608df32f | Translated (100%) | 2 | 2 |
| 441d47b793d-5be3-4d9a-96c9-065b0dc1cc11 | Translated (95%) | 30% | 30% |
| 442541f7d37-1c55-4e80-80cf-419aa357db61 | Translated (100%) | 4 | 4 |
| 4437b0eeca3-cbf6-4fc9-9dea-4d62c4ccd994 | Translated (100%) | 4 | 4 |
| 444e77da64c-464c-4b24-8be1-4642560c2db7 | Translated (100%) | 4 | 4 |
| 445118bb759-d9eb-4df9-a0c4-5bbf66847393 | Translated (100%) | 35% | 35% |
| 4468a9701e7-c195-476c-8924-bf141965ada8 | Translated (100%) | 6 | 6 |
| 44765b60820-4fc1-4225-b684-232778b2623f | Translated (100%) | 6 | 6 |
| 448493bf384-0ba2-4952-ac4b-1e84e3364244 | Translated (100%) | 6 | 6 |
| 4493df3ce75-6e11-4ab4-9c9d-c82746bc26df | Translated (100%) | 40% | 40% |
| 4505f843b7a-e795-42de-84a6-44cd971a7706 | Translated (100%) | 8 | 8 |
| 451061c36da-d483-4ab9-88a7-2ae33b893b24 | Translated (100%) | 8 | 8 |
| 452a52a7ddb-f16a-4c61-89ab-5b6bf4f621a8 | Translated (100%) | 8 | 8 |
| 453bf2bd60f-0f2d-4d27-8b7b-1f63792607c6 | Translated (100%) | 45% | 45% |
| 4541b57aef9-3e8d-453f-8c92-6710643aec61 | Translated (100%) | 10 | 10 |
| 455554deef5-9ac2-4d1a-8cb0-ee29446af259 | Translated (100%) | 10 | 10 |
| 45613125aac-5238-4d1c-8c15-f0433788f282 | Translated (100%) | 10 | 10 |
| 45787e65bc5-437d-40d0-84de-a504c41b841a | Translated (100%) | 50% | 50% |
| 458c0c32ff7-e957-4a7c-b855-42b8fd654c71 | Translated (100%) | 12 | 12 |
| 459d6bb2761-d968-4829-9767-3c4a18d56753 | Translated (0%) | -- | -- |
| 46029fba865-fe4c-4562-bdd8-d347f5e9248b | Translated (100%) | 11 | 11 |
| 4618cf52301-7ff0-4f2c-b41b-6ff9b9f7c6c0 | Translated (98%) | Retail and Hospitality only | 仅限零售和宾馆接待 |
| 46212165095-2f25-4686-8581-08b903b22acd | Translated (0%) | Meet the percentage reduction requirements above. | 满足上述百分比减少要求。 |
| 46387aad48b-214e-4b1b-82b5-da76b9bc1554 | Translated (100%) | AND | 以及 |
| 4649c7c4253-8b41-4954-9fa6-e07c91d829db | Translated (83%) | Appliance and Process Water. | 电器和工艺用水。 |
| 4659c7c4253-8b41-4954-9fa6-e07c91d829db | Translated (0%) | Install equipment within the project scope that meets the minimum requirements in Table 2, 3, 4, or 5. | 在项目内安装符合表 2、3、4 或 5 中最低要求的设备。 |
| 4669c7c4253-8b41-4954-9fa6-e07c91d829db | Translated (0%) | One point is awarded for meeting all applicable requirements in any one table. | 满足任意一个表格中的所有适用要求即可获得 1 分。 |
| 4679c7c4253-8b41-4954-9fa6-e07c91d829db | Translated (0%) | All applicable equipment listed in each table must meet the standard. | 每个表格中列出的所有适用设备都必须符合标准。 |
| 468fd2e795f-b42a-4197-8c44-805c6af7fe3a | Translated (0%) | Retail projects can earn a second point for meeting the requirements of two tables. | 零售项目在满足两个表格的要求之后可以再获得 1 分。 |
| 46988a7ad29-7d69-485b-ba6b-f9c5dd6d2057 | Translated (100%) | Table 2. | 表 2. |
| 47088a7ad29-7d69-485b-ba6b-f9c5dd6d2057 | Translated (0%) | Compliant commercial washing machines | 符合要求的商用洗衣机 |
| 471dde4e942-1d47-4f18-bdeb-c5f963b875a6 | Translated (0%) | To use Table 2, the project must process at least 120,000 lbs (57 606 kg) of laundry per year. | 要使用表 2，项目每年必须至少洗涤 120000 磅（57606 千克）的衣物。 |
| 4721785fef2-3521-41a0-8f2c-9e9193685e1d | Translated (0%) | Washing machine | 洗衣机 |
| 4731049f8c6-db41-4019-86fc-f5900a42d06d | Translated (100%) | Requirement (IP units) | 要求（IP 单位） |
| 4743945d4bc-652c-4602-a002-92682f59255c | Translated (CM) | Requirement (SI units) | 要求（SI 单位） |
| 47576bd0667-3d5e-4aa2-87a6-7d54a20d5556 | Translated (0%) | On-premise, minimum capacity 2,400 lbs (1 088 kg) per 8-hour shift | 现场部署，每 8 小时轮班的最小容量为 2400 磅（1088 千克） |
| 476b45b4838-3218-4223-8adf-0e1cafde14e4 | Translated (0%) | Maximum 1.8 gals per pound * | 每磅最多 1.8 加仑* |
| 477cc1153a0-bf15-4ae9-bc19-390104a317cd | Translated (0%) | Maximum 7 liters per 0.45 kilograms * | 每 0.45 千克最多 7 升* |
| 478cbaeb077-3335-40f2-b3a4-d37b91f3cb58 | Translated (0%) | * Based on equal quantities of heavy, medium, and light soil laundry. | * 基于等量的重度、中度和轻度脏污的衣服。 |
| 479196e3505-9a4a-4262-8cde-c3f3dae499e9 | Translated (100%) | Table 3. | 表 3. |
| 480196e3505-9a4a-4262-8cde-c3f3dae499e9 | Translated (0%) | Standards for commercial kitchen equipment | 商用厨房设备的标准 |
| 48184787786-6f73-4aab-b602-2dc3152522e7 | Translated (72%) | To use Table 3, the project must serve at least 100 meals per day of operation. | 要使用表 3，项目每个运营日至少必须有 100 次供餐。 |
| 48284787786-6f73-4aab-b602-2dc3152522e7 | Translated (0%) | All process and appliance equipment listed in the category of kitchen equipment and present on the project must comply with the standards. | 厨房设备目录中列出的，以及项目中存在的所有工艺和电器设备都必须符合标准。 |
| 483c0b4b25b-af38-43d3-bf87-249b0b67a229 | Translated (100%) | Kitchen equipment | 厨房设备 |
| 48497d78410-f716-4737-989e-d51c4c7e1735 | Translated (100%) | Requirement (IP units) | 要求（IP 单位） |
| 485b7962459-e4c7-4f50-87b2-db70ae67562a | Translated (100%) | Requirement (SI units) | 要求（SI 单位） |
| 4863ad8bad8-a5e8-4b62-8610-cadd18984e90 | Translated (100%) | Dishwasher | 洗碗机 |
| 4875e9073e0-31a5-4080-bf60-dcd203d4d543 | Translated (100%) | Undercounter | 台下式 |
| 488327a44a7-dccc-42d9-93c8-1ceb11b7ca3c | Translated (100%) | ENERGY STAR | 能源之星 |
| 489a602599f-b907-4458-b2c8-b08e79d587d8 | Translated (100%) | ENERGY STAR or performance equivalent | “能源之星”（ENERGY STAR） 或同等性能 |
| 490a3dd9d28-217c-4f8c-9280-7868118ff47c | Translated (100%) | Stationary, single tank, door | 固定，单缸，门式 |
| 491f5913a02-933f-4db8-a7f8-20da0126c326 | Translated (100%) | ENERGY STAR | 能源之星 |
| 492463de712-1d39-4ec2-a0df-654e911d4ff5 | Translated (100%) | ENERGY STAR or performance equivalent | “能源之星”（ENERGY STAR） 或同等性能 |
| 49384253a3a-8b29-4bd7-aaeb-4f86cadbaf62 | Translated (100%) | Single tank, conveyor | 单缸，传送式 |
| 494bc867479-aa4f-4935-99ce-ebb156adde03 | Translated (100%) | ENERGY STAR | 能源之星 |
| 4958c0b5174-b46e-4d53-a499-7afe0bcf77c9 | Translated (100%) | ENERGY STAR or performance equivalent | “能源之星”（ENERGY STAR） 或同等性能 |
| 496a23b3d8a-9709-4c88-9a3c-d9ef04c534ab | Translated (100%) | Multiple tank, conveyor | 多缸，传送式 |
| 4976e1599b0-e99c-41fd-87ba-fe956de83b79 | Translated (100%) | ENERGY STAR | 能源之星 |
| 4988db0fe42-007d-4126-acf7-68a0a95d4fc5 | Translated (100%) | ENERGY STAR or performance equivalent | “能源之星”（ENERGY STAR） 或同等性能 |
| 4998cfdb260-7e69-4826-87e5-6427101121d7 | Translated (100%) | Flight machine | 履带式 |
| 50023751e92-3535-4b9e-972d-e91772252e42 | Translated (100%) | ENERGY STAR | 能源之星 |
| 501fa019b9b-c9e3-4d6a-adf4-cdf82a1072cd | Translated (100%) | ENERGY STAR or performance equivalent | “能源之星”（ENERGY STAR） 或同等性能 |
| 502ce5980c5-7c04-495e-8762-5660ea332156 | Translated (100%) | Food steamer | 食物蒸笼 |
| 503f2fe0407-3159-4fdf-a73c-f2160b27b735 | Translated (0%) | Batch (no drain connection) | 批量（未连接排水管） |
| 50438ab3eee-cf08-46e8-b655-06016e4e3f9d | Translated (0%) | ≤ 2 gal/hour/pan including condensate cooling water | ≤ 2 加仑/小时/盘，包括冷凝冷却水 |
| 505b3c65859-7755-4d49-8fcf-eb17fe7e9931 | Translated (82%) | ≤ 7.5 liters/hour/pan including condensate cooling water | ≤ 7.5 升/小时/盘，包括冷凝冷却水 |
| 5061a0ead1b-eb14-498b-aefe-7740fd7c67c6 | Translated (0%) | Cook-to-order (with drain connection) | 按订单烹饪（已连接排水管） |
| 50720699b47-2409-40ff-99ae-a647f26dc053 | Translated (96%) | ≤ 5 gal/hour/pan including condensate cooling water | ≤ 5 加仑/小时/盘，包括冷凝冷却水 |
| 50869db4339-1e61-40aa-b379-5f4174dded35 | Translated (CM) | ≤ 19 liters/hour/pan including condensate cooling water | ≤ 19 升/小时/盘，包括冷凝冷却水 |
| 509b1732fd0-9a04-4bd4-a9af-0fa5dfb00ba9 | Translated (CM) | Combination oven, | 组合烤箱， |
| 5101e9d66f1-4fed-4988-a2b1-1a6664f8469e | Translated (CM) | Countertop or stand | 台式或直立式 |
| 511112f3093-f4e5-444b-be2f-714e37591b78 | Translated (74%) | ≤ 1.5 gal/hour/panincluding condensate cooling water | ≤ 1.5 加仑/小时/盘，包括冷凝冷却水 |
| 5120b86460f-69e8-48f8-97af-e032b9207253 | Translated (100%) | ≤ 5.7 liters/hour/pan including condensate cooling water | ≤ 5.7 升/小时/盘，包括冷凝冷却水 |
| 513ec03757e-7524-40f8-8645-f73da70df719 | Translated (100%) | Roll-in | 旋转式 |
| 514e5c1356b-f564-4401-9806-a01f9b66d981 | Translated (96%) | ≤ 1.5 gal/hour/pan including condensate cooling water | ≤ 1.5 加仑/小时/盘，包括冷凝冷却水 |
| 515e2526884-1daf-4692-b027-d004197fed05 | Translated (CM) | ≤ 5.7 liters/hour/pan including condensate cooling water | ≤ 5.7 升/小时/盘，包括冷凝冷却水 |
| 5168d623c9b-ec8c-4ea1-a9bf-48eecb35c810 | Translated (0%) | Food waste disposer | 食物残渣处理机 |
| 51769352c54-cf83-4549-822e-fd59ad84c87b | Translated (0%) | Disposer | 处理机 |
| 5185e754700-f77b-4529-81d6-51e5ac4bf684 | Translated (0%) | 3-8 gpm, full load condition, 10 minute automatic shutoff; or 1 gpm, no-load condition | 3-8 gpm，满载情况，10 分钟自动关闭；或 1 gpm，空载情况 |
| 5191d6458e1-82c1-4a53-aa2c-4ecb15d452f9 | Translated (87%) | 11–30 lpm, full load condition, 10-min automatic shutoff; or 3.8 lpm, no-load condition | 11–30 lpm，满载情况，10 分钟自动关闭；或 3.8 lpm，空载情况 |
| 5206c760acf-f928-4ec4-806e-581011e662e3 | Translated (0%) | Scrap collector | 废料收集器 |
| 52156c80138-2497-4642-8dc1-631b419c34e5 | Translated (0%) | Maximum 2 gpm makeup water | 最多 2 gpm 补充水 |
| 5222658a839-db45-41d0-961e-dfd2990bf059 | Translated (87%) | Maximum 7.6 lpm makeup water | 最多 7.6 lpm 补充水 |
| 5233e25f857-6d3e-4ac2-aeec-5c880ff8a8f7 | Translated (0%) | Pulper | 碎浆机 |
| 524fdd4cb26-d291-4e2a-9a83-2c5305a53198 | Translated (100%) | Maximum 2 gpm makeup water | 最多 2 gpm 补充水 |
| 525995df6ab-6b74-4803-82b9-2c85410aaf24 | Translated (CM) | Maximum 7.6 lpm makeup water | 最多 7.6 lpm 补充水 |
| 52667d02339-8863-4951-ac22-74b107ed6a68 | Translated (0%) | Strainer basket | 过滤篮 |
| 5275deb963c-b0c8-4322-ac95-8a1554be68e7 | Translated (0%) | No additional water usage | 不会另外使用水 |
| 528804ab95b-efdc-4d05-85e7-fbcc7ca04c7c | Translated (100%) | No additional water usage | 不会另外使用水 |
| 5297806e91c-11c8-4f1e-bfad-1acc8c72a06d | Translated (100%) | gpm = gallons per minute | gpm = 每分钟加仑数 |
| 5300e84fc14-abfe-48bc-ad54-25fcbf13168d | Translated (0%) | gph = gallons per hour | gph = 每小时加仑数 |
| 531fc78ee1c-2527-47c7-846a-b3864d0c4d8a | Translated (100%) | lpm = liters per minute | lpm = 每分钟升数 |
| 532b26426dd-c591-4859-894b-1f8a5733ef11 | Translated (0%) | lph = liters per hour | lph = 每小时升数 |
| 533e0e752cf-6b45-41ce-b563-cbcc0b103d6d | Translated (100%) | Table 4. | 表 4. |
| 534e0e752cf-6b45-41ce-b563-cbcc0b103d6d | Translated (0%) | Compliant laboratory and medical equipment | 符合要求的实验室和医疗设备 |
| 535ba14c961-0b0a-447e-ad3a-34aac0a47a7f | Translated (0%) | To use Table 4, the project must be a medical or laboratory facility. | 要使用表 4，项目必须为医疗或实验室设施。 |
| 53615c8a410-6a4d-45fd-8131-85b7198f6570 | Translated (0%) | Lab equipment | 实验室设备 |
| 537de340860-d68e-47eb-bbda-0613e3728534 | Translated (100%) | Requirement (IP units) | 要求（IP 单位） |
| 5386fa4e655-0a5c-4aa2-996b-f429ee01c980 | Translated (CM) | Requirement (SI units) | 要求（SI 单位） |
| 53998719a07-29c5-475d-a8e1-5e267e61dd53 | Translated (0%) | Reverse-osmosis water purifier | 反渗透净水器 |
| 5406a6c2d42-7107-4dea-934c-f780b1f382cb | Translated (0%) | 75% recovery | 75% 回收 |
| 541616dd357-067c-4adb-93c0-5282879abeb6 | Translated (100%) | 75% recovery | 75% 回收 |
| 542499ebedf-2ece-49ba-96cd-570ff84af929 | Translated (0%) | Steam sterilizer | 蒸汽灭菌器 |
| 54364b0332a-0ed7-4111-9c13-51ae5cb3b85a | Translated (0%) | For 60-inch sterilizer, 6.3 gal/U.S. tray | 对于 60 英寸的灭菌器，6.3 加仑/美式托盘 |
| 544606fcda1-fe07-4ab6-8d4d-c6bead753a9d | Translated (96%) | For 48-inch sterilizer, 7.5 gal/U.S. tray | 对于 48 英寸的灭菌器，7.5 加仑/美式托盘 |
| 545ba5147f0-d260-492b-9b20-60ef2c10fe3c | Translated (0%) | For 1520-mm sterilizer, 28.5 liters/DIN tray | 对于 1520 毫米的灭菌器，28.5 升/DIN 托盘 |
| 546548e6dff-ac2e-474c-97c6-c3d4d2adb954 | Translated (100%) | For 1220-mm sterilizer, 28.35 liters/DIN tray | 对于 1220 毫米的灭菌器，28.35 升/DIN 托盘 |
| 547dac58683-346c-4cd8-9849-b75018bec369 | Translated (0%) | Sterile process washer | 消毒过程洗涤器 |
| 5480f8be7eb-35c6-41d7-869d-11cf44b49a3d | Translated (0%) | 0.35 gal/U.S. tray | 0.35 加仑/美式托盘 |
| 549791bec10-32a1-48bb-9221-e3f39e14acf3 | Translated (0%) | 1.3 liters/DIN tray | 1.3 升/DIN 托盘 |
| 550136e8234-d36b-494f-a597-e0ac653dbb7c | Translated (0%) | X-ray processor, 150 mm or more in any dimension | X 射线处理器，在任意维度上均为 150 毫米或更多 |
| 551fcdc8489-6d7c-4bcf-aee2-69758bf5f725 | Translated (0%) | Film processor water recycling unit | 薄膜处理器水回收装置 |
| 552d3226f31-d35b-40ea-8474-9720e3bffcf8 | Translated (0%) | Digital imager, all sizes | 数字成像仪，所有尺寸 |
| 5539b2f2ff3-f5a0-4fd1-8f32-7f9adc34ef42 | Translated (71%) | No water use | 不用水 |
| 5548b07ddbf-9e95-4e8f-974d-934b39b6e170 | Translated (100%) | Table 5. | 表 5. |
| 5558b07ddbf-9e95-4e8f-974d-934b39b6e170 | Translated (0%) | Compliant municipal steam systems | 符合要求的市政蒸汽系统 |
| 556bd246f02-038d-4d56-b7ea-1dfef173f8bd | Translated (0%) | To use Table 5, the project must be connected to a municipal or district steam system that does not allow the return of steam condensate. | 要使用表 5，项目必须连接到不允许蒸汽冷凝水回流的市政或地区蒸汽系统。 |
| 557b0d48911-f113-42b3-8c92-310315c40c53 | Translated (0%) | Steam system | 蒸汽系统 |
| 558f74412f8-8bb9-419c-b79d-48f59bbd70e8 | Translated (0%) | Standard | 标准 |
| 559f7ea8902-9752-40d9-875e-c1be677b883d | Translated (0%) | Steam condensate disposal | 蒸汽冷凝水处理 |
| 5608b338b43-a368-4a24-a5c5-ca8d04d59656 | Translated (0%) | Cool municipally supplied steam condensate (no return) to drainage system with heat recovery system or reclaimed water | 使用热回收系统或再生水冷却排放到排水系统中的由市政提供的蒸汽冷凝水（无回流） |
| 56162fefbf0-6354-4be2-bec4-4783c94c8701 | Translated (100%) | OR | 或 |
| 562c4891539-49ab-4460-a4bd-fda7c1652a0c | Translated (0%) | Reclaim and use steam condensate | 再生并利用蒸汽冷凝水 |
| 5639848b20e-4bb6-49e8-9f09-4b73c7ff33af | Translated (0%) | 100% recovery and reuse | 100% 回收和再利用 |
| 5645898ad65-8983-455d-83f7-b1a72a1edb51 | Translated (100%) | Energy and Atmosphere | 能源与大气 (Energy and Atmosphere) |
| 565ff7e4544-17b7-4a6b-b704-649d870216b3 | Translated (70%) | EA Prerequisite: Fundamental Commissioning and Verification | EA 先决条件：基本调试和校验 (Fundamental Commissioning and Verification) |
| 5668c4a088f-8e8a-481c-9482-9a4d0c548aab | Translated (100%) | Required | 必要项 |
| 5674a1b3f56-07f8-477f-bbfe-b2932f37f509 | Translated (CM) | ID&C | ID&C |
| 568752e243e-e002-493e-b1a5-e569a9c8a657 | Translated (CM) | This prerequisite applies to | 该先决条件适用于 |
| 56986cecd39-3ebc-4bce-b65e-49811495f24c | Translated (100%) | Commercial Interiors | 商业室内 |
| 570199a3cbf-de66-49d3-a40b-4f06a9d3897e | Translated (100%) | Retail | 零售 |
| 571e490ad59-a85e-409a-82b1-a8f63e7716b4 | Translated (100%) | Hospitality | 宾馆接待 |
| 572fc8d4046-2248-4265-8088-74c3d093ad75 | Translated (100%) | Intent | 目的 |
| 573c65eb31f-0992-412b-a555-b3a04cf0f20d | Translated (0%) | To support the design, construction, and eventual operation of a project that meets the owner’s project requirements for energy, water, indoor environmental quality, and durability. | 为符合业主对项目能源、水、室内环境质量和耐久性要求的项目的设计、施工和最后运营提供支持。 |
| 574f51e4acc-e1f7-4c35-a15c-eafcd7727877 | Translated (100%) | Requirements | 要求 |
| 575c2deeb60-a349-4880-9e5f-c41c04e223c1 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 576c50de4a3-0ecd-411d-a219-0d7e1c432ac8 | Translated (0%) | Commissioning Process Scope | 调试过程范围 |
| 5776f8c3965-67cb-44c0-b67d-4d56f0a6bdb8 | Translated (0%) | Complete the following commissioning (Cx) process activities for mechanical, electrical, plumbing, and renewable energy systems and assemblies, in accordance with ASHRAE Guideline 0-2005 and ASHRAE Guideline 1.1–2007 for HVAC&R Systems, as they relate to energy, water, indoor environmental quality, and durability. | 根据适用于 HVAC&R 系统的 ASHRAE 指南 0-2005 和 ASHRAE 指南 1.1–2007（与能源、水、室内环境质量和耐久性相关）完成以下机械、电气、管道和可再生能源系统与组件的调试 (Cx) 过程活动。 |
| 57843e4a045-9a7e-437d-a351-3640efa61455 | Translated (0%) | Requirements for exterior enclosures are limited to inclusion in the owner’s project requirements (OPR) and basis of design (BOD), as well as the review of the OPR, BOD and project design. | 外部围护结构的要求仅限于业主项目要求 (OPR) 和设计基础要求 (BOD) 中包括的内容，以及对 OPR、BOD 和项目设计的审查。 |
| 57943e4a045-9a7e-437d-a351-3640efa61455 | Translated (0%) | NIBS Guideline 3-2012 for Exterior Enclosures provides additional guidance. | 适用于外围护结构的 NIBS 指南 3-2012 提供了额外的指南。 |
| 580f5987059-6b36-488e-9326-9823e5f9471a | Translated (0%) | Develop the OPR. | 制定 OPR。 |
| 5814a8099f4-562e-47ff-95ff-d9d4692ad823 | Translated (0%) | Develop a BOD. | 制定 BOD。 |
| 5823ccdd3d0-86e0-4227-8983-630614797aba | Translated (79%) | The commissioning authority (CxA) must do the following: | 调试机构 (CxA) 必须完成以下任务： |
| 583388ba8ac-9e31-4b41-84f0-c6868d411f93 | Translated (0%) | Review the OPR, BOD, and project design. | 审查 OPR、BOD 和项目设计。 |
| 5840bbce5b0-5824-4da6-808d-77dcd991d880 | Translated (89%) | Develop and implement a Cx plan. | 制定和实施 Cx 计划。 |
| 58560b358d0-9dd0-46cd-823e-4065d81fb355 | Translated (0%) | Confirm incorporation of Cx requirements into the construction documents. | 确认将 Cx 要求整合到施工文件中。 |
| 5869743baa7-f758-4755-8911-42744e51f421 | Translated (0%) | Develop construction checklists. | 制定施工清单。 |
| 587eaa14a54-1738-465f-b562-9294af66f5cf | Translated (0%) | Develop a system test procedure. | 制定系统测试程序。 |
| 588cbd93f82-5eb8-45c0-a1d2-011e86f3337b | Translated (0%) | Verify system test execution. | 验证系统测试执行。 |
| 589628d91de-6f2f-4019-825b-a5572c3b7c48 | Translated (0%) | Maintain an issues and benefits log throughout the Cx process. | 在 Cx 过程期间维护问题和益处日志。 |
| 59068cb8d26-91c6-4d03-b0bf-6b41412e4be5 | Translated (0%) | Prepare a final Cx process report. | 准备最终的 Cx 过程报告。 |
| 591499ee4d0-7c93-4852-96ad-510a0748e37c | Translated (0%) | Document all findings and recommendations and report directly to the owner throughout the process. | 在过程中记录所有结果和建议，并直接汇报给业主。 |
| 59276056e12-44e2-4da8-afa0-c45ae5dc7fa3 | Translated (0%) | The review of the exterior enclosure design may be performed by a qualified member of the design or construction team (or an employee of that firm) who is not directly responsible for design of the building envelope. | 外围护结构设计的审查可由具有资质的设计或施工团队中不直接负责建筑外围护结构设计的成员（或者该公司的员工）执行。 |
| 593a9d76e94-5d08-47bb-a187-c3e386a3b62e | Translated (100%) | Commissioning Authority | 调试计划 |
| 594d2fdc5ce-cb22-4c2e-a16d-89d29ff38c2d | Translated (0%) | By the end of the design development phase, engage a commissioning authority with the following qualifications. | 在设计开发阶段结束时，让具有以下资质的调试机构参与进来。 |
| 59541b20602-659d-47ea-9988-f393ed1936a6 | Translated (0%) | The CxA must have documented commissioning process experience on at least two building projects with a similar scope of work. | CxA 必须有证据表明至少在两个具有类似工作范围的建筑项目上执行过调试过程。 |
| 59641b20602-659d-47ea-9988-f393ed1936a6 | Translated (0%) | The experience must extend from early design phase through at least 10 months of occupancy; | 相关经验必须从早期设计阶段一直延伸到入驻至少 10 个月之后。 |
| 597f433c8b9-6a93-4cde-adab-32972b5d2cd5 | Translated (0%) | The CxA may be a qualified employee of the owner, an independent consultant, or an employee of the design or construction firm who is not part of the project’s design or construction team, or a disinterested subcontractor of the design or construction team. | CxA 可以是有资质的业主员工、独立顾问、设计或建筑公司的员工（不是项目设计或施工团队成员），或者是没有利益关系的设计或施工团队分包商。 |
| 598bfd04337-6032-4d40-92e2-2413838c52fb | Translated (0%) | For projects smaller than 20,000 square feet (1 860 square meters), the CxA may be a qualified member of the design or construction team. | 对于小于 20000 平方英尺（1860 平方米）的项目，CxA 可以是有资质的设计或施工团队成员。 |
| 599bfd04337-6032-4d40-92e2-2413838c52fb | Translated (0%) | In all cases, the CxA must report his or her findings directly to the owner. | 在任何情况下，CxA 都必须直接向业主报告他/她的结果。 |
| 60076dfb0e4-4b0a-4dca-8620-485e25b0e4af | Translated (0%) | Project teams that intend to pursue EA Credit Enhanced Commissioning should note a difference in the CxA qualifications: for the credit, the CxA may not be an employee of the design or construction firm nor a subcontractor to the construction firm. | 计划争取 EA 得分点：增强调试 (Enhanced Commissioning) 的项目团队应注意 CxA 资质的不同：对于该得分点来说，CxA 不得为设计或施工单位的员工，也不得为施工单位的分包商。 |
| 601c5983581-95a0-492f-babd-56cadc7bb082 | Translated (0%) | Current Facilities Requirements and Operations and Maintenance Plan | 当前设施要求以及运营和维护计划 |
| 6025193d3eb-caa3-4d78-ac18-863eca70be7b | Translated (0%) | Prepare and maintain a current facilities requirements and operations and maintenance plan that contains the information necessary to operate the building efficiently. | 准备并维护包含高效运营建筑所需信息的当前设施要求以及运营和维护计划。 |
| 6035193d3eb-caa3-4d78-ac18-863eca70be7b | Translated (0%) | The plan must include the following: | 计划必须包括： |
| 60446aa03fc-1245-46a9-9c2e-8168b0a88bce | Translated (0%) | a sequence of operations for the building; | 建筑运营顺序； |
| 605d5571063-f23b-41b0-b9d6-f6770d958f00 | Translated (0%) | the building occupancy schedule; | 建筑入驻安排； |
| 606dcd57e36-5dc7-4802-a1d8-e33889c99eca | Translated (0%) | equipment run-time schedules; | 设备运行时间安排； |
| 6076ffe68e4-c965-43cd-b90e-39ba87d0c521 | Translated (0%) | setpoints for all HVAC equipment; | 所有暖通空调设备的设定温度； |
| 60844b78bf4-63fa-49f1-b48f-f2191830fb9b | Translated (0%) | set lighting levels throughout the building; | 设置建筑中的照明水平； |
| 60919782144-6b48-4c73-9df7-7db7fdbff1f0 | Translated (0%) | minimum outside air requirements; | 室外空气最低要求； |
| 6102f856a84-9f61-449c-8d80-b9d547eed497 | Translated (88%) | any changes in schedules or setpoints for different seasons, days of the week, and times of day; | 不同季节、日期、时间的时间表或设定点的任何变化。 |
| 6119dded085-bf34-46f3-baa1-84e1597e3c56 | Translated (0%) | a systems narrative describing the mechanical and electrical systems and equipment; | 介绍机械和电气系统和设备的系统描述； |
| 61266831bed-da57-4653-b00f-4d0d9cab07e6 | Translated (0%) | a preventive maintenance plan for building equipment described in the systems narrative; and | 系统描述中所述建筑设备的预防性维护计划；以及 |
| 613147cbd76-7893-4323-a0a7-113724d7144c | Translated (0%) | a commissioning program that includes periodic commissioning requirements, ongoing commissioning tasks, and continuous tasks for critical facilities. | 包括关键设施定期调试要求、持续调试任务和连续任务的调试方案。 |
| 6145d2f7fa1-4f19-4042-9f3e-d967b07c2347 | Translated (0%) | Interior Design and Construction projects are responsible for completing the above tasks for all systems and equipment included in their scope, including items furnished by the base building, but modified or relocated as part of tenant fit-out. | 室内设计和施工项目负责针对范围内的所有系统和设备完成上述任务，包括基本建筑提供的物品，不包括租户装修导致的修改或搬迁。 |
| 6155d2f7fa1-4f19-4042-9f3e-d967b07c2347 | Translated (0%) | Information such as sequences of operations, schedules, equipment setpoints, and outside air requirements for tenant equipment and spaces must be coordinated with base building requirements. | 租户设备和空间的运营顺序、日程安排、设备设定点以及外部空气要求等信息都必须根据基本建筑要求进行协调。 |
| 616b8915618-59c6-4b98-bfb2-a0de5429464a | Translated (0%) | EA Prerequisite: Minimum Energy Performance | EA 先决条件：最低能源表现 (Minimum Energy Performance) |
| 617846f8b0a-82bd-4dc1-a05a-0e820003569c | Translated (100%) | Required | 必要项 |
| 618a934fb09-254f-4915-921f-ad5fe45cd7fb | Translated (CM) | ID&C | ID&C |
| 619d34a85f2-dd9f-4c17-8380-4d18920d25f9 | Translated (CM) | This prerequisite applies to | 该先决条件适用于 |
| 62035bc1ca8-2d37-440b-9614-344beb546c31 | Translated (100%) | Commercial Interiors | 商业室内 |
| 621b6c9cb41-60c8-4225-bffb-b3d608648594 | Translated (100%) | Retail | 零售 |
| 622c5f6263b-23df-49b7-b449-3103a495e603 | Translated (100%) | Hospitality | 宾馆接待 |
| 62318a3e473-ecce-48bf-8b2a-4cc38e977c73 | Translated (100%) | Intent | 目的 |
| 62487c84af2-04a9-43c8-8b04-d7e5d074499b | Translated (0%) | To reduce the environmental and economic harms of excessive energy use by achieving a minimum level of energy efficiency for the building and its systems. | 通过实现建筑及其系统的最低等级节能以减少过度使用能源带来的环境和经济危害。 |
| 625a422aca9-9f8a-4311-8a56-fa11768efb9a | Translated (100%) | Requirements | 要求 |
| 626c3b0176b-afbb-4539-83c4-b2931d227d93 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 6271f94fc90-96d9-4344-b5b3-eb6b8d5356a3 | Translated (CM) | Option 1. | 选项 1. |
| 6281f94fc90-96d9-4344-b5b3-eb6b8d5356a3 | Translated (0%) | Tenant-Level Energy Simulation | 租户级别的能耗模拟 |
| 629190eebfc-d85b-4422-a5f3-35419742f89e | Translated (0%) | Demonstrate a 3% improvement in the proposed performance rating compared with the baseline performance rating for portions of the building within the tenant’s scope of work. | 相比租户工作范围中的建筑部分的基线性能等级，建议的性能等级提高 3%。 |
| 630190eebfc-d85b-4422-a5f3-35419742f89e | Translated (0%) | Calculate the baseline according to ANSI/ASHRAE/IESNA Standard 90.1–2010, Appendix G, with errata (or a USGBC-approved equivalent standard for projects outside the U.S.), using a simulation model for all tenant project energy use. | 根据 ANSI/ASHRAE/IESNA 标准 90.1–2010，附录 G，带勘误表（对于美国以外的项目来说，为 USGBC 认可的等效标准），使用所有租户项目能耗的模拟模型来计算基线。 |
| 631126bf2bf-7fd3-466a-828b-a7a37eae62a9 | Translated (0%) | Projects must meet the minimum percentage savings before application of renewable energy systems. | 项目在使用可再生能源系统之前必须满足最低的节能百分比。 |
| 6322db9a2ad-fcd9-4ee4-b3e8-76e53b2e1944 | Translated (0%) | The proposed design must meet the following criteria: | 建议的设计必须满足以下条件： |
| 6331e299b88-38db-454d-9cb0-4037ec431b7f | Translated (0%) | compliance with the mandatory provisions of ANSI/ASHRAE/IESNA Standard 90.1–2010, with errata, or a USGBC–approved equivalent standard for projects outside the U.S.); | 符合 ANSI/ASHRAE/IESNA 标准 90.1–2010，带勘误表（对于美国以外的项目来说，为 USGBC 认可的等效标准）的强制规定； |
| 63431cfc466-e73b-4066-b5e0-a94ea8e6ed00 | Translated (0%) | inclusion of all energy consumption and costs within and associated with the tenant project; and | 包括所有租户项目内部及与其相关联的能耗和成本；以及 |
| 635f8681dd3-5f49-4f18-96c2-2a40f6e48a54 | Translated (0%) | comparison against a baseline tenant project that complies with Standard 90.1–2010, Appendix G, with errata but without addenda (or a USGBC-approved equivalent standard outside the U.S.). | 对比符合标准 90.1–2010，附录 G，带勘误表但不带附文（或者是 USGBC 认可的美国以外的等效标准）的基线租户项目。 |
| 636e61bc798-72ef-4fd1-89c3-7f6a7e1409df | Translated (0%) | Exception: the baseline project envelope must be modeled according to Table G3.1(5) (baseline), Sections a–e, and not Section f. | 例外：基线项目外围护结构必须根据表 G3.1(5)（基线），a–e 部分，而不是 f 部分来建模。 |
| 6371c476fb3-a9d1-4ae0-85ef-05fb74c39f86 | Translated (0%) | Document the energy modeling input assumptions for unregulated loads. | 记录未管制负载的能源建模输入假设。 |
| 6381c476fb3-a9d1-4ae0-85ef-05fb74c39f86 | Translated (0%) | Unregulated loads should be modeled accurately to reflect the actual expected energy consumption of the tenant project. | 应对未管制负载进行精确建模以反映租户项目实际的预期能耗。 |
| 6390c9dd2ca-1c86-48ec-b831-2e8bf319cd8c | Translated (0%) | If unregulated loads are not identical for both the baseline and the proposed performance ratings, and the simulation program cannot accurately model the savings, follow the exceptional calculation method (ANSI/ASHRAE/IESNA Standard 90.1–2010, G2.5). | 如果基线和建议性能等级的未管制负载均不相同，并且模拟项目无法准确建模节能，请按照特殊的计算方法 (ANSI/ASHRAE/IESNA 标准 90.1–2010, G2.5) 操作。 |
| 6400c9dd2ca-1c86-48ec-b831-2e8bf319cd8c | Translated (0%) | Alternatively, use the COMNET modeling guidelines and procedures to document measures that reduce unregulated loads. | 另外，使用 COMNET 建模指南和程序来记录减少未管制负载的措施。 |
| 6417bee67ec-e9e5-4a44-a177-1ac26151e28e | Translated (0%) | Retail only | 仅限零售 |
| 642a038ed08-a77d-4234-bdc6-5ed7c4859d5d | Translated (0%) | For Option 1, Tenant-Level Energy Simulation, process loads for retail may include refrigeration equipment, cooking and food preparation, clothes washing, and other major support appliances.. | 对于选项 1，租户级别的能耗模拟，零售业的工艺负载可包括制冷设备、烹饪与食物准备、洗衣和其他主要的辅助电器。 |
| 643a038ed08-a77d-4234-bdc6-5ed7c4859d5d | Translated (0%) | Many of the industry standard baseline conditions for commercial kitchen equipment and refrigeration are defined in Appendix 3, Tables 1–4. | 商用厨房设备和制冷的很多行业标准基线状况都在附录 3，表 1-4 中进行了定义。 |
| 644a038ed08-a77d-4234-bdc6-5ed7c4859d5d | Translated (0%) | No additional documentation is necessary to substantiate these predefined baseline systems as industry standard. | 无需使用额外的文件来证明这些预定义的基线系统为行业标准。 |
| 6455e9f973f-0c1e-4acc-b1e8-83d636f1661e | Translated (100%) | OR | 或 |
| 6460fde21f0-6080-433a-a7f9-94c2f3712bbb | Translated (100%) | Option 2. | 选项 2. |
| 6470fde21f0-6080-433a-a7f9-94c2f3712bbb | Translated (0%) | Prescriptive Compliance | 规范性合规 |
| 648c2c3debb-a096-4174-aff3-3023a2a3a82b | Translated (90%) | Comply with the mandatory and prescriptive provisions of ANSI/ASHRAE/IESNA Standard 90.1–2010, with errata (or a USGBC-approved equivalent standard for projects outside the U.S.). | 符合 ANSI/ASHRAE/IESNA 标准 90.1–2010，带勘误表（对于美国以外的项目来说，为 USGBC 认可的等效标准）的强制和规范性规定。 |
| 649b731d336-5faa-4030-9a8b-12766d7f11c8 | Translated (0%) | Reduce connected lighting power density by 5% below ASHRAE 90.1-2010 using the space-by-space method or by applying the whole-building lighting power allowance to the entire tenant space. | 使用逐个空间的方法或对整个承租空间使用整栋建筑的照明功率容限，将相关的照明功率密度降至比 ASHRAE 90.1-2010 标准允许的值低 5%。 |
| 65064dcf050-c3bf-47a8-8b99-ecf0bc42d454 | Translated (0%) | Install ENERGY STAR appliances, office equipment, electronics, and commercial food service equipment (HVAC, lighting, and building envelope products are excluded) for 50% (by rated-power) of the total ENERGY STAR eligible products in the project. | 为项目中所有符合“能源之星”（ENERGY STAR）标准的产品中的 50%（按额定功率）安装“能源之星”（ENERGY STAR）电器、办公设备、电子产品和商用食品服务设备（不包括暖通空调、照明和建筑外围护结构产品）。 |
| 65164dcf050-c3bf-47a8-8b99-ecf0bc42d454 | Translated (73%) | Projects outside the U.S. may use a performance equivalent to ENERGY STAR. | 美国以外的项目可以使用与“能源之星”（ENERGY STAR）类似的标准。 |
| 65297447e1f-b98c-46aa-a325-2d71015c8fc5 | Translated (74%) | EA Prerequisite: Fundamental Refrigerant Management | EA 先决条件: 基础冷媒管理 (Fundamental Refrigerant Management) |
| 653a41941d1-5f0c-4417-aea1-4d937096f50e | Translated (100%) | Required | 必要项 |
| 654097eb234-dd24-4740-b2cd-1a64a5b73b19 | Translated (CM) | ID&C | ID&C |
| 655fff1c325-0eaa-4f80-9920-afd25e2be31d | Translated (CM) | This prerequisite applies to | 该先决条件适用于 |
| 656d2ed0fcb-b683-4907-9fbd-2dbed0983a9e | Translated (100%) | Commercial Interiors | 商业室内 |
| 6571b040b65-332d-4a17-b9ce-4a5e53448bdd | Translated (100%) | Retail | 零售 |
| 658a578ad3a-65b2-4c73-8b6e-7bd7102f34d8 | Translated (100%) | Hospitality | 宾馆接待 |
| 659701bd42d-ff24-40c6-b213-573a0d17711e | Translated (100%) | Intent | 目的 |
| 660e50fc3ff-2827-4430-8e2d-35c21c9ab065 | Translated (99%) | To reduce stratospheric ozone depletion. | 减少对平流层臭氧的消耗。 |
| 66185b20785-f3a1-4066-aa80-2a3ffb70d59a | Translated (100%) | Requirements | 要求 |
| 6621d522305-e6c9-406a-b7cd-fd4f452dd2ec | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 66324770d98-dc76-44ff-a86c-fcecdee1ca79 | Translated (0%) | Do not use chlorofluorocarbon (CFC)-based refrigerants in new heating, ventilating, air-conditioning, and refrigeration (HVAC&R) systems. | 在新的供暖、通风、空调和制冷 (HVAC&R) 系统中不使用氯氟化碳型冷媒制冷机。 |
| 66424770d98-dc76-44ff-a86c-fcecdee1ca79 | Translated (0%) | When reusing existing HVAC&R equipment, complete a comprehensive CFC phase-out conversion before project completion. | 再利用既有 HVAC&R 设备时，在项目完成之前进行全面的 CFC 淘汰改造。 |
| 66524770d98-dc76-44ff-a86c-fcecdee1ca79 | Translated (99%) | Phase-out plans extending beyond the project completion date will be considered on their merits. | 超出项目完成日期的淘汰方案将酌情考虑。 |
| 6668202e90d-7ad1-429e-b526-13d750c1bf60 | Translated (0%) | Existing small HVAC&R units (defined as containing less than 0.5 pound [225 grams] of refrigerant) and other equipment, such as standard refrigerators, small water coolers, and any other equipment that contains less than 0.5 pound (225 grams) of refrigerant, are exempt. | 现有的小型 HVAC&R 装置（定义为包含的制冷剂不足 0.5 磅 [225 克]）和其他设备，如标准冰箱、小型水冷却器和其他任何含有不足 0.5 磅 [225 克] 制冷剂的设备，均不包括在内。 |
| 667441cb5e0-90a7-411f-9814-68759fd47252 | Translated (0%) | EA Credit: Enhanced Commissioning | EA 得分点: 增强调试 (Enhanced Commissioning) |
| 668f22da267-19b6-4e7c-ae63-70bf372996c3 | Translated (100%) | ID&C | ID&C |
| 6695ef06aa4-b2c7-4f96-bd66-674033f908e9 | Translated (100%) | 4-5 points | 4-5 分 |
| 670b399b4fe-b964-4ec5-b1e2-a85315b8343f | Translated (100%) | This credit applies to | 该得分点适用于 |
| 67199ca8526-be76-4a89-b422-3a05c03e1e85 | Translated (100%) | Commercial Interiors (4-5 points) | 商业室内（4-5 分） |
| 672c8352a7b-5b34-4e41-ab6c-37757c2ea104 | Translated (100%) | Retail (4-5 points) | 零售（4-5 分） |
| 673bbcec184-cea7-4f4d-bedb-a32527e661c3 | Translated (100%) | Hospitality (4-5 points) | 宾馆接待（4-5 分） |
| 67431b68edf-a86e-451d-b274-cb5a5195fe8a | Translated (100%) | Intent | 目的 |
| 67528ef3595-8423-4eb2-bf06-e2a0fd5d0e0b | Translated (98%) | To further support the design, construction, and eventual operation of a project that meets the owner’s project requirements for energy, water, indoor environmental quality, and durability. | 为符合业主对项目能源、水、室内环境质量和耐久性要求的项目的设计、施工和最后运营提供进一步支持。 |
| 6766c6811f5-17d3-47d1-8acc-585c5d74a631 | Translated (100%) | Requirements | 要求 |
| 67722be9198-f06d-487a-853c-912c9c00b1e5 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 67818a7c31b-c395-4170-bb1c-4b138b966270 | Translated (CM) | Option 1. | 选项 1. |
| 67918a7c31b-c395-4170-bb1c-4b138b966270 | Translated (0%) | Enhanced Commissioning (4 points) | 增强调试（4 分） |
| 6802460a7d3-cae6-4560-b570-820107fe37c8 | Translated (89%) | Projects must complete the following commissioning process (CxP) activities for mechanical, electrical, domestic hot water, and renewable energy systems and assemblies in accordance with ASHRAE Guideline 0–2005 and ASHRAE Guideline 1.1–2007 for HVAC&R systems, as they relate to energy, water, indoor environmental quality, and durability. | 项目必须根据适用于 HVAC&R 系统的 ASHRAE 指南 0-2005 和 ASHRAE 指南 1.1–2007（与能源、水、室内环境质量和耐久性相关）完成以下机械、电气、生活热水和可再生能源系统与组件的调试过程 (CxP) 活动。 |
| 681ab4018cd-fa27-4dbf-94f4-189ec4e1042c | Translated (0%) | Review contractor submittals. | 审查承建商递交的材料。 |
| 682be030a0f-87b4-48bd-adcd-5f6859fa9e8b | Translated (73%) | Include systems manual requirements in construction documents. | 在施工文件中包括系统手册要求。 |
| 6832934f3ae-3709-4e87-8bd3-b27c4ec1e641 | Translated (0%) | Include operator and occupant training requirements in construction documents. | 将运营商和驻户培训要求包含在施工文件中。 |
| 6841e1a847a-9d74-42e5-ab0c-064429cc511f | Translated (0%) | Verify systems manual updates and delivery. | 验证系统手册更新与交付。 |
| 685beceab2d-3a9c-4555-a7c7-5c1f0adaf151 | Translated (0%) | Verify operator and occupant training delivery and effectiveness. | 验证运营商和驻户培训交付和有效性。 |
| 686c4abf70f-9963-4b11-93f6-bc7cb4ea2ba8 | Translated (0%) | Verify seasonal testing. | 验证季节测试。 |
| 687cb332724-81ff-40e0-87be-cdc203dda1ec | Translated (0%) | Review building operations 10 months after substantial completion. | 在实质性完成之后 10 个月审查建筑运营。 |
| 6887f39592e-872b-4129-a25a-9a5f36c03769 | Translated (0%) | Develop an on-going commissioning plan. | 制定持续的调试计划。 |
| 6894dd1372c-e9f9-49f1-b0e4-781b6687e14f | Translated (100%) | OR | 或 |
| 690f9e858de-1343-4a75-8aab-6b221243f373 | Translated (CM) | Option 2. | 选项 2. |
| 691f9e858de-1343-4a75-8aab-6b221243f373 | Translated (84%) | Monitoring-Based Commissioning (5 points) | 基于监测的调试（5 分） |
| 69207d5759e-87c5-40a5-bdad-9f17663fce4f | Translated (71%) | Achieve Option 1. | 实现选项 1. |
| 6930538089b-224c-4827-9a16-95e4af889f1e | Translated (0%) | Develop monitoring-based procedures and identify points to be measured and evaluated to assess performance of energy- and water-consuming systems. | 制定基于监测的程序并确定要测量和评估的分数，以评估耗能和用水系统的性能。 |
| 694ff990bfa-3187-4e28-bc25-6dea0c41d3f1 | Translated (0%) | Include the procedures and measurement points in the commissioning plan. | 将程序和测量分数包含在调试计划内。 |
| 695ff990bfa-3187-4e28-bc25-6dea0c41d3f1 | Translated (70%) | Address the following: | 说明以下内容： |
| 69633c8251f-9118-4e14-bb1a-949fc8e5d7b7 | Translated (0%) | roles and responsibilities; | 角色与职责； |
| 6977e55427f-daa1-47fb-b14e-34cecbabc4c1 | Translated (0%) | measurement requirements (meters, points, metering systems, data access); | 测量要求（仪表、分数、计量系统、数据访问）； |
| 698d07fcfd9-2998-4d7a-ad79-34f3f1f8cf94 | Translated (0%) | the points to be tracked, with frequency and duration for trend monitoring; | 要跟踪的分数，以及趋势监测的频率和持续时间； |
| 6994dbf450d-f753-47e6-90dc-483314953652 | Translated (0%) | the limits of acceptable values for tracked points and metered values (where appropriate, predictive algorithms may be used to compare ideal values with actual values); | 可接受的跟踪分数和计量值限制（如果合适的话，可以使用预测算法来比较理想值和实际值）； |
| 700663ebeec-8b5a-486f-8e4e-bb3a069b0ae6 | Translated (0%) | the elements used to evaluate performance, including conflict between systems, out-of-sequence operation of systems components, and energy and water usage profiles; | 用于评估性能的元素，包括系统之间的冲突、系统组件的失序运营，以及能源和水的使用情况； |
| 701dc861e31-8638-44e2-9043-6e72bf706fd3 | Translated (0%) | an action plan for identifying and correcting operational errors and deficiencies; | 确定和纠正运营错误和不足的行动计划； |
| 702edc1f6c8-48ad-4637-9be9-5aedda9373da | Translated (0%) | training to prevent errors; | 培训以防止错误； |
| 703132a52a6-0bfe-4e8a-9939-3615db152753 | Translated (0%) | planning for repairs needed to maintain performance; | 计划保持性能所需的维修； |
| 704202b8405-abc5-4672-a535-b07d515f7bb5 | Translated (0%) | the frequency of analyses in the first year of occupancy (at least quarterly); | 入驻第一年的分析频率（至少每季度）； |
| 705f1f0cd5f-e408-4858-b3e3-f7f46efdc53c | Translated (0%) | Update the systems manual with any modifications or new settings, and give the reason for any modifications from the original design. | 更新系统手册，加入修改或新设置，并说明修改原设计的原因。 |
| 70673db6d5f-b998-40ba-856c-f50657a217c9 | Translated (85%) | EA Credit: Optimize Energy Performance | EA 得分点：能源效率优化 (EA Credit: Optimize Energy Performance) |
| 7079875781e-dcbb-46eb-a9f3-676e82a7b665 | Translated (100%) | ID&C | ID&C |
| 70835dd16d1-0a85-4e0a-9fa8-911a39413a5f | Translated (100%) | 1–25 points | 1-25 分 |
| 7090538da03-2592-4763-a2b6-03654dd10cdf | Translated (100%) | This credit applies to | 该得分点适用于 |
| 7109072ed71-f341-429f-a85c-258dfd41d8eb | Translated (100%) | Commercial Interiors (1–25 points) | 商业室内（1-25 分） |
| 711b85bd3e8-8af6-4bf4-89ef-eaf729639f11 | Translated (CM) | Retail (1–25 points) | 零售（1-25 分） |
| 712b811e542-6ec9-4f73-9dad-dcadad1a0d5f | Translated (99%) | Hospitality(1–25 points) | 宾馆接待（1-25 分） |
| 713bb05a9c5-7d0d-431c-bdbc-67868fe579e0 | Translated (100%) | Intent | 目的 |
| 714e8bbac22-cc46-47f5-97a0-b6d16c1a956a | Translated (97%) | To achieve increasing levels of energy performance beyond the prerequisite standard to reduce environmental and economic harms associated with excessive energy use. | 实现比先决条件要求更高的节能等级，以减少能源过量使用所引发的环境和经济危害。 |
| 7157c778d68-ce33-4c2e-9636-9e89dd3d1406 | Translated (100%) | Requirements | 要求 |
| 71692c8e31e-41e6-440a-bbcd-6657ac6187b9 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 717eec65f4d-9742-48cb-add6-abd0f2591a3f | Translated (0%) | Establish an energy performance target no later than the schematic design phase. | 在设计企划期间或之前建立能源表现目标。 |
| 718eec65f4d-9742-48cb-add6-abd0f2591a3f | Translated (0%) | The target must be established as KBtu per square foot-year (kW per square meter-year) of source energy use. | 建立的目标必须以能耗的每平方英尺年 KBtu（每平方米年 kW）为单位。 |
| 719651c1a97-9f32-4022-bfce-cb55a01490c4 | Translated (0%) | Select one of the options below. | 选择以下选项之一。 |
| 720792b253c-e3d4-4214-8641-e3445e6abe0c | Translated (100%) | Option 1. | 选项 1. |
| 721792b253c-e3d4-4214-8641-e3445e6abe0c | Translated (0%) | Tenant-Level Energy Simulation (1–25 points) | 租户级别的能耗模拟（1-25 分） |
| 722a8d692e1-bff2-413b-977a-5e797adaf8cb | Translated (0%) | Analyze efficiency measures during the design process and account for the results in design decision-making. | 分析设计阶段的效率措施，有助于设计决策制定。 |
| 723a8d692e1-bff2-413b-977a-5e797adaf8cb | Translated (0%) | Analysis can include energy simulation of efficiency opportunities, energy simulation analyses for similar projects, or published data from energy analyses performed for similar projects (such as AEDGs). | 分析可包括对节能机会的能耗模拟、对类似项目的能耗模拟分析，或者是为类似项目执行的能耗分析公布的数据（如 AEDG）。 |
| 724c23dc9f6-b9a8-479f-b6a6-8f018f46e4aa | Translated (0%) | Analyze efficiency measures focused on load reduction and HVAC-related strategies; passive measures are acceptable. | 分析专注于降低负载和暖通空调相关策略的节能措施；接受被动措施。 |
| 725c23dc9f6-b9a8-479f-b6a6-8f018f46e4aa | Translated (0%) | Project the potential energy savings and cost implications for all affected systems. | 预测所有受影响系统的潜在节能和成本影响。 |
| 72641465868-44ac-4118-b50f-b6018ed9d090 | Translated (0%) | Follow the criteria in EA Prerequisite Minimum Energy Performance to demonstrate a percentage improvement in the proposed tenant project performance rating compared with the baseline. | 按照 EA 先决条件：最低能源表现 (Minimum Energy Performance) 中的标准来展示建议的租户项目性能等级高于基线的百分比。 |
| 727e08214ef-e728-42f9-ad9a-739f687943de | Translated (100%) | Table 1. | 表 1. |
| 728e08214ef-e728-42f9-ad9a-739f687943de | Translated (0%) | Points for percentage improvement in energy performance | 能源表现百分比提高的分数 |
| 729dcf79a9e-c450-4eb4-b3e0-1a7f2eef3b74 | Translated (0%) | Interior construction | 室内施工 |
| 73098c5f9d4-f400-468d-a5af-395d46cd3592 | Translated (100%) | Points | 分数 |
| 731c1c2117d-afc8-4d7e-866f-7ac8fdf6cf81 | Translated (100%) | 4% | 4% |
| 7325c6f4c68-5aec-4bb5-8223-aa5b8bf9b07d | Translated (100%) | 4 | 4 |
| 733b53a72b5-e04d-4432-8ca3-0481a1c9af1e | Translated (100%) | 5% | 5% |
| 734c987d42c-aec6-414e-b28f-3f08d2669aed | Translated (100%) | 6 | 6 |
| 73537cc0584-8794-4b66-851f-22eead8ff9d9 | Translated (100%) | 6% | 6% |
| 7366b571561-905f-4865-80c5-640632e6965c | Translated (100%) | 8 | 8 |
| 737ac2de9ab-90a2-4cbe-b3ad-8c5fc65bbd56 | Translated (100%) | 7% | 7% |
| 73834bfc21f-0d0e-44cc-8368-1ef3a5886415 | Translated (100%) | 10 | 10 |
| 73956890b89-2eb4-43ed-8897-5d9c85c5d7fe | Translated (100%) | 8% | 8% |
| 740d026f31a-c329-4ecd-9865-2d2b0fbdbfba | Translated (100%) | 11 | 11 |
| 741e0568e0e-3fe2-44b6-bc12-fc3c661b1b9b | Translated (100%) | 9% | 9% |
| 742721b857f-8994-4d41-932f-397b3948319e | Translated (100%) | 12 | 12 |
| 7432d83276a-27bd-4d1b-a9ae-d205394adf7a | Translated (100%) | 10% | 10% |
| 744ad22ef0e-e7c5-4a24-9d26-a64e277171fe | Translated (100%) | 13 | 13 |
| 745e7c4f379-06d9-4540-a773-713d67f553c8 | Translated (100%) | 11% | 11% |
| 7468d2b8c59-1579-4273-9a2c-9bac0959d7fa | Translated (100%) | 14 | 14 |
| 74798c86ce2-13c6-4113-8a77-cecab898fd1d | Translated (100%) | 12% | 12% |
| 74804a890c2-646b-4b24-a06c-a635387bb34b | Translated (100%) | 15 | 15 |
| 749838a4e94-7e4c-4f20-81e9-3bce5c5a4039 | Translated (100%) | 13% | 13% |
| 7506ff932f9-7d58-4308-9fdc-b1824539b319 | Translated (100%) | 16 | 16 |
| 7519fec7f86-b0fb-4f6f-9590-362001a58298 | Translated (100%) | 14% | 14% |
| 7527cb95178-c3ab-4cd9-bbef-d6c59717d16c | Translated (100%) | 17 | 17 |
| 75305323e5a-c806-4734-87a5-f286b05bdd0d | Translated (100%) | 15% | 15% |
| 754b24cf60a-74f2-4adf-a6e3-b0880eb32201 | Translated (100%) | 18 | 18 |
| 755befbe909-ccc7-4393-9d9a-2fb89914f35c | Translated (100%) | 16% | 16% |
| 75630bd152c-eb8f-445b-a2dc-8930536c3c5a | Translated (100%) | 19 | 19 |
| 7578a2dec0d-9c32-4892-a1e8-64f6e78d200e | Translated (100%) | 17% | 17% |
| 7583647475b-a87a-4ddc-98f2-a0080217dc50 | Translated (100%) | 20 | 20 |
| 759f15aa1f1-20d1-430a-9d0c-ba8a6be7c8bb | Translated (100%) | 18% | 18% |
| 760681c820c-64b4-4db1-9800-2a48c8924090 | Translated (100%) | 21 | 21 |
| 761cdf0eb9b-5915-4954-8acf-a30bea167977 | Translated (100%) | 20% | 20% |
| 7627739f4b0-28a3-4775-86d1-a8a056a71e21 | Translated (100%) | 22 | 22 |
| 76335279279-11ce-4966-8a59-bb2f3d197f04 | Translated (100%) | 22% | 22% |
| 764d1556614-0a13-4699-8739-5b66e7fda83c | Translated (100%) | 23 | 23 |
| 76562a14d75-4c35-4e1b-8f3e-30a0ebe3f7cd | Translated (100%) | 24% | 24% |
| 766e813cc46-ebb2-4860-aaf7-6eb6812d6e50 | Translated (100%) | 24 | 24 |
| 767e5c54170-ae1e-41dc-a786-7615f45184a7 | Translated (100%) | 28% | 28% |
| 768fb9f75b4-9c1c-4aee-ad09-fda54f4776d5 | Translated (100%) | 25 | 25 |
| 769cf2a15cb-427c-422b-9b5a-e0dfb56ab994 | Translated (100%) | Retail only | 仅限零售 |
| 770e4beec55-5591-4ef4-9618-bd2e0599c271 | Translated (0%) | For all process loads, define a clear baseline to compare with proposed improvements. | 对于所有的工艺负载，定义明确的基线以与提议的改进作比较。 |
| 771e4beec55-5591-4ef4-9618-bd2e0599c271 | Translated (0%) | The baselines in Appendix 3, Tables 1–4, represent industry standards and may be used without additional documentation. | 附录 3，表 1–4 中的基线代表行业标准，无需额外提供文件证明即可使用。 |
| 772e4beec55-5591-4ef4-9618-bd2e0599c271 | Translated (0%) | Calculate the baseline and design as follows: | 按照以下方式计算基线和设计： |
| 7733dba72aa-24d6-4e2c-afdc-282ab0262d66 | Translated (74%) | <1186>Appliances and equipment</1186><1187>.</1187> | 电器和设备。 |
| 7743dba72aa-24d6-4e2c-afdc-282ab0262d66 | Translated (0%) | For appliances and equipment not covered in Appendix 3, Tables 1–4 indicate hourly energy use for proposed and budget equipment, along with estimated daily use hours. | 对于附录 3，表 1-4 中未涵盖的电器和设备，表明建议和符合预算的设备每小时的能耗，以及预估的每天使用时间。 |
| 7753dba72aa-24d6-4e2c-afdc-282ab0262d66 | Translated (0%) | Use the total estimated appliance/equipment energy use in the energy simulation model as a plug load. | 在能耗模拟模型中使用预估的电器/设备总能耗作为插座负荷。 |
| 7763dba72aa-24d6-4e2c-afdc-282ab0262d66 | Translated (0%) | Reduced use time (schedule change) is not a category of energy improvement in this credit. | 减少使用时间（计划更改）在该得分点中不是能源改善的类别。 |
| 7773dba72aa-24d6-4e2c-afdc-282ab0262d66 | Translated (0%) | ENERGY STAR ratings and evaluations are a valid basis for performing this calculation. | “能源之星”(ENERGY STAR) 等级和评估是执行该计算的有效依据。 |
| 77879559bc8-02b8-4a98-858c-77e8d22ca891 | Translated (0%) | <1189>Display lighting</1189><1190>.</1190> | 显示照明。 |
| 77979559bc8-02b8-4a98-858c-77e8d22ca891 | Translated (0%) | For display lighting, use the space by space method of determining allowed lighting power under ANSI/ASHRAE/IESNA Standard 90.1–2010, with errata (or a USGBC-approved equivalent standard for projects outside the U.S.), to determine the appropriate baseline for both the general building space and the display lighting. | 对于显示照明，根据 ANSI/ASHRAE/IESNA 标准 90.1–2010，带勘误表（对于美国以外的项目来说，为 USGBC 认可的等效标准）使用确定照明功率容限的逐个空间方法，来确定同时适合常规建筑空间和显示照明的基线。 |
| 78060ff7b6f-29fd-479d-b047-f594fca5a15a | Translated (94%) | <1192>Refrigeration</1192><1193>.</1193> | 制冷。 |
| 78160ff7b6f-29fd-479d-b047-f594fca5a15a | Translated (0%) | For hard-wired refrigeration loads, model the effect of energy performance improvements with a simulation program designed to account for refrigeration equipment. | 对于硬连线的制冷负荷，通过设计用于说明制冷设备的模拟程序来对能源表现改善的效果建模。 |
| 782b590a8d9-5781-4a77-8a31-969b3e4ce978 | Translated (100%) | Option 2. | 选项 2. |
| 783b590a8d9-5781-4a77-8a31-969b3e4ce978 | Translated (0%) | Prescriptive Compliance (1–16 points) | 规范性合规（1-16 分） |
| 7846eb5ccba-ac44-4b55-9ef3-ade97a054d5f | Translated (0%) | Use any combination of the strategies in any or all of the categories below. | 使用以下任意或全部类别的任意策略组合。 |
| 7857c639bfd-3ee7-41e5-944a-a6ebf443f957 | Translated (0%) | Base Building Systems (2-6 points) | 基本建筑系统（2-6 分） |
| 786b4a62e46-177f-4c8e-ac9e-16315147589e | Translated (0%) | For base building systems that serve the project, as well as any applicable improvements that are part of the project, document compliance with the following according to base building type and climate zone. | 对于为项目提供服务的基本建筑系统，以及项目中的任何适用改进，根据基本建筑类型和气候区证明符合以下内容。 |
| 787b4a62e46-177f-4c8e-ac9e-16315147589e | Translated (87%) | For projects outside the U.S., consult ASHRAE/ASHRAE/IESNA Standard 90.1–2010, Appendixes B and D, to determine the appropriate climate zone. | 美国以外的项目可以参考 ASHRAE/ASHRAE/IESNA 标准 90.1-2010 附录 B 和 D 来确定合适的气候区。 |
| 7887023e0ca-1f12-4f70-82ab-99c8f4d26810 | Translated (0%) | Building Envelope, Opaque (2 points) | 建筑外围护结构，不透明（2 分） |
| 7891849f38d-9c5d-4f50-afa7-008fc782fdc2 | Translated (0%) | Comply with the recommendations in the appropriate ASHRAE 50% Advanced Energy Design Guide for all roofs, walls, floors, slabs, doors, vestibules, and continuous air barriers. | 符合适当的 ASHRAE 50% 高级能源设计指南中有关所有屋面、墙壁、地板、板材、门、前庭和连续空气屏障的建议。 |
| 79053e4001e-136a-4bd0-87fe-c4346859556c | Translated (87%) | Building Envelope, Glazing (2 points) | 建筑外围护结构，玻璃（2 分） |
| 7917619c67f-a084-455f-af42-7e016fe824df | Translated (0%) | Comply with the recommendations in the appropriate ASHRAE 50% Advanced Energy Design Guide for all vertical fenestration. | 符合适当的 ASHRAE 50% 高级能源设计指南中有关所有垂直开窗的建议。 |
| 7925afe98a7-183e-41e5-9686-32a6ae57e713 | Translated (0%) | HVAC Equipment Efficiency (2 points) | 暖通空调设备效率（2 分） |
| 793f1edae24-ad86-452b-b386-778b2b559300 | Translated (0%) | For all base building HVAC systems that serve the project, comply with the recommendations in the appropriate ASHRAE 50% Advanced Energy Design Guide. | 对于为项目提供服务的所有基本建筑暖通空调系统，符合适当的 ASHRAE 50% 高级能源设计指南中的建议。 |
| 79437aa899d-7f6b-4e13-bbcb-30fd77e6b178 | Translated (76%) | HVAC Systems (2 points) | 暖通空调系统（2 分） |
| 7954b2e503c-0ae7-4bb8-815c-84a48c48cc17 | Translated (0%) | HVAC Zoning and Controls (2 points) | 暖通空调分区和控制（2 分） |
| 7961d428670-3421-4767-917e-5ca6d6deb05b | Translated (0%) | For the tenant fit-out of spaces, provide a separate control zone for each solar exposure and interior space. | 对于租户的空间装修，为每个日光直射和室内空间提供单独的控制区。 |
| 7971d428670-3421-4767-917e-5ca6d6deb05b | Translated (0%) | Provide controls capable of sensing space conditions and modulating the HVAC system in response to space demand for all private offices and other enclosed spaces (e.g., conference rooms, classrooms). | 提供能够在所有私人办公室和其他封闭空间（如会议室、教室）中感应空间状况并根据空间需求调整暖通空调系统的控制装置。 |
| 798365b62ac-8b47-43bf-a04c-417596119e86 | Translated (0%) | Interior Lighting Power (1–4 points) | 室内照明功率（1-4 分） |
| 79947fca56d-833b-44ad-ae48-bcdf16db5534 | Translated (75%) | Lighting Power Density (1–4 points) | 照明功率密度（1-4 分） |
| 800a2ab8bca-c1b1-41e5-b512-3477cf045292 | Translated (80%) | Reduce connected lighting power density below that allowed by ASHRAE/IESNA Standard 90.1–2010, either using the space-by-space method or applying the whole-building lighting power allowance to the entire tenant space. | 使用逐个空间的方法或对整个承租空间使用整栋建筑的照明功率容限，降低相关的照明功率密度，使其比 ASHRAE/IESNA 90.1-2010 标准允许的值低。 |
| 801a2ab8bca-c1b1-41e5-b512-3477cf045292 | Translated (100%) | Points are awarded according to Table 2. | 根据表 2 获得分数。 |
| 80245522101-6c44-48d9-afdf-7dadf45ba157 | Translated (CM) | Table 2. | 表 2. |
| 80345522101-6c44-48d9-afdf-7dadf45ba157 | Translated (0%) | Points for percentage reduction in lighting power density | 照明功率密度百分比减少的分数 |
| 804ca36431e-6c2a-48f6-929e-d1a7dfeab390 | Translated (0%) | Percentage below standard LPD | 低于标准 LPD 的百分比 |
| 805a745e119-ab86-460f-874d-f114fa81d03e | Translated (100%) | Points | 分数 |
| 806857ca769-d61d-403c-a2c5-57a58a08c06c | Translated (100%) | 10% | 10% |
| 807f8a7f435-2e36-4584-92d4-6b0d45011229 | Translated (100%) | 1 | 1 |
| 808c3376c3e-f048-4c94-9804-e026ffed11e6 | Translated (100%) | 15% | 15% |
| 80941226a5b-1887-474e-a2d2-3ac73b5cb141 | Translated (100%) | 2 | 2 |
| 8109f920c11-d967-4f3f-96f3-2a7a23b2e0f8 | Translated (100%) | 20% | 20% |
| 8116338916f-6389-4a4c-9b25-a335a8a4a793 | Translated (100%) | 3 | 3 |
| 81246bd9218-274a-48fa-aa44-d64cca3aad01 | Translated (100%) | 25% | 25% |
| 8130067995a-90e6-4a87-a85f-d28f7e082aa6 | Translated (100%) | 4 | 4 |
| 81493328758-454c-4b16-8504-c770917718cd | Translated (89%) | Interior Lighting Controls (1–2 points) | 室内照明控制（1-2 分） |
| 815b086b96b-edad-4b12-a249-ff22f7ed1889 | Translated (0%) | Daylighting Controls (1 point) | 自然采光控制（1 分） |
| 8163c1c4264-d2e7-4c1c-b4fc-75c653dff05e | Translated (0%) | Install daylight-responsive controls in all regularly occupied daylit spaces within 15 feet (4.5 meters) of windows and under skylights for at least 25% of the connected lighting load. | 在距离窗户 15 英尺（4.5 米）之内以及天窗下方所有有自然采光的常用空间中，为至少 25% 的相关照明负荷安装自然采光响应控制装置。 |
| 8173c1c4264-d2e7-4c1c-b4fc-75c653dff05e | Translated (0%) | Daylight controls must switch or dim electric lights in response to daylight illumination in the space. | 自然采光控制装置必须根据空间中的自然采光照度来开关或调暗电灯。 |
| 818f066c1dc-bab4-48c5-ad47-c7d4da9e0c92 | Translated (0%) | Occupancy Sensor Lighting Controls (1 point) | 用户感应器照明控制（1 分） |
| 81939228979-11e6-4a15-9a33-5719d8ec789b | Translated (87%) | Install occupancy sensors for at least 75% of the connected lighting load. | 至少为 75% 的相关照明负荷安装用户感应器。 |
| 820d6db6071-09ac-4ebf-8c2e-f8118248c6ca | Translated (0%) | Equipment and Appliances (1–2 points) | 设备与电器（1-2 分） |
| 8217e952553-7c03-4c7f-a4ff-a0e4ae7223f0 | Translated (80%) | ENERGY STAR Equipment and Appliances (1–2 points) | “能源之星”（ENERGY STAR）设备与电器（1-2 分） |
| 822fb4ace8c-7c8b-4da6-a955-e8d977218882 | Translated (74%) | Install ENERGY STAR appliances, office equipment, electronics, and commercial food service equipment (HVAC, lighting, and building envelope products are excluded from this credit) or performance equivalent for projects outside the U.S.. | 安装“能源之星”（ENERGY STAR）电器、办公设备、电子产品和商用食品服务设备（本得分点不包括暖通空调、照明和建筑外围护结构产品），美国以外的项目需安装性能相当的设备。 |
| 823fb4ace8c-7c8b-4da6-a955-e8d977218882 | Translated (0%) | Calculate their percentage of the total (by rated-power) ENERGY STAR–eligible products in the project. | 计算项目中所有符合“能源之星”（ENERGY STAR）的产品的百分比（按照额定功率）。 |
| 824fb4ace8c-7c8b-4da6-a955-e8d977218882 | Translated (100%) | Points are awarded according to Table 3. | 根据表 3 获得分数。 |
| 825dbbdc251-dff6-4c7a-bb80-c4f4568bd381 | Translated (CM) | Table 3. | 表 3. |
| 826dbbdc251-dff6-4c7a-bb80-c4f4568bd381 | Translated (0%) | Points for installing ENERGY STAR equipment and appliances | 安装“能源之星”（ENERGY STAR）设备和电器的分数 |
| 827e206374a-8a8a-4872-a5f3-68cef472fe09 | Translated (0%) | Percentage of ENERGY STAR products | “能源之星”（ENERGY STAR）产品的百分比 |
| 82867537818-e3a0-45b5-b5df-4ad68259e589 | Translated (100%) | Points | 分数 |
| 829e22a37a1-fdca-4f9d-a7c6-78163e19a6b3 | Translated (100%) | 70% | 70% |
| 8306fd4d96f-9540-484a-a8b5-2888c179f97a | Translated (100%) | 1 | 1 |
| 831bddb5da0-b7fc-4585-88a4-8a1c393fb16a | Translated (100%) | 90% | 90% |
| 832c6d455e8-9323-4929-9b1c-0c56af67aeb1 | Translated (100%) | 2 | 2 |
| 833afbdd3b6-6389-4e4d-9a7c-b884947ee68f | Translated (100%) | Retail only | 仅限零售 |
| 834bebea62c-79cc-43fa-92a3-ce7124e00784 | Translated (0%) | All projects pursuing Option 2 must also comply with the prescriptive measures in Appendix 3, Tables 1–4, for 90% of total energy consumption for all process equipment. | 对于所有争取选项 2 的项目，其所有工艺设备总能耗的 90% 都必须符合附录 3，表 1-4 中的规范性指标。 |
| 8355d342ef2-f971-4dbc-a1d7-28d8e1a08921 | Translated (85%) | EA Credit: Advanced Energy Metering | EA 得分点:高阶能源计量 (Advanced Energy Metering) |
| 836b6e9cede-48cf-4544-9694-d84efe226495 | Translated (100%) | ID&C | ID&C |
| 83770def2ca-8047-46bc-8fce-bb098bc94c4d | Translated (100%) | 1–2 points | 1-2 分 |
| 8388126f15d-1f8a-46b9-a836-0a9f9e618e2c | Translated (100%) | This credit applies to | 该得分点适用于 |
| 839bcea05a7-4f7d-40ce-af9c-f92482b50416 | Translated (100%) | Commercial Interiors (1-2 points) | 商业室内（1-2 分） |
| 8402343a092-e1de-4797-b7f7-36d3edbfa0b6 | Translated (100%) | Retail (1-2 points) | 零售（1-2 分） |
| 841c31081b6-68b4-430c-b1c1-53c4432ecb4a | Translated (100%) | Hospitality (1-2 points) | 宾馆接待（1-2 分） |
| 84284ef0cd1-e10c-475b-8a5f-947a56f537bd | Translated (100%) | Intent | 目的 |
| 8434bef6a75-199b-4e7c-898f-07ecbec47b59 | Translated (0%) | To support energy management and identify opportunities for additional energy savings by tracking building-level and system-level energy use. | 通过跟踪建筑级以及系统级的能耗来支持能源管理并确定节省更多能源的机会。 |
| 844c0197563-1af8-4335-81e4-efe22eb751f1 | Translated (100%) | Requirements | 要求 |
| 8456356dedf-9c66-4c2a-aa4e-867d07655b70 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 8465a38558c-114a-46bb-8ce6-efd24c690825 | Translated (CM) | Option 1. | 选项 1. |
| 8475a38558c-114a-46bb-8ce6-efd24c690825 | Translated (0%) | <1274>Metering</1274> (1 point) | <Bold>计量</Bold>（1 分） |
| 8486a75f4eb-abab-4024-9e61-12d24b70de16 | Translated (0%) | Install new or use existing tenant-level energy meters to provide tenant-level data representing total tenant energy consumption (electricity, natural gas, chilled water, steam, fuel oil, propane, biomass, etc.). | 新装或使用既有的租户级别的能源表来提供租户级别的数据，以推算租户的总能耗（电力、天然气、冷却水、蒸汽、燃油、丙烷、生物质能等）。 |
| 8496a75f4eb-abab-4024-9e61-12d24b70de16 | Translated (0%) | Utility-owned meters are acceptable. | 可使用公用事业公司的仪表。 |
| 850a0dc280d-9715-4503-8b46-5598e3571b7d | Translated (0%) | Commit to sharing with USGBC the resulting energy consumption data and electrical demand data (if metered) for a five-year period beginning on the date the project accepts LEED certification. | 承诺在从项目接受 LEED 认证开始的 5 年内与 USGBC 分享得到的能耗数据和电力需求数据（如果已计量）。 |
| 851a0dc280d-9715-4503-8b46-5598e3571b7d | Translated (0%) | At a minimum, energy consumption must be tracked at one-month intervals. | 能耗至少需要每月跟踪一次。 |
| 852a7f460a3-6999-49be-83c3-a390d18d6bb5 | Translated (0%) | This commitment must carry forward for five years or until the space changes ownership or lessee. | 该承诺必须执行 5 年，或者直至空间变更所有权或承租人。 |
| 8531c4ef6fd-7c35-43b7-97a9-b798b586301b | Translated (100%) | Option 2. | 选项 2. |
| 8541c4ef6fd-7c35-43b7-97a9-b798b586301b | Translated (71%) | <1275>Advanced Metering</1275> (2 points) | 高级计量（2 分） |
| 855085ded1d-9d8e-4123-a621-3aec16df67b0 | Translated (0%) | Install <1276>advanced energy metering</1276> for the following<1277>:</1277> | 为以下项安装高级能源计量装置： |
| 856c7c2e3ca-a66c-4973-97f5-33468971cb77 | Translated (0%) | all energy sources used in the tenant space; and | 承租空间中使用的所有能源；以及 |
| 85789a45977-4b97-46b3-bcb2-bd08c139e30f | Translated (0%) | any individual energy end uses that represent 10% or more of the total annual consumption of the tenant space. | 任何占据承租空间年度总能耗 10% 或更多比例的单独的终端能耗。 |
| 8585bdf4b63-e24e-4800-bc22-143d0ef72f7f | Translated (0%) | The advanced energy metering must have the following characteristics. | 高级能源计量必须具有以下特性。 |
| 8599f6f7d67-918e-480a-97c0-350b4c6519ee | Translated (0%) | Meters must be permanently installed, record at intervals of one hour or less, and transmit data to a remote location. | 仪表必须永久性地安装，按照每小时或更短时间的间隔来记录，并将数据传输到远程位置。 |
| 86018a8950b-0b5c-4bc7-985e-b0a0d560b0c2 | Translated (0%) | Electricity meters must record both consumption and demand. | 电表必须记录消耗量和需求量。 |
| 86118a8950b-0b5c-4bc7-985e-b0a0d560b0c2 | Translated (0%) | Whole-building electricity meters should record the power factor, if appropriate. | 如果可以的话，整栋建筑的电表应记录功率因数。 |
| 86243a25fec-bd41-4dad-a32c-30f357d1d68e | Translated (0%) | The data collection system must use a local area network, building automation system, wireless network, or comparable communication infrastructure. | 数据收集系统必须使用本地局域网、建筑自动化系统、无线网络或类似的通信基础设施。 |
| 863ce45a6b8-d66a-4009-8df1-21a65da44416 | Translated (0%) | The system must be capable of storing all meter data for at least 18 months. | 系统必须能够保存至少 18 个月的所有仪表数据。 |
| 86444cc8434-539a-4156-9d6b-9b89892fc169 | Translated (0%) | The data must be remotely accessible. | 必须能够远程访问数据。 |
| 8659c662f81-accb-4248-ae5f-a7595acee70d | Translated (0%) | All meters in the system must be capable of reporting hourly, daily, monthly, and annual energy use. | 系统中的所有仪表都必须能够每小时、每天、每月进行报告，以及报告年度能耗。 |
| 8665c193ddd-4063-41b1-a9c1-f2bfadefd697 | Translated (85%) | EA Credit: Renewable Energy Production | EA 得分点：可再生能源生产 (Renewable Energy Production) |
| 867fe33557f-8ec3-4926-b511-b8fc613fb378 | Translated (100%) | ID&C | ID&C |
| 86844429209-ea57-4abc-b5a0-c1976144fe95 | Translated (100%) | 1–3 points | 1-3 分 |
| 8698aaed67c-99c6-42ca-9983-53a729cd38ac | Translated (100%) | This credit applies to | 该得分点适用于 |
| 8709f8ccd17-85b4-4456-a275-c27450d15c14 | Translated (100%) | Commercial Interiors (1–3 points) | 商业室内（1-3 分） |
| 8710f0fb4d1-6af6-43e4-9fcf-45744c2ead5d | Translated (100%) | Retail (1–3 points) | 零售（1-3 分） |
| 872d18f3613-09d4-4b9b-9691-2988347d83d3 | Translated (100%) | Hospitality (1–3 points) | 宾馆接待（1-3 分） |
| 87327d36862-5314-41a7-8ab1-96b2b4f8b764 | Translated (100%) | Intent | 目的 |
| 874a40a3b9c-1dee-4551-8654-2cab4b5bcd43 | Translated (0%) | To reduce the environmental and economic harms associated with fossil fuel energy by increasing self-supply of renewable energy. | 增加可再生能源的自给，减少与化石燃料能源相关的环境和经济危害。 |
| 875034b0006-0fe2-44ef-8763-d4dce9a3030d | Translated (100%) | Requirements | 要求 |
| 8768f945cb1-54dc-422e-a3b3-9c9f863f1110 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 877748564b0-3750-4fe5-a6a1-076164f3e067 | Translated (72%) | Use tenant renewable energy systems to offset the project’s energy cost. | 使用租户可再生能源系统补偿项目能源成本。 |
| 878748564b0-3750-4fe5-a6a1-076164f3e067 | Translated (0%) | Calculate the project’s percentage of renewable energy by the following equation: | 按照以下公式计算项目可再生能源的百分比： |
| 8799372254f-00c4-4f2d-b217-44d83616197f | Translated (87%) | % renewable energy = | 可再生能源百分比 = |
| 8804850d12e-5c35-4ede-a09d-492354002ed2 | Translated (0%) | Equivalent cost of usable energy produced by renewable energy system | 可再生能源系统产生的可用能源的对应成本 |
| 88190ab6845-c6eb-4c0d-bd53-ffc3eabb3736 | Translated (0%) | Total building annual energy cost | 建筑年度总能源成本 |
| 882041d6281-37c0-499c-8e94-855195334de1 | Translated (0%) | Use the project’s annual energy cost, calculated in EA Prerequisite Minimum Energy Performance, if Option 1 was pursued; otherwise use the U.S. Department of Energy’s Commercial Buildings Energy Consumption Survey (CBECS) database to estimate energy use and cost. | 如果争取选项 1，则使用在 EA 先决条件：最低能源表现 (Minimum Energy Performance) 中计算的项目年度能源成本；否则使用美国能源部商业建筑能耗调查 (CBECS) 数据库来预估能耗和成本。 |
| 8834e57b0c0-5a97-41f0-8afa-426ca6871663 | Translated (0%) | The use of solar gardens or community renewable energy systems is allowed if both of the following requirements are met. | 如果满足以下两个要求，则允许使用太阳能花园或社区可再生能源系统。 |
| 8842602d634-dd52-4a8e-8842-66fa516f24dd | Translated (0%) | The project owns the system or has signed a lease agreement for a period of at least 10 years. | 项目拥有该系统，或者签署了至少为期 10 年的租赁协议。 |
| 88555291afa-1e3b-48f3-a818-62a8f09e33f9 | Translated (0%) | The system is located with the same utility service area as the facility claiming the use. | 该系统与声称使用它的设施位于同一个公用事业公司服务区域内。 |
| 88683c9c31f-4f64-4608-8002-89888a65ca04 | Translated (0%) | Credit is based on the percentage of ownership or percentage use assigned in the lease agreement. | 得分点基于所有权的百分比或者租赁协议中分配的用量百分比。 |
| 88783c9c31f-4f64-4608-8002-89888a65ca04 | Translated (100%) | Points are awarded according to Table 1. | 根据表 1 获得分数。 |
| 888d5938499-0b6c-4164-9acc-e3ee3705347a | Translated (CM) | Table 1. | 表 1. |
| 889d5938499-0b6c-4164-9acc-e3ee3705347a | Translated (0%) | Points for renewable energy | 可再生能源的分数 |
| 890511a2e56-e7bd-4deb-865f-0521b6f8c6b4 | Translated (94%) | Percentage renewable energy | 可再生能源百分比 |
| 891718d793b-2e27-485d-93a5-f04b518b555b | Translated (100%) | Points | 分数 |
| 89204ea1418-4bee-4d9c-a506-c4b792a20212 | Translated (CM) | 1% | 1% |
| 893c4c0f79f-ab8e-4fb5-a336-68cd58636eed | Translated (100%) | 1 | 1 |
| 89479396d98-d710-42a2-b973-4806a83a86f6 | Translated (100%) | 3% | 3% |
| 895be5369b1-77d9-4c3d-9ccd-dfa46bcdb98d | Translated (100%) | 2 | 2 |
| 896a9a5f78f-dcf8-49d7-b4cf-93257a0374a9 | Translated (100%) | 5% | 5% |
| 897b631e59d-c7a7-4bc2-bee7-e2e7f48fbf01 | Translated (100%) | 3 | 3 |
| 8988fc78b28-e4f6-4440-91fd-66911acd7929 | Translated (85%) | EA Credit: Enhanced Refrigerant Management | EA 得分点：增强冷媒管理 (Enhanced Refrigerant Management) |
| 8998bb5bc36-c4d8-45e7-9dfc-d9f25fda0e26 | Translated (100%) | ID&C | ID&C |
| 90091e2ac7b-0255-46ad-a6b9-ce7001ff853e | Translated (100%) | 1 point | 1 分 |
| 901a937c92a-e0cd-47da-b4b3-fd338e8943da | Translated (CM) | This credit applies to | 该得分点适用于 |
| 90290bca53a-ebc9-4b9f-a1d6-0ad029d17f93 | Translated (100%) | Commercial Interiors (1 point) | 商业室内（1 分） |
| 9036808eafc-bacb-4dc5-bd53-721a8a7dc82f | Translated (100%) | Retail (1 point) | 零售（1 分） |
| 904c7084f2c-c0ef-419d-8e5e-e53fcc29a37b | Translated (100%) | Hospitality (1 point) | 宾馆接待（1 分） |
| 905390f96e7-bf20-4c8c-aa74-941994fa2ce2 | Translated (100%) | Intent | 目的 |
| 906d4189c7b-5bad-4139-8866-99e434f5443a | Translated (99%) | To reduce ozone depletion and support early compliance with the Montreal Protocol while minimizing direct contributions to climate change. | 减少臭氧消耗并支持尽早遵守蒙特利尔议定书，同时尽量减少对气候改变的促进影响。 |
| 9070e79ca53-f54f-40d2-853a-5a3a488169ef | Translated (100%) | Requirements | 要求 |
| 908ce685c65-556a-450d-b76d-db9469e2f429 | Translated (CM) | CI, Hospitality | 商业室内，宾馆接待 |
| 9098b142ed3-8f2c-426c-81fd-e22839b2bd4e | Translated (100%) | Option 1. | 选项 1. |
| 9108b142ed3-8f2c-426c-81fd-e22839b2bd4e | Translated (0%) | No Refrigerants or Low-Impact Refrigerants (1 point) | 无制冷剂或影响较小的制冷剂（1 分） |
| 911b90c1595-01bd-431e-a738-562c332588ef | Translated (0%) | Do not use refrigerants, or use only refrigerants (naturally occurring or synthetic) that have an ozone depletion potential (ODP) of zero and a global warming potential (GWP) of less than 50. | 不使用制冷剂，或仅使用不会潜在破坏臭氧层 (ODP) 和全球变暖潜能值小于 50 的制冷剂（天然或人工合成）。 |
| 912c83e15e5-7951-4141-961f-6ae82ca2ffae | Translated (100%) | OR | 或 |
| 913e7d21717-9291-4659-bbcc-426183a2fa23 | Translated (CM) | Option 2. | 选项 2. |
| 914e7d21717-9291-4659-bbcc-426183a2fa23 | Translated (0%) | Calculation of Refrigerant Impact (1 point) | 制冷剂影响计算（1 分） |
| 91547544d81-f182-4641-8279-7a6fadee94c6 | Translated (80%) | Select refrigerants that are used in heating, ventilating, air-conditioning, and refrigeration (HVAC&R) equipment to minimize or eliminate the emission of compounds that contribute to ozone depletion and climate change. | 选择供暖、通风、空调和制冷 (HVAC&R) 设备中使用的制冷剂以尽量减少或消除促使臭氧消耗和气候改变的化合物排放。 |
| 91647544d81-f182-4641-8279-7a6fadee94c6 | Translated (0%) | The combination of all new and existing base building and tenant HVAC&R equipment that serve the project must comply with the following formula: | 所有为项目服务的新的和既有的基本建筑和租户 HVAC&R 设备组合都必须符合以下公式： |
| 917d58be129-ef90-431f-9d72-d7ad5a574d12 | Translated (0%) | IP units | IP 单位 |
| 918699f3f91-bd87-41f5-bc0d-f2a5d880082e | Translated (100%) | LCGWP | LCGWP |
| 9191f57f4c5-636a-477a-b3c6-0bc647f8c700 | Translated (CM) | + | + |
| 9204694b595-b1c4-414c-b0b2-854ba3b349dc | Translated (100%) | LCODP | LCODP |
| 921bee4e493-65f5-4083-9c60-a7bdaf02cf04 | Translated (100%) | x | x |
| 9229ddb6299-2662-4a42-82ea-dd70c31ef4a6 | Translated (CM) | 10<1346>5</1346> | 10<1346>5</1346> |
| 9232da7da6c-07ec-4df9-9190-430d45a9a743 | Translated (0%) | ≤ | ≤ |
| 9241a453f89-a5a8-492d-8bc0-2e02e25b0f66 | Translated (100%) | 100 | 100 |
| 925318025ec-86e2-458c-abcd-41305bea98f1 | Translated (0%) | SI units | SI 单位 |
| 9268bb27b61-2def-4bb0-9f03-4073e260b0f8 | Translated (100%) | LCGWP | LCGWP |
| 927ee15327e-0d1a-4d63-97f9-1bd405bd7157 | Translated (0%) | + | + |
| 928e792df72-6170-4017-86a7-b5bf806f0b06 | Translated (100%) | LCODP | LCODP |
| 929fe2d7e04-ff6d-4791-881d-bb0045eed2f8 | Translated (100%) | x | x |
| 930b5b46bd1-f590-4486-bb75-0182cb5157df | Translated (0%) | 10<1372>5</1372> | 10<1372>5</1372> |
| 93107337305-1bf7-4b46-9a96-eadaa411e8a5 | Translated (0%) | ≤ | ≤ |
| 9322f36f5eb-5122-43b3-9f2d-45d048b252b7 | Translated (100%) | 13 | 13 |
| 933d7be60fb-248c-434b-bc87-32bffd38aa99 | Translated (100%) | Calculation definitions for LCGWP + LCODP x 10<1383>5</1383> ≤ 100 | LCGWP + LCODP x 10<1383>5</1383> ≤ 100 的计算定义 |
| 934eb816139-9fc6-4a9d-8f65-96dcfb52019e | Translated (0%) | (IP units) | （IP 单位） |
| 935f7e1951b-dfb0-410c-ac68-904ded9497e8 | Translated (0%) | Calculation definitions for LCGWP + LCODP x 10<1392>5</1392> ≤ 13 | LCGWP + LCODP x 10<1392>5</1392> ≤ 13 的计算定义 |
| 936ec3a340d-63b5-487b-b86d-59df98e8812f | Translated (96%) | (SI units) | （SI 单位） |
| 937bd9e48d1-2b5d-4c2d-b7c4-6f459687e799 | Translated (100%) | LCODP = [ODPr x (Lr x Life +Mr) x Rc]/Life | LCODP = [ODPr x (Lr x Life +Mr) x Rc]/Life |
| 938dd197a8b-adb8-4bc8-850b-1e5c26c28994 | Translated (100%) | LCODP = [ODPr x (Lr x Life +Mr) x Rc]/Life | LCODP = [ODPr x (Lr x Life +Mr) x Rc]/Life |
| 93941137745-a4df-44b6-8273-a7984ac09c1a | Translated (100%) | LCGWP = [GWPr x (Lr x Life +Mr) x Rc]/Life | LCGWP = [GWPr x (Lr x Life +Mr) x Rc]/Life |
| 940ce1dbd27-0511-4d70-ab65-1fd6c3509675 | Translated (100%) | LCGWP = [GWPr x (Lr x Life +Mr) x Rc]/Life | LCGWP = [GWPr x (Lr x Life +Mr) x Rc]/Life |
| 94184e8f54c-a2ed-4ae7-954e-ee5f2d080f9e | Translated (83%) | LCODP: Lifecycle Ozone Depletion Potential (lb CFC 11/Ton-Year) | LCODP：生命周期中的臭氧消耗潜能值 （磅 CFC 11/吨-年） |
| 942d272186c-7be3-4f66-b41d-b13d8d9bd0dc | Translated (85%) | LCODP: Lifecycle Ozone Depletion Potential (kg CFC 11/(kW/year)) | LCODP：生命周期中的臭氧消耗潜能值 （千克 CFC 11/（kW/年）） |
| 94392fc0d08-67ad-4c41-b7e5-ecf0ce35ccd8 | Translated (0%) | LCGWP: Lifecycle Direct Global Warming Potential (lb CO<1418>2</1418>/Ton-Year) | LCGWP：生命周期中的全球变暖潜能值 （磅 CO<1418>2</1418>/吨-年） |
| 9441ba086f6-12f3-4d86-83c9-351704130ba9 | Translated (87%) | LCGWP: Lifecycle Direct Global Warming Potential (kg CO<1422>2</1422>/kW-year) | LCGWP：生命周期中的全球变暖潜能值 （千克 CO<1422>2</1422>/kW-年） |
| 94542a42b82-0d9b-46b2-9598-e2430394dc62 | Translated (0%) | GWPr: Global Warming Potential of Refrigerant (0 to 12,000 lb CO<1426>2</1426>/lbr) | GWPr：制冷剂的全球变暖潜能值 （0 到 12,000 lb CO<1426>2</1426>/lbr） |
| 9463a202d75-24ee-469b-bc9a-cf5fd890a9e1 | Translated (89%) | GWPr: Global Warming Potential of Refrigerant (0 to 12,000 kg CO<1430>2</1430>/kg r) | GWPr：制冷剂的全球变暖潜能值 （0 到 12,000 kg CO<1430>2</1430>/kg r） |
| 947c5175bf8-0d31-4c30-ae99-3547aa0515ce | Translated (0%) | ODPr: Ozone Depletion Potential of Refrigerant (0 to 0.2 lb CFC 11/lbr) | ODPr：制冷剂的臭氧消耗潜能值 （0 到 0.2 lb CFC 11/lbr） |
| 94807c7067b-9a1b-4014-844f-0ac8977dc360 | Translated (89%) | ODPr: Ozone Depletion Potential of Refrigerant (0 to 0.2 kg CFC 11/kg r) | ODPr：制冷剂的臭氧消耗潜能值 （0 到 0.2 kg CFC 11/kg r） |
| 9490e4e48bc-ad46-4299-8286-746c246029cb | Translated (0%) | Lr: Refrigerant Leakage Rate (2.0%) | Lr：制冷剂泄漏率 (2.0%) |
| 9503c06eaba-86a9-4e90-bee4-3a00fd8e63d7 | Translated (100%) | Lr: Refrigerant Leakage Rate (2.0%) | Lr：制冷剂泄漏率 (2.0%) |
| 9518dd20fad-8fc4-4c2f-ad56-005d63a6d232 | Translated (0%) | Mr: End-of-life Refrigerant Loss (10%) | Mr：使用寿命结束时的制冷剂损失 (10%) |
| 952b6db8da8-403d-4669-86e4-b7202fed05e1 | Translated (100%) | Mr: End-of-life Refrigerant Loss (10%) | Mr：使用寿命结束时的制冷剂损失 (10%) |
| 9530aeced90-8843-4039-8ea9-72eba3564eb6 | Translated (87%) | Rc: Refrigerant Charge (0.5 to 5.0 lbs of refrigerant per ton of gross AHRI rated cooling capacity) | Rc：制冷剂更换 （每吨 AHRI 额定总制冷量 0.5 到 5.0 lbs 制冷剂） |
| 95450019739-e01a-4242-8a07-fdda03060466 | Translated (88%) | Rc: Refrigerant Charge (0.065 to 0.65 kg of refrigerant per kW of AHRI rated or Eurovent Certified cooling capacity) | Rc：制冷剂更换 （每千瓦 AHI 额定或 Eurovent 认证的制冷量 0.065 到 0.65 kg 制冷剂） |
| 955957245c3-64f5-43b6-b8e5-3c98c90ea2ce | Translated (88%) | Life: Equipment Life (10 years; default based on equipment type, unless otherwise demonstrated) | Life：设备使用寿命 （10 年；除非另有说明，否则默认基于设备类型） |
| 956b27a0f94-d17d-471f-99b1-9790cfcf23b0 | Translated (100%) | Life: Equipment Life (10 years; default based on equipment type, unless otherwise demonstrated) | Life：设备使用寿命 （10 年；除非另有说明，否则默认基于设备类型） |
| 9573fa41a02-8d44-4f76-8284-b4549f1caaae | Translated (84%) | For multiple types of equipment, calculate a weighted average of all base building HVAC&R equipment, using the following formula: | 对于多种类型的设备，必须使用以下公式计算所有建筑基础 HVAC&R 设备的加权平均值： |
| 9580b97f9d8-5c9e-4c8f-a7a4-9f7147c8a418 | Translated (100%) | IP units | IP 单位 |
| 95989b13a32-ed51-4560-a711-042ad09d446c | Translated (100%) | SI units | SI 单位 |
| 9601e8ec9f5-7a89-4a00-8387-b3179cbf3678 | Translated (0%) | ∑ ( LCGWP + LCODP x 10<1472>5</1472> ) x Qunit | ∑ ( LCGWP + LCODP x 10<1472>5</1472> ) x Qunit |
| 961d73e368e-22b1-4767-aa65-a7237a51b82d | Translated (0%) | ≤ 100 | ≤ 100 |
| 962bec69f7b-ef1c-4039-8487-f3fb8c3360e2 | Translated (100%) | ∑ ( LCGWP + LCODP x 10<1479>5</1479> ) x Qunit | ∑ ( LCGWP + LCODP x 10<1479>5</1479> ) x Qunit |
| 96300d6422c-3aca-452e-988b-d017d4eae604 | Translated (CM) | ≤ 13 | ≤ 13 |
| 9643a602118-18a8-42a3-81cc-532a12c5cf15 | Translated (0%) | ——————————————————————————— | ——————————————————————————— |
| 9651821da72-7b13-465c-b4dd-ba323b440bdc | Translated (100%) | Qtotal | Qtotal |
| 96689813926-edc5-44cb-bda6-707d15ca446c | Translated (0%) | ——————————————————————— | ——————————————————————— |
| 96798004ef1-49ed-4c6e-adeb-4f13328ee2ac | Translated (100%) | Qtotal | Qtotal |
| 968f43f9178-1e9d-4127-b3c3-dc8fc0c2b054 | Translated (0%) | Calculation definitions for [ ∑ (LCGWP + LCODP x 10<1499>5</1499>) x Qunit ] / Qtotal ≤ 100 | [ ∑ (LCGWP + LCODP x 10<1499>5</1499>) x Qunit ] / Qtotal ≤ 100 的计算定义 |
| 969d736764f-f27b-4efd-8a37-7db5147d0dd6 | Translated (100%) | (IP units) | （IP 单位） |
| 970bad37d57-2e1b-4900-8c17-08a4b521cb9e | Translated (100%) | Calculation definitions for [ ∑ (LCGWP + LCODP x 10<1508>5</1508>) x Qunit ] / Qtotal ≤ 13 | [ ∑ (LCGWP + LCODP x 10<1508>5</1508>) x Qunit ] / Qtotal ≤ 13 的计算定义 |
| 9714353c26b-dd51-48d9-bd58-e80df2139af1 | Translated (100%) | (SI units) | （SI 单位） |
| 972bc38c84d-b5e5-4806-bc43-e9d81a30617a | Translated (96%) | Qunit = Gross AHRI rated cooling capacity of an individual HVAC or refrigeration unit (Tons) | Qunit = 单个暖通空调或制冷装置的 AHRI 额定总制冷量（吨） |
| 973dd85ce2a-f721-4486-9a0a-2e4e3c39bf6c | Translated (99%) | Qunit = Eurovent Certified cooling capacity of an individual HVAC or refrigeration unit (kW) | Qunit = 单个暖通空调或制冷装置的 Eurovent 认证制冷量（KW） |
| 97468a75dc8-d8dd-417c-a9df-5cbb4aa8c020 | Translated (95%) | Qtotal = Total gross AHRI rated cooling capacity of all HVAC or refrigeration | Qtotal = 所有暖通空调或制冷装置的 AHRI 额定总制冷量 |
| 9757ec00134-3ebb-4ec6-9ee5-fb3977e36e06 | Translated (100%) | Qtotal = Total Eurovent Certified cooling capacity of all HVAC or refrigeration (kW) | Qtotal = 所有暖通空调或制冷装置的 Eurovent 认证总制冷量（KW） |
| 9761db49a2c-c6cd-428d-8525-c80fc6962cd6 | Translated (0%) | Retail CI | 商业室内零售 |
| 977b09d3299-d9a3-4e04-973c-9a1633fb4cd1 | Translated (0%) | Meet Option 1 or 2 for all HVAC systems. | 所有暖通空调系统满足选项 1 或 2。 |
| 97832240253-4e5f-42ca-86bb-38d8f003e0e3 | Translated (0%) | Stores with commercial refrigeration systems must comply with the following. | 使用商用制冷系统的商店必须符合以下要求。 |
| 979edf49487-3194-43b5-ab5a-ff02b164df82 | Translated (0%) | Use only non-ozone-depleting refrigerants. | 仅使用不消耗臭氧层的制冷剂。 |
| 980291205bb-c43a-4b56-9d53-fc9d942692e1 | Translated (0%) | Select equipment with an average HFC refrigerant charge of no more than 1.75 pounds of refrigerant per 1,000 Btu/h (2.72 kg of refrigerant per kW) total evaporator cooling load. | 对于选择的设备，其蒸发器全部制冷负载的平均 HFC 制冷剂补充量为每 1,000 Btu/h（每 KW 2.72 kg 制冷剂）不超过 1.75 磅。 |
| 981290bf155-ce5e-4713-919f-ba14489a5f79 | Translated (0%) | Demonstrate a predicted store-wide annual refrigerant emissions rate of no more than 15%. | 证明商店内部的预计年制冷剂排放率不超过 15%。 |
| 982290bf155-ce5e-4713-919f-ba14489a5f79 | Translated (0%) | Conduct leak testing using the procedures in GreenChill’s best practices guideline for leak tightness at installation. | 在安装时使用 GreenChill 密封性最佳实践指南中的程序进行泄漏测试。 |
| 9837e7317a4-a53b-44f1-80a2-959cdeb7d5c6 | Translated (0%) | Alternatively, stores with commercial refrigeration systems may provide proof of attainment of EPA GreenChill’s silver-level store certification for newly constructed stores. | 另外，使用商用制冷系统的商店需提供证据来证明新建的商店通过了 EPA GreenChill 银级商店认证。 |
| 98413c71182-bdae-4a89-8226-b8c18b97d8fa | Translated (89%) | EA Credit: Green Power and Carbon Offsets | EA 得分点：绿色电力和碳补偿 (Green Power and Carbon Offsets) |
| 9855052a91a-44c4-49e6-8fc7-3c61201ff0a7 | Translated (100%) | ID&C | ID&C |
| 98662dd3e25-ff4a-4f19-a356-8d40645c78e6 | Translated (100%) | 1–2 points | 1-2 分 |
| 9873f1a1113-c2e8-4e41-bf80-283002abb7f9 | Translated (100%) | This credit applies to | 该得分点适用于 |
| 988c21bf7c5-4d2e-428c-b9a4-91ca116b2f1a | Translated (100%) | Commercial Interiors (1–2 points) | 商业室内（1-2 分） |
| 989dd488aea-f13f-4c73-9cfd-fc1087c7ea4b | Translated (100%) | Retail (1–2 points) | 零售（1-2 分） |
| 9902acba5a5-24dd-4b06-92c7-e70c2fd2dc5d | Translated (100%) | Hospitality (1–2 points) | 宾馆接待（1-2 分） |
| 9918684fd68-6bbd-4b38-83d4-c139d229580e | Translated (100%) | Intent | 目的 |
| 99235266d23-2477-452f-8f54-db924d531cc1 | Translated (0%) | To encourage the reduction of greenhouse gas emissions through the use of grid-source, renewable energy technologies and carbon mitigation projects. | 鼓励通过使用电网可再生能源技术和碳减排项目来减少温室气体排放。 |
| 993535a9dd1-777c-4d2b-9a92-58061b2c101d | Translated (100%) | Requirements | 要求 |
| 994d40354f2-c150-413e-bd60-54a6b760c005 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 995b2e77678-5fbd-4ab1-9001-c45900105896 | Translated (0%) | Engage in a contract for qualified resources that have come online since January 1, 2005, for a minimum of five years, to be delivered at least annually. | 针对符合条件的资源签署合同，2005 年 1 月 1 日生效，至少为期 5 年，至少要每年交付一次。 |
| 996b2e77678-5fbd-4ab1-9001-c45900105896 | Translated (0%) | The contract must specify the provision of at least 50% or 100% of the project’s energy from green power, carbon offsets, or renewable energy certificates (RECs). | 合同必须指定至少通过绿色电力、碳补偿或可再生能源认证 (REC) 提供 50% 或 100% 的项目能源。 |
| 99779749158-d02d-4761-8551-58b267963190 | Translated (0%) | Green power and RECs must be Green-e Energy certified or the equivalent. | 绿色电力和 REC 必须通过 Green-e 能源认证或类似认证。 |
| 99879749158-d02d-4761-8551-58b267963190 | Translated (0%) | RECs can only be used to mitigate the effects of Scope 2, electricity use. | REC 只能用于减轻范围 2，电力使用的影响。 |
| 999a6b8f8df-efd8-4f54-99c8-9c155a815a80 | Translated (0%) | Carbon offsets may be used to mitigate Scope 1 or Scope 2 emissions on a metric ton of carbon dioxide–equivalent basis and must be Green-e Climate certified, or the equivalent. | 碳补偿可用于在一公吨二氧化碳当量的基础上减轻范围 1 或范围 2 排放，并且必须通过 Green-e 气候认证或类似认证。 |
| 1000dfa943b5-2718-499f-a055-e1d375ccb906 | Translated (0%) | For U.S. projects, the offsets must be from greenhouse gas emissions reduction projects within the U.S. | 对于美国项目，补偿必须来自美国的温室气体减排项目。 |
| 1001f3623855-b040-4a94-b8fe-a7fd2eeb7b2c | Translated (81%) | Determine the percentage of green power or offsets based on the quantity of energy consumed, not the cost. | 基于能耗量，而不是成本来确定绿色电力或补偿的百分比。 |
| 1002f3623855-b040-4a94-b8fe-a7fd2eeb7b2c | Translated (100%) | Points are awarded according to Table 1. | 根据表 1 获得分数。 |
| 100324ef1758-cb57-4457-a6d1-4752c90db44e | Translated (CM) | Table 1. | 表 1. |
| 100424ef1758-cb57-4457-a6d1-4752c90db44e | Translated (0%) | Points for energy from green power or carbon offsets | 来自绿色电力或碳补偿的能源的分数 |
| 10058bd28f50-6d87-4f1d-8d83-919f285f1c3c | Translated (0%) | Percentage of total energy addressed by green power, RECs and/or offsets | 绿色电力、REC 和/或补偿解决的全部能源的百分比 |
| 1006e6c834b0-4dd2-49df-90c0-833fcdb26a06 | Translated (100%) | Points | 分数 |
| 1007facc291e-b81b-4142-851c-efae8563d3f5 | Translated (CM) | 50% | 50% |
| 1008ec8afde9-4aeb-46af-b977-0c67501243aa | Translated (100%) | 1 | 1 |
| 1009e19c2b3e-1905-419e-a712-32ef0a9379ed | Translated (100%) | 100% | 100% |
| 1010c1141b73-8fe5-4e15-830d-d096d7f7eb46 | Translated (100%) | 2 | 2 |
| 10118b794704-0145-4cdd-99e3-7109b8fdc45a | Translated (95%) | Use the project’s annual energy consumption, calculated in EA Prerequisite Minimum Energy Performance, if Option 1 was pursued; otherwise use the U.S. Department of Energy’s Commercial Buildings Energy Consumption Survey (CBECS) database to estimate energy use. | 如果争取选项 1，则使用在 EA 先决条件：最低能源表现 (Minimum Energy Performance) 中计算的项目的年度能耗；否则使用美国能源部商业建筑能耗调查 (CBECS) 数据库来预估能耗。 |
| 1012a062d3c7-cf43-4016-a273-b6de94e74614 | Translated (100%) | Materials and Resources (MR) | 材料与资源 (MR) |
| 1013995e2a99-0617-4211-905d-d38cef661a0f | Translated (75%) | MR Prerequisite: Storage and Collection of Recyclables | MR 先决条件：可回收物存储和收集 (Storage and Collection of Recyclables) |
| 10145b653f0f-e3f5-4bc5-bde4-270ecc156c45 | Translated (100%) | Required | 必要项 |
| 1015e311b55d-456f-46b4-9907-a2212c651be7 | Translated (CM) | ID&C | ID&C |
| 1016a3798a5d-0856-45fc-938d-7d1face0e199 | Translated (CM) | This prerequisite applies to | 该先决条件适用于 |
| 1017c609ffce-381e-43c5-bc44-63586dc277f8 | Translated (100%) | Commercial Interiors | 商业室内 |
| 1018659bca1b-63d6-430a-9768-45450bffbfb6 | Translated (100%) | Retail | 零售 |
| 10198eb2239a-93fb-4ccd-b64e-79cc6273ba94 | Translated (100%) | Hospitality | 宾馆接待 |
| 10200834d479-a180-4013-8c0d-c87596d1823a | Translated (100%) | Intent | 目的 |
| 10215c48be37-3515-48dd-ad9c-9b3e97245470 | Translated (78%) | To reduce the waste that is generated by building occupants and hauled to and disposed of in landfills. | 减少由建筑驻户产生并被运送到和弃置于掩埋场的废弃物。 |
| 10220188b4d6-8d97-4249-9f19-a103f9164a20 | Translated (100%) | Requirements | 要求 |
| 1023104b2b8a-0eef-4ad7-a6ca-5545d33863f2 | Translated (77%) | CI, Hospitality CI | 商业室内，宾馆接待商业室内 |
| 1024f1a5e4ac-224c-4ecf-9bb6-d0244b8fe280 | Translated (0%) | Provide dedicated areas accessible to waste haulers and building occupants for the collection and storage of recyclable materials for the entire building. | 提供专门的区域，可以让废弃物清运商和建筑驻户收集和存放整栋建筑的可回收材料。 |
| 1025f1a5e4ac-224c-4ecf-9bb6-d0244b8fe280 | Translated (0%) | Collection and storage areas may be separate locations. | 收集和存放区域可位于不同的地点。 |
| 1026f1a5e4ac-224c-4ecf-9bb6-d0244b8fe280 | Translated (0%) | Recyclable materials must include mixed paper,<1557> </1557> corrugated cardboard,<1558> </1558> glass, plastics, and metals. | 可回收材料必须包括混合纸、<1557></1557>硬纸板、<1558></1558>玻璃、塑料和金属。 |
| 1027f1a5e4ac-224c-4ecf-9bb6-d0244b8fe280 | Translated (0%) | Take appropriate measures for the safe collection, storage, and disposal of two of the following: batteries, mercury-containing lamps, and electronic waste. | 采取适当的措施安全地收集、存放和处理电池、含汞灯和电子垃圾中的两种。 |
| 102821f0362a-7928-41c4-9e65-f60e5fcfd4e5 | Translated (100%) | Retail CI | 商业室内零售 |
| 1029f80f4682-85e2-447a-bbf2-18f7e6e600a3 | Translated (0%) | Conduct a waste stream study to identify the retail project’s top five recyclable waste streams, by either weight or volume, using consistent metrics. | 使用一致的度量方法，进行废弃物流研究以确定零售项目中数量最多（按照重量或体积）的 5 种可回收废弃物流。 |
| 1030f80f4682-85e2-447a-bbf2-18f7e6e600a3 | Translated (0%) | Based on the waste stream study, list the top four waste streams for which collection and storage space will be provided. | 根据废弃物流研究，列出将要为其提供收集和存放空间的数量最多的 4 种废弃物流。 |
| 1031f80f4682-85e2-447a-bbf2-18f7e6e600a3 | Translated (0%) | If no information is available on waste streams for the project, use data from similar operations to make projections. | 如果没有关于项目废弃物流的信息，请使用类似运营的数据来进行预测。 |
| 1032f80f4682-85e2-447a-bbf2-18f7e6e600a3 | Translated (0%) | Retailers with existing stores of similar size and function can use historical information from their other locations. | 已有类似规模和功能的商店的零售商可以使用其他商店的历史信息。 |
| 1033110bc853-4ead-44e9-8e2b-1e286f8f56b6 | Translated (0%) | Provide dedicated areas accessible to waste haulers and building occupants for the separation, collection, and storage of recyclable materials for at least the top four recyclable waste streams identified by the waste study. | 提供专门的区域，可以让废弃物清运商和建筑驻户分离、收集和存放至少由废弃物研究确定的数量最多的 4 种可回收废弃物流的可回收材料。 |
| 1034110bc853-4ead-44e9-8e2b-1e286f8f56b6 | Translated (0%) | Locate the collection and storage bins close the source of recyclable waste. | 将收集和存放箱放在靠近可回收废弃物来源的位置。 |
| 1035110bc853-4ead-44e9-8e2b-1e286f8f56b6 | Translated (0%) | If any of the top four waste streams are batteries, mercury-containing lamps, or electronic waste, take appropriate measures for safe collection, storage, and disposal. | 如果数量最多的 4 种废弃物流中包括电池、含汞灯或电子垃圾，请采取适当的措施以安全收集、存放和处理。 |
| 103647c94ab8-9930-4cf3-a98f-d3496e3976ec | Translated (79%) | MR Prerequisite: Construction and Demolition Waste Management Planning | MR 先决条件：营建和拆建废弃物管理计划 (Construction and Demolition Waste Management Planning) |
| 1037a14c6a41-a8d0-4026-a3fd-36492b27eec4 | Translated (100%) | Required | 必要项 |
| 10381afb73e0-1766-4406-9af3-a44450ede1e2 | Translated (CM) | ID&C | ID&C |
| 1039800e3665-3d7a-4261-9dd6-de026bd4a9d5 | Translated (CM) | This prerequisite applies to | 该先决条件适用于 |
| 1040286ae3e6-f11d-48e3-b00c-e474917f4560 | Translated (100%) | Commercial Interiors | 商业室内 |
| 1041ace16f10-39a6-48f6-b71f-b13ec4a966df | Translated (100%) | Retail | 零售 |
| 10423b22e2e8-9f41-43c5-b30c-18ed67dcec60 | Translated (100%) | Hospitality | 宾馆接待 |
| 10432d9468be-f045-4d5d-bea1-8f212d274a4e | Translated (100%) | Intent | 目的 |
| 10441477edf8-f114-43f6-8066-28cc2b83da4a | Translated (0%) | To reduce construction and demolition waste disposed of in landfills and incineration facilities by recovering, reusing, and recycling materials. | 回收、再利用材料，减少在填埋场和焚化设施中处理的营建和拆建废弃物。 |
| 1045daa34a8b-868e-41e8-8213-4b582f6b18f1 | Translated (100%) | Requirements | 要求 |
| 104636fad510-b831-47ac-9b0c-020d2792f7b5 | Translated (0%) | CI, Retail CI, Hospitality CI | 商业室内，零售商业室内，宾馆接待商业室内 |
| 10475b35a91f-5b04-416a-8663-550427f5c2f1 | Translated (73%) | Develop and implement a construction and demolition waste management plan: | 制定和执行营建和拆建废弃物管理计划。 |
| 10486640108c-882f-4596-b358-5d2dc21652f6 | Translated (0%) | Establish waste diversion goals for the project by identifying at least five materials (both structural and nonstructural) targeted for diversion. | 确定至少 5 种将要进行转化的材料（建筑和非建筑材料），建立项目的废弃物转化目标。 |
| 10496640108c-882f-4596-b358-5d2dc21652f6 | Translated (0%) | Approximate a percentage of the overall project waste that these materials represent. | 预估这些材料占整体项目废弃物的百分比。 |
| 10504d3e2e23-9d0b-438c-be13-e38f6d690582 | Translated (0%) | Specify whether materials will be separated or comingled and describe the diversion strategies planned for the project. | 指定是否要分离或混合材料，并介绍为项目规划的转化策略。 |
| 10514d3e2e23-9d0b-438c-be13-e38f6d690582 | Translated (0%) | Describe where the material will be taken and how the recycling facility will process the material. | 介绍材料将要被送往何处，以及回收设施如何处理这些材料。 |
| 1052ecb6ae7a-d0e7-4c86-92ac-bf70a3e67469 | Translated (0%) | Provide a final report detailing all major waste streams generated, including disposal and diversion rates. | 提供最终报告，详细说明产生的所有主要的废弃物流，包括处理和转化率。 |
| 10536a552e70-361b-4ed6-ab9b-14d0d9f23e36 | Translated (0%) | Alternative daily cover (ADC) does not qualify as material diverted from disposal. | 替代日常封盖物 (ADC) 不符合处理后转化材料的标准。 |
| 10546a552e70-361b-4ed6-ab9b-14d0d9f23e36 | Translated (0%) | Land-clearing debris is not considered construction, demolition, or renovation waste that can contribute to waste diversion. | 场地清理产生的碎片不被视作可进行废弃物转化的施营建、拆建或改造废弃物。 |
| 105589e8d9da-e50c-4c8c-a412-e79ec7961469 | Translated (0%) | MR Credit: Long-Term Commitment | MR 得分点：长期承诺 (Long-Term Commitment) |
| 105687ccee82-0544-46a4-9c94-ec806ca5bdde | Translated (100%) | ID&C | ID&C |
| 105722562bba-4aef-4110-9334-4ad34ba23cfa | Translated (100%) | 1 point | 1 分 |
| 10586451fd91-f57c-4992-be9a-8c2ea6b912b7 | Translated (CM) | This credit applies to | 该得分点适用于 |
| 10596f5402f1-745b-41ba-b1a8-6338c10660dd | Translated (CM) | Commercial Interiors (1 point) | 商业室内（1 分） |
| 1060ae68ba45-bc10-40bf-b1f3-f9a8effd928a | Translated (CM) | Retail (1 point) | 零售（1 分） |
| 1061573b0b64-6c23-42a8-a159-ea9e1f2cd3f9 | Translated (CM) | Hospitality (1 point) | 宾馆接待（1 分） |
| 1062bb640a1b-4d6c-41e5-af42-bde4eb8cf89c | Translated (100%) | Intent | 目的 |
| 1063a47c16da-7d38-42e5-9d27-ab7122f5d059 | Translated (0%) | To encourage choices that will conserve resources and reduce environmental harm from materials manufacturing and transport for tenants’ relocation. | 鼓励能够节约资源和减少租户搬迁所带来的材料制造和运输对环境危害的选项。 |
| 10642df270cf-b09c-434f-a548-010fb4e86a04 | Translated (100%) | Requirements | 要求 |
| 10654f8ed7e6-6e85-4830-858f-92b2dad518f6 | Translated (CM) | CI, Retail CI, Hospitality CI | 商业室内，零售商业室内，宾馆接待商业室内 |
| 1066e09c6a03-2814-4678-aac0-163957698aae | Translated (88%) | The occupant or tenant must commit to remain in the same location for at least 10 years. | 用户或租户必须承诺保持在同一处所至少 10 年。 |
| 10673fb5856d-3f98-49c8-86c4-31f641d3034f | Translated (70%) | MR Credit: Interiors Life-Cycle Impact Reduction | MR 得分点：降低室内生命周期中的影响 (Interiors Life-Cycle Impact Reduction) |
| 1068b5751b39-90bb-4a32-9ba1-11262d43b1d1 | Translated (100%) | ID&C | ID&C |
| 106980ee5bf6-676c-410a-affc-6a9a105e110c | Translated (100%) | 1-5 points | 1-5 分 |
| 1070c6f5e634-e32a-419d-9511-c7770d0839d9 | Translated (100%) | This credit applies to | 该得分点适用于 |
| 1071764f8571-db3d-4e52-a29a-ad334d2cd0eb | Translated (100%) | Commercial Interiors (1-4 points) | 商业室内（1-4 分） |
| 1072b46b37cc-3cc5-4af7-bb88-faf11d3467ec | Translated (100%) | Retail (1-5 points) | 零售（1-5 分） |
| 1073b5db5142-b8c9-48e1-863f-d521b16aed07 | Translated (100%) | Hospitality (1-4 points) | 宾馆接待（1-4 分） |
| 1074c931b873-9299-4a9c-bb77-6d26553d1a42 | Translated (100%) | Intent | 目的 |
| 10753c28fa7c-a3ee-4773-8d02-003ab5542629 | Translated (0%) | To encourage adaptive reuse and optimize the environmental performance of products and materials. | 鼓励适应性再利用和优化产品和材料的环境方面表现。 |
| 10763a133137-5406-4722-9ad6-823ef466da86 | Translated (100%) | Requirements | 要求 |
| 107789955136-cab4-4fe9-b09a-4c7881b4f872 | Translated (100%) | CI, Retail CI, Hospitality CI | 商业室内，零售商业室内，宾馆接待商业室内 |
| 1078499a8782-e322-408b-974f-206fd58170b1 | Translated (99%) | <1572>Option 1</1572>. | 选项 1. |
| 1079499a8782-e322-408b-974f-206fd58170b1 | Translated (0%) | Interior Reuse (2 points) | 室内再利用（2 分) |
| 10800b78906d-2f6a-47e6-b2a9-0f0c03d3d1fa | Translated (0%) | Reuse or salvage interior nonstructural elements for at least 50% of the surface area. | 再利用或回收至少 50% 表面区域的室内非建筑元素。 |
| 10810b78906d-2f6a-47e6-b2a9-0f0c03d3d1fa | Translated (85%) | Hazardous materials that are remediated as a part of the project must be excluded from the calculation. | 作为项目一部分被修复的有害物质必须从计算中排除。 |
| 10825c876554-2af6-433c-910f-4caa1e01c6d9 | Translated (100%) | AND/OR | 和/或 |
| 10835cd53578-d8e0-4f67-a44b-ea151e603361 | Translated (CM) | Option 2. | 选项 2. |
| 10845cd53578-d8e0-4f67-a44b-ea151e603361 | Translated (76%) | Furniture Reuse (1 point) | 家具再利用（1 分) |
| 10855cd3c077-71c7-42c9-8167-0856b3b1a24b | Translated (73%) | Reuse, salvage, or refurbish furniture and furnishings for at least 30% of the total furniture and furnishings cost. | 重复利用、回收或翻新占到家具和陈设总成本 30% 的家具和陈设。 |
| 1086bb0d06ca-b7d2-4ff4-ab75-13fbaa04279b | Translated (100%) | AND/OR | 和/或 |
| 1087c43ab3b8-02cd-4650-be98-b4d8c9c51848 | Translated (CM) | Option 3. | 选项 3. |
| 1088c43ab3b8-02cd-4650-be98-b4d8c9c51848 | Translated (0%) | Design for Flexibility (1 point ID&C, 2 points Retail CI) | 灵活性设计（1 分 ID&C，2 分商业室内零售） |
| 1089740ac721-523a-4be2-a46e-d0d6f58d00aa | Translated (0%) | Conduct an integrative planning process to increase the useful life of the project space. | 进行整合规划过程以增加项目空间的使用寿命。 |
| 1090740ac721-523a-4be2-a46e-d0d6f58d00aa | Translated (0%) | Increase project space flexibility, ease of adaptive use, and recycling of building materials while considering differential durability and premature obsolescence over building design life and individual component service lives. | 增加项目空间灵活性、适应性使用的简便性，以及回收建筑材料，同时考虑在建筑设计寿命和单个组件使用寿命期间的差异耐久性和过早过时问题。 |
| 1091740ac721-523a-4be2-a46e-d0d6f58d00aa | Translated (0%) | Use at least three of the following strategies. | 使用以下策略中的至少三种。 |
| 1092bca6666b-fbf5-4891-be31-4ee9fff1dfb3 | Translated (0%) | Install accessible systems (floor or ceiling) for at least 50% of the project floor area to allow for flexible use of space and access to systems (under floor distribution systems) not entangled with other building systems. | 为至少 50% 的项目建筑面积安装易于使用的系统，以允许灵活使用不与其他建筑系统有关联的空间和系统（在楼层分布系统下）。 |
| 1093a4fd79a4-509b-409a-b6b0-18906a5fb97c | Translated (0%) | Design at least 50% of interior nonstructural walls, ceilings, and floors to be movable or demountable. | 将至少 50% 的室内非建筑墙壁、屋顶和地板设计为可以移动或可拆卸的。 |
| 10945edbdf2a-8f9d-4f53-8370-256f8927a19c | Translated (0%) | Ensure that at least 50%, by cost, of nonstructural materials have integral labels (radio frequency identification, engraving, embossing, or other permanent marking) containing information on material origin, properties, date of manufacture, in compliance with Canadian Standards Association CSA Z782-06 Guideline for Design for Disassembly and Adaptability in Buildings. | 确保至少 50%（按成本）的非建筑材料有内置标签（无线射频识别、雕刻、压花，或其他永久性标志），包含材料原产地、属性、生产日期等信息，符合加拿大标准协会适用于建筑中可拆卸和适应性设计的 CSA Z782-06 指南。 |
| 10959bdc1970-91eb-4dea-908c-5016fa911d23 | Translated (0%) | Include in at least one major component or systems purchase contract a clause specifying sub-contractor, vendor, or on site take back system. | 包括至少一份主要组件或系统购买合同，指定分包商、供应商或现场收回系统的条款。 |
| 109602dce3eb-4f8c-4bfa-82ea-f929854fd79a | Translated (0%) | Ensure that at least 50% of nonstructural materials, by cost, are reusable or recyclable, as defined by the Federal Trade Commission Guide for Use of Environmental Marketing Claims, 260.12. | 按照美国联邦贸易委员会 (Federal Trade Commission)《环境营销声明使用指南》(Guide for Use of Environmental Marketing Claims) 260.12 的规定，确保至少 50%（按成本）的非建筑材料可重复利用或可回收。 |
| 109721f5912e-726c-4740-9f81-99602e2bba50 | Translated (0%) | Implement flexible power distribution (i.e., plug-and-play) systems for at least 50% of the project floor area so that lighting, data, voice, and other systems can be easily reconfigured and repurposed. | 至少为 50% 的项目建筑面积部署灵活的配电（即插即用）系统，从而可以轻松地对照明、数据、语音和其他系统进行重新配置和改变用途。 |
| 1098adc037a4-7ad0-45ce-a8cf-1a7e3c5ae455 | Translated (0%) | Implement a flexible lighting control system with plug and play components such as wall controls, sensors, and dimming ballasts for a minimum of 50% of the lighting load. | 至少为 50% 的照明负荷部署灵活的照明控制系统，带有壁挂式控制装置、感应器和调光镇流器等即插即用组件。 |
| 1099adc037a4-7ad0-45ce-a8cf-1a7e3c5ae455 | Translated (0%) | The system shall allow for reconfiguring and repurposing of luminaires and controls without rewiring such as having the capability to group and assign luminaires into zones and change those zones as needed. | 系统应允许在不重新接线的情况下对光源和控制装置进行重新配置和改变用途，从而能够将光源分组和分配到各个区域，并根据需要更改这些区域。 |
| 1100adc037a4-7ad0-45ce-a8cf-1a7e3c5ae455 | Translated (0%) | Also, the system shall be flexible so that as a space changes functions, the lighting levels can change to suit the needs of the space without rewiring or removing or adding luminaires. | 并且，系统应足够灵活，从而可以在空间更改功能之后，在不重新接线以及拆除或添加光源的情况下更改照明等级以适应空间需求。 |
| 11016bf08be6-92e9-47c1-a83e-ac70f17d9496 | Translated (0%) | MR Credit: Building Product Disclosure and Optimization—Environmental Product Declarations | MR 得分点：建筑产品的分析公示和优化 - 产品环境要素声明 (Building Product Disclosure and Optimization—Environmental Product Declarations) |
| 1102f1b41499-b786-4aab-b663-a444ab917947 | Translated (100%) | ID&C | ID&C |
| 1103b6fe618c-fd7c-4876-b318-64d09bd32ae6 | Translated (100%) | 1-2 points | 1-2 分 |
| 1104254aa0dc-6f07-491c-b768-62892a883fee | Translated (CM) | This credit applies to | 该得分点适用于 |
| 11057d1d2dc4-ac87-46f0-b553-bebcc2c674ae | Translated (0%) | Commercial Interiors (2 points) | 商业室内（2 分) |
| 110647603cd1-5fa9-465a-b2fa-5510a22496d0 | Translated (90%) | Retail (2 points) | 零售（2 分） |
| 1107761c83a8-65ff-4284-acb9-ab816e1453aa | Translated (90%) | Hospitality (2 points) | 宾馆接待（2 分） |
| 110830c33aae-6821-4ca7-85d6-58357fc4751b | Translated (100%) | Intent | 目的 |
| 1109bf68935c-2555-449a-b228-da84c1b8c043 | Translated (0%) | To encourage the use of products and materials for which life-cycle information is available and that have environmentally, economically, and socially preferable life-cycle impacts. | 鼓励使用提供生命周期信息且具有有利的环境、经济和社会生命周期影响的产品和材料。 |
| 1110bf68935c-2555-449a-b228-da84c1b8c043 | Translated (0%) | To reward project teams for selecting products from manufacturers who have verified improved environmental life-cycle impacts. | 对从已经证明改善环境生命周期影响的制造商那里选择产品的项目团队进行奖励。 |
| 1111465eb5fa-0509-46df-871e-fde10efd0ba1 | Translated (100%) | Requirements | 要求 |
| 111200b50e35-9000-4b0a-b57d-61e4277c37b0 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 11135d9e164a-fbbf-431b-84a0-a6390160fd9d | Translated (0%) | Achieve one or more of the options below, for a maximum of 2 points. | 实现以下一个或多个选项，最多获得 2 分。 |
| 1114d2513e86-baba-4e50-b411-d37bb9ea1cc7 | Translated (100%) | Option 1. | 选项 1. |
| 1115d2513e86-baba-4e50-b411-d37bb9ea1cc7 | Translated (0%) | Environmental Product Declaration (EPD) (1 point) | 环保产品声明 (EPD)（1 分） |
| 1116eebd9b8d-4696-4eba-ba53-c1848130cb23 | Translated (0%) | Use at least 20 different permanently installed products sourced from at least five different manufacturers that meet one of the disclosure criteria below. | 使用至少 20 种不同的永久安装的产品，这些产品采购自至少 5 个不同的满足以下某一种信息披露标准的制造商。 |
| 111702e5f168-0512-4499-914b-b4e1586469ec | Translated (0%) | Product-specific declaration. | 产品特定声明。 |
| 11189c73e058-3188-4a4b-bfca-f949e3941761 | Translated (0%) | Products with a publicly available, critically reviewed life-cycle assessment conforming to ISO 14044 that have at least a cradle to gate scope are valued as one quarter (1/4) of a product for the purposes of credit achievement calculation. | 具有公开的、经过严格审查的生命周期评估（符合至少有“从摇篮到大门”范围的 ISO 14044）的产品在进行得分点计算时按产品成本的四分之一 (1/4) 估价。 |
| 1119be421a41-b884-407d-8634-8df3384205f3 | Translated (0%) | Environmental Product Declarations which conform to ISO 14025, 14040, 14044, and EN 15804 or ISO 21930 and have at least a cradle to gate scope. | 符合 ISO 14025、14040、14044 和 EN 15804 或 ISO 21930 且至少有“从摇篮到大门”范围的环保产品声明。 |
| 1120c719ed11-7605-4cdc-8321-263041c75c80 | Translated (0%) | Industry-wide (generic) EPD -- Products with third-party certification (Type III), including external verification, in which the manufacturer is explicitly recognized as a participant by the program operator are valued as one half (1/2) of a product for purposes of credit achievement calculation. | 行业范围内的（通用）EPD - 具有第三方认证（III 类）（包括制造商被计划运营商明确视为参与者的外部验证）的产品在得分点计算中按产品成本的一半 (1/2) 估价。 |
| 1121b4ca09de-0fc5-4a56-8747-c0d40dbcae51 | Translated (84%) | Product-specific Type III EPD -- Products with third-party certification (Type III), including external verification in which the manufacturer is explicitly recognized as the participant by the program operator are valued as one whole product for purposes of credit achievement calculation. | 产品特定的 III 类 EPD - 具有第三方认证（III 类）（包括制造商被计划运营商明确视为参与者的外部验证）的产品在得分点计算中按整个产品价值估价。 |
| 11220195c5f6-4433-4946-b403-c7881e87dca6 | Translated (0%) | USGBC approved program – Products that comply with other USGBC approved environmental product declaration frameworks. | 经过 USGBC 批准的计划 - 符合其他经过 USGBC 批准的环保产品声明框架的产品。 |
| 11232636b161-ba67-4468-88fe-3691c492b73b | Translated (100%) | Option 2. | 选项 2. |
| 11242636b161-ba67-4468-88fe-3691c492b73b | Translated (0%) | Multi-Attribute Optimization (1 point) | 多属性优化（1 分） |
| 1125a687703d-99b0-4154-90b0-2f621ca0b1e0 | Translated (0%) | Use products that comply with one of the criteria below for 50%, by cost, of the total value of permanently installed products in the project. | 对于占到项目中永久安装的产品总价值 50%（按成本）的产品，使用符合以下条件之一的产品。 |
| 1126a687703d-99b0-4154-90b0-2f621ca0b1e0 | Translated (0%) | Products will be valued as below. | 产品估价方法如下。 |
| 112794b8b8e8-7d9d-47df-bf12-7d36485966cd | Translated (0%) | Third party certified products that demonstrate impact reduction below industry average in at least three of the following categories are valued at 100% of their cost for credit achievement calculations. | 对于经过第三方认证的产品（在以下至少三个类别中减少的影响低于行业平均水平），在得分点计算中按成本的 100% 估价。 |
| 11286963a67b-ec91-4f52-81ff-0f07125cf85b | Translated (0%) | global warming potential (greenhouse gases), in CO<1585>2</1585>e; | 全球变暖潜能值（温室气体），单位为 CO<1585>2</1585>e； |
| 11296d3ce7d6-44a3-48c1-a190-22ce456b4d15 | Translated (0%) | depletion of the stratospheric ozone layer, in kg CFC-11; | 平流层臭氧层消耗，单位为 kg CFC-11； |
| 1130da3d8666-9f6e-41b8-a3cc-0e4152f844b3 | Translated (0%) | acidification of land and water sources, in moles H+ or kg SO<1586>2</1586>; | 土地和水资源酸化，单位为摩尔 H+ 或 kg SO<1586>2</1586>； |
| 113118d52638-462f-48f5-ba77-e8c28afdeddc | Translated (0%) | eutrophication, in kg nitrogen or kg phosphate; | 藻类污染，单位为 kg 氮或 kg 磷； |
| 1132b2aace4b-e30c-41ac-a3c2-eeddc75ce4c7 | Translated (0%) | formation of tropospheric ozone, in kg NOx, kg O3, or kg ethene; and | 平流层臭氧形成，单位为 kg NOx、kg O3 或 kg 乙烯；以及 |
| 1133b2beacd6-e77e-47c0-a596-0d38d7ccd2ff | Translated (0%) | depletion of nonrenewable energy resources, in MJ. | 非可再生能源消耗，单位为 MJ。 |
| 11345832a05f-0d87-4810-a18b-f7829bdaa37b | Translated (80%) | USGBC approved program -- Products that comply with other USGBC approved multi-attribute frameworks. | 经过 USGBC 批准的计划 - 符合其他经过 USGBC 批准的多属性框架的产品。 |
| 11352730975f-8766-4b62-8b29-8410a141f1c6 | Translated (0%) | For credit achievement calculation, products sourced (extracted, manufactured, purchased) within 100 miles (160 km) of the project site are valued at 200% of their base contributing cost. | 对于得分点计算，从项目场址 100 英里（160 公里）范围内获得（开采、制造、购买）的产品将按照基本贡献成本的 200% 估价。 |
| 1136f2490a01-ef0e-41a7-abc7-51d3db04e90c | Translated (0%) | Structure and enclosure materials may not constitute more than 30% of the value of compliant building products. | 结构和外围护结构材料不得超过符合要求的建筑产品价值的 30%。 |
| 1137343378b7-acc1-4493-8572-5d3612e41f2b | Translated (74%) | For all options | 对于所有选项 |
| 1138e1428fc4-3b6f-44ea-8aee-7419a0f3bcd2 | Translated (0%) | Meet the requirements of the credit above and include furniture and furnishings within the project’s scope of work. | 满足上述得分点的要求，并包括项目工作范围内的家具和陈设。 |
| 1139c5660a76-ee22-4e90-8cdc-d50b866f6842 | Translated (81%) | MR Credit: Building Product Disclosure and Optimization – Sourcing of Raw Materials | MR 得分点：建筑产品的分析公示和优化 - 原材料的来源和采购 (Building Product Disclosure and Optimization – Sourcing of Raw Materials) |
| 114063c13cfb-cb90-4fcd-b40a-38674208bc5c | Translated (100%) | ID&C | ID&C |
| 114166a15c05-4f58-4f38-9f69-abaa97176628 | Translated (100%) | 1–2 points | 1-2 分 |
| 1142fdaf8124-3941-419e-b911-abe9c6d343da | Translated (100%) | This credit applies to | 该得分点适用于 |
| 1143141f2485-308f-4fad-b980-838717c214fb | Translated (100%) | Commercial Interiors (1–2 points) | 商业室内（1-2 分） |
| 1144d3ecd5fa-9b4a-4bdb-89b0-9186d1683e70 | Translated (100%) | Retail (1–2 points) | 零售（1-2 分） |
| 11457e5a4e61-8858-455c-ad03-d376bae014d5 | Translated (100%) | Hospitality (1–2 points) | 宾馆接待（1-2 分） |
| 114668e786ac-4c82-4c8c-af3a-6bf1abb351cc | Translated (100%) | Intent | 目的 |
| 11473fc3fdc8-2b37-44c7-9e60-3b3e8c35b085 | Translated (89%) | To encourage the use of products and materials for which life cycle information is available and that have environmentally, economically, and socially preferable life cycle impacts. | 鼓励使用提供生命周期信息且具有有利的环境、经济和社会生命周期影响的产品和材料。 |
| 11483fc3fdc8-2b37-44c7-9e60-3b3e8c35b085 | Translated (0%) | To reward project teams for selecting products verified to have been extracted or sourced in a responsible manner. | 奖励选择被证明以负责的方式开采或采购的产品的项目团队。 |
| 1149b14cace0-8e7c-489e-8b52-44d25f4bf481 | Translated (100%) | Requirements | 要求 |
| 1150c39a09ef-d592-4bc4-9cea-54e0e4999923 | Translated (CM) | CI, Retail CI, Hospitality CI | 商业室内，零售商业室内，宾馆接待商业室内 |
| 115161f63ea5-ef9c-4e68-8420-70cc004d051c | Translated (CM) | Option 1. | 选项 1. |
| 115261f63ea5-ef9c-4e68-8420-70cc004d051c | Translated (0%) | Raw Material Source and Extraction Reporting (1 point) | 原材料来源和开采报告（1 分） |
| 11535772d67e-fd20-44f1-8807-6ec7b990dfc1 | Translated (0%) | Use at least 20 different permanently installed products from at least five different manufacturers that have publicly released a report from their raw material suppliers which include raw material supplier extraction locations, a commitment to long-term ecologically responsible land use, a commitment to reducing environmental harms from extraction and/or manufacturing processes, and a commitment to meeting applicable standards or programs voluntarily that address responsible sourcing criteria | 使用至少 20 种不同的永久安装的产品，这些产品来自至少 5 个不同的制造商，这些制造商均有来自其原材料供应商的公开发布的报告，包括原材料供应商开采位置、对长期土地使用生态责任的承诺、对减少开采和/或制造过程中环境危害的承诺，以及对自愿满足涉及负责任的采购标准的适用标准或计划的承诺。 |
| 1154e0a78e81-4c29-43e0-998d-aec75f5d868d | Translated (0%) | Products sourced from manufacturers with self-declared reports are valued as one half (1/2) of a product for credit achievement. | 采购自具有自我声明报告的制造商的产品在得分点计算中将按照产品成本的一半 (1/2) 来估价。 |
| 11556db10cfc-eae3-4d4a-8c5c-75f1693c6999 | Translated (0%) | Third-party verified corporate sustainability reports (CSR) which include environmental impacts of extraction operations and activities associated with the manufacturer’s product and the product’s supply chain, are valued as one whole product for credit achievement calculation. | 具有经过第三方认证的企业可持续性报告 (CSR)（包括与制造商的产品和产品供应链相关的开采操作和活动的环境影响）的产品在得分点计算时将按照整个产品价值来估价。 |
| 11566db10cfc-eae3-4d4a-8c5c-75f1693c6999 | Translated (0%) | Acceptable CSR frameworks include the following: | 可接受的 CSR 框架包括： |
| 1157ecf1c390-2efa-40fb-a41c-3365cef6555b | Translated (0%) | Global Reporting Initiative (GRI) Sustainability Report | 全球报告倡议 (GRI) 可持续性报告 |
| 1158a2237932-0194-4c56-9841-bb103aa80545 | Translated (0%) | Organisation for Economic Co-operation and Develoment (OECD) Guidelines for Multinational Enterprises | 跨国企业的经济合作与开发组织 (OECD) 指南 |
| 115906f45492-9bbb-47e4-9f41-c834ed2767d1 | Translated (0%) | U.N. | 联合国 |
| 116006f45492-9bbb-47e4-9f41-c834ed2767d1 | Translated (0%) | <1597>Global Compact</1597>: Communication of Progress | <Bold>全球影响</Bold>：进度沟通 |
| 11612b3bf738-01cd-4f82-9f83-8f7d6eb13d9e | Translated (0%) | <1598>ISO 26000:</1598> 2010 Guidance on Social Responsibility | <1598>ISO 26000：</1598>2010 社会责任指南 |
| 1162e7bf879b-5e83-49f3-9fc8-380632819086 | Translated (0%) | <1599>USGBC approved program<1600>: </1600> </1599>Other USGBC approved programs meeting the CSR criteria. | <1599>USGBC 批准的计划：</1599>其他符合 CSR 标准的经过 USGBC 批准的计划。 |
| 11634b62f680-082a-44d6-a9b8-61cf5020bf8f | Translated (100%) | Option 2. | 选项 2. |
| 11644b62f680-082a-44d6-a9b8-61cf5020bf8f | Translated (0%) | Leadership Extraction Practices (1 point) | 领先开采实践（1 分） |
| 1165b2e7cff8-8b7f-426b-a142-c732881c4c15 | Translated (80%) | Use products that meet at least one of the responsible extraction criteria below for at least 25%, by cost, of the total value of permanently installed building products in the project. | 对于至少占到项目中永久安装的建筑产品总价值 25%（按成本）的产品，使用至少符合以下负责任的开采条件之一的产品。 |
| 116665fc3b53-9987-4441-a1eb-cf2df3ac815f | Translated (0%) | Extended producer responsibility. | 扩大生产商责任。 |
| 116765fc3b53-9987-4441-a1eb-cf2df3ac815f | Translated (0%) | Products purchased from a manufacturer (producer) that participates in an extended producer responsibility program or is directly responsible for extended producer responsibility. | 从参与或直接负责扩大生产商责任计划的制造商（生产商）那里采购的产品。 |
| 116865fc3b53-9987-4441-a1eb-cf2df3ac815f | Translated (0%) | Products meeting extended producer responsibility criteria are valued at 50% of their cost for the purposes of credit achievement calculation. | 符合扩大生产商责任标准的产品在得分点计算时将按照成本的 50% 来估价。 |
| 116932d99018-f22c-4967-8e4e-430bc0bb7488 | Translated (0%) | Bio-based materials. | 生物基材料。 |
| 117032d99018-f22c-4967-8e4e-430bc0bb7488 | Translated (0%) | Bio-based products must meet the Sustainable Agriculture Network’s Sustainable Agriculture Standard. | 生物基产品必须符合可持续农业网络 (Sustainable Agriculture Network) 的可持续农业标准 (Sustainable Agriculture Standard)。 |
| 117132d99018-f22c-4967-8e4e-430bc0bb7488 | Translated (0%) | Bio-based raw materials must be tested using ASTM Test Method D6866 and be legally harvested, as defined by the exporting and receiving country. | 生物基原材料必须使用 ASTM 测试方法 D6866 进行测试，并且在收集时要符合出口国和进口国的法律要求。 |
| 117232d99018-f22c-4967-8e4e-430bc0bb7488 | Translated (0%) | Exclude hide products, such as leather and other animal skin material. | 不包括兽皮产品，如皮革及其他动物皮料。 |
| 117332d99018-f22c-4967-8e4e-430bc0bb7488 | Translated (90%) | Products meeting bio-based materials criteria are valued at 100% of their cost for the purposes of credit achievement calculation. | 符合生物基材料标准的产品在得分点计算时将按照成本的 100% 来估价。 |
| 1174c23afac0-605b-4527-bc78-7ee28519644d | Translated (0%) | Wood products. | 木制品。 |
| 1175c23afac0-605b-4527-bc78-7ee28519644d | Translated (0%) | Wood products must be certified by the Forest Stewardship Council or USGBC-approved equivalent. | 木制品必须通过森林管理委员会 (Forest Stewardship Council) 或 USGBC 认可的当地等效标准的认证。 |
| 1176c23afac0-605b-4527-bc78-7ee28519644d | Translated (93%) | Products meeting wood products criteria are valued at 100% of their cost for the purposes of credit achievement calculation. | 符合木制品标准的产品在得分点计算时将按照成本的 100% 来估价。 |
| 11776cdec266-2e1c-4927-b220-dd50c03cf229 | Translated (93%) | Materials reuse<1607>.</1607> | 材料再利用。 |
| 11786cdec266-2e1c-4927-b220-dd50c03cf229 | Translated (0%) | Reuse includes<1608> </1608>salvaged, refurbished, or reused products. | 再利用包括回收利用、翻新或再利用的产品。 |
| 11796cdec266-2e1c-4927-b220-dd50c03cf229 | Translated (93%) | Products meeting materials reuse criteria are valued at 100% of their cost for the purposes of credit achievement calculation. | 符合材料再利用标准的产品在得分点计算中将按照成本的 100% 来估价。 |
| 118099cd665b-6e3b-4d25-983d-2a346bc9f471 | Translated (94%) | Recycled content. | 回收物质含量。 |
| 118199cd665b-6e3b-4d25-983d-2a346bc9f471 | Translated (0%) | Recycled content is the sum of postconsumer recycled content plus one-half the preconsumer recycled content, based on cost. | 回收物质含量是消费后回收物质含量加上消费前回收物质含量一半的总和（基于成本）。 |
| 118299cd665b-6e3b-4d25-983d-2a346bc9f471 | Translated (93%) | Products meeting recycled content criteria are valued at 100% of their cost for the purposes of credit achievement calculation | 符合回收物质含量标准的产品在得分点计算中将按照成本的 100% 来估价。 |
| 1183a8c96bac-e252-4c32-a0d0-dd3114a280c7 | Translated (0%) | USGBC approved program<1613>.</1613> | 经过 USGBC 认证的计划。 |
| 1184a8c96bac-e252-4c32-a0d0-dd3114a280c7 | Translated (0%) | Other USGBC approved programs meeting leadership extraction criteria. | 其他经过 USGBC 认证且满足领先开采标准的计划。 |
| 1185a03ebe01-d828-47b7-a188-5443173fbc1e | Translated (98%) | For credit achievement calculation, products sourced (extracted, manufactured andpurchased) within 100 miles (160 km) of the project site are valued at 200% of their base contributing cost. | 从项目场址 100 英里（160 公里）范围内获得（开采、制造、购买）的产品在得分点计算中将按照基本贡献成本的 200% 估价。 |
| 1186a03ebe01-d828-47b7-a188-5443173fbc1e | Translated (0%) | For credit achievement calculation, the base contributing cost of individual products compliant with multiple responsible extraction criteria is not permitted to exceed 100% its total actual cost (before regional multipliers) and double counting of single product components compliant with multiple responsible extraction criteria is not permitted and in no case is a product permitted to contribute more than 200% of its total actual cost. | 对于得分点计算，符合多个负责任的开采条件的单个产品的基本贡献成本不允许超过其实际总成本的 100%（在区域乘数之前），并且不允许重复计入符合多个负责任的开采条件的单个产品组件。无论什么情况，产品都不允许贡献超过其实际总成本 200% 的成本。 |
| 1187d25c321b-0ffe-49cc-a216-80c9c1dfb8d6 | Translated (100%) | Structure and enclosure materials may not constitute more than 30% of the value of compliant building products. | 结构和外围护结构材料不得超过符合要求的建筑产品价值的 30%。 |
| 1188153b4704-17ae-43fd-9224-51e4b36548ae | Translated (CM) | For all options | 对于所有选项 |
| 11894bf40e43-4178-4132-b4ec-414f04fe98e9 | Translated (CM) | Meet the requirements of the credit above and include furniture and furnishings within the project’s scope of work. | 满足上述得分点的要求，并包括项目工作范围内的家具和陈设。 |
| 1190b6cc5156-2bdb-4dda-bd84-e5d8dfb9d9e0 | Translated (76%) | MR Credit: Building Product Disclosure and Optimization – Material Ingredients | MR 得分点：建筑产品的分析公示和优化 - 材料成分 (Building Product Disclosure and Optimization – Material Ingredients) |
| 11912dad6e33-f891-42bb-acde-e4a544a87369 | Translated (100%) | ID&C | ID&C |
| 1192f5f6eb2c-3562-48be-9161-cc87be24f4fa | Translated (100%) | 1-2 points | 1-2 分 |
| 1193bc8bd267-5255-480e-b3f3-c706c5f3ade4 | Translated (CM) | This credit applies to | 该得分点适用于 |
| 1194b933f016-49b3-47d7-a0aa-f6d66d465d00 | Translated (100%) | Commercial Interiors (1-2 points) | 商业室内（1-2 分） |
| 11952ef7b867-6037-46f4-9cc9-b1463b91764b | Translated (CM) | Retail (1-2 points) | 零售（1-2 分） |
| 1196e37d157b-207a-4c25-bd7c-8a593fa6ef6a | Translated (CM) | Hospitality (1-2 points) | 宾馆接待（1-2 分） |
| 1197887c6cec-497e-49c3-9d5f-13fd839a8580 | Translated (100%) | Intent | 目的 |
| 119869e8fa01-bdd0-41b0-915e-fac680b6b848 | Translated (99%) | To encourage the use of products and materials for which life-cycle information is available and that have environmentally, economically, and socially preferable life-cycle impacts. | 鼓励使用提供生命周期信息且具有有利的环境、经济和社会生命周期影响的产品和材料。 |
| 119969e8fa01-bdd0-41b0-915e-fac680b6b848 | Translated (0%) | To reward project teams for selecting products for which the chemical ingredients in the product are inventoried using an accepted methodology and for selecting products verified to minimize the use and generation of harmful substances. | 奖励选择产品中的化学成分使用可接受的方法列出的产品，以及选择被证明可以最大程度减少有害物质使用和产生的产品的项目团队。 |
| 120069e8fa01-bdd0-41b0-915e-fac680b6b848 | Translated (0%) | To reward raw material manufacturers who produce products verified to have improved life-cycle impacts. | 奖励生产的产品被证明有改善的生命周期影响的原材料制造商。 |
| 1201cb6d3e70-4d52-45de-bbd1-ee5a02258c2e | Translated (100%) | Requirements | 要求 |
| 12023606be52-3a48-4ae1-8870-c659cdfcd8e6 | Translated (CM) | CI, Retail CI, Hospitality CI | 商业室内，零售商业室内，宾馆接待商业室内 |
| 12031f6af3bd-f2ec-4055-8f31-970f2832ac4b | Translated (CM) | Option 1. | 选项 1. |
| 12041f6af3bd-f2ec-4055-8f31-970f2832ac4b | Translated (0%) | Material Ingredient Reporting (1 point) | 材料成分报告（1 分） |
| 1205b02e157e-acaf-4b9e-bef1-6503ad9b8192 | Translated (0%) | Use at least 20 different permanently installed products from at least five different manufacturers that use any of the following programs to demonstrate the chemical inventory of the product to at least 0.1% (1000 ppm). | 使用至少 20 种不同的永久安装的产品，这些产品来自至少 5 个不同的制造商，而这些制造商使用以下任一计划来公布含量占产品总量 0.1% (1000 ppm) 以上的化学物质清单。 |
| 1206c7774096-0456-47c0-b8ab-6872f35ad907 | Translated (0%) | <1621>Manufacturer Inventory</1621>. | <Italic>制造商清单</Italic>。 |
| 1207c7774096-0456-47c0-b8ab-6872f35ad907 | Translated (0%) | The manufacturer has published complete content inventory for the product following these guidelines: | 制造商按照这些指南公布了完整的产品成分清单： |
| 1208d40ef9ce-3348-4d2c-9bf4-103e5ae040b0 | Translated (0%) | A publicly available inventory of all ingredients identified by name and Chemical Abstract Service Registration Number (CASRN) | 包含由名称和化学文摘服务社登记编号 (CASRN) 标识的所有成分的公开清单。 |
| 120918b9189d-bbc1-49db-b24d-9102a4edd4a6 | Translated (0%) | Materials defined as trade secret or intellectual property may withhold the name and/or CASRN but must disclose role, amount and GreenScreen benchmark, as defined in GreenScreen v1.2. | 被定义为商业秘密或知识产权的材料可以不公开名称和/或 CASRN，但必须披露 GreenScreen v1.2 中规定的作用、数量和 GreenScreen 基准。 |
| 121030baa832-503c-46d2-9b12-ab6bdfcab073 | Translated (0%) | Health Product Declaration. | 健康产品声明。 |
| 121130baa832-503c-46d2-9b12-ab6bdfcab073 | Translated (0%) | The end use product has a published, complete Health Product Declaration with full disclosure of known hazards in compliance with the Health Product Declaration open Standard. | 最终用途产品具有公开的、完整的健康产品声明，其中根据健康产品声明公开标准完整披露了已知的危险。 |
| 1212f8a9ebb2-8501-4dcf-9aa8-82072d8358c4 | Translated (74%) | Cradle to Cradle. | 从摇篮到摇篮。 |
| 1213f8a9ebb2-8501-4dcf-9aa8-82072d8358c4 | Translated (0%) | The end use product has been certified at the Cradle to Cradle v2 Basic level or Cradle to Cradle v3 Bronze level. | 最终用途产品已经通过了从摇篮到摇篮 v2 基本等级或从摇篮到摇篮 v3 铜级认证。 |
| 1214d3b38b29-c4b0-4b5a-a89a-a40118589eca | Translated (100%) | USGBC approved program<1625>.</1625> | 经过 USGBC 认证的计划。 |
| 1215d3b38b29-c4b0-4b5a-a89a-a40118589eca | Translated (0%) | Other USGBC approved programs meeting the material ingredient reporting criteria. | 其他经过 USGBC 认证且符合材料成分报告标准的计划。 |
| 12169ba60055-3a18-4328-bc19-84e04cc74989 | Translated (100%) | AND/OR | 和/或 |
| 121760235c83-365b-4c10-94ac-950e8e0328ee | Translated (0%) | Option 2: Material Ingredient Optimization (1 point) | 选项 2：材料成分优化（1 分） |
| 12184b1e570d-82c5-4006-9d4c-945eee935353 | Translated (77%) | Use products that document their material ingredient optimization using the paths below for at least 25%, by cost, of the total value of permanently installed products in the project. | 对于至少占到项目中永久安装的产品总价值 25%（按成本）的产品，使用通过以下途径记录材料成分优化的产品。 |
| 1219d1f727ac-245e-4406-b031-ad145850ae91 | Translated (74%) | <1627>GreenScreen v1.2 Benchmark</1627><1628>.</1628> | <Italic>GreenScreen v1.2 基准。</Italic> |
| 1220d1f727ac-245e-4406-b031-ad145850ae91 | Translated (0%) | Products that have fully inventoried chemical ingredients to 100 ppm that have no Benchmark 1 hazards: | 完全列出化学成分（达 100 ppm）且没有基准 1 危害的产品： |
| 1221697466bd-d02b-4b10-a482-821690aaa244 | Translated (0%) | If any ingredients are assessed with the GreenScreen List Translator, value these products at 100% of cost. | 如果有任何成分通过 GreenScreen List Translator 的评估，则按照 100% 的成本对这些产品估价。 |
| 12221870d64d-e7f5-4435-b76e-317d793ddb9d | Translated (73%) | If all ingredients are have undergone a full GreenScreen Assessment, value these products at 150% of cost. | 如果所有成分都通过了完整的 GreenScreen 评估，则按照 150% 的成本对这些产品估价。 |
| 1223454d358d-15ed-4590-a92c-d13b18d6d700 | Translated (78%) | <1629>Cradle to Cradle Certified</1629><1630>.</1630> | <Italic>经过从摇篮到摇篮的认证。</Italic> |
| 1224454d358d-15ed-4590-a92c-d13b18d6d700 | Translated (0%) | End use products are certified Cradle to Cradle. | 最终用途产品经过从摇篮到摇篮的认证。 |
| 1225454d358d-15ed-4590-a92c-d13b18d6d700 | Translated (88%) | Products will be valued as follows: | 产品估价方法如下： |
| 122657e9ffb1-c177-4af3-92a7-99f37a12d263 | Translated (0%) | Cradle to Cradle v2 Gold: 100% of cost | 从摇篮到摇篮 v2 金级：100% 的成本 |
| 12270d30ab80-b176-4839-9950-fbf1d00157fd | Translated (92%) | Cradle to Cradle v2 Platinum: 150% of cost | 从摇篮到摇篮 v2 铂金级：150% 的成本 |
| 122842313074-ff20-4587-9df0-280cabbece2b | Translated (83%) | Cradle to Cradle v3 Silver: 100% of cost | 从摇篮到摇篮 v3 银级：100% 的成本 |
| 1229b2638101-9c90-44c6-b2ce-0284b7b4ceba | Translated (77%) | Cradle to Cradle v3 Gold or Platinum: 150% of cost | 从摇篮到摇篮 v3 金级或铂金级：150% 的成本 |
| 1230432679a3-24a1-414c-a10e-0b5be99bfa2f | Translated (0%) | <1631>International Alternative Compliance Path – REACH Optimization</1631>. | <Italic>国际替代合规路径 - REACH 优化</Italic>。 |
| 1231432679a3-24a1-414c-a10e-0b5be99bfa2f | Translated (0%) | End use products and materials that do not contain substances that meet REACH criteria for substances of very high concern. | 不包含符合高关注度物质 REACH 标准的最终用途产品和材料。 |
| 1232432679a3-24a1-414c-a10e-0b5be99bfa2f | Translated (0%) | If the product contains no ingredients listed on the REACH Authorization or Candidate list, value at 100% of cost. | 如果产品不包含 REACH 授权或候选列表中列出的成分，则按照 100% 的成本估价。 |
| 1233014c9736-25b7-4eef-a453-628424e26cc4 | Translated (100%) | USGBC approved program<1633>.</1633> | 经过 USGBC 认证的计划。 |
| 1234014c9736-25b7-4eef-a453-628424e26cc4 | Translated (0%) | Products that comply with USGGBC approved building product optimization criteria. | 符合经过 USGGBC 批准的建筑产品优化标准的产品。 |
| 1235c6e55c7d-7dd8-44f3-a073-4edcdd9198e0 | Translated (100%) | AND/OR | 和/或 |
| 123692b8136a-7591-4e4c-93a0-c3ba3ff2f5fb | Translated (0%) | Option 3: Product Manufacturer Supply Chain Optimization (1 point) | 选项 3：产品制造商供应链优化（1 分） |
| 123762864f17-dd0a-4a62-b808-435e397cb753 | Translated (0%) | Use building products <1635>for at least 25%, by cost, of the total value of permanently installed products in the<1636> </1636></1635><1637>project</1637> that: | <1635>对于至少占到<1636></1636></1635><1637>项目</1637>中永久安装的产品总价值 25% 的产品，使用以下建筑产品： |
| 12381d359d1c-098f-467d-93af-db934096620a | Translated (95%) | http://new.usgbc.org/glossary/term/4727 | http://new.usgbc.org/glossary/term/4727 |
| 1239a13fbf96-b290-4761-92ab-87caa8f1cfe5 | Translated (0%) | Are sourced from product manufacturers who engage in validated and robust safety, health, hazard, and risk programs which at a minimum document at least 99% (by weight) of the ingredients used to make the building product or building material, and | 采购自参与经过验证且可靠的安全、健康、危险和风险计划的产品制造商，这些计划记录了用于制造建筑产品或建筑材料的原料中至少 99%（按重量）的成分，以及 |
| 124040bb2a75-c8d1-4458-8433-7559dbfd1b2b | Translated (0%) | Are sourced from product manufacturers with independent third party verification of their supply chain that at a minimum verifies: | 采购自会对其供应链进行独立第三方验证的产品制造商，且至少应验证： |
| 124131c28a70-98b1-4065-88ea-00ab92e3a608 | Translated (0%) | Processes are in place to communicate and transparently prioritize chemical ingredients along the supply chain according to available hazard, exposure and use information to identify those that require more detailed evaluation | 有现成的工艺，可在供应链上根据提供的危险、披露和使用信息沟通化学成分并透明地对其排定优先级，以确定需要更详细评估的成分。 |
| 12420178e4f7-8a0e-44c4-891f-d6b3be2f4e12 | Translated (0%) | Processes are in place to identify, document, and communicate information on health, safety and environmental characteristics of chemical ingredients | 有现成的工艺来确定、记录和沟通有关化学成分健康、安全和环境特性的信息。 |
| 12434f7c6481-f700-409c-b771-74b175c8dcf2 | Translated (0%) | Processes are in place to implement measures to manage the health, safety and environmental hazard and risk of chemical ingredients | 有现成的工艺来执行措施以管理化学成分的健康、安全及环境危害和风险。 |
| 12445f169bd1-00f4-4ace-8066-b5275cf02215 | Translated (0%) | Processes are in place to optimize health, safety and environmental impacts when designing and improving chemical ingredients | 有现成的工艺在设计和改进化学成分时来优化健康、安全和环境影响。 |
| 12454a25893f-1941-4b2e-9bb5-02ea92fea60e | Translated (0%) | Processes are in place to communicate, receive and evaluate chemical ingredient safety and stewardship information along the supply chain | 有现成的工艺来沟通、接收和评估供应链上的化学成分安全和管理信息。 |
| 1246f14f7f3b-0dd9-480c-b552-e8e5b146841e | Translated (0%) | Safety and stewardship information about the chemical ingredients is publicly available from all points along the supply chain | 有关化学成分的安全和管理信息在供应链上的所有阶段都公开提供。 |
| 1247239e47f1-7777-4308-9a74-8f06628764ea | Translated (93%) | Products meeting Option 3 criteria are valued at 100% of their cost for the purposes of credit achievement calculation. | 符合选项 3 标准的产品在得分点计算中将按照成本的 100% 来估价。 |
| 1248e45f2483-94ff-43d1-b8d0-4f80dfd672bb | Translated (87%) | For credit achievement calculation of options 2 and 3, products sourced (extracted, manufactured, andpurchased) within 100 miles (160 km) of the project site are valued at 200% of their base contributing cost. | 对于选项 2 和 3 的得分点计算，从项目场址 100 英里（160 公里）范围内获得（开采、制造、购买）的产品将按照基本贡献成本的 200% 估价。 |
| 1249e45f2483-94ff-43d1-b8d0-4f80dfd672bb | Translated (0%) | For credit achievement calculation, the value of individual products compliant with either option 2 or 3 can be combined to reach the 25% threshold but products compliant with both option 2 and 3 may only be counted once. | 对于得分点计算，符合选项 2 或 3 的单个产品的价值可以合并以达到 25% 的阈值，但是符合选项 2 和 3 的产品只能计入一次。 |
| 12505e10b333-3af7-4f7b-85f6-8fb936c9a662 | Translated (100%) | Structure and enclosure materials may not constitute more than 30% of the value of compliant building products. | 结构和外围护结构材料不得超过符合要求的建筑产品价值的 30%。 |
| 125136a59b85-ff04-4195-9946-3790ea87cec3 | Translated (CM) | For all options | 对于所有选项 |
| 1252c1e9e41e-fd04-4bd5-913f-791b5b2af3e8 | Translated (100%) | Meet the requirements of the credit above and include furniture and furnishings within the project’s scope of work. | 满足上述得分点的要求，并包括项目工作范围内的家具和陈设。 |
| 1253f7fa0a07-fcb7-4d63-b728-cad034bca3be | Translated (82%) | MR Credit: Construction and Demolition Waste Management | MR 得分点：营建和拆建废弃物管理 (Construction and Demolition Waste Management) |
| 1254dd2eb426-306e-499b-a8e7-8359930d59d3 | Translated (100%) | ID&C | ID&C |
| 1255705958b9-8aa7-4651-969b-64e6fdc8ec3a | Translated (100%) | 1–2 points | 1-2 分 |
| 1256b5945cba-9262-4662-bbaf-f1d6c489e38e | Translated (100%) | This credit applies to | 该得分点适用于 |
| 1257c5155bbf-cbd4-4d82-bdb0-64ea6987bf57 | Translated (100%) | Commercial Interiors (1–2 points) | 商业室内（1-2 分） |
| 1258c7a0466e-deb6-46e5-8d57-a8b22d865ce8 | Translated (100%) | Retail (1–2 points) | 零售（1-2 分） |
| 1259f2fec3d4-a658-431f-bb71-10228042c3f7 | Translated (100%) | Hospitality (1–2 points) | 宾馆接待（1-2 分） |
| 126019fd6973-a882-454b-819a-e6d98d2b488e | Translated (100%) | Intent | 目的 |
| 12617a9fbb43-a8d1-4a27-aa61-88a513355df7 | Translated (100%) | To reduce construction and demolition waste disposed of in landfills and incineration facilities by recovering, reusing, and recycling materials. | 回收、再利用材料，减少在填埋场和焚化设施中处理的营建和拆建废弃物。 |
| 1262560d7daa-3fcc-43bc-b780-4cfedbaea16a | Translated (100%) | Requirements | 要求 |
| 1263eda6f2b1-5ea8-4450-b445-986b8f245116 | Translated (100%) | CI, Retail CI, Hospitality CI | 商业室内，零售商业室内，宾馆接待商业室内 |
| 12646bdab832-a188-4701-88d6-017d70020200 | Translated (93%) | Recycle and/or salvage nonhazardous construction and demolition materials. | 回收和/或再利用无害的营建和拆建材料。 |
| 12656bdab832-a188-4701-88d6-017d70020200 | Translated (94%) | Calculations can be by weight or volume but must be consistent throughout. | 可以按照重量或体积来计算，但是要前后保持一致。 |
| 126606481dd1-b905-4b46-b3c7-523398cfb69e | Translated (0%) | Exclude excavated soil, land-clearing debris, and alternative daily cover (ADC). | 不包括挖出的土壤、场地清理产生的碎片和替代日常封盖物 (ADC)。 |
| 126706481dd1-b905-4b46-b3c7-523398cfb69e | Translated (0%) | Include wood waste converted to fuel (biofuel) in the calculations; other types of waste-to-energy are not considered diversion for this credit. | 在计算中包括转换为燃料（生物燃料）的木材废料；其他类型的“废物到能量”在该得分点中并不被视为转化。 |
| 1268f8c62c32-7e8c-4273-9d6c-4c6980fe9303 | Translated (0%) | However, for projects that cannot meet credit requirements using reuse and recycling methods, waste-to-energy systems may be considered waste diversion if the European Commission Waste Framework Directive 2008/98/EC and Waste Incineration Directive 2000/76/EC are followed and Waste to Energy facilities meet applicable European Committee for Standardization (CEN) EN 303 standards. | 但是，对于无法使用再利用和回收方法满足得分点要求的项目，如果遵守欧盟废弃物框架指令 2008/98/EC 和废弃物焚烧指令 2000/76/EC，且“废物到能量”设施符合适用的欧盟标准化 (CEN) EN 303 标准，则“废物到能量”系统可被视为废弃物转化。 |
| 12698cde5927-6d9d-47c6-a994-075eb64b8b0b | Translated (100%) | Option 1. | 选项 1. |
| 12708cde5927-6d9d-47c6-a994-075eb64b8b0b | Translated (80%) | Diversion (1–2 points) | 转化（1-2 分） |
| 1271a2a467eb-d6ca-4ae1-9897-21337fc748f6 | Translated (98%) | Path 1. | 途径 1. |
| 1272a2a467eb-d6ca-4ae1-9897-21337fc748f6 | Translated (0%) | Divert 50% and Three Material Streams (1 point) | 转化 50% 和三种材料流（1 分） |
| 1273810bfdd9-3fd8-4f1f-9e27-4da3d531227a | Translated (0%) | Divert at least 50% of the total construction and demolition material; diverted materials must include at least three material streams. | 转化至少 50% 的营建和拆建材料；转化的材料至少必须包括三种材料流。 |
| 1274ad90b82e-1210-46e3-9f8e-62d05fb73be9 | Translated (100%) | OR | 或 |
| 12754f211653-eecd-4765-9012-13a8aea68cd6 | Translated (100%) | Path 2. | 途径 2. |
| 12764f211653-eecd-4765-9012-13a8aea68cd6 | Translated (88%) | Divert 75% and Four Material Streams (2 points) | 转化 75% 和四种材料流（2 分） |
| 1277ed5ecf26-f08b-45d4-866a-f38ea3a109a9 | Translated (97%) | Divert at least 75% of the total construction and demolition material; diverted materials must include at least four material streams. | 转化至少 75% 的营建和拆建材料；转化的材料至少必须包括四种材料流。 |
| 127824a48fd9-c2c3-4a36-a05d-5990cc91e22b | Translated (100%) | OR | 或 |
| 12797b65956b-92db-4700-bb6f-6e51fcdf671c | Translated (CM) | Option 2. | 选项 2. |
| 12807b65956b-92db-4700-bb6f-6e51fcdf671c | Translated (75%) | Reduction of Total Waste Material (2 points) | 减少废弃物材料总量（2 分） |
| 1281b4ffc993-aa38-4f5e-bcf6-a84b46a11319 | Translated (0%) | Do not generate more than 2.5 pounds of construction waste per square foot (12.2 kilograms of waste per square meter) of the building’s floor area. | 在每平方英尺的建筑面积中产生的建筑废弃物不超过 2.5 磅（每平方米 12.2 千克废弃物）。 |
| 1282a8523c47-1258-4692-a0a9-7c709a40a24b | Translated (84%) | Indoor Environmental Quality (EQ) | 室内环境质量 (EQ) |
| 128363817526-0bdc-4b86-9470-f0c6fedb21ac | Translated (75%) | EQ Prerequisite: Minimum Indoor Air Quality Performance | EQ 先决条件：最低室内空气质量表现 (Minimum Indoor Air Quality Performance) |
| 1284fd644f8c-9a95-43c8-8929-a1e51eb1fd63 | Translated (100%) | Required | 必要项 |
| 1285ac5eae9d-8e4e-4628-adae-9bcb0339ddc7 | Translated (CM) | ID&C | ID&C |
| 128614182c5a-c221-4171-9132-fe935d05c1a7 | Translated (CM) | This prerequisite applies to | 该先决条件适用于 |
| 1287a0ac8c1d-b862-4ba6-b5d2-3eadf156c238 | Translated (100%) | Commercial Interiors | 商业室内 |
| 12884c40c3af-01a3-4279-8922-7d47c1263875 | Translated (100%) | Retail | 零售 |
| 128907c2b337-1229-4dc7-a651-b63feb307f13 | Translated (100%) | Hospitality | 宾馆接待 |
| 12905acc3fcb-73dc-446b-9474-58df5177bf93 | Translated (100%) | Intent | 目的 |
| 1291e3c430c3-ffa3-464b-a057-ea0113c47371 | Translated (0%) | To contribute to the comfort and well-being of building occupants by establishing minimum standards for indoor air quality (IAQ). | 建立室内空气质量 (IAQ) 最低标准，有助于改善建筑驻户的舒适和健康。 |
| 12927a8788f5-e1fd-481f-9d8c-bdd0adc4d287 | Translated (100%) | Requirements | 要求 |
| 1293e4df85c9-3fae-43d9-8efd-61f844ea8770 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 1294ab92e97b-5efe-4874-bbef-2f5322ab91a0 | Translated (0%) | Meet the requirements for both ventilation and monitoring. | 满足通风和监测要求。 |
| 1295f0427b31-7fdc-4b2c-a2cc-52e0a9513632 | Translated (100%) | Ventilation | 通风 |
| 12964cd34599-452e-4553-877f-77b8b9c7cd87 | Translated (70%) | Mechanically Ventilated Spaces | 采用机械通风的空间 |
| 12979845c431-f4db-42f7-b8f9-6f6024280734 | Translated (0%) | For mechanically ventilated spaces (and for mixed-mode systems when the mechanical ventilation is activated), chose one of the following cases. | 对于采用机械通风的空间（以及启用机械通风后的混合模式系统），选择以下某一种情况。 |
| 12982ce018ce-f322-4e7d-908a-334ce28f3105 | Translated (100%) | Case 1. | 情况 1. |
| 12992ce018ce-f322-4e7d-908a-334ce28f3105 | Translated (0%) | Systems Able to Meet Required Outdoor Airflow Rates | 系统能够满足要求的新风流速 |
| 130009515960-d348-4a74-bfda-258d69c42bef | Translated (100%) | Option 1. | 选项 1. |
| 130109515960-d348-4a74-bfda-258d69c42bef | Translated (0%) | ASHRAE Standard 62.1–2010 | ASHRAE 标准 62.1–2010 |
| 1302aa0bf56b-ad9a-453b-a549-ba120959c61e | Translated (0%) | Determine the minimum outdoor air intake flow for mechanical ventilation systems using the ventilation rate procedure from ASHRAE 62.1–2010 or a local equivalent, whichever is more stringent and meet the minimum requirements of ASHRAE Standard 62.1–2010, Sections 4–7, Ventilation for Acceptable Indoor Air Quality (with errata), or a local equivalent, whichever is more stringent. | 使用 ASHRAE 62.1–2010 或本地对应标准（以更严格的为准）的通风率程序确定机械通风系统的最小新风进气量，并满足 ASHRAE 标准 62.1–2010，第 4 到 7 部分室内空气质量达标的通风（带勘误表）或本地对应标准（以更严格的为准）的最低要求。 |
| 130329fafa52-87fb-41ae-a7f9-4a6ac514a00d | Translated (100%) | Option 2. | 选项 2. |
| 130429fafa52-87fb-41ae-a7f9-4a6ac514a00d | Translated (0%) | CEN Standards EN 15251–2007 and EN 13779–2007 | CEN 标准 EN 15251–2007 和 EN 13779–2007 |
| 13058dce085a-3884-4b78-b8e2-a589a4c4918b | Translated (0%) | Projects outside the U.S. may instead meet the minimum outdoor air requirements of Annex B of Comité Européen de Normalisation (CEN) Standard EN 15251–2007, Indoor environmental input parameters for design and assessment of energy performance of buildings addressing indoor air quality, thermal environment, lighting and acoustics; and meet the requirements of CEN Standard EN 13779–2007, Ventilation for nonresidential buildings, Performance requirements for ventilation and room conditioning systems, excluding Section 7.3, Thermal environment; 7.6, Acoustic environment; A.16; and A.17. | 美国以外的项目需满足欧洲标准化委员会 (CEN) 附录 B 标准 EN 15251–2007 用于设计和评估与室内空气质量、热环境、照明和声学效果相关的建筑能源性能的室内环境输入参数的最小新风量要求；并满足 CEN 标准 EN 13779–2007 非住宅建筑中的通风，通风和房间空调系统性能要求，第 7.3 节 - 热环境，7.6 - 声学效果环境，A.16 和 A.17 除外。 |
| 1306f67c3668-9d4d-4949-ad12-ef04115a96dc | Translated (100%) | Case 2. | 情况 2. |
| 1307f67c3668-9d4d-4949-ad12-ef04115a96dc | Translated (92%) | Systems Unable to Meet Required Outdoor Airflow Rates | 系统无法满足要求的新风流速 |
| 130831c08a69-de8a-45aa-9032-226975b86d71 | Translated (0%) | If meeting the outdoor airflow rates in Case 1 is not feasible because of the physical constraints of the existing ventilation system, complete an engineering assessment of the system’s maximum outdoor air delivery rate. | 如果由于现有通风系统的物理限制，无法满足情况 1 中的新风流速，需对系统的最大新风输送率进行工程评估。 |
| 130931c08a69-de8a-45aa-9032-226975b86d71 | Translated (0%) | Supply the maximum possible to reach the minimum setpoint in Case 1 and not less than 10 cubic feet per minute (5 liters per second) of outdoor air per person. | 提供尽可能大的值以满足情况 1 中的最低设定点，且不少于每人每分钟 10 立方英尺（每秒 5 升）的新风。 |
| 13102b8227bc-2787-4a2f-a76e-80b22bac2b02 | Translated (77%) | Naturally Ventilated Spaces | 采用自然通风的空间 |
| 13113e2b2e38-01d3-4473-8db7-b7c477ce8a22 | Translated (0%) | For naturally ventilated spaces (and for mixed-mode systems when the mechanical ventilation is inactivated), determine the minimum outdoor air opening and space configuration requirements using the natural ventilation procedure from ASHRAE Standard 62.1–2010 or a local equivalent, whichever is more stringent. | 对于采用自然通风（以及混合模式系统禁用机械通风时）的空间，使用 ASHRAE 标准 62.1–2010 或本地对应标准（以更严格的为准）中的自然通风程序确定最小新风开口和空间配置要求。 |
| 13123e2b2e38-01d3-4473-8db7-b7c477ce8a22 | Translated (0%) | Confirm that natural ventilation is an effective strategy for the project by following the flow diagram in the Chartered Institution of Building Services Engineers (CIBSE) Applications Manual AM10, March 2005, Natural Ventilation in Nondomestic Buildings, Figure 2.8 and meet the requirements of ASHRAE Standard 62.1–2010, Section 4, or a local equivalent, whichever is more stringent. | 按照英国皇家注册设备工程师协会 (CIBSE) 应用手册 AM10，2005 年 3 月，非住宅建筑自然通风中图 2.8 所示的流程图确定自然通风是一种有效的项目策略，并满足 ASHRAE 标准 62.1–2010，第 4 部分或本地对应标准（以更严格的为准）的要求。 |
| 13139d496aa0-ce3a-495c-b380-ab5f2baa8344 | Translated (0%) | All Spaces | 所有空间 |
| 13148f3a74f7-b867-4f4f-b4cf-37c2d62c096b | Translated (0%) | The indoor air quality procedure defined in ASHRAE Standard 62.1–2010 may not be used to comply with this prerequisite. | 不得使用 ASHRAE 标准 62.1–2010 中定义的室内空气质量程序来满足该先决条件。 |
| 13151044ea75-c741-41b9-bf43-b9b103347457 | Translated (100%) | Monitoring | 监测 |
| 13169c5b9976-af2a-49e8-a977-69dd019c8a28 | Translated (100%) | Mechanically Ventilated Spaces | 采用机械通风的空间 |
| 13170613dc75-5d0c-44e4-a0b1-ecbfd591ccc2 | Translated (77%) | For mechanically ventilated spaces (and for mixed-mode systems when the mechanical ventilation is activated), monitor outdoor air intake flow as follows: | 对于采用机械通风（以及混合模式系统启用机械通风时）的空间，按照以下方式监测室外进气量： |
| 1318993c6b5c-6907-44fd-a679-47bd6270b11a | Translated (0%) | For variable air volume systems with an outdoor air intake in the project scope of work, provide a direct outdoor airflow measurement device capable of measuring the minimum outdoor air intake flow with an accuracy of +/–10% of the design minimum outdoor airflow rate, as defined by the ventilation requirements above. | 按照上述通风要求的规定，对于在项目工作范围内带有新风进气的可变风量系统，提供能够测量最低新风进气量的直接新风测量设备，精度为设计的最低新风量的 +/–10%。 |
| 1319993c6b5c-6907-44fd-a679-47bd6270b11a | Translated (0%) | An alarm must indicate when the outdoor airflow value varies by 15% or more from the outdoor airflow setpoint. | 如果新风量相比新风设定值改变了 15% 或更多，则必须发出警报。 |
| 1320e3061af8-bed4-45a2-bae7-c28ba5f3dc08 | Translated (0%) | For constant-volume systems included in the project scope of work, balance outdoor airflow to the design minimum outdoor airflow rate defined by ASHRAE Standard 62.1–2010 (with errata), or higher. | 对于项目工作范围内包含的定量系统，将新风平衡为 ASHRAE 标准 62.1–2010（带勘误表）定义的最低新风设计流速或更大的值。 |
| 1321e3061af8-bed4-45a2-bae7-c28ba5f3dc08 | Translated (0%) | Install a current transducer on the supply fan, an airflow switch, or similar monitoring device. | 在送风风扇上安装电流传感器、气流开关或类似的监测设备。 |
| 1322ca95e87c-8f29-4751-b4b3-f0614995da9c | Translated (100%) | Naturally Ventilated Spaces | 采用自然通风的空间 |
| 132370b6507b-6f91-46bb-ae55-f42e1d396036 | Translated (78%) | For naturally ventilated spaces (and for mixed-mode systems when the mechanical ventilation is inactivated), comply with at least one of the following. | 对于采用自然通风（以及混合模式系统禁用机械通风时）的空间，符合以下至少一种情况。 |
| 1324a7994ca2-a1e6-47d3-b671-32555bb24e50 | Translated (0%) | Provide a direct exhaust airflow measurement device capable of measuring the exhaust airflow with an accuracy of +/–10% of the design minimum exhaust airflow rate. | 提供能够测量排气气流的排气气流直接测量设备，精度为最低排气气流设计速度的 +/–10%。 |
| 1325a7994ca2-a1e6-47d3-b671-32555bb24e50 | Translated (85%) | An alarm must indicate when airflow values vary by 15% or more from the exhaust airflow setpoint. | 如果气流值相比排气气流设定值改变了 15% 或更多，则必须发出警报。 |
| 13266d6622e6-ba5f-4a3e-ba11-18a1319c27d7 | Translated (0%) | Provide automatic indication devices on all natural ventilation openings intended to meet the minimum opening requirements. | 在用于满足最低开口要求的所有自然通风开口处提供自动指示设备。 |
| 13276d6622e6-ba5f-4a3e-ba11-18a1319c27d7 | Translated (0%) | An alarm must indicate when any one of the openings is closed during occupied hours. | 在使用期间如果有任意开口关闭，则必须发出警报。 |
| 13284950910d-01d3-4036-895e-30518e79d1a6 | Translated (0%) | Monitor carbon dioxide (CO<1664>2</1664>) concentrations within each thermal zone. | 在每个热区中监测二氧化碳 (CO<1664>2</1664>) 浓度。 |
| 13294950910d-01d3-4036-895e-30518e79d1a6 | Translated (0%) | CO<1665>2</1665> monitors must be between 3 and 6 feet (900 and 1 800 millimeters) above the floor and within the thermal zone. | CO<1665>2</1665> 监测仪必须距地面 3 到 6 英尺（900 到 1800 毫米），并且必须位于热区中。 |
| 13304950910d-01d3-4036-895e-30518e79d1a6 | Translated (0%) | CO<1666>2</1666> monitors must have an audible or visual indicator or alert the building automation system if the sensed CO<1667>2</1667> concentration exceeds the setpoint by more than 10%. | CO<1666>2</1666> 监测仪必须有声音或视觉指示器，或者在感应到 CO<1667>2</1667> 浓度超过设定值 10% 或更多时向建筑自动化系统发出警报。 |
| 13314950910d-01d3-4036-895e-30518e79d1a6 | Translated (0%) | Calculate appropriate CO<1668>2</1668> setpoints by using the methods in ASHRAE 62.1–2010, Appendix C. | 使用 ASHRAE 62.1–2010，附录 C 中的方法计算合适的 CO<1668>2</1668> 设定值。 |
| 133270b0896f-bd5b-48e3-859d-fa99fc63e89f | Translated (70%) | EQ Prerequisite: Environmental Tobacco Smoke Control | EQ 先决条件：环境烟控 (Environmental Tobacco Smoke Control) |
| 1333783862f1-350d-4512-9dca-d25c8dc0799e | Translated (100%) | Required | 必要项 |
| 13344941a197-b296-495d-af02-61abd26ec1f0 | Translated (CM) | ID&C | ID&C |
| 133565bd7ca4-0669-4dda-82ef-89ddfdc5f875 | Translated (CM) | This prerequisite applies to | 该先决条件适用于 |
| 13364275885f-2c03-438a-921d-23ae73488135 | Translated (100%) | Commercial Interiors | 商业室内 |
| 1337a0f954ae-3659-47df-a0cb-617c923b11a5 | Translated (100%) | Retail | 零售 |
| 133884431263-fb42-41dc-ac7b-2a11d84a95e1 | Translated (100%) | Hospitality | 宾馆接待 |
| 133931641522-cc0a-467b-9d05-3742ae36414f | Translated (100%) | Intent | 目的 |
| 1340db087f6d-7f63-4e72-99de-4a3e00386ca5 | Translated (96%) | To prevent or minimize exposure of building occupants, indoor surfaces, and ventilation air distribution systems to environmental tobacco smoke. | 防止或尽量减少让建筑驻户、室内表面和通风空气配送系统接触环境烟害。 |
| 1341189737cf-c821-447e-a4f0-0786ecf80ba8 | Translated (100%) | Requirements | 要求 |
| 1342ff2eb44a-7599-4990-a11d-a846f2f29266 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 13434f070869-31ab-4b03-aa54-0b86289e2129 | Translated (0%) | Locate the project in a building that prohibits smoking by all occupants and users both inside the building and outside the building except in designated smoking areas located at least 25 feet (7.5 meters) from all entries, outdoor air intakes, and operable windows. | 将项目置于这种建筑中：禁止所有驻户和用户在建筑内和建筑外吸烟，但指定的吸烟区除外，这些吸烟区与所有入口、室外进气口和活动窗的距离至少为 25 英尺（7.5 米）。 |
| 13444f070869-31ab-4b03-aa54-0b86289e2129 | Translated (0%) | Also prohibit smoking outside the property line in spaces used for business purposes. | 还要禁止在商务用途空间的地界线以外吸烟。 |
| 13451113ea74-ccea-42ef-bae7-e10c271c7ccc | Translated (0%) | If the requirement to prohibit smoking within 25 feet (7.5 meters) cannot be implemented because of code, provide documentation of these regulations. | 如果因为法规而导致无法执行在 25 英尺（7.5 米）内禁止吸烟的要求，请提供这些法规文件。 |
| 1346edb987f7-8ff0-4c20-9c77-e8e1328b6f80 | Translated (0%) | Signage must be posted within 10 feet (3 meters) of all building entrances indicating the no-smoking policy. | 必须在距所有建筑入口 10 英尺（3 米）的距离内设置标志，指示无烟政策。 |
| 1347f38e1dcc-4674-47a3-8aa6-6f17fbd53105 | Translated (89%) | EQ Credit: Enhanced Indoor Air Quality Strategies | EQ 得分点：增强室内空气质量策略 (Enhanced Indoor Air Quality Strategies) |
| 1348488fb805-0488-4c71-b556-747cdde127ba | Translated (100%) | ID&C | ID&C |
| 13497b881347-3d9b-4830-b72d-552323f11b57 | Translated (100%) | 1–2 points | 1-2 分 |
| 13502e5a2024-4272-4cbf-ab5c-5aa855171ef1 | Translated (100%) | This credit applies to | 该得分点适用于 |
| 1351cb8ff910-0718-42fa-a632-d686757e42c7 | Translated (100%) | Commercial Interiors (1–2 points) | 商业室内（1-2 分） |
| 13526a4f2ac5-46ce-494a-88f2-4f69b39e6d07 | Translated (100%) | Retail (1–3 points) | 零售（1-3 分） |
| 135363445f83-de61-4c42-8a25-154a9dd8b1ea | Translated (100%) | Hospitality (1–2 points) | 宾馆接待（1-2 分） |
| 1354f6d390f7-38ae-4850-9e75-c60e27bb957b | Translated (100%) | Intent | 目的 |
| 13557aa2b6b7-560e-4fdf-b794-1afa7c9810f0 | Translated (0%) | To promote occupants’ comfort, well-being, and productivity by improving indoor air quality. | 通过提高室内空气质量改善驻户的舒适、健康和生产效率。 |
| 1356b27e2819-4d54-4f4e-b7f1-170df2f28406 | Translated (100%) | Requirements | 要求 |
| 13572047c83e-1a1a-4a11-acad-4fedb49c9d80 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 1358be68f263-b737-423e-8965-a582264c5a32 | Translated (CM) | Option 1. | 选项 1. |
| 1359be68f263-b737-423e-8965-a582264c5a32 | Translated (0%) | Enhanced IAQ Strategies (1 point) | 增强 IAQ 策略（1 分） |
| 1360f4f2bea3-2d2a-4cc3-b06e-6abb9df29fb8 | Translated (0%) | Comply with the following requirements, as applicable. | 符合以下要求（如适用）。 |
| 1361909570de-1db9-400f-96d1-5fb4ae12309e | Translated (93%) | Mechanically ventilated spaces: | 采用机械通风的空间： |
| 1362e7f54351-b659-4440-97cd-e91ac299f526 | Translated (94%) | entryway systems; | 入口通道系统； |
| 13634040e5e8-db13-4ec9-af64-6b9e6ef32081 | Translated (0%) | interior cross-contamination prevention; and | 防止室内交叉污染；以及 |
| 13643ebb41a8-3cca-44a6-bec3-533a259bbc15 | Translated (0%) | filtration. | 过滤。 |
| 13656e515771-52aa-4a44-a4ec-ae9176e9a289 | Translated (93%) | Naturally ventilated spaces: | 采用自然通风的空间： |
| 1366d7170414-f8ac-4dd7-936e-ec72bc5ab0c0 | Translated (71%) | entryway systems; and | 入口通道系统；以及 |
| 1367de135661-d5a6-4f1a-85aa-46c6ae849d1c | Translated (0%) | natural ventilation design calculations. | 自然通风设计计算。 |
| 136862e122fc-b421-43f8-8c7b-749b016de886 | Translated (0%) | Mixed-mode systems: | 混合模式系统： |
| 136950a0ffc4-25e9-42f4-a64f-bdf1bcb924b2 | Translated (100%) | entryway systems; | 入口通道系统； |
| 13704c157037-b2f1-451e-adff-2cda9881c754 | Translated (80%) | interior cross-contamination prevention; | 防止室内交叉污染； |
| 13719481ebd0-168a-401d-b0cd-258084025842 | Translated (96%) | filtration; | 过滤； |
| 13728f4f7d1e-d5ae-4820-b14a-e7329ab3ca73 | Translated (83%) | natural ventilation design calculations; and | 自然通风设计计算；以及 |
| 13734a5db068-45f8-4225-8a9b-243bac4aab76 | Translated (0%) | mixed-mode design calculations. | 混合模式设计计算。 |
| 1374870df114-a242-42b7-b430-3009cf1d7461 | Translated (70%) | A. Entryway Systems | A. 入口通道系统 |
| 1375a8976f49-dfab-4187-a72d-5cf206495947 | Translated (84%) | Install permanent entryway systems at least 10 feet (3 meters) long in the primary direction of travel to capture dirt and particulates entering the building at regularly used exterior entrances. | 在主要行进方向上安装至少 10 英尺（3 米）长的永久入口通道系统，以在常用外部入口处捕捉进入建筑的灰尘和颗粒物。 |
| 1376a8976f49-dfab-4187-a72d-5cf206495947 | Translated (0%) | Acceptable entryway systems include permanently installed grates, grilles, slotted systems that allow for cleaning underneath, rollout mats, and any other materials manufactured as entryway systems with equivalent or better performance. | 可接受的入口通道系统包括永久安装的格栅、鞋筛、允许清洗底部的开槽系统、活动地垫，以及其他任何作为具有同等或更高性能的入口通道系统生产的材料。 |
| 1377a8976f49-dfab-4187-a72d-5cf206495947 | Translated (0%) | Maintain all on a weekly basis. | 每周维护一次。 |
| 13783bdd9bdb-14f4-463d-981a-bad4e57c3d71 | Translated (70%) | B. Interior Cross-Contamination Prevention | B. 防止室内交叉污染 |
| 13794ffb9ef8-ea03-488a-a3d3-038e72098696 | Translated (0%) | Sufficiently exhaust each space where hazardous gases or chemicals may be present or used (e.g., garages, housekeeping and laundry areas, copying and printing rooms), using the exhaust rates determined in EQ Prerequisite Minimum Indoor Air Quality Performance or a minimum of 0.50 cfm per square foot (2.54 l/s per square meter), to create negative pressure with respect to adjacent spaces when the doors to the room are closed. | 使用 EQ 先决条件：最低室内空气质量表现 (Minimum Indoor Air Quality Performance) 中确定的排气率或者最少为每平方英尺 0.50 cfm（每平方米 2.54 l/s）的排气率，对可能存在或使用危险气体或化学物质的空间（如车库、家务和洗衣区、复印和打印室）进行充分排气，从而在房间门关闭后相对于相邻空间形成负压。 |
| 13804ffb9ef8-ea03-488a-a3d3-038e72098696 | Translated (99%) | For each of these spaces, provide self-closing doors and deck-to-deck partitions or a hard-lid ceiling. | 对于每个这样的空间，请提供自掩门和楼层间隔板，或者硬盖天花板。 |
| 1381af3dad33-27de-49e4-ab78-76ec573ab4aa | Translated (0%) | C. Filtration | C. 过滤 |
| 1382e19abc60-3c09-48d9-9a90-8133cc9831e2 | Translated (0%) | Each ventilation system that supplies outdoor air to occupied spaces must have particle filters or air-cleaning devices that meet one of the following filtration media requirements: | 每个为使用空间提供新风的通风系统都必须使用满足以下过滤介质要求之一的颗粒过滤器或空气清洁设备： |
| 138309f66bca-68b0-4e84-9566-dcc6285c5ccd | Translated (72%) | minimum efficiency reporting value (MERV) of 13 or higher, in accordance with ASHRAE Standard 52.2–2007; or | 最低效率报告值 (MERV) 为 13 或更高，符合 ASHRAE 标准 52.2–2007；或 |
| 13844297861a-58f4-476f-876f-070426043df3 | Translated (86%) | Class F7 or higher as defined by CEN Standard EN 779–2002, Particulate Air Filters for General Ventilation, Determination of the Filtration Performance. | F7 或更高类别，由 CEN 标准 EN 779–2002，“一般通风用空气粒子过滤器”，“过滤性能确定”定义。 |
| 13850f71a91c-e0e8-4990-99ef-7b2463a2b077 | Translated (0%) | Replace all air filtration media after completion of construction and before occupancy. | 在施工完成和使用之前更换所有空气过滤介质。 |
| 13865cbb8029-8403-4c0c-ac06-6b119db07a0d | Translated (75%) | D. Natural Ventilation Design Calculations | D. 自然通风设计计算 |
| 1387926886d2-2286-4fe1-b68f-3f1aad3d2135 | Translated (0%) | Demonstrate that the system design for occupied spaces employs the appropriate strategies in Chartered Institution of Building Services Engineers (CIBSE) Applications Manual AM10, March 2005, Natural Ventilation in Non-Domestic Buildings, Section 2.4. | 表明使用空间的系统设计采用了英国皇家注册设备工程师协会 (CIBSE) 应用手册 AM10，2005 年 3 月，非住宅建筑自然通风第 2.4 节中合适的策略。 |
| 1388453e25ba-15f0-474a-a6a6-c26d86461733 | Translated (70%) | E. Mixed-Mode Design Calculations | E. 混合模式设计计算 |
| 13899c2dff85-8b2f-4dd9-ace8-594dbaa2f83d | Translated (0%) | Demonstrate that the system design for occupied spaces complies with CIBSE Applications Manual 13–2000, Mixed Mode Ventilation. | 表明使用空间的系统设计符合 CIBSE 应用手册 13–2000，混合模式通风的要求。 |
| 1390c09f9b20-755d-4193-8b41-8ac12eb779e5 | Translated (100%) | Option 2. | 选项 2. |
| 1391c09f9b20-755d-4193-8b41-8ac12eb779e5 | Translated (75%) | Additional Enhanced IAQ Strategies (1 point ID&C, 2 points Retail CI) | 其他增强 IAQ 策略（ID&C 1 分，商业室内零售 2 分） |
| 13926fc83c77-2345-4526-a6df-198ebbe180bc | Translated (100%) | Comply with the following requirements, as applicable. | 符合以下要求（如适用）。 |
| 13937b581581-679d-4f82-82c2-b39817b0350c | Translated (0%) | Mechanically ventilated spaces (select one): | 采用机械通风的空间（选择一项）： |
| 139427f487c5-ec4a-48c8-b9e2-fdff54b8a846 | Translated (0%) | exterior contamination prevention; | 防止室外污染； |
| 13953a611501-3c99-4750-9f4c-60d97718c263 | Translated (89%) | increased ventilation; | 提高换气量； |
| 1396290715d2-0fd9-4662-a6d7-ebb785b8c748 | Translated (0%) | carbon dioxide monitoring; or | 二氧化碳监测；或 |
| 13978f661575-4f9f-4e1c-9119-c306d1e6cbde | Translated (0%) | additional source control and monitoring. | 其他来源控制和监测。 |
| 1398d15fcac0-abcc-4249-a7c7-f3745a372685 | Translated (87%) | Naturally ventilated spaces (select one): | 采用自然通风的空间（选择一项）： |
| 1399c80c9b36-61fb-4334-ad8e-d6039ce39d49 | Translated (100%) | exterior contamination prevention; | 防止室外污染； |
| 140063e8c759-9bba-4de2-b6ea-3d154465049f | Translated (86%) | additional source control and monitoring; or | 其他来源控制和监测；或 |
| 1401fdea10d5-68a1-4751-9366-bd5759cbe647 | Translated (0%) | natural ventilation room by room calculations. | 自然通风逐个房间计算。 |
| 14022ffec30b-0ec1-4d04-8eaf-69534edba864 | Translated (0%) | Mixed-mode systems (select one): | 混合模式系统（选择一项）： |
| 14035a863e90-5701-459e-88c4-4bd3e4509774 | Translated (100%) | exterior contamination prevention; | 防止室外污染； |
| 1404878b7470-e02b-4fc6-8d90-53d8f5e38fef | Translated (CM) | increased ventilation; | 提高换气量； |
| 14055bfda1ed-2712-410d-97c1-d8658a41762d | Translated (100%) | additional source control and monitoring; or | 其他来源控制和监测；或 |
| 1406367337c2-b016-4337-bd46-eca3942401d8 | Translated (83%) | natural ventilation room-by-room calculations. | 自然通风逐个房间计算。 |
| 14070ec11243-5692-4f28-a609-7a875e9cd338 | Translated (70%) | A. Exterior Contamination Prevention | A. 防止室外污染 |
| 140816819139-0023-4911-8a33-a91b1f0bb631 | Translated (0%) | Design the project to minimize and control the entry of pollutants into the building. | 设计项目以尽量减少和控制污染物进入建筑。 |
| 140916819139-0023-4911-8a33-a91b1f0bb631 | Translated (0%) | Ensure through the results of computational fluid dynamics modeling, Gaussian dispersion analyses, wind tunnel modeling, or tracer gas modeling that outdoor air contaminant concentrations at outdoor air intakes are below the thresholds listed in Table 1 (or local equivalent for projects outside the U.S., whichever is more stringent). | 通过计算流体动力学建模、高斯分布分析、风洞建模或示踪气体建模的结果确保室外进气口的新风污染物浓度低于表 1 中列出的阈值（美国以外的项目为本地对应值，以更严格的为准）。 |
| 1410070e54a7-86a4-47ee-952a-f4613bd24cd7 | Translated (100%) | Table 1. | 表 1. |
| 1411070e54a7-86a4-47ee-952a-f4613bd24cd7 | Translated (0%) | Maximum concentrations of pollutants at outdoor air intakes | 室外进气口污染物的最大浓度 |
| 14124cba4c63-92c6-473f-9a92-2be0465e1177 | Translated (0%) | Pollutants | 污染物 |
| 1413a6de7c49-f0e6-4a69-8db4-6b07933109ad | Translated (96%) | Maximum concentration | 最大浓度 |
| 1414e23d7ee0-b0c8-42fd-ac2d-2628ed8d8c96 | Translated (100%) | Standard | 标准 |
| 1415f13911b1-46e7-4882-8241-edf184ca0c24 | Translated (0%) | Those regulated by National Ambient Air Quality Standards (NAAQS) | 受国家环境空气质量标准 (NAAQS) 管制的污染物 |
| 1416c35f8948-fe3c-447c-acae-71c33987be99 | Translated (0%) | Allowable annual average | 允许的年平均值 |
| 1417f715fceb-c84f-47c1-8774-825ba0b8b821 | Translated (100%) | OR | 或 |
| 14189e98c605-11d7-4db8-b94b-0775b482318f | Translated (0%) | 8-hour or 24-hour average where an annual standard does not exist | 如果没有年标准值，则为 8 小时或 24 小时平均值 |
| 14191636b853-d368-49f0-b400-43d693b0a4be | Translated (100%) | OR | 或 |
| 142058afc9fd-9096-4f18-86e8-bbcbbbc2cb7c | Translated (0%) | Rolling 3-month average | 连续 3 个月的平均值 |
| 14219adcaedb-5421-48d4-bc71-86e314c1d5d2 | Translated (72%) | National Ambient Air Quality Standards (NAAQS) | 国家环境空气质量标准 (NAAQS) |
| 1422ab4eaa65-7a13-4da9-9ba9-15b5f35d27de | Translated (96%) | B. Increased Ventilation | B. 提高换气量 |
| 142329cb5f5d-4ca9-4625-9aba-b14433fb286f | Translated (0%) | Increase breathing zone outdoor air ventilation rates to all occupied spaces by at least 30% above the minimum rates as determined in EQ Prerequisite Minimum Indoor Air Quality Performance. | 至少将所有使用空间的呼吸区域新风通风率增加到高于 EQ 先决条件：最低室内空气质量表现 (Minimum Indoor Air Quality Performance) 确定的最低速率的 30%。 |
| 14248fd45a6f-99ee-47ad-9963-1fbda54f6979 | Translated (78%) | C. Carbon Dioxide Monitoring | C. 二氧化碳监测 |
| 1425dbd9a567-977e-481d-856c-ade097c0b8e6 | Translated (0%) | Monitor CO<1720>2</1720> concentrations within all densely occupied spaces. | 监测所有密集使用空间的 CO<1720>2</1720> 浓度。 |
| 1426dbd9a567-977e-481d-856c-ade097c0b8e6 | Translated (82%) | CO<1721>2</1721> monitors must be between 3 and 6 feet (900 and 1 800 millimeters) above the floor. | CO<1721>2</1721> 监测仪必须距地面 3 到 6 英尺（900 到 1800 毫米）。 |
| 1427dbd9a567-977e-481d-856c-ade097c0b8e6 | Translated (100%) | CO<1722>2</1722> monitors must have an audible or visual indicator or alert the building automation system if the sensed CO<1723>2</1723> concentration exceeds the setpoint by more than 10%. | CO<1722>2</1722> 监测仪必须有声音或视觉指示器，或者在感应到 CO<1723>2</1723> 浓度超过设定值 10% 或更多时向建筑自动化系统发出警报。 |
| 1428dbd9a567-977e-481d-856c-ade097c0b8e6 | Translated (90%) | Calculate appropriate CO<1724>2</1724> setpoints using methods in ASHRAE 62.1–2010, Appendix C. | 使用 ASHRAE 62.1–2010，附录 C 中的方法计算合适的 CO<1724>2</1724> 设定值。 |
| 14290b1f0566-34db-4ccf-9eba-cb05117e038c | Translated (80%) | D. Additional Source Control and Monitoring | D. 其他来源控制和监测。 |
| 14301e783051-e0f3-4809-8528-6715de4464f9 | Translated (0%) | For spaces where air contaminants are likely, evaluate potential sources of additional air contaminants besides CO<1727>2</1727>. | 对于可能出现空气污染的空间，评估除 CO<1727>2</1727> 之外的其他空气污染物潜在来源。 |
| 14311e783051-e0f3-4809-8528-6715de4464f9 | Translated (0%) | Develop and implement a materials-handling plan to reduce the likelihood of contaminant release. | 制定和实施材料处理计划，以减少污染物释放的可能性。 |
| 14321e783051-e0f3-4809-8528-6715de4464f9 | Translated (0%) | Install monitoring systems with sensors designed to detect the specific contaminants. | 安装带有设计用于监测特定污染物的感应器的监测系统。 |
| 14331e783051-e0f3-4809-8528-6715de4464f9 | Translated (0%) | An alarm must indicate any unusual or unsafe conditions. | 只要出现不常见或不安全的情况，必须发出警报。 |
| 1434d2aee8b6-77c1-4843-9307-8ffe2a6052d3 | Translated (75%) | E. Natural Ventilation Room-by-Room Calculations | E. 自然通风逐个房间计算 |
| 1435badfc8eb-0e66-4ef7-974c-eb532a155bd6 | Translated (0%) | Follow CIBSE AM10, Section 4, Design Calculations, to predict that room-by-room airflows will provide effective natural ventilation. | 按照 CIBSE AM10，第 4 部分，设计计算来预测每个房间的气流能够提供有效的自然通风。 |
| 14364be490b2-ce08-4dfb-b654-96c419573ed8 | Translated (82%) | EQ Credit: Low-Emitting Materials | EQ 得分点：低逸散材料 (Low-Emitting Materials) |
| 14376e598fef-b86e-489c-bd34-425ff82fa954 | Translated (100%) | ID&C | ID&C |
| 14386181ee33-372d-49b5-b065-a36d3f85fa27 | Translated (100%) | 1–3 points | 1-3 分 |
| 14396d27b039-d9ad-4aac-a2af-b1aad8c0c9ca | Translated (100%) | This credit applies to | 该得分点适用于 |
| 14409352d2c5-f01e-46f8-b3a9-386da991ae08 | Translated (100%) | Commercial Interiors (1–3 points) | 商业室内（1-3 分） |
| 144158990ae7-e43e-4168-8bdc-1757ac19a818 | Translated (100%) | Retail (1–3 points) | 零售（1-3 分） |
| 144200503a05-0056-4e34-b9e5-6e1279b87f1b | Translated (100%) | Hospitality (1–3 points) | 宾馆接待（1-3 分） |
| 144313e58cae-7195-4a08-ac97-49f41d510094 | Translated (100%) | Intent | 目的 |
| 144442a18b48-00aa-4a10-bf09-1c44f7052b76 | Translated (0%) | To reduce concentrations of chemical contaminants that can damage air quality, human health, productivity, and the environment. | 减少会影响空气质量、人体健康、生产效率和环境的化学污染物的浓度。 |
| 14459b431a71-7130-4972-b9f0-15c0f76d039b | Translated (100%) | Requirements | 要求 |
| 1446826e1037-1f59-4d69-851f-893ebe335cc6 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 14472f9ddb52-f2e5-4638-8094-a3e163e1beb1 | Translated (0%) | This credit includes requirements for product manufacturing as well as project teams. | 该得分点包括对产品制造和项目团队的要求。 |
| 14482f9ddb52-f2e5-4638-8094-a3e163e1beb1 | Translated (0%) | It covers volatile organic compound (VOC) emissions into indoor air and the VOC content of materials, as well as the testing methods by which indoor VOC emissions are determined. | 它涵盖了排放到室内空气中的挥发性有机化合物 (VOC)、材料的 VOC 含量，以及确定室内 VOC 排放的测试方法。 |
| 14492f9ddb52-f2e5-4638-8094-a3e163e1beb1 | Translated (0%) | Different materials must meet different requirements to be considered compliant for this credit. | 不同的材料必须符合该得分点规定的不同要求。 |
| 14502f9ddb52-f2e5-4638-8094-a3e163e1beb1 | Translated (0%) | The building interior and exterior are organized in seven categories, each with different thresholds of compliance. | 建筑室内和室外分为 7 个类别，每个都有不同的合规阈值。 |
| 14512f9ddb52-f2e5-4638-8094-a3e163e1beb1 | Translated (0%) | The building interior is defined as everything within the waterproofing membrane. | 建筑室内被定义为防水膜内的所有项目。 |
| 14522f9ddb52-f2e5-4638-8094-a3e163e1beb1 | Translated (0%) | The building exterior is defined as everything outside and inclusive of the primary and secondary weatherproofing system, such as waterproofing membranes and air- and water-resistive barrier materials. | 建筑室外被定义为主要和辅助防水系统及其外部的所有项目，如防水膜以及防止空气和水的屏障材料。 |
| 14533e6214eb-61e8-41b8-adfa-e880942bb3e6 | Translated (100%) | Option 1. | 选项 1. |
| 14543e6214eb-61e8-41b8-adfa-e880942bb3e6 | Translated (70%) | Product Category Calculations | 产品类别计算 |
| 1455c7ee6524-2f2e-4244-adb1-8985981f9a77 | Translated (0%) | Achieve the threshold level of compliance with emissions and content standards for the number of product categories listed in Table 2. | 实现表 2 中所列产品类别数量的排放和含量标准的合规阈值等级。 |
| 145608f4e9d7-c195-423d-b718-215aa8bd43ea | Translated (100%) | Table 1. | 表 1. |
| 145708f4e9d7-c195-423d-b718-215aa8bd43ea | Translated (0%) | Thresholds of compliance with emissions and content standards for 6 categories of materials | 6 类材料排放和含量标准的合规阈值 |
| 14581dc1d381-a9ec-44c9-8ac5-309c846c06ba | Translated (86%) | Category | 类别 |
| 14596206e2ab-3567-443b-8f42-a5c74b3dcaf1 | Translated (96%) | Threshold | 阈值 |
| 146014eb9fa1-9f72-41fe-9444-bc28fb6a26e2 | Translated (0%) | Emissions and content requirements | 排放和含量要求 |
| 1461586e5454-63ff-4e70-8726-b903e81930e6 | Translated (0%) | Interior paints and coatings applied on site | 场址内使用的室内涂料和涂层 |
| 14620373400b-e741-4296-b5ee-fad18915774d | Translated (0%) | At least 90%, by volume, for emissions; 100% for VOC content | 排放为至少 90%（按照体积）；VOC 含量为 100% |
| 1463595937e4-9dac-4e57-867f-0d895ba52780 | Translated (0%) | General Emissions Evaluation for paints and coatings applied to walls, floors, and ceilings | 对墙壁、地板和天花板使用的涂料和涂层进行常规排放评估 |
| 14645aaf9d7b-796b-4cdf-84b0-39914c2d7dc5 | Translated (0%) | VOC content requirements for wet applied products | 加水使用的产品的 VOC 含量要求 |
| 1465d5848c3e-67e2-416b-a6f0-9a879b6236b8 | Translated (0%) | Interior adhesives and sealants applied on site (including flooring adhesive) | 场址内使用的室内粘着剂和密封胶（包括地板粘着剂） |
| 1466568cb86e-adb0-468d-811e-17772dbd1135 | Translated (100%) | At least 90%, by volume, for emissions; 100% for VOC content | 排放为至少 90%（按照体积）；VOC 含量为 100% |
| 1467eb234ead-8020-40b9-9420-6bcae4a2ac2b | Translated (0%) | General Emissions Evaluation | 常规排放评估 |
| 146839f216ab-77a3-4a71-9157-37570ab0b1ac | Translated (100%) | VOC content requirements for wet applied products | 加水使用的产品的 VOC 含量要求 |
| 1469df5496d8-6136-47f1-844c-13d3976b2886 | Translated (0%) | Flooring | 地板 |
| 147017ac0b45-fe32-4e7d-a24f-0d13ddc9ab7e | Translated (100%) | 100% | 100% |
| 1471b06dd660-87d7-4276-9d07-9ad730075323 | Translated (100%) | General Emissions Evaluation | 常规排放评估 |
| 14728257a600-e68e-4e33-ae74-24546bcd2350 | Translated (100%) | Composite wood | 复合木材 |
| 1473d71d6f61-a632-4bab-a38a-eba9604355d8 | Translated (0%) | 100% not covered by other categories | 100% 不被其他类别涵盖 |
| 1474c341f900-cf3d-4b7d-95be-0c6da57b4478 | Translated (70%) | Composite Wood Evaluation | 复合木材评估 |
| 14753baa5046-e42a-4b45-9c40-27ecd1f000aa | Translated (0%) | Ceilings, walls, thermal, and acoustic insulation | 天花板、墙壁、隔热与隔音 |
| 1476e7587dec-2870-46de-b6dd-974f94563a32 | Translated (100%) | 100% | 100% |
| 147740b6bd1d-acce-47ed-b4c1-2b49ad50905c | Translated (100%) | General Emissions Evaluation | 常规排放评估 |
| 1478eb224cda-b72a-4181-9ee8-4d08572a1296 | Translated (100%) | Furniture | 家具 |
| 1479a487a73e-d240-4f51-b9e4-af9c5291042c | Translated (0%) | At least 90%, by cost | 至少 90%（按成本） |
| 1480df13a2f3-76b1-45d7-9234-45ed2db7164a | Translated (0%) | Furniture Evaluation | 家具评估 |
| 14815fa44f9e-55eb-4bdc-8d37-84d1d8d1083d | Translated (100%) | Table 2. | 表 2. |
| 14825fa44f9e-55eb-4bdc-8d37-84d1d8d1083d | Translated (0%) | Points for number of compliant categories of products | 产品合规类别数量的分数 |
| 1483ea67968a-6369-4c9d-bc3e-ab71304007da | Translated (0%) | Compliant categories | 合规类别 |
| 1484fa9a069b-ac8b-45e8-aa07-2ebcc0e7306b | Translated (100%) | Points | 分数 |
| 148578b6c377-7862-4b24-a7a1-3e953864e65f | Translated (CM) | 3 | 3 |
| 1486eb7e9cd6-d6eb-4519-a4a2-fbfd189965ac | Translated (CM) | 1 | 1 |
| 1487de2adcb0-ed38-42ce-a37b-6d2e3908f0d0 | Translated (CM) | 5 | 5 |
| 1488baa13026-efaf-4775-b7a1-dfc65ad57856 | Translated (CM) | 2 | 2 |
| 14892e798642-400a-4014-b483-84a2b666811b | Translated (CM) | 6 | 6 |
| 149042c61061-4698-4bdf-a4d0-f0cb253c6d7d | Translated (CM) | 3 | 3 |
| 149155eb40ea-deef-4084-85ad-7d24c54a81ef | Translated (100%) | Option 2. | 选项 2. |
| 149255eb40ea-deef-4084-85ad-7d24c54a81ef | Translated (0%) | Budget Calculation Method | 预算计算方法 |
| 1493d4f8e84f-8d3f-4dab-9466-ceec3f8a7d31 | Translated (0%) | If some products in a category do not meet the criteria, project teams may use the budget calculation method (Table 3). | 如果类别中的某些产品不符合标准，项目团队可以使用预算计算方法（表 3）。 |
| 14947c2461b7-c200-4f4c-8111-6bc863a90c18 | Translated (100%) | Table 3. | 表 3. |
| 14957c2461b7-c200-4f4c-8111-6bc863a90c18 | Translated (0%) | Points for percentage compliance, under budget calculation method | 预算计算方法下的合规百分比分数 |
| 1496f7a55114-f5e2-4a11-bd23-ec084e240495 | Translated (0%) | Percentage of total | 总百分比 |
| 1497bb51e89d-0f5e-44d2-ba0a-f83ccbefa097 | Translated (100%) | Points | 分数 |
| 14988d5a04e5-b940-4a4b-9b61-ae88fe62dce9 | Translated (0%) | ≥ 50% and < 70% | ≥ 50% 且 < 70% |
| 14993b5f7e9d-95a1-497b-bf32-31874f219924 | Translated (100%) | 1 | 1 |
| 15009307d020-bfd9-40cf-b0dd-e5cc84414be5 | Translated (100%) | ≥ 70% and < 90% | ≥ 70% 且 < 90% |
| 1501ba8dba4c-8e16-4bc6-91b8-7f5f64c12475 | Translated (100%) | 2 | 2 |
| 15024425b67b-99e7-4a23-b200-02c56706276e | Translated (0%) | ≥ 90% | ≥ 90% |
| 15034b929493-0386-4c4b-835a-ce0e7b7900c6 | Translated (CM) | 3 | 3 |
| 1504db1be485-ea15-4864-92a2-0b6866359c6f | Translated (0%) | The budget method organizes the building interior into five assemblies: | 预算方法将建筑室内分为 5 种组件： |
| 1505d6281773-0e98-4dad-a304-afc8f1ba1983 | Translated (86%) | flooring; | 地板； |
| 15062dc8f19c-594d-44f4-a119-b7fbc68905b2 | Translated (0%) | ceilings; | 天花板； |
| 1507da1f9e75-18ab-46d6-a99b-3c5f90ec55d0 | Translated (0%) | walls; | 墙壁； |
| 15081489f07c-e608-49a1-ad2a-9fc93908fe83 | Translated (0%) | thermal and acoustic insulation; | 隔热与隔音层； |
| 1509f231d364-1a1f-4d10-8307-379cf07e6ec6 | Translated (0%) | furniture; and | 家具；且 |
| 1510706e8ecc-9c74-48f6-8d9e-b04f47dc4c5f | Translated (0%) | Walls, ceilings, and flooring are defined as building interior products; each layer of the assembly, including paints, coatings, adhesives, and sealants, must be evaluated for compliance. | 墙壁、天花板和地板被定义为建筑室内产品；组件的每一层，包括涂料、涂层、粘着剂和密封胶都必须进行合规评估。 |
| 1511706e8ecc-9c74-48f6-8d9e-b04f47dc4c5f | Translated (0%) | Insulation is tracked separately. | 单独跟踪隔热和隔音层。 |
| 151214190fc8-7159-490a-80c6-76c46057818c | Translated (0%) | Determine the total percentage of compliant materials according to Equation 1. | 根据等式 1 确定合规材料的总百分比。 |
| 1513e6a44a13-a834-4ff8-be31-d7410a4fd7af | Translated (98%) | Equation 1. | 等式 1. |
| 1514e6a44a13-a834-4ff8-be31-d7410a4fd7af | Translated (0%) | Total percentage compliance | 合规总百分比 |
| 1515d322a1e6-b8c4-4b35-a89f-b551b380671c | Translated (0%) | Total % compliant for projects with furniture = | 带有家具的项目的合规总百分比 = |
| 151631dc70f7-0c9e-493f-ae3b-0b5505294f3b | Translated (0%) | (% compliant walls + % compliant ceilings + % compliant flooring + % compliant insulation) + (% compliant furniture) | （合规墙壁的百分比 + 合规天花板的百分比 + 合规地板的百分比 + 合规隔热与隔音层的百分比） + （合规家具的百分比） |
| 1517710185f9-f143-493a-9c72-4bf59b611ab9 | Translated (100%) | 5 | 5 |
| 15188c2f148a-79db-480c-9d90-0a2c3159145a | Translated (100%) | Equation 2. | 等式 2. |
| 15198c2f148a-79db-480c-9d90-0a2c3159145a | Translated (0%) | System percentage compliant | 合规系统百分比 |
| 152063dee4ba-a6f9-4602-bf05-f871a7907d9b | Translated (0%) | Flooring, walls, ceilings,<1888> </1888> insulation % compliant = | 地板、墙壁、天花板、<1888></1888>隔热与隔音层合规百分比 = |
| 15213ef9a1cc-9b62-4a6f-ae6c-45ca8e78bdcb | Translated (0%) | (compliant surface area of layer 1 + compliant surface area of layer 2 + compliant surface area of layer 3 + …) | （第一层的合规表面积 + 第二层的合规表面积 + 第三层的合规表面积 + …） |
| 15220637ee92-b531-4324-b30b-acbb1cab952e | Translated (89%) | total surface area of layer 1 + total surface area of layer 2 + total surface area of layer 3 + …) | 第一层的总表面积 + 第二层的总表面积 + 第三层的总表面积 + …） |
| 15235d20065c-51f6-49dc-a8f4-4f577b734739 | Translated (100%) | X 100 | X 100 |
| 15243e338253-3b00-42fa-bc90-bd8871c7d2b6 | Translated (100%) | Equation 3. | 等式 3. |
| 15253e338253-3b00-42fa-bc90-bd8871c7d2b6 | Translated (0%) | Furniture systems compliant, using ANSI/BIFMA evaluation | 家具系统合规，使用 ANSI/BIFMA 评估 |
| 15268f34e714-5009-4359-95bb-7fe340228641 | Translated (0%) | % compliant for furniture = | 家具合规百分比 = |
| 152717a991b9-131a-4e63-bb64-54d86382ceea | Translated (0%) | 0.5 x cost compliant with §7.6.1 of ANSI/BIFMA e3-2011 + cost compliant with §7.6.2 of ANSI/BIFMA e3-2011 | 0.5 x 符合 ANSI/BIFMA e3-2011 §7.6.1 的成本 + 符合 ANSI/BIFMA e3-2011 §7.6.2 的成本 |
| 1528780752d6-dc53-4cc1-8466-3336967a1c06 | Translated (71%) | total furniture cost | 家具总成本 |
| 15299717b64d-1b55-44bd-bab0-c0dd528846ca | Translated (0%) | X 100 | X 100 |
| 15304e2abe2a-7dde-4678-982d-d48160768138 | Translated (0%) | Calculate surface area of assembly layers based on the manufacturer’s documentation for application. | 根据制造商的申请文件计算组件层的表面积。 |
| 15311fdff0c5-57ae-4975-99a8-787802b13b3d | Translated (0%) | If 90% of an assembly meets the criteria, the system counts as 100% compliant. | 如果 90% 的组件符合标准，系统将认为 100% 符合标准。 |
| 15321fdff0c5-57ae-4975-99a8-787802b13b3d | Translated (86%) | If less than 50% of an assembly meets the criteria, the assembly counts as 0% compliant. | 如果少于 50% 的组件符合标准，系统将认为完全不符合标准。 |
| 15330c359f48-4364-42ce-aa01-170f6e556a3b | Translated (0%) | Manufacturers’ claims<1919>.</1919> | 制造商要求。 |
| 15340c359f48-4364-42ce-aa01-170f6e556a3b | Translated (0%) | Both first-party and third-party statements of product compliance must follow the guidelines in CDPH SM V1.1–2010, Section 8. | 有关产品合规的第一方和第三方声明都必须符合 CDPH SM V1.1–2010，第 8 部分的指南。 |
| 15350c359f48-4364-42ce-aa01-170f6e556a3b | Translated (0%) | Organizations that certify manufacturers’ claims must be accredited under ISO Guide 65. | 证明制造商声明的组织必须通过 ISO Guide 65 的认证。 |
| 1536ae42de10-587b-4b5f-b05f-a31444daef44 | Translated (0%) | Laboratory requirements<1920>.</1920> | 实验室要求。 |
| 1537ae42de10-587b-4b5f-b05f-a31444daef44 | Translated (0%) | Laboratories that conduct the tests specified in this credit must be accredited under ISO/IEC 17025 for the test methods they use. | 进行本得分点中指定测试的实验室使用的测试方法必须经过 ISO/IEC 17025 认证。 |
| 153835893449-5bc4-4679-8519-55efa864ae49 | Translated (95%) | Emissions and Content Requirements | 排放和含量要求 |
| 1539f346094f-80ee-4524-84b0-f073bb745267 | Translated (0%) | To demonstrate compliance, a product or layer must meet all of the following requirements, as applicable. | 为了表明合规性，产品或层必须满足以下所有要求（如适用）。 |
| 15408f00a8eb-5e85-4570-afaa-3b18641579dc | Translated (0%) | Inherently nonemitting sources. | 固有的非排放源。 |
| 15418f00a8eb-5e85-4570-afaa-3b18641579dc | Translated (0%) | Products that are inherently nonemitting sources of VOCs (stone, ceramic, powder-coated metals, plated or anodized metal, glass, concrete, clay brick, and unfinished or untreated solid wood flooring) are considered fully compliant without any VOC emissions testing if they do not include integral organic-based surface coatings, binders, or sealants. | 作为本身不会排放 VOC 的产品（石材、陶瓷、粉末涂层金属、镀或电镀的金属、玻璃、混凝土、粘土砖和未抛光的或未经处理的实木地板）如果不带整合的有机表面涂层、粘合剂或密封胶，那么无需任何 VOC 排放测试即可被认为是符合标准的。 |
| 15425e0ee25a-1ed1-411b-a285-49305241c32b | Translated (93%) | General emissions evaluation. | 常规排放评估。 |
| 15435e0ee25a-1ed1-411b-a285-49305241c32b | Translated (0%) | Building products must be tested and determined compliant in accordance with California Department of Public Health (CDPH) Standard Method v1.1–2010, using the applicable exposure scenario. | 建筑产品必须按照加利福尼亚州公共健康部 (CDPH) 标准方法 V1.1-2010，使用适用的暴露场景进行测试并确认符合标准。 |
| 15445e0ee25a-1ed1-411b-a285-49305241c32b | Translated (0%) | The default scenario is the private office scenario. | 默认场景是私人办公室。 |
| 15455e0ee25a-1ed1-411b-a285-49305241c32b | Translated (0%) | The manufacturer’s or third-party certification must state the exposure scenario used to determine compliance. | 制造商或第三方认证必须声明用于确定合规性的暴露场景。 |
| 15465e0ee25a-1ed1-411b-a285-49305241c32b | Translated (0%) | Claims of compliance for wet-applied products must state the amount applied in mass per surface area. | 加水使用的产品的符合性声明必须说明单位表面区域的用量（重量）。 |
| 154756341a9f-cda1-43c9-b06f-37202c0efa85 | Translated (0%) | Manufacturers’ claims of compliance with the above requirements must also state the range of total VOCs after 14 days (336 hours), measured as specified in the CDPH Standard Method v1.1: | 制造商满足上述要求的符合性声明还必须说明按照 CDPH 标准方法 v1.1 测得的 14 天（336 小时）后的总 VOC 范围： |
| 1548bf211b17-3096-4dbe-aeec-ccf65a867d3a | Translated (0%) | 0.5 mg/m<1926>3 </1926>or less; | 0.5 mg/m<1926>3 </1926>或更少； |
| 1549a4b9a48e-ee3a-4265-a924-cb8e8e3c2aa5 | Translated (0%) | between 0.5 and 5.0 mg/m<1928>3</1928>; or | 在 0.5 和 5.0 mg/m<1928>3</1928> 之间；或 |
| 1550ef204259-2371-49e5-a0d8-e5e26abf3293 | Translated (86%) | 5.0 mg/m<1930>3 </1930>or more. | 5.0 mg/m<1930>3 </1930>或更多。 |
| 15517a53119e-35ac-48db-a010-fc1a2c0e7eea | Translated (0%) | Projects outside the U.S. may use products tested and deemed compliant in accordance with either (1) the CDPH standard method (2010) or (2) the German AgBB Testing and Evaluation Scheme (2010). | 美国以外的项目可以使用根据 (1) CDPH 标准方法 (2010) 或 (2) 德国 AgBB 测试与评估方案 (2010) 测试并符合要求的产品。 |
| 15527a53119e-35ac-48db-a010-fc1a2c0e7eea | Translated (0%) | Test products either with (1) the CDPH Standard Method (2010), (2) the German AgBB Testing and Evaluation Scheme (2010), (3) ISO 16000-3: 2010, ISO 16000-6: 2011, ISO 16000-9: 2006, ISO 16000-11:2006 either in conjunction with AgBB, or with French legislation on VOC emission class labeling, or (4) the DIBt testing method (2010). | 使用 (1) CDPH 标准方法 (2010)，(2) 德国 AgBB 测试和评估方案 (2010)，(3) ISO 16000-3: 2010、ISO 16000-6: 2011、ISO 16000-9: 2006、ISO 16000-11:2006 与 AgBB 或法国有关 VOC 排放等级标签相关的法规相结合，或 (4) DIBt 测试方法 (2010) 来测试产品。 |
| 15537a53119e-35ac-48db-a010-fc1a2c0e7eea | Translated (0%) | If the applied testing method does not specify testing details for a product group for which the CDPH standard method does provide details, use the specifications in the CDPH standard method. | 如果使用的测试方法没有指定产品组的测试详情，而 CDPH 标准方法已经提供了这些详情，则使用 CDPH 标准方法中的规范。 |
| 15547a53119e-35ac-48db-a010-fc1a2c0e7eea | Translated (0%) | U.S. projects must follow the CDPH standard method. | 美国项目必须遵循 CDPH 标准方法。 |
| 155595ef9c6b-68c7-4a05-9b50-6a8207570740 | Translated (0%) | Additional VOC content requirements for wet-applied products. | 加水使用的产品的其他 VOC 含量要求。 |
| 155695ef9c6b-68c7-4a05-9b50-6a8207570740 | Translated (0%) | In addition to meeting the general requirements for VOC emissions (above), on-site wet-applied products must not contain excessive levels of VOCs, for the health of the installers and other tradesworkers who are exposed to these products. | 除了满足常规的 VOC 排放要求（上述），为了安装人员和其他接触这些产品的工人的安全着想，场址内加水使用的产品不得具有过高的 VOC 含量。 |
| 155795ef9c6b-68c7-4a05-9b50-6a8207570740 | Translated (91%) | To demonstrate compliance, a product or layer must meet the following requirements, as applicable. | 为了表明合规性，产品或层必须满足以下要求（如适用）。 |
| 155895ef9c6b-68c7-4a05-9b50-6a8207570740 | Translated (0%) | Disclosure of VOC content must be made by the manufacturer. | VOC 含量的披露必须由制造商完成。 |
| 155995ef9c6b-68c7-4a05-9b50-6a8207570740 | Translated (0%) | Any testing must follow the test method specified in the applicable regulation. | 任何测试都必须遵循适用法规中指定的测试方法。 |
| 1560dfe83fa2-91ed-4e4b-959e-8413fad869a1 | Translated (0%) | All paints and coatings wet-applied on site must meet the applicable VOC limits of the California Air Resources Board (CARB) 2007, Suggested Control Measure (SCM) for Architectural Coatings, or the South Coast Air Quality Management District (SCAQMD) Rule 1113, effective June 3, 2011. | 所有在场址内加水使用的涂料和涂层都必须满足美国加州空气资源委员会 (CARB) 2007，建筑涂层的建议控制措施 (SCM)，或 2011 年 6 月 3 日生效的“南海岸空气质量管理地区”（South Coast Air Quality Management District (SQAQMD)) 规则 1113 中适用的 VOC 限制。 |
| 1561614013fc-ec38-4ea3-abea-7c57497a79ea | Translated (0%) | All adhesives and sealants wet-applied on site must meet the applicable chemical content requirements of SCAQMD Rule 1168, July 1, 2005, Adhesive and Sealant Applications, as analyzed by the methods specified in Rule 1168. | 所有在场址内加水使用的粘着剂和密封胶都必须满足 2005 年 7 月 1 日生效的 SCAQMD 规则 1168 粘着剂和密封胶应用中适用的化学品含量要求（由规则 1168 中指定的方法分析）。 |
| 1562614013fc-ec38-4ea3-abea-7c57497a79ea | Translated (0%) | The provisions of SCAQMD Rule 1168 do not apply to adhesives and sealants subject to state or federal consumer product VOC regulations. | SCAQMD 规则 1168 的条款不适用于受州或联邦消费品 VOC 法规约束的粘着剂和密封胶。 |
| 1563f70eb570-e1a2-4f6e-b6a1-7e393d12d512 | Translated (0%) | For projects outside the U.S., all paints, coatings, adhesives, and sealants wet-applied on site must either meet the technical requirements of the above regulations, or comply with applicable national VOC control regulations, such as the European Decopaint Directive (2004/42/EC), the Canadian VOC Concentration Limits for Architectural Coatings, or the Hong Kong Air Pollution Control (VOC) Regulation. | 对于美国以外的项目，所有在场址内加水使用的涂料、涂层、粘着剂和密封胶必须满足上述法规的技术要求，或符合适用的国家 VOC 控制法规，如欧洲 Decopaint 指令 (2004/42/EC)、加拿大建筑涂层 VOC 浓度限值，或香港空气污染控制 (VOC) 法规。 |
| 1564ba177e2a-3c79-44e1-a279-3efc1ef25764 | Translated (0%) | If the applicable regulation requires subtraction of exempt compounds, any content of intentionally added exempt compounds larger than 1% weight by mass (total exempt compounds) must be disclosed. | 如果适用法规要求减去豁免化合物，那么任何有意添加的重量大于总重量 1% 的豁免化合物含量均必须披露。 |
| 15655baca52e-1f67-4386-98e3-204aff014cd2 | Translated (0%) | If a product cannot reasonably be tested as specified above, testing of VOC content must comply with ASTM D2369-10; ISO 11890, part 1; ASTM D6886-03; or ISO 11890-2. | 如果产品无法按上述指定方法进行合理测试，那么 VOC 含量的测试必须符合 ASTM D2369-10；ISO 11890，第 1 部分；ASTM D6886-03 或 ISO 11890-2。 |
| 15669a70bf00-b6a1-4d9d-97ba-cb385081979d | Translated (0%) | For projects in North America, methylene chloride and perchloroethylene may not be intentionally added in paints, coatings, adhesives, or sealants. | 对于北美的项目，不得有意将二氯甲烷和全氯乙烯添加到涂料、涂层、粘着剂或密封胶中。 |
| 1567f219aac3-68ea-4a81-9872-988fdce74349 | Translated (99%) | Composite Wood Evaluation. | 复合木材评估。 |
| 1568f219aac3-68ea-4a81-9872-988fdce74349 | Translated (0%) | Composite wood, as defined by the California Air Resources Board, Airborne Toxic Measure to Reduce Formaldehyde Emissions from Composite Wood Products Regulation, must be documented to have low formaldehyde emissions that meet the California Air Resources Board ATCM for formaldehyde requirements for ultra-low-emitting formaldehyde (ULEF) resins or no added formaldehyde resins. | 美国加州空气资源委员会 (CARB) 旨在降低复合木材产品中甲醛排放的毒空气污染物控制措施法规定义的复合木材必须被证明甲醛排放量较低，符合美国加州空气资源委员会 ATCM 对超低甲醛排放 (ULEF) 树脂或不添加甲醛树脂的甲醛要求。 |
| 156989c49f1f-f7d0-424c-bc56-1387a86d8932 | Translated (0%) | Salvaged and reused architectural millwork more than one year old at the time of occupancy is considered compliant, provided it meets the requirements for any site-applied paints, coatings, adhesives, and sealants. | 在入驻时使用已经超过一年的回收和再利用建筑木制品被认为是符合标准的，前提是满足任何在场址内使用的涂料、涂层、粘着剂和密封胶的要求。 |
| 1570349b3f4c-b94c-4d95-95aa-1fb730e8e26a | Translated (94%) | Furniture evaluation. | 家具评估。 |
| 1571349b3f4c-b94c-4d95-95aa-1fb730e8e26a | Translated (0%) | New furniture and furnishing items<1934> </1934>must be tested in accordance with ANSI/BIFMA Standard Method M7.1–2011. | 新的家具和陈设项目必须根据 ANSI/BIFMA 标准方法 M7.1–2011 进行测试。 |
| 1572349b3f4c-b94c-4d95-95aa-1fb730e8e26a | Translated (0%) | Comply with ANSI/BIFMA e3-2011 Furniture Sustainability Standard, Sections 7.6.1 and 7.6.2, using either the concentration modeling approach or the emissions factor approach. | 使用浓度建模方法或排放因数方法符合 ANSI/BIFMA e3-2011 家具可持续性标准，第 7.6.1 和 7.6.2 部分。 |
| 1573349b3f4c-b94c-4d95-95aa-1fb730e8e26a | Translated (0%) | Model the test results using the open plan, private office, or seating scenario in ANSI/BIFMA M7.1, as appropriate. | 在适当的情况下，使用 ANSI/BIFMA M7.1 中的开放式私人办公室或座位场景对测试结果建模。 |
| 1574349b3f4c-b94c-4d95-95aa-1fb730e8e26a | Translated (0%) | USGBC-approved equivalent testing methodologies and contaminant thresholds are also acceptable. | 也接受 USGBC 认可的当地等效标准测试方法和污染物阈值。 |
| 1575349b3f4c-b94c-4d95-95aa-1fb730e8e26a | Translated (0%) | For classroom furniture, use the standard school classroom model in CDPH Standard Method v1.1. | 对于教室家具，使用 CDPH 标准方法 v1.1 中的标准学校教室模型。 |
| 1576349b3f4c-b94c-4d95-95aa-1fb730e8e26a | Translated (0%) | Documentation submitted for furniture must indicate the modeling scenario used to determine compliance. | 提交的家具文件必须指定用于确定合规的建模场景。 |
| 15776f9b9af8-0e55-4bfe-aa8d-10ff557de8cb | Translated (94%) | Salvaged and reused furniture more than one year old at the time of use is considered compliant, provided it meets the requirements for any site-applied paints, coatings, adhesives, and sealants. | 在入驻时使用已经超过一年的回收和再利用家具被认为是符合标准的，前提是满足任何在场址内使用的涂料、涂层、粘着剂和密封胶的要求。 |
| 1578ae03ed19-47ae-4172-ae8a-544fef7cf58f | Translated (81%) | EQ Credit: Construction Indoor Air Quality Management Plan | EQ 得分点：施工期室内空气质量管理计划 (Construction Indoor Air Quality Management Plan) |
| 15796fb8d82e-e4c3-48cc-8555-ea42cfa7099b | Translated (100%) | ID&C | ID&C |
| 1580892f43fa-bc42-4ba1-82d5-22f4fd22fb5f | Translated (100%) | 1 point | 1 分 |
| 15818227d690-428b-41e0-af3e-2856425a0324 | Translated (100%) | This credit applies to | 该得分点适用于 |
| 1582132a34fe-b6c8-46ff-9d4a-dd5fb885225a | Translated (CM) | Commercial Interiors (1 point) | 商业室内（1 分） |
| 158343d6788c-3611-445f-a2a0-e0d0fb929e39 | Translated (CM) | Retail (1 point) | 零售（1 分） |
| 158483496662-362c-41f3-b7fe-81b5d0c8b5d3 | Translated (CM) | Hospitality (1 point) | 宾馆接待（1 分） |
| 15854e82c5f8-3141-4e6b-b12e-f81c109eef12 | Translated (100%) | Intent | 目的 |
| 1586f65ef227-63d9-4da5-a6c9-8abb6a76a421 | Translated (0%) | To promote the well-being of construction workers and building occupants by minimizing indoor air quality problems associated with construction and renovation. | 尽量减少与施工和改造相关的室内空气质量问题，改善施工工人和建筑驻户的健康。 |
| 1587e9641ba6-b4e3-4104-9c6a-667b80bba147 | Translated (100%) | Requirements | 要求 |
| 1588ac0d6716-f02c-4f47-a2fd-bc356298a8f0 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 1589c5746829-726e-43c5-9225-ebdc01636e19 | Translated (77%) | Develop and implement an indoor air quality (IAQ) management plan for the construction and preoccupancy phases of the building. | 在建筑的施工和入驻前阶段制定和实施室内空气质量 (IAQ) 管理计划。 |
| 1590c5746829-726e-43c5-9225-ebdc01636e19 | Translated (84%) | The plan must address all of the following. | 该方案必须针对以下所有项目。 |
| 1591ad2212cb-1271-42d8-bed8-c2a8e69ea056 | Translated (82%) | During construction, meet or exceed all applicable recommended control measures of the Sheet Metal and Air Conditioning National Contractors Association (SMACNA) IAQ Guidelines for Occupied Buildings under Construction, 2nd edition, 2007, ANSI/SMACNA 008–2008, Chapter 3. | 在施工期间，满足或超出美国金属散热与空气调节承包商协会 (SMACNA)《施工过程中使用建筑的 IAQ 指南》(IAQ Guidelines For Occupied Buildings Under Construction)，2007 年第 2 版，ANSI/SMACNA 008-2008，第 3 章中建议的所有适用控制措施。 |
| 1592282e7957-44b7-4e27-b4d1-5167048943f8 | Translated (0%) | Protect absorptive materials stored on-site and installed from moisture damage. | 防止在场址内保存和安装的吸收性材料受潮损坏。 |
| 159315f4c5ca-7300-430e-828e-59f1a95a318b | Translated (0%) | Do not operate permanently installed air-handling equipment during construction unless filtration media with a minimum efficiency reporting value (MERV) of 8, as determined by ASHRAE 52.2–2007, with errata (or equivalent filtration media class of F5 or higher, as defined by CEN Standard EN 779–2002, Particulate Air Filters for General Ventilation, Determination of the Filtration Performance), are installed at each return air grille and return or transfer duct inlet opening such that there is no bypass around the filtration media. | 施工期间不要操作永久安装的空气处理设备，除非有由 ASHRAE 52.2–2007，带勘误表（或由 CEN 标准 EN 779–2002，“一般通风用空气粒子过滤器”，“过滤性能确定”定义的 F5 或更高值的等效过滤介质等级）确定的最低效率报告值（MERV）为 8 的过滤介质安装在每个回风口以及回风或送风管道入口，这样空气就不会绕过过滤介质。 |
| 159415f4c5ca-7300-430e-828e-59f1a95a318b | Translated (0%) | Immediately before occupancy, replace all filtration media with the final design filtration media, installed in accordance with the manufacturer’s recommendations. | 在入驻之前将所有过滤介质更换为最终设计的过滤介质，并根据制造商的建议安装。 |
| 1595fd211cfe-e94d-417d-aedd-3dc1cd4d1ab2 | Translated (0%) | Prohibit the use of tobacco products inside the building and within 25 feet (7.5 meters) of the building entrance during construction. | 施工期间，禁止在建筑中以及距建筑入口 25 英尺（7.5 米）的范围内使用烟草产品。 |
| 1596fab079d4-832e-4c98-b331-349d7dbed573 | Translated (79%) | EQ Credit: Indoor Air Quality Assessment | EQ 得分点：室内空气质量评估 (Indoor Air Quality Assessment) |
| 1597470b7cbe-9a27-4878-9459-fd5951003dfe | Translated (100%) | ID&C | ID&C |
| 15986abedbd2-e8a0-415e-84d4-df9775116d0a | Translated (100%) | 1–2 points | 1-2 分 |
| 159964d791ae-d85c-4502-befa-48203cc9d618 | Translated (100%) | This credit applies to | 该得分点适用于 |
| 1600ecd2d595-436e-4820-8067-f305b36bc1f0 | Translated (100%) | Commercial Interiors (1–2 points) | 商业室内（1-2 分） |
| 16010142f081-7849-41df-a0bb-a3bdeecaaaa0 | Translated (100%) | Retail (1–2 points) | 零售（1-2 分） |
| 1602a0f587f3-2195-4689-ba54-9dfb79d15fb1 | Translated (100%) | Hospitality (1–2 points) | 宾馆接待（1-2 分） |
| 16032a0c6432-83fd-44ce-a42b-c3535048fa27 | Translated (100%) | Intent | 目的 |
| 1604f7a2fa0c-98d2-4487-9a8b-e71a9fad3d61 | Translated (0%) | To establish better quality indoor air in the building after construction and during occupancy. | 在施工后以及入驻期间在建筑中形成质量更高的室内空气。 |
| 1605e2f07fa3-7844-4f8a-bcd7-daebdbfce64f | Translated (100%) | Requirements | 要求 |
| 16060fa40a2a-3df3-4a52-8f64-ea320db0984b | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 16078161c2db-8967-4fe9-9857-028926fdfcf7 | Translated (0%) | Select one of the following two options, to be implemented after construction ends and the building has been completely cleaned. | 选择以下两个将要在施工结束以及建筑完全清洁之后实施的选项之一。 |
| 16088161c2db-8967-4fe9-9857-028926fdfcf7 | Translated (0%) | All interior finishes, such as millwork, doors, paint, carpet, acoustic tiles, and movable furnishings (e.g., workstations, partitions), must be installed, and major VOC punch list items must be finished. | 所有室内面层，如木制品、门、涂料、地毯、声学瓷砖和可移动的陈设（如工作站、隔墙）都必须安装，且主要的 VOC 剩余工作清单项目都必须完成。 |
| 16098161c2db-8967-4fe9-9857-028926fdfcf7 | Translated (0%) | The options cannot be combined. | 选项无法合并。 |
| 1610be2cd946-b232-4c06-85ca-80531ebb413f | Translated (100%) | Option 1. | 选项 1. |
| 1611be2cd946-b232-4c06-85ca-80531ebb413f | Translated (0%) | Flush-Out (1 point) | 吹洗（1 分） |
| 1612e0c5ae93-56a6-4131-ad8b-9cec99744e87 | Translated (100%) | Path 1. | 途径 1. |
| 1613e0c5ae93-56a6-4131-ad8b-9cec99744e87 | Translated (0%) | Before Occupancy | 入驻前 |
| 16142f4f0071-f4d7-443f-9f3d-ae8d288d6525 | Translated (72%) | Install new filtration media and perform a building flush-out by supplying a total air volume of 14,000 cubic feet of outdoor air per square foot (4 267 140 liters of outdoor air per square meter) of gross floor area while maintaining an internal temperature of at least 60°F (15°C) and no higher than 80<1945/>F (27°C) and relative humidity no higher than 60%. | 安装新的过滤介质并按照每平方英尺总建筑面积总共提供 14000 立方英尺新风（每平方米提供 4267140 升新风）的标准执行建筑吹洗，同时保持至少 60° F (15°C) 且不超过 80<1945/>F (27°C) 的内部温度和不超过 60% 的相对湿度。 |
| 161552e8afe5-07d1-470d-8165-0431587edb2c | Translated (100%) | OR | 或 |
| 1616e6d9bffb-1878-40eb-9bcc-85b502e41a2f | Translated (CM) | Path 2. | 途径 2. |
| 1617e6d9bffb-1878-40eb-9bcc-85b502e41a2f | Translated (0%) | During Occupancy | 入驻期间 |
| 1618d3931b05-de8e-4f3e-be5d-abe3f437f514 | Translated (78%) | If occupancy is desired before the flush-out is completed, the space may be occupied only after delivery of a minimum of 3,500 cubic feet of outdoor air per square foot (1 066 260 liters of outdoor air per square meter) of gross floor area while maintaining an internal temperature of at least 60°F (15°C) and no higher than 80<1947/>F (27°C) and relative humidity no higher than 60%.. | 如果需要在吹洗完成之前入驻，则只能在按照每平方英尺总建筑面积至少提供 3,500 立方英尺新风（每平方米提供 1066260 升新风）的标准提供了新风之后入驻，同时保持至少 60° F (15°C) 且不超过 80<1947/>F (27°C) 的内部温度和不超过 60% 的相对湿度。 |
| 16198ff0dccf-cbf1-4aa4-be00-52a6b70d0b11 | Translated (77%) | Once the space is occupied, it must be ventilated at a minimum rate of 0.30 cubic foot per minute (cfm) per square foot of outdoor air (1.5 liters per second per square meter of outside air) or the design minimum outdoor air rate determined in EQ Prerequisite Minimum Indoor Air Quality Performance, whichever is greater. | 空间被使用之后，必须按照最少每平方英尺每分钟 (cfm) 0.30 立方英尺（每平方米每秒钟 1.5 升）新风的标准，或者由 EQ 先决条件：最低室内空气质量表现 (Minimum Indoor Air Quality Performance) 规定的最少设计新风量（以较大的值为准）进行通风。 |
| 16208ff0dccf-cbf1-4aa4-be00-52a6b70d0b11 | Translated (81%) | During each day of the flush-out period, ventilation must begin at least three hours before occupancy and continue during occupancy. | 在每天进行吹洗时，至少需要在入驻前 3 小时开始通风，并且在使用期间持续通风。 |
| 16218ff0dccf-cbf1-4aa4-be00-52a6b70d0b11 | Translated (85%) | These conditions must be maintained until a total of 14,000 cubic feet per square foot of outdoor air (4 270 liters of outdoor air per square meter) has been delivered to the space. | 必须遵守这些条件，直到按照每平方英尺 14000 立方英尺（每平方米 4270 升新风）的标准向空间中注入新风之后为止。 |
| 1622fe5e3b3f-b242-4618-9dc4-9ff6850d1996 | Translated (100%) | OR | 或 |
| 162369e6c8e1-8e2a-4d9e-b7f9-408863e478d7 | Translated (CM) | Option 2. | 选项 2. |
| 162469e6c8e1-8e2a-4d9e-b7f9-408863e478d7 | Translated (0%) | Air Testing (2 points) | 空气测试（2 分） |
| 16257e25f9e0-fbb3-4804-9300-9748eea8815b | Translated (0%) | After construction ends and before occupancy, but under ventilation conditions typical for occupancy, conduct baseline IAQ testing using protocols consistent with the methods listed in Table 1 for all occupied spaces. | 施工结束后入驻之前，在入驻常见的通风条件下，使用与表 1 所列方法一致的方案对所有使用空间进行基线 IAQ 测试。 |
| 16267e25f9e0-fbb3-4804-9300-9748eea8815b | Translated (0%) | Use current versions of ASTM standard methods, EPA compendium methods, or ISO methods, as indicated. | 使用指定的当前版本的 ASTM 标准方法、EPA 纲要方法或 ISO 方法。 |
| 16277e25f9e0-fbb3-4804-9300-9748eea8815b | Translated (75%) | Laboratories that conduct the tests for chemical analysis of formaldehyde and volatile organic compounds must be accredited under ISO/IEC 17025 for the test methods they use. | 进行甲醛和挥发性有机化合物化学分析测试的实验室使用的测试方法必须经过 ISO/IEC 17025 的认证。 |
| 16287e25f9e0-fbb3-4804-9300-9748eea8815b | Translated (0%) | Retail projects may conduct the testing within 14 days of occupancy. | 零售项目可以在入驻 14 天之内进行测试。 |
| 16294f029689-c4e5-45fe-8d20-d2ad36fae71b | Translated (0%) | Demonstrate that contaminants do not exceed the concentration levels listed in Table 1. | 证明污染物不超过表 1 中列出的浓度等级。 |
| 1630d02eb063-063b-4264-8a8a-6845652b1a0d | Translated (100%) | Table 1. | 表 1. |
| 1631d02eb063-063b-4264-8a8a-6845652b1a0d | Translated (0%) | Maximum concentration levels, by contaminant and testing method | 最大浓度等级，按照污染物和测试方法 |
| 1632356f2d0f-9210-4721-87f5-7dd722a15dba | Translated (100%) | Contaminant | 污染物 |
| 163384b48356-9cdb-4d95-a2d7-4acdfa002227 | Translated (100%) | Maximum concentration | 最大浓度 |
| 163423e730b5-aaf2-4128-96ae-6831795f9a8c | Translated (0%) | Maximum concentration (Healthcare only) | 最大浓度（仅医疗保健） |
| 1635d3024878-8e6d-4be5-96b7-baf0ab8dd83a | Translated (0%) | ASTM and U.S. EPA methods | ASTM 和 U.S. EPA 方法 |
| 16366a0213c9-6cfd-40b1-baf2-f89ba1ac184f | Translated (100%) | ISO method | ISO 方法 |
| 1637238ba48c-2719-496e-be10-02bede8be4ed | Translated (100%) | Formaldehyde | 甲醛 |
| 163834c236d1-3a27-4bd3-92e3-04f571ac0a9e | Translated (0%) | 27 ppb | 27 ppb |
| 16394190407f-1850-495b-a506-43873171ad1a | Translated (0%) | 16.3 ppb | 16.3 ppb |
| 16404674cc36-d543-4f39-8aef-8a5d2123fabb | Translated (0%) | ASTM D5197; EPA TO-11 or EPA Compendium Method IP-6 | ASTM D5197；EPA TO-11 或 EPA 纲要方法 IP-6 |
| 1641f0cdf131-07e2-4b42-b179-a10eeba55360 | Translated (100%) | ISO 16000-3 | ISO 16000-3 |
| 164218a05202-2e04-403d-a136-8964fad592c5 | Translated (0%) | Particulates (PM10 for all buildings; PM2.5 for buildings in EPA nonattainment areas, or local equivalent) | 微粒（所有建筑为 PM10；EPA 不达标区域的建筑为 PM2.5，或本地对应建筑） |
| 1643846e18c2-f1dd-4a72-b30f-deee78d6c61a | Translated (86%) | PM10: 50 micrograms per cubic meter | PM10：每立方米 50 微克 |
| 16443c86a32e-4fa5-4c9f-914c-a59787905a26 | Translated (89%) | PM2.5: 15 micrograms per cubic meter | PM2.5：每立方米 15 微克 |
| 164579af3a3e-a565-4826-8500-a71b687c7c5e | Translated (100%) | 20 micrograms per cubic meter | 每立方米 20 微克 |
| 164695ded8e9-d845-4e67-94b2-d3b7434eca1e | Translated (76%) | EPA Compendium Method IP-10 | EPA 纲要方法 IP-10 |
| 164763af5e4d-0a1d-47fa-b9b8-1a662524da95 | Translated (100%) | ISO 7708 | ISO 7708 |
| 1648ab136847-c171-4433-aba8-53696c26fc53 | Translated (0%) | Ozone (for buildings in EPA nonattainment areas) | 臭氧（对于 EPA 不达标区域的建筑） |
| 1649ae711581-c9ca-4001-825b-fa974fc97b41 | Translated (0%) | 0.075 ppm | 0.075 ppm |
| 165070d2126a-d357-4a20-8542-9afd7fe9c2bf | Translated (100%) | 0.075 ppm | 0.075 ppm |
| 1651ee5a735c-fc29-49e0-bbcb-a759b36b4c1d | Translated (0%) | ASTM D5149 - 02 | ASTM D5149 - 02 |
| 16525d2cb869-4351-40a9-8b01-179b20deeba1 | Translated (100%) | ISO 13964 | ISO 13964 |
| 1653fa83b30c-216f-43f6-96d2-4fc50bf85b14 | Translated (100%) | Total volatile organic compounds (TVOCs) | 挥发性有机化合物总量 (TVOC) |
| 1654b19b755b-4c48-43e6-910e-7db9bfffa411 | Translated (100%) | 500 micrograms per cubic meter | 每立方米 500 微克 |
| 16552b950dd2-2c8d-4e1d-b46c-c8ed4cb6ca77 | Translated (100%) | 200 micrograms per cubic meter | 每立方米 200 微克 |
| 16566d4ec511-21df-4ca8-9853-ef52a317fa17 | Translated (70%) | EPA TO-1, TO-15, TO-17, or EPA Compendium Method IP-1 | EPA TO-1、TO-15、TO-17 或 EPA 纲要方法 IP-1 |
| 16579b20e505-ce61-4aaf-831f-aa6c198c9fd1 | Translated (100%) | ISO 16000-6 | ISO 16000-6 |
| 165815e5c18b-14f1-49ba-8dbe-e580ec113fc7 | Translated (0%) | Target chemicals listed in CDPH Standard Method v1.1, Table 4-1, except formaldehyde | CDPH 标准方法 v1.1，表 4-1 中列出的目标化学品，甲醛除外 |
| 16598015a428-0342-44d3-8e3e-9ef9366037aa | Translated (0%) | CDPH Standard Method v1.1–2010, Allowable Concentrations, Table 4-1 | CDPH 标准方法 v1.1–2010，允许的浓度，表 4-1 |
| 166041aad379-4b8c-4806-9cc4-7bbfccc16094 | Translated (100%) | CDPH Standard Method v1.1–2010, Allowable Concentrations, Table 4-1 | CDPH 标准方法 v1.1–2010，允许的浓度，表 4-1 |
| 1661da2cc9cc-0328-471f-9ee5-12d80a89d597 | Translated (0%) | ASTM D5197; EPA TO-1, TO-15, TO-17 | ASTM D5197；EPA TO-1，TO-15，TO-17 |
| 1662ff6a03f8-55f9-43e0-a142-f3b401255201 | Translated (0%) | ISO 16000-3, 16000-6 | ISO 16000-3，16000-6 |
| 16636003e5a5-35c8-489a-93d5-7bdc54def308 | Translated (100%) | Carbon monoxide (CO) | 一氧化碳 (CO) |
| 16644df3906e-7421-4cc4-a3b3-090a44a7a89f | Translated (0%) | 9 ppm; no more than 2 ppm above outdoor levels | 9 ppm；不超过室外等级 2 ppm |
| 16652c220e04-b64b-429b-988a-967cc760aa22 | Translated (100%) | 9 ppm; no more than 2 ppm above outdoor levels | 9 ppm；不超过室外等级 2 ppm |
| 166618ce8cd7-7b30-4c18-a7bc-a26c14f0cf54 | Translated (83%) | EPA Compendium Method IP-3 | EPA 纲要方法 IP-3 |
| 16671a095509-c389-487b-90b6-ab9e83a90512 | Translated (100%) | ISO 4224 | ISO 4224 |
| 166865edc055-751e-4496-b928-4a758d48e35e | Translated (0%) | ppb = parts per billion; ppm = parts per million; μg/cm = micrograms per cubic meter | ppb = 十亿分之一；ppm = 百万分之一；μg/cm = 每立方米微克 |
| 1669ef6dcff3-2836-436e-8553-57b54e9c7855 | Translated (74%) | Conduct all measurements before occupancy but during normal occupied hours, with the building ventilation system started at the normal daily start time and operated at the minimum outdoor airflow rate for the occupied mode throughout the test. | 在入驻前的正常使用时间段内进行所有测试，且建筑通风系统要在正常的日常启动时间启动，并且以使用模式下的最低新风气流量运行。 |
| 167077ab0de7-5e97-4a20-8c29-0bd1601f8e88 | Translated (0%) | For each sampling point where the concentration exceeds the limit, take corrective action and retest for the noncompliant contaminants at the same sampling points. | 对于浓度超出限值的每个采样点，采取纠正措施并在相同的采样点上重新测试不符合标准的污染物。 |
| 167177ab0de7-5e97-4a20-8c29-0bd1601f8e88 | Translated (99%) | Repeat until all requirements are met. | 重复操作，直到满足所有要求。 |
| 1672ec79008b-2b82-4fa7-80b3-9dfc11f7331f | Translated (82%) | EQ Credit: Thermal Comfort | EQ 得分点：热舒适 (Thermal Comfort) |
| 1673d1a180f4-74fb-4b0d-a446-539064f69cb5 | Translated (100%) | ID&C | ID&C |
| 16741830fe2d-f9b6-4b34-b0c8-2cc1a7fd6718 | Translated (100%) | 1 point | 1 分 |
| 16758bdfc662-bc37-4777-b8e8-1e1a82a4fa73 | Translated (CM) | This credit applies to | 该得分点适用于 |
| 167683f3ef53-025f-4f68-9d48-9ce541ee2c02 | Translated (CM) | Commercial Interiors (1 point) | 商业室内（1 分） |
| 16773fa5730b-e203-40cd-bc35-4201d6e9ce45 | Translated (100%) | Retail (1 point) | 零售（1 分） |
| 1678320f23eb-0e2b-473c-a6bc-da3ba403d295 | Translated (100%) | Hospitality (1 point) | 宾馆接待（1 分） |
| 1679f2a27a3f-6842-47a9-9e31-cc141413d92f | Translated (100%) | Intent | 目的 |
| 168084d95143-eab7-4e7b-9c1e-ee947d0f8494 | Translated (0%) | To promote occupants’ productivity, comfort, and well-being by providing quality thermal comfort. | 提供优质的热舒适，改善驻户的生产效率、舒适性和健康。 |
| 1681e09070fa-807d-4039-9972-d65ee0f8cef9 | Translated (100%) | Requirements | 要求 |
| 16829c12c718-824b-4059-b7a3-b69e637621ac | Translated (0%) | Meet the requirements for both thermal comfort design and thermal comfort control. | 满足热舒适设计和热舒适控制的要求。 |
| 1683b48157b7-b7c1-42ed-a6f2-ebc02b75ea5a | Translated (79%) | Thermal Comfort Design | 热舒适设计 |
| 16844f537d67-afb9-4162-89c8-471d01274aa1 | Translated (100%) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 1685f1e9d8ca-c254-427f-aef2-0d08e16d356b | Translated (CM) | Option 1. | 选项 1. |
| 1686f1e9d8ca-c254-427f-aef2-0d08e16d356b | Translated (CM) | ASHRAE Standard 55-2010 | ASHRAE 标准 55-2010 |
| 16874afee4da-9b6e-4c34-92c2-7ae2a3a77d75 | Translated (0%) | Design heating, ventilating, and air-conditioning (HVAC) systems and the building envelope to meet the requirements of ASHRAE Standard 55–2010, Thermal Comfort Conditions for Human Occupancy with errata or a local equivalent. | 设计供暖、通风和空调 (HVAC) 系统和建筑外围护结构，以符合 ASHRAE 标准 55–2010，可供人类使用的热舒适条件，带勘误表或本地对应标准的要求。 |
| 1688f79ae912-1b20-4a54-9017-6b0eb6a03947 | Translated (72%) | For natatoriums, demonstrate compliance with ASHRAE HVAC Applications Handbook, 2011 edition, Chapter 5, Places of Assembly, Typical Natatorium Design Conditions, with errata. | 对于游泳馆来说，证明符合 ASHRAE 暖通空调应用手册 (ASHRAE HVAC Applications Handbook)，2011 版，第 5 章，集会场所，典型的游泳馆设计条件，带勘误表的要求。 |
| 168975ad36f2-0644-4be4-a660-8c83930ab0b2 | Translated (100%) | OR | 或 |
| 16904b9cd60b-ed1e-4639-b338-8d3dc2ca64c2 | Translated (CM) | Option 2. | 选项 2. |
| 16914b9cd60b-ed1e-4639-b338-8d3dc2ca64c2 | Translated (0%) | ISO and CEN Standards | ISO 和 CEN 标准 |
| 16928eca3143-f7bb-4ae9-b7f8-f6c9025e7318 | Translated (0%) | Design HVAC systems and the building envelope to meet the requirements of the applicable standard: | 设计暖通空调系统和建筑外围护结构以符合适用标准的要求： |
| 169361791316-5737-416a-a96e-afeb2ff4521f | Translated (84%) | ISO 7730:2005, Ergonomics of the Thermal Environment, analytical determination and interpretation of thermal comfort, using calculation of the PMV and PPD indices and local thermal comfort criteria; and | ISO 7730:2005，热环境的人体工程学，使用 PMV 和 PPD 指数的计算和本地热舒适性标准分析确定以及解释热舒适性；和 |
| 169402655d94-e72c-476d-83d0-4b9d0ebffbb0 | Translated (81%) | CEN Standard EN 15251:2007, Indoor Environmental Input Parameters for Design and Assessment of Energy Performance of Buildings, addressing indoor air quality, thermal environment, lighting, and acoustics, Section A2. | CEN 标准 EN 15251:2007，用于设计和评估与室内空气质量、热环境、照明和声学效果相关的建筑能源性能的室内环境输入参数，第 A2 部分。 |
| 1695c04844f9-fd23-4925-813b-7badcbf8dc15 | Translated (77%) | Thermal Comfort Control | 热舒适控制 |
| 1696a14a60cf-4fa4-4198-b142-3ef34c65d675 | Translated (100%) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 169715d3869d-a983-4d1c-b8c7-88491b745f2a | Translated (0%) | Provide individual thermal comfort controls for at least 50% of individual occupant spaces. | 至少为 50% 的个人使用空间提供独立的热舒适控制装置。 |
| 169815d3869d-a983-4d1c-b8c7-88491b745f2a | Translated (75%) | Provide group thermal comfort controls for all shared multioccupant spaces. | 为所有的共享多驻户空间提供群组热舒适控制装置。 |
| 1699c6c338e7-49ca-46f8-b245-9e54f4d179b8 | Translated (0%) | Thermal comfort controls allow occupants, whether in individual spaces or shared multioccupant spaces, to adjust at least one of the following in their local environment: air temperature, radiant temperature, air speed, and humidity. | 热舒适控制装置可以让独立空间或共享多驻户空间的驻户在他们的本地环境中调节其中至少一项：气温、辐射温度、气流速度和湿度。 |
| 1700bdcf98da-fbbc-429b-b50a-d0ef6b4768b7 | Translated (0%) | Hospitality only | 仅限宾馆接待 |
| 1701ef0531da-450b-4617-9a4f-c22c138bb892 | Translated (0%) | Guest rooms are assumed to provide adequate thermal comfort controls and are therefore not included in the credit calculations. | 假定客房能够提供充分的热舒适控制，因此不包含在得分点计算中。 |
| 170246064634-fa3f-48c1-ac8f-9e37ac5dc4db | Translated (100%) | Retail only | 仅限零售 |
| 17036bad3169-9d67-4550-93c1-ce9930efacf5 | Translated (0%) | Meet the above requirements for at least 50% of the individual occupant spaces in office and administrative areas. | 在办公室和管理区域至少 50% 的个人使用空间中满足上述要求。 |
| 17049f135ebc-2cdf-4c8c-9daa-12dff6327fc8 | Translated (0%) | EQ Credit: Interior Lighting | EQ 得分点：室内照明 (Interior Lighting) |
| 17051fe711de-60d1-4a53-9764-641d227c7207 | Translated (100%) | ID&C | ID&C |
| 17064a090bac-dc13-411e-9882-8902b48b2db6 | Translated (100%) | 1–2 points | 1-2 分 |
| 170789327173-1c90-4945-bd1b-7b41b6568331 | Translated (100%) | This credit applies to | 该得分点适用于 |
| 17088ba3cfd3-c15d-40f8-b730-9b594ede857c | Translated (100%) | Commercial Interiors (1–2 points) | 商业室内（1-2 分） |
| 1709bfa998ee-3ef2-464d-8626-123ebd053823 | Translated (100%) | Retail (2 points) | 零售（2 分） |
| 1710a598d66b-13b8-4cb1-abe0-773959c837fe | Translated (100%) | Hospitality (1–2 points) | 宾馆接待（1-2 分） |
| 17112d761c0e-a588-428d-b0c2-4f8b093df5ad | Translated (100%) | Intent | 目的 |
| 17127ef15b4b-6e79-4adf-95dc-e2dd1b57bb98 | Translated (83%) | To promote occupants’ productivity, comfort, and well-being by providing high-quality lighting. | 提供高质量照明，改善驻户的生产效率、舒适性和健康。 |
| 171381814a95-914a-4d77-a22e-412bed8a2533 | Translated (100%) | Requirements | 要求 |
| 17144008af91-99e4-4253-95e6-0a0f5949c86e | Translated (CM) | CI, Hospitality | 商业室内，宾馆接待 |
| 17154d95658a-ea97-434b-aaee-60d9d7b660ba | Translated (0%) | Select one or both of the following two options. | 选择以下两个选项之一或全部。 |
| 17166ac5b226-2340-48d4-9090-017b8e54498f | Translated (100%) | Option 1. | 选项 1. |
| 17176ac5b226-2340-48d4-9090-017b8e54498f | Translated (76%) | Lighting Control (1 point) | 照明控制（1 分） |
| 1718616285fd-fd3f-40fe-8e22-8649da927291 | Translated (0%) | For at least 90% of individual occupant spaces,<2109> </2109>provide individual lighting controls that enable occupants to adjust the lighting to suit their individual tasks and preferences, with at least three lighting levels or scenes (on, off, midlevel). | 为至少 90% 的个人使用空间<2109></2109>提供独立照明控制，可以让驻户调节照明以适合他们各自的任务和偏好，并且具有至少三种照明等级或场景（开、关、中等）。 |
| 1719616285fd-fd3f-40fe-8e22-8649da927291 | Translated (0%) | Midlevel is 30% to 70% of the maximum illumination level (not including daylight contributions). | 中等是最大照明等级的 30% 到 70%（不包括自然采光的影响）。 |
| 1720b3df47d3-f844-484a-9567-de06c5efa212 | Translated (0%) | For all shared multioccupant spaces, meet all of the following requirements. | 所有共享多驻户空间都要满足以下全部要求。 |
| 1721b1d1e3f8-6c1e-4989-a79b-e0b0e54612fb | Translated (0%) | Have in place multizone control systems that enable occupants to adjust the lighting to meet group needs and preferences, with at least three lighting levels or scenes (on, off, midlevel). | 提供多区控制系统，可以让驻户调节照明以满足群组需求和偏好，并且具有至少三种照明等级或场景（开、关、中等）。 |
| 17227d4676d7-e904-4135-b236-c7595b5a8aea | Translated (0%) | Lighting for any presentation or projection wall must be separately controlled. | 演示或投影墙的照明必须单独控制。 |
| 1723148008f7-734d-4e26-b160-6c397247e82d | Translated (0%) | Switches or manual controls must be located in the same space as the controlled luminaires. | 开关或手动控制必须与受控的光源位于同一个空间中。 |
| 1724148008f7-734d-4e26-b160-6c397247e82d | Translated (0%) | A person operating the controls must have a direct line of sight to the controlled luminaires. | 操作控制装置的人必须能够直接看到受控的光源。 |
| 1725ed14cfde-5a88-44e2-b715-9c4c295afd40 | Translated (100%) | Hospitality only | 仅限宾馆接待 |
| 17263c86b7f3-40fa-4258-aa59-7dc7683ad5b9 | Translated (93%) | Guest rooms are assumed to provide adequate lighting controls and are therefore not included in the credit calculations. | 假定客房能够提供充分的照明控制，因此不包含在得分点计算中。 |
| 17274d6e7b3c-96c8-420f-9de9-7a58292bfcf6 | Translated (100%) | AND/OR | 和/或 |
| 1728d5af3bd9-dfa0-4395-9e58-e2da7e84e582 | Translated (CM) | Option 2. | 选项 2. |
| 1729d5af3bd9-dfa0-4395-9e58-e2da7e84e582 | Translated (84%) | Lighting Quality (1 point) | 照明质量（1 分） |
| 1730264afe5f-fb62-4885-b9ca-2f3ce866f3ab | Translated (77%) | Choose four of the following strategies. | 从以下策略中选择 4 项。 |
| 173105e41085-49ba-47a2-a352-8d5e2639e0bc | Translated (0%) | For all regularly occupied spaces, use light fixtures with a luminance of less than 2,500 cd/m<2116>2</2116> between 45 and 90 degrees from nadir. | 在所有常用空间中，使用从最低点开始 45 到 90 度的范围内照度不足 2,500 cd/m<2116>2</2116> 的照明灯具。 |
| 17322d08b9b1-2d43-455b-98d0-7d734491ad8a | Translated (0%) | Exceptions include wallwash fixtures properly aimed at walls, as specified by manufacturer’s data, indirect uplighting fixtures, provided there is no view down into these uplights from a regularly occupied space above, and any other specific applications (i.e. adjustable fixtures). | 例外情况包括正确对准墙壁的壁灯（制造商数据指定）、间接上照灯具（前提是没有从上方的常用空间向下看这些灯具的视野），以及其他任何具体应用（即，可调节的灯具）。 |
| 1733a5d9be29-1f77-4900-91eb-5da88350fb08 | Translated (0%) | For the entire project, use light sources with a CRI of 80 or higher. | 在整个项目中使用 CRI 为 80 或更大值的光源。 |
| 1734a5d9be29-1f77-4900-91eb-5da88350fb08 | Translated (0%) | Exceptions include lamps or fixtures specifically designed to provide colored lighting for effect, site lighting, or other special use. | 例外情况包括专门设计以为实现效果、场址照明或其他特殊用途提供彩色灯光的灯具。 |
| 1735922518c8-41a4-4f29-8bdf-3c0d92c24f62 | Translated (0%) | For at least 75% of the total connected lighting load, use light sources that have a rated life (or L70 for LED sources) of at least 24,000 hours (at 3-hour per start, if applicable). | 在至少 75% 的全部相关照明负荷中，使用额定寿命（LED 光源为 L70）至少为 24000 小时（如适用，每次启动后使用 3 小时）的光源。 |
| 1736a3de6627-bf2d-4126-bfbd-af25e9a31a4c | Translated (0%) | Use direct-only overhead lighting for 25% or less of the total connected lighting load for all regularly occupied spaces. | 在所有常用空间中，对 25% 或更少的全部相关照明负荷使用直射光吸顶灯。 |
| 17376d5aa83d-1aec-48f8-9185-864244cebf75 | Translated (0%) | For at least 90% of the regularly occupied floor area, meet or exceed the following thresholds for area-weighted average surface reflectance: 85% for ceilings, 60% for walls, and 25% for floors. | 在至少 90% 的常用建筑面积中，满足或超出面积权重平均表面反射率的以下阈值：天花板为 85%、墙壁为 60%，地板为 25%。 |
| 173802a970cd-1af2-47e7-9f19-eb774f3f83b2 | Translated (0%) | If furniture is included in the scope of work, select furniture finishes to meet or exceed the following thresholds for area-weighted average surface reflectance: 45% for work surfaces, and 50% for movable partitions. | 如果家具包括在工作范围内，选择家具表层以满足或超出面积权重平均表面反射率的以下阈值：工作表面为 45%，可移动的隔墙为 50%。 |
| 1739588533a8-7f0b-48db-b5eb-ad1f7b9586fd | Translated (0%) | For at least 75% of the regularly occupied floor area, meet a ratio of average wall surface illuminance (excluding fenestration) to average work plane (or surface, if defined) illuminance that does not exceed 1:10. | 在至少 75% 的常用建筑面积中，满足墙壁表面平均照度（不包括开窗）与工作平面（或表面，如定义）平均照度的比值不超过 1:10 这一标准。 |
| 1740588533a8-7f0b-48db-b5eb-ad1f7b9586fd | Translated (0%) | Must also meet strategy E, strategy F, or demonstrate area-weighted surface reflectance of at least 60% for walls. | 还必须满足策略 E、策略 F，或证明墙壁的面积权重表面反射率至少为 60%。 |
| 1741169db42f-be06-487e-ba60-fd6e9c09fa8b | Translated (84%) | For at least 75% of the regularly occupied floor area, meet a ratio of average ceiling illuminance (excluding fenestration) to work surface illuminance that does not exceed 1:10. | 在至少 75% 的常用建筑面积中，满足天花板平均照度（不包括开窗）与工作表面照度的比值不超过 1:10 这一标准。 |
| 1742169db42f-be06-487e-ba60-fd6e9c09fa8b | Translated (89%) | Must also meet option E, option F, or demonstrate area-weighted surface reflectance of at least 85% for ceilings. | 还必须满足选项 E、选项 F，或证明天花板的面积权重表面反射率至少为 85%。 |
| 17439e0f9aa3-85c7-4fc5-8d61-7b625fd226d4 | Translated (100%) | Retail CI | 商业室内零售 |
| 17448f35bf10-e404-47d7-8afa-01f26f537fea | Translated (0%) | For at least 90% of the individual occupant spaces in office and administrative areas, provide individual lighting controls. | 在办公室和管理区域至少 90% 的个人使用空间中，提供单独的照明控制装置。 |
| 17450fec1cba-7777-42f5-9151-b3b8d328c478 | Translated (0%) | In sales areas, provide controls that can reduce the ambient light levels to a midlevel (30% to 70% of the maximum illumination level not including daylight contributions). | 在销售区域中，提供可以将环境亮度降低至中等程度（最大照度等级的 30% 到 70%，不包括自然采光影响）的控制装置。 |
| 174603926b00-464a-48ea-a434-7efd9c32f0d8 | Translated (74%) | EQ Credit: Daylight | EQ 得分点：自然采光 (Daylight) |
| 174781eb905d-f39d-40e5-a1fc-3dab2f01fe07 | Translated (100%) | ID&C | ID&C |
| 17487012bd9d-d989-49d7-a2c6-0324de2db76a | Translated (100%) | 1–3 points | 1-3 分 |
| 17490e320aae-0d4a-4e69-a0c1-bc148c8f63ec | Translated (100%) | This credit applies to | 该得分点适用于 |
| 1750d6183f37-e6f1-4cca-b66b-f46269638322 | Translated (100%) | Commercial Interiors (1–3 points) | 商业室内（1-3 分） |
| 1751895371e7-8c52-4529-bbf9-069001c3c179 | Translated (100%) | Retail (1–3 points) | 零售（1-3 分） |
| 1752965311c5-c1cc-4b23-8567-7bfa5e4d4e6a | Translated (100%) | Hospitality (1–3 points) | 宾馆接待（1-3 分） |
| 17538ec3f7b9-84da-47ec-9c72-e26428c9b77f | Translated (100%) | Intent | 目的 |
| 1754fe6e5279-1ffb-4cb0-988d-9112c00701f0 | Translated (0%) | To connect building occupants with the outdoors, reinforce circadian rhythms, and reduce the use of electrical lighting by introducing daylight into the space. | 将建筑驻户与室外相关联，加强昼夜节律，并通过将自然光引入空间来减少电力照明的使用。 |
| 1755a7b5f11c-bade-40c7-9cc4-1b1154be09ce | Translated (100%) | Requirements | 要求 |
| 1756da2fc086-13fd-4b5f-b464-75f4e8ab156a | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 175759845e51-4a82-4ef6-a5eb-2c8793339641 | Translated (0%) | Provide manual or automatic (with manual override) glare-control devices for all regularly occupied spaces. | 在所有常用空间中提供手动或自动（带手动超控）眩光控制设备。 |
| 17585865a110-816c-41c7-bd7f-fd39e7cdfdf3 | Translated (74%) | Select one of the following three options. | 选择以下三个选项之一。 |
| 17592e7e5c0f-70cc-48dd-ad0e-be041f5f0837 | Translated (100%) | Option 1. | 选项 1. |
| 17602e7e5c0f-70cc-48dd-ad0e-be041f5f0837 | Translated (0%) | Simulation: Spatial Daylight Autonomy and Annual Sunlight Exposure (2–3 points) | 模拟：空间全自然光照明和年度日光照射（2-3 分） |
| 1761b63ac6ee-6284-4e13-aa88-d618475989b0 | Translated (0%) | Demonstrate through annual computer simulations that spatial daylight autonomy<2131>300/50%</2131> (sDA<2132>300/50%</2132>) of at least 55%, 75%, or 90% is achieved. | 通过每年的计算机模拟来证明实现至少 55%、75% 或 90% 的空间全自然光照明 <2131>300/50%</2131> (sDA<2132>300/50%</2132>) 。 |
| 1762b63ac6ee-6284-4e13-aa88-d618475989b0 | Translated (0%) | Use regularly occupied floor area. | 使用常用建筑面积。 |
| 1763b63ac6ee-6284-4e13-aa88-d618475989b0 | Translated (0%) | Healthcare projects should use the perimeter area determined under EQ Credit Quality Views. | 医疗保健项目应使用 EQ 得分点：优良视野 (Quality Views) 确定的周边区域。 |
| 1764b63ac6ee-6284-4e13-aa88-d618475989b0 | Translated (100%) | Points are awarded according to Table 1. | 根据表 1 获得分数。 |
| 1765f3138100-71f4-4529-a9a0-76f28551a156 | Translated (CM) | Table 1. | 表 1. |
| 1766f3138100-71f4-4529-a9a0-76f28551a156 | Translated (0%) | Points for daylit floor area: Spatial daylight autonomy | 自然采光建筑面积：空间全自然光照明的分数 |
| 1767e9737831-7c30-4567-9a4f-050744ac46bb | Translated (73%) | sDA (for regularly occupied floor area) | sDA（用于常用建筑面积） |
| 1768615c12ff-e2df-45fa-b973-5962a4534d41 | Translated (100%) | Points | 分数 |
| 17699998f31d-6150-42cc-9a09-768c1261b951 | Translated (CM) | 55% | 55% |
| 177061570b5a-a02b-4df5-9549-22f5980eefee | Translated (CM) | 2 | 2 |
| 17712d08a38f-2ec0-4345-bad3-19173ab8397f | Translated (CM) | 75% | 75% |
| 177237f62477-9882-4371-ae82-8d47996c656a | Translated (CM) | 3 | 3 |
| 1773a2fa85d8-730f-4cae-ac5c-4b597506b5f1 | Translated (100%) | AND | 以及 |
| 177402785be6-7ac6-4fe7-bd87-fa6c9858fd0f | Translated (0%) | Demonstrate through annual computer simulations that annual sunlight exposure<2158>1000,250</2158> (ASE<2159>1000,250</2159>) of no more than 10% is achieved. | 通过每年的计算机模拟证明实现了不超过 10% 的年度日光照射<2158>1000,250</2158> (ASE<2159>1000,250</2159>)。 |
| 177502785be6-7ac6-4fe7-bd87-fa6c9858fd0f | Translated (0%) | Use the regularly occupied floor area that is daylit per the sDA<2160>300/50%</2160> simulations. | 按照 sDA<2160>300/50%</2160> 模拟使用有自然采光的常用建筑面积。 |
| 17761fbf05fa-9315-498e-aa76-c9e8655f2602 | Translated (0%) | The sDA and ASE calculation grids should be no more than 2 feet (600 millimeters) square and laid out across the regularly occupied area at a work plane height of 30 inches (76 millimeters) above finished floor (unless otherwise defined). | sDA 和 ASE 计算网格不应超过 2 英尺（600 毫米）见方，并且在高于完工地板 30 英寸（76 毫米）的工作面高度上横跨常用区域（除非另有规定）。 |
| 17771fbf05fa-9315-498e-aa76-c9e8655f2602 | Translated (0%) | Use an hourly time-step analysis based on typical meteorological year data, or an equivalent, for the nearest available weather station. | 使用基于最近气象站的典型气象年数据，或者类似数据的每小时时间步长分析。 |
| 1778fe6299b7-b4da-42af-89e3-43669e4fa92e | Translated (0%) | Include any permanent interior obstructions and moveable furniture and partitions. | 包括任何永久室内阻碍物和可移动家具和隔墙。 |
| 1779cb1edaa4-183b-48e7-a460-e72547cbec01 | Translated (100%) | OR | 或 |
| 178001562a1e-8e54-4471-a641-fae236509ef5 | Translated (CM) | Option 2. | 选项 2. |
| 178101562a1e-8e54-4471-a641-fae236509ef5 | Translated (0%) | Simulation: Illuminance Calculations (1–2 points) | 模拟：照度计算（1-2 分） |
| 1782b509fe48-c5e3-4171-8af0-5faf249489d4 | Translated (0%) | Demonstrate through computer modeling that illuminance levels will be between 300 lux and 3,000 lux for 9 a.m. and 3 p.m., both on a clear-sky day at the equinox, for the floor area indicated in Table 2. | 通过计算机建模证明表 2 中指定的建筑面积在春分（且为晴天）那天的照度等级在上午 9 点和下午 3 点之间将为 300 lux 到 3,000 lux。 |
| 1783b509fe48-c5e3-4171-8af0-5faf249489d4 | Translated (100%) | Use regularly occupied floor area. | 使用常用建筑面积。 |
| 1784b509fe48-c5e3-4171-8af0-5faf249489d4 | Translated (CM) | Healthcare projects should use the perimeter area determined under EQ Credit Quality Views. | 医疗保健项目应使用 EQ 得分点：优良视野 (Quality Views) 确定的周边区域。 |
| 178501752d76-78cc-4594-acf4-44143787d084 | Translated (100%) | Table 2. | 表 2. |
| 178601752d76-78cc-4594-acf4-44143787d084 | Translated (73%) | Points for daylit floor area: Illuminance calculation | 自然采光建筑面积：照度计算的分数 |
| 17872f2d80dd-6c5e-4dad-8f94-fb375bcd7147 | Translated (76%) | Percentage of regularly occupied floor area | 常用建筑面积的百分比 |
| 17883451d865-c95f-41a1-8d8f-a3eda8c70a5b | Translated (100%) | Points | 分数 |
| 1789bdc8bb81-99c4-4529-bb07-def3acddfcd4 | Translated (CM) | 75% | 75% |
| 1790cf24da86-c912-405f-8061-5c64897ecfbe | Translated (CM) | 1 | 1 |
| 1791e4dae9bd-d66b-4924-a9d6-fd5ef2f1e251 | Translated (CM) | 90% | 90% |
| 1792b45144aa-ff82-4578-afdf-0260cf23f187 | Translated (CM) | 2 | 2 |
| 1793d2803850-9ae5-4383-b014-c5cdc990b2cd | Translated (0%) | Calculate illuminance intensity for sun (direct component) and sky (diffuse component) for clear-sky conditions as follows: | 在晴天按照以下方式计算太阳（直射部分）和天空（漫反射部分）的照度强度： |
| 1794a3bad59b-86ff-4f82-bc7c-b870aba6b170 | Translated (75%) | Use typical meteorological year data, or an equivalent, for the nearest available weather station. | 使用最近气象站的典型气象年数据，或者类似数据。 |
| 17952ef55e71-4937-4ed2-b3b1-8deb50500f0d | Translated (0%) | Select one day within 15 days of September 21 and one day within 15 days of March 21 that represent the clearest sky condition. | 分别在 9 月 21 日和 3 月 21 日前后 15 天中选择一天，作为最晴朗的天空情况。 |
| 17963820c936-9a3e-4616-861f-ecf231c9ad26 | Translated (0%) | Use the average of the hourly value for the two selected days. | 使用所选 2 天的每小时值的平均值。 |
| 179717016bee-d6c8-4814-a9ca-a77325b69e12 | Translated (0%) | Exclude blinds or shades from the model. | 模型中不包括百叶窗或窗帘。 |
| 1798670630f2-cb89-41ed-a35a-972bd3289682 | Translated (100%) | Include any permanent interior obstructions and moveable furniture and partitions. | 包括任何永久室内阻碍物和可移动家具和隔墙。 |
| 17997dc304e4-054d-4aae-b226-16a4b6001479 | Translated (100%) | OR | 或 |
| 180020091d42-6b62-4b3e-b7a3-9011142fb76b | Translated (CM) | Option 3. | 选项 3. |
| 180120091d42-6b62-4b3e-b7a3-9011142fb76b | Translated (80%) | Measurement (2-3 points) | 测量（2-3 分） |
| 1802e62f4bd6-30e4-4da4-908f-00d93e2f762e | Translated (0%) | Achieve illuminance levels between 300 lux and 3,000 lux for the floor area indicated in Table 3. | 使表 3 中指定的建筑面积实现 300 lux 到 3,000 lux 的照度等级。 |
| 1803e25b864b-7260-4aa9-b47c-26c62f8e232a | Translated (100%) | Table 3. | 表 3. |
| 1804e25b864b-7260-4aa9-b47c-26c62f8e232a | Translated (70%) | Points for daylit floor area: Measurement | 自然采光建筑面积：测量的分数 |
| 18056dc6fcd7-385b-41fa-b23b-c436ebdfe862 | Translated (100%) | Percentage of regularly occupied floor area | 常用建筑面积的百分比 |
| 18066d290c92-131f-4e44-aaec-f4a8d41ce6f7 | Translated (100%) | Points | 分数 |
| 180750a416cb-44fc-45c6-879e-f35a4b1015d3 | Translated (CM) | 75 | 75 |
| 18085d9ecde2-c542-40fa-9fd0-a9209704281c | Translated (CM) | 2 | 2 |
| 1809b305a4e1-5ce6-42bb-8db0-031e73f05eff | Translated (CM) | 90 | 90 |
| 1810025e91ec-e0db-4733-a650-590aebf67db1 | Translated (CM) | 3 | 3 |
| 18113c194b60-3d7d-4923-aff2-11278a6d4674 | Translated (0%) | With furniture, fixtures, and equipment in place, measure illuminance levels as follows: | 在安放好家具、器具和设备之后，按照以下方式测量照度等级： |
| 1812837dcc8b-4a22-4027-a609-df7f075b019c | Translated (0%) | Measure at appropriate work plane height during any hour between 9 a.m. and 3 p.m. | 在上午 9 点和下午 3 点之间的任意一个小时期间，在合适的工作面高度上测量。 |
| 18139df67329-ed3f-4683-bd55-7ad219a4988e | Translated (0%) | Take one measurement in any regularly occupied month, and take a second as indicated in Table 4. | 在任意常用月份进行一次测量。并按照表 4 的规定进行第二次测量。 |
| 1814b87427f4-ec6f-499b-8883-b4b16c591f9d | Translated (0%) | For spaces larger than 150 square feet (14 square meters), take measurements on a maximum 10 foot (3 meter) square grid. | 对于大于 150 平方英尺（14 平方米）的空间，在最多 10 英尺（3 米）见方的方格中进行测量。 |
| 1815fa93e70e-23a3-43c8-906f-280d2cd16def | Translated (79%) | For spaces 150 square feet (14 square meters) or smaller, take measurements on a maximum 3 foot (900 millimeters) square grid. | 对于小于等于 150 平方英尺（14 平方米）的空间，在最多 3 英尺（900 毫米）见方的方格中进行测量。 |
| 1816188a0c95-3078-48b7-98c6-02bd7627324c | Translated (100%) | Table 4. | 表 4. |
| 1817188a0c95-3078-48b7-98c6-02bd7627324c | Translated (0%) | Timing of measurements for illuminance | 照度测量时间 |
| 18188701f19a-f046-4dbc-bc8f-1e505e9a651a | Translated (0%) | If first measurement is taken in … | 如果第一次测量的时间为... |
| 18192b378096-028e-4659-9179-185cb1e5fb98 | Translated (0%) | take second measurement in … | 则进行第二次测量的时间为... |
| 1820f1705168-1471-46cb-ae8c-038308bd998f | Translated (0%) | January | 一月 |
| 18210003e07c-e3da-45a7-a758-7df22982cfc3 | Translated (0%) | May-September | 五月-九月 |
| 182299b35b55-abfc-4380-9693-a4d4b3e1ae8c | Translated (0%) | February | 二月 |
| 1823b690bf22-bd32-44dd-83f5-28adc4020bd5 | Translated (0%) | June-October | 六月-十月 |
| 1824542bfad1-b867-4cf1-a350-fd2de392873c | Translated (0%) | March | 三月 |
| 18253ecc9720-4776-4400-b34e-0141f22d8a9a | Translated (0%) | June-July, November-December | 六月-七月，十一月-十二月 |
| 1826244ea54d-5784-4ed4-8a75-b1214441c46a | Translated (0%) | April | 四月 |
| 1827ac1c4d9a-43d8-4d4c-907b-ef1e7ed52c6b | Translated (0%) | August-December | 八月-十二月 |
| 182816710d37-bdf1-4f26-935c-3e8fd78cad02 | Translated (0%) | May | 五月 |
| 182942a5214e-966e-4e54-bf61-88320d0aa31f | Translated (0%) | September-January | 九月-次年一月 |
| 18300613bc26-df4c-448b-87ff-899cf553fa19 | Translated (0%) | June | 六月 |
| 18316ef94d01-9db6-413a-a1c0-d00c93637d81 | Translated (0%) | October-February | 十月-次年二月 |
| 1832f9d478e3-58ca-426f-87f0-2408d70121b1 | Translated (0%) | July | 七月 |
| 183338ef666f-741a-4f3b-be8d-8b15958adf09 | Translated (0%) | November-March | 十一月-次年三月 |
| 1834fc5ce92c-d068-4ffd-8c88-b75ab79e9899 | Translated (0%) | August | 八月 |
| 1835b8cb9913-b189-4134-9823-d86c9b005ccc | Translated (0%) | December-April | 十二月-次年四月 |
| 1836d38e8133-8d18-409e-ad30-6d1b12303cf7 | Translated (0%) | September | 九月 |
| 183767f77847-44fa-4957-981e-b0192eeb7477 | Translated (0%) | December-January, May-June | 十二月-次年一月，五月-六月 |
| 18389e0d6c3c-6a99-44fb-8d7d-48a6fd44f322 | Translated (0%) | October | 十月 |
| 1839b8b1d26b-563d-4b84-8a99-681a18d1f2e2 | Translated (0%) | February-June | 二月-六月 |
| 184049a3a34e-8004-4c60-8f64-169551aa771a | Translated (0%) | November | 十一月 |
| 184121f07987-56c8-40cd-94e3-4ad7fdcec6ff | Translated (0%) | March-July | 三月-七月 |
| 1842c8f18bd0-9e3b-49f4-99cd-2f9bb74a6a4d | Translated (0%) | December | 十二月 |
| 184314d817cc-5b96-448f-8862-7ee8b640b785 | Translated (0%) | April-August | 四月-八月 |
| 1844f567e937-d0f6-4558-a2d3-b7bdc06cd245 | Translated (82%) | EQ Credit: Quality Views | EQ 得分点：优良视野 (Quality Views) |
| 1845b3b4c452-a605-42b0-bc66-f7b2d2441da8 | Translated (100%) | ID&C | ID&C |
| 1846b74df283-56e8-4961-8c58-8324d89d5f22 | Translated (100%) | 1 point | 1 分 |
| 184771f7148b-d5f3-4a19-a451-3102fb423e60 | Translated (CM) | This credit applies to | 该得分点适用于 |
| 1848c8577585-2b44-4586-af23-496363223555 | Translated (CM) | Commercial Interiors (1 point) | 商业室内（1 分） |
| 1849e23baae5-c981-413d-a22a-3a27439a2b73 | Translated (CM) | Retail (1 point) | 零售（1 分） |
| 1850c1ae9d30-d141-46fd-a4c6-1b4a80ef2326 | Translated (CM) | Hospitality (1 point) | 宾馆接待（1 分） |
| 185110953e4d-8de6-45cb-ab30-6fe50faa0403 | Translated (100%) | Intent | 目的 |
| 185263de54aa-3cb0-4231-9663-5dfec942ed7f | Translated (0%) | To give building occupants a connection to the natural outdoor environment by providing quality views. | 通过提供优良视野，让建筑驻户与自然室外环境相关联。 |
| 185352ac7026-c965-49e4-b594-4ee69b4c92fd | Translated (100%) | Requirements | 要求 |
| 1854543e7bc2-386d-4ccb-ab94-f3177ef16acd | Translated (99%) | CI, Retail, Hospitality, | 商业室内，零售，宾馆接待， |
| 18552af4a0da-df77-4483-b375-823d4eef3dd3 | Translated (0%) | Achieve a direct line of sight to the outdoors via vision glazing for 75% of all regularly occupied floor area. | 为全部常用建筑面积的 75% 提供观景窗，从而能够直接看到室外。 |
| 185603f40334-2285-40e2-85a6-01725b9093ab | Translated (0%) | View glazing in the contributing area must provide a clear image of the exterior, not obstructed by frits, fibers, patterned glazing, or added tints that distort color balance. | 相关区域的观景窗必须提供清楚的室外视野，不会受到玻璃、纤维、有图案的玻璃，或添加的色彩（扭曲色彩平衡）的阻挡。 |
| 18578f603cf5-7b3c-4dcc-8b70-6570b1ca0fef | Translated (0%) | Additionally, 75% of all regularly occupied floor area must have at least two of the following four kinds of views: | 此外，全部常用建筑面积的 75% 都必须拥有以下 4 种视野中的至少 2 种： |
| 1858bc10b966-60df-4c43-bc2e-b9a55ccaa50d | Translated (0%) | multiple lines of sight to vision glazing in different directions at least 90 degrees apart; | 多条视线从不同方向（至少相隔 90 度）看到观景窗； |
| 18590a054bbb-a457-4b39-88ed-ded5460c8a59 | Translated (0%) | views that include at least two of the following: (1) flora, fauna, or sky; (2) movement; and (3) objects at least 25 feet (7.5 meters) from the exterior of the glazing; | 视野包括其中至少两种：(1) 植物、动物或天空；(2) 移动景观；以及 (3) 距离玻璃外部至少 25 英尺（7.5 米）的物体； |
| 1860ae34004b-5b49-48d0-af4e-d15f7b983fcb | Translated (0%) | unobstructed views located within the distance of three times the head height of the vision glazing; and | 不受阻碍的视野位于观景窗顶部高度三倍的距离之内；以及 |
| 18611c855dc0-e193-42c7-bb28-1f0433b1f1f8 | Translated (0%) | views with a view factor of 3 or greater, as defined in “Windows and Offices; A Study of Office Worker Performance and the Indoor Environment.” | 按照“窗户与办公室：办公室工作人员的绩效和室内环境的研究”(Windows and Offices: A Study of Office Worker Performance and the Indoor Environment) 的定义，视野系数为 3 或更大的视野 |
| 1862be48a148-2a92-4327-9271-cf13677c373a | Translated (0%) | Include in the calculations any permanent interior obstructions. | 将永久室内阻碍物包括在计算中。 |
| 1863be48a148-2a92-4327-9271-cf13677c373a | Translated (0%) | Movable furniture and partitions may be excluded. | 可移动家具和隔断可排除在外。 |
| 1864d74f1a33-207b-46c8-92c9-821ec3e41eb3 | Translated (0%) | Views into interior atria may be used to meet up to 30% of the required area. | 可看到室内中庭的视野可用于满足最多 30% 的所需面积。 |
| 18651fed097c-73d3-428b-8d7c-14d886af3823 | Translated (83%) | EQ Credit: Acoustic Performance | EQ 得分点：声环境表现 (Acoustic Performance) |
| 1866477f370b-c10f-46d7-8450-77c5d3c566c1 | Translated (100%) | ID&C | ID&C |
| 18672a44236b-4469-4e78-9277-053e0b9987a7 | Translated (100%) | 2 points | 2 分 |
| 186887dcd890-c869-4e8c-8f46-e057a5e8de73 | Translated (100%) | This credit applies to | 该得分点适用于 |
| 1869bac424e9-b6c5-43ab-b269-a49bbf169b56 | Translated (100%) | Commercial Interiors (2 points) | 商业室内（2 分) |
| 1870f0e7821a-4690-4903-a281-8c25fba03c73 | Translated (100%) | Hospitality (2 points) | 宾馆接待（2 分） |
| 187148bd7eac-fa38-4bb9-bf29-3a97db0f059f | Translated (100%) | Intent | 目的 |
| 18726e5107d1-54a5-4b12-a891-0c1524e2c015 | Translated (0%) | To provide workspaces and classrooms that promote occupants’ well-being, productivity, and communications through effective acoustic design. | 提供通过有效的声学效果设计来改善驻户健康、生产效率和沟通的工作空间和教室。 |
| 18738aa890fc-27bd-48c8-a099-8d39c032f5d5 | Translated (100%) | Requirements | 要求 |
| 1874105c7cbc-628a-4b49-9585-9b7d8a0d5b85 | Translated (CM) | CI, Hospitality | 商业室内，宾馆接待 |
| 1875b0ff0168-c7bb-465f-a852-1c3caadd978d | Translated (0%) | For all occupied spaces, meet the following requirements, as applicable, for HVAC background noise, sound isolation, reverberation time, and sound reinforcement and masking. | 对于所有使用空间来说，满足以下适用于暖通空调背景噪音、隔音、混响时间以及扩音和掩蔽的要求（如适用）。 |
| 18761d598513-94e7-4fc8-8fed-9adc12e185c2 | Translated (0%) | HVAC Background Noise | 暖通空调背景噪音 |
| 1877f79811dc-d886-4f0a-bb68-95447647fc36 | Translated (0%) | Achieve maximum background noise levels from heating, ventilating, and air conditioning (HVAC) systems per 2011 ASHRAE Handbook, HVAC Applications, Chapter 48, Table 1; AHRI Standard 885-2008, Table 15; or a local equivalent. | 按照 2011 ASHRAE 手册，暖通空调应用，第 48 章，表 1；AHRI 标准 885-2008，表 15；或本地对应标准，让供暖、通风和空调 (HVAC) 系统实现最大背景噪音等级。 |
| 1878f79811dc-d886-4f0a-bb68-95447647fc36 | Translated (0%) | Calculate or measure sound levels. | 计算或测量声音强度。 |
| 18796c4e3d7b-8337-443c-96c6-bf0181a30835 | Translated (0%) | For measurements, use a sound level meter that conforms to ANSI S1.4 for type 1 (precision) or type 2 (general purpose) sound measurement instrumentation, or a local equivalent. | 在测量中，使用符合 ANSI S1.4 1 类（精确）或 2 类（一般用途）声音测量仪器，或本地对应标准的声级表。 |
| 18809954d6e0-f4d7-4df6-9b34-8b27fe7c53ea | Translated (0%) | Comply with design criteria for HVAC noise levels resulting from the sound transmission paths listed in ASHRAE 2011 Applications Handbook, Table 6; or a local equivalent. | 符合 ASHRAE 2011 应用手册，表 6；或本地对应标准中所列声音传播路径的暖通空调噪音等级设计标准。 |
| 1881a8230c80-b99a-44b4-b449-8d0507b7c7e6 | Translated (0%) | Sound Transmission | 声音传播 |
| 1882d4dbe54c-9734-4d95-ba2f-8960707685d3 | Translated (0%) | Meet the composite sound transmission class (STC<2306>C</2306>) ratings listed in Table 1, or local building code, whichever is more stringent. | 满足表 1 中所列复合声音传播等级 (STCc)，或本地建筑法规（以更严格的为准）。 |
| 1883be7abe59-c0de-4eb8-a88b-9243085278e8 | Translated (100%) | Table 1. | 表 1. |
| 1884be7abe59-c0de-4eb8-a88b-9243085278e8 | Translated (0%) | Maximum composite sound transmission class ratings for adjacent spaces | 相邻空间最大复合声音传播等级 |
| 188514030fcf-e7f1-4e88-871e-59032c67f63c | Translated (0%) | Adjacency combinations | 相邻组合 |
| 1886ce70dc7a-3039-42d2-a454-64558afab57c | Translated (0%) | STC<2315>C</2315> | STC<2315>C</2315> |
| 1887b9507292-bbdf-438d-a8dd-f051c0abf7bd | Translated (0%) | Residence (within a multifamily residence), hotel or motel room | 住宅（在多户住宅中）、酒店或汽车旅馆房间 |
| 18888782a23d-3cc4-4db7-b2e5-5a3846227567 | Translated (0%) | Residence, hotel or motel room | 住宅、酒店或汽车旅馆房间 |
| 1889c3facc2a-bb43-4a17-b3d7-ccaddd185353 | Translated (100%) | 55 | 55 |
| 18909d957d4d-3533-499f-9c02-72d7060cabaf | Translated (100%) | Residence, hotel or motel room | 住宅、酒店或汽车旅馆房间 |
| 189108aaeb36-86bc-445f-b7e6-a7315b9bb222 | Translated (0%) | Common hallway, stairway | 公用走廊，楼梯 |
| 1892c4dcb4f4-4f4a-409b-b732-f599188f967d | Translated (100%) | 50 | 50 |
| 18936b695857-3136-4a9f-b17d-3c85ef908757 | Translated (CM) | Residence, hotel or motel room | 住宅、酒店或汽车旅馆房间 |
| 1894f37c6092-3020-43e2-8779-e80bcfcc65c2 | Translated (100%) | Retail | 零售 |
| 18952dd2c7e2-6066-4072-a74d-4d74cdf2b4ff | Translated (100%) | 60 | 60 |
| 189652a3ec6f-c9d2-4247-a4c5-1c8df435dc18 | Translated (100%) | Retail | 零售 |
| 18978ab9a20a-a049-4c9a-9696-782bb864b66f | Translated (100%) | Retail | 零售 |
| 1898ef9bd5d0-bae9-461b-ab08-06218a9c2a2e | Translated (100%) | 50 | 50 |
| 1899c5e486b0-c288-4cc2-983b-a6a9f90bc0ec | Translated (0%) | Standard office | 标准办公室 |
| 190054b23bf8-7147-4534-a350-cd963bc7bc87 | Translated (100%) | Standard office | 标准办公室 |
| 19012f4a6e96-818c-4235-a253-d819089aea6f | Translated (100%) | 45 | 45 |
| 1902ee24ecc1-64bd-4274-b659-7f7cb5b88deb | Translated (0%) | Executive office | 行政办公室 |
| 19037bdba501-657e-4e13-a59e-a0c85cd03784 | Translated (100%) | Executive office | 行政办公室 |
| 1904af60b56f-fc83-4850-93b6-2a5e8ec489bf | Translated (100%) | 50 | 50 |
| 19051667ac15-a432-4193-9020-8ae6a3c4a119 | Translated (70%) | Conference room | 会议室 |
| 1906a1e48839-a97a-4820-911a-cfcd30d8dcf2 | Translated (100%) | Conference room | 会议室 |
| 1907ac9019e7-38e4-4c76-aa30-46d7dcb43315 | Translated (100%) | 50 | 50 |
| 19081b0f3961-9391-4976-8972-6af3908f43ba | Translated (71%) | Office, conference room | 办公室，会议室 |
| 19099c9b08a4-7434-435f-b97f-2d9671963662 | Translated (0%) | Hallway, stairway | 走廊，楼梯 |
| 1910258a1019-242c-4418-81e3-4d7bdd12adc6 | Translated (100%) | 50 | 50 |
| 191156abca75-2c62-4273-9b89-1ca34c7c4c07 | Translated (0%) | Mechanical equipment room | 机械设备间 |
| 19124832a419-a023-4711-91c4-25e34505d57b | Translated (0%) | Occupied area | 使用空间 |
| 1913d97b5b1d-77a1-480d-a372-eb68bd0da79b | Translated (100%) | 60 | 60 |
| 191463f2c5f8-f747-448e-80d4-212222d7b8aa | Translated (0%) | Reverberation Time | 混响时间 |
| 1915cf51bc64-05a6-4e52-914d-92db03db7c27 | Translated (0%) | Meet the reverberation time requirements in Table 2 (adapted from Table 9.1 in the Performance Measurement Protocols for Commercial<2426> Buildings<2427/>).</2426> | 满足表 2 中的混响时间要求（修改自商业建筑能效测量方案表 9.1<2426><2427/>）。</2426> |
| 19166f7142d3-f172-431c-917f-1259bbd4e226 | Translated (0%) | Adapted from ASHRAE (2007d), ASA (2008), ANSI (2002), and CEN (2007) | 修改自 ASHRAE (2007d)、ASA (2008)、ANSI (2002) 和 CEN (2007) |
| 19178567ebd3-9c81-4042-a313-dfcc4b72f40d | Translated (100%) | Table 2. | 表 2. |
| 19188567ebd3-9c81-4042-a313-dfcc4b72f40d | Translated (75%) | Reverberation time requirements | 混响时间要求 |
| 1919c0f74392-e2dc-488d-b089-2590f4a81dd8 | Translated (79%) | Room type | 房间类型 |
| 19203c440818-50ff-4ac5-8358-0bc1485ffd5c | Translated (0%) | Application | 应用 |
| 19218ffb3994-c306-4241-a0c1-31b57f6ceb8f | Translated (0%) | T60 (sec), at 500 Hz, 1000 Hz, and 2000 Hz | T60（秒），500 Hz、1000 Hz 和 2000 Hz 时 |
| 19221374872d-9bbd-4b04-842e-9e4984ffab27 | Translated (0%) | Apartment and condominium | 单层公寓和共管式公寓 |
| 1923551fa050-9b7c-426a-820e-9ece00385b06 | Translated (0%) | — | — |
| 192463de0536-a9b9-4437-9975-89f14894a566 | Translated (100%) | < 0.6 | < 0.6 |
| 1925b03de5c3-1804-4cac-8603-dcee118cfa71 | Translated (0%) | Hotel/motel | 酒店/汽车旅馆 |
| 1926a64cb366-75db-488e-8a4f-606afa703b37 | Translated (0%) | Individual room or suite | 单个房间或套间 |
| 192745a0b26b-9dca-4da8-8723-a9c5c53d96b7 | Translated (100%) | < 0.6 | < 0.6 |
| 1928ffaf0a27-b225-41c1-b1ea-b17bf123314c | Translated (0%) | Meeting or banquet room | 会议室或宴会厅 |
| 19297f5cedf4-2aae-422a-ba62-7b974c7f4de0 | Translated (100%) | < 0.8 | < 0.8 |
| 1930a68916e3-65c6-45c9-b072-9494cb996abf | Translated (0%) | Office building | 办公楼 |
| 1931328c7733-2cc7-4ced-a4e0-55e55559e3d6 | Translated (0%) | Executive or private office | 行政或私人办公室 |
| 1932314adec9-4f51-4931-9aed-25e2ac41513d | Translated (100%) | < 0.6 | < 0.6 |
| 193378c2dcde-2b44-4995-8990-cda823b0666b | Translated (CM) | Conference room | 会议室 |
| 19340534dca7-f57d-4af8-b99e-b09cdfa36629 | Translated (100%) | < 0.6 | < 0.6 |
| 193592dc83f4-a3d7-4760-879a-ce62de3f2ff0 | Translated (0%) | Teleconference room | 电话会议室 |
| 1936faec872e-736c-4db3-8862-b1f1b496e5ff | Translated (100%) | < 0.6 | < 0.6 |
| 1937e84f7a9e-445d-49df-8a7f-e703d443db29 | Translated (0%) | Open-plan office without sound masking | 没有声掩蔽的开放式办公室 |
| 19384dcf069d-db68-4793-9d6d-52aef8ff3dc4 | Translated (100%) | < 0.8 | < 0.8 |
| 1939468315b0-b8ee-4fd3-abf5-9df8987edc8b | Translated (87%) | Open-plan office with sound masking | 有声掩蔽的开放式办公室 |
| 1940573600dc-515e-41b6-b677-0454f85385e3 | Translated (100%) | 0.8 | 0.8 |
| 194179e15c13-a1c0-48e7-8bff-4cf75476e4cb | Translated (0%) | Courtroom | 法庭 |
| 19426dd7e028-1477-4b2c-8e8b-a802207cb5b4 | Translated (0%) | Unamplified speech | 不扩音的讲话 |
| 1943ad60c0ee-6ce9-41bf-8074-47bc87506c49 | Translated (0%) | < 0.7 | < 0.7 |
| 194421b52db7-ecb7-40e9-b3de-a1f59d33022c | Translated (84%) | Amplified speech | 扩音的讲话 |
| 1945b1c6cbf8-d90d-4d2e-9709-87f028caf505 | Translated (100%) | < 1.0 | < 1.0 |
| 19460a5be20d-10cb-4843-b18f-f0c4bd593d48 | Translated (0%) | Performing arts space | 艺术表演空间 |
| 1947028f9fa3-c5ff-421d-ae6b-0936d293234f | Translated (0%) | Drama theaters, concert and recital halls | 戏剧院、音乐会和独奏厅 |
| 1948b10bc0f2-2639-4d19-9af4-99d89f625dba | Translated (0%) | Varies by application | 因应用而不同 |
| 19491723e1f2-0ac5-416c-a4ac-c3f28b28398e | Translated (0%) | Laboratories | 实验室 |
| 1950b0a86e36-d436-4401-91e0-59e3aae1d9c1 | Translated (0%) | Testing or research with minimal speech communication | 在尽可能少地进行语言沟通的情况下测试或研究 |
| 19514f3f53fc-afa3-45ac-a986-64b7e0870ef8 | Translated (100%) | < 1.0 | < 1.0 |
| 1952874494b4-6d0c-4bb4-950f-cce68151acd0 | Translated (0%) | Extensive phone use and speech communication | 经常使用电话和进行语言沟通 |
| 1953a40c3ecd-916f-42de-b583-6b4fdc19da58 | Translated (100%) | < 0.6 | < 0.6 |
| 19547ac40602-91df-424c-8d5c-bc046acd654f | Translated (0%) | Church, mosque, synagogue | 教堂、清真寺、犹太教堂 |
| 1955f82ed85e-4a0e-4dad-a33c-80aad67cb258 | Translated (0%) | General assembly with critical music program | 有重要音乐节目的集会 |
| 1956841eaa90-ae0e-44b0-9576-3c7909a2c5f8 | Translated (100%) | Varies by application | 因应用而不同 |
| 1957ce49be67-6bdd-4bee-8c7c-0d6c0429c750 | Translated (100%) | Library | 图书馆 |
| 19582b8ca7c2-57e4-4380-9398-343947496bdb | Translated (100%) | < 1.0 | < 1.0 |
| 19594896c1b4-f505-4e16-a84c-c74fb4e10443 | Translated (0%) | Indoor stadium, gymnasium | 室内体育场，体育馆 |
| 196083c7811d-305e-4c5d-82c1-3a94311ce4b4 | Translated (0%) | Gymnasium and natatorium | 体育馆和游泳馆 |
| 196132f079dd-d98e-4118-be50-8bdfb754a12a | Translated (100%) | < 2.0 | < 2.0 |
| 1962b9ca5858-8ee2-468f-b9d6-75d5d8dad515 | Translated (0%) | Large-capacity space with speech amplification | 带扩音效果且可容纳很多人的空间 |
| 196387949ef4-7533-4def-83d3-8a75ef43ab5d | Translated (100%) | < 1.5 | < 1.5 |
| 196425dae9b5-3ed5-4642-9c59-776daafcb8bc | Translated (0%) | Classroom | 教室 |
| 1965c5e1843e-6dc9-40b8-b1a7-52989589fb78 | Translated (100%) | — | — |
| 1966af30bbce-573b-4805-aa08-a865619c0c76 | Translated (CM) | < 0.6 | < 0.6 |
| 1967cedaa582-0016-48e3-9f7c-91e9c2ddfeac | Translated (0%) | Sound Reinforcement and Masking Systems | 扩音和掩蔽系统 |
| 19684451dd70-ad72-4901-93e2-3c557c90d5cf | Translated (0%) | Sound Reinforcement | 扩音 |
| 1969c94d3199-2fda-435d-aa53-58c9555e31ed | Translated (0%) | For all large conference rooms and auditoriums seating more than 50 persons, evaluate whether sound reinforcement and AV playback capabilities are needed. | 对于所有可容纳超过 50 人的大型会议室和礼堂，评估是否需要扩音和影音播放功能。 |
| 19704b9c9d8b-8456-4207-84d8-2d3c956fb0c4 | Translated (0%) | If needed, the sound reinforcement systems must meet the following criteria: | 如果需要，扩音系统必须满足以下条件： |
| 1971c43f4929-bdb0-4dbc-86fd-737d44867e2f | Translated (0%) | Achieve a speech transmission index (STI) of at least 0.60 or common intelligibility scale (CIS) rating of at least 0.77 at representative points within the area of coverage to provide acceptable intelligibility. | 在覆盖范围内具有代表性的点上实现至少 0.60 的语言传输指数 (STI) 或者至少 0.77 的普通清晰度范围 (CIS) 等级，以提供可接受的清晰度。 |
| 1972f699332a-d7a8-4be1-864f-51afb4f3b7b7 | Translated (0%) | Have a minimum sound level of 70 dBA. | 最低音量为 70 dBA。 |
| 1973a0853d87-0390-4b69-a75d-9ac4d460a353 | Translated (0%) | Maintain sound-level coverage within +/–3 dB at the 2000 Hz octave band throughout the space. | 对于整个空间，在 2000 Hz 倍频带上使音量覆盖范围保持在 +/–3 dB 范围内。 |
| 1974e958f320-ef41-4b85-af8d-43457449d728 | Translated (0%) | Masking Systems | 掩蔽系统 |
| 1975961f76f2-7176-45b3-8679-d323dbeb9d40 | Translated (0%) | For projects that use masking systems, the design levels must not exceed 48 dBA. | 对于使用掩蔽系统的项目，设计音量不得超过 48 dBA。 |
| 1976961f76f2-7176-45b3-8679-d323dbeb9d40 | Translated (0%) | Ensure that loudspeaker coverage provides uniformity of +/–2 dBA and that speech spectra are effectively masked. | 确保扬声器覆盖范围提供 +/–2 dBA 的一致性，且能够有效掩蔽语谱。 |
| 1977499454da-772c-4e55-9504-f65a4952e65a | Translated (91%) | Innovation (IN) | 创新 (IN) |
| 19783dbde7ec-d886-4fa3-a94a-a807d0b42562 | Translated (0%) | IN Credit: Innovation | IN 得分点：创新 (Innovation) |
| 1979d27abfe2-ee7f-4ec4-8891-3937fd167154 | Translated (100%) | ID&C | ID&C |
| 1980dcbec9b2-8aec-419f-8945-d6975f7c691d | Translated (100%) | 1–5 points | 1-5 分 |
| 19815d6c8207-03ed-4119-9726-f17da95546a5 | Translated (100%) | This credit applies to | 该得分点适用于 |
| 1982574c49bd-f68c-4039-ae79-53cfb7ce5c81 | Translated (100%) | Commercial Interiors (1–5 points) | 商业室内（1-5 分） |
| 198306e01f06-02f0-460f-be74-167274da2598 | Translated (100%) | Retail (1–5 points) | 零售（1-5 分） |
| 19840ed5610e-f252-4e52-a0c0-f27c6261a0b6 | Translated (100%) | Hospitality (1–5 points) | 宾馆接待（1-5 分） |
| 19852f41c40d-51dd-4ab9-ab22-2143f5e6691c | Translated (100%) | Intent | 目的 |
| 1986fcc05531-1d8c-4186-a5c5-4690cea90a03 | Translated (0%) | To encourage projects to achieve exceptional or innovative performance | 鼓励项目实现优良表现或创新表现 |
| 1987408d466c-b27c-48fd-b70f-80733050aa21 | Translated (100%) | Requirements | 要求 |
| 1988540e07e0-7c31-49fc-ae1a-a8d28c33db89 | Translated (CM) | CI, Retail, Hospitality | 商业室内，零售，宾馆接待 |
| 1989991b2d2c-8979-46c5-b59c-d1e1769277a4 | Translated (0%) | Project teams can use any combination of innovation, pilot, and exemplary performance strategies. | 项目团队可使用创新、试点和优良表现策略的任意组合。 |
| 19909ad4e91e-bf81-4d65-a1b1-be6cf2dea0a5 | Translated (100%) | Option 1. | 选项 1. |
| 19919ad4e91e-bf81-4d65-a1b1-be6cf2dea0a5 | Translated (0%) | <2635>Innovation</2635> <2636>(1 point)</2636> | 创新（1 分） |
| 19924598b049-e2bb-4c83-840e-a6f1baf7756e | Translated (84%) | Achieve significant, measurable environmental performance using a strategy not addressed in the LEED green building rating system. | 使用 LEED 绿色建筑评估体系中没有涉及到的策略实现突出的环境表现。 |
| 19936d7fe9de-cf3b-455d-8c8f-c50580ffc34a | Translated (78%) | Identify the following: | 确定以下内容： |
| 1994fad81e2a-ecd8-4a81-a45f-cf32655bc17e | Translated (99%) | the intent of the proposed innovation credit; | 提议的创新得分点的意图； |
| 1995e0b6d1fd-9055-45fc-907f-4c8cf503f1b2 | Translated (84%) | proposed requirements for compliance; | 提议的合规性要求； |
| 1996735e12e6-dba3-430c-8ae3-9e0b4f6047f3 | Translated (89%) | proposed submittals to demonstrate compliance; and | 建议要递交的以证明满足得分要求的材料；以及 |
| 199788685b8e-bc6f-405f-81ed-ae37d9046acc | Translated (0%) | the design approach or strategies used to meet the requirements. | 用于满足要求的设计方法和策略 |
| 19986ce88fe9-6071-4817-a60d-f0c618c47964 | Translated (100%) | AND/OR | 和/或 |
| 19994c9c21af-71a4-43ab-8568-c9cb3152f14f | Translated (CM) | Option 2. | 选项 2. |
| 20004c9c21af-71a4-43ab-8568-c9cb3152f14f | Translated (0%) | Pilot (1 point) | 试点（1 分） |
| 2001fd7671a7-6275-468c-bf8f-98693bed9a93 | Translated (0%) | Achieve one pilot credit from USGBC’s LEED Pilot Credit Library | 从 USGBC 的 LEED 试行得分点库中获得一个试行得分点 |
| 200211bfb712-cc40-4480-9c52-cc1daf5e9d6f | Translated (100%) | AND/OR | 和/或 |
| 2003876c1ed9-6e4a-4386-9d66-e34cf2b362c3 | Translated (CM) | Option 3. | 选项 3. |
| 2004876c1ed9-6e4a-4386-9d66-e34cf2b362c3 | Translated (98%) | Additional Strategies | 其他策略 |
| 2005f897a0da-96cb-49b8-b7c5-060a5e0f8e98 | Translated (78%) | <2639>Innovation</2639> <2640>(1-3 points)</2640> | 创新（1-3 分） |
| 2006b47ddef1-0c12-4ca6-b169-bc6830152ec6 | Translated (0%) | Defined in Option 1 above. | 已在上面的选项 1 中定义。 |
| 200772f750d5-dab1-4a5f-8cdc-8c08208daced | Translated (78%) | <2641>Pilot</2641> <2642>(1-3 points)</2642> | 试点（1-3 分） |
| 200825b21603-1e44-4f40-b921-acc6cd2d3bc2 | Translated (89%) | Meet the requirements of Option 2. | 满足选项 2 的要求。 |
| 2009ce4fea50-bbbb-4a6e-94bd-6b72bd7fa8ba | Translated (96%) | <2643>Exemplary Performance</2643> <2644>(1–2 points)</2644> | 优良表现（1-2 分） |
| 201003b64244-7ce7-44c7-b191-f9b858ee5451 | Translated (74%) | Achieve exemplary performance in an existing LEED v4 prerequisite or credit that allows exemplary performance, as specified in the LEED Reference Guide, v4 edition. | 在允许 v4 版 LEED 参考指南中指定的优良表现的既有 LEED v4 先决条件或得分点中实现优良表现。 |
| 201103b64244-7ce7-44c7-b191-f9b858ee5451 | Translated (0%) | An exemplary performance point is typically earned for achieving double the credit requirements or the next incremental percentage threshold. | 一般可通过实现双倍得分点要求或下一个增量百分比阈值来获得优良表现分数。 |
| 2012b98f76c2-1f95-4edd-b3dc-57f389948037 | Translated (70%) | IN Credit: LEED Accredited Professional | IN 得分点：LEED AP |
| 20139f9cafc5-5c37-45be-b6bd-843d6f82c4f9 | Translated (100%) | ID&C | ID&C |
| 20142b762d8b-4ae0-46dd-a739-6b981fed992e | Translated (100%) | 1 point | 1 分 |
| 2015b9c3972d-d3ec-4084-8a09-c170449f7268 | Translated (CM) | This credit applies to | 该得分点适用于 |
| 2016c89e85f3-9afa-40d0-9271-6c1220a45077 | Translated (CM) | Commercial Interiors (1 point) | 商业室内（1 分） |
| 2017bbdd695c-9557-4035-b86c-eefdab559740 | Translated (CM) | Retail (1 point) | 零售（1 分） |
| 2018301dc5e3-7aa6-4666-a2e8-4506cf752455 | Translated (CM) | Hospitality (1 point) | 宾馆接待（1 分） |
| 201995dc59a4-a664-4f7f-bcde-3bdc99ddd2c3 | Translated (100%) | Intent | 目的 |
| 2020ea63c2d7-660a-4507-8786-77d3b0a19cd8 | Translated (0%) | To encourage the team integration required by a LEED project and to streamline the application and certification process. | 鼓励 LEED 项目要求的团队整合，以及简化应用和认证过程。 |
| 202192b02bae-2167-41cb-82f1-43efc284d2c3 | Translated (100%) | Requirements | 要求 |
| 2022fcde7dd3-b967-4963-a0d9-c4557a571611 | Translated (85%) | CI, Retail CI, Hospitality | 商业室内，零售商业室内，宾馆接待 |
| 2023401e13d3-ca8e-43e6-96c2-6fd6e4f0a1da | Translated (0%) | At least one principal participant of the project team must be a LEED Accredited Professional (AP) with a specialty appropriate for the project. | 项目团队中至少有一个主要参与者必须是拥有适合该项目的专长的 LEED AP。 |
| 2024e58f6477-2b69-412a-be68-b64e542d68e1 | Translated (100%) | Regional Priority (RP) | 地域优先 (RP) |
| 2025b011535f-965d-4d03-b736-750f21517a81 | Translated (0%) | RP Credit: Regional Priority | RP 得分点：地域优先 (Regional Priority) |
| 2026bcd56d4d-d4e8-486b-844d-0fa99d88a7d3 | Translated (100%) | ID&C | ID&C |
| 202766a1090c-6d75-438f-ba40-72e5e46a8738 | Translated (CM) | 4 points | 4 分 |
| 2028504ac918-2605-416b-ad0e-3371b2148314 | Translated (CM) | This credit applies to | 该得分点适用于 |
| 2029089eaac9-1fc4-45c7-a797-10e814a671d9 | Translated (CM) | Commercial Interiors (1-4 points) | 商业室内（1-4 分） |
| 20303b54f73c-a5f2-4f38-b583-67d7a8b4ab1b | Translated (CM) | Retail (1-4 points) | 零售（1-4 分） |
| 2031f30f3c6a-fd0e-49f3-b0e4-35e63a444d92 | Translated (CM) | Hospitality (1-4 points) | 宾馆接待（1-4 分） |
| 20321c48e836-68db-49c8-9370-bb4b1fe6eed9 | Translated (100%) | Intent | 目的 |
| 20332037309d-c982-47bd-90cb-3e7e6513e103 | Translated (79%) | To provide an incentive for the achievement of credits that address geographically specific environmental, social equity, and public health priorities. | 为获得解决特定地域优先环境、社会公平和公众健康问题的得分点提供激励措施。 |
| 2034e2d0d924-e8f8-4b87-94b7-1ed9d8c358d8 | Translated (100%) | Requirements | 要求 |
| 2035e1ecdf23-9871-49c8-a28f-f1214768fac3 | Translated (CM) | CI, Retail CI, Hospitality | 商业室内，零售商业室内，宾馆接待 |
| 203674362291-988b-410c-92a7-dbe599ee7cfd | Translated (0%) | Earn up to four of the six Regional Priority credits. | 最多可获得六个地域优先 (Regional Priority) 得分点中的四个。 |
| 203774362291-988b-410c-92a7-dbe599ee7cfd | Translated (0%) | These credits have been identified by the USGBC regional councils and chapters as having additional regional importance for the project’s region. | 这些得分点已被 USGBC 地区委员会和分会确定为对项目的区域有额外的区域重要性。 |
| 203874362291-988b-410c-92a7-dbe599ee7cfd | Translated (96%) | A database of Regional Priority credits and their geographic applicability is available on the USGBC website, <2655>http://www.usgbc.org</2655>. | USGBC 网站 <2655>http://www.usgbc.org</2655> 上提供了地域优先得分点及其地域适用性的数据库。 |
| 2039dc296da1-9301-4d02-886d-79b923546651 | Translated (100%) | http://www.usgbc.org | http://www.usgbc.org |
| 20400a1cad05-7b4b-4c71-a517-4e483ea9a2dc | Translated (0%) | One point is awarded for each Regional Priority credit achieved, up to a maximum of four. | 每获得一个地域优先得分点即可获得一分，最多获得四分。 |
| 20416dfaf93b-7965-40e0-bcd2-b81088dc03f1 | Translated (100%) | Appendices | 附录 |
| 2042061691ab-936c-4d49-b0b4-b5ffe6856467 | Translated (0%) | Appendix 1. | 附录 1 |
| 2043061691ab-936c-4d49-b0b4-b5ffe6856467 | Translated (0%) | Use Types and Categories | 使用类型和类别 |
| 2044143384a9-fb23-4abf-97b7-5f5b95024b80 | Translated (100%) | Table 1. | 表 1. |
| 2045143384a9-fb23-4abf-97b7-5f5b95024b80 | Translated (100%) | Use Types and Categories | 使用类型和类别 |
| 20468bda9010-2e27-40e4-9009-de3043d03738 | Translated (100%) | Category | 类别 |
| 2047c00dc358-ec0c-4b35-a20d-751a63f5e2e7 | Translated (0%) | Use type | 使用类型 |
| 20481c7db79a-dfd1-4b99-8804-7da750f859ca | Translated (0%) | Food retail | 食品零售 |
| 2049234919d2-9e61-48d6-a754-9dc0dac5d262 | Translated (100%) | Supermarket | 超市 |
| 20508c1b47e6-866a-4ffc-aa38-b01d9d646dff | Translated (0%) | Grocery with produce section | 有农产品专区的杂货店 |
| 20514c40a0ca-29bf-457c-82f2-4d5f3893b74e | Translated (0%) | Community-serving retail | 社区服务零售 |
| 20528d7b5e0e-9812-43e6-934c-471da52c2284 | Translated (0%) | Convenience store | 便利店 |
| 20534e66304c-4126-4c97-a32e-164555b9e7fd | Translated (0%) | Farmers market | 农贸市场 |
| 205427c8b2e2-0262-48c0-a9d8-2594b0e43d78 | Translated (0%) | Hardware store | 五金店 |
| 2055e7d1d38c-2f32-42cb-a2e4-5877f9d75116 | Translated (100%) | Pharmacy | 药店 |
| 20564df7410e-a498-4075-94ac-107e64d15426 | Translated (0%) | Other retail | 其他零售 |
| 2057d3dc14bc-745c-4f20-88e4-96dbf0b272ba | Translated (0%) | Services | 服务 |
| 205851059113-e58b-424f-9204-2076450c0cb5 | Translated (100%) | Bank | 银行 |
| 2059199055fc-4ad6-41bc-8607-601b09babde7 | Translated (0%) | Family entertainment venue (e.g., theater, sports) | 家庭娱乐场所（如剧场、体育） |
| 20604c4c6d9b-1035-410a-a442-0238a5828cc0 | Translated (0%) | Gym, health club, exercise studio | 健身房、健身俱乐部、健身室 |
| 20615370554f-5351-41e5-a512-78b00ef53fd5 | Translated (0%) | Hair care | 头发护理 |
| 20621b84d5db-0c17-4fbb-8d23-5342e13f4da8 | Translated (0%) | Laundry, dry cleaner | 洗衣店、干洗店 |
| 206367c29c1b-1e3f-416d-80d5-9350d19460e2 | Translated (0%) | Restaurant, café, diner (excluding those with only drive-thru service) | 餐厅、咖啡厅、快餐店（只有免下车服务的除外） |
| 20646df1f849-1aba-4495-9517-1528e89250c4 | Translated (0%) | Civic and community facilities | 公民及社区设施 |
| 2065893284a4-61c2-4821-9d7f-00b13ae54569 | Translated (0%) | Adult or senior care (licensed) | 成人或老人护理（有许可） |
| 206615647e84-9ec7-43cd-8a2f-f1231eb79e14 | Translated (0%) | Child care (licensed) | 儿童护理（有许可） |
| 20672f5f782f-915d-4ca4-bb0a-b126bc868fbd | Translated (0%) | Community or recreation center | 社区或娱乐中心 |
| 206858e851d2-57c0-466b-9662-34dee6b7c967 | Translated (0%) | Cultural arts facility (museum, performing arts) | 文化艺术设施（博物馆、艺术表演） |
| 20693923b3f8-4c01-4aeb-b7d0-67d1a8768a8d | Translated (0%) | Education facility (e.g., K—12 school, university, adult education center, vocational school, community college) | 教育设施（如 K—12 学校、大学、成人教育中心、职业学校、社区学院） |
| 207090fde308-ae32-41c5-a289-adb64ba41762 | Translated (0%) | Government office that serves public on-site | 在现场服务公众的政府办公室 |
| 207180332ddb-8aa5-4bba-b056-81b6dc3218b7 | Translated (0%) | Medical clinic or office that treats patients | 治疗病人的医疗诊所或办公室 |
| 2072a83c3e65-d537-41fb-807f-a6a9f4444d52 | Translated (97%) | Place of worship | 教堂 |
| 20732e55c5a4-197b-4302-9554-8ea3ab341d1d | Translated (0%) | Police or fire station | 警察或消防局 |
| 207438fed83b-6b10-42cf-8695-4635d0d76462 | Translated (96%) | Post office | 邮局 |
| 2075aa100c41-2cf4-4a7a-a237-0df97d4652c7 | Translated (0%) | Public library | 公共图书馆 |
| 207644cf42d6-a767-4db4-bd86-115f81aee0ab | Translated (0%) | Public park | 公园 |
| 2077133f3b3c-a9b5-409c-979b-3c355b34b78f | Translated (0%) | Social services center | 社会服务中心 |
| 2078b6e25b94-50a1-4283-b411-c3a2cac27fb9 | Translated (0%) | Community anchor uses (BD&C and ID&C only) | 社区精神支柱使用（仅 BD&C 和 ID&C） |
| 2079837abc86-4b0e-4c30-aeed-d6c9863da67b | Translated (0%) | Commercial office (100 or more full-time equivalent jobs) | 商务办公室（100 或更多全职工作） |
| 208021cae48d-23d4-4649-90fe-7e1117767bff | Translated (0%) | Housing (100 or more dwelling units) | 住房（100 或更多居住单元） |
| 20810a5163bd-0582-49d6-9835-8381c78a432f | Translated (0%) | Adapted from Criterion Planners, INDEX neighborhood completeness indicator, 2005. | 修改自标准规划师，INDEX 社区完整性指标，2005。 |
| 2082a4da3244-0d75-44b5-b5f2-3d13cb0e95c4 | Translated (100%) | Appendix 2. | 附录 2 |
| 2083a4da3244-0d75-44b5-b5f2-3d13cb0e95c4 | Translated (99%) | Default Occupancy Counts | 默认入驻人数 |
| 2084dc8b9041-69a0-4e60-9f18-0e285312b900 | Translated (0%) | Use Table 1 to calculate default occupancy counts. | 使用表 1 计算默认入驻人数。 |
| 2085dc8b9041-69a0-4e60-9f18-0e285312b900 | Translated (0%) | Only use the occupancy estimates if occupancy is unknown. | 如果入驻人数未知，则使用入驻人数预估值。 |
| 20861fa71161-69e9-4219-9284-6ea921655ee6 | Translated (0%) | For the calculation, use gross floor area, not net or leasable floor area. | 在计算中使用总建筑面积，而不是净建筑面积或可租赁的建筑面积。 |
| 20871fa71161-69e9-4219-9284-6ea921655ee6 | Translated (0%) | Gross floor area is defined as the sum of all areas on all floors of a building included within the outside faces of the exterior wall, including common areas, mechanical spaces, circulation areas, and all floor penetrations that connect one floor to another. | 总建筑面积被定义为包括在外墙外表面之中的建筑所有楼层所有面积之和，包括公共面积、机械空间、循环区域，以及连接不同楼层的所有楼层穿透。 |
| 20881fa71161-69e9-4219-9284-6ea921655ee6 | Translated (0%) | To determine gross floor area, multiply the building footprint (in square feet or square meters) by the number of floors in the building. | 要确定总建筑面积，需要用建筑占地面积（单位为平方英尺或平方米）乘以建筑楼层数。 |
| 20891fa71161-69e9-4219-9284-6ea921655ee6 | Translated (0%) | Exclude underground or structured parking from the calculation. | 计算中不包括地下或构筑的停车场。 |
| 20901a840675-051d-4f2b-a49c-44fa09907360 | Translated (100%) | Table 1. | 表 1. |
| 20911a840675-051d-4f2b-a49c-44fa09907360 | Translated (76%) | Default Occupancy Numbers | 默认入驻人数 |
| 2092711b0c37-a487-4e78-9532-b016d92113f4 | Translated (0%) | Gross square feet per occupant | 每个驻户的总平方英尺数 |
| 20932caf4005-1679-469d-9e65-7337244b475e | Translated (87%) | Gross square meters per occupant | 每个驻户的总平方米数 |
| 2094320b6d52-5d5d-41c3-80da-0cfd0a4125cc | Translated (0%) | Employees | 员工 |
| 2095b7e4da0d-5378-4717-9420-64bfed51df1f | Translated (0%) | Transients | 临时驻户 |
| 209613ce672d-5bbb-42d4-8fe0-c6583685eddd | Translated (100%) | Employees | 员工 |
| 209707ef802c-d6e5-400f-b7af-c468ba74dc34 | Translated (CM) | Transients | 临时驻户 |
| 20981d428852-3d12-431a-a5cf-93901ba13aa0 | Translated (100%) | General office | 常规办公室 |
| 2099fe6aa798-7b5d-424d-8566-adbd4046cbe6 | Translated (100%) | 250 | 250 |
| 2100e5876143-4522-4e05-8e87-aa15355fd83b | Translated (100%) | 0 | 0 |
| 2101603c84af-c214-4b7c-9ee6-176c58fc2cf3 | Translated (100%) | 23 | 23 |
| 2102e94deeef-2a83-4366-a7d7-b746a5dbfd70 | Translated (100%) | 0 | 0 |
| 210302ca6d7a-353b-452c-8793-7f4875d0121e | Translated (0%) | Retail, general | 常规零售 |
| 210424fa825d-1ed2-4f67-8586-4ffdbee4c161 | Translated (100%) | 550 | 550 |
| 21056ebb3150-b710-42c0-97b1-1ef5f88dd019 | Translated (100%) | 130 | 130 |
| 21061bfdb406-a813-4ca3-ba6e-e375392f5c25 | Translated (100%) | 51 | 51 |
| 2107dcfb75e3-465a-4408-b3ac-79244917f3f8 | Translated (100%) | 12 | 12 |
| 210816b36778-2f09-47e7-b51c-4b5c869c1ba1 | Translated (0%) | Retail or service (e.g., financial, auto) | 零售或服务（如，金融、汽车） |
| 210920c392f2-2549-4e18-9783-4109324d20ff | Translated (100%) | 600 | 600 |
| 2110cf2070da-2bf4-425a-b2a4-b365e258a701 | Translated (100%) | 130 | 130 |
| 2111bf54c919-b59f-49c3-9382-81288f515f3f | Translated (100%) | 56 | 56 |
| 21125ba01610-ed1f-4e7e-838e-6b056d0959b4 | Translated (100%) | 12 | 12 |
| 211365b76eaf-a4a8-4235-a24f-985f64b4ae8f | Translated (100%) | Restaurant | 餐馆 |
| 211418c2c96a-1bfc-4458-acd5-75d986c513b4 | Translated (100%) | 435 | 435 |
| 2115e92717d7-b130-4a9b-8410-d07234375b9a | Translated (100%) | 95 | 95 |
| 2116ceed3888-bad8-46bb-aab5-3d2d89453f7f | Translated (100%) | 40 | 40 |
| 2117004984a4-fabc-4ca3-b57b-80e1b2bacf0e | Translated (100%) | 9 | 9 |
| 2118750b97d3-12a6-4703-be3f-4729b6abb80a | Translated (0%) | Grocery store | 杂货店 |
| 21192704fd23-30e5-48eb-9fcf-b31c202e7b29 | Translated (100%) | 550 | 550 |
| 2120b08a68d7-222e-495f-84a5-6d528bbfd02a | Translated (100%) | 115 | 115 |
| 21213eae3efa-8c33-4c11-80c6-40d61dec80d9 | Translated (100%) | 51 | 51 |
| 212246a30f45-1bb1-45ab-89af-e44bd0b3d9e1 | Translated (100%) | 11 | 11 |
| 2123b9f709bf-004c-4268-8109-a3220d5934b6 | Translated (0%) | Medical office | 医疗办公室 |
| 21246f0faa29-50bd-4ec3-a047-000ea5d4a28c | Translated (100%) | 225 | 225 |
| 212573dc46df-bda8-47f7-9d51-5e780dd44d98 | Translated (100%) | 330 | 330 |
| 212694335414-0df8-490c-bd42-22fa246694fa | Translated (100%) | 21 | 21 |
| 2127cf15e681-430a-4843-ab0a-343edd0e6b66 | Translated (100%) | 31 | 31 |
| 212854f7e2ad-56d9-434f-be70-20434db5407b | Translated (0%) | R&D or laboratory | 研发或实验室 |
| 21295bc4105c-862b-4b8b-9399-c051566ccb52 | Translated (100%) | 400 | 400 |
| 21308a3a0ebd-691c-49c1-a708-cc020bad773b | Translated (100%) | 0 | 0 |
| 21315e9a00c0-d905-469c-946d-58f9745929c1 | Translated (100%) | 37 | 37 |
| 213208ced50b-e90e-4ed7-971e-ad7e1577e72f | Translated (100%) | 0 | 0 |
| 21330f6c5261-d5ad-47e8-b13f-b0606dfa90ee | Translated (0%) | Warehouse, distribution | 仓库，配送 |
| 2134af15989c-52c6-4258-9052-717472d6caf5 | Translated (100%) | 2,500 | 2,500 |
| 21353ca3bfd1-eb3e-42fc-89f6-d091dccdbe59 | Translated (100%) | 0 | 0 |
| 21364adbbbc3-3adc-4570-bfff-45e4c9f5f435 | Translated (100%) | 232 | 232 |
| 2137657cea96-3ef9-4f44-b46d-ccb7d19e3c16 | Translated (100%) | 0 | 0 |
| 2138ae6482c7-c0c3-4632-b486-8b4f447615a3 | Translated (0%) | Warehouse, storage | 仓库，存储 |
| 213990f529b9-bdea-40b4-b662-ffe17ffe0b60 | Translated (100%) | 20,000 | 20,000 |
| 2140346c1d75-f4f9-4395-a37a-56f03e984e99 | Translated (100%) | 0 | 0 |
| 214150d7080d-edfb-4092-ad0b-f0520d89a288 | Translated (100%) | 1860 | 1860 |
| 214295411f11-1178-41e8-abfa-ffc94cf91f02 | Translated (100%) | 0 | 0 |
| 2143138e78e7-093d-4172-acae-2e9b7f76eac5 | Translated (0%) | Hotel | 酒店 |
| 2144e5b43761-1fce-4845-95c0-b80044f9db8f | Translated (100%) | 1,500 | 1,500 |
| 21453da27e9e-2a8b-48e4-979b-34133fc2711c | Translated (100%) | 700 | 700 |
| 2146a9a6ee25-36f7-493b-9588-b50d566d63e6 | Translated (100%) | 139 | 139 |
| 214752f3313e-9a89-4128-92e4-f61641af64cc | Translated (100%) | 65 | 65 |
| 214853d12ce9-d683-496f-acf1-c6d1b257938f | Translated (0%) | Educational, daycare | 教育，日托 |
| 21497905e586-8921-49c3-ad74-512221520960 | Translated (100%) | 630 | 630 |
| 2150943261b0-6853-4222-95b8-f92bb0221d0f | Translated (100%) | 105 | 105 |
| 21514703c469-f92a-40f1-bcd5-021fa947b6e0 | Translated (100%) | 59 | 59 |
| 215265434d13-ee6b-47c5-90bb-6ef99f824a85 | Translated (100%) | 10 | 10 |
| 21530a1a53a1-8321-4737-a3dd-e4c943443fd6 | Translated (0%) | Educational, K–12 | 教育，K-12 |
| 215457984fd2-c0aa-4800-8983-52db35e5ce19 | Translated (100%) | 1,300 | 1,300 |
| 2155651b7e77-8259-4417-ab36-58efd03abc5d | Translated (100%) | 140 | 140 |
| 21564500c5f2-2465-45e4-b5f4-d3818af48b05 | Translated (100%) | 121 | 121 |
| 21575f6d780c-6bd6-4120-9699-df90365ac719 | Translated (100%) | 13 | 13 |
| 2158b92bf4eb-a5d3-462d-be84-8408e4339944 | Translated (0%) | Educational, postsecondary | 教育，高等教育 |
| 2159276b9d5e-0269-49dc-87c6-f5281862206a | Translated (100%) | 2,100 | 2,100 |
| 21602302f82c-5a0f-44ae-b85d-c3768c0082bc | Translated (100%) | 150 | 150 |
| 216150277812-cefa-4e77-a721-1d10a565894b | Translated (100%) | 195 | 195 |
| 21621cd2892e-b8a0-432f-b79f-a8463e53d2a3 | Translated (CM) | 14 | 14 |
| 216398702866-5058-4b41-81ab-53331df0be7e | Translated (0%) | Sources: | 来源： |
| 21648a9bf3e1-4e9c-48bc-929e-27c4dac548b2 | Translated (0%) | ANSI/ASHRAE/IESNA Standard 90.1–2004 (Atlanta, GA, 2004). | ANSI/ASHRAE/IESNA 标准 90.1–2004（佐治亚州亚特兰大，2004）。 |
| 2165d4773eaf-344a-4727-b6fe-0630cb589788 | Translated (0%) | 2001 Uniform Plumbing Code (Los Angeles, CA) | 2001 统一给排水规范（加州洛杉矶）(2001 Uniform Plumbing Code (Los Angeles, CA)) |
| 2166c523c0a7-3bb2-4d60-b719-170717f93de4 | Translated (0%) | California Public Utilities Commission, 2004–2005 Database for Energy Efficiency Resources (DEER) Update Study (2008). | 加州公共事业委员会，2004–2005 能效资源数据库 (DEER) 更新研究 (2008)(California Public Utilities Commission, 2004–2005 Database for Energy Efficiency Resources (DEER) Update Study (2008))。 |
| 21672fdb586e-c626-4a6e-9133-d3f43cad3ff7 | Translated (0%) | California State University, Capital Planning, Design and Construction Section VI, Standards for Campus Development Programs ( Long Beach, CA, 2002). | 加州州立大学，资本规划、设计和建筑第 VI 部分，校园开发计划标准（加州长滩 2002）(California State University, Capital Planning, Design and Construction Section VI, Standards for Campus Development Programs ( Long Beach, CA, 2002))。 |
| 21687dbb444c-0da2-4b75-91d8-2c5e13e7de2d | Translated (0%) | City of Boulder Planning Department, Projecting Future Employment—How Much Space per Person (Boulder, 2002). | 博尔德市规划部门，预测未来就业 - 人均空间（博尔德，2002）(City of Boulder Planning Department, Projecting Future Employment—How Much Space per Person (Boulder, 2002))。 |
| 216945a3c000-29fe-45b6-8d3c-3e50a576136c | Translated (0%) | Metro, 1999 Employment Density Study (Portland, OR 1999). | 麦德龙，1999 就业密度研究（俄勒冈州波特兰，1999）(Metro, 1999 Employment Density Study (Portland, OR 1999))。 |
| 217055dd6fce-a989-4851-b22c-fddf2afdf82e | Translated (0%) | American Hotel and Lodging Association, Lodging Industry Profile Washington, DC, 2008. | 美国酒店及住宿协会，住宿业简介，华盛顿特区，2008 (American Hotel and Lodging Association, Lodging Industry Profile Washington, DC, 2008)。 |
| 2171848e7c0e-2479-43e3-b3d3-b2b535bc5d0f | Translated (0%) | LEED for Core & Shell Core Committee, personal communication (2003 - 2006). | LEED 核心与外壳核心委员会，个人通信 (2003 - 2006)(LEED for Core & Shell Core Committee, personal communication (2003 - 2006))。 |
| 21726f819b19-01a9-4406-9f70-c358b29623d9 | Translated (70%) | LEED for Retail Core Committee, personal communication (2007) | LEED 零售核心委员会，个人通信 (2007) (LEED for Retail Core Committee, personal communication (2007))。 |
| 217378336b17-e35a-482b-98e4-0c0bad65351c | Translated (0%) | OWP/P, Medical Office Building Project Averages (Chicago, 2008). | OWP/P，医疗办公大楼项目平均值（芝加哥，2008）(OWP/P, Medical Office Building Project Averages (Chicago, 2008))。 |
| 2174eda61316-50bb-46f9-a3ad-50c559020b16 | Translated (0%) | OWP/P, University Master Plan Projects (Chicago, 2008). | OWP/P，大学总体规划项目（芝加哥，2008）(OWP/P, University Master Plan Projects (Chicago, 2008))。 |
| 21750aae44a8-47de-4ff3-99ec-c02ed602e641 | Translated (0%) | U.S. General Services Administration, Childcare Center Design Guide (Washington, DC,2003). | 美国总务管理局，儿童看护中心设计指南（华盛顿特区，2003）(U.S. General Services Administration, Childcare Center Design Guide (Washington, DC,2003))。 |
| 217616915fc1-0103-43b7-b1ef-30ac3cdd3779 | Translated (100%) | Appendix 3. | 附录 3 |
| 217716915fc1-0103-43b7-b1ef-30ac3cdd3779 | Translated (0%) | Retail Process Load Baselines | 零售过程负荷基线 |
| 2178f8fc5cde-41ba-4f9e-9509-23ec6393cebd | Translated (100%) | Table 1a. | 表 1a. |
| 2179f8fc5cde-41ba-4f9e-9509-23ec6393cebd | Translated (0%) | Commercial kitchen appliance prescriptive measures and baseline for energy cost budget (IP units) | 商用厨房电器的规范性指标和能源费用预算基线（IP 单位） |
| 21802642ab8f-a783-443b-84ff-87c84e906407 | Translated (0%) | Baseline energy usage for energy modeling path | 能耗模拟途径的基线能耗 |
| 21813438a2ac-28cd-4afc-8727-cf9c46823fcb | Translated (0%) | Levels for prescriptive path | 规范性途径的等级 |
| 2182b0728618-2820-4590-a1e1-7e5f2320de6c | Translated (0%) | Appliance type | 电器类型 |
| 2183a94a5f96-7bd5-4786-a807-1060595a1aa6 | Translated (100%) | Fuel | 燃料 |
| 2184488cd2f7-0537-4e54-8afd-0e96ab023d42 | Translated (0%) | Function | 功能 |
| 2185c1a2484e-fd26-404b-846d-87a08dbfc3b4 | Translated (0%) | Baseline efficiency | 基线效率 |
| 2186f22c5cd4-8be2-4285-a873-08ffd17e2cd7 | Translated (0%) | Baseline idle rate | 基线闲时功率 |
| 21871781fba8-d630-460c-ab6f-fa52a4ad4553 | Translated (0%) | Prescriptive efficiency | 规范性效率 |
| 2188996678ca-c5c6-4b33-a584-4afa3de26734 | Translated (77%) | Prescriptive idle rate | 规范性闲时功率 |
| 21898f19edcb-8565-4812-9be0-b44be2b0cf7d | Translated (0%) | Broiler, underfired | 底燃式锅炉 |
| 2190c6e54047-bcdd-4fb4-97c3-a8946e15d12a | Translated (0%) | Gas | 天然气 |
| 2191df61714e-2d49-4c57-b308-1f8b68572f7e | Translated (100%) | Cooking | 烹饪 |
| 219274827842-0206-4d97-9577-ff8d1fc69d9a | Translated (100%) | 30% | 30% |
| 2193db855584-1cbf-4298-92c1-231f838a2071 | Translated (0%) | 16,000 Btu/h/ft<3117>2</3117> peak input | 16,000 Btu/h/ft<3117>2</3117> 峰值输入 |
| 2194055ec11e-3f24-4b42-92c5-a64892b15b4d | Translated (100%) | 35% | 35% |
| 21953b8ad93d-dceb-426a-a442-179cf770e829 | Translated (100%) | 12,000 Btu/h/ft<3124>2</3124> peak input | 12,000 Btu/h/ft<3124>2</3124> 峰值输入 |
| 21960e3a07dc-9b97-46d6-9ca7-c170917b2295 | Translated (0%) | Combination ovens, steam mode (P = pan capacity) | 组合烤箱，蒸汽模式（P = 盘容量） |
| 2197382cb1f6-751c-492d-bd3c-0353874b4198 | Translated (0%) | Elec | 电力 |
| 21980feead49-47a7-418b-b575-a29d1f9d016b | Translated (100%) | Cooking | 烹饪 |
| 2199db1b80cf-3a2f-47ba-81d1-1522016e6ec5 | Translated (0%) | 40% steam mode | 40% 蒸汽模式 |
| 22008671545e-d825-4370-a2fc-de5ccae960e7 | Translated (0%) | 0.37P+4.5 kW | 0.37P+4.5 kW |
| 22016e5bb050-c0c3-4254-ab34-65db3524873d | Translated (100%) | 50% steam mode | 50% 蒸汽模式 |
| 22024d0e3e01-2b6a-4c49-bb70-85b66bc328ab | Translated (0%) | 0.133P+0.6400 kW | 0.133P+0.6400 kW |
| 2203646592fd-61d8-402b-bc8a-d96ac6fd0648 | Translated (0%) | Combination ovens, steam mode | 组合烤箱，蒸汽模式 |
| 22049253201b-672d-48df-987c-3e0950417199 | Translated (100%) | Gas | 天然气 |
| 2205455927ce-9f8f-4af7-a690-a4e13d17dfcf | Translated (100%) | Cooking | 烹饪 |
| 2206d41730a4-01f7-43a6-ac71-c515319b1f33 | Translated (100%) | 20% steam mode | 20% 蒸汽模式 |
| 2207529b0b6f-f3a8-4a2a-8a37-7067097b6f01 | Translated (0%) | 1,210P+35,810 Btu/h | 1,210P+35,810 Btu/h |
| 220808d75e5f-c599-46f4-92f2-86089230c7dc | Translated (100%) | 38% steam mode | 38% 蒸汽模式 |
| 22099845342d-577e-44b5-94cd-487f6708a179 | Translated (0%) | 200P+6,511 Btu/h | 200P+6,511 Btu/h |
| 2210d088101b-988b-4b3a-88c2-a1bb35c750e4 | Translated (83%) | Combination ovens, convection mode | 组合烤箱，对流模式 |
| 2211c3300e24-c681-4844-8213-2106d7e4f9fc | Translated (100%) | Elec | 电力 |
| 2212abc925b9-468e-4d94-aaea-6e07b7264d1a | Translated (100%) | Cooking | 烹饪 |
| 22133273a313-4825-408e-8ff6-5752ae0eaeb5 | Translated (0%) | 65% convection mode | 65% 对流模式 |
| 2214cfa192b0-c0ea-47c6-b361-0558e71bd812 | Translated (0%) | 0.1P+1.5 kW | 0.1P+1.5 kW |
| 2215a63228fc-c0e8-40f7-b322-0d0e9cdcc8e8 | Translated (100%) | 70% convection mode | 70% 对流模式 |
| 2216c4f02c84-3385-4e57-b6d2-34cbf20c08a7 | Translated (0%) | 0.080P+0.4989 kW | 0.080P+0.4989 kW |
| 2217e91c7bd7-3bbb-43e3-981a-9d78b11e5014 | Translated (CM) | Combination ovens, convection mode | 组合烤箱，对流模式 |
| 22189bce3ef9-1a86-4915-b689-1633c4c0eaf0 | Translated (100%) | Gas | 天然气 |
| 2219549698cb-6709-4be8-a717-bffa5c893bfc | Translated (100%) | Cooking | 烹饪 |
| 2220da32c086-8be5-4f7f-a429-f8dc461558af | Translated (100%) | 35% convection mode | 35% 对流模式 |
| 22211429625c-399b-469f-9cd1-07f19278b318 | Translated (0%) | 322P+13,563 Btu/h | 322P+13,563 Btu/h |
| 2222bfcd204e-47fd-410d-8df3-1a7554ba2357 | Translated (100%) | 44% convection mode | 44% 对流模式 |
| 22234f3de291-275e-4021-b75b-616715f983f3 | Translated (0%) | 150P+5,425 Btu/h | 150P+5,425 Btu/h |
| 2224ca819223-29cd-47b5-a439-c7a188a83456 | Translated (0%) | Convection oven, full-size | 对流烤箱，全尺寸 |
| 22256e8d332e-2adf-4b7f-952a-4e05b1787867 | Translated (100%) | Elec | 电力 |
| 2226658da2e0-df5c-4e4e-a081-2583a6d0010c | Translated (100%) | Cooking | 烹饪 |
| 222792b25c5e-1614-428c-8791-344660eefe5d | Translated (100%) | 65% | 65% |
| 2228047a8a56-ac39-4f95-af78-d29ca4517834 | Translated (100%) | 2.0 kW | 2.0 kW |
| 2229a4258cf8-8177-43d6-a8a5-b63906dde293 | Translated (100%) | 71% | 71% |
| 22300526c567-3f0a-40d8-8292-5b02572ee812 | Translated (100%) | 1.6 kW | 1.6 kW |
| 223152b5dfb1-20df-4f45-8049-411cc55d9a09 | Translated (100%) | Convection oven, full-size | 对流烤箱，全尺寸 |
| 22329c8d3418-7334-43ba-be83-546d430ed785 | Translated (100%) | Gas | 天然气 |
| 2233e370c7c2-f313-46f3-9af4-3c697957909e | Translated (100%) | Cooking | 烹饪 |
| 22343dfea01e-99d1-415a-ae46-615287f9d859 | Translated (100%) | 30% | 30% |
| 2235d6700284-9f95-42bf-b56d-1619741d0ec7 | Translated (0%) | 18,000 Btu/h | 18,000 Btu/h |
| 2236a2738753-f9bc-4ae6-b822-9def4eed1428 | Translated (100%) | 46% | 46% |
| 2237bb93d73a-1996-470f-b1eb-12ed7009af81 | Translated (100%) | 12,000 Btu/h | 12,000 Btu/h |
| 223846ed564f-ef29-4956-8587-ffbeb53563ac | Translated (78%) | Convection oven, half-size | 对流烤箱，半尺寸 |
| 2239cba04903-ab14-4e83-a7a2-65e1b8860169 | Translated (100%) | Elec | 电力 |
| 2240db04b348-99bf-4b6d-a5c9-baa54cbea478 | Translated (100%) | Cooking | 烹饪 |
| 224146366788-0f92-47d5-acf6-b0c9332c9604 | Translated (100%) | 65% | 65% |
| 22425b9e6da2-4d40-4110-9158-af289ba62ddf | Translated (100%) | 1.5 kW | 1.5 kW |
| 2243601fc516-c759-44f6-8728-05617d2dbf5c | Translated (CM) | 71% | 71% |
| 22441a47e8ff-59bf-49c4-b723-3d21ce3b6cf2 | Translated (100%) | 1.0 kW | 1.0 kW |
| 224572b0efad-ba1d-4c97-b590-8b56435cfcc7 | Translated (0%) | Conveyor oven, > 25-inch belt | 链条式平炉，> 25 英寸传送带 |
| 22460da53743-0604-4208-b76e-833aa0895775 | Translated (100%) | Gas | 天然气 |
| 22474b260b30-fb3c-4ac5-8286-3c3aa45bf9fa | Translated (100%) | Cooking | 烹饪 |
| 224883f901a9-74a3-4c3b-9162-999a80329925 | Translated (100%) | 20% | 20% |
| 2249e96a3fd0-c78a-4edb-8505-05d8462da227 | Translated (CM) | 70,000 Btu/h | 70,000 Btu/h |
| 22506a9e8268-d58f-4798-97a3-119d2b2316eb | Translated (CM) | 42% | 42% |
| 225128061acb-56d9-4e31-a2d9-1678f349f65c | Translated (CM) | 57,000 Btu/h | 57,000 Btu/h |
| 2252f03d9729-6b7c-41bd-9efb-9bf06c203bd1 | Translated (99%) | Conveyor oven, ≤ 25-inch belt | 链条式平炉，≤ 25 英寸传送带 |
| 2253687b2522-f695-4c3b-8f2d-0d0775ba27b0 | Translated (100%) | Gas | 天然气 |
| 225446e4af73-212c-4908-993e-2ffb8f9c9961 | Translated (100%) | Cooking | 烹饪 |
| 22555ab0c21d-373b-4dc6-9bbc-307fd8334bbc | Translated (100%) | 20% | 20% |
| 22564abf3fa6-9689-428d-9324-6fd0dcb81214 | Translated (100%) | 45,000 Btu/h | 45,000 Btu/h |
| 2257a1dea4af-8010-4dd1-9193-7876b2e66bf8 | Translated (100%) | 42% | 42% |
| 22580ec1d08b-f1f5-4e0e-a2b7-32182cff055d | Translated (CM) | 29,000 Btu/h | 29,000 Btu/h |
| 2259bc3b45bb-2f1a-4207-966a-ae5e56085c66 | Translated (0%) | Fryer | 炸锅 |
| 22602be1b66d-e716-4bf9-aaae-0d830910f086 | Translated (100%) | Elec | 电力 |
| 2261f01cb27a-c36e-40b3-b107-955ac7447823 | Translated (100%) | Cooking | 烹饪 |
| 2262fbfc07be-f9f5-42e4-b7c8-8e0035b6fcae | Translated (100%) | 75% | 75% |
| 22634fb25aee-4f70-4123-b92a-674729cd3ec5 | Translated (100%) | 1.05 kW | 1.05 kW |
| 226444f29d94-c200-4235-a6cc-56c354450412 | Translated (CM) | 80% | 80% |
| 226542a9c0e9-83fb-4380-800a-6a38308b21f7 | Translated (100%) | 1.0 kW | 1.0 kW |
| 22663bd5a6ab-50a7-42d6-a94f-77cf64dd3821 | Translated (100%) | Fryer | 炸锅 |
| 226744ff1dfe-ea13-4fa5-bf15-e60be4cf13ad | Translated (100%) | Gas | 天然气 |
| 2268f2512223-eb35-4905-ad72-5fce40a9a900 | Translated (100%) | Cooking | 烹饪 |
| 226912d5b15a-5b56-4873-9e78-43e259990f75 | Translated (100%) | 35% | 35% |
| 22704cdb7b80-8347-4b17-a308-29ab3010e224 | Translated (100%) | 14,000 Btu/h | 14,000 Btu/h |
| 22712bd957d7-bcf1-41b2-9b7c-4aeef518042c | Translated (100%) | 50% | 50% |
| 2272bcd3955e-f116-4a56-8ce5-f2a342c78ec9 | Translated (100%) | 9,000 Btu/h | 9,000 Btu/h |
| 2273adfc9f00-4995-44ba-b7ed-ca35de5304b7 | Translated (0%) | Griddle (based on 3 ft model) | 煎锅（基于 3 ft 模型） |
| 227426d3b873-29ad-4953-a586-df6997ac9dc7 | Translated (100%) | Elec | 电力 |
| 2275fd034fc1-0bee-4712-86f1-e497033e0971 | Translated (100%) | Cooking | 烹饪 |
| 22767c605ba8-de16-480b-9cf7-d1fe7c5fcdbe | Translated (100%) | 60% | 60% |
| 2277e1bd11d0-bb14-47b2-9114-baa9e4fdf0f2 | Translated (100%) | 400 W/ft<3371>2</3371> | 400 W/ft<3371>2</3371> |
| 2278e362933d-d531-4fbc-ac89-705a8eedfdcd | Translated (100%) | 70% | 70% |
| 2279723a5248-3480-418d-87f4-332d3e38952e | Translated (100%) | 320 W/ft<3378>2</3378> | 320 W/ft<3378>2</3378> |
| 22800850dcf9-122a-4b52-8104-eccc75e034e7 | Translated (100%) | Griddle (based on 3 ft model) | 煎锅（基于 3 ft 模型） |
| 2281a628b6ca-2e55-4ff3-b3df-0cbe14b6600b | Translated (100%) | Gas | 天然气 |
| 22827e292e50-d06c-4678-8e2f-28e4b8d68590 | Translated (100%) | Cooking | 烹饪 |
| 2283e1fa8fc2-9ffc-41c4-91c9-6cdefafe3d34 | Translated (100%) | 30% | 30% |
| 228468591f72-8333-4770-bf95-1831cf776c05 | Translated (0%) | 3,500 Btu/h/ft<3394>2</3394> | 3,500 Btu/h/ft<3394>2</3394> |
| 22854c27bcfe-28f3-4a02-9a38-51e2f2c01673 | Translated (100%) | 38% | 38% |
| 228678edcc28-3911-4434-9247-30219bcb294b | Translated (100%) | 2,650 Btu/h/ft<3401>2</3401> | 2,650 Btu/h/ft<3401>2</3401> |
| 228742d67a40-6f95-4269-bb59-31d3c4a02938 | Translated (0%) | Hot food holding cabinets (excluding drawer warmers and heated display), 0 < V < 13 ft³ (V = volume) | 热食物储藏柜（不包括抽屉式加热器和加热显示器），0 < V < 13 ft³ (V = 体积) |
| 2288b7e67835-d615-455d-a7bf-4995c087bef7 | Translated (100%) | Elec | 电力 |
| 2289074fa8f7-f428-41db-a639-f51ee6291685 | Translated (100%) | Cooking | 烹饪 |
| 229044805a93-7060-406e-87b7-4e084caa3d96 | Translated (0%) | na | na |
| 2291f5b111bf-c0e1-4206-93ac-596a62efeb5b | Translated (100%) | 40 W/ft<3417>3</3417> | 40 W/ft<3417>3</3417> |
| 229297b17a44-0edf-4ed1-b6d0-83c5145f5d01 | Translated (100%) | na | na |
| 22935dd72f02-6ecc-486a-b127-87acab1f1be0 | Translated (0%) | 21.5V Watts | 21.5V 瓦特 |
| 229480f0068f-ad37-41e6-96ec-1045840ee0eb | Translated (90%) | Hot food holding cabinets (excluding drawer warmers and heated display), 13 ≤ V < 28 ft³ | 热食物储藏柜（不包括抽屉式加热器和加热显示器），13 ≤ V < 28 ft³ |
| 2295528b7bdf-8be2-4922-95b5-7302251cf712 | Translated (100%) | Elec | 电力 |
| 2296b336e1b5-3acf-4f99-b3eb-957b9f052f95 | Translated (100%) | Cooking | 烹饪 |
| 2297a111e56b-dc66-43a3-8e15-c7d39dd1201a | Translated (CM) | na | na |
| 22987f6ca541-49d0-499e-b067-6c7c2d170ebe | Translated (0%) | 40 W/ft<3439>3</3439> | 40 W/ft<3439>3</3439> |
| 22992bde0ff6-9e5a-475e-9c14-ab0738017e59 | Translated (100%) | na | na |
| 2300821260ab-f68c-4c97-ba36-037da853d1e0 | Translated (0%) | 2.0V + 254 Watts | 2.0V + 254 瓦特 |
| 2301b40c4cd5-1986-4fe6-a923-b64193ef4b5b | Translated (85%) | Hot food holding cabinets (excluding drawer warmers and heated display), 28 ft³ ≤ V | 热食物储藏柜（不包括抽屉式加热器和加热显示器），28 ft³ ≤ V |
| 23027a1e40e2-2d3c-41a8-a8fe-7d0a6e8d30b7 | Translated (100%) | Elec | 电力 |
| 2303c07eeec5-ef84-4644-abaf-b28d27a9af3f | Translated (100%) | Cooking | 烹饪 |
| 23048aee6f40-2d0d-4626-8dae-840586a4d573 | Translated (CM) | na | na |
| 2305a039dfa1-8919-4b1d-8a66-eae9ebbfba89 | Translated (100%) | 40 W/ft<3461>3</3461> | 40 W/ft<3461>3</3461> |
| 2306ec361dfd-f435-4fac-821b-f2bc31c86b78 | Translated (CM) | na | na |
| 2307fa5e8b18-c530-4957-842e-d72698b1ad5d | Translated (CM) | 3.8V + 203.5 Watts | 3.8V + 203.5 瓦特 |
| 23088e37269a-8617-45de-bf18-2f2b6585af84 | Translated (0%) | Large vat fryer | 大桶炸锅 |
| 2309415e86aa-36ca-438f-bce9-8bd39465a08d | Translated (100%) | Elec | 电力 |
| 2310c5fd328f-4f1c-419a-ad1a-992511d426a5 | Translated (100%) | Cooking | 烹饪 |
| 2311f7229ec6-0536-4b7f-8d2c-62e6a73ecbd3 | Translated (CM) | 75% | 75% |
| 2312ccb0119a-0ea6-47f9-b6c7-f76e32d7333d | Translated (100%) | 1.35 kW | 1.35 kW |
| 23130ef36868-066b-4044-8ad0-ecded76a178b | Translated (CM) | 80% | 80% |
| 2314c184b56e-e5e4-4bbf-a9bd-4b40c26efb0e | Translated (CM) | 1.1 kW | 1.1 kW |
| 2315a1b1ea0e-2ad5-4d66-94b4-1cf766fad857 | Translated (100%) | Large vat fryer | 大桶炸锅 |
| 2316c511c548-f6d4-490a-8f5e-5b12805a3b0b | Translated (100%) | Gas | 天然气 |
| 23171d636e21-1b10-4add-91e2-ba58f8c5c392 | Translated (100%) | Cooking | 烹饪 |
| 23182ba7093c-2ef3-4e3e-99a4-f244c95f6792 | Translated (CM) | 35% | 35% |
| 231973b1f6ec-531b-41e9-97a6-001775ebeed2 | Translated (CM) | 20,000 Btu/h | 20,000 Btu/h |
| 2320dcfe1683-04de-4a41-af69-237a580ba5bc | Translated (CM) | 50% | 50% |
| 2321a3308562-7b6c-4e61-b6a2-a6e6171fbcf3 | Translated (CM) | 12,000 Btu/h | 12,000 Btu/h |
| 232219701ac1-af4a-4c2f-9ba9-2a3ceff49ed3 | Translated (0%) | Rack oven, double | 搁架式烤炉，双 |
| 232377ea04ab-de91-4045-ab4e-71aecd031bae | Translated (100%) | Gas | 天然气 |
| 2324b30e6758-23f9-447d-a8d1-5897087a45d6 | Translated (100%) | Cooking | 烹饪 |
| 232517d5b815-f50b-417d-bbdc-9ffec0f3060a | Translated (CM) | 30% | 30% |
| 2326a3ad39d9-2722-4ff1-a0c7-441e4e3a29d0 | Translated (CM) | 65,000 Btu/h | 65,000 Btu/h |
| 2327a61786c5-f443-429c-b36d-0fc682b3d20e | Translated (CM) | 50% | 50% |
| 232833786ed7-c1b7-4797-9348-05252a9de869 | Translated (CM) | 35,000 Btu/h | 35,000 Btu/h |
| 2329251b064b-2f36-43d5-b256-c5233f13f1e2 | Translated (78%) | Rack oven, single | 搁架式烤炉，单 |
| 2330bf012254-2eae-459b-977d-6a942b02ca6e | Translated (100%) | Gas | 天然气 |
| 233137a9ea7c-3690-4b73-a6e2-f9f122fba116 | Translated (100%) | Cooking | 烹饪 |
| 2332c3e17173-b413-4de9-8bfb-117c775fe6dc | Translated (CM) | 30% | 30% |
| 2333c59580f3-1bce-45dc-bb2a-189a01e36c28 | Translated (CM) | 43,000 Btu/h | 43,000 Btu/h |
| 23348052bb2f-0eb5-42bb-9952-8dc75317ea69 | Translated (CM) | 50% | 50% |
| 2335d64e85bf-9ed2-4d26-ab30-580b9b0272c5 | Translated (CM) | 29,000 Btu/h | 29,000 Btu/h |
| 2336ab22488b-93b4-48c5-b633-f9f4167457bd | Translated (0%) | Range | 炉灶 |
| 2337047e22ba-ae3b-4abf-99a0-ea7571f79e03 | Translated (100%) | Elec | 电力 |
| 23386a79b8a8-e2ac-4cc9-8f85-b409a330d89a | Translated (100%) | Cooking | 烹饪 |
| 23392342fad8-ae0a-488b-b53c-e0a1e6dcf683 | Translated (CM) | 70% | 70% |
| 234035776f78-6ab3-48b1-95e0-d25b0c272aac | Translated (CM) | 80% | 80% |
| 2341c7312df8-6aab-48bd-bbb3-69242d611d64 | Translated (100%) | Range | 炉灶 |
| 23420241477d-0e33-4b71-ae3e-6826a78a16f8 | Translated (100%) | Gas | 天然气 |
| 2343f7d40382-1c5f-43a9-911c-6749aaae0392 | Translated (100%) | Cooking | 烹饪 |
| 2344406fdfad-4298-4615-987a-4598e0bab5d2 | Translated (CM) | 35% | 35% |
| 2345d9df7936-56b3-44fa-a2c4-c4792cd5191c | Translated (CM) | na | na |
| 23464cb93717-c5f6-41b5-a920-753d25f29882 | Translated (0%) | 40% and no standing pilots | 40% 且无竖直引燃器 |
| 23472a7eaedf-b16a-48b3-b346-0eeb94ae9193 | Translated (100%) | na | na |
| 2348bc28760d-37ee-4da4-8927-5e9a5e3da5fd | Translated (0%) | Steam cooker, batch cooking | 蒸汽锅，批量烹饪 |
| 23498e1a39ad-f9db-4a92-9649-ee9c9e4d4227 | Translated (100%) | Elec | 电力 |
| 2350e9fd74c8-0dca-45a5-a2a1-6715c3e49fca | Translated (100%) | Cooking | 烹饪 |
| 2351f5869210-1837-40d7-b912-31ffb228e74b | Translated (CM) | 26% | 26% |
| 2352a19b2a9c-f66d-44c4-8832-b7dae774caf6 | Translated (0%) | 200 W/pan | 200 W/盘 |
| 2353853f1658-e537-4ceb-ba39-1daab8b90710 | Translated (100%) | 50% | 50% |
| 2354197bd068-edcf-4efd-8824-2f9d7c0cb3c5 | Translated (CM) | 135 W/pan | 135 W/盘 |
| 23559c96b31a-e073-46a4-9627-ed93544d60eb | Translated (100%) | Steam cooker, batch cooking | 蒸汽锅，批量烹饪 |
| 2356c894f0c3-0b2c-444f-a9e7-4ef94289f28b | Translated (100%) | Gas | 天然气 |
| 23574dd2a9f5-769f-4312-a1a8-4cab57fa18cb | Translated (100%) | Cooking | 烹饪 |
| 2358dd39d489-dcce-4ae8-afc4-606d44cd5ef8 | Translated (CM) | 15% | 15% |
| 23593491c6ec-51be-4c04-bfe5-bf47b48d30dd | Translated (0%) | 2,500 Btu/h/pan | 2,500 Btu/h/盘 |
| 236011b6d9d0-3be2-427d-be2d-2b65168297d5 | Translated (100%) | 38% | 38% |
| 236132160855-6833-4572-ada0-7c923a32daba | Translated (CM) | 2,100 Btu/h/pan | 2,100 Btu/h/盘 |
| 23626b7c5113-e5ee-4de1-82c8-4cdb4f2a70ab | Translated (0%) | Steam cooker, high production or cook to order | 蒸汽锅，高产量或按订单烹饪 |
| 23630c329318-b122-4f7e-afad-adc0a3a03eb4 | Translated (100%) | Elec | 电力 |
| 23644cd862d7-90c9-4cfd-808c-ec36a055ebef | Translated (100%) | Cooking | 烹饪 |
| 236523404591-200a-4ca9-bab0-5a51441b0d1c | Translated (CM) | 26% | 26% |
| 2366f08b1b8a-e578-4e2d-af3e-cf98f13ff617 | Translated (CM) | 330 W/pan | 330 W/盘 |
| 236717344767-1769-4cef-9be4-67201194bab1 | Translated (CM) | 50% | 50% |
| 23683108bd9f-b1ff-4870-9d2f-ea1142b3ade2 | Translated (CM) | 275 W/pan | 275 W/盘 |
| 236993353fb7-dd62-42ec-b31a-1bed84ff9dcf | Translated (100%) | Steam cooker, high production or cook to order | 蒸汽锅，高产量或按订单烹饪 |
| 237095851bf0-e6ff-48a3-a669-3749ddc15343 | Translated (100%) | Gas | 天然气 |
| 23714748a188-22bc-40fc-8a75-1067ff9dbc44 | Translated (100%) | Cooking | 烹饪 |
| 2372a52699e0-1def-4307-855c-8da73879349f | Translated (CM) | 15% | 15% |
| 2373c486204c-a0fb-4608-91ef-027cb4878022 | Translated (CM) | 5,000 Btu/h/pan | 5,000 Btu/h/盘 |
| 2374815978c7-82a0-40ff-8c3a-ca58f26520c9 | Translated (CM) | 38% | 38% |
| 23753dfca6b0-0427-4e4a-8fd2-155e7db79453 | Translated (CM) | 4,300 Btu/h/pan | 4,300 Btu/h/盘 |
| 23764a796d6f-9900-4f3b-bfb9-77958da71dd3 | Translated (0%) | Toaster | 烤面包机 |
| 2377d17254ef-321d-484c-823d-97b77a580064 | Translated (100%) | Elec | 电力 |
| 2378f7910cfe-aa97-4a55-a061-0cec46d782d0 | Translated (100%) | Cooking | 烹饪 |
| 2379b4914769-b1ef-4ac9-9714-4d689f225e59 | Translated (100%) | — | — |
| 23807ec32adb-6d84-4cce-a697-5cf5ef1204d2 | Translated (0%) | 1.8 kW average operating energy rate | 1.8 kW 平均运行功率 |
| 23812bafab8b-92b7-4f12-9e1b-018321d438a2 | Translated (100%) | na | na |
| 2382ff4a2356-328a-49c7-95e0-628935aa067f | Translated (100%) | 1.2 kW average operating energy rate | 1.2 kW 平均运行功率 |
| 2383b6a45cf6-042f-415c-b470-c06798bc6ab8 | Translated (0%) | Ice machine, IMH (ice-making head, H = ice harvest), H <3702>></3702> 450 lb/day | 制冰机，IMH（制冰头，H = 产冰量），H <3702>></3702> 450 lb/天 |
| 23848eae45ce-11c5-4887-a5e2-928776564162 | Translated (100%) | Elec | 电力 |
| 23858d93a62d-fb7f-4792-a4a8-557438f203cd | Translated (0%) | Ice | 制冰 |
| 23866469639e-978d-4b6e-a905-4a798f15bdee | Translated (0%) | 6.89 - 0.0011H kWh/100 lb ice | 6.89 - 0.0011H kWh/100 lb 冰 |
| 238769c02368-5cf4-4a10-989f-a2ea4a94d6ec | Translated (100%) | na | na |
| 238825442a3f-1f72-40df-b263-31130191859d | Translated (0%) | 37.72*H<3718>-0.298 </3718>kWh/100 lb ice | 37.72*H<3718>-0.298 </3718>kWh/100 lb 冰 |
| 2389dc8f554f-e65e-4cf8-91e2-7cbc4556c36d | Translated (100%) | na | na |
| 23906453fcf6-05a4-4516-881f-3427b58b2734 | Translated (75%) | Ice machine, IMH (ice-making head), H <3725>< </3725>450 lb/day | 制冰机，IMH（制冰头），H <3725>< </3725>450 lb/天 |
| 2391da4a8787-5fd2-4cc0-b537-011d87553be9 | Translated (100%) | Elec | 电力 |
| 239232e1c01b-4d0f-42e2-bc4a-3af5164c4781 | Translated (CM) | Ice | 制冰 |
| 2393f230b9ac-2088-47ad-aa9e-4a42f143a709 | Translated (0%) | 10.26 – 0.0086H kWh/100 lb ice | 10.26 – 0.0086H kWh/100 lb 冰 |
| 2394abfc2ba0-3abb-4749-a97c-cf9db935249a | Translated (100%) | na | na |
| 2395c8c64701-a93b-4071-8c7e-971515780bef | Translated (CM) | 37.72*H<3741>-0.298 </3741>kWh/100 lb ice | 37.72*H<3741>-0.298 </3741>kWh/100 lb 冰 |
| 23962598c7f2-4049-4787-9c13-0a171321729e | Translated (CM) | na | na |
| 2397174dce4f-9fff-4232-9805-c2541da76a9f | Translated (0%) | Ice machine, RCU (remote condensing unit, w/o remote compressor, H < 1,000 lb/day | 制冰机，RCU（远程冷凝装置，带/不带远程压缩机，H < 1,000 lb/天 |
| 2398c2cb468c-e709-4f9b-ac6b-baa6d5b142a8 | Translated (100%) | Elec | 电力 |
| 2399ecdaf4c8-e375-4fb4-b2a3-3074e9c8ca9c | Translated (CM) | Ice | 制冰 |
| 2400c2c979ab-5680-4f3a-aa9b-3a60380fc031 | Translated (91%) | 8.85 - 0.0038H kWh/100lb ice | 8.85 - 0.0038H kWh/100 lb 冰 |
| 2401fcc5ab7b-9d1a-415c-9394-167cff00b5a2 | Translated (100%) | na | na |
| 24022a83be04-f7a6-42bd-a640-4d9f27524c87 | Translated (0%) | 22.95*H<3763>-0.258 </3763>+ 1.00 kWh/100 lb ice | 22.95*H<3763>-0.258 </3763>+ 1.00 kWh/100 lb 冰 |
| 2403b3c3824f-46e0-4aa8-b380-d2b939fa7908 | Translated (100%) | na | na |
| 2404963f5b0d-70d1-47f9-af79-e0ba25199887 | Translated (75%) | Ice machine, RCU (remote condensing unit), 1600 > H <3770>></3770> 1000 lb/day | 制冰机，RCU（远程冷凝装置），1600 > H <3770>></3770> 1000 lb/天 |
| 2405bbde376f-a2d3-4429-8346-2fe39017e3a9 | Translated (100%) | Elec | 电力 |
| 2406a587b810-12a2-40e3-9af9-cb8392691bd1 | Translated (90%) | ice | 制冰 |
| 240737659af7-9fcb-4eb8-a408-76a32a28a8e7 | Translated (0%) | 5.10 kWh/100 lb ice | 5.10 kWh/100 lb 冰 |
| 240853b795a6-e62e-4eeb-99ff-8a0b7e82446f | Translated (100%) | na | na |
| 240928969178-12de-4c06-844d-6a2c1a9c5d25 | Translated (0%) | 22.95*H<3786>-0.258</3786> + 1.00 kWh/100 lb ice | 22.95*H<3786>-0.258</3786> + 1.00 kWh/100 lb 冰 |
| 24102def7aac-ed58-4bca-825f-d1479531c7b6 | Translated (CM) | na | na |
| 24112eab841b-7fe0-4a73-a03a-75e70c74da44 | Translated (0%) | Ice machine, RCU (remote condensing unit), H ≥ 1600 lb/day | 制冰机，RCU（远程冷凝装置），H ≥ 1600 lb/天 |
| 24123e7e7ad9-5516-40db-b202-8ca14ef7c72f | Translated (100%) | Elec | 电力 |
| 2413b2adaa99-22f9-4fce-a985-ad6ea5268eb8 | Translated (CM) | Ice | 制冰 |
| 24146845ef4e-b99b-498e-99e4-7d1e5d2ad2f7 | Translated (CM) | 5.10 kWh/100lb ice | 5.10 kWh/100 lb 冰 |
| 241575a5921a-b490-4c6c-aaa1-4e99c275d453 | Translated (CM) | na | na |
| 24162860e8cd-4535-4027-a1b9-bcf01496281b | Translated (0%) | -0.00011*H + 4.60 kWh/100 lb ice | -0.00011*H + 4.60 kWh/100 lb 冰 |
| 2417bff72f1b-312c-430e-abfa-f0676ad02c3f | Translated (CM) | na | na |
| 2418dbe13f0a-20a5-414a-8731-6529bef9d586 | Translated (73%) | Ice machine, SCU (self-contained unit), H < 175 lb/day | 制冰机，SCU（独立装置），H < 175 lb/天 |
| 24199f71be11-c254-4158-8a26-27d28e4c9d0a | Translated (100%) | Elec | 电力 |
| 2420292ae7f2-f1f6-477d-8c21-f75f8fe69a1f | Translated (CM) | Ice | 制冰 |
| 2421803691ae-662d-4fb8-88df-35ad050ce99e | Translated (0%) | 18.0 - 0.0469H kWh/100lb ice | 18.0 - 0.0469H kWh/100 lb 冰 |
| 2422b926ad76-76a8-4586-92de-ce3cfd7564fa | Translated (100%) | na | na |
| 2423f784546e-2eb5-424b-98e3-27443d2c0a2d | Translated (CM) | 48.66*H<3829>-0.326</3829> + 0.08 kWh/100 lb ice | 48.66*H<3829>-0.326</3829> + 0.08 kWh/100 lb 冰 |
| 2424bd1122b2-e84e-4ad3-813a-8d10b504d657 | Translated (CM) | na | na |
| 2425da51ff3c-6381-45f7-baff-27577ec7256a | Translated (0%) | Ice machine self-contained unit, H <3836>></3836> 175 lb/day | 制冰机独立装置，H <3836>></3836> 175 lb/天 |
| 2426656723c9-57a7-44e1-ad13-c087cff27c24 | Translated (100%) | Elec | 电力 |
| 2427a189f3b5-d3b2-4646-910b-bc64fed96949 | Translated (CM) | Ice | 制冰 |
| 2428f5ff84b0-6b49-4853-8bd9-5ace6f0153d1 | Translated (CM) | 9.80 kWh/100 lb ice | 9.80 kWh/100 lb 冰 |
| 2429b660d583-e63f-4a29-82f2-fe5dc658ca91 | Translated (CM) | na | na |
| 24305d0f98fd-eda1-40b4-9748-a4845259d573 | Translated (CM) | 48.66*H<3852>-0.326</3852> + 0.08 kWh/100 lb ice | 48.66*H<3852>-0.326</3852> + 0.08 kWh/100 lb 冰 |
| 2431030555c6-5932-42ac-93fb-7b0db27f02ea | Translated (CM) | na | na |
| 24326ac51efb-cced-4dc5-83b7-1f98abc125a1 | Translated (0%) | Ice machine, water-cooled ice-making head, H <3859>></3859> 1436 lb/day (must be on chilled loop) | 制冰机，水冷制冰头，H <3859>></3859> 1436 lb/天（必须在冷却回路上） |
| 24330ca262ff-f2e0-4c1d-a411-5132b3acef3e | Translated (100%) | Elec | 电力 |
| 24345d5737a1-6be6-42d4-b542-33974f6e973e | Translated (CM) | Ice | 制冰 |
| 2435092d6c17-c930-438c-8c90-de87932ad123 | Translated (CM) | 4.0 kWh/100 lb ice | 4.0 kWh/100 lb 冰 |
| 2436de9323bc-35d4-4313-acc5-45f250f75762 | Translated (CM) | na | na |
| 243795d74ce8-ebbb-4921-82a1-40ff9a1e9e95 | Translated (100%) | 3.68 kWh/100 lb ice | 3.68 kWh/100 lb 冰 |
| 2438a883f416-c1c2-4b89-ae88-b6fad868beb4 | Translated (CM) | na | na |
| 2439a05c733b-dc79-4f34-b0f1-ffc8928491d3 | Translated (78%) | Ice machine, water-cooled ice-making head, 500 lb/day < H < 1436 (must be on chilled loop) | 制冰机，水冷制冰头，500 lb/天 < H < 1436（必须在冷却回路上） |
| 2440f6a24442-6d7d-4f64-87ee-bf9d889ff8b5 | Translated (100%) | Elec | 电力 |
| 2441ef5d127b-dfb5-498d-8734-01a361498d3e | Translated (CM) | Ice | 制冰 |
| 2442661f52e4-d9c8-465a-b6b9-44cdbb9f7f6f | Translated (91%) | 5.58 – 0.0011H kWh/100 lb ice | 5.58 – 0.0011H kWh/100 lb 冰 |
| 244390125db6-432d-4a44-b60e-aced90c068b4 | Translated (100%) | na | na |
| 24440c7427ec-7a4c-40a4-aa9f-c6baccb367e9 | Translated (96%) | 5.13 - 0.001H kWh/100 lb ice | 5.13 - 0.001H kWh/100 lb 冰 |
| 2445cd7ca529-a503-4800-9634-9bbe675f0da0 | Translated (100%) | na | na |
| 2446431211bc-f0b9-4a0c-b6ac-0ed86811fb5f | Translated (97%) | Ice machine, water-cooled ice-making head, H < 500 lb/day (must be on chilled loop) | 制冰机，水冷制冰头，H < 500 lb/天（必须在冷却回路上） |
| 2447ef677b5a-b48c-4fcf-b985-48e1a73291c6 | Translated (CM) | Elec | 电力 |
| 2448f9c2be16-8c78-4add-9270-0336549fce48 | Translated (CM) | Ice | 制冰 |
| 2449980c6440-6bc3-4983-8b34-a3d59f1c01c0 | Translated (91%) | 7.80 – 0.0055H kWh/100 lb ice | 7.80 – 0.0055H kWh/100 lb 冰 |
| 24504446b3df-6b67-48dd-8256-4c84fb393dd0 | Translated (100%) | na | na |
| 2451f9304185-e6de-451b-a937-3eaefac7cb77 | Translated (91%) | 7.02 - 0.0049H kWh/100 lb ice | 7.02 - 0.0049H kWh/100 lb 冰 |
| 24526f3ffe3d-17c4-4ed0-bc0f-fb6214d9d136 | Translated (100%) | na | na |
| 2453e9ce61a9-fc0a-4134-bf0f-ad808143381b | Translated (0%) | Ice machine, water-cooled once-through (open loop) | 制冰机，水冷直流（开回路） |
| 2454897e5c20-9ebb-424b-8db3-13de390e7cc5 | Translated (100%) | Elec | 电力 |
| 24559eca2e82-b335-4a56-966e-535e4a7de3f9 | Translated (CM) | Ice | 制冰 |
| 245607d6f1d5-c62f-4a8a-9ead-74ec9e3a17ff | Translated (0%) | Banned | 禁止 |
| 2457269ddfd1-63db-4b5c-aa60-80eaf95a4df9 | Translated (100%) | Banned | 禁止 |
| 24580387911b-85b6-40b4-8b90-a694defbce20 | Translated (CM) | Banned | 禁止 |
| 2459e4732bf4-47a5-42e0-877f-1fc3fd20d32a | Translated (CM) | Banned | 禁止 |
| 24605f9f721f-6383-49e9-a94b-33b927b5d746 | Translated (83%) | Ice machine, water-cooled SCU (self-contained unit), H < 200 lb/day (must be on chilled loop) | 制冰机，水冷 SCU（独立装置），H < 200 lb/天（必须在冷却回路上） |
| 2461296f2a70-a366-4517-988f-f1ed2cfa4162 | Translated (100%) | Elec | 电力 |
| 2462f8f03aef-824a-4c87-adea-b80740307d40 | Translated (CM) | Ice | 制冰 |
| 2463e43f17f8-b954-473d-bf3f-d0ea7aeb82eb | Translated (91%) | 11.4 – 0.0190H kWh/100 lb ice | 11.4 – 0.0190H kWh/100 lb 冰 |
| 24648da2f679-b21e-48d9-981e-a5507bb0d66c | Translated (100%) | na | na |
| 246513bc5cd5-276a-4f65-bc50-adbe3f79e61d | Translated (91%) | 10.6 - 0.177H kWh/100 lb ice | 10.6 - 0.177H kWh/100 lb 冰 |
| 2466502ac216-a138-44e5-8a50-f980c3c4d09f | Translated (100%) | na | na |
| 24679c3fb592-a229-476e-9068-6098e67f8dbe | Translated (93%) | Ice machine, water-cooled self-contained unit, H <3965>></3965> 200 lb/day (must be on chilled loop) | 制冰机，水冷独立装置，H <3965>></3965> 200 lb/天（必须在冷却回路上） |
| 24680339773b-93ce-439a-82c9-48059b64bd5d | Translated (100%) | Elec | 电力 |
| 2469c9dfc0c9-e588-4128-8721-b89735f9cad3 | Translated (CM) | Ice | 制冰 |
| 24707086efb3-0ea0-42bb-a3d0-2c0b15b1fefd | Translated (CM) | 7.6 kWh/100 lb ice | 7.6 kWh/100 lb 冰 |
| 2471a128eab4-c4e1-4222-a8ed-92500e880188 | Translated (CM) | na | na |
| 24725d8e75cf-f94c-4dfd-91e5-09251da41e12 | Translated (100%) | 7.07 kWh/100 lb ice | 7.07 kWh/100 lb 冰 |
| 24736c4e1efc-ef89-4e77-94e2-cb2900b2b397 | Translated (CM) | na | na |
| 2474ab77ad5a-e5f1-4816-a361-39d600224e83 | Translated (0%) | Chest freezer, solid or glass door | 卧式冷冻柜，实心或玻璃门 |
| 247599233e38-324c-42ef-aa95-1aee18b5a29b | Translated (100%) | Elec | 电力 |
| 2476b79d6088-7dbc-466d-b533-80c49a98175b | Translated (0%) | Refrig | 制冷 |
| 2477d48dc8fa-7194-4c6e-aa38-a15642e5484a | Translated (0%) | 0.45V + 0.943 kWh/day | 0.45V + 0.943 kWh/天 |
| 2478f759e3d4-9da2-4382-b617-93b1fd667723 | Translated (100%) | na | na |
| 2479a15b3714-33d8-43a2-b3fd-4909165ca9e7 | Translated (0%) | ≤ 0.270V + 0.130 kWh/day | ≤ 0.270V + 0.130 kWh/天 |
| 2480ad61dfd2-5e46-4c75-90a9-b8f55bf51002 | Translated (CM) | na | na |
| 248108029457-267f-4d56-b28f-902212e5b248 | Translated (89%) | Chest refrigerator, solid or glass door | 卧式冰柜，实心或玻璃门 |
| 24821a23a553-d39b-4267-9717-4a56d1bc243c | Translated (100%) | Elec | 电力 |
| 24832adc7794-76f3-429f-9da5-7b0040089ba9 | Translated (CM) | Refrig | 制冷 |
| 24840e7c906a-c9a2-47c0-9dd0-e80a0dc840fd | Translated (CM) | 0.1V + 2.04 kWh/day | 0.1V + 2.04 kWh/天 |
| 2485b776d206-82fa-413a-a34a-1805c2230d0b | Translated (CM) | na | na |
| 248624a25e48-7ae3-4311-8a5b-eaca80208dfc | Translated (CM) | ≤ 0.125V + 0.475 kWh/day | ≤ 0.125V + 0.475 kWh/天 |
| 2487a022e487-8544-40ef-a122-0ce423c10960 | Translated (CM) | na | na |
| 2488933c2bf7-0645-4b14-81fb-b04346520fb7 | Translated (0%) | Glass-door reach-in freezer, 0 < V < 15 ft³ | 手入式玻璃门冰柜， 0 < V < 15 ft³ |
| 2489789364c8-7b9d-45f9-9df8-7b0ac00f31c1 | Translated (100%) | Elec | 电力 |
| 249054c46d14-b859-4709-ba39-bd1b2849c5cb | Translated (CM) | Refrig | 制冷 |
| 2491172ce7c8-0b70-45f2-a609-4004215c620a | Translated (CM) | 0.75V + 4.10 kWh/day | 0.75V + 4.10 kWh/天 |
| 249259851181-2b6f-441c-987e-f0121686c11c | Translated (CM) | na | na |
| 249388257fec-79e4-4f3b-8caa-9c68e1f04bf1 | Translated (CM) | ≤ 0.607V + 0.893 kWh/day | ≤ 0.607V + 0.893 kWh/天 |
| 2494fc997e38-5e8f-4eff-99bc-15239a570773 | Translated (CM) | na | na |
| 249587e1b04e-8db8-40b6-995d-adb919f257c0 | Translated (99%) | Glass-door reach-in freezer, 15 ≤ V < 30 ft³ | 手入式玻璃门冰柜， 15 ≤ V < 30 ft³ |
| 2496b2d8968e-4a16-4568-8012-73d80e3025df | Translated (CM) | Elec | 电力 |
| 24979275ecb9-cd90-4201-b9ec-871b6a90ba9f | Translated (CM) | Refrig | 制冷 |
| 2498d6bf2a03-ec3f-485c-8dfe-8bdb6ce8caa9 | Translated (99%) | .75V + 4.10 kWh/day | 0.75V + 4.10 kWh/天 |
| 2499ed597371-e728-4586-970b-fe400c27c702 | Translated (CM) | na | na |
| 250019b36a0d-c420-4a78-a5cd-982d52dfdc95 | Translated (0%) | ≤ 0.733V – 1.00 kWh/day | ≤ 0.733V – 1.00 kWh/天 |
| 2501c666922a-9bb8-471d-9834-095521ef503a | Translated (100%) | na | na |
| 2502087fc655-880f-44c0-89f2-73fb6d88f4f9 | Translated (99%) | Glass-door reach-in freezer, 30 ≤ V < 50 ft³ | 手入式玻璃门冰柜，30 ≤ V < 50 ft³ |
| 2503528b0c4c-fcfe-4fc9-a81e-c54bc2212e21 | Translated (CM) | Elec | 电力 |
| 2504f477c657-4a3b-40b4-b5c3-d1d6b3cddbfb | Translated (CM) | Refrig | 制冷 |
| 25056c330c3f-3959-4c32-9b81-55911fd17673 | Translated (CM) | .75V + 4.10 kWh/day | 0.75V + 4.10 kWh/天 |
| 2506e7f7f392-66bc-4ed7-bb19-424df370f3bd | Translated (CM) | na | na |
| 25078d9ee4f0-e456-4e7e-97d3-b3dbbef5298a | Translated (CM) | ≤ 0.250V + 13.50 kWh/day | ≤ 0.250V + 13.50 kWh/天 |
| 25089eec0109-3713-4679-969d-8d8caff826f6 | Translated (CM) | na | na |
| 2509b592584c-88db-4e98-939f-f602a3c9375b | Translated (88%) | Glass-door reach-in freezer, 50 ≤ V ft³ | 手入式玻璃门冰柜，50 ≤ V ft³ |
| 251004574f65-7d85-45ea-b7aa-67371267d043 | Translated (100%) | Elec | 电力 |
| 25111b821842-9ad1-479f-9658-f526aa7e1ecb | Translated (CM) | Refrig | 制冷 |
| 25128106a98b-57ca-405a-8a67-77ed9cff4f9d | Translated (CM) | 0.75V + 4.10 kWh/day | 0.75V + 4.10 kWh/天 |
| 2513553d0c01-d857-42fc-aafd-ae008e705294 | Translated (CM) | na | na |
| 2514db30f0e8-54de-408e-a266-2db9822a279d | Translated (CM) | ≤ 0.450V + 3.50 kWh/day | ≤ 0.450V + 3.50 kWh/天 |
| 251530dab585-a32f-46e7-91b5-648f4ad90389 | Translated (CM) | na | na |
| 2516682436b0-5e66-4c05-be94-326f06805d9f | Translated (91%) | Glass-door reach-in refrigerator, 0 < V < 15 ft³ | 手入式玻璃门冰箱，0 < V < 15 ft³ |
| 2517735ba783-fdcc-4b7c-b8d6-52e873143665 | Translated (100%) | Elec | 电力 |
| 25185a440926-6c7d-4dea-9e42-2f290fb1c139 | Translated (CM) | Refrig | 制冷 |
| 25195f0db354-18ba-44b6-bf3e-3505e1401c77 | Translated (CM) | 0.12V + 3.34 kWh/day | 0.12V + 3.34 kWh/天 |
| 25209976e7fb-296f-45b6-aa03-0644630676f9 | Translated (CM) | na | na |
| 252140a9b56a-7d52-4900-b2af-9eecb6d98889 | Translated (CM) | ≤ 0.118V + 1.382 kWh/day | ≤ 0.118V + 1.382 kWh/天 |
| 25227dbba160-c13f-413b-8539-8a7a4a9ecdfd | Translated (CM) | na | na |
| 25238df5a5cc-f138-4571-9d23-2dccf6f71bf3 | Translated (99%) | Glass-door reach-in refrigerator, 15 ≤ V < 30 ft³ | 手入式玻璃门冰箱，15 ≤ V < 30 ft³ |
| 2524b2cfcf94-260a-4de9-9e21-f31e40f0bad1 | Translated (CM) | Elec | 电力 |
| 2525f3dbcdbc-a2b8-4702-90e8-8d1550f02303 | Translated (CM) | Refrig | 制冷 |
| 252617db7ce6-6478-462e-b0a3-4329db87d93b | Translated (CM) | 0.12V + 3.34 kWh/day | 0.12V + 3.34 kWh/天 |
| 2527dc15db99-de3d-4602-9180-a8cd772fc03d | Translated (CM) | na | na |
| 2528663af8bf-7bff-48a7-bd19-9d4dfe7571da | Translated (CM) | ≤ 0.140V + 1.050 kWh/day | ≤ 0.140V + 1.050 kWh/天 |
| 2529a70a3069-bccd-460e-809a-f22f4658ce51 | Translated (CM) | na | na |
| 2530aee3bf69-c7f6-4ee9-b8ab-d6f9e637e446 | Translated (CM) | Glass-door reach-in refrigerator, 30 ≤ V < 50 ft³ | 手入式玻璃门冰箱，30 ≤ V < 50 ft³ |
| 2531c93c25dd-d0fa-4e79-8ba9-672c637c64f7 | Translated (CM) | Elec | 电力 |
| 2532140b33c5-afd6-411e-9402-999e388ab2d6 | Translated (CM) | Refrig | 制冷 |
| 25333119e13b-d2f3-4ba5-a16c-948b66f7f0b4 | Translated (CM) | 0.12V + 3.34 kWh/day | 0.12V + 3.34 kWh/天 |
| 2534e6df88c9-c312-4a5e-b23a-d1c08ad8b5f7 | Translated (CM) | na | na |
| 25353e2c2ce3-006a-4550-99cc-c28d7fec4705 | Translated (CM) | ≤ 0.088V + 2.625 kWh/day | ≤ 0.088V + 2.625 kWh/天 |
| 2536e74c3c39-4945-46ad-b763-5ff7efd5e703 | Translated (CM) | na | na |
| 2537cb0e332a-4334-4185-80b4-6e49deac7866 | Translated (89%) | Glass-door reach-in refrigerator, 50 ≤ V ft³ | 手入式玻璃门冰箱，50 ≤ V ft³ |
| 2538eecb0bef-7e16-4504-bb26-92efd4acff0b | Translated (100%) | Elec | 电力 |
| 25399061ac4d-3b54-42d2-ae45-b2148229aadb | Translated (CM) | Refrig | 制冷 |
| 2540e7bd945e-d390-44f2-9b28-cc16662dec56 | Translated (CM) | 0.12V + 3.34 kWh/day | 0.12V + 3.34 kWh/天 |
| 254116e3d062-2562-4bd3-916a-131704314dbe | Translated (CM) | na | na |
| 25422f88d30e-8d33-477c-a473-73b6a6605d7c | Translated (CM) | ≤ 0.110V + 1.500 kWh/day | ≤ 0.110V + 1.500 kWh/天 |
| 2543f977c156-5a8a-42ef-9003-52212a14c9c8 | Translated (CM) | na | na |
| 2544f1da66e7-9a23-43df-994c-19e57aa800f7 | Translated (91%) | Solid-door reach-in freezer, 0 < V < 15 ft³ | 手入式实心门冰柜，0 < V < 15 ft³ |
| 25455b8adfdc-af7b-44e3-9e84-d8a5f6c916e9 | Translated (100%) | Elec | 电力 |
| 2546c9d06686-abf2-4495-a652-f2355da1d3e2 | Translated (CM) | Refrig | 制冷 |
| 2547e39e2075-e07a-48f1-bd2f-8de26fe8328b | Translated (CM) | 0.4V + 1.38 kWh/day | 0.4V + 1.38 kWh/天 |
| 2548bc07f0a0-1c9c-4cfc-9bb7-fec0e2e9388c | Translated (CM) | na | na |
| 25496068869b-5e28-421c-b821-632a420592c5 | Translated (CM) | ≤ 0.250V + 1.25 kWh/day | ≤ 0.250V + 1.25 kWh/天 |
| 2550a446fafb-618a-486b-a197-ce6b30b68be2 | Translated (CM) | na | na |
| 25515ff4ad71-900c-4bc6-867d-8f15bc2228ba | Translated (99%) | Solid-door reach-in freezer, 15 ≤ V < 30 ft³ | 手入式实心门冰柜，15 ≤ V < 30 ft³ |
| 2552dbe43e64-1005-45ff-93f9-2aa6b85fdd0a | Translated (CM) | Elec | 电力 |
| 2553c79a0f21-86a2-402d-b6b4-d951c717ab92 | Translated (CM) | Refrig | 制冷 |
| 25549e26af4d-02de-47aa-9681-3d847f3a47e6 | Translated (CM) | 0.4V + 1.38 kWh/day | 0.4V + 1.38 kWh/天 |
| 255554bef810-c89e-450e-8c30-22ebe553c949 | Translated (CM) | na | na |
| 25569e1cc9a8-2854-489a-aa62-8b32764a10d5 | Translated (CM) | ≤ 0.400V – 1.000 kWh/day | ≤ 0.400V – 1.000 kWh/天 |
| 25572a5bcb66-3c92-45fc-8e39-0520d3f5b4f8 | Translated (CM) | na | na |
| 25589c2ee76e-39a3-44eb-a998-e7053bd00869 | Translated (CM) | Solid-door reach-in freezer, 30 ≤ V < 50 ft³ | 手入式实心门冰柜，30 ≤ V < 50 ft³ |
| 25594329fd0f-43cf-4f91-a015-60a6cee8bb03 | Translated (CM) | Elec | 电力 |
| 25602d307b12-f3b9-42f4-82f5-0cf3149f1fee | Translated (CM) | Refrig | 制冷 |
| 2561c42a7c1c-5868-4f5c-9a7d-229012467b93 | Translated (CM) | 0.4V + 1.38 kWh/day | 0.4V + 1.38 kWh/天 |
| 2562774f4738-a941-4e01-8733-b98bdf14ae32 | Translated (CM) | na | na |
| 256351f7ba5b-cbda-443d-9493-c78044c76526 | Translated (CM) | ≤ 0.163V + 6.125 kWh/day | ≤ 0.163V + 6.125 kWh/天 |
| 25643a55f04b-cbda-4556-848b-1355f8302ebe | Translated (CM) | na | na |
| 256564249b7b-074d-4761-8b67-3538584ed44e | Translated (89%) | Solid-door reach-in freezer, 50 ≤ V ft³ | 手入式实心门冰柜，50 ≤ V ft³ |
| 25666593e330-d447-45d9-861c-c04118daa3ed | Translated (100%) | Elec | 电力 |
| 25676585a42a-1e52-4e1f-8637-27fd95a5108d | Translated (CM) | Refrig | 制冷 |
| 2568b1bd6aa0-88fe-47f2-9808-9e08014341ca | Translated (CM) | 0.4V + 1.38 kWh/day | 0.4V + 1.38 kWh/天 |
| 25691942bf72-4126-47e4-8357-6a7b88dedb91 | Translated (CM) | na | na |
| 257023caa183-f0f0-411a-96e7-9debc8f11fdc | Translated (CM) | ≤ 0.158V + 6.333 kWh/day | ≤ 0.158V + 6.333 kWh/天 |
| 2571b3ef5eee-3bfa-40b3-bdcb-8f188af1ffa9 | Translated (CM) | na | na |
| 2572107ab665-407a-4a8e-b5a2-2c8722806d8d | Translated (91%) | Solid-door reach-in refrigerator, 0 < V < 15 ft³ | 手入式实心门冰箱，0 < V < 15 ft³ |
| 257397684ea0-d0d9-454f-8ea8-259ebf416115 | Translated (100%) | Elec | 电力 |
| 257447d714bb-de68-45ed-a4ff-d1c67d7174fb | Translated (CM) | Refrig | 制冷 |
| 25759973e70e-cd04-4b07-ba11-de9b28c97120 | Translated (CM) | 0.1V + 2.04 kWh/day | 0.1V + 2.04 kWh/天 |
| 25762d5f975a-e1bb-4596-abe9-0fe78dfd5e74 | Translated (CM) | na | na |
| 25770a91d3c0-ce7d-4b9c-9080-ac6b950c1851 | Translated (CM) | ≤ 0.089V + 1.411 kWh/day | ≤ 0.089V + 1.411 kWh/天 |
| 25782bd37f81-ca00-41bc-8b42-4f22d7c4aa65 | Translated (CM) | na | na |
| 257952b7a9a2-f230-496a-8f8a-11263df468df | Translated (99%) | Solid-door reach-in refrigerator, 15 ≤ V < 30 ft³ | 手入式实心门冰箱，15 ≤ V < 30 ft³ |
| 258041c9219c-ef0e-484a-a495-6c5e28da9324 | Translated (CM) | Elec | 电力 |
| 2581151670cd-180d-4adf-b3c7-8e752b74dea2 | Translated (CM) | Refrig | 制冷 |
| 2582b7984b14-cc16-4904-b314-56f98cba4739 | Translated (CM) | 0.1V + 2.04 kWh/day | 0.1V + 2.04 kWh/天 |
| 2583b4be7449-0598-46d6-8b82-541951f68cbd | Translated (CM) | na | na |
| 25841aa8923f-0c02-4f4d-ae7c-ba11229e912e | Translated (CM) | ≤ 0.037V + 2.200 kWh/day | ≤ 0.037V + 2.200 kWh/天 |
| 258554b5dc10-864c-4c19-81f8-342f438a1afa | Translated (CM) | na | na |
| 2586b41d9705-bf48-449a-91db-acd4420417bd | Translated (CM) | Solid-door reach-in refrigerator, 30 ≤ V < 50 ft³ | 手入式实心门冰箱，30 ≤ V < 50 ft³ |
| 2587e25c425f-a3cc-497e-81bc-5c6810d7d15c | Translated (CM) | Elec | 电力 |
| 2588a57351a6-0531-465b-89c5-9073b0ecc9a8 | Translated (CM) | Refrig | 制冷 |
| 25892c117c82-c336-4044-aa3c-40a2b7e0b8a1 | Translated (CM) | 0.1V + 2.04 kWh/day | 0.1V + 2.04 kWh/天 |
| 25900a2432dc-4498-4da1-b3a8-573b2d74dfbc | Translated (CM) | na | na |
| 25918bdb571e-d5b3-40e1-a6cb-6a00c590298f | Translated (CM) | ≤ 0.056V + 1.635 kWh/day | ≤ 0.056V + 1.635 kWh/天 |
| 259244f346a0-dbf2-4ffa-8d4b-14385268f0ef | Translated (CM) | na | na |
| 2593773901f3-8279-41d7-beb7-fdcedc92fd9f | Translated (89%) | Solid-door reach-in refrigerator, 50 ≤ V ft³ | 手入式实心门冰箱，50 ≤ V ft³ |
| 25946e62d5f1-abf9-4449-a7ac-71aca32cfdf1 | Translated (100%) | Elec | 电力 |
| 2595538443b6-14ca-4a4a-a922-7bef93805a02 | Translated (CM) | Refrig | 制冷 |
| 2596dc511a45-5769-4fc7-9ae3-141c065f9eee | Translated (CM) | 0.1V + 2.04 kWh/day | 0.1V + 2.04 kWh/天 |
| 2597c62ffc9e-0279-49b0-8ec8-faf598609175 | Translated (CM) | na | na |
| 259898253f5f-98ce-4a8a-9b99-87d8aaa775df | Translated (CM) | ≤ 0.060V + 1.416 kWh/day | ≤ 0.060V + 1.416 kWh/天 |
| 2599f80883cd-abe6-4ccb-a39b-00b7d8eb08b6 | Translated (CM) | na | na |
| 2600f2869a45-300a-4ec2-b06e-8989c39fa9bc | Translated (70%) | Clothes washer | 洗衣机 |
| 2601367c12b2-cde9-4883-a400-2da5d3889812 | Translated (100%) | Gas | 天然气 |
| 26029ffc68cc-be13-4efb-8b3f-7a1f4746933e | Translated (0%) | Sanitation | 卫生 |
| 2603e4f91cad-bee9-40ad-8156-0e994de18bc6 | Translated (0%) | 1.72 MEF | 1.72 MEF |
| 2604635afa4e-bc80-448c-901a-903f7d2dfbbd | Translated (100%) | na | na |
| 26054d6dfbef-74cf-4efa-acf2-1c57bbcc3cf7 | Translated (100%) | 2.00 MEF | 2.00 MEF |
| 26061acf4f33-9ab4-495e-afb9-2a82aec58bbb | Translated (CM) | na | na |
| 2607aa625d08-293d-4b69-aa45-057d721e3e6b | Translated (0%) | Door-type dish machine, high temp | 门式洗碗机，高温 |
| 2608efec964c-57e1-4b8e-bba9-60d8b93f711e | Translated (100%) | Elec | 电力 |
| 26093b74010d-92e2-4316-8a04-3bc05afcb63e | Translated (100%) | Sanitation | 卫生 |
| 2610c749b446-b9ef-4892-9541-e1289e91bf9d | Translated (100%) | na | na |
| 2611fca7fc53-ea52-4260-9e87-59dd93505725 | Translated (100%) | 1.0 kW | 1.0 kW |
| 26123c8c938d-fa87-45a6-9149-0c392c09e4e7 | Translated (CM) | na | na |
| 2613826ae678-7cfd-4ad3-a4eb-3800ca6020fa | Translated (100%) | 0.70 kW | 0.70 kW |
| 2614be8980a1-a5cc-4907-9ec5-6b3ab8c46f2d | Translated (87%) | Door-type dish machine, low temp | 门式洗碗机，低温 |
| 26157dacc82c-3c16-4783-a070-3de6b139ff34 | Translated (100%) | Elec | 电力 |
| 26166e741908-e2ff-4e7c-8c8c-9531c76d35b4 | Translated (CM) | Sanitation | 卫生 |
| 261737748c20-19a8-448c-995c-e307ec5d8ab6 | Translated (CM) | na | na |
| 26182d832263-f25a-4d48-a8d4-28244ac55f66 | Translated (CM) | 0.6 kW | 0.6 kW |
| 26196ad83e8d-770e-4baa-a32d-469f040392f2 | Translated (100%) | na | na |
| 262090e2710c-f0ed-4812-8191-dab4b2668600 | Translated (CM) | 0.6 kW | 0.6 kW |
| 262129cf5b6e-0ac1-4869-9bb0-f09aade28ee6 | Translated (0%) | Multitank rack conveyor dish machine, high temp | 多缸架传送式洗碗机，高温 |
| 2622afaf6d42-40d1-413f-8084-1e549a0e285e | Translated (100%) | Elec | 电力 |
| 2623425d8c8a-56f1-42ec-b518-e064fc7430fb | Translated (CM) | Sanitation | 卫生 |
| 26245a7cd5ae-092e-4f57-a4fd-d9fed2e48b51 | Translated (CM) | na | na |
| 262598223f82-f37d-4c5c-bdc1-618140332e8e | Translated (CM) | 2.6 kW | 2.6 kW |
| 2626ece52016-64cc-46b6-bc0a-748f79d8de85 | Translated (CM) | na | na |
| 262710d25b44-d7da-4d48-a5ba-b39ce39bebf0 | Translated (CM) | 2.25 kW | 2.25 kW |
| 26282c74c041-10c9-428d-a696-3c3ec8bf2c49 | Translated (91%) | Multitank rack conveyor dish machine, low temp | 多缸架传送式洗碗机，低温 |
| 2629e19b0eff-1439-4d62-9508-f6269f8e632d | Translated (100%) | Elec | 电力 |
| 2630b9f19830-3720-4dfa-a820-eb015e373777 | Translated (CM) | Sanitation | 卫生 |
| 263117eafed2-4e22-48d2-a76e-abe2cb3cdb79 | Translated (CM) | na | na |
| 26324fb21e5a-301c-4760-aa5a-b4b1e8585881 | Translated (CM) | 2.0 kW | 2.0 kW |
| 2633bcd829f2-9932-48fd-9570-c861f19da75b | Translated (CM) | na | na |
| 26343288a28a-47b7-4806-b662-113f1aa9f618 | Translated (CM) | 2.0 kW | 2.0 kW |
| 2635718bf4a2-bb9e-4d98-82b7-2dd3f729dcd4 | Translated (91%) | Single-tank rack conveyor dish machine, high temp | 单缸架传送式洗碗机，高温 |
| 263637eb4991-f200-4323-b845-641f3d865c5f | Translated (100%) | Elec | 电力 |
| 26378b05441b-e38a-4f1e-a5b3-3f69780bc0c2 | Translated (CM) | Sanitation | 卫生 |
| 2638d987c3ac-8f63-4dfa-98db-6dc02328e488 | Translated (CM) | na | na |
| 2639ebeb0862-62f7-441b-b898-e4ee18346beb | Translated (CM) | 2.0 kW | 2.0 kW |
| 2640b8b81afa-955a-46a0-b726-979353eff0ca | Translated (CM) | na | na |
| 26417139b626-79d1-4863-9454-958ac8a306a1 | Translated (CM) | 1.5 kW | 1.5 kW |
| 2642ffdd6243-14fc-4073-9d31-8a258a1aeb5f | Translated (91%) | Single-tank rack conveyor dish machine, low temp | 单缸架传送式洗碗机，低温 |
| 26430a283b61-0c2f-460d-96e1-884f103422da | Translated (100%) | Elec | 电力 |
| 264499728564-93df-4863-b83c-92f42c790f04 | Translated (CM) | Sanitation | 卫生 |
| 26453928504e-3326-4b15-8485-c6a673193864 | Translated (CM) | na | na |
| 2646992fc91b-98f5-41d7-87da-6f728e363717 | Translated (CM) | 1.6 kW | 1.6 kW |
| 2647be43c774-0da2-4ee4-a596-02ee91a79ed9 | Translated (CM) | na | na |
| 26482b37bafc-2dbe-4421-9421-b1d203b953d4 | Translated (CM) | 1.5 kW | 1.5 kW |
| 2649deca48b9-2e14-447b-b634-d121e7ed9715 | Translated (87%) | Undercounter dish machine, high temp | 台下式洗碗机，高温 |
| 26501f245334-bc5b-4fb0-ae4c-d515eb6df6ad | Translated (100%) | Elec | 电力 |
| 2651af2e2dc5-9652-489a-af55-340815c4335c | Translated (CM) | Sanitation | 卫生 |
| 2652fa85da3f-3204-48c9-9a8c-c1056a562b94 | Translated (CM) | na | na |
| 2653bbec2171-6bce-48d2-b548-98265dcc088a | Translated (CM) | 0.9 kW | 0.9 kW |
| 2654ef6b8509-7fc3-475d-9437-a1bad3fd3de9 | Translated (CM) | na | na |
| 2655ce95c210-a2b4-4991-846d-6c4140195e37 | Translated (CM) | 0.5 kW | 0.5 kW |
| 2656832e3cd0-1411-4834-a14b-2b564d947c31 | Translated (87%) | Undercounter dish machine, low temp | 台下式洗碗机，低温 |
| 2657375e9dc9-bb7a-40a5-b9f8-4c75172aca3d | Translated (100%) | Elec | 电力 |
| 2658a8f02beb-70b6-456f-ae3d-f9eee16f62cd | Translated (CM) | Sanitation | 卫生 |
| 26596c82bd39-358f-4020-a15f-46bdfbc326dd | Translated (CM) | na | na |
| 2660938ce35e-d043-44be-bf3d-0fe4c7598db4 | Translated (CM) | 0.5 kW | 0.5 kW |
| 266118f889d2-9c73-4962-8de7-236966689343 | Translated (CM) | na | na |
| 266242a3cb54-7bae-440b-9ebf-58d688b3ac50 | Translated (CM) | 0.5 kW | 0.5 kW |
| 2663bc54a810-2c4a-4325-986f-46e7faf3b460 | Translated (0%) | The energy efficiency, idle energy rates, and water use requirements, where applicable, are based on the following test methods: | 能效、闲时功率和用水要求（如适用）均基于以下测试方法： |
| 2664e1e58056-d680-4927-95e9-e5ab0ecca4c3 | Translated (0%) | ASTM F1275 Standard Test Method for Performance of Griddles | ASTM F1275 煎锅性能标准测试方法 |
| 2665f30858e0-9a4a-41b5-b1c6-4ce69dc973e3 | Translated (0%) | ASTM F1361 Standard Test Method for Performance of Open Deep Fat Fryers | ASTM F1361 敞口式深底油炸锅性能标准测试方法 |
| 2666fe960e70-34cb-45ec-8c52-768d0a93c161 | Translated (75%) | ASTM F1484 Standard Test Methods for Performance of Steam Cookers | ASTM F1484 蒸汽炊具性能标准测试方法 |
| 26673449d64d-d666-47a8-a59c-dfaed374a67c | Translated (78%) | ASTM F1496 Standard Test Method for Performance of Convection Ovens | ASTM F1496 对流烤箱性能标准测试方法 |
| 2668cfa4f97a-1ab1-49b8-92b4-7725dc9dfad4 | Translated (80%) | ASTM F1521 Standard Test Methods for Performance of Range Tops | ASTM F1521 炉灶盖性能标准测试方法 |
| 2669f3380973-322c-4930-b6a0-6b21cce9462a | Translated (86%) | ASTM F1605 Standard Test Method for Performance of Double-Sided Griddles | ASTM F1605 双面煎锅性能标准测试方法 |
| 267052ead4f1-80bd-4f60-b4c4-a5b94f0f04ac | Translated (87%) | ASTM F1639 Standard Test Method for Performance of Combination Ovens | ASTM F1639 组合烤箱性能标准测试方法 |
| 26711a7b6ce7-63ec-416a-9345-6d99e37d9c03 | Translated (84%) | ASTM F1695 Standard Test Method for Performance of Underfired Broilers | ASTM F1695 底燃式锅炉性能标准测试方法 |
| 2672040df818-7e6c-4c92-851a-46fe92168d8a | Translated (0%) | ASTM F1696 Standard Test Method for Energy Performance of Single-Rack Hot Water Sanitizing, ASTM Door-Type Commercial Dishwashing Machines | ASTM F1696 单架热水消毒，ASTM 门式商用洗碗机能效标准测试方法 |
| 2673b1855a04-6610-4667-8e27-51f7b4da13a6 | Translated (0%) | ASTM F1704 Standard Test Method for Capture and Containment Performance of Commercial Kitchen Exhaust Ventilation Systems | ASTM F1704 商用厨房排烟通风系统捕捉和容纳性能标准测试方法 |
| 2674dcb5de6e-bd53-4598-80f6-2f66c0075175 | Translated (87%) | ASTM F1817 Standard Test Method for Performance of Conveyor Ovens | ASTM F1817 链条式平炉性能标准测试方法 |
| 2675d6fac274-3126-493a-84ac-76ce31224e62 | Translated (78%) | ASTM F1920 Standard Test Method for Energy Performance of Rack Conveyor, Hot Water Sanitizing, Commercial Dishwashing Machines | ASTM F1920 架传送式热水消毒商用洗碗机能效标准测试方法 |
| 267643c03433-f601-405d-8c09-7cfd345edaeb | Translated (87%) | ASTM F2093 Standard Test Method for Performance of Rack Ovens | ASTM F2093 搁架式烤炉性能标准测试方法 |
| 26775bd31e20-cea3-46d7-9c83-98fb43cc3788 | Translated (72%) | ASTM F2140 Standard Test Method for Performance of Hot Food Holding Cabinets | ASTM F2140 热食物储藏柜性能标准测试方法 |
| 2678a103c32a-2594-49d0-8328-db9469e4a55b | Translated (78%) | ASTM F2144 Standard Test Method for Performance of Large Open Vat Fryers | ASTM F2144 敞口式大桶炸锅性能标准测试方法 |
| 2679da80896d-9d1d-4f7f-a07e-80496b0784c4 | Translated (70%) | ASTM F2324 Standard Test Method for Prerinse Spray Valves | ASTM F2324 预清洗喷雾阀标准测试方法 |
| 2680e86cc69d-f1ee-445d-b16f-0727d9c21168 | Translated (87%) | ASTM F2380 Standard Test Method for Performance of Conveyor Toasters | ASTM F2380 链条式烤面包机性能标准测试方法 |
| 26815be991ed-df40-4b0b-87dc-30f88d580d24 | Translated (0%) | ARI 810-2007: Performance Rating of Automatic Commercial Ice Makers | ARI 810-2007：商用自动制冰机性能等级 |
| 2682b234058e-2ee1-45b1-b84a-702c6441ae3b | Translated (0%) | ANSI/ASHRAE Standard 72–2005: Method of Testing Commercial Refrigerators and Freezers with temperature setpoints at 38°F for medium-temp refrigerators, 0°F for low-temp freezers, and -15°F for ice cream freezers | ANSI/ASHRAE 标准 72–2005：商用冰箱和冰柜测试方法，中等温度冰箱的温度设定值为 38°F，低温冰柜为 0°F，冰淇淋冷柜为 -15°F。 |
| 26835a710255-72a2-4e09-bdf7-37e553a7a1b1 | Translated (100%) | Table 1b. | 表 1b. |
| 26845a710255-72a2-4e09-bdf7-37e553a7a1b1 | Translated (89%) | Commercial Kitchen Appliance Prescriptive Measures and Baseline for Energy Cost Budget (SI units) | 商用厨房电器的规范性指标和能源费用预算基线（SI 单位） |
| 2685bb659772-6e58-47b7-b236-64be85c09fbf | Translated (100%) | Baseline energy usage for energy modeling path | 能耗模拟途径的基线能耗 |
| 2686721869c7-19c3-487a-82fa-1a17ef1cdc11 | Translated (100%) | Levels for prescriptive path | 规范性途径的等级 |
| 2687dcbe50fa-9299-4584-8c3a-5b8baa891e05 | Translated (100%) | Appliance type | 电器类型 |
| 2688dbc03f89-99fc-4b63-bd00-ebd6d3b24705 | Translated (100%) | Fuel | 燃料 |
| 2689a1b142b0-39b0-422a-9ab8-55b0803b9391 | Translated (100%) | Function | 功能 |
| 2690bbc5bd1e-d687-4308-9f54-38790ddc0819 | Translated (100%) | Baseline efficiency | 基线效率 |
| 2691ae87dd0b-746c-4e30-b796-9ed631b84748 | Translated (100%) | Baseline idle rate | 基线闲时功率 |
| 2692cb02fcb4-65aa-43a4-8467-51457e7ad117 | Translated (100%) | Prescriptive efficiency | 规范性效率 |
| 2693dd56182e-bc16-41c5-b6b8-900b807fa565 | Translated (100%) | Prescriptive idle rate | 规范性闲时功率 |
| 2694b5db72ea-1776-45cf-b4f7-6cefe1dc3881 | Translated (100%) | Broiler, underfired | 底燃式锅炉 |
| 26956f4323b9-1554-46b7-8f7d-381dfea7a355 | Translated (100%) | Gas | 天然气 |
| 2696293293b0-9127-41e6-b1e6-5ac6990863fe | Translated (100%) | Cooking | 烹饪 |
| 2697abb49437-9808-4666-9c21-f782ae05a352 | Translated (100%) | 30% | 30% |
| 269807c3897e-c4ea-44c1-ba76-5f6c77af2589 | Translated (0%) | 50.5 kW/m<4659>2</4659> | 50.5 kW/m<4659>2</4659> |
| 2699c3496833-5b13-42a8-b987-a614f35bf8df | Translated (100%) | 35% | 35% |
| 2700b6d46cd2-de1b-4e4d-9347-46934dc917b5 | Translated (CM) | 37.9 kW/m<4666>2</4666> | 37.9 kW/m<4666>2</4666> |
| 270104bcfb47-3699-4193-b060-283d979f31f8 | Translated (96%) | Combination oven, steam mode (P = pan capacity) | 组合烤箱，蒸汽模式（P = 盘容量） |
| 270279e91425-7151-4efd-a7e0-e35735f42453 | Translated (100%) | Elec | 电力 |
| 27032562be06-65f3-4e8f-8e01-56effec96359 | Translated (100%) | Cooking | 烹饪 |
| 2704ae4ad7d1-88f5-4af7-9735-073f3f3ae885 | Translated (CM) | 40% steam mode | 40% 蒸汽模式 |
| 27051b3dfeb4-3db8-4819-bbdd-c6a8e0e3fbe1 | Translated (CM) | 0.37P+4.5 kW | 0.37P+4.5 kW |
| 2706096c0298-2281-46e6-b4e1-35fc99744c1c | Translated (CM) | 50% steam mode | 50% 蒸汽模式 |
| 2707a214a3c3-a1e9-43b3-94a8-8d5ad7925a67 | Translated (CM) | 0.133P+0.6400 kW | 0.133P+0.6400 kW |
| 2708b2443d98-13ea-4688-bcbb-53b1e329042a | Translated (93%) | Combination oven, steam mode | 组合烤箱，蒸汽模式 |
| 27090e192f89-68cc-448d-a5f8-16622be9ddfe | Translated (100%) | Gas | 天然气 |
| 2710b0de5d9a-43b9-4a5e-b841-9b533394703f | Translated (100%) | Cooking | 烹饪 |
| 2711381b55cb-7b26-4970-a238-c156b524fe04 | Translated (CM) | 20% steam mode | 20% 蒸汽模式 |
| 27124917c8d3-715d-4f2a-a2db-6e034a9aa1e8 | Translated (0%) | (1 210P+ | (1 210P+ |
| 2713be9b5702-9961-4eaa-a570-d223d3ec550e | Translated (0%) | 35 810)/3 412 kW | 35 810)/3 412 kW |
| 2714884cd82c-4bff-4d67-9c13-67c5cbe4b6e5 | Translated (100%) | 38% steam mode | 38% 蒸汽模式 |
| 2715b46f08d9-5b7f-4df2-b346-cc5440eb9149 | Translated (0%) | (200P+6 511)/ | (200P+6 511)/ |
| 271697a05ee9-0ebc-46c3-bafd-7792e004fe73 | Translated (0%) | 3 412 kW | 3 412 kW |
| 2717880381b6-70cb-4514-9785-5a576dc03bbe | Translated (93%) | Combination oven, convection mode | 组合烤箱，对流模式 |
| 2718e7f23a72-a4b5-4781-89ca-72309951699b | Translated (100%) | Elec | 电力 |
| 27193c98cc5e-93a9-47ef-b660-312fd164f6ea | Translated (100%) | Cooking | 烹饪 |
| 2720966b1513-fdfb-4555-a617-74031942685b | Translated (CM) | 65% convection mode | 65% 对流模式 |
| 2721c223513a-ce0a-44a8-b764-4bd4aa55837a | Translated (CM) | 0.1P+1.5 kW | 0.1P+1.5 kW |
| 2722832ab524-e7ba-40ef-ba7e-0fa3f5dc4d05 | Translated (CM) | 70% convection mode | 70% 对流模式 |
| 272360007dbf-c1fe-4491-999c-5cd73611d4d0 | Translated (CM) | 0.080P+0.4989 kW | 0.080P+0.4989 kW |
| 2724cbdea3d9-434a-46b2-a16d-c89ae1aa8f0a | Translated (100%) | Combination oven, convection mode | 组合烤箱，对流模式 |
| 272549807d93-6d3e-4d2c-b63f-3e7cc414e00a | Translated (100%) | Gas | 天然气 |
| 272620b8192f-4919-4378-a6cb-d0ac3f1f107b | Translated (100%) | Cooking | 烹饪 |
| 2727d54c5b63-9b3d-4a10-954a-bceaf04818c6 | Translated (CM) | 35% convection mode | 35% 对流模式 |
| 2728fbfb3a9f-0793-44ed-a065-60fa97b9bbd8 | Translated (0%) | (322P+ | (322P+ |
| 2729d755dadd-8f49-477a-9ec0-8a963674c276 | Translated (0%) | 13 563)/ | 13 563)/ |
| 27305df2f7fc-5de5-4ca8-bb69-caa67920c5bc | Translated (100%) | 3412 kW | 3412 kW |
| 2731fcad9451-1a03-4cc1-a5b3-350fef982c77 | Translated (100%) | 44% convection mode | 44% 对流模式 |
| 2732fd131e71-baae-49a8-8b41-d648fd142f99 | Translated (0%) | (150P+5 425)/ | (150P+5 425)/ |
| 27332ec33e17-3d29-4f44-baf5-a66fedf68525 | Translated (100%) | 3412 kW | 3412 kW |
| 2734ebe77d8f-173b-4f28-950e-418c762feeff | Translated (CM) | Convection oven, full-size | 对流烤箱，全尺寸 |
| 2735b3fbe3b5-5513-4e1a-bcad-4303dbacacf7 | Translated (100%) | Elec | 电力 |
| 2736d608fcc9-104e-425f-ab64-856578d5ae04 | Translated (100%) | Cooking | 烹饪 |
| 2737f786e707-7df0-4f7b-97aa-6c562121012c | Translated (CM) | 65% | 65% |
| 2738384d88f4-6cec-49fa-b7a6-00bf4a29200a | Translated (CM) | 2.0 kW | 2.0 kW |
| 273956c88d20-1fe3-41c8-b1b3-586b3d1b538d | Translated (CM) | 71% | 71% |
| 2740d3661567-e5db-4f2c-9248-ef2de70d1815 | Translated (CM) | 1.6 kW | 1.6 kW |
| 27414989f0cd-7117-4c09-8cbe-a99437db6f18 | Translated (CM) | Convection oven, full-size | 对流烤箱，全尺寸 |
| 27423fbf7721-e693-4cb2-b4f5-c5cd2a69af5b | Translated (100%) | Gas | 天然气 |
| 274317c75268-215f-48b0-a438-c18897cc9a5d | Translated (100%) | Cooking | 烹饪 |
| 274426a0b7ee-78ea-4a80-8144-2443ebc47808 | Translated (CM) | 30% | 30% |
| 2745a5eddeab-e236-4521-bfed-168391556997 | Translated (CM) | 5.3 kW | 5.3 kW |
| 2746a92b9f88-6fb1-4626-af25-a77b900a0dba | Translated (CM) | 46% | 46% |
| 27476bcabf8e-e98c-4022-8265-bdab2e7bd7af | Translated (CM) | 3.5 kW | 3.5 kW |
| 27485b3b2638-39b4-451f-bb49-370d3efeb47f | Translated (100%) | Convection oven, half-size | 对流烤箱，半尺寸 |
| 27490012a18a-8606-448b-85c1-44ce58d35bdf | Translated (CM) | Elec | 电力 |
| 2750c0cccf41-d758-4c45-9d77-343c7105b74a | Translated (100%) | Cooking | 烹饪 |
| 27514e0b0035-5ac9-40d5-ae4c-b75047d09a58 | Translated (CM) | 65% | 65% |
| 2752513fe817-fd51-4e4f-9e95-acff7953de4a | Translated (CM) | 1.5 kW | 1.5 kW |
| 27535f8fc151-a46d-449f-9cd6-58ac8818fbbd | Translated (CM) | 71% | 71% |
| 27543ede2885-9c5b-47e6-b7f1-7d4faca1bd65 | Translated (CM) | 1.0 kW | 1.0 kW |
| 2755dbc8c40e-4fa7-4bef-8b0c-d5c8f82aa4f3 | Translated (71%) | Conveyor oven, > 63.5 cm belt | 链条式平炉，> 63.5 cm 传送带 |
| 2756852e3464-7e3e-4873-a63b-77f8e23c6bc0 | Translated (100%) | Gas | 天然气 |
| 2757a2fac096-1cd3-4c19-b424-d33b78e468b8 | Translated (100%) | Cooking | 烹饪 |
| 275860367f6d-07bd-49e3-b6f3-b1d56eadd37c | Translated (CM) | 20% | 20% |
| 2759d23ceb66-6fc6-493c-99e2-f547fbebfd2a | Translated (100%) | 20.5 kW | 20.5 kW |
| 27606ebe41b4-d5f1-4cc2-93c4-0bbecb0ffe85 | Translated (CM) | 42% | 42% |
| 2761f3b08f82-2f8a-4c24-b0cc-1ceabf79f38e | Translated (CM) | 16.7 kW | 16.7 kW |
| 2762de3404bd-8944-4332-bd69-39e2dd7afe31 | Translated (98%) | Conveyor oven, < 63.5 cm belt | 链条式平炉，< 63.5 cm 传送带 |
| 27637e93961a-4646-4af6-9b11-31bac866edbf | Translated (CM) | Gas | 天然气 |
| 276415c5a7a8-97dd-45b0-8d13-5041f141b493 | Translated (100%) | Cooking | 烹饪 |
| 27654070163d-876c-4b51-8bb1-229dec9b24e1 | Translated (CM) | 20% | 20% |
| 2766f85451e6-5916-4fb2-8b08-39add294b622 | Translated (CM) | 13.2 kW | 13.2 kW |
| 276732faaec9-3e6a-48b1-b0c1-e7265ea1fbe6 | Translated (CM) | 42% | 42% |
| 27682fc0fc02-112f-429f-a3f2-2d63c4499251 | Translated (CM) | 8.5 kW | 8.5 kW |
| 2769671883f4-89e1-4dcb-85d6-3ed046d2aa19 | Translated (100%) | Fryer | 炸锅 |
| 2770fa3e67c9-b5b2-4025-9bd1-4179daef6687 | Translated (CM) | Elec | 电力 |
| 2771a180679b-6b7c-4ec1-829a-54cd7b41a361 | Translated (100%) | Cooking | 烹饪 |
| 27721ac71f48-9a58-40f7-87d1-8f2277ef89bc | Translated (CM) | 75% | 75% |
| 2773986da6ef-55a2-4dda-b61e-5911e4cbba67 | Translated (CM) | 1,05 kW | 1,05 kW |
| 27748f1b2743-bc7d-475c-9d4d-82f71f636e2e | Translated (CM) | 80% | 80% |
| 2775317e3909-9746-48f9-9e5e-a15dcdb8739a | Translated (CM) | 1.0 kW | 1.0 kW |
| 27767f143019-e0bc-4cc5-996b-c0db4cba0fe0 | Translated (CM) | Fryer | 炸锅 |
| 27778c8dbe0b-e7f0-4d7e-8432-48ed0de99396 | Translated (100%) | Gas | 天然气 |
| 2778492bde12-23da-4e4e-89cd-da40f3bfdfbd | Translated (100%) | Cooking | 烹饪 |
| 2779b3c054ca-2658-41ae-b096-33ff309b303f | Translated (CM) | 35% | 35% |
| 27801e166caa-b7c6-4a25-9008-17b70bac9904 | Translated (CM) | 4.1 kW | 4.1 kW |
| 2781cb5d37c7-4391-4e3d-b03a-4ae56bbe983f | Translated (CM) | 50% | 50% |
| 2782c4c78f40-3bda-4f49-b291-6328e4777085 | Translated (CM) | 2.64 kW | 2.64 kW |
| 2783a33afaea-37cc-4b09-97a1-9eb2846f4a43 | Translated (76%) | Griddle (based on 90-cm model) | 煎锅（基于 90 cm 型号） |
| 2784c0c48a74-f894-4ce8-9916-6f204e85fcd5 | Translated (100%) | Elec | 电力 |
| 27858b8fedfa-23c1-4876-9685-cb3cc58e0a43 | Translated (100%) | Cooking | 烹饪 |
| 2786d2c9a658-f868-4bd2-aff3-3f2c9109c2b6 | Translated (CM) | 60% | 60% |
| 27874a9fb7f1-b1a8-474b-a1aa-5df73387f9eb | Translated (CM) | 4.3 kW/m<4928>2</4928> | 4.3 kW/m<4928>2</4928> |
| 2788bcda403e-8c86-4082-bee4-7ef4f7a57838 | Translated (CM) | 70% | 70% |
| 2789a7893923-b726-4a83-99b5-2d9d15bb98c1 | Translated (0%) | 3 .45 kW/m<4935>2</4935> | 3 .45 kW/m<4935>2</4935> |
| 27901c5c6bd0-548d-4869-9c63-ef1c77dda04f | Translated (100%) | Griddle (based on 90-cm model) | 煎锅（基于 90 cm 型号） |
| 2791fff73351-43e1-4b4b-91a1-fb8f31cf52ff | Translated (100%) | Gas | 天然气 |
| 2792cc0c0706-c71e-4a39-bedb-87d0c761bc8d | Translated (100%) | Cooking | 烹饪 |
| 2793e1d57e49-0b97-4452-8877-27cf12ea8d0d | Translated (CM) | 30% | 30% |
| 2794ecf65094-b001-418b-a979-c88d81aa2b86 | Translated (CM) | 11 kW/m<4951>2</4951> | 11 kW/m<4951>2</4951> |
| 2795d8934602-e77f-425f-8565-9e7dbcf73540 | Translated (CM) | 33% | 33% |
| 2796a1b1b483-81c6-4b95-bec9-3621caf1b4cf | Translated (CM) | 8.35 kW/m<4958>2</4958> | 8.35 kW/m<4958>2</4958> |
| 2797e9f6d695-2d67-477b-90dc-16165e2b37f6 | Translated (90%) | Hot food holding cabinets (excluding drawer warmers and heated display) 0 < V < 0.368 m<4962>3</4962> (V = volume) | 热食物储藏柜（不包括抽屉式加热器和加热显示器），0 < V < 0.368 m<4962>3</4962> (V = 体积) |
| 27986fb9d37c-7e54-4686-bb30-66108944ac71 | Translated (100%) | Elec | 电力 |
| 2799466bac79-92d9-484b-85d2-f3047c57d630 | Translated (100%) | Cooking | 烹饪 |
| 2800ccfa8e3c-bcdd-4207-ab76-19ef23de4f93 | Translated (CM) | na | na |
| 280103af9030-1624-4db8-9cb8-fcaa510b23a5 | Translated (100%) | 1.4 kW/m<4975>3</4975> | 1.4 kW/m<4975>3</4975> |
| 28024be7abfe-fe2c-4139-8097-6b559e3bece9 | Translated (100%) | na | na |
| 2803ad496156-5514-42f4-8e21-47cf21b97b30 | Translated (0%) | (21.5*V)/0.0283 kW/m<4982>3</4982> | (21.5*V)/0.0283 kW/m<4982>3</4982> |
| 28044a0d2a40-043b-4549-9507-867398f0bcea | Translated (90%) | Hot food holding cabinets (excluding drawer warmers and heated display), 0.368 ≤ V < 0.793 m<4986>3</4986> | 热食物储藏柜（不包括抽屉式加热器和加热显示器），0.368 ≤ V < 0.793 m<4986>3</4986> |
| 280516fc2f6a-b6be-46cc-b4ce-2f528692c9ea | Translated (100%) | Elec | 电力 |
| 2806bafe0789-fa9b-42c0-9272-8924d1fd35bc | Translated (100%) | Cooking | 烹饪 |
| 28071ed9caf7-e818-4a8f-bb4d-c560d7847dd8 | Translated (CM) | na | na |
| 2808fbc88428-b56d-4b95-83f1-866bac48bdcc | Translated (100%) | 1.4 kW/m<4999>3</4999> | 1.4 kW/m<4999>3</4999> |
| 28095f8300ca-d334-4846-b71e-4ade42d7f62d | Translated (CM) | na | na |
| 2810ab538798-da6d-48ec-9f49-4a491f7eb648 | Translated (0%) | (2.0*V + 254)/0.0283 kW/m<5006>3</5006> | (2.0*V + 254)/0.0283 kW/m<5006>3</5006> |
| 281193eb4619-3c2b-4d96-bb34-1926c1f711bc | Translated (89%) | Hot food holding cabinets (excluding drawer warmers and heated display), 0.793 m<5010>3</5010> ≤ V | 热食物储藏柜（不包括抽屉式加热器和加热显示器），0.793 m<5010>3</5010> ≤ V |
| 281201021259-6ae7-4caf-9228-63b0a12f5419 | Translated (100%) | Elec | 电力 |
| 2813023f4af8-f53e-4fa7-82df-31230f36f0fc | Translated (100%) | Cooking | 烹饪 |
| 2814df184bcd-cb46-4bc5-ab6e-3853746d7cb2 | Translated (CM) | na | na |
| 28155776b841-831c-4e78-b916-3eb0c1d17d0e | Translated (100%) | 1.4 kW/m<5023>3</5023> | 1.4 kW/m<5023>3</5023> |
| 281610d4b168-93f4-4fdd-959e-5835a26327a6 | Translated (CM) | na | na |
| 2817006dfe3d-deb1-4867-b6e8-b1c7e139aeb3 | Translated (100%) | (3.8*V + 203.5)/0.0283 kW/m<5030>3</5030> | (3.8*V + 203.5)/0.0283 kW/m<5030>3</5030> |
| 2818b4cce97d-a05a-4848-a596-a2e1eded782e | Translated (100%) | Large vat fryer | 大桶炸锅 |
| 28196c4b877e-96b0-4dc9-84c4-74a9952eb45f | Translated (CM) | Elec | 电力 |
| 28204d15681a-a67f-440e-bdf7-9c5eb6edfac9 | Translated (100%) | Cooking | 烹饪 |
| 2821c4cf18a9-2401-440e-97a9-2c0979b9494b | Translated (CM) | 75% | 75% |
| 28229f22a9fe-b578-4418-a13a-aff8e6c194ce | Translated (CM) | 1.35 kW | 1.35 kW |
| 2823c15c83ff-1e6c-475d-9f24-be851d9b4b5a | Translated (CM) | 80% | 80% |
| 2824e8d55f7f-34c5-4e76-a071-0acac704a29e | Translated (CM) | 1.1 kW | 1.1 kW |
| 28253a52039e-690b-467e-b4f1-28c8afbd78f1 | Translated (100%) | Large vat fryer | 大桶炸锅 |
| 28268628c9f2-32b4-4d13-b845-db44c31b1170 | Translated (CM) | Gas | 天然气 |
| 28270ac448fe-5bf4-4389-bc80-5963fc6d5515 | Translated (100%) | Cooking | 烹饪 |
| 2828ab6b081d-2d01-48db-82a3-99bdeb8951fa | Translated (CM) | 35% | 35% |
| 28299020f107-8d86-46b6-86d1-8525540bb0f7 | Translated (CM) | 5.86 kW | 5.86 kW |
| 28303fdf351b-af50-47ac-a238-5653062d57d7 | Translated (CM) | 50% | 50% |
| 283197e43622-6f5b-4861-8d59-dae21a9b669e | Translated (CM) | 3.5 kW | 3.5 kW |
| 2832ac2b77ec-d100-4090-96d3-2897a872cdb3 | Translated (100%) | Rack oven, double | 搁架式烤炉，双 |
| 2833e1d8d14f-ba74-49fb-9ffa-bf93de0edbeb | Translated (CM) | Gas | 天然气 |
| 2834b704785f-c5eb-44f5-ad07-beef08c0cae3 | Translated (100%) | Cooking | 烹饪 |
| 283526ce6d22-fe17-4eb8-9725-bfbc98dd02d5 | Translated (CM) | 30% | 30% |
| 28361f80d642-322b-4813-bb99-2e4f80aad0e0 | Translated (CM) | 19 kW | 19 kW |
| 2837c91499a3-e403-45ef-9302-761fb82c0a23 | Translated (CM) | 50% | 50% |
| 28384e1fbb79-37b9-4402-bc4d-6ebe1feb57fd | Translated (CM) | 10.25 kW | 10.25 kW |
| 2839efa6cbd3-0cdc-4dee-982f-1520675059fc | Translated (100%) | Rack oven, single | 搁架式烤炉，单 |
| 2840b6f1e82b-c959-44de-9004-c0da0ea72b8a | Translated (CM) | Gas | 天然气 |
| 284142131a1a-1ab3-4cf6-8c68-ae6653b679f1 | Translated (100%) | Cooking | 烹饪 |
| 2842bbf242e6-9b9d-4f0f-aae2-d6ab2b71db4e | Translated (CM) | 30% | 30% |
| 2843f390b663-353c-4ed6-b25c-ec5425450a94 | Translated (CM) | 12.6 kW | 12.6 kW |
| 2844b99e3a78-0b38-49ef-85aa-f18580bb1322 | Translated (CM) | 50% | 50% |
| 284594a495d3-ec0b-40e4-9545-a7bd3a3c4bad | Translated (CM) | 8.5 kW | 8.5 kW |
| 284661be24ab-f97b-43ba-99e1-ec0106f046ee | Translated (100%) | Range | 炉灶 |
| 28477e187d54-5b77-416a-9f86-ed77f95f961e | Translated (CM) | Elec | 电力 |
| 284860121f1b-ce15-4a8d-a217-f8ef015d3841 | Translated (100%) | Cooking | 烹饪 |
| 2849e54a30a7-572d-48ba-9f4f-eefacf891714 | Translated (CM) | 70% | 70% |
| 285088d374a0-c96b-4e2f-880b-44a6867e0435 | Translated (CM) | na | na |
| 28513ab99b9c-368c-4566-9df6-ff323d50f261 | Translated (100%) | 80% | 80% |
| 2852e3ff9301-94d0-4126-8cde-8def24ac29c4 | Translated (CM) | na | na |
| 285328d09f1b-5fcd-477d-93fa-ffe7e0846129 | Translated (100%) | Range | 炉灶 |
| 285468440842-32b1-4993-8af7-3c3d1943f6d2 | Translated (CM) | Gas | 天然气 |
| 285538facaf1-825a-45ff-9b8a-5696d13a58ca | Translated (100%) | Cooking | 烹饪 |
| 2856fcab5c83-e309-42b9-a4ce-cabfb711b22a | Translated (CM) | 35% | 35% |
| 2857fa0c499f-dba0-472d-bc88-27a0e6ed35a1 | Translated (CM) | na | na |
| 2858a48d5ae5-135c-433f-a4a6-8ce5cc5d5145 | Translated (CM) | 40% and no standing pilots | 40% 且无竖直引燃器 |
| 28595ab8b801-309c-43d6-ba6f-e525d59e582a | Translated (CM) | na | na |
| 2860a67a970b-5e34-4443-8c04-80ab76d0b74b | Translated (CM) | Steam cooker, batch cooking | 蒸汽锅，批量烹饪 |
| 28616df67d2e-7534-45ac-90e3-5d8e6d868113 | Translated (100%) | Elec | 电力 |
| 28623b92c7e6-558e-47f5-8a74-f94dfcf03c65 | Translated (100%) | Cooking | 烹饪 |
| 286381bda648-199e-40b4-8b5b-af8675ffdf29 | Translated (CM) | 26% | 26% |
| 2864a1aef77a-4260-40cf-ab71-ff52b3e3cd0a | Translated (CM) | 200 W/pan | 200 W/盘 |
| 28655b999224-caf7-4405-b929-e0d4293ba56d | Translated (100%) | 50% | 50% |
| 28661d47f14e-43de-4e0c-be3c-d07a3bf7e0b6 | Translated (CM) | 135 W/pan | 135 W/盘 |
| 286714472f18-e313-45b0-bd85-83d132851a0d | Translated (100%) | Steam cooker, batch cooking | 蒸汽锅，批量烹饪 |
| 28683ac650be-5e3c-418b-b741-b8f6f10fb497 | Translated (100%) | Gas | 天然气 |
| 28694178af32-2495-46a4-9710-f7426e1cfd38 | Translated (100%) | Cooking | 烹饪 |
| 2870e3b44ecd-5522-4e52-8705-e2f69b97634c | Translated (CM) | 15% | 15% |
| 2871e89bf38a-e5a1-409a-92eb-ceea3a6940a1 | Translated (CM) | 733 W/pan | 733 W/盘 |
| 2872f9f5743b-c370-4d1d-a35f-60c5f02975f1 | Translated (CM) | 38% | 38% |
| 28733cd24629-8e91-445b-8379-753832c0d138 | Translated (CM) | 615 W/pan | 615 W/盘 |
| 2874bfa2514f-8afd-4106-874e-87777bd2a529 | Translated (100%) | Steam cooker, high production or cook to order | 蒸汽锅，高产量或按订单烹饪 |
| 287587ec9a3a-1801-4ddb-afb1-8d7215924f77 | Translated (CM) | Elec | 电力 |
| 28762ec78999-c02d-41c0-bbd8-56e8af3ccfdf | Translated (100%) | Cooking | 烹饪 |
| 287746ebede7-180b-4d72-9b4f-8199051a69a1 | Translated (CM) | 26% | 26% |
| 28787c0a9af3-0a5c-4d79-863a-5aff978b98ff | Translated (CM) | 330 W/pan | 330 W/盘 |
| 28799a82be64-4ec1-4775-82e8-d6ecd5ff43b2 | Translated (CM) | 50% | 50% |
| 2880998f76d6-baa4-4689-9b4b-aeab48f65aed | Translated (CM) | 275 W/pan | 275 W/盘 |
| 28817b7d24e3-200b-41af-b2e9-008856427e5f | Translated (100%) | Steam cooker, high production or cook to order | 蒸汽锅，高产量或按订单烹饪 |
| 288213e0c939-f3ca-418f-bf31-e2b9061e1f78 | Translated (CM) | Gas | 天然气 |
| 28839c479135-5c79-4182-bbb2-d95b8fabbf7a | Translated (100%) | Cooking | 烹饪 |
| 2884c26a1c9b-e5a8-4c5c-81fa-e43a7d6fce36 | Translated (CM) | 15% | 15% |
| 2885426fad68-147c-4943-9ae8-8ac33f1b6777 | Translated (CM) | 1.47 kW/pan | 1.47 kW/盘 |
| 28863d558f7b-e96e-46c9-a6ea-81b1bc968923 | Translated (CM) | 38% | 38% |
| 28873f49af99-e3c1-45bc-a69d-d05544781432 | Translated (CM) | 1.26 kW/pan | 1.26 kW/盘 |
| 288889a69067-6b29-4bf9-a066-24b4804ae5c1 | Translated (100%) | Toaster | 烤面包机 |
| 288928a241cb-1882-4d4a-a75d-698f9811e43f | Translated (CM) | Elec | 电力 |
| 289078ac4af2-f087-4034-932d-5fef0c752c91 | Translated (100%) | Cooking | 烹饪 |
| 2891032a8348-1bd9-407c-8d50-7ad9c52f66ac | Translated (100%) | na | na |
| 2892012ee885-b578-4cb9-88b2-1e16cf8d3166 | Translated (CM) | 1.8 kW average operating energy rate | 1.8 kW 平均运行功率 |
| 289324bdf86d-4bdf-409f-91cc-eada9d0bd71c | Translated (CM) | na | na |
| 28949f97b34f-0133-469e-9df5-b8e91323b05f | Translated (CM) | 1.2 kW average operating energy rate | 1.2 kW 平均运行功率 |
| 289555b58900-2415-4604-bef7-2c2b6f07b672 | Translated (86%) | Ice machine, IMH (ice making head, H = ice harvest) H <5268>≥</5268> 204 kg/day | 制冰机，IMH（制冰头，H = 产冰量），H <5268>≥</5268> 204 kg/天 |
| 2896384c51b5-dd25-4e8e-99c1-c9a651d1b2fa | Translated (100%) | Elec | 电力 |
| 28978263c19c-d92e-495c-b6cf-39595044f660 | Translated (CM) | Ice | 制冰 |
| 289859eef978-f048-4ae5-9a31-f719aa5b4e1a | Translated (0%) | 0.0015 - 5.3464E<5278>-07</5278> kWh/kg ice | 0.0015 - 5.3464E<5278>-07</5278> kWh/kg 冰 |
| 2899d415c53b-fcac-4cfa-9ea1-100546637c7c | Translated (100%) | na | na |
| 2900323b7d59-e369-49e3-83e5-9a2eb351845a | Translated (0%) | ≤ 13.52*H<5285>-0.298</5285> kWh/100 kg ice | ≤ 13.52*H<5285>-0.298</5285> kWh/100 kg 冰 |
| 2901c58c9da9-b26e-4135-a0bc-b416080c1ca8 | Translated (100%) | na | na |
| 29022d1379c2-d62f-4238-bfda-762a10b4f379 | Translated (80%) | Ice machine, IMH (ice making head), H < 204 kg/day | 制冰机，IMH（制冰头），H < 204 kg/天 |
| 29034b90b09c-a089-459d-a253-b15932112a63 | Translated (100%) | Elec | 电力 |
| 290401f16280-93d8-445f-a77c-b6ba36214c18 | Translated (CM) | Ice | 制冰 |
| 29054548dd22-5f46-4ef3-8e58-5f49e51f0a9e | Translated (0%) | 0.2262 - 4.18E<5301>-04</5301> kWh/kg ice | 0.2262 - 4.18E<5301>-04</5301> kWh/kg 冰 |
| 2906aa54e04b-b466-43a7-acb0-65de241f6207 | Translated (100%) | na | na |
| 290766f9e82f-1158-45b2-820d-921c2f2be324 | Translated (CM) | ≤ 13.52*H<5308>-0.298</5308> kWh/100 kg ice | ≤ 13.52*H<5308>-0.298</5308> kWh/100 kg 冰 |
| 29080047c687-ec61-43c3-9752-6d82761ae3ad | Translated (CM) | na | na |
| 29095b96dafc-30bb-466b-b789-7efc180c25d8 | Translated (96%) | Ice machine, RCU <5315> </5315>(remote condensing unit, w/o remote compressor) H < 454 kg/day | 制冰机，RCU（远程冷凝装置，不带远程压缩机），H < 454 kg/天 |
| 2910b2c9b1c8-ec53-4a50-a47e-e06f3609a4c4 | Translated (100%) | Elec | 电力 |
| 291136f83ac0-1af2-47a1-8dda-a6a82f696fe8 | Translated (CM) | Ice | 制冰 |
| 291289693ea5-78f9-417c-951f-1c46db7054e4 | Translated (0%) | 0.1951 - 1.85E<5325>-04</5325> kWh/kg ice | 0.1951 - 1.85E<5325>-04</5325> kWh/kg 冰 |
| 2913ab14ee83-da85-49a2-91f1-573373570ea5 | Translated (100%) | na | na |
| 291453ee2f96-ac26-4bb4-8fd3-75c653569649 | Translated (0%) | ≤ 111.5835H<5332>-0.258</5332>) + 2.205 kWh/100 kg ice | ≤ 111.5835H<5332>-0.258</5332>) + 2.205 kWh/100 kg 冰 |
| 29150688e5b9-eb12-4f8a-a9ec-55323c5ea38e | Translated (100%) | na | na |
| 2916befbad22-46fc-4225-9383-d968784be150 | Translated (97%) | Ice machine, RCU (remote condensing unit) 726 > H <5339>≥</5339> 454 kg/day | 制冰机，RCU（远程冷凝装置），726 > H <5339>≥</5339> 454 kg/天 |
| 29174a318275-c3f6-4142-aaba-be52b30a3bab | Translated (100%) | Elec | 电力 |
| 291874fc9001-a928-4706-9ef9-c7616e32dcfa | Translated (CM) | Ice | 制冰 |
| 29190f1ddf4f-4eb8-4187-875a-846222fd25f6 | Translated (0%) | 0.1124 kWh/kg ice | 0.1124 kWh/kg 冰 |
| 292016e8caf5-c5c6-4bf4-91d2-586e8d30313e | Translated (100%) | na | na |
| 2921a240d91f-a1b0-4f4e-97aa-55ca2f0703b7 | Translated (CM) | ≤ 111.5835H<5355>-0.258</5355>) + 2.205 kWh/100 kg ice | ≤ 111.5835H<5355>-0.258</5355>) + 2.205 kWh/100 kg 冰 |
| 2922a3c9d69e-baf8-4918-b718-9aeb048795f3 | Translated (CM) | na | na |
| 2923f1e193f4-8a69-48e5-a3d8-a124db492087 | Translated (96%) | Ice machine, RCU (remote condensing unit), H <5362>></5362> 726kg/day | 制冰机，RCU（远程冷凝装置），H <5362>></5362> 726 kg/天 |
| 2924ee72a382-e4cf-4547-be3a-b865efba9208 | Translated (100%) | Elec | 电力 |
| 292551ab831e-9a21-4ec9-851c-c4d6c0d6b923 | Translated (CM) | Ice | 制冰 |
| 292652b6a534-5037-4c5e-adb9-8a588e95c7be | Translated (CM) | 0.1124 kWh/kg ice | 0.1124 kWh/kg 冰 |
| 2927054c952a-2b33-46a2-8d16-02e4c51792bd | Translated (CM) | na | na |
| 2928a1ded5e5-4fb9-48f6-aff5-e1173c02748a | Translated (0%) | ≤ -0.00024H + 4.60 kWh/100 kg ice | ≤ -0.00024H + 4.60 kWh/100 kg 冰 |
| 29297faf68bf-bc85-4de8-b11a-d7c72644ce3e | Translated (100%) | na | na |
| 2930a089f3c1-b5f5-470d-ad17-afadd655e9d8 | Translated (81%) | Ice machine, SCU (self contained unit), H < 79 kg/day | 制冰机，SCU（独立装置），H < 79 kg/天 |
| 2931b0c5b959-c4cc-4e90-b59e-2f36c7251daa | Translated (100%) | Elec | 电力 |
| 293209ff909a-61e5-4fb2-b2c7-126385ea2f73 | Translated (CM) | Ice | 制冰 |
| 29336c33ea07-6c38-4378-9655-b7cc10eede97 | Translated (0%) | 0.3968 - 2.28E<5393>-03</5393> kWh/kg ice | 0.3968 - 2.28E<5393>-03</5393> kWh/kg 冰 |
| 2934f8255980-63e7-4bd5-8aa7-6efeedf72c95 | Translated (100%) | na | na |
| 29354691a1ba-50e7-4a52-b225-1284ec0d79e8 | Translated (0%) | 236.59H<5400>-0.326 </5400>+0.176 kWh/100 kg ice | 236.59H<5400>-0.326 </5400>+0.176 kWh/100 kg 冰 |
| 2936fcabb71f-e3d0-4841-bfe8-89e9207184d0 | Translated (100%) | na | na |
| 293763445e3f-14e3-4f30-9cd3-417bec8f6cd7 | Translated (98%) | Ice machine, SCU (self contained unit), H <5407>≥</5407> 79 kg/day | 制冰机，SCU（独立装置），H <5407>≥</5407> 79 kg/天 |
| 2938fc666f33-e407-4085-9d53-f1edfb4dd425 | Translated (CM) | Elec | 电力 |
| 2939293a3ebb-a9eb-4629-9aaa-d4b97b4babe9 | Translated (CM) | Ice | 制冰 |
| 294037a6c8a8-6bfd-4471-b279-41417f5c7f94 | Translated (CM) | 0.2161 kWh/kg ice | 0.2161 kWh/kg 冰 |
| 2941506d140f-831a-4f1c-a950-977a25ff7704 | Translated (CM) | na | na |
| 294282f41010-08c4-4794-9d29-7f5556f9b214 | Translated (CM) | 236.59H<5423>-0.326 </5423>+0.176 kWh/100 kg ice | 236.59H<5423>-0.326 </5423>+0.176 kWh/100 kg 冰 |
| 2943153c4279-49aa-48c0-b5f6-b2b72d0bb737 | Translated (CM) | na | na |
| 29444bdc38bb-a2f5-4648-ba25-96128636a22f | Translated (90%) | Ice machine, water-cooled ice-making head, H <5430>≥</5430> 651 kg/day(must be on a chilled loop) | 制冰机，水冷制冰头，H <5430>≥</5430> 651 kg/天（必须在冷却回路上） |
| 2945ef42538a-c37c-4a8e-8422-6aa55816f4eb | Translated (100%) | Elec | 电力 |
| 2946803b849d-018e-4486-9fed-224e21fac95f | Translated (CM) | Ice | 制冰 |
| 2947a7ce8b15-966a-4b64-b707-e7150f51bb84 | Translated (CM) | 0.0882 kWh/kg ice | 0.0882 kWh/kg 冰 |
| 2948d8f532ad-189b-4be2-be9e-2288edd9743a | Translated (CM) | na | na |
| 29494a8e74d1-dec6-48be-b70a-559c7f932ada | Translated (0%) | ≤ 8.11 kWh/100 kg ice | ≤ 8.11 kWh/100 kg 冰 |
| 29506695871d-96cd-4ec2-b352-294b533e6b0f | Translated (100%) | na | na |
| 29517ae642bf-c3b3-4ae2-b72b-088966ad284b | Translated (90%) | Ice machine, water-cooled ice-making head, 227 <5452>≤ </5452>H < 651 kg/day (must be on a chilled loop) | 制冰机，水冷制冰头，227 <5452>≤ </5452>H < 651 kg/天（必须在冷却回路上） |
| 295289bb7874-1e52-4965-99a6-43164980f3a4 | Translated (100%) | Elec | 电力 |
| 2953d9dec34c-e5b7-43fc-b8e0-f310fb934b2c | Translated (CM) | Ice | 制冰 |
| 2954a2fcf0b4-907d-44fb-9c7d-af2bd70098b8 | Translated (0%) | 0.1230 - 5.35E<5462>-05</5462> kWh/kg ice | 0.1230 - 5.35E<5462>-05</5462> kWh/kg 冰 |
| 2955f8f07db5-f926-4ae2-9771-1708dd0deb59 | Translated (100%) | na | na |
| 2956ee45406c-03ef-49f0-9c4d-d0d041e3f043 | Translated (0%) | ≤ 11.31 - 0.065H kWh/100 kg ice | ≤ 11.31 - 0.065H kWh/100 kg 冰 |
| 295753033974-dd5f-4f1c-bee2-ccbccceab77b | Translated (100%) | na | na |
| 29580f4b2b6a-1db8-4af9-870c-f0f3fab22c7c | Translated (96%) | Ice machine, water-cooled ice-making head, H < 227 kg/day( must be on a chilled loop) | 制冰机，水冷制冰头，H < 227 kg/天（必须在冷却回路上） |
| 295911855609-2755-413f-9654-fcd8f3ef35c1 | Translated (CM) | Elec | 电力 |
| 2960d5ee9037-39c0-4b26-8b60-a9d3b2206f3b | Translated (CM) | Ice | 制冰 |
| 2961c4c02bcc-656a-44c2-bb74-8b2a6bc73fca | Translated (0%) | 0.1720 - 2.67E<5484>-04</5484> kWh/kg ice | 0.1720 - 2.67E<5484>-04</5484> kWh/kg 冰 |
| 2962383fe9ee-5e37-4b60-a943-a1594685c900 | Translated (100%) | na | na |
| 2963ed351109-73b8-47fe-bb91-d6d639ea9022 | Translated (0%) | ≤ 15.48 - 0.0238H kWh/100 kg ice | ≤ 15.48 - 0.0238H kWh/100 kg 冰 |
| 296415be56a5-b0fc-4df1-8f7a-306a50dc7c6a | Translated (100%) | na | na |
| 29652b4cf5cc-4a17-4f96-a918-87ff41732f15 | Translated (CM) | Ice machine, water-cooled once-through (open loop) | 制冰机，水冷直流（开回路） |
| 29662f82d16f-e977-418e-87d4-7f6302245aa1 | Translated (CM) | Elec | 电力 |
| 29678f0b8639-5bf2-45b8-b764-71f550ab9226 | Translated (CM) | Ice | 制冰 |
| 2968b5cf8a0f-6d36-49b2-b89e-4aa886d20d46 | Translated (100%) | Banned | 禁止 |
| 2969d1d4e1e9-af5e-4530-ba69-680f8de80fd0 | Translated (CM) | Banned | 禁止 |
| 2970bfa4aec8-05cb-456d-b856-9bf4af12421e | Translated (CM) | Banned | 禁止 |
| 297159f8ccbf-ca65-4646-b9b5-4abbe8e42274 | Translated (CM) | Banned | 禁止 |
| 297250940681-758e-4f67-aa32-003d99594d36 | Translated (84%) | Ice machine, water cooled SCU (self-contained unit) H < 91 kg/day (must be on a chilled loop) | 制冰机，水冷 SCU（独立装置），H < 91 kg/天（必须在冷却回路上） |
| 297334d40768-2993-42ad-8540-41dbb4f972e7 | Translated (100%) | Elec | 电力 |
| 29745190a6a8-6afb-4013-8fd4-442d86716a90 | Translated (CM) | Ice | 制冰 |
| 2975a55ff1f4-d103-4da5-87f2-b94b4f686994 | Translated (0%) | 0.2513 - 9.23E<5527>-04</5527> kWh/kg ice | 0.2513 - 9.23E<5527>-04</5527> kWh/kg 冰 |
| 2976830b12e7-9a12-4104-925f-34e42e3fa730 | Translated (100%) | na | na |
| 29773a948819-1d1c-4a95-9d92-32e780c553a5 | Translated (0%) | ≤ 23.37-0.086H kWh/100 kg ice | ≤ 23.37-0.086H kWh/100 kg 冰 |
| 2978f6d34c8d-30c4-4c1d-b17c-d20b40f0a386 | Translated (100%) | na | na |
| 29795bf638bb-f030-461e-b17d-c41e0718cbf3 | Translated (98%) | Ice machine, water cooled SCU (self-contained unit) H <5540>></5540> 91 kg/day (must be on a chilled loop) | 制冰机，水冷 SCU（独立装置），H <5540>></5540> 91 kg/天（必须在冷却回路上） |
| 298056db9c17-949d-40cb-9b9c-3dd7587399e4 | Translated (CM) | Elec | 电力 |
| 2981220b09e1-2ba2-44cc-85e9-43f210ac79c2 | Translated (0%) | Ice | 制冰 |
| 29829ea642a6-4911-4baf-b92a-2f171448ced1 | Translated (100%) | 0.1676 kWh/kg ice | 0.1676 kWh/kg 冰 |
| 29836d06fa2c-e7fa-4e4c-a281-4e2bf8a8683e | Translated (CM) | na | na |
| 29842e14168f-89d9-4078-808c-807eb3421b51 | Translated (100%) | 15.57 kWh/100 kg ice | 15.57 kWh/100 lb 冰 |
| 298578501af3-ac4c-4b36-8ad4-bdaa8d3f1724 | Translated (CM) | na | na |
| 298623032058-3898-4816-9eb7-a9fa1e8592ee | Translated (CM) | Chest freezer, solid or glass door | 卧式冷冻柜，实心或玻璃门 |
| 298762bed23b-8e06-45d4-b8ad-fab001af91e6 | Translated (CM) | Elec | 电力 |
| 29885c624f8a-f45c-4818-90a6-43dc340f71d1 | Translated (CM) | Refrig | 制冷 |
| 2989a7caad3d-3f0e-4bbc-beec-2de6e934f844 | Translated (CM) | 15.90V + 0.943 kWh/day | 15.90V + 0.943 kWh/天 |
| 299016f0526b-18e7-486f-af57-9485c38d5b5c | Translated (CM) | na | na |
| 299175653f86-98a0-469a-8ef0-1b8d7e4c607e | Translated (100%) | 9.541V + 0.130 kWh/day | 9.541V + 0.130 kWh/天 |
| 2992dd5adef9-1a19-4e9e-99ee-4e7b286cfdd5 | Translated (CM) | na | na |
| 2993ae372ff7-df70-4b47-90ce-3283f5a46183 | Translated (CM) | Chest refrigerator, solid or glass door | 卧式冰柜，实心或玻璃门 |
| 2994414777f5-9ffe-4812-89e9-cb43ec423169 | Translated (CM) | Elec | 电力 |
| 2995e52a07ae-e5bf-4bae-9a74-051fb46c885f | Translated (CM) | Refrig | 制冷 |
| 29963d3f48e1-95ef-4e35-a545-ad2a8e7c87c6 | Translated (CM) | 3.53V + 2.04 kWh/day | 3.53V + 2.04 kWh/天 |
| 29971f8370e5-9d21-4e6e-8c7c-d277c2ba9594 | Translated (CM) | na | na |
| 29983051b020-c2fd-41c4-9bde-3c02d9addca5 | Translated (CM) | ≤ 4.417 V + 0.475 kWh/day | ≤ 4.417 V + 0.475 kWh/天 |
| 2999873d08c0-9b76-4d49-bf02-cb6ce45352fc | Translated (CM) | na | na |
| 3000a1b430e6-ecbc-4752-89ec-cff3af819dbf | Translated (80%) | Glass-door reach-in freezer, 0 < V < 0.42 m<5604>3</5604> | 手入式玻璃门冰柜，0 < V < 0.42 m<5604>3</5604> |
| 30010466ad55-a9a1-4216-9562-2ef40c2a8a46 | Translated (100%) | Elec | 电力 |
| 30028f69b113-fe10-4ac5-9ab9-846a234d1b1e | Translated (CM) | Refrig | 制冷 |
| 300342b5f8f1-cc84-4f68-af14-c4c177368ad2 | Translated (CM) | 26.50V + 4.1 kWh/day | 26.50V + 4.1 kWh/天 |
| 30047c4ade3b-c096-4aa4-8c9a-9c2685f62aa9 | Translated (CM) | na | na |
| 3005da2226f8-bda5-480b-bd83-63d4111634f8 | Translated (CM) | ≤ 21.449V + 0.893 kWh/day | ≤ 21.449V + 0.893 kWh/天 |
| 3006a7f12eee-2130-4fb7-b2e1-018a6aa574de | Translated (CM) | na | na |
| 3007b6684345-8aec-4862-9fea-026570826ff2 | Translated (99%) | Glass-door reach-in freezer, 0.42 ≤ V < 0.85 m<5626>3</5626> | 手入式玻璃门冰柜，0.42 ≤ V < 0.85 m<5626>3</5626> |
| 30084758aa4b-ecf6-4f67-87f4-e7a60e2b204a | Translated (CM) | Elec | 电力 |
| 3009a9783dd7-e442-4baf-bec1-99d0ddcd87cc | Translated (CM) | Refrig | 制冷 |
| 30109a22222a-dd31-44c4-8650-57007e065f30 | Translated (CM) | 26.50V + 4.1 kWh/day | 26.50V + 4.1 kWh/天 |
| 3011ac9bb55d-3e30-44c6-b38e-c5e9f9a5eae5 | Translated (CM) | na | na |
| 30129af5d2eb-409b-4eba-8784-f482d4e9d21d | Translated (CM) | ≤ 25.901V – 1.00 kWh/day | ≤ 25.901V – 1.00 kWh/天 |
| 3013759490f0-e666-4a4b-b1ed-bd531c0ab96c | Translated (CM) | na | na |
| 301423d56c3c-e1a4-4841-9b9c-2bbec7698ba0 | Translated (CM) | Glass-door reach-in freezer, 0.85 ≤ V < 1.42 m<5648>3</5648> | 手入式玻璃门冰柜，0.85 ≤ V < 1.42 m<5648>3</5648> |
| 301581e59de5-e4a8-446b-9f6d-5e3242044890 | Translated (CM) | Elec | 电力 |
| 3016b15071a6-3f49-4ebb-af64-ae91831f58d1 | Translated (CM) | Refrig | 制冷 |
| 3017e13d767d-43e1-4e54-bd14-55fcee6a57c8 | Translated (CM) | 26.50V + 4.1 kWh/day | 26.50V + 4.1 kWh/天 |
| 30181cffa571-b392-4167-a4e7-8e06d00a37ba | Translated (CM) | na | na |
| 301931d45c70-d470-497d-84cc-e7a8c7864ec6 | Translated (CM) | ≤ 8.834V + 13.50 kWh/day | ≤ 8.834V + 13.50 kWh/天 |
| 30206a7c44af-4dd9-4171-8679-daa2f648f403 | Translated (CM) | na | na |
| 3021af366053-d92f-4742-9137-973b2fb00f2e | Translated (90%) | Glass-door reach-in freezer, 1.42 ≤ V m<5670>3</5670> | 手入式玻璃门冰柜，1.42 ≤ V m<5670>3</5670> |
| 3022fe6f5920-e01e-4791-b0bb-aae665d332ab | Translated (100%) | Elec | 电力 |
| 3023712fa0ff-955c-4de6-8c2c-4944cd599ff4 | Translated (CM) | Refrig | 制冷 |
| 3024e0ea9e35-0fbe-4540-8303-1ad2a2e0e2a1 | Translated (CM) | 26.50V + 4.1 kWh/day | 26.50V + 4.1 kWh/天 |
| 3025e343b85c-2ee2-4a0c-ae16-4e8c4fb7190c | Translated (CM) | na | na |
| 3026c9ca788d-2989-445d-b64c-4a159b80a3fe | Translated (CM) | ≤ 15.90V + 3.50 kWh/day | ≤ 15.90V + 3.50 kWh/天 |
| 3027b94b2b72-7dba-405c-ae03-5ee329689a36 | Translated (CM) | na | na |
| 302850de7b97-4f04-458e-9c14-d47a71f39055 | Translated (91%) | Glass-door reach-in refrigerator, 0 < V < 0.42m<5692>3</5692> | 手入式玻璃门冰箱，0 < V < 0.42m<5692>3</5692> |
| 3029bdf8c9ee-76b1-4698-a7de-7b15446d6a15 | Translated (100%) | Elec | 电力 |
| 30306ae55afa-555c-47d9-9960-ba58ba716002 | Translated (CM) | Refrig | 制冷 |
| 30318d6ec1cd-730c-4462-93de-748201018845 | Translated (CM) | 4.24V + 3.34 kWh/day | 4.24V + 3.34 kWh/天 |
| 3032bbd0045b-d3d5-4d42-b75e-3d6c62653d75 | Translated (CM) | na | na |
| 30331c49f3ae-d1f1-4125-97e4-b0b740c9b2f4 | Translated (CM) | ≤ 4.169V + 1.382 kWh/day | ≤ 4.169V + 1.382 kWh/天 |
| 3034a552aff3-f8e5-4217-8be9-b158970677bd | Translated (CM) | na | na |
| 3035dac3c633-3f96-4f56-a0a5-2544a8222723 | Translated (99%) | Glass-door reach-in refrigerator, 0.42 ≤ V < 0.85 m<5714>3</5714> | 手入式玻璃门冰箱，0.42 ≤ V < 0.85 m<5714>3</5714> |
| 3036bf2aa7b9-b633-4746-bf00-2a00b5637819 | Translated (CM) | Elec | 电力 |
| 3037794496a8-52a3-41d1-8e82-1052ad3800d8 | Translated (CM) | Refrig | 制冷 |
| 303898f9b713-99e8-44ea-a5f1-bf3eedf0e014 | Translated (CM) | 4.24V + 3.34 kWh/day | 4.24V + 3.34 kWh/天 |
| 3039b525b32f-d638-492e-bd83-7a6a0e1227cd | Translated (CM) | na | na |
| 3040d0010aec-d4cc-406b-8b73-cbd2769ac10a | Translated (CM) | ≤ 4.947V + 1.050 kWh/day | ≤ 4.947V + 1.050 kWh/天 |
| 3041a780f632-8f94-4cd0-b2f4-2b5ceffb19a1 | Translated (CM) | na | na |
| 304271af5be0-cdb6-4581-9b17-f6a5cd57d34c | Translated (CM) | Glass-door reach-in refrigerator, 0.85 ≤ V < 1.42 m<5736>3</5736> | 手入式玻璃门冰箱，0.85 ≤ V < 1.42 m<5736>3</5736> |
| 3043e6f27335-571a-4e54-ab03-bae36886895d | Translated (CM) | Elec | 电力 |
| 304454cb9767-f4bf-4b32-add8-85302db50407 | Translated (CM) | Refrig | 制冷 |
| 304575497ffb-d7df-4283-b903-e40fa224e1b6 | Translated (CM) | 4.24V + 3.34 kWh/day | 4.24V + 3.34 kWh/天 |
| 30467dde1e47-8b40-415a-b0b4-23a99fb98869 | Translated (CM) | na | na |
| 30474b27ea91-5347-4140-aab8-e522a9b312d2 | Translated (CM) | ≤ 3.109V + 2.625 kWh/day | ≤ 3.109V + 2.625 kWh/天 |
| 30489f795389-ed84-4b7a-8c9b-e053b61b9034 | Translated (CM) | na | na |
| 30497f7f2916-a747-4336-8eb0-4477c7d5df86 | Translated (91%) | Glass-door reach-in refrigerator, 1.42 ≤ V m<5758>3</5758> | 手入式玻璃门冰箱，1.42 ≤ V m<5758>3</5758> |
| 305061276414-f73b-4386-a85b-10b7eaf836d3 | Translated (100%) | Elec | 电力 |
| 30514f61eda0-6a9d-49bb-8738-56ed8ff0c787 | Translated (CM) | Refrig | 制冷 |
| 30526617a54c-d16a-4754-9814-bc7df250de7b | Translated (CM) | 4.24V + 3.34 kWh/day | 4.24V + 3.34 kWh/天 |
| 305325d0ea00-d3a0-427c-90c8-a27f789e3673 | Translated (CM) | na | na |
| 3054bacb4163-e0b5-4cfe-9757-510ff5ca6cc9 | Translated (CM) | ≤ 3.887V + 1.500 kWh/day | ≤ 3.887V + 1.500 kWh/天 |
| 305554c67126-683c-4240-877c-5ca9a769c6ad | Translated (CM) | na | na |
| 3056d3fe6264-16d0-4691-bad7-35d1b516bd44 | Translated (91%) | Solid-door reach-in freezer, 0 < V < 0.42 m<5780>3</5780> | 手入式实心门冰箱，0 < V < 0.42 m<5780>3</5780> |
| 3057b7daefcd-64c2-4d47-aa9d-36f61a93f93f | Translated (100%) | Elec | 电力 |
| 30583f92b7bb-b9cd-4ee4-8345-8a3fbbd64b50 | Translated (CM) | Refrig | 制冷 |
| 305935ced5d0-29bc-4567-8af6-6e4b54c2af11 | Translated (CM) | 14.13V + 1.38 kWh/day | 14.13V + 1.38 kWh/天 |
| 3060989c231b-366e-4098-be69-48bc91e5d9d2 | Translated (CM) | na | na |
| 306176a900b5-1064-48a1-a88f-0f9f1879850e | Translated (CM) | ≤ 8.834V + 1.25 kWh/day | ≤ 8.834V + 1.25 kWh/天 |
| 3062de0c9f20-829f-49d0-b196-dfdb6ecf491e | Translated (CM) | na | na |
| 306320e574bc-fe3f-4eea-9d6b-dd018626b091 | Translated (99%) | Solid-door reach-in freezer, 0.42 ≤ V < 0.85 m<5802>3</5802> | 手入式实心门冰柜，0.42 ≤ V < 0.85 m<5802>3</5802> |
| 30647fb08ad3-c6c2-4070-a7a6-2a2fca037827 | Translated (CM) | Elec | 电力 |
| 30650332ca3b-b26a-4191-929e-8b9fb125e49b | Translated (CM) | Refrig | 制冷 |
| 306620b6f208-f3a8-4899-82e4-d32eb4a135b6 | Translated (CM) | 14.13V + 1.38 kWh/day | 14.13V + 1.38 kWh/天 |
| 306736447468-5e37-431b-9d59-09cc651561cb | Translated (CM) | na | na |
| 3068086c221c-3759-42be-b046-e2a31ee74358 | Translated (CM) | ≤ 4.819V – 1.000 kWh/day | ≤ 4.819V – 1.000 kWh/天 |
| 3069b165aa05-ba15-472d-b067-21c867fa19c4 | Translated (CM) | na | na |
| 307020fd73b8-6306-4215-8de7-884742da6a54 | Translated (CM) | Solid-door reach-in freezer, 0.85 ≤ V < 1.42 m<5824>3</5824> | 手入式实心门冰柜，0.85 ≤ V < 1.42 m<5824>3</5824> |
| 30716ce30105-e312-4644-b29c-5b153bc8e2b8 | Translated (CM) | Elec | 电力 |
| 30722262e706-7d40-4d49-9e3f-6fca80cd1f4a | Translated (CM) | Refrig | 制冷 |
| 30737bb7b4a1-ced6-4a48-8117-e72b2cfa7510 | Translated (CM) | 14.13V + 1.38 kWh/day | 14.13V + 1.38 kWh/天 |
| 3074d26998ae-8ca3-4cd1-a9a4-1ad74931de2d | Translated (CM) | na | na |
| 3075b88b0bb4-b61e-4f7c-85f4-b6896117ee48 | Translated (CM) | ≤ 5.760V + 6.125 kWh/day | ≤ 5.760V + 6.125 kWh/天 |
| 30762fd1f449-668a-4b76-ac19-625acdcf2636 | Translated (CM) | na | na |
| 30773b32ce1e-cdc8-4c03-8e3b-d3569f9dab2d | Translated (91%) | Solid-door reach-in freezer, 1.42 ≤ V m<5846>3</5846> | 手入式实心门冰柜，1.42 ≤ V m<5846>3</5846> |
| 30787e118e46-e331-46e9-b338-0ea8821ff225 | Translated (100%) | Elec | 电力 |
| 30790a5f5795-0c5c-4425-9acf-5f4e0c6c5485 | Translated (CM) | Refrig | 制冷 |
| 3080441d60f7-bc50-4008-80cc-467429c743a8 | Translated (CM) | 14.13V + 1.38 kWh/day | 14.13V + 1.38 kWh/天 |
| 3081217e35f1-1037-4673-8a85-4494303927b1 | Translated (CM) | na | na |
| 308200f072a3-37f5-4bc7-ae95-cd8c1ca31eef | Translated (CM) | ≤ 5.583V + 6.333 kWh/day | ≤ 5.583V + 6.333 kWh/天 |
| 3083817e6549-dce1-418a-a71d-c55c9d451ab9 | Translated (CM) | na | na |
| 3084803d1a93-d244-4bae-8d27-853433536ad4 | Translated (91%) | Solid-door reach-in refrigerator, 0 < V < 0.42m<5868>3</5868> | 手入式实心门冰箱，0 < V < 0.42m<5868>3</5868> |
| 3085e709ac6a-7cb7-4da6-920e-8183673eb905 | Translated (100%) | Elec | 电力 |
| 308662ca5ac7-c422-4b6d-9e42-b1374d2ff15d | Translated (CM) | Refrig | 制冷 |
| 3087563add8b-3879-44ec-8cef-21aa03107074 | Translated (CM) | 3.53V + 2.04 kWh/day | 3.53V + 2.04 kWh/天 |
| 3088a4465857-d88c-49af-97d8-de3f9d1f3274 | Translated (CM) | na | na |
| 3089af60cab0-73ec-4c0b-84d5-a0ed950ffea0 | Translated (CM) | ≤ 3.145V + 1.411 kWh/day | ≤ 3.145V + 1.411 kWh/天 |
| 30904cb67434-d060-41f0-a0f7-42098ddb8855 | Translated (CM) | na | na |
| 3091a205d9b0-4bcb-464d-97c7-20d1fc26b539 | Translated (99%) | Solid-door reach-in refrigerator, 0.42 ≤ V < 0.85 m<5890>3</5890> | 手入式实心门冰箱，0.42 ≤ V < 0.85 m<5890>3</5890> |
| 3092e048826a-1bd4-4eff-8f10-735005a23199 | Translated (CM) | Elec | 电力 |
| 30933444912c-76ad-4ea7-b02d-edc17d08527c | Translated (CM) | Refrig | 制冷 |
| 30940e2e7e47-b47a-4e64-bd7f-c63d36eb8624 | Translated (CM) | 3.53V + 2.04 kWh/day | 3.53V + 2.04 kWh/天 |
| 30956e07e898-b78c-4c84-bbb4-be0202bbe845 | Translated (CM) | na | na |
| 3096a265f8d5-2e8e-421b-bc79-7e231272f4f4 | Translated (CM) | ≤ 1.307V + 2.200 kWh/day | ≤ 1.307V + 2.200 kWh/天 |
| 3097adaf7316-8932-4d65-b9a1-9894c9b0fffb | Translated (CM) | na | na |
| 3098a09105db-ac49-45a0-8471-f07309fa4d3a | Translated (CM) | Solid-door reach-in refrigerator, 0.85 ≤ V < 1.42 m<5912>3</5912> | 手入式实心门冰箱，0.85 ≤ V < 1.42 m<5912>3</5912> |
| 309963b60819-93a0-4894-89b3-d88baa74d425 | Translated (CM) | Elec | 电力 |
| 3100e15e01d0-eaae-42cc-b91a-26ab1b68dfd1 | Translated (CM) | Refrig | 制冷 |
| 31017dbc47d2-a6dc-4959-9bda-edfd2a133d42 | Translated (CM) | 3.53V + 2.04 kWh/day | 3.53V + 2.04 kWh/天 |
| 3102a289c448-eb7b-496a-8807-a23860248c1e | Translated (CM) | na | na |
| 31038a69d3cb-7d32-4880-bcfd-9a6e0c8c119b | Translated (CM) | ≤ 1.979V + 1.635 kWh/day | ≤ 1.979V + 1.635 kWh/天 |
| 3104ec859e0a-3c96-47c5-a481-340fa7f48223 | Translated (CM) | na | na |
| 3105ad76a85e-dae5-4bf9-8a8f-bf441cb7d71e | Translated (91%) | Solid-door reach-in refrigerator, 1.42 ≤ V m<5934>3</5934> | 手入式实心门冰箱，1.42 ≤ V m<5934>3</5934> |
| 3106bd0c3ac8-b715-4e68-ad22-1fb1da926d9c | Translated (100%) | Elec | 电力 |
| 3107a79f8d93-4abb-433f-9e4e-177b33b1b8cc | Translated (CM) | Refrig | 制冷 |
| 310880d41bc6-cb39-4331-80c3-3bd2540fa093 | Translated (CM) | 3.53V + 2.04 kWh/day | 3.53V + 2.04 kWh/天 |
| 31099e4b9b8a-240a-4239-8dfa-a471d45fe02e | Translated (CM) | na | na |
| 3110f0df8d5b-909b-453a-950f-a1296da64e90 | Translated (CM) | ≤ 2.120V + 1.416 kWh/day | ≤ 2.120V + 1.416 kWh/天 |
| 3111f73428e6-0cf4-425c-823d-84cb22ee0afe | Translated (CM) | na | na |
| 311251e24f7e-493c-44c2-870f-ed126b9b2f5e | Translated (CM) | Clothes washer | 洗衣机 |
| 3113d572eaea-1e1a-41c9-be28-cb8f5b959496 | Translated (CM) | Gas | 天然气 |
| 31145ff74f9e-becd-41ee-b615-a9bb756e385c | Translated (CM) | Sanitation | 卫生 |
| 31158443ece7-0c0f-42f9-97c8-f28c537f5620 | Translated (100%) | 1.72 MEF | 1.72 MEF |
| 31161fddf36b-3aac-4a3f-9842-a514c15b3c59 | Translated (100%) | 2.00 MEF | 2.00 MEF |
| 311703c22077-bc3a-44c5-b00c-32e973fbfcaf | Translated (100%) | Door-type dish machine, high temp | 门式洗碗机，高温 |
| 31186ffdd538-58f3-44ff-afa2-c6fa5bf20d80 | Translated (CM) | Elec | 电力 |
| 3119f891ff29-84dd-4814-a0ce-88e3ac8b2e86 | Translated (CM) | Sanitation | 卫生 |
| 31204a532d21-2eb7-478a-929a-3d1c875b0ad7 | Translated (CM) | na | na |
| 3121047b6219-e81b-4a75-80e5-59586e1cfb9c | Translated (CM) | 1.0 kW | 1.0 kW |
| 3122203b8c9c-6cb8-4820-bf9f-1754510250ea | Translated (CM) | na | na |
| 31237e1c64df-120c-4912-bbc4-23d890d7ca44 | Translated (CM) | 0.70 kW | 0.70 kW |
| 3124eaf8edda-1966-4bc9-a37a-e79fba5f2983 | Translated (CM) | Door-type dish machine, low temp | 门式洗碗机，低温 |
| 31258a45d0e6-3a89-49b3-bf83-68e6b0a301c4 | Translated (CM) | Elec | 电力 |
| 3126be2c1ed7-f635-42b3-aebe-3488b60d0670 | Translated (CM) | Sanitation | 卫生 |
| 3127c4b7635c-f9fb-45d4-a910-f330f8ff6a9c | Translated (CM) | na | na |
| 312864fda67e-8680-4437-9b7e-86f582622782 | Translated (CM) | 0.6 kW | 0.6 kW |
| 312985b5a319-b0cd-44aa-9bde-efd5803b40d9 | Translated (CM) | na | na |
| 31303f3298dc-6f8e-4da2-a162-92c26a26a639 | Translated (CM) | 0.6 kW | 0.6 kW |
| 31318cdbf17a-7dfe-4cd5-b6bf-73c7a4f57544 | Translated (CM) | Multitank rack conveyor dish machine, high temp | 多缸架传送式洗碗机，高温 |
| 313263e56a2d-c67e-4443-991b-ab24db69a65d | Translated (CM) | Elec | 电力 |
| 3133153f9256-bc7b-4da1-a5db-3fe8c2e3d4bf | Translated (CM) | Sanitation | 卫生 |
| 3134c34f67e1-ffd0-43d9-b006-4b234ef4d6b6 | Translated (CM) | na | na |
| 313539b436de-462a-45a6-a884-ce0f94a5d5f2 | Translated (CM) | 2.6 kW | 2.6 kW |
| 31363bb6783c-9a10-417b-87cd-9ce84ca7c2b2 | Translated (CM) | na | na |
| 31374a78c24a-941e-4182-a0e6-f6a093365bf1 | Translated (CM) | 2.25 kW | 2.25 kW |
| 313856706b15-36bc-4aef-917d-4aac9d84ef5a | Translated (CM) | Multitank rack conveyor dish machine, low temp | 多缸架传送式洗碗机，低温 |
| 31394c84d79f-b448-4958-b3ce-144a59e63e4b | Translated (CM) | Elec | 电力 |
| 31401a741baf-c4d1-4c7a-858f-363f09732732 | Translated (CM) | Sanitation | 卫生 |
| 3141674f35b8-f443-4673-884e-676d9f1ff104 | Translated (CM) | na | na |
| 3142a16d90e0-b44c-4840-a4c0-d6f011d6f87a | Translated (CM) | 2.0 kW | 2.0 kW |
| 3143dadb6ca3-8d4e-42ea-a55b-570f414750a3 | Translated (CM) | na | na |
| 31449d76c69e-1234-4de0-a915-1cf1aadd4657 | Translated (CM) | 2.0 kW | 2.0 kW |
| 31459500ce6f-0a04-4e08-8482-8778e364a759 | Translated (CM) | Single-tank rack conveyor dish machine, high temp | 单缸架传送式洗碗机，高温 |
| 3146a523584a-d09c-4edc-b753-026d8facb0e7 | Translated (CM) | Elec | 电力 |
| 314747032676-315e-4d7f-8913-65a9dc812b9c | Translated (CM) | Sanitation | 卫生 |
| 3148969d8de8-bf2b-4582-bcb5-d059827c1b34 | Translated (CM) | na | na |
| 31493251efed-3376-4c1d-a40c-83ee879e6a2e | Translated (0%) | 2.0 kW | 2.0 kW |
| 3150d2a733a2-52a9-478f-8694-e1585eaf4973 | Translated (0%) | na | na |
| 31512eda9a07-90cf-432c-a0c9-d7ef80370881 | Translated (CM) | 1.5 kW | 1.5 kW |
| 31526a9018ea-67e3-4dc5-bb39-280a6335322d | Translated (CM) | Single-tank rack conveyor dish machine, low temp | 单缸架传送式洗碗机，低温 |
| 31539d23e236-fb49-4290-9489-ce0f72d11906 | Translated (CM) | Elec | 电力 |
| 315497f6596f-c038-4629-8d30-1788003340df | Translated (CM) | Sanitation | 卫生 |
| 31552df0a4a2-8a00-4dae-9129-4695ca3e5c0c | Translated (CM) | na | na |
| 3156bb591a9b-e1a2-4b39-a5fe-a461b52c484a | Translated (CM) | 1.6 kW | 1.6 kW |
| 31574e398c1e-edea-44cf-a3e1-374e75d3486a | Translated (CM) | na | na |
| 315874a3b5bc-cbd1-4350-a887-4eb27c839f1b | Translated (CM) | 1.5 kW | 1.5 kW |
| 31596dd9e59b-bfc9-4402-811a-a62f07183c42 | Translated (CM) | Undercounter dish machine, high temp | 台下式洗碗机，高温 |
| 31606495d9ef-a470-4130-afea-f1f5038d5016 | Translated (CM) | Elec | 电力 |
| 316128362414-8b00-4800-981b-00e0a1295e48 | Translated (CM) | Sanitation | 卫生 |
| 316230b5d38d-7e0a-42d6-85bb-7c3d2c5c9ee4 | Translated (CM) | na | na |
| 3163b87a9e5f-b025-4050-a30d-63f43c5ee200 | Translated (CM) | 0.9 kW | 0.9 kW |
| 3164cd6a9f13-c90f-40f1-a885-37178db48dfa | Translated (CM) | na | na |
| 3165d0ec4204-0ca2-440c-bdb1-9ca0f3c45eab | Translated (CM) | 0.5 kW | 0.5 kW |
| 316676d70e20-026a-4f34-8d15-5d2a1bbb1eb7 | Translated (CM) | Undercounter dish machine, low temp | 台下式洗碗机，低温 |
| 3167831cf6a3-fde8-4431-9ae4-c788de44d140 | Translated (CM) | Elec | 电力 |
| 316827925052-ec57-4b91-ab4e-b140573f7de3 | Translated (CM) | Sanitation | 卫生 |
| 3169483d7d95-88b0-4a6a-924d-2316ababc5bd | Translated (CM) | na | na |
| 31701981441e-2e29-4e5a-9c47-e0f194904e09 | Translated (CM) | 0.5 kW | 0.5 kW |
| 3171e26f2c09-3b8d-477c-ae59-47c6fbd856f3 | Translated (CM) | na | na |
| 3172892cfb6d-b4c4-4b02-b1ca-4e689e0124fe | Translated (CM) | 0.5 kW | 0.5 kW |
| 317347f280fa-c80c-4758-a6d8-6475cc91b7b3 | Translated (CM) | The energy efficiency, idle energy rates, and water use requirements, where applicable, are based on the following test methods: | 能效、闲时功率和用水要求（如适用）均基于以下测试方法： |
| 3174ceb2d8d8-da63-4784-8aa6-ec3d4e01a534 | Translated (CM) | ASTM F1275 Standard Test Method for Performance of Griddles | ASTM F1275 煎锅性能标准测试方法 |
| 3175f0c9e299-1644-439b-a05b-a38b0d8db316 | Translated (CM) | ASTM F1361 Standard Test Method for Performance of Open Deep Fat Fryers | ASTM F1361 敞口式深底油炸锅性能标准测试方法 |
| 3176495dd46a-ccd6-4d66-9910-ee87a9360901 | Translated (CM) | ASTM F1484 Standard Test Methods for Performance of Steam Cookers | ASTM F1484 蒸汽炊具性能标准测试方法 |
| 31778f74c8e7-0aa4-4450-8ac5-e5f4d56a60dd | Translated (CM) | ASTM F1496 Standard Test Method for Performance of Convection Ovens | ASTM F1496 对流烤箱性能标准测试方法 |
| 3178bcba6495-445f-4b0c-88ea-c8b29d925ed9 | Translated (CM) | ASTM F1521 Standard Test Methods for Performance of Range Tops | ASTM F1521 炉灶盖性能标准测试方法 |
| 31796c3c4c97-64a2-4b16-a7d2-f963bfed559c | Translated (CM) | ASTM F1605 Standard Test Method for Performance of Double-Sided Griddles | ASTM F1605 双面煎锅性能标准测试方法 |
| 3180ff0d74cc-f720-4fe2-80f4-f1c6c88131d1 | Translated (CM) | ASTM F1639 Standard Test Method for Performance of Combination Ovens | ASTM F1639 组合烤箱性能标准测试方法 |
| 3181430160f6-2186-4a28-8a31-89b4e4dc84fa | Translated (CM) | ASTM F1695 Standard Test Method for Performance of Underfired Broilers | ASTM F1695 底燃式锅炉性能标准测试方法 |
| 3182d754a6d1-093a-454a-8f2f-60b45e3648f6 | Translated (CM) | ASTM F1696 Standard Test Method for Energy Performance of Single-Rack Hot Water Sanitizing, ASTM Door-Type Commercial Dishwashing Machines | ASTM F1696 单架热水消毒，ASTM 门式商用洗碗机能效标准测试方法 |
| 31838c148474-e073-4be1-b05b-5bbd1d5490ba | Translated (CM) | ASTM F1704 Standard Test Method for Capture and Containment Performance of Commercial Kitchen Exhaust Ventilation Systems | ASTM F1704 商用厨房排烟通风系统捕捉和容纳性能标准测试方法 |
| 318454cf717d-0124-4042-acb2-726bab0ca8fe | Translated (CM) | ASTM F1817 Standard Test Method for Performance of Conveyor Ovens | ASTM F1817 链条式平炉性能标准测试方法 |
| 318554768ccf-4394-4e6a-9d7c-ae774e303097 | Translated (CM) | ASTM F1920 Standard Test Method for Energy Performance of Rack Conveyor, Hot Water Sanitizing, Commercial Dishwashing Machines | ASTM F1920 架传送式热水消毒商用洗碗机能效标准测试方法 |
| 31862be0c5dd-f84f-496e-9842-b4dd23993902 | Translated (CM) | ASTM F2093 Standard Test Method for Performance of Rack Ovens | ASTM F2093 搁架式烤炉性能标准测试方法 |
| 31876c6f5c3d-76e7-492a-b88a-9faf7f9e9e0c | Translated (CM) | ASTM F2140 Standard Test Method for Performance of Hot Food Holding Cabinets | ASTM F2140 热食物储藏柜性能标准测试方法 |
| 3188bc2fb134-6704-45bc-92a7-82578c2373da | Translated (CM) | ASTM F2144 Standard Test Method for Performance of Large Open Vat Fryers | ASTM F2144 敞口式大桶炸锅性能标准测试方法 |
| 3189bc4a80fc-ac28-4a24-89d4-c67d475a596c | Translated (CM) | ASTM F2324 Standard Test Method for Prerinse Spray Valves | ASTM F2324 预清洗喷雾阀标准测试方法 |
| 3190f63b5087-a185-40ac-8c52-cfc617ec13df | Translated (CM) | ASTM F2380 Standard Test Method for Performance of Conveyor Toasters | ASTM F2380 链条式烤面包机性能标准测试方法 |
| 31912f8e0de6-3bc6-400e-bcd3-8805a1d93106 | Translated (CM) | ARI 810-2007: Performance Rating of Automatic Commercial Ice Makers | ARI 810-2007：商用自动制冰机性能等级 |
| 319209ed229d-e399-4fd9-ac9e-c00ad3b464b7 | Translated (97%) | ANSI/ASHRAE Standard 72–2005: Method of Testing Commercial Refrigerators and Freezers with temperature setpoints at 38°F (3°C) for mediumtemp refrigerators, -18°C for low-temp freezers, and -26°C for ice cream freezers. | ANSI/ASHRAE 标准 72–2005：商用冰箱和冰柜测试方法，中等温度冰箱的温度设定值为 38°F (3°C)，低温冰柜为 -18°C，冰淇淋冷柜为 -26°C。 |
| 31939d799208-e539-4069-9bd4-5e1005676590 | Translated (100%) | Table 2. | 表 2. |
| 31949d799208-e539-4069-9bd4-5e1005676590 | Translated (0%) | Supermarket refrigeration prescriptive measures and baseline for energy cost budget | 超市制冷的规范性指标和能源费用预算基线 |
| 31952f1215d8-51fe-410a-bd5c-7cf9d21da7d3 | Translated (100%) | Item | 项目 |
| 31963ccea164-dcaa-460d-a386-22ad3aced0dd | Translated (0%) | Attribute | 属性 |
| 31973d7cdbfb-8479-4fbb-a682-1dbfef80fb99 | Translated (0%) | Prescriptive measure | 规范性指标 |
| 31983d412da4-a7cf-4db3-9819-652d3d9387c6 | Translated (76%) | Baseline for energy modeling path | 能耗模拟途径的基线 |
| 31992d55c645-ec7b-4e95-8e65-2a4ee2ab069a | Translated (0%) | Commercial Refrigerator and Freezers | 商用冰箱和冰柜 |
| 32000b457ce7-3460-4666-a30e-cf0af5fcd7ed | Translated (77%) | Energy Use Limits | 能耗限制 |
| 32010dcc6a23-37c7-4e26-bdb4-831e9e156f78 | Translated (0%) | ASHRAE 90.1-2010 Addendum g. Table 6.8.1L | ASHRAE 90.1-2010 附录 g. 表 6.8.1L |
| 3202cab4f9fb-8c53-4383-b4dd-9f24db28fc1e | Translated (100%) | ASHRAE 90.1-2010 Addendum g. Table 6.8.1L | ASHRAE 90.1-2010 附录 g. 表 6.8.1L |
| 3203a5cac9c0-1717-4fac-b52c-ff3b7b68b9f4 | Translated (73%) | Commercial Refrigeration Equipment | 商用制冷设备 |
| 3204edf60da1-cbd2-456d-8a95-e65b4681faca | Translated (100%) | Energy Use Limits | 能耗限制 |
| 32057fca3886-9691-43b0-a1b5-2d8f3cf852a6 | Translated (92%) | ASHRAE 90.1-2010 Addendum g. Table 6.8.1M | ASHRAE 90.1-2010 附录 g. 表 6.8.1M |
| 3206b0fecc18-7e0d-4d41-bc50-eb138e75625c | Translated (100%) | ASHRAE 90.1-2010 Addendum g. Table 6.8.1M | ASHRAE 90.1-2010 附录 g. 表 6.8.1M |
| 3207c4eb7e50-265a-46e0-890d-2904cc40a9ff | Translated (100%) | Table 3. | 表 3. |
| 3208c4eb7e50-265a-46e0-890d-2904cc40a9ff | Translated (75%) | Walk-in coolers and freezers prescriptive measures and baseline for energy cost budget | 步入式制冷机和冰柜的规范性指标和能源费用预算基线 |
| 3209044691cb-be66-43f4-b49b-1f791be33f7b | Translated (100%) | Item | 项目 |
| 32102ef1ca67-6a62-477d-8b6e-03413f3f9b66 | Translated (CM) | Attribute | 属性 |
| 321184e0c4e9-bb43-4237-aefe-535e633f1a78 | Translated (CM) | Prescriptive measure | 规范性指标 |
| 3212d2868808-1614-467c-b59b-43386c8ee715 | Translated (CM) | Baseline for energy modeling path | 能耗模拟途径的基线 |
| 321365e32fb9-1c55-4783-8c9b-e70de342c082 | Translated (0%) | Envelope | 外围护结构 |
| 321428943ce3-a445-46d5-80b0-bfb880b05150 | Translated (0%) | Freezer insulation | 冰柜保温 |
| 321532796bcb-9a85-41f1-a8fb-1754851802ef | Translated (0%) | R-46 | R-46 |
| 32161cc6d3f4-0d96-4428-b002-15f6b3558e9b | Translated (0%) | R-36 | R-36 |
| 32172379db20-c802-40cd-9ce4-0a970682cdbe | Translated (0%) | Cooler insulation | 制冷机保温 |
| 321856583ed9-c1ad-4950-b0f8-a91fadc3bae0 | Translated (0%) | R-36 | R-36 |
| 32191c6f1aa5-bdf8-4118-b048-ca4e231cd755 | Translated (0%) | R-20 | R-20 |
| 3220b7d407af-235a-48a4-92c0-10236758c8a0 | Translated (0%) | Automatic closer doors | 自动闭门装置 |
| 3221acd26883-a9fa-47b8-bdac-653f61552156 | Translated (100%) | Yes | 有 |
| 32227cdfabfc-fac5-41da-acf7-d3136132e8fa | Translated (100%) | No | 无 |
| 3223ee003fc6-7507-41d8-bc95-f65e989e3cb6 | Translated (0%) | High-efficiency low- or no-heat reach-in doors | 高效低热或无热手入式门 |
| 3224d0e1f7bb-0583-40be-9112-baeaa400cd49 | Translated (0%) | 40W/ft (130W/m) of door frame (low temperature), 17W/ft (55W/m) of door frame (medium temperature) | 40W/ft (130W/m) 门框（低温），17W/ft (55W/m) 门框（中等温度） |
| 322563a51f72-394a-46f0-b99e-6fd8d38fee33 | Translated (100%) | 40W/ft (130W/m) of door frame (low temperature), 17W/ft (55W/m) of door frame (medium temperature) | 40W/ft (130W/m) 门框（低温），17W/ft (55W/m) 门框（中等温度） |
| 3226a5c5bc27-4d9e-4136-a473-ad5add31da95 | Translated (0%) | Evaporator | 蒸发器 |
| 32273359bde9-feaa-44c6-be8f-5df0feb8cd77 | Translated (0%) | Evaporator fan motor and control | 蒸发器风扇电机和控制 |
| 32281a3c25e9-48a7-4c0b-8e12-e86ee5d56300 | Translated (0%) | Shaded pole and split phase motors prohibited; use PSC or EMC motors | 禁止罩极式和分相电动机；商用 PSC 或 EMC 电机 |
| 3229d57296df-3c54-48d7-ae1d-b9684c7786c1 | Translated (0%) | Constant-speed fan | 定速风扇 |
| 323032ca2783-f01d-4bd7-aa75-cce7c8a81a6d | Translated (0%) | Hot gas defrost | 热气除霜 |
| 32319f5bbe34-25b9-4222-b2f9-22d44fdab018 | Translated (0%) | No electric defrosting. | 无电动除霜。 |
| 3232c5b2d990-80c0-4a52-ae29-d71f483e1b8a | Translated (0%) | Electric defrosting | 电动除霜 |
| 3233ea272035-b1a0-4785-85f6-7a2269275f42 | Translated (0%) | Condenser | 冷凝器 |
| 3234ccaf4e5f-364a-4618-8adf-20997f36f7f4 | Translated (74%) | Air-cooled condenser fan motor and control | 气冷冷凝器风扇电机和控制 |
| 32357c0d23c2-bcaa-4cb9-8299-30af289d16d1 | Translated (79%) | Shaded pole and split phase motors prohibited; use PSC or EMC motors; add condenser fan controllers | 禁止罩极式和分相电动机；商用 PSC 或 EMC 电机；添加冷凝器风扇控制器 |
| 3236b39b1ed5-27b0-404b-b466-44d84c247932 | Translated (0%) | Cycling one-speed fan | 循环单速风扇 |
| 323779d572d2-8c4b-4dcf-b216-bf36bdac8fab | Translated (0%) | Air Cooled condenser design approach | 气冷冷凝器设计方法 |
| 323835273a17-e9f6-48c2-a7c7-f7f5949f2696 | Translated (0%) | Floating head pressure controls or ambient subcooling | 浮头压力控制或环境低温冷却 |
| 3239973dda6b-3804-49d7-bccc-2b5e0577cfc1 | Translated (0%) | 10°F (-12°C) to 15°F | 10°F (-12°C) 到 15°F |
| 32400fdf6abd-020d-4825-bbf5-6fb6b35f0546 | Translated (0%) | (-9°C) dependent on suction temperature | (-9°C) 依赖于吸气温度 |
| 32418f779f48-46ab-4138-901b-ac9d02b9bec0 | Translated (100%) | Lighting | 照明 |
| 32426cf53d58-1ffe-4f08-9b6f-5cd673ee34ba | Translated (0%) | Lighting power density (W/sq.ft.) | 照明功率密度 (W/平方英尺) |
| 324398d72daa-cdbb-4cc0-b37c-d1cfc8a248ce | Translated (0%) | 0.6 W/sq.ft. | 0.6 W/平方英尺 |
| 3244696109bf-79ee-4c0b-a6a7-fcf044bf896f | Translated (0%) | (6.5 W/sq. meter) | (6.5 W/平方米) |
| 324537a61120-4fd5-4556-8217-4ae5e259aa86 | Translated (100%) | 0.6 W/sq.ft. | 0.6 W/平方英尺 |
| 324635217811-36b2-4625-9a62-e6b5007e1699 | Translated (CM) | (6.5 W/sq. meter) | (6.5 W/平方米) |
| 3247830964b2-d8a0-4b69-8d65-4da8624edd75 | Translated (100%) | Commercial Refrigerator and Freezers | 商用冰箱和冰柜 |
| 3248181d6c77-2b16-453f-bde4-9047d676b3c8 | Translated (CM) | Energy Use Limits | 能耗限制 |
| 32499b81b91a-164b-4b67-af16-5508647bbdb0 | Translated (100%) | N/A | 不适用 |
| 3250f4c6866b-c033-4961-a9d9-dc5bc3bc3388 | Translated (0%) | Use an Exceptional Calculation Method if attempting to take savings | 如果尝试实现节约，则使用特殊计算方法 |
| 3251f7e2aa37-a040-4a1b-826b-790ee5834e02 | Translated (100%) | Commercial Refrigerator and Freezers | 商用冰箱和冰柜 |
| 32521a5f1c9a-2d1d-4ce9-aab9-64681efe3e35 | Translated (100%) | Energy Use Limits | 能耗限制 |
| 3253053dcc03-e243-47ba-ab1a-a1e1b5a175ce | Translated (100%) | N/A | 不适用 |
| 32549d979539-a84a-4935-b068-c6023e8f14e7 | Translated (CM) | Use an Exceptional Calculation Method if attempting to take savings | 如果尝试实现节约，则使用特殊计算方法 |
| 325581ed5d27-e0de-41b1-977f-f007c1bb9958 | Translated (100%) | Table 4. | 表 4. |
| 325681ed5d27-e0de-41b1-977f-f007c1bb9958 | Translated (82%) | Commercial kitchen ventilation prescriptive measures and baseline for energy cost budget | 商用厨房通风的规范性指标和能源费用预算基线 |
| 3257bfd5c235-6074-4f04-af4f-05854859927b | Translated (100%) | Strategies | 策略 |
| 3258b598ec14-0143-40b8-9b55-185e96bffb1c | Translated (100%) | Prescriptive measure | 规范性指标 |
| 32599d9af698-2a0d-455d-9f4a-06dcb8726973 | Translated (0%) | Baseline | 基线 |
| 3260fce82e91-74c4-4284-bf09-ea031cf57fc0 | Translated (0%) | Kitchen hood control | 厨房油烟控制 |
| 32616f7b7543-faff-4800-b049-7861ae35387b | Translated (0%) | ASHRAE 90.1-2010 Section 6.5.7.1, except that Section 6.5.7.1.3 and Section 6.5.7.1.4 shall apply if the total kitchen exhaust airflow rate exceeds 2,000 cfm (960 L/s) (as opposed to 5,000 cfm (2,400 L/s) noted in the ASHRAE 90.1-2010 requirements) | 如果厨房总排气气流速度超过 2,000 cfm (960 L/s)（相对于 ASHRAE 90.1-2010 要求中注明的 5,000 cfm (2,400 L/s)），ASHRAE 90.1-2010 第 6.5.7.1 部分（第 6.5.7.1.3 部分和第 6.5.7.1.4 部分除外）将适用。 |
| 32621801149c-6201-43ea-bffa-26850e1ef062 | Translated (0%) | ASHRAE 90.1-2010 Section 6.5.7.1 and Section G3.1.1 Exception (d) where applicable | ASHRAE 90.1-2010 第 6.5.7.1 部分和第 G3.1.1 部分例外情况 (d)（如适用） |